COMPETITIVE AND SUSTAINABLE GROWTH

(GROWTH)

PROGRAMME

Deliverable D16: Policy perspectives

Version 1.7

August 2003

Authors: Markus Maibach, Chris Nash, Peter Mackie,  with contributions from partners

Contract: 1999-AM.11157 Project Coordinator: ITS, University of Leeds Funded by the European Commission 5th Framework – Transport RTD

UNITE Partner Organisations ITS/UNIVLEEDS (UK), DIW (De), NEI (Nl), CES/KUL (Be), TIS.PT (Pt), IWW/UNIKARL (De), VTI (Se), IER/USTUTT (De), CERAS/ENPC (Fr), HERRY (Au), EIET/ULPGC (Es), ISIS (It), STRATEC (Be), SYSTEMA (Gr), VATT (Fi), ECOPLAN (Ch), INFRAS (Ch), EKONO (Fi), EKI (Se)

UNITE

1999-AM.11157

UNIfication of accounts and marginal costs for Transport Efficiency

Deliverable 16: Policy perspectives

This document should be referenced as:

August 2003

Version No: 	1.7

Authors:	Markus Maibach, Chris Nash, Peter Mackie, Esko Niskanen, with contributions from partners

PROJECT INFORMATION

Contract no: 1999-AM.11157:

UNIfication of accounts and marginal costs for Transport Efficiency

Website:	www.its.leeds.ac.uk/UNITE

Commissioned by: European Commission – DG TREN; Fifth Framework Programme

Lead Partner: Institute for Transport Studies, University of Leeds (UK)

Partners:   ITS/UNIVLEEDS (UK), DIW (De), NEI (Nl), CES/KUL (Be), TIS.PT (Pt), IWW/UNIKARL (De), VTI (Se), IER/USTUTT (De), CERAS/ENPC (Fr), HERRY (Au), EIET/ULPGC (Es), ISIS (It), STRATEC (Be), SYSTEMA (Gr), VATT (Fi), ECOPLAN (Ch), INFRAS (Ch), EKONO (Fi), EKI (Se)

DOCUMENT CONTROL INFORMATION

Status: 			Internal draft

Distribution:			European Commission, Advisory Board, Partners

Availability:			Not for circulation

Filename:			UNITE D16 final version

Quality assurance:

Co-ordinator’s review:

Signed:          			   			Date:

Table of Contents

1	Introduction	1

2	The policy issues	1

2.2	Overview	1

2.3	Marginal costs	2

2.4	Accounts	3

2.5	Integration	4

3	Marginal Cost Research	4

3.1	Introduction	4

3.2	Methodology	5

3.2.1.	Infrastructure costs	5

3.2.2.	Supplier operating cost	7

3.2.3.	User cost	7

3.2.4.	Accident cost	10

3.2.5.	Environment	11

3.3	Empirical results	12

3.4	Conclusions	13

4	Accounts	23

4.1	The use of accounts	23

4.2	Elaboration of accounts	25

4.2.1.	Templates for costs and revenues	26

4.2.2.	Estimation and valuation of costs and revenues	29

4.2.3.	Relevant Indicators	31

4.3	Interpretation of UNITE results	32

4.3.1.	Strategic monitoring	32

4.3.2.	Monitoring progress towards sustainable transport	36

4.3.3.	Monitoring financial viability	39

4.3.4.	Monitoring Equity	43

4.3.5.	Monitoring Efficiency of Pricing Schemes	44

4.4	Future development of accounts	47

5	Integration	49

5.1	Alternative concepts of integration	49

5.2	Integration of transport pricing within the macro-economic context	50

6	Overall conclusions	53

7	References	56

# Introduction

Throughout the UNITE programme we have been concerned to liaise with policy makers so as to ensure that the work was relevant to the real world of policy.  We twice addressed the DG TREN committee of Government experts on transport infrastructure charging and heard from them about the pricing initiatives being taken at member state level.  Throughout the project, we have had an advisory panel (G. Ericsson, T. Howes, P. Tena, T. Worsley) who have given their views and expertise at key stages in the project. We also carried out interviews within the various modal directorates within DG TREN.

Three years ago, when we submitted the proposal for UNITE, we defined the work programme around three core aspects – transport accounts, marginal costs, and integration of approaches.  Now, we have developed a set of pilot transport accounts for the member states, we have a series of marginal cost case studies investigating the various elements of marginal costs in a range of contexts.  We have reviewed the prospects for and methods of generalising the marginal cost results, and we have carried out studies on the integration of the accounting and marginal cost approaches.

The purpose of this report is to consider the implications for policy making of our findings.  Section 2 summarises the issues to be considered.  In section 3, we attempt to answer the questions as they relate to marginal costs, in section 4 as they relate to accounts, and in section 5 as they relate to integration.  Finally we draw out the key conclusions.

# The policy issues

## 2.1

## Overview

The UNITE work programme relates to policy in a number of complex and interrelated ways:

- First, it is directly relevant to transport pricing.  The marginal cost approach provides estimates for efficient pricing in different traffic situations.  Even though pricing policy in transport involves consideration of multiple objectives and constraints, an important starting point for policy is the pattern of efficient prices by mode, area type and route type.  The marginal cost case studies provide relevant information to help populate that approach.  However, it is unrealistic to expect a comprehensive set of marginal costs to be derived from such an approach on its own.  In practice, we need to rely on social accounts data as a generic source of information, and to derive approximate or “average” marginal costs information from such  data using such evidence on cost/output relationships as can be found in the literature.  It is the use of case study and accounts data together which is likely to be the most practical means of generating practical marginal cost estimates which feed into pricing policy.

- Secondly, the work programme is relevant to the creation and maintenance of a set of consistent social accounts for the transport sector as required by EU directives.  To achieve consistency across modes and countries is a formidable task to which we believe UNITE has made a contribution.

- Thirdly, in practice, pricing policy may involve balancing a mixture of considerations.  Efficiency is clearly one but notions of equity fairness, cost recovery and revenue raising are others.  Thus, second-best questions such as how to set efficient prices in relation to marginal cost in the transport sector while achieving a given budgetary result, or how to set transport sector prices in relation to marginal cost given distortions in related sectors elsewhere in the economy are clearly relevant policy issues which may draw on both marginal cost and accounts information and which the integration strand of UNITE has addressed.

- Fourthly, the information both from marginal costs and accounts may provide relevant inputs to decisions relating to other decisions such as decisions on investment and to non-price regulation.  The interrelationship between pricing and efficient investment is an issue of considerable policy interest, both in an economic sense and in relation to the case for Trust Funds and other ways of ring-fencing revenues for transport investments.  Such issues are likely to be particularly relevant for the accession countries.

In the next section, we discuss the information requirements and the progress made by UNITE towards fulfilling these, in more depth.

## 2.2	Marginal costs

The concept of marginal social costs of transport use is, we believe, a clear one. However, there are numerous issues involved in operationalising the concept.  These are at the levels of methodology, quantification and use for policy analysis and development.

In terms of methodology, it is necessary to spell out what the main methods are for the estimation of different cost elements.  These will involve a mixture of engineering, accounting and econometric methods.  It is unlikely that any one of these alone will be adequate;  a hybrid will be required.  In dealing with this issue, the UNITE project has built upon the recommendations of the High Level Group for Pricing, which proposed core elements of useful methodologies.

In terms of quantification, it is clearly necessary to develop estimates of marginal social costs which are useable for policy purposes.  This means that we need:

- engineering relationships (such as speed/flow/capacity/ on road and rail);

- relevant values (of time, safety, pollution costs etc) and that we need to be able to apply these in a suitably disaggregated way;

- differentiated by mode, area type, route type, vehicle type, time period;

- and suitable indications of levels of accuracy, ranges of uncertainty.

In terms of policy it is necessary to provide guidance on how the results may legitimately be used for pricing at sector and sub-sector level.  In particular, the issue of modelling demand/supply re-equilibration with respect to price changes needs discussion.  In designing efficient price schemes, the demand/supply feedback needs to be taken into account.

## 2.3	Accounts

There are two objectives of the Accounts strand of the work in UNITE.  The first is to generate a set of transport social accounts at national level.  The second is to provide information on cost levels for particular cost categories, to which suitable cost allocation or similar accounting methods can be applied to obtain estimates of marginal costs, accepting that these will involve a degree of averaging, or even that average cost equals marginal cost for particular cost or traffic categories across the range of traffic volumes being considered.

On Methodology, key issues include agreement on the relevant cost categories, the basis for disaggregation across area types, vehicle types etc, and the basis for valuation of time, safety risk, pollution etc.  As with much European work, issues arise as to the extent to which commonality is required so as to facilitate comparisons across the EU.  This could be:

- commonality of method – e.g. agreement that in transport accounts road maintenance costs are to be handled in a common manner across Member States;

- commonality of values – for example, that common values of travel time are to be used across Member States;

We think there is a stronger case for the first rather than the second.  If local values of time relative to (say) fuel costs are very different in Greece from Germany, we think the social accounts should respect and reflect that difference.

Practical issues arise where Member States have different data or different conventions for using that data.  Achieving a truly consistent set of accounts to common standards will require commitment by the EU and Member States.

In terms of results, the main issues concern the levels of aggregation/disaggregation at which it is both feasible and useful to present the results.  Comparisons of results across Member States are, of course, interesting in themselves, but comparisons between cities, inter-urban routes, rural roads etc, in different countries would be more interesting because these would be closer to comparing like with like.  However, even the limited degree of disaggregation we sought to achieve proved impossible in many cases.

We would argue for a high degree of caution in the direct use of transport accounting data for policy-making.  Neither an efficiency, nor on equity grounds do we see a strong case for 100% cost recovery, and in any case questions arise as to which costs it is legitimate to recover from users in aggregate.  However, we do consider accounts to be useful as a source of data for estimating marginal costs (for some cost categories) and as a source of evidence relating to equity issues and to the determination of the budget constraint by political authorities.

## 2.4	Integration

The integration strand of the research has sought to build a bridge between the marginal cost and accounts strands and to model different pricing scenarios under various second-best conditions.  This part of the work inevitably relies more on economic theory and modelling to make a complex reality more tractable.

In this strand of the work, we posit that the authorities are interested in the macroeconomic (ie multi-sectoral) performance of transport pricing strategies and also that for practical tariff-setting, budget constraints will be relevant.  In that context, key issues include the information requirement for efficient second-best pricing, the ways to represent the interfaces between the transport sector and other sectors in the economy, and the ways in which accounts information might sensibly be used alongside marginal cost information in such a context.  The key output is to help develop an understanding of the effects of those credible restrictions on the pattern  of price : cost relationships relative to the first best pattern.  Understanding what drives the direction and magnitude of changes relative to that base can give very useful insights for policy which we discuss further below.

In order to pursue these objectives both partial and general equilibrium models were utilised to examine alternative pricing strategies and the relevance of various constraints on efficient pricing. In addition, a review was undertaken of pricing doctrines and the way in which they influence policy in the various member states.

# Marginal Cost Research

## Introduction

The following table presents the wide variety of  case studies undertaken by the modes and cost categories they cover.

Table 1. Quantitative overview of the case studies by mode and cost category

| Category | Road | Rail | Air | Inland Waterways | Maritime | Total – by cost category |
| --- | --- | --- | --- | --- | --- | --- |
| Infrastructure costs | 2 | 2 | 1 | 1 | 2 | 8 |
| Supplier operating costs | 0 | 2 | 1 | 0 | 0 | 3 |
| Congestion costs | 6 | 1 | 1 | 0 | 0 | 8 |
| Mohring effect | 0 | 1 | 1 | 1 | 0 | 3 |
| Accident costs | 3 | 2 | 0 | 1 | 1 | 7 |
| Environmental costs | 6 | 3 | 0 | 1 | 1 | 11 |
| Total – by mode | 17 | 11 | 4 | 4 | 4 | 40 |
| Source: D3, Unite – update |  |  |  |  |  |  |

As can be seen, not surprisingly the largest number of the case studies cover  road transport. This is explained in particular by the relative importance of the  costs for this mode of transport. Some of the costs are considered of no or minor significance for certain modes and therefore no case studies were implemented. For example accidents in aviation are rare, and most costs are internal while the role of congestion in waterborne transport is normally insignificant.

- In the following, we will bring together the main conclusions of the UNITE case studies firstly on methodology and secondly on empirical results.

## Methodology

In this section we summarise our conclusions about the appropriate methodologies for use in estimating marginal social cost and the extent to which marginal cost estimates may be adapted or transferred from one context to another. We consider transferability in terms of methodology, input values, functional relationships and output values, and look at each cost category in turn.

### Infrastructure costs

For road and rail, previous studies tend to use a (top-down) cost accounting approach, based on a simple division into fixed and variable costs. Very few included regression analysis to derive marginal costs. The cost accounting approach basically divides costs into categories and allocates them to the output measure deemed most appropriate. This is a simple practical approach for which the data is usually readily available. For an example of tis approach from a specific UK study see Table 2. In this case, marginal cost is estimated as around 50% of average cost, with marginal cost varying between vehicle types mainly on the basis of standard axle kilometres. However, this result depends on a specific set of assumptions about which cost elements are fixed and which are variable, and there is no agreement between different studies on this issue, or on the question of with which output measure they vary; different countries tend to use different conventions.

Table 2.

Values of Cost Drivers and inclusion in MC Analysis in the UK (Sansom et al, 2002)

| Description | PCU -km | av.gwt -km | max gwt -km | sa - km | Include in MC? |
| --- | --- | --- | --- | --- | --- |
| Long-life pavements |  |  |  | 100% |  |
| Resurfacing |  |  |  | 100% |  |
| Overlay |  |  |  | 100% |  |
| Surface dressing | 20% | 80% |  |  |  |
| Patching and minor repairs |  | 20% |  | 80% |  |
| Drainage | 100% |  |  |  |  |
| Bridges and remedial earthworks |  | 100% |  |  | - |
| Footways, cycle tracks & kerbs |  | 100% |  |  | - |
| Fences and barriers | 33% | 67% |  |  | - |
| Verges, traffic signs and crossings | 100% |  |  |  | - |
| Sweeping and cleaning | 100% |  |  |  | - |
| Road markings | 10% | 90% |  |  |  |
| winter maintenance & misc. | 100% |  |  |  | - |
| Street lighting | 100% |  |  |  | - |
| Policing and traffic wardens | 100% |  |  |  | - |

Note: av.gwt– average gross vehicle weight; max gwt – maximum gross vehicle weight; sa – standard axles (a measure of the relative damage due to axle weights).  The costs attributed to pedestrians for roads other than motorways (50% of the categories from Fences and barriers through to Street lighting) are removed prior to allocation to motorised vehicles.

Source   Sansom et al (2001)

Both econometric and engineering approaches can help with this problem. But data requirements are heavy, and it cannot be expected that such studies will be undertaken every time marginal cost estimates are required. Moreover, the econometric approach is best for getting general information about cost elasticities; it cannot identify the impact of different types of vehicles in great detail because of multicollinearity between the explanatory variables. The engineering approach is therefore needed for this.

In practice many studies will continue to use data from accounts, with cost elasticities and vehicle relativities borrowed from other studies. However even these parameters vary with context. For instance cost elasticities vary with traffic density, and relative marginal costs of heavy vehicles are higher when infrastructure quality is low. This seems to be less of a problem for rail where cost elasticities are generally lower.  Also, many authorities do not decide expenditure on the grounds of necessity, but in relation to budgetary reasons. More research on cost elasticities is needed, particularly for road infrastructure, where considerable variation in the results was found. Our case studies found evidence of a cost elasticity of around 0.8 for road maintenance and renewal, but with much lower figures for less heavily used roads, and 0.2 for rail maintenance and renewal.

Much less work has been done for air and water transport; it appears that marginal infrastructure costs are very low, but that result needs further work to confirm it. For nodes (airports and ports) estimates of marginal operating costs are also needed; a regression approach has been used but again data requirements are heavy. It has to be considered, that cost data in a liberalised transport sector are owned by private companies and thus difficult to compile. Again the marginal cost appears to be very low.

Supplier operating cost

As with infrastructure costs, our preferred approach is the econometric approach, but there are big problems with it. The first is getting appropriate data. Usually data is only available as time series data for an entire company, or for a cross section of different entire companies (or both). One company includes a wide variety of types of service, but it is difficult to estimate the costs of each type of service accurately from such data, and variables representing different types of output are highly correlated. The result is that the econometric approach is most useful to provide evidence on  economies of scale and scope at an aggregate level. Cost elasticities appear to be transferable; usually as in the air case here they are found to be around one with respect to vehicle kilometres, implying constant costs for this category of costs, meaning that an approach based on fully allocated cost should be adequate as an estimate of marginal cost.

The main approach for more detailed estimates of marginal cost is the cost accounting approach. Accounts are the obvious source of the relevant data, but usually business accounts of the companies concerned rather than national accounts, which are too aggregated.  The standard formula for supplier operating cost is along the lines of:

Cost = a + b* train hours +c* vehicle kilometres + d* peak vehicle requirement

As with infrastructure costs, this estimates the costs resulting from the pattern of service provided. To estimate costs per passenger kilometre it is necessary to understand how the service pattern will respond to changes in traffic. This point is returned to in the next section in the discussion of Mohring effects.

Logic, rather than econometrics, is used to determine which output variable determines the level of each cost category. This approach does not necessarily deal adequately with effects of peaks on staffing levels, or with variability by type of vehicles. In situations of peak demand, many costs are attributed entirely to the times and locations providing the peak vehicle requirements. This may result in huge differences between peak and off-peak MCs. To estimate this accurately, a better approach is to do a complete vehicle and crew scheduling exercise to identify numbers required and cost.

### User cost

Under user costs we consider three elements; congestion costs, scarcity costs and Mohring effects. Congestion arises where one vehicle delays another; scarcity costs where one vehicle prevents another from gaining access to the network and Mohring effects where additional traffic leads to increased frequency of a public transport service leading to external benefits for existing users.

For road, studies using speed flow relationships, either for individual links or on a network basis are commonplace. Many existing models are available which may be used. Yet there is still considerable variability of results. The UNITE urban case studies produced results that are an order of magnitude less than some other case studies, such as the TRENEN case studies.  This may simply be because the UNITE case studies concentrated on locations which were not particularly congested, but this is not always the case. For instance Greater Brussels was one case study where the UNITE figure is very much lower than those in TRENEN. Another explanation is that UNITE has computed congestion costs as they would be were a perfectly differentiated congestion charge introduced, allowing for the substantial reduction of congestion via rerouting that would then take place. This will be much lower either than current congestion costs or congestion costs were a simple cordon or area charge introduced. A third reason may be that UNITE has looked at relatively large areas rather than just city centres as many previous studies have done. When these areas were broken down, of course congestion was found to vary substantially by section of the network.

Speed flow relationships as well as demand patterns and traffic data cannot be readily transferred due to differences in the standard of infrastructure, traffic laws and in  behaviour between countries. Resulting output values are only transferable with regard to situations where these factors are similar, as well as having similar traffic levels and values of time. If the only difference is in income, values of time may be transferred using an appropriate income elasticity.

Data from the accounts may be of some help here. In the accounts we estimated the average cost of congestion relative to speed flow conditions. If the average and marginal cost curves were linear, then it is easy to show that this is equal to the difference between marginal and average cost, which is the marginal external congestion cost. Given known non linearities, it is likely that in fact this value forms a lower bound to the marginal external cost of congestion. However, the accounts give this at an aggregate level for the country as a whole. Thus whilst this value may be of some use for monitoring the average level of prices relative to marginal cost on the average, it is no help in formulating the details of pricing policy for congestion.

For rail and air, there is very limited evidence on congestion. Within UNITE the methodology we used was regression analysis relating delays to volume of traffic. Much more detailed studies of the relationship between delays and capacity utilisation have been undertaken by Railtrack, the rail infrastructure provider in the UK. The basic regression methodology used in these studies should be readily transferable provided that data may be obtained.  It is more doubtful whether relationships and outputs can be transferred given differences in the infrastructure and the level and mix of traffic. The Railtrack work required the use of different parameters even for different parts of the British rail network. The assessment of opportunity costs of scarce railway and airport slots remains a difficult and under researched area which is a priority for future research. The analysis of slot trading mechanisms and marginal willingness to pay for additional slots (for air and rail transport) might be a possibility to improve the situation.

The Mohring effect is the effect by which increasing volumes of traffic lead to improved services, thus benefiting existing users. For the Mohring effect, which only applies to scheduled transport, an approach based on first principles assuming that operators increase service frequencies in direct proportion to increases in patronage (see  Sansom et al, 2001, section 5.5) may be used to give a simple formula. This may also be adapted to cases where other behavioural rules apply.

For high frequency regular interval services, the benefit of an additional passenger is:

B = 0.5*h*v*ef

Where B = Mohring benefit from an additional passenger (euros)

h = headway between services (mins)

v = value of waiting time (euros/min)

ef = elasticity of service frequency with respect to traffic volume

If the service is a long way from being regular interval than an accurate calculation requires knowledge of the distribution of desired departure times and the precise timetable before and after the increase in traffic.  Where passengers are adapting to a known timetable, and are indifferent between travelling earlier or later, then the mean schedule delay becomes 0.25h instead of 0.5h as in the above formula, and v must be replaced by the value of a departure time shift, rather than waiting time. It is usually lower than the value of waiting time.

The rail case study put forward theoretical arguments to suppose that frequency will increase with the square root of traffic levels (ef = 0.5), and showed that this was in fact the case for inter urban rail services in Sweden. In such a case, it is necessary to allow for the economies of scale in supplier operating cost that result from higher load factors.

The air and scheduled inter-modal freight case studies found a much more varied pattern of response, but in both cases there were significant benefits to existing users from increasing traffic levels. In other words, the UNITE case studies show that this basic result, that increases in traffic lead to external benefits to existing users, applies not just to urban public transport but also to inter urban rail, air and scheduled freight services. In each case users value the improved frequencies and increase in through services resulting. However, the value of these benefits relative to other cost elements is obviously smaller for longer distance trips.  Moreover, price differentiation may recover a larger proportion of consumer surplus as revenue for longer distance trips since more complex price structures may be used.  Thus, whilst in principle the Mohring effect provides an argument for subsidising all scheduled public transport services, its practical importance is greatest for local public transport.

This methodology is general, subject to knowledge of the way in which operators respond to changes in traffic levels, and the value users place on these changes. Values of time and of departure time shift are transferable given knowledge of relevant income elasticities. If frequencies are optimal then the Swedish rail case study shows that marginal social cost should be the same, irrespective of the way frequency adapts to traffic levels.

### Accident cost

Many studies in the past have failed correctly to distinguish between internal and external accident costs and used simple average figures. The risk elasticity method and theory presented in UNITE is summarised in the equation for marginal external accident cost below

MCje =r(a+b+c)1−θ+E+θrc

where r represents accident risk, a the value of statistical life (VOSL), b ditto for relatives and friends, c the costs for the rest of society, θ the proportion of accident cost that falls on the traveller of type j and E the risk elasticity (ie the relationship between accidents and traffic volume).

It is clear from this that marginal external costs of accidents will differ when any of these parameters differs, and that direct transfers of values from one context to another may only be done if it is believed that all these parameters are unchanged.

We expect the external marginal cost to be high if:

- the accident risk r is high

- the cost per accident is high (a+b+c);

- most of the costs fall on other groups (θ≈0);

- the risk increases when the traffic increases (E>0);

- or a large part of the accident cost is paid by the society at large (c).

We believe that this method is suitable for all modes in all member states. We cannot foresee any more general form of the external marginal accident cost, except that risk avoiding behaviour should be introduced. This involves formidable practical issues in terms of estimation, however.

Our formula requires knowledge of:

- The accident risk for the mode and context. Such data is usually available and is derivable from the UNITE accounts.

- The relevant risk elasticity. UNITE research has extended the knowledge of risk elasticities and how they vary but this is still an area of uncertainty. However, estimation of elasticities is difficult on a case by case basis.

- The value of a statistical life, which may generally be transferred using data on real incomes. Within UNITE, a general VOSL of 1.5 Mio Euro (European average) has been used. This most sensitive unit value can be adjusted to different countries according to GDP per capita. The value is based on the state of the art of willingness to pay studies to avoid fatality risks.

- The proportion of costs borne by the injurer, which will need to be estimated locally, as it varies with legal and insurance company provisions.

Comment MM:	
Maybe it would be good to add a sentence on the relevance for pricing (road pricing versus differentiated insurance systems). Marginal external accident per km costs are low, but there are other cost drivers (age groups, drug risks, local elements etc.).

### Environment

Bottom up impact pathway studies are the best way of calculating environmental costs, and are generic enough to be applied to all modes of transport. However, there are several areas of controversy surrounding the use of this methodology such as amibiguities over the effects of particles and nitrate aerosols on human health, the valuation of mortality, ozone impacts and the potential omission of other impacts which cannot be quantified.

Physical impacts (except global warming) are difficult to transfer to other countries/regions/contexts. A direct transfer of costs due to air pollution cannot be recommended; a generalisation methodology should account for local scale conditions such as population density and meteorology and regional scale costs per tonne of pollutant emitted in a certain area. Most important are dose-response functions to value the impact of PM10 concentration to human health.

Noise cost estimates are also difficult to generalise due to their local nature and dependence on background noise level.

Whilst exposure response functions can be generalised, inputs to dispersion models, due to their site-dependence, cannot be generalised. Specific exhaust and noise emission factors for vehicle types can be generalised, accounting for driving characteristics and average speed. Generalisation of emissions related to vehicle fleets is not recommended. Economic values may be transferred using real income and the Purchasing Power Parity structure.

Regarding global warming, damage cost estimates are transferable as location of emissions is irrelevant. It has to be considered, there is a wide range of unit cost estimations and that unit values are rather sensitive for the comparison of different transport modes.

To the extent that for environmental costs other than noise marginal costs are assumed equal to average costs, accounts do at least produce countrywide averages of marginal cost to monitor overall price levels.  For noise, average cost is likely to exceed average marginal cost. This of course means that an optimal level of noise nuisance cannot necessarily be achieved by pricing alone; nevertheless, in determining optimal traffic levels the marginal cost of noise is still the appropriate concept to use.

## Empirical results

In this section we present an overview of some of the empirical results from the UNITE case studies, and seek conclusions on the relative importance of the different cost elements by mode and context as well as the degree to which the magnitude of estimates for that cost category varies. It should be noted that in general each cost category for each mode was studied in a different case study, so that the various cost categories cannot simply be added together. Moreover the output variables that were used in the different studies varied, so to make them comparable various assumptions had to be made. The bar charts should therefore be seen as illustrative of general tendencies rather than as precise numerical estimates. Where a range of results is produced by different case studies the bar charts show the upper and lower values.

Figures 1a-b summarise the results for car travel. It is clear that for car the dominant element is congestion (especially for urban trips), however, this varies greatly between case studies. (Note the ranges for congestion costs are smaller than those reported in D15 as here we report network averages rather than results from single road segments). Accident costs are also comparatively large, particularly in urban areas. Also, in urban areas noise is very important, particularly at night, and air pollution is also significant. Many of the environmental costs also vary due to differences in traffic densities and speeds. Areas of high traffic speed and density report lower marginal noise and air costs, as an additional car has less impact than in quieter areas. Marginal infrastructure costs, air pollution and global warming costs for car are all relatively small.

The most striking feature of these diagram is the enormous variation from case to case for the costs of congestion, noise and air pollution.

Figures 2a-b produces the same diagram for heavy goods vehicles. Here congestion costs are similar as for car, but a smaller component of the overall marginal cost, whilst infrastructure costs are somewhat greater. Noise costs are important for urban HGV travel, and air pollution costs remain important for interurban routes, and in most cases these are over a factor of ten higher than for cars.  One urban  study in a low density traffic area finds night-time noise the dominant factor. (Ranges for congestion costs are again smaller than those reported in D15 for the same reason as described above).

Figures 3a-b show the results for rail passengers. Naturally, supplier operating costs are the dominant element, and are many times higher in the peak than in the off peak.  Urban congestion and inter-urban air pollution costs are the next most significant items. Air pollution and global warming costs varying with type of traction and (where relevant) prime energy source for electric traction. Marginal infrastructure costs for passenger services are low. It should be noted that in the inter urban rail case study, supplier operating costs are strictly for lengthening trains; for purposes of comparability they have been expressed per train kilometre given typical train loadings but this is somewhat misleading given the nature of the case study. Also when capacity is expanded solely by lengthening trains  there is no Mohring effect, so these two cost categories certainly cannot be added together.

Figure 4 shows comparable figures for rail freight, except that we do not have a supplier operating cost case study for freight. Air pollution, global warming and noise are all of similar orders of magnitude to marginal infrastructure costs. Infrastructure costs are higher than for passenger rail due to the higher gross tonne km per train of freight.

Finally figure 5 shows the results for air transport. Of course supplier operating cost is dominant, but these results suggest that both congestion costs and the Mohring effects  are more important than environmental effects. In circumstances where line density is low, the Mohring effect is quite important, leading at least to an a priori case for subsidy for scheduled air services. (For consistency across cost categories, we have assumed an average journey length of  930km as was used in the environmental cost case study. For this reason results reported here may differ from those in D15).

## Conclusions

The UNITE case studies have produced estimates for a wide range of circumstances and using a wide variety of approaches. In some cases (such as supplier operating costs for rail and air) these have been the subject of much previous analysis; in others, such as infrastructure costs for water transport, the existing literature is very sparse.

These case studies illustrate that there is no unique ‘state of the art’ approach for the estimation of marginal costs. For instance, for infrastructure and supplier operating costs, there may be a preference for econometric estimation, but it is seldom possible to do that at a level of detail that lends itself directly to pricing decisions; a mixture of econometric research with engineering and cost accounting approaches therefore is necessary.  For user costs, road congestion has been extensively investigated using either single link or network models based on speed flow relationships and junction delay formula. For rail and air congestion a regression based approach has also been developed but work in this area is new; there are few existing studies. No estimates of scarcity costs for these modes have been found and this is a priority for future work.

- For accidents, a correct methodology has been developed which requires inputs from a variety of sources; the most difficult being risk elasticities. The impact pathway approach is recommended for environmental costs, but further research on transferability of results would be worthwhile.

- The quantitative results suggest that marginal infrastructure costs are generally low; it is supplier operating costs, congestion costs, the Mohring effect and in some circumstances elements of environmental costs, particularly noise, that are the most important categories.

In terms of external costs, for the car, as expected, generally congestion costs dominates, followed in the case of urban areas only, by noise and air pollution (especially for diesel cars) with global warming generally much smaller. Accident costs are also significant in urban areas. In general, the differences between areas are quite large. Outside urban areas, noise and air pollution costs are generally much smaller.

When load factors are taken into account, external costs of air and rail are generally much smaller than car. The exception to this is congestion costs, where our case studies, as was noted earlier, yield lower results for cars than in other comparable car studies. The congestion costs in our air case study, Madrid airport, appear to be of the same order of magnitude as for car when allowing for load factors, although the results for rail appear to be somewhat lower. We have no quantification of pure scarcity costs, which can be important for rail and air.

Figure 1a

Figure 1b

Figure 2a

Figure 2b

Figure 3a

Figure 3b

Figure 4

Figure 5

# Accounts

The use of accounts

Transport accounts are an important tool to show performance of the transport sector for a specific spatial entity and a specific period of time. The following table contrasts the information provided by accounts and marginal cost estimates.

Table 3  Commons and differences between marginal cost and accounts information

|  | Marginal Cost Information | Transport Accounts |
| --- | --- | --- |
| Use | Input figures for efficient pricing | Output/Impact monitoring of the transport sector |
| Cost information | Unit costs/specific information: Additional costs of an additional transport unit, related to additional vehicle km | Total costs and revenues/ aggregate information: Total and average costs/revenues for a specific entity |
| Differentiation, systems delimitation | Different type of vehicles, different traffic situations, different exposure levels | Regional boundaries (e.g. national level), different time periods, different transport modes |
| Elaboration | Bottom up, based on analysis of a cost function | Top down, based on national transport statistics |
| Use of information | Pricing decisions | Transport monitoring |

D14 (Link et.al. 2002) has identified seven purposes for transport accounts:

Strategic Monitoring	
The basic information of accounts helps to understand the structure, the level and the development of costs and revenues on a national level. On a policy level, this can be used for

- Relevance analysis: To identify important cost elements within different transport modes,

- Comparison: To compare different countries, transport modes and different spatial aggregation levels,

- Forecasting: To develop early diagnosis of trends in the transport sector and use for policy scenarios.

Monitoring financial viability

Accounts contain information which is directly relevant for financial purposes. Most important are infrastructure costs, supplier operating costs and related revenues or subsidies. Hence one part of social accounts is an aggregate of transport operators accounts (e.g. public transport suppliers, privatised motorway companies, airport operators etc.), which sets the financial costs in relation to total social costs.

Accounts make visible the financial performance of private and state owned transport entities, showing as well explicit and hidden subsidies. These subsidies can be provided for specific transport services, such as public service obligation (e.g. of public transport), to cover deficits of transport operators, or to cross-subsidise specific transport services.

The financial balance within the accounts can be used to answer three types of questions:

- what is the total cost of the sector in question?

- to what extent is this cost borne by users as opposed to taxpayers or the community at large?

The results certainly provide a starting point for considering issues of finance and equity.

Since cost recovery is an important aim for business units, the monitoring of financial viability is still one of the most important aims of transport accounts. In actual fact, it was one of the very initial aims of national initiatives to develop transport accounts (e.g. Germany, Switzerland, Austria). One has to consider however, that cost recovery indicators need assumptions for the allocation of costs and revenues. These allocation rules cannot be provided on a scientific level and are very difficult to generalise.  Moreover, there are good economic arguments for believing that 100% cost recovery is not necessarily the best policy.  This issue is returned to in Section 5.

Monitoring progress towards sustainable transport

Social costs and revenues are showing the performance of the transport sector in monetary terms. This common monetary denominator is also used to make visible important sustainability issues like the quality of the transport sector (e.g. expressed as delays or congestion), safety problems (social costs of accidents), and environmental problems (social costs of environmental nuisances). The respective indicators can be integrated in regular monitoring tools on a national basis. EEA for example (see TERM 2001) is using total environmental cost information as one out of 20 periodic indicators on a European level.

Monitoring Equity

Equity issues are very important in the public discussion of transport policy and refer to different dimensions such as

- Regional equity: Costs and revenues of urban and rural areas, different state levels and different nations.

- Equal treatment of different transport services: Most important are those being in direct competition, such as road and rail (passenger and freight), high speed rail and air transport services.

- Equity between social groups: Well elaborated and sophisticated accounts can be used as well for the presentation of equity considerations between different income groups or between consumers and producers. This is clearly an interesting task of future research.

Equity is addressing also the polluter pays principle being one of the central claims for sustainable transport policy. It has to be considered that there are different interpretations possible for the implementation of this principle. Accounts are useful to present the basic information for this task, although they may typically not provide the level of disaggregation necessary for their full resolution.

Monitoring Efficiency of pricing schemes

Accounts provide different elements of information on efficiency: Firstly accounts provide information on the development of social cost or subsidies of transport, which are clearly relevant in considering the efficiency performance of the transport system. The lower average costs, the better the efficiency. But it is important to note that the UNITE pilot accounts do not cover all efficiency aspects. The work on integration (D4, Proost et.al. 2001) has shown that accounts should be enlarged by the measurement of the development of the consumer surplus of the transport sector as a whole in order to have a comprehensive magnitude of overall efficiency. This is not an easy task and can be better fulfilled by using Cost-Benefit-Analysis for specific projects.

Secondly accounts provide information on average prices (e.g. revenues per unit of transport) in the transport sector. The more these prices converge with marginal cost figures, the higher the efficiency.

Thirdly accounts information can be used for benchmarking and comparing different country performances. This is a link to the aim of strategic monitoring. Finally the level of subsidies per unit of transport is not only an indicator for equity but also for efficiency. The higher the subsidies, the higher the risk of efficiency losses (especially in the public transport sector).

Development of marginal cost estimates

Accounts are providing aggregate information, which can be allocated to specific transport services (top down allocation). Parts of such information can be used for aggregate proxies of marginal social costs. Average variable infrastructure costs or average variable environmental or operating costs for example can be used as general information for marginal cost figures (e.g. on a national basis). Such figures are helpful as a starting point for development of more detailed marginal cost estimates for specific circumstances.

Provision of basic data for transport statistics

Accounts are in general a product of national statistics providing a set of aggregate transport data. This information can be used like any other statistic information for very different purposes, as well for future research and for further elaboration of indicators (see below).

## Elaboration of accounts

UNITE has studied optimal structures of transport accounts on a practical and on a theoretical level. The experience of the elaboration of the national pilot accounts can be used as well for policy recommendations for the future elaboration and development of accounts.

### Templates for costs and revenues

It is useful to make a distinction between basic transport accounts and further sophistication (ideal accounts). Within UNITE, the basic transport accounts were elaborated as so called pilot accounts. The experience with this task showed, that the templates developed are appropriate; in general it is possible to collect the relevant basic data and to estimate related costs and revenues on a national level. The following major difficulties have to be challenged:

- The allocation of costs and revenues to certain type of vehicles, especially the differentiation between freight and passenger transport and respective type of vehicles (especially for road transport).

- The differentiation of costs between fixed and variable parts.

- The aggregation of costs and revenues of individual transport service units (e.g. public transport, airports, ports) to national figures.

The following recommendations for a further use and development of national accounts refer to the pilot accounts experience within UNITE.

a)	Basic transport information

In order to produce the accounts and to elaborate the relevant indicators, the following basic data per country (or other spatial entities) and per year should be provided:

Table 4  Basic transport information for the accounts

| Basic data | Specification |
| --- | --- |
| Socio economic indicators | Population, Land area, GDP (real and nominal, growth rates, employment rate, consumer price index) |
| Transport infrastructure | Network length, capital stock |
| Transport performance | Passenger transport: Passenger carried, Passenger km Freight transport: Goods transported, Tonne km Number of vehicles Vehicle km |
| Safety performance | Number of accidents (injuries, fatalities) |
| Environmental performance | Direct transport emissions (NOx, PM10, NMVOC, SO2, CO2) People exposed to noise |

b)	Differentiation of accounts per mode

The more differentiated, the more relevant are the results of the accounts for transport policy. There is however a trade-off between data quality or accurate allocation methods and a sophisticated differentiation. The following table shows a useful level of differentiation.

Table 5  Differentiation of accounts according to modes and further criteria

| Transport modes | Network and institutional differentiation1) | Means and user breakdown1) |
| --- | --- | --- |
| Road | -Motorways -Inter-Urban roads -Urban and Local Roads | –Motorcycles –Passenger cars –Buses (private and public) –Light goods vehicles –Heavy goods vehicles (HGV) |
| Rail | -All rail | –Passenger transport –Freight transport |
| Other public transport | – | –Trams –Metro –Trolley buses – Public buses |
| Aviation | -Airports -Air transport | –Passenger –Freight |
| Inland waterway | -Inland waterways -Inland waterway harbours | – |
| Maritime shipping | -Seaports | – |

c)	Basic templates for costs and benefits

Due to different institutional frameworks, different relevance of costs and differentiation, the structure of the costs and revenue information of different transport modes is not exactly the same. The following table shows the basic structure of the relevant information for different modes as it was defined for the UNITE pilot accounts. Future improvement could refer to supplier operating costs for air and waterborne transport.

Table 6a  Basic templates  for transport accounts per mode – cost side

| Cost and revenue information | Road | Rail | Urban Public Transport | Aviation | Shipping (Canals/ Ports) |
| --- | --- | --- | --- | --- | --- |
| Costs |  |  |  |  |  |
| Core information |  |  |  |  |  |
| Infrastructure Costs | X | X | X | X | X |
| Capital costs | X | X | X | X | X |
| Running costs (Variable/Fixed) | X | X | X | X | X |
| Traffic control costs | - | - | - | X | - |
| Supplier Operating Costs | - | X | X | - | - |
| Accident costs (user external)1) | X | X | X | X | X |
| Environmental costs | X | X | X | X | X |
| Air pollution | X | X | X | X | X |
| Global warming | X | X | X | X | X |
| Noise2) | X | X | X | X | X |
| Additional information | X | X | X | X | X |
| Congestion/Delay costs3) | X | X | X | X | - |
| Time costs | X | X | X | X | - |
| Fuel costs | X | X | X | X | - |
| Accident costs (user internal)4) | X | X | X | X | X |
| From this: risk value | X | X | X | X | X |
| Environmental costs | X | X | X | X | X |
| Nature and landscape, soil and water pollution | X | X | X | X | X |
| Nuclear risk | - | X | X | - | - |

Table 6b  Basic template for transport accounts  per mode – revenue side

| Cost and revenue information | Road | Rail | Urban Public Transport | Aviation | Shipping 1) |
| --- | --- | --- | --- | --- | --- |
| Revenues |  |  |  |  |  |
| Directly related to a specific cost category | X | X | X | X | X |
| Charges for infrastructure usage | X | X | X | X | X |
| Fixed5) | X | X | X | X | X |
| Variable | X | X | X | X | X |
| User tariffs | - | X | X | X | X |
| Compensation for concessionary fares | - | X | X | - | - |
| Charges for traffic control | - | - | - | X | - |
| Other transport specific revenues |  |  |  |  |  |
| Fuel tax | X | X | X | X | X |
| Annual vehicle tax | X | - | - | - | - |
| Sales tax | X | - | - | - | - |
| VAT on fuel tax/other charges | X | X | X | X | X |
| Security charges | - | - | - | X | - |
| Non transport related revenues | - | X | X | X | X |
| Subsidies |  | X | X | X | X |

1) Refers to those parts of road accident costs which are not borne by road users and insurance companies but by the public sector and third parties. – 2) metro, tram and trolley bus, diesel bus. – 3) Expressed as delay costs. – 4) Refers to those parts of accident costs which are caused and borne by road users and insurance companies. – 5) Charges not depending on the mileage.

### Estimation and valuation of costs and revenues

a)	Collection of input data

The estimation of costs and revenues is based on the following inputs:

- Transport statistics: Basic transport indicators, such as transport volumes, number of accidents etc,

- Financial information of transport service companies and infrastructure operators: Investment time series, expenses, costs and revenues, business balances,

- Modelling of specific information such as environmental nuisances, additional information to estimate fixed and variable costs,

- Additional information on economic costs (hidden subsidies etc.).

The following table indicates the sources needed to produce the relevant figures.

Table 7  Basic sources for the estimation of costs and revenues per transport mode

| Cost and revenue information |  | Road |  | Rail |  | Urban Public Transport |  | Aviation |  | Shipping (Canals/ Ports) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Costs |  |  |  |  |  |  |  |  |  |  |  |
| Core information |  |  |  |  |  |  |  |  |  |  |  |
| Infrastructure Costs |  | T/M (B) |  | T/M/B |  | T/M/B |  | M/B |  | M/B |  |
| Supplier Operating Costs |  |  |  | B |  | B |  | B |  | B |  |
| Accident costs (user external)1) |  | T/M |  | T/M |  | T/M |  | T/M |  | T/M |  |
| Environmental costs |  | T/M |  | T/M |  | T/M |  | T/M |  | T/M |  |
| Air pollution |  | T/M |  | T/M |  | T/M |  | T/M |  | T/M |  |
| Global warming |  | T/M |  | T/M |  | T/M |  | T/M |  | T/M |  |
| Noise2) |  | T/M |  | T/M |  | T/M |  | T/M |  | T/M |  |
| Additional information |  |  |  |  |  |  |  |  |  |  |  |
| Congestion/Delay costs3) |  | M |  | B/M |  | B/M |  | B/M |  |  |  |
| Accident costs (user internal)4) |  | T/M |  | T/M |  | T/M |  | T/M |  | T/M |  |
| Environmental costs |  | M |  | M |  | M |  | M |  | M |  |
| Revenues |  |  |  |  |  |  |  |  |  |  |  |
| Directly related to a specific cost category | T/B |  | B |  | B |  | B |  | B |  |  |
| Charges for infrastructure usage | T/B |  | B |  | B |  | B |  | B |  |  |
| User tariffs |  |  | B |  | B |  | B |  | B |  |  |
| Compensation for concessionary fares |  |  | B |  | B |  |  |  |  |  |  |
| Charges for traffic control |  |  |  |  |  |  | B |  |  |  |  |
| Other transport specific revenues |  |  |  |  |  |  |  |  |  |  |  |
| Fuel tax | T |  | T |  | T |  | T |  | T |  |  |
| Annual vehicle tax | T |  |  |  |  |  |  |  |  |  |  |
| Sales tax | T |  |  |  |  |  |  |  |  |  |  |
| VAT on fuel tax/other charges | T |  | T |  | T |  | T |  | T |  |  |
| Security charges |  |  |  |  |  |  | B |  |  |  |  |
| Non transport related revenues |  |  | B |  | B |  | B |  | B |  |  |
| Subsidies | A |  | A |  | A |  | A |  | A |  |  |

T	Transport Statistics	
B	Business Reports	
M	Modelling	
A	Additional information

b)	Valuation methods

In order to guarantee certain comparability between countries and different transport modes, it is necessary to use a standardised approach for valuation and value transfer. Based on the findings in D14 (Link et.al), we can recommend the following main approaches:

- Value transfer between countries: Unit values should be transferred from one county to another by using a GDP per capita correction.

- Infrastructure costs can be produced out of annual expenditure data using the perpetual inventory method. The values should be expressed at constant prices (with the price base being the account’s year), using real interest rates.

- Other financial costs and revenues should be valued in nominal terms of the account’s year.

- Delay costs can be modelled by estimating the difference between existing and uncongested traffic conditions. The latter refer to a situation in off peak situations (private transport) or according to transport schedules (public transport).

- Accidents costs should be produced by using a risk value for the valuation of fatalities. We recommend an average risk value for Western Europe of 1.5 million €. The external part can be produced by subtracting the level of insurance premia paid by different transport modes.

- Environmental costs should be modelled with specific emission and valuation models (e.g. ExernE, impact-pathway approach). The unit values for different environmental nuisances should be based – if possible - on national estimates. If not available, general dose-response-functions (as used by the ExernE model) can be used.

If possible the unit values should be based on national estimates. UNITE however has carefully considered how and when value transfers from one country to another is possible.

### Relevant Indicators

Accounts can be seen – this is shown by the different purposes – as an open tool for monitoring. In order to facilitate the interpretation, it is useful to define some basic indicators. The following table provides an overview according to the different purposes identified above.

Table 8  Basic indicators for the interpretation of transport costs and revenues

| Purpose | Indicators |
| --- | --- |
| Strategic Monitoring | - Total costs per GDP/per capita - Infrastructure costs per network km - Annual growth rate of costs and revenues |
| Monitoring progress towards sustainable transport | - External accident and environmental costs per capita/per GDP |
| Monitoring financial viability | - Infrastructure cost recovery rate, net balance of financial costs |
| Monitoring Equity | - Average accident and environmental costs per pkm and tkm - Subsidies for public transport |
| Monitoring Efficiency of pricing schemes | - Average variable cost - average price, compared with marginal cost - (subsidies per transport mode) |

In the following section we present results on these indicators.

## Interpretation of UNITE results

The following figures are based on the results of the UNITE pilot accounts. Since data quality differs between countries, we show the results only for those countries with reliable and comparable figures.

### Strategic monitoring

Based on preliminary results, Transport in Western Europe with toad, rail and air causes external accident costs of € 36 billion and environmental costs of € 90 billion (road, rail and air transport considered). Infrastructure costs amount to € 164 billion (road, rail and air transport).

The following figures provide country wise information.

a)	Costs in relation to GDP

Total core costs (infrastructure costs, external accident and environmental costs, supplier operating costs rail not included) amount from 2 % up to 5% of national GDP. Two exceptions (Greece, Hungary) show significantly higher ratios. This ratio considers road, rail and aviation, where total country figures can be obtained. Most important (around 60% in average) are related to infrastructure costs, followed by environmental and external accident costs. Road transport contributes more than 75% to total costs.

Figure 6  Total core costs of road, rail and air transport 1998 in % of GDP.

b)	Infrastructure costs per km

The following two figures show infrastructure costs per capita for road and rail per km of network. The differences between countries reflect different network capacities, different type of infrastructure (e.g. tunnels, open sections etc.) and different unit cost levels for construction and maintenance. Thus countries in mountainous regions (e.g. Switzerland, Austria) reflect higher costs per km, whereas the costs in countries with flat areas (e.g. Spain, Finland) are considerably lower.

Figure 7a: Road infrastructure costs in € per network km

Figure 7b: Rail infrastructure costs in € per network km

c)	Growth rates

The following figures show the %change of costs for road and rail between 1998 and 2005. Most of the countries face a growth of core cost elements (infrastructure, external accident and environmental costs) in this period. Note that the quantitative results for the forecast 2005 are less robust than the 1998 results. The results in the table can be used to illustrate a general trend. Expected growth rates for rail are comparably lower. There are however significant differences between countries, mainly depending on their investment priorities and initial modal split.

Figure 8a: Core costs road. Development 1998-2005.

Figure 8b: Core costs rail. Development 1998-2005.

### Monitoring progress towards sustainable transport

a)	Accident costs

The following figures are showing accident costs per vkm for different countries. The external part - which is borne by society - lies between 10 and 15% (road) and 2 and 25% (rail). The differences in total costs between countries are reflected by different accident rates and different valuations of value of statistical life. The differences of the external parts reflect different insurance systems.

Figure 9a  Road accident costs 1998.

For Hungary no separation between external and internal accident costs was possible.

Figure 9b  Rail accident costs 1998.

b)	Environmental costs

Environmental costs are compared by costs per capita. The following two figures show the results for road and rail. Most important are air pollution costs, which amount to 50% -75% of total environmental costs. Differences between countries reflect different pollution levels and different valuation of damages (depending on GDP per capita). The values for road vary between 100 and 350 Euro per capita. For rail the values are significantly lower (between 2 and 17 Euro). Most important are noise costs. Countries with diesel traction show also significant shares of air pollution costs for rail.

Figure 10a  Environmental costs road transport.

Figure 10b Environmental costs rail.

### Monitoring financial viability

a)	Road transport

We present two different indicators. Figure 11 shows the ratio for direct allocatable revenues (flat or variable road charges such as vignettes, road tolls). The results per country reflect as well the importance of this type of user charges in relation to other taxes. It can be shown that road user charges are still of minor importance. Some countries with tolled roads (such as Portugal, Spain, France, Italy) reach a share of 15 to 20% of cost coverage.

Figure 11a Road infrastructure cost recovery. Total infrastructure costs and directly allocatable fixed and variable charges for infrastructure use

.

The next figure also includes transport related charges and taxes (vehicle tax, fuel tax, VAT on fuel tax) which form however transport specific tax burdens which are usually considered as road revenues in national infrastructure accounts (e.g. Germany, Austria, Switzerland). The results show that the coverage ratio is in every country above 100%: Road transport related charges and taxes exceed financially relevant infrastructure costs.

Figure 11b Road infrastructure cost recovery. Total infrastructure costs and total revenues (fixed and variable charges, vehicle and fuel taxes)

b)	Rail transport

The relation of track charges and infrastructure costs reflect the cost recovery ratio for rail. The picture for Western Europe is quite heterogeneous. The ratios are between 0% (Belgium, Italy, Luxembourg) and more than 100% (UK).  This reflects very different policies towards rail infrastructure charges and the degree to which services as opposed to infrastructure is subsidised in the different countries.

It should be noted that we concluded above that marginal cost pricing might lead to a cost recovery ratio of the order of 20%, although strictly this applies only to maintenance and renewal costs rather than total infrastructure costs.

.

Figure 12 Rail infrastructure cost recovery. Total infrastructure costs and directly allocatable track and station charges for infrastructure use.

c)	Air transport

The cost recovery ratios are shown for airports in different countries. Since a separation between aviation and non aviation related costs and revenues are very difficult, the next figure shows the cost recovery ratio for the airports as a whole, but considering only aviation related revenues. The ratios are differing quite significantly between countries, depending on the function of the airport and the business models (state owned or privatised airports).

Figure 13  Air transport infrastructure cost recovery. Total infrastructure costs and directly allocatable airport revenues.

### Monitoring Equity

The UNITE pilot accounts have not compiled results for regional or social differences within countries. Thus the only major equity issues are already shown in the figures above, relating to the differences in the extent to which users of the different modes cover their infrastructure costs.

In general road transport users pay for the infrastructure they use, whereas rail users do not. However, the much lower marginal infrastructure and external costs for rail provide some economic justification for such a disparity. Figure 14 compares specific average cost ratios between road and rail.

Figure 14 Ratios road – rail (rail = 1) of selected countries

Passenger Transport (average costs per pkm):

Austria: Figures refer to vehicles < 3.5 tonnes

Germany: Figures refer to motorcycles, passenger cars and busses

Freight transport (average costs per tkm):

Figures refer to HGV + LGV for Finland, Germany, Italy, Netherlands, Sweden	 and UK

Austria: Figures refer to vehicles > 3.5 tonnes

Example: Average accident and environmental costs for road passenger transport in Austria are 9 times higher than rail costs.

### Monitoring Efficiency of Pricing Schemes

We have already shown above a comparison of revenues and infrastructure costs.  Given evidence that road marginal infrastructure costs are somewhat below average costs, it is clear that charges on average exceed marginal infrastructure costs for road.  For rail the picture is more variable.  For efficient pricing, the margin between marginal infrastructure costs and prices should of course equate to external cost. However, the only element of external cost for which the average value may provide a good approximation to average marginal cost is environmental cost.  Accordingly, average environmental costs for road and rail are presented below.

The following figure is showing the results for several countries. They amount up to 0.04 Euro per vkm for road and 1.4 Euro per track km. The variance between countries is rather high due to different traffic density, different pollution levels and different valuation of environmental nuisances (according to GDP differences).

Figure 15: Average environmental costs road 1998. Costs in € per total vkm (all road modes).

Remark: For Denmark no noise costs available.

Figure 16: Average environmental costs rail 1998. Costs in € per total vkm (all rail modes).

Remarks: For Denmark no noise costs available.

One indicator of pricing efficiency might be the level of subsidy. Results for this variable for rail transport are shown in Fig 17. As explained above, divergences between average and marginal cost mean that we would generally expect a certain level of subsidies to be desirable for rail transport. However, the variability in Fig 17 is enormous, and is undoubtedly partly explained by differences in accounting conventions rather than in the true level of subsidies.

Figure 17 Subsidies for rail transport in selected countries

Future development of accounts

The pilot accounts elaborated in UNITE provide a sound basis for the future development of accounts. This basis can be used for the further elaboration of accounts on a national level, further improvements and differentiations on a scientific basis and the development of a comprehensive European data basis for costs and revenues of the transport sector.

D14 (Link. et.al.) has recommended three elements which could be elaborated further, either on national level (extension of the national approach described above) or on expert basis (National or EU research).

- Disaggregated regional accounts: Most interesting are urban accounts (see FISCUS 2001) and specific corridor accounts. The outputs are comparable to the national accounting elements. Most tricky is appropriate systems delimitation.  It is useful to choose a system delimitation which follows the institutional set up (for example urban public transport network and operators). Otherwise the allocation problems are very dominant.

- Disaggregated accounts with social dimensions: The social accounts can be described as a differentiation according to income or other user groups. Such accounts go along the logic of incident analysis and can be used for the evaluation of distributional effects of specific policy measures.

- Social welfare accounts: As recommended in D4 (Proost et.al. 2001), social welfare accounts are able to show the level of welfare. A comparison of different time periods for example can be used to show the change of welfare (based on specific policy measures for instance).

Directive 1108/70 requests periodic information on basic transport data and information on costs (expenses) and revenues. Due to a rather weak implementation, the information is not available on a comprehensive basis.

An improved approach should relate to the outcome and experience of the UNITE pilot accounts. At the same time it has to be considered, that the efforts to produce such figures are rather high and the incentives rather low. In addition it will become more difficult in the future to have access to data of privatised institutions. Thus a pragmatic but coherent approach should be used. Although it would be interesting to produce annual accounts based on the recommendations made in Chapter 5.2, the risk of incomplete and low enforcement will be too high.

Thus we recommend that the elaboration of accounts information should concentrate on basic information for expenses, revenues and indicators for social costs which are usually available in public finance and transport statistics and can be compiled in an easy way.

The following table illustrates the necessary information.

Table 9. Basic information on EU-level for the future development of national accounts

| Information | Use/Compilation on EU-level |
| --- | --- |
| Basic Socioeconomic and transport figures | Basic information to produce indicators |
| Road infrastructure expenses (investments, running expenses and revenues (charges/taxes) | Transformation to cost information acc. to standardised method Benchmarking of costs per km, Cost recovery rates (HGV, Pass. Car) |
| Infrastructure accounts of national rail infrastructure operators Costs and track charges | Financial cost recovery of rail infrastructure by track charges |
| Infrastructure accounts of international airport operators: Aviation costs and landing/ airport charges | Financial cost recovery of airport infrastructure (aviation) by user charges |
| Infrastructure accounts of international ports | Financial cost recovery of ports by user charges |
| Additional information on subsidies per mode Subsidies for public service obligations Public loans (à fonds perdu, reduced interests) for infrastructure and transport operators | Subsidy monitoring / State aids |
| Add. Information on accident and environmental costs per mode: Indicators for - accidents/fatalities per mode - Insurance expenses - Air pollution emissions - Noise exposed population | Estimation of accident and environmental costs Sustainability monitoring (e.g. EEA/TERM) |
| Add. Information on delays/punctuality per mode | Quality monitoring of the transport sector. |

Based on such figures, it is possible to produce periodical updates of national social cost accounts, which can be compared on a European level. In order to strengthen the quality of information, it will be useful that countries would be able to present their own social cost estimates (e.g. based on expert studies or national statistics).

# Integration

## Alternative concepts of integration

We can distinguish two concepts of integration.  The first is a “hardwired” concept in which the marginal cost and accounts data are combined together to create a common and unified set of data which is then used as the basis for policy-making.  This has been referred to as the “ideal accounts” approach, which would show relevant marginal cost figures side by side with average/total cost figures.

On the basis of experience with UNITE, we have two different sets of reservations about this approach.  The first is that the theoretical interpretation of an ideal account, and hence its use for policy, is not capable of resolution. There is no single “correct” way in which marginal cost and social accounting information can be integrated together to provide unique guidance for policy-making.  It is better to think of policy-making as relating to various objectives and constraints - efficiency, equity, financial sustainability, practicality and so on - to which both marginal cost and accounting data contribute information.

A second, more pragmatic reservation is that the “ideal account” approach could be very inflexible.  What might suit the laws, history and culture of Sweden might not suit Germany, or France, or the UK, or Portugal.  It is important in practice to develop methods, approaches and data systems which are sufficiently flexible to be applied in a wide range of political, social and economic circumstances.  This will be even more the case in the context of the accession countries.  A flexible approach is much preferable to a rigid model which, even if it could be established and agreed upon (which we doubt), would need many resources to maintain and update it.

Both in principle and from the point of view of practical politics, therefore, we conclude that a less ambitious concept of integration is required.  A general and plausible definition of integration must mean usage of marginal cost and accounts information jointly but as two different sets of data.  The main relevant policy context for integration is in consideration of transport pricing packages.  Some elements of such packages may be based on accounts information (eg fixed charges should reflect revenue raising needs).  Other elements (variability with use, vehicle type etc) may be based on pure marginal cost information.  For some cost categories, marginal costs may, in fact, be accounts-based approximations which use engineering or accounting rules of thumb to set the marginal cost/output relationships.

Despite their obvious limitations, given how laborious and difficult it is to derive marginal cost estimates across all relevant dimensions of the problem, we think careful use of accounts information in conjunction with well-founded engineering relationships is a very practical approach which helps in estimating “average marginal costs” and in transferring estimated marginal costs between contexts.

It has been usual to refer to the marginal cost approach and accounts approach as competing approaches to transport pricing.  This is naïve - practical policy-making relies on both with the possibility of considering marginal cost pricing and average cost pricing as special cases within a spectrum.

## Integration of transport pricing within the macro-economic context

In D4, we set out the principles of transport pricing as follows:

“Short-run marginal social cost pricing is the benchmark for an efficient transport pricing policy.  This principle holds where there are no other price distortions in the economy and no income distribution concerns.  The SRMSC contains the marginal resoure costs, marginal infrastructure cost, scarcity or external congestion costs and external environmental and accident costs.  The SRMSC principle ensures that existing infrastructure is used as efficiently as possible.  Whether the infrastructure is at an efficient level or not and whether it is optimally maintained or not has an impact on the level of the SRMSC but not on the SRMSC principle”

(UNITE D4 page (iii))

That statement points us towards some of the key policy questions in transport price-setting:-

- How robust is the SRMSC pricing rule?

- What if there are price distortions elsewhere in the economy?

- What if there are budget constraints?

- What if notions of distributive justice, fairness, equity are relevant considerations in social utility?

- What if there are legal or ownership constraints within the transport sector such that some transport sub-sectors are priced commercially while others are not?

This area is explored in the economic modelling work of D13.  The main policy implications of that work are now drawn out and discussed.

In the integration work-stream, a number of different models were used to test the implications of alternative pricing rules – namely simple average cost pricing, marginal social cost pricing and Ramsey pricing (marginal cost pricing with and without a budget constraint.

The first set of models were partial equilibrium models for specific cities,  It was found that in every case, simple average cost pricing based on accounts information reduced welfare relative to the status quo.  This is a very powerful result which shows that tariffs based on naïve cost recovery principles perform poorly in efficiency terms.  As expected, marginal social cost pricing increased welfare relative to the status quo.  Ramsey pricing lay between average and marginal costs pricing, but generally was significantly worse in welfare performance than pure marginal social cost pricing.

Partial equilibrium models of the transport sector cannot, by their nature, take account of distortions elsewhere in the economy or deal adequately with the sources and uses of revenue.  Therefore, two general equilibrium models were used to explore this;  a simple model for Belgium and a much larger model for Switzerland.  The Belgian model was able to address the issue of the distribution of costs and benefits by income group.  These models do provide a conceptual framework within which the questions posed above can be addressed.

The result for Belgium compare the performance of average and marginal cost pricing, allowing the revenue surplus to be distributed either via lower labour taxes or via higher social security transfers.  As with the partial equilibrium approach, average cost pricing made all income groups worse off by on average half of one percent of their income relative to the baseline.  Marginal cost pricing makes most groups better off by around half of one percent, but the incidence depends crucially on how the revenue surplus from the sector is distributed and on the social weight attached to a euro accruing to low versus high income groups

Another feature of the Belgian study is that in the Baseline Case, the revenue to cost ratios are 2.5 for road, 0.38 for rail, and 0.37 for other public transport.  Applying sub-sector cost recovery constraints pushes prices in the wrong direction relative to marginal social costs, and causes welfare losses.  Clearly we have not performed a social cost-benefit analysis of Belgian railways and public transport, and cannot comment on the social value for money of 1 euro of public revenue support.  However, we can say in

general terms that if the transport sector is composed of a roads sub-sector subject to increasing SRMSC with use and a public transport sub-sector subject to falling SRMSC with use, then it will be inefficient to impose budgetary equilibrium at sub-sector level and it will be efficient to generate surpluses from the roads sector and to provide revenue support for public transport.  Clearly, efficient levels of investment and disinvestments will depend on cost-benefit analysis principles (see Dings et al (2002) for an interesting discussion).

The Swiss model does not distinguish between income groups, but it does distinguish the impacts of various pricing scenarios on urban and rural households.  A crucial question is the charging treatment of transit traffic through Switzerland since higher charge scenarios generate windfall gains for Swiss domestic taxpayers within the model.

The Swiss model also shows some very interesting results.  Again, marginal cost pricing performs better than average cost pricing, though the magnitude of the welfare effects are rather smaller than in the Belgian study (-0.1 to +0.2 of one percent of income).  Urban households lose from marginal social cost pricing while rural households gain – thus there would be spatial redistribution effects.  The size of the welfare changes depends on the flexibility with which the budget constraint is applied (ie preferably at transport sector level rather than at modal level). The results demonstrate as well the sensitivities of the model approach in regard to levels of congestion. The more congestion in a country, the higher the marginal cost prices, the more likely the revenues exceed total infrastructure costs. On the other hand, the Swiss case illustrates also a political trade off. The application of marginal social cost pricing is not in line with national transport policy goals, which favours traffic diversion from road to rail.

The results of the economic modelling would suggest important conclusions for policy-makers.  First, transport accounts should not be used simplistically, as a way of introducing and enforcing average cost pricing on all modes.  This would be worse, probably significantly so, than maintaining the unsatisfactory status quo.  Secondly, transport accounts – whilst valuable sources of information – do not provide a comprehensive basis on which to judge whether pricing reform has been beneficial.  In particular, transport accounts do not normally contain measures of benefits, nor of how these are distributed across the population.  To assess this, some form of economic model with suitable elasticities and sectoral relationships is required.  Thirdly, the change in net revenue raised in the transport sector under pricing strategies, requires careful examination; no firm conclusions can be drawn without tracing through, or at least assuming, how this money is raised or used.  Moreover, where budget constraints need to be imposed on transport for revenue-raising purposes, they should be imposed as flexibly as possible; budget constraints at sector level will be less damaging than cost recovery constraints on individual modes.

One major issue that has not been dealt with in UNITE is how to reconcile efficient transport pricing policies with institutional arrangements that promote technical and allocative efficiency in production.  It is sometimes argued that this can only be achieved by private provision with 100% cost recovery.  We do not believe this approach to be either necessary or sufficient for efficiency in production.  Alternative approaches such as competitive tendering or franchising may ensure efficient production even of subsidised commodities.  The widespread pressure of monopoly in the transport sector necessitates regulation and means that 100% cost recovery is quite compatible with inefficiency in production.

A more tricky problem is that of appropriate incentives for investment under short run marginal social cost pricing.  Any infrastructure authority, public or private, that is keen to increase its revenue will have an incentive to restrict capacity to push the appropriate price up.  On the other hand, a policy of long run marginal social cost pricing, provided that the infrastructure authority may only charge capital costs (including an appropriate profit margin) when it incurs them will provide appropriate incentives to expand capacity as long as users are willing to pay the costs involved.  However, this may be at the cost of considerable price distortion until the investment takes place.  A key issue in choosing between the two is the length of the lag in adjusting capacity to demand, versus the lag in customers adjusting demand to price (for instance by changing location).

These issues are outside the scope of UNITE, but are important issues for future research.

# Overall conclusions

# The UNITE project has served to illustrate that a variety of methods are needed in order to estimate marginal social cost, working together to overcome difficulties in the availability of appropriate data. A pragmatic approach, using a combination of cost allocation, econometric and engineering models is needed, with the precise approach differing between cost categories. The most important category of external cost in general for the transport sector is congestion (especially for road transport in urban areas), although various elements of environmental cost can be important particularly in urban areas. Both congestion and environmental costs vary greatly from context to context, so great caution needs to be exercised in transferring results from one context to another. External accident costs appear to be less important than was previously believed, mainly because earlier studies had failed to identify correctly the external element of these costs.

A general warning should be given that many earlier studies of all cost categories used methodologies that would now be considered inappropriate and their results should be used with great care. The main priorities for further research are the treatment of congestion and scarcity in rail and air transport, and the development of better methods for transferring results from one context to another.

The accounts have been shown to yield useful information particularly for monitoring and for the estimation of many categories of marginal cost. They provide crucial evidence on the relative importance of different categories of cost and whether these are reducing or increasing over time. Ideally they would be more disaggregate, giving data for instance for individual cities or types of area rather than purely national data, and extended to give some indicators of changes in user benefits over time, and how this is distributed across the population. Given the amount of work involved in them we do not see it as worthwhile to collect this information every year, since year to year changes are typically small; every 3-5 years would suffice for monitoring purposes.

The key lesson emerging from the work on integration is that both marginal cost estimates and accounts information are needed for a practical approach to pricing policy formulation (and indeed accounts may often provide the basic data for estimating marginal cost). But we should avoid the crude use of accounts information. For instance, consideration of how much revenue to raise within the transport sector needs examination of sources and uses of revenue elsewhere in the economy; the approach of allowing for this by means of a simple shadow price of public funds in the accounts does not allow for the different possibilities in terms of sources and uses of revenue. Even in the presence of budget constraints we should not resort to simple measures such as the use of accounts to implement average cost pricing.  Moreover, where budget constraints are needed, they should be implemented flexibly, with provision for cross subsidisation between modes, if they are to do least damage to economic efficiency.  The simple imposition of 100% cost recovery rules on all modes is likely to worsen, rather than improve, economic efficiency compared with the status quo.

# References

Dings, J M W, et al, 2002: Return on roads, C E Delft, Netherlands

Link, H, et .al,. 2002:  	UNITE Future approach to accounts  UNITE D14

Proost, S, et .al, 2001:	 UNITE Integration approach UNITE D4

Sansom, T, et al, 2000: The overall UNITE Methodology, UNITE D1

TERM 2001:	Are we moving in the right direction? Indicators and environment integration in the EU, European Environmental Agency, Copenhagen 2001

FISCUS 2000: Viegas, J, et. al, 2000: . Cost evaluation and financing schemes for urban transport systems, FISCUS final report

Literature