| ## Appendix D1 – Detailed description of the Norwegian National Model System for Passenger Travel (NTM) Appendix D2 – Calculation of the changes in operating costs due to an increase in rail frequency Appendix D3 – Model calculations for the scenarios CONTRACT N° : GMA2/2000/32056-S12.335673 PROJECT N° : GMA2/2000/32056-S12.335673 ACRONYM : SPECTRUM TITLE : Study of Policies regarding Economic instruments Complementing Transport Regulation and the Undertaking of physical Measures PROJECT CO-ORDINATOR : Dr Susan Grant-Muller, Institute for Transport Studies PARTNERS : Institute for Transport Studies, University of Leeds (ITS) University of Antwerp (UA) Transport studies unit, University of Oxford (OXFJQ) Technical Research Centre of Finland (VTT) University of Technology Vienna (TUV-IVV) University of Las Palmas (EIET) University of Budapest (BUTE) Center for Economic Studies (K.U.Leuven) Institute of Studies for the Integration of Systems (ISIS) Institute of Transport Economics, Oslo (TOI) University of Gdansk (UG) PROJECT START DATE : 01/09/2002 DURATION : 36 months Date of issue of this report : 24th January 2005 |  |  |
| --- | --- | --- |
|  | Project funded by the European Community under the ‘Competitive and Sustainable Growth’ Programme (1998-2002) |  |

This document has been produced by the participants in the SPECTRUM project, under the 5th Framework Programme for RTD. It reflects solely the authors' views. The European Commission is in no way responsible or liable for the contents of this document or the use made of it.

## Appendix D1 – Detailed description of the Norwegian National Model System for Passenger Travel (NTM)

The model system is designed to produce medium and long-term passenger travel forecasts. The model system represents the passenger travel market for long distance travel (over 100 Kilometres one way) in detail and a wide range of decisions including car ownership, mode and destination choice, and frequency of travel. Long distance travel is segmented into 5 travel purposes (business work, visit, recreation and other private travel purposes) covering 5 travel modes (car, air, rail, bus and boat).

The model system is designed to deal with different scenarios defined in terms of factors that influence demand for travel. Thus not only trends in factors such as relative prices, level of transport services, income, and population are to be accounted for but also compositional changes in population that will have a bearing on the demand for travel. Therefore the model system had to retain a high degree of disaggregation.

The model system has the following components:

- Disaggregate and aggregate license holding models

- Disaggregate car ownership models

- Disaggregate models for long distance travel demand comprising models for travel frequency and models for mode and destination

- Segmentation model

- Network model

i) Aggregate and Disaggregate Licence Holding Models

Disaggregate logit models estimates licence holding at the household level. Two separate models are estimated for this purpose. The first model predicts licence holding of the head of the household and his or her partner. The second model predicts the licence holding of other adults in the household. License holding predictions from these models are compared with aggregate licence holding predictions and calibrated to produce similar results. The aggregate licence holding predictions contain trend effects that are not captured in the cross-sectional models. The aggregate licence holding model was constructed in such a manner to capture licence-holding penetration in population over time. The aggregate licence holding model is based on the aggregate licence holding data from Norway.

ii) Disaggregate Car Ownership Models

Three binomial logit models estimate car ownership at a household level when there are 1, 2 or 3 or more persons of age 18 or older in the household. In these models, car ownership is explained by household's socio-economic. These variables are:

- Sex and age

- Income

- Couple with children

- Number of children of age 0-12

- Dummy for large towns

iv) Disaggregate Models for Long Distance Travel Demand

These models consist of travel frequency (trip generation) models and joint travel destination mode choice models for each travel purpose.

Disaggregate binomial logit models estimate number of tours at individual level, for various travel purposes. Trip frequency is explained by the socio-economic characteristics of an individual in a household, household characteristics, car availability, level of urbanization of the place of residence of the individual, employment density and logsum variable from the joint destination and mode choice models. The logsum variable describes accessibility of a location to alternative destinations.

Joint travel destination and mode choice models predict travel destination and the mode of travel among car, bus, boat, rail and air on an individual level. Mode/destination models are structured logit models. Variables such as population density, employment density, total employment, hotel and restaurant employment, retail employment, student enrolment and total population were used to describe attraction of destinations. A zonal system based on 1428 communities in Norway was used for this purpose.

Attributes of different modes such as different cost and time components, and frequency of service describe mode choice. Network data was used for the long distance models. For this purpose car, bus, boat, rail, and air networks were coded in terms of 1428 municipalities in Norway in the EMME/2 system. Mode choice is also described by other variables such as car availability within the household and income of the individual.

In the estimation of joint destination and mode choice models, the method of sampling alternative destinations was adopted. Twenty stratified destinations in addition to the chosen destination were used for estimation. Figure D1.1 shows the structure of the frequency and mode/destination choice models.

Figure D1.1. Structure of the frequency and mode/destination choice models.

The main variables in the frequency and mode/destination choice models are presented in Table D1.2.

Table D1.2. The main variable in the frequency and mode/destination choice models

| Level of service, car | Level of service, public transport | The individual | Zones / Attractions at destination |
| --- | --- | --- | --- |
| - Travel time ”door to door” - Fuel cost - Toll cost / ferry cost - Car availability (license holding and car ownership) | - Onboard time - Access/egress time - Waiting time (based on frequency) - Number of transfers - Fare | - Sex and age - Income (personal or for household) - Employment / profession - Household category and car availability - Zone/geography/centrality | - Population - Employment in different sectors of industry - Hotels (hotel beds) - Cottages / holiday houses - Transport services |

v) Segmentation model

The segmentation model relies on the results from licence holding and car availability model.

For every zone the population is divided into 7 each sex and age groups and into five groups depending on car availability as follows:

1.No licences, no cars in the household

2.No licence, but car in the household

3.Licence, but no car in the household

4.Licence, number of cars >= number of licences

5.Licence, number of cars < number of licences

Mode/choice model are run for each of these five groups, with different dummies for car availability.

v) Network model

The network model is based on 1428 zones. The model consists of road network and networks for rail, boat, bus and air consisting of about 80 transit lines for rail, 110 transit lines for boat, 600 transit lines for bus and 220 transit lines for air.

EMME/2 is used for assignment and calculating level of service data. Generalised cost of travel is used for car assignment. The following figure shows the base network in the Southern part of Norway.

Figure D1.3. Base network in the Southern part of Norway

The model system has been validated against data including mode share and some flows on different networks

Table D1.4. Income elasticity

|  | Business | Visit | Other private purposes | Total |
| --- | --- | --- | --- | --- |
| Car | 0.51 | 0.38 | 0.37 | 0.44 |
| Bus | 0.04 | 0.12 | 0.15 | 0.16 |
| Boat | -0.29 | 0.09 | 0.17 | 0.11 |
| Rail | 0.24 | 0.12 | 0.14 | 0.16 |
| Air | 0.75 | 0.45 | 0.47 | 0.59 |
| Total | 0.59 | 0.35 | 0.34 | 0.42 |

Table D1.5. Price elasticity values

|  | Totalt NTM5 |
| --- | --- |
| Car | -0.11 |
| Bus | -0.37 |
| Boat | -0.37 |
| Rail | -0.30 |
| Air | -0.35 |

Appendix D2 – Calculation of the changes in operating costs due to an increase in rail frequency

Table D2.1. Changes in cost for rail operator

| 1% increase in frequency pr year 2001-2012 = 11.57% increase |  |  |
| --- | --- | --- |
| Unit cost pr train pr day (Nkr) | 29886 | (Based on 15% diesel trains) |
| Unit cost pr train pr day (Euro) | 3628 |  |
| Number of trains in reference scenario: | 145 |  |
| 1% increase in frequency pr year implies | 16.8 | Extra trains |
| Extra cost because of increased frequency: | 60950 | Euro pr day |
| Cost pr train-km (Nkr 2003) | 15.5 |  |
| Cost pr train-km (Euro) | 1.88 |  |
| Train-km pr day in 2012 reference (according to NTM5) | 82587 |  |
| 1% increase in frequency pr year implies | 92142 | Train-km pr day |
| Increase in train-km pr day | 9555 |  |
| Extra cost because of increased frequency: | 17979 | Euro pr day |
| Total | 78930 | Euro pr day |

Appendix D3 – Model calculations for the scenarios

Table D3.1. Changes in number of trips  per day

|  | Alt. | Total | Car | Bus | Boat | Rail | Air |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference scenario 2012 | 0 | 209886 | 144961 | 7369 | 6273 | 15965 | 35318 |
| Fuel price +25% | 1 | 208352 | 142112 | 7555 | 6413 | 16316 | 35956 |
| Fuel price+50% | 2 | 207002 | 139603 | 7723 | 6538 | 16633 | 36505 |
| Air fare-25% | 3 | 210556 | 143541 | 7255 | 6099 | 15771 | 37890 |
| Air frequency +1% per year | 4 | 210186 | 144285 | 7333 | 6214 | 15871 | 36483 |
| Rail fare -25% | 5 | 210153 | 144346 | 7319 | 6238 | 17073 | 35177 |
| Rail frequency +1% per year | 6 | 210028 | 144638 | 7349 | 6263 | 16527 | 35252 |
| Speed change, road network: B | 7 | 206819 | 139122 | 7722 | 6767 | 16657 | 36550 |
| Rail fare –25% | 8 | 204881 | 132310 | 7946 | 6902 | 19050 | 38673 |
| Fuel +25%, rail fare-10%, rail&air freq +1% | 9 | 206139 | 134733 | 7784 | 6783 | 18703 | 38136 |
| Speed change: A | 10 | 208446 | 142178 | 7532 | 6583 | 16276 | 35876 |
| Fuel +25%, rail fare-10%, rail&air freq +1% | 11 | 206406 | 135271 | 7761 | 6729 | 18638 | 38007 |
| Rail fare +10% | 12 | 209795 | 145170 | 7387 | 6285 | 15588 | 35365 |
| Rail fare -10% | 13 | 209985 | 144732 | 7350 | 6260 | 16376 | 35266 |
| Fuel cost -25% | 14 | 211681 | 148300 | 7161 | 6110 | 15564 | 34546 |
| Fuel +25%, rail fare-10%, rail freq +1%/year | 15 | 208603 | 141547 | 7512 | 6389 | 17325 | 35831 |
| Fuel +25%, rail fare+10%, rail freq +1%/year | 16 | 208401 | 142004 | 7552 | 6416 | 16492 | 35938 |
| Fuel –25%, Speed change A | 17 | 210190 | 145459 | 7323 | 6420 | 15877 | 35112 |
| Railfare –10%, rail freq +1%/year, Speed A | 18 | 208696 | 141615 | 7490 | 6558 | 17280 | 35753 |

Table D3.2. Percentage change in no of trips relative to the reference scenario in 2012

| Prosentvis endring i antall turer | % change in trips relative to the reference scenario |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario | Alt | Total | Car | Bus | Boat | Rail | Air |
| Fuel price +25% | 1 | -0.7% | -2.0% | 2.5% | 2.2% | 2.2% | 1.8% |
| Fuel price+50% | 2 | -1.4% | -3.7% | 4.8% | 4.2% | 4.2% | 3.4% |
| Air fare-25% | 3 | 0.3% | -1.0% | -1.6% | -2.8% | -1.2% | 7.3% |
| Air frequency +1% per year | 4 | 0.1% | -0.5% | -0.5% | -0.9% | -0.6% | 3.3% |
| Rail fare -25% | 5 | 0.1% | -0.4% | -0.7% | -0.6% | 6.9% | -0.4% |
| Rail frequency +1% per year | 6 | 0.1% | -0.2% | -0.3% | -0.2% | 3.5% | -0.2% |
| Speed change, road network: B | 7 | -1.5% | -4.0% | 4.8% | 7.9% | 4.3% | 3.5% |
| Package 1 | 8 | -2.4% | -8.7% | 7.8% | 10.0% | 19.3% | 9.5% |
| Package 2 | 9 | -1.8% | -7.1% | 5.6% | 8.1% | 17.2% | 8.0% |
| Speed change: A | 10 | -0.7% | -1.9% | 2.2% | 4.9% | 2.0% | 1.6% |
| Package 3 | 11 | -1.7% | -6.7% | 5.3% | 7.3% | 16.7% | 7.6% |
| Rail fare +10% | 12 | 0.0% | 0.1% | 0.2% | 0.2% | -2.4% | 0.1% |
| Rail fare -10% | 13 | 0.0% | -0.2% | -0.3% | -0.2% | 2.6% | -0.1% |
| Fuel cost -25% | 14 | 0.9% | 2.3% | -2.8% | -2.6% | -2.5% | -2.2% |
| Fuel +25%, rail fare-10%, rail freq +1%/year | 15 | -0.6% | -2.4% | 1.9% | 1.8% | 8.5% | 1.5% |
| Fuel +25%, rail fare+10%, rail freq +1%/year | 16 | -0.7% | -2.0% | 2.5% | 2.3% | 3.3% | 1.8% |
| Fuel –25%, Speed change A | 17 | 0.1% | 0.3% | -0.6% | 2.3% | -0.6% | -0.6% |
| Railfare –10%, rail freq +1%/year, Speed A | 18 | -0.6% | -2.3% | 1.6% | 4.5% | 8.2% | 1.2% |