| Appendix C1 – Detailed description of the model EURORAIL # Appendix C2 – Tables of the passenger case study London-Brussels-Paris CONTRACT N° : GMA2/2000/32056-S12.335673 PROJECT N° : GMA2/2000/32056-S12.335673 ACRONYM : SPECTRUM TITLE : Study of Policies regarding Economic instruments Complementing Transport Regulation and the Undertaking of physical Measures PROJECT CO-ORDINATOR : Dr Susan Grant-Muller, Institute for Transport Studies PARTNERS : Institute for Transport Studies, University of Leeds (ITS) University of Antwerp (UA) Transport studies unit, University of Oxford (OXFJQ) Technical Research Centre of Finland (VTT) University of Technology Vienna (TUV-IVV) University of Las Palmas (EIET) University of Budapest (BUTE) Center for Economic Studies (K.U.Leuven) Institute of Studies for the Integration of Systems (ISIS) Institute of Transport Economics, Oslo (TOI) University of Gdansk (UG) PROJECT START DATE : 01/09/2002 DURATION : 36 months Date of issue of this report : 24th January 2005 |  |  |
| --- | --- | --- |
|  | Project funded by the European Community under the ‘Competitive and Sustainable Growth’ Programme (1998-2002) |  |

This document has been produced by the participants in the SPECTRUM project, under the 5th Framework Programme for RTD. It reflects solely the authors' views. The European Commission is in no way responsible or liable for the contents of this document or the use made of it.

Appendix C1 – Detailed description of the model EURORAIL

# Introduction

A passenger Case Study will be undertaken involving a multi-modal analysis of passenger traffic on the London-Paris-Brussels corridor.  This would involve analysis of rail, air and road/ferry services and would consider previous relevant research relating to rail services and the Channel Tunnel Rail Link.

This will examine combinations of economic, regulatory and physical measures suggested by the local case studies and the literature review.

The following modes will be examined in the London-Brussels-Paris Case Study:

This modeling approach implies that the instruments examined in the case study will concern several modes (rail, air and car).

A number of studies have been undertaken in relation to the Channel Crossing traffic including, e.g. (Heddbaut 1996), (PETS 2000) and (EUROSIL 1997). The present study will build on these studies and others to examine the impact of a battery of transport policy instruments.

# Model overview

The model used is an adapted version of the so-called EURORAIL model used in 1998 to forecast market segments for international traffic using the Channel Tunnel. The basic structure is a series of mode choice models for the modes: rail, air, road (ferry or Le Shuttle). This is based on a binomial logit model framework, where two modes are considered at any one time.

This model requires three main groups of inputs:

- Demand matrices between each pair of zones by each of the current cross-Channel modes

- Network variables for each pair of zones

- Logit model parameters regarding the importance of the various network variables and passengers’ sensitivity to the variables

# Demand matrices

The demand matrices for the base year in the model runs will utilise different passenger surveys, including the International Passenger Survey (IPS), Sea Passengers returns, Accompanied Passengers Vehicles return, Air Passengers returns, Travel Trends supplement survey. The OD information is available for 1991 and 1996 thereby allowing examination of the influence of the Channel Tunnel on travel patterns. Tables 1 and 2 show country aggregated OD demand data for the base year 1996.

Table C1.1. Passenger journeys by sea (aggregated zones) thousand journeys

|  | Greater London | Rest of SE | Rest of UK | UK Unspecified | Total |
| --- | --- | --- | --- | --- | --- |
| Total France | 3344 | 7361 | 4787 | 54 | 15546 |
| Total Belgium-Lux | 776 | 1396 | 708 | 46 | 2927 |
| Total Netherlands | 553 | 685 | 1033 | 36 | 2308 |
| Total Germany | 1118 | 857 | 1338 | 40 | 3353 |
| Italy | 124 | 170 | 356 | 10 | 660 |
| Switzerland-Austria | 59 | 159 | 145 | 2 | 365 |
| Scandinavia | 87 | 158 | 478 | 10 | 732 |
| Spain/Portugal | 214 | 362 | 1059 | 11 | 1646 |
| Total above countries | 6275 | 11148 | 9905 | 209 | 27537 |

Sources: (Department for Transport 2003)

Table C1.2. Passenger journeys by Channel Tunnel (aggregated zones)

Thousand journeys

|  | Greater London | Rest of SE | Rest of UK | UK Unspecified | Total |
| --- | --- | --- | --- | --- | --- |
| Total France | 3555 | 3767 | 1858 | 19 | 9200 |
| Total Belgium-Lux | 932 | 860 | 298 | 29 | 2120 |
| Total Netherlands | 239 | 280 | 208 | 19 | 745 |
| Total Germany | 229 | 266 | 310 | 18 | 823 |
| Italy | 44 | 83 | 79 | 1 | 207 |
| Switzerland-Austria | 60 | 45 | 35 | 1 | 141 |
| Scandinavia | 22 | 2 | 2 | 0 | 25 |
| Spain/Portugal | 53 | 41 | 102 | 0 | 196 |
| Total above countries | 5135 | 5344 | 2892 | 87 | 13458 |

Source: (Department for Transport 2003)

Demand matrices for future years are estimated using simple growth rates to each modal matrix. These growth rates will be derived from analysis of historic trends and consideration to possible differential growth rates for different zones.

In Table 3 examples of recent traffic forecasts for the Channel Tunnel are shown, although other forecasts are available (DETR by Arthur D. 2000).

Table C1.3. Cross-Channel Traffic Growth Forecasts

|  | 1998-2000 (% p.a.) | 2001-2005 (% p.a.) | 2006-2010 (% p.a.) | 2011-2025 (% p.a.) |
| --- | --- | --- | --- | --- |
| Business inbound | 1.0 | 3.0 | 3.0 | 2.0 for 2011-2015, declining 0.5% every five year interval |
| Business outbound | 1.0 | 3.0 | 3.0 | 2.0 for 2011-2015, declining 0.5% every five year interval |
| Leisure inbound | 6.0 | 4.0 | 4.0 | 3.0 for 2011-2015, declining 0.5% every five year interval |
| Leisure outbound | 5.0 | 5.0 | 3.0 | 3.0 for 2011-2015, declining 0.5% every five year interval |
| Total | 5.0 | 4.0 | 4.0 | 3.0 for 2011-2015, declining 0.5% every five year interval |

Source: (DETR by Arthur D. 2000)

# Network variables

This includes information about fare, travel time, access and egress, service frequency for each pair of zones. Two main components of data are required for each mode:

- A representation of the physical network incl. details regarding distance

- Details concerning services offered by the operators (fares, service frequencies and routes) (air, ferry and rail).

As part of the previous application of the Eurorail model these data are available for the relevant network variables.

In addition, information about vehicle operating cost parameters has been based on (Affuso, Masson et al. 2003)

# Logit model parameters

The following generalised cost parameters will be used to calibrate the mode choice functions:

- Value of time (EUR/hr)

- In-vehicle time (as proportion of physical time)

- Headway effect (equivalent penalty in in-vehicle minutes to reflect effect of service frequency)

- Access time (as proportion of physical time)

- Interchange (mins) (Equivalent penalty in in-vehicle minutes to reflect the physical inconvenience of interchange).

- Board (mins) (Penalty for waiting to board. Calculated as twice the average wait time in terminal)

These parameters are available from the initial application of the Eurorail model to examine the effects of Regional Eurostar services (DETR by Arthur D. 2000).

In addition, information about demand elasticities is required (in particular fare and service elasticities). Again, elasticity values are available from the previous application of the model. Prior to the calibration of the choice functions the appropriateness of the elasticities will be reviewed. Otherwise, a significant collection of possible elasticity values exist from: BTE's Transport Elasticities Database Online http://dynamic.dotrs.gov.au/bte/tedb/index.cfm

# Outputs

This model provides forecasts for international traffic for a 30-year period. As such demand forecasts will be produced for business and leisure trips (with leisure trips further split into independent holiday, package tours and short-stay and excursions) covering each of the four modes - rail, air, car (Le Shuttle and ferry). Modal split and actual passenger numbers will be the main output. Forecasts of demand require assumptions on service level and fares for the different modes over the forecast period. Do-something scenarios can be tested with respect to network changes, service changes, tariff changes for all modes, as well as general socio-economic changes (e.g. population and income changes). The following impacts will be considered in the London-Brussels-Paris case study:-

- Forecast demand (by market segment)

- Revenue and costs (for infrastructure managers and service operators)

- User benefits

- Externalities

- Congestion costs, the Mohring effect for public transport, accidents, air pollution, global warming, noise (all from PETS and UNITE)

# Equity and efficiency

The model is well suited to determine effects in relation to the efficiency dimension where changes in consumers’ surplus, producers’ surplus and externalities can be estimated.

More difficulties concern the consideration to equity due to limited information about the socio-economic characteristics within overall demand. Possibilities are restricted to equity implications as regard to different zones (with different average income levels) and differences between leisure and business travelers.

# Appendix C2 – Tables of the passenger case study London-Brussels-Paris

# Table C2.1. Traffic data for Channel Tunnel

| Number per year | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 | 2003 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Le Shuttle Cars (vehicles) |  | 1222722 | 2076954 | 2319160 | 3351348 | 3260166 | 2784493 | 2529757 | 2335625 | 2278999 |
| Le Shuttle Coaches (vehicles) |  | 23383 | 57971 | 64579 | 96324 | 82074 | 79460 | 75402 | 71911 | 71942 |
| Eurostar (passengers) |  | 2920309 | 4866566 | 6004268 | 6307849 | 6596247 | 7130417 | 6947135 | 6602817 | 6314795 |

Source: Eurotunnel (various years) Annual Traffic Figures, www.eurotunnel.com

| Annual pct growth | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 | 2003 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Car (vehicles) | 69.9% | 11.7% | 44.5% | -2.7% | -14.6% | -9.1% | -7.7% | -2.4% |
| Coach (vehicles) | 147.9% | 11.4% | 49.2% | -14.8% | -3.2% | -5.1% | -4.6% | 0.0% |
| Rail (passengers) | 66.6% | 23.4% | 5.1% | 4.6% | 8.1% | -2.6% | -5.0% | -4.4% |

Source: Eurotunnel (various years) Annual Traffic Figures, www.eurotunnel.com

| Passenger modal split (%) | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Car | 27.6% | 25.9% | 25.3% | 29.1% | 29.7% | 26.2% | 24.8% | 24.5% |
| Coach | 31.1% | 36.1% | 33.7% | 36.6% | 32.7% | 31.9% | 32.6% | 32.2% |
| Rail | 41.2% | 38.0% | 41.0% | 34.3% | 37.6% | 41.9% | 42.6% | 43.3% |
| Total | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |

Source: (Department for Transport 2003) Transport Statistics for Great Britain 2003 and own calculations.

# Table C2.2. Sea Passenger Trends

| Sea Passenger Trends (Cross-Channel) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 2000 | 2001 | 2002 | 2003 |
| Eastern Channel | 16923 | 16301 | 16838 | 15176 |
| Dover - Calais | 14970 | 14679 | 15406 | 14084 |
| Dover - Ostend | 814 | 663 | 266 | n |
| Dover - Dunkerque | 75 | 306 | 445 | 547 |
| Folkestone - Boulogne | 440 | 5 | 0 | 0 |
| Ramsgate - Ostend | 76 | 88 | 117 | 137 |
| Newhaven - Dieppe | 313 | 337 | 379 | 397 |
| Other routes | 234 | 224 | 225 | 11 |
| Western Channel | 4274 | 4514 | 4666 | 4357 |
| Plymouth - Roscoff | 458 | 447 | 498 | 478 |
| Poole - Cherbourg | 402 | 510 | 537 | 532 |
| Portsmouth - Caen | 909 | 889 | 894 | 942 |
| Portsmouth - Cherbourg | 848 | 942 | 939 | 772 |
| Portsmouth - Le Havre | 741 | 803 | 859 | 755 |
| Portsmouth - St Malo | 479 | 489 | 502 | 457 |
| Other Routes | 437 | 434 | 438 | 421 |
| Pct growth |  |  |  |  |
| Eastern Channel |  | -3.7% | 3.3% | -9.9% |
| Dover - Calais |  | -1.9% | 5.0% | -8.6% |
| Dover - Ostend |  | -18.6% | -59.9% |  |
| Dover - Dunkerque |  | 308.0% | 45.4% | 22.9% |
| Folkestone - Boulogne |  | -98.9% | -100.0% |  |
| Ramsgate - Ostend |  | 15.8% | 33.0% | 17.1% |
| Newhaven - Dieppe |  | 7.7% | 12.5% | 4.7% |
| Other routes |  | -4.3% | 0.4% | -95.1% |
| Western Channel |  | 5.6% | 3.4% | -6.6% |
| Plymouth - Roscoff |  | -2.4% | 11.4% | -4.0% |
| Poole - Cherbourg |  | 26.9% | 5.3% | -0.9% |
| Portsmouth - Caen |  | -2.2% | 0.6% | 5.4% |
| Portsmouth - Cherbourg |  | 11.1% | -0.3% | -17.8% |
| Portsmouth - Le Havre |  | 8.4% | 7.0% | -12.1% |
| Portsmouth - St Malo |  | 2.1% | 2.7% | -9.0% |
| Other Routes |  | -0.7% | 0.9% | -3.9% |

Source: (Department for Transport 2003) Transport Statistics Bulletin, Sea Passenger Bulletin

# Table C2.3. Air Passenger Trends

| Total # passengers | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 | 2003 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LHR-Paris Charles de Gaule | 1893785 | 1839086 | 1846290 | 1883927 | 1964524 | 2120079 | 2150791 | 1970345 |
| LHR – Paris Orly | 498722 | 433607 | 451056 | 428877 | 378915 | 38002 |  |  |
| LGW-Paris Charles de Gaule | 314173 | 305433 | 314002 | 322498 | 312083 | 281754 | 278064 | 288997 |
| LGW - Paris Orly | 1044 | 188 |  |  |  |  |  |  |
| London City – Paris | 42910 | 95242 | 77830 | 72121 | 96101 | 88140 | 71331 | 43035 |
| London City - Paris Orly | 34514 | 4 |  |  |  |  | 13322 | 83775 |
| Stansted-Paris | 126524 | 125685 | 119704 | 119030 | 161889 | 178081 | 178921 | 34766 |
| LHR-Brussels | 974172 | 1068698 | 1064499 | 1080766 | 1163877 | 1022320 | 968982 | 838337 |
| LGW-Brussels | 126640 | 173807 | 227837 | 227946 | 118963 | 54050 | 49356 | 71831 |
| London City – Brussels | 41808 | 61066 | 74728 | 77138 | 86940 | 56283 | 52774 | 70348 |
| Stansted-Brussels | 60735 | 62512 | 90455 | 133295 | 32573 | 1031 |  |  |
| Total Paris | 2911672 | 2799245 | 2808882 | 2826453 | 2913512 | 2706056 | 2692429 | 2420918 |
| Total Brussels | 1203355 | 1366083 | 1457519 | 1519145 | 1402353 | 1133684 | 1071112 | 980516 |
| TOTAL | 4115027 | 4165328 | 4266401 | 4345598 | 4315865 | 3839740 | 3763541 | 3401434 |
| Annual Pct Growth |  |  |  |  |  |  |  |  |
| London – Paris |  | -3.9% | 0.3% | 0.6% | 3.1% | -7.1% | -0.5% | -10.1% |
| London – Brussels |  | 13.5% | 6.7% | 4.2% | -7.7% | -19.2% | -5.5% | -8.5% |
| London - Paris/Brussels |  | 1.2% | 2.4% | 1.9% | -0.7% | -11.0% | -2.0% | -9.6% |

Source: Civil Aviation Autority (various years) International Air Passenger Traffic to/from UK airports

# Table C2.4. Social Marginal Costs by Mode for United Kingdom, France, Belgium and EU15 (c per passenger kilometre)

|  |  | FR | BE | LU | UK | EU15 |
| --- | --- | --- | --- | --- | --- | --- |
| Accidents | Air |  |  |  |  |  |
| Congestion | Air |  |  |  |  |  |
| Global Warming | Air | 1.31 | 1.39 | 1.39 | 1.16 | 1.24 |
| Infrastructure | Air |  |  |  |  |  |
| Noise | Air | 0.06 | 0.05 | 0.01 | 0.09 | 0.07 |
| Air pollution | Air | 0.08 | 0.07 | 0.07 | 0.03 | 0.04 |
| MSC TOTAL | Air | 1.45 | 1.51 | 1.47 | 1.29 | 1.36 |
| Accidents | Bus | 0.09 | 0.18 | 0.17 | 0.06 | 0.11 |
| Congestion | Bus | 0.10 | 0.11 | 0.11 | 0.09 | 0.10 |
| Global Warming | Bus | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| Infrastructure | Bus | 0.17 | 0.18 | 0.18 | 0.15 | 0.16 |
| Noise | Bus | 0.01 | 0.02 | 0.02 | 0.02 | 0.01 |
| Air pollution | Bus | 0.65 | 0.46 | 0.46 | 0.14 | 0.29 |
| MSC TOTAL | Bus | 1.04 | 0.98 | 0.96 | 0.48 | 0.69 |
| Accidents | Car | 1.09 | 1.85 | 1.6 | 0.55 | 1.05 |
| Congestion | Car | 0.40 | 0.48 | 0.48 | 0.44 | 0.44 |
| Global Warming | Car | 0.16 | 0.17 | 0.17 | 0.14 | 0.15 |
| Infrastructure | Car | 0.63 | 0.44 | 0.44 | 0.36 | 0.39 |
| Noise | Car | 0.04 | 0.06 | 0.06 | 0.05 | 0.04 |
| Air pollution | Car | 0.72 | 0.68 | 0.68 | 0.35 | 0.42 |
| MSC TOTAL | Car | 3.03 | 3.67 | 3.42 | 1.89 | 2.50 |
| Accidents | Ferry |  |  |  |  |  |
| Congestion | Ferry |  |  |  |  |  |
| Global Warming | Ferry | 0.25 | 0.27 | 0.27 | 0.22 | 0.24 |
| Infrastructure | Ferry |  |  |  |  |  |
| Noise | Ferry |  |  |  |  |  |
| Air pollution | Ferry | 5.59 | 4.99 | 4.99 | 2.32 | 3.08 |
| MSC TOTAL | Ferry | 5.85 | 5.26 | 5.26 | 2.54 | 3.32 |
| Accidents | Rail | 0.05 | 0.06 | 0.06 | 0.05 | 0.05 |
| Congestion | Rail | 0.06 | 0.07 | 0.07 | 0.06 | 0.06 |
| Global Warming | Rail | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| Infrastructure | Rail | 0.61 | 0.48 | 0.48 | 5.67 | 0.43 |
| Noise | Rail | 0.03 | 0.04 | 0.04 | 0.03 | 0.03 |
| Air pollution | Rail | 0.36 | 0.36 | 0.36 | 0.19 | 0.22 |
| MSC TOTAL | Rail | 1.13 | 1.03 | 1.03 | 6.01 | 0.81 |

# References