COMPETITIVE AND SUSTAINABLE GROWTH

(GROWTH)

PROGRAMME

Deliverable 12 Annex 5 The Pilot Accounts for ITALY

- Version 2.1

- June 2003

Authors: Andrea Ricci, Riccardo Enei, Rita Esposito, Patrizia Fagiani, Ferdinando Giammichele, Gianluca Leone, Donato Pellegrini (ISIS),

Heike Link, Louise Stewart (DIW), Peter Bickel (IER)

Contract: 1999-AM.11157 Project Co-ordinator: ITS, University of Leeds Funded by the European Commission 5th Framework – Transport RTD

UNITE Partner Organisations ITS/UNIVLEEDS (UK), DIW (De), NEI (Nl), CES/KUL (Be), TIS.PT (Pt), IWW/UNIKARL (De), VTI (Se), IER/USTUTT (De), CERAS/ENPC (Fr), HERRY (Au), EIET/ULPGC (Es), ISIS (It), STRATEC (Be), SYSTEMA (Gr), JP-TRANSPLAN (Fi), VATT (Fi), ECOPLAN (Ch), INFRAS (Ch), EKI (Se)

- UNITE

1999-AM.11157

- UNIfication of accounts and marginal costs for Transport Efficiency

- Pilot Accounts – Results for Italy

This document should be referenced as:

A. Ricci, R. Enei, R. Esposito, P. Fagiani, F. Giammichele, G. Leone, D. Pellegrini (ISIS), UNITE (UNIfication of accounts and marginal costs for Transport Efficiency) Working Funded by 5th Framework RTD Programme. ITS, University of Leeds, Leeds, December 2001.

June 2003

Version No: 2.0

Authors: as above

- PROJECT  INFORMATION

- Contract no: 1999-AM.11157:

- UNIfication of accounts and marginal costs for Transport Efficiency

- Website: www.its.leeds.ac.uk/unite

- Commissioned by: European Commission – DG TREN; Fifth Framework Programme

- Lead Partner:	Institute for Transport Studies, University of Leeds (UK)

Partners: 	ITS/UNIVLEEDS (UK), DIW (De), NEI (Nl), CES/KUL (Be), TIS.PT (Pt), IWW/UNIKARL (De), VTI (Se), IER/USTUTT (De), CERAS/ENPC (Fr), HERRY (Au), EIET/ULPGC (Es), ISIS (It), STRATEC (Be), SYSTEMA (Gr), JP-TRANSPLAN (Fi), VATT (Fi), ECOPLAN (Ch), INFRAS (Ch), EKONO (Fi), EKI (Se)

- DOCUMENT  CONTROL  INFORMATION

- Status: 	Accepted

Distribution:	UNITE partners

Availability:	Public (only once status is “Accepted” as part of Deliverable 12)

Filename:	UNITE D12 Annex 5

Quality assurance:

Co-ordinator’s review:	CAN

Signed: 	Date:

- Table of contents

1	Introduction	8

1.1	Study context and objectives of this annex report	8

1.2	The accounts approach of UNITE	9

1.2.1	Aims of the pilot accounts	9

1.2.2	Core, supplementary and excluded data in the pilot accounts	9

1.2.3	The six UNITE pilot account cost categories	10

1.2.4	The transport modes covered in the pilot accounts	12

1.3	Results presentation and guidelines for interpretation	13

1.4	The structure of this annex report	14

2	Description of input data	15

2.1	Overview on the Italian transport sector and basic input data used for all cost and revenue categories	15

2.1.1	Road transport	17

2.1.2	Rail transport	17

2.1.3	Public transport – tram, metro, bus	20

2.1.4	Aviation	21

2.1.5	Waterborne transport: inland waterway navigation and maritime shipping	23

2.2	Input data per cost/revenue category	23

2.2.1	Infrastructure costs	23

2.2.2	Supplier operating costs	27

2.2.3	Delay costs due to congestion	27

2.2.3.1	The study by Amici della Terra	28

2.2.3.2	The study by ACI – ANFIA	30

2.2.4	Accident costs	31

2.2.5	Environmental costs	35

2.2.5.1	General data for airborne emissions	35

2.2.5.2	General data for the calculation of costs due to air pollution	36

2.2.5.3	Basic data for airborne emissions in Italy	39

2.2.5.4	Global warming	45

2.2.5.5	Noise	46

2.2.5.6	Nature and landscape	47

2.2.5.7	Summary	47

2.2.6	Taxes, charges, subsidies	48

3	Methodological issues	52

3.1	Methodology for estimating infrastructure costs	52

3.2	Methodology for estimating supplier operating costs	55

3.3	Methodology for estimating delay costs due to congestion	56

3.4	Methodology for estimating accident costs	56

3.5	Methodology for estimating environmental costs	62

3.5.1	Air pollution	62

3.5.1.1	General approach	62

3.5.1.2	Monetary values	67

3.5.1.3	Discussion of uncertainties	68

3.5.2	Global warming	71

3.5.3	Noise	72

3.5.4	Methodology for 1996 and for the forecast to 2005	73

3.6	Methodology for estimating taxes, charges and subsidies	74

3.6.1	General issues	74

3.6.2	Methodological issues per mode	75

3.7	Forecasting methodology	78

3.7.1	MURE National - Transport scenario description	79

3.7.2	Logistic Substitution Models	81

4	Results	83

4.1	Infrastructure costs	83

4.1.1	Road transport.	83

4.1.2	Rail transport	84

4.1.3	Public Transport (tram and metro)	84

4.1.4	Inland waterways	85

4.1.5	Aviation	85

4.1.6	Infrastructure running costs	86

4.2	Supplier operating costs	87

4.2.1	National and non national rail sector	87

4.2.2	Public Transport	89

4.3	Delay costs due to congestion	90

4.4	Accident costs	90

4.4.1	Results for 1998- total costs by category and main cost bearer	90

4.4.2	Allocation of total costs to modes and types of infrastructure	91

4.4.3	Average costs in 1998	92

4.4.4	Results for 1996	93

4.4.5	Results for 2005	95

4.5	Environmental Costs	96

4.4.6	Results for 1998	96

4.4.7	Results for 1996 and 2005	100

4.6	Taxes, charges, subsidies	102

4.6.1	Road transport	102

4.6.2	Rail transport – FS	103

4.6.3	Non-national railways	105

4.6.4	Public transport	105

4.6.5	Aviation	106

4.6.6	Maritime and inland waterways transport	106

5	Summary of results for Italy	108

5.1	Road transport	108

5.2	Rail transport – national rail carrier Ferrovie dello Stato S.p.A. – FS and other Italian  railway companies	118

5.3	Public transport: tram, metro and trolley bus	124

5.4	Aviation	130

5.5	Inland waterways transport	136

6	Conclusions	142

6.1	Relevance of the accounts for the transport policy	143

6.2	Open questions and future improvements	143

4	References	145

5	Glossary	150

6	Abbreviations	153

7	Abbreviations used in data tables	154

List of Tables

Table 1 The modes, network differentiation, transport means and  user breakdown in the Italian pilot accounts	13

Table 2 Basic indicators for Italy 1996 and 1998	15

Table 3 Basic transport related indicators for Italy 1998 per mode	16

Table 4 Road mileage driven in Italy – in million v-km	18

Table 5 Rail Network Extension - Italy (km)	19

Table 6 Train-km of Italian National Rail (FS) 1996, 1998	19

Table 7 Share of urban and interurban public transport in Italy 1996, 1998	20

Table 8 Means of public transport per cost category  and modal transport account for Italy	21

Table 9 Italy’s Air Traffic Performance	22

Table 10 Sources and quality of input data for estimating infrastructure costs by transport mode	26

Table 11 Congestion costs estimation - Basic input data (Lombard, Molocchi 1998)	29

Table 12 Time lost in 1995 in the Italian cities with more than 500 000 inhabitants due to congestion	29

Table 13 Time lost in 1995 in urban areas in Italy	30

Table 14 External road congestion costs in 1995	30

Table 15 Road accident cases by network type in Italy 1998 1	33

Table 16 Total number of casualties in Italy 1998	33

Table 17 Material damages in Italy road transport 1998	34

Table 18 Sources and quality of input data for estimating accident costs by transport mode	34

Table 19 Sources and quality of input data for estimating accident costs by cost category	35

Table 20 Emissions caused by fuel production processes	36

Table 21 Environmental data in the EcoSense database	37

Table 22 Energy consumption 1996 and 1998 (in tonnes) - Road transport -	40

Table 23 Energy consumption 1996 and 1998 (in tonnes and kWh) - Public transport -	41

Table 24 Energy consumption 1996 and 1998 (in tonnes and kWh) - Rail transport -	41

Table 25 Energy consumption 1996 and 1998 (in tonnes ) - Air transport -	42

Table 26 Direct transport emissions in Italy 1998	43

Table 27 Direct transport emissions in Italy 1996	45

Table 28 Italian population exposed to road and railway noise 1997	46

Table 29 Source and quality of input data for estimating environmental costs	47

Table 30 Sources and quality of input data for estimating taxes, charges and subsidies by cost category	48

Table 31 Forecast capital costs for transport mode in Italy 2005 – € million 1998 –	54

Table 32 Average costs for medical treatment in Italy 1998 - in € per casualty -	58

Table 33 Average costs for the calculation of production losses - in € per casualty -	59

Table 34 Administrative costs: unit costs per accident 1998  – € million 1998 –	60

Table 35 Average unit costs per material damage of road accidents 1998 - € per v-km -	60

Table 36 Health and environmental effects included in the analysis of air pollution costs	65

Table 37 Quantification of human health impacts due to air pollution1)	68

Table 38 Monetary values for health impacts (factor costs, European average) - € million at 1998 prices -	69

Table 39 Valuation of health effects (factor costs, European average) from noise exposure - € million at 1998 prices -	73

Table 40 Capital value, capital costs and running costs for road transport in Italy 1996-1998 – € million 1998 –	83

Table 41 Capital value, capital costs and running costs for rail, tram-metro, inland waterways and air transport in Italy 1996 and 1998 - € million at 1998 prices -	84

Table 42 Infrastructure running costs in Italy 1998 - € million at 1998 prices -	87

Table 43 Supplier operating costs for Italian rail sector 1996 - € million at 1998 prices -	88

Table 44 Supplier operating cost for Italian rail sector 1998  - € million at 1998 prices -	88

Table 45 Supplier Operating Costs- Public Transport 1996  - € million at 1998 prices -	89

Table 46 Supplier Operating Costs for Public Transport 1998  - € million at 1998 prices -	89

Table 47 Supplier Operating Costs for Public Transport 2005 - € million at 1998 prices -	90

Table 48 Total internal and external accident costs in Italy – 1998 - € million at 1998 prices -	91

Table 49 Total external costs in Italy by vehicle type – road transport - € million at 1998 prices -	92

Table 50 Total external costs in other transport modes - € million at 1998 prices -	92

Table 51 Average accident costs in Italy 1998 – road transport - €/1000 v-km -	93

Table 52 Average accident costs in Italy 1998 – other transport modes - €/1000 v-km -	93

Table 53 Total accident costs in 1996 - € million at 1998 prices -	94

Table 54 Average accident costs in Italy 1996 – road transport - €/1000 v-km -	94

Table 55 Average accident costs in Italy 1996 – other transport modes  - €/1000 v-km -	95

Table 56 Total accident costs in Italy 2005 - € million at 1998 prices -	95

Table 57 Average accident costs in Italy 2005 – road transport - €/1000 v-km -	96

Table 58 Average accident costs in Italy 2005 – other transport modes - €/1000 v-km -	96

Table 59 Environmental costs for Italy 1998 - € million at 1998 prices -	97

Table 60 Environmental costs road transport Italy 1998 - Disaggregation by vehicle type - € million at 1998 prices -	98

Table 61 Environmental costs road transport Italy 1998 – Disaggregation by vehicle and road type - € million at 1998 prices -	99

Table 62 Average environmental costs for Italy 1998  - €/1000 v-km -	100

Table 63 Environmental costs for Italy 1996  - € million at 1998 prices -	101

Table 64 Environmental Costs for Italy 2005 - € million at 1998 prices -	102

Table 65 Road Transport Revenues in Italy 1996, 1998 and 2005 - € million at 1998 prices -	103

Table 66 Revenues from taxes and charges in rail transport – FS Spa - € million at 1998 prices -	104

Table 67 Subsidies for FS 1996 and 1998 - € million at 1998 prices -	104

Table 68 Tariff revenues and subsidies of non-national railways - € million at 1998 prices -	105

Table 69 Tariff revenues and subsidies of public transport - € million at 1998 prices -	105

Table 70 Taxes and charges of air transport in Italy 1996 and 1998 - € million at 1998 prices -	106

Table 71 Taxes of maritime transport sector in Italy 1994 - € million at 1998 prices -	107

Table 72 Italian road account for 1996, 1998 and 2005  - € million at 1998 prices -	109

Table 73 Average variable costs of road transport per vehicle km: Italy - €/km at 1998 prices -	113

Table 74 Total costs of road transport: Italy - € million at 1998 prices -	114

Table 75 Total costs of road transport: Italy - € million at 1998 prices -	115

Table 76 Total costs of road transport: Italy - € million at 1998 prices -	116

Table 77 Total costs of road transport: Italy - € million at 1998 prices -	117

Table 78 Italian rail account 1996, 1998 and 2005  – € million at 1998 prices –	119

Table 79 Average variable costs of rail transport per vehicle km National and non national rail - €/train km at 1998 prices -	122

Table 80 Total costs of rail transport (National and non national rail) - € million at 1998 prices -	123

Table 81 Italian account for metro, tram, trolley bus 1996, 1998 and 2005  - € million at 1998 prices -	125

Table 82 Average variable costs of metro, tram, trolley bus per vehicle km: Italy - €/km at 1998 prices –	128

Table 83 Total costs of metro, tram, trolley bus: Italy - € million at 1998 prices -	129

Table 84 Italian air transport account for 1996, 1998 and 2005  - € million at 1998 prices -	131

Table 85 Average variable costs of Aviation per LTO cycle: Italy – €/LTO cycle at 1998 prices –	134

Table 86 Total costs of Aviation: Italy - € million at 1998 prices -	135

Table 87 Italian inland waterways account for 1996, 1998 and 2005  - € million at 1998 prices -	137

Table 88 Average variable costs of inland waterways and maritime shipping per vehicle km - €/km at 1998 prices -	139

Table 89 Total costs of inland waterways and maritime shipping: Italy - € million at 1998 prices -	140

# Introduction

## Study context and objectives of this annex report

This annex report contains the full version of the Italian pilot account developed within the UNITE project. It serves as background report for the results presented in the core body of Deliverable 12 and gives more detailed descriptions on the methodology used and the input data and their reliability and quality. However, the general and detailed discussion of the accounts approach was presented in Link et al. (2000b) and will be summarised only in this document. This annex report discusses methodologies only in so far as they are necessary background information for understanding the results and describes rather the application of methodology to the Italian case. Furthermore, in addition to the core accounts for 1998 this annex report also presents the results for 1996 and a forecast for 2005. This annex report was produced by ISIS, with contributions from DIW and IER. DIW contributed to infrastructure costs calculations, while IER contributed to environmental costs calculation.

In order to put this annex report into the context of the UNITE project a summary of the aims and research areas of UNITE is given here. The UNITE project endeavours to provide accurate information about the costs, benefits and revenues of all transport modes including the underlying economic, financial, environmental and social factors. To achieve this goal, three main areas of research are carried out, known as “transport accounts”, “marginal costs” and “integration of approaches”. This annex report belongs to the research area “transport accounts”. For a better understanding of the results presented here it has to be borne in mind that the UNITE project distinguishes between ideal accounts on the one hand and the pilot accounts on the other hand. The ideal accounts reflect the perfect situation with the utmost disaggregation, showing factors such as the time and location and duration of individual trips, all the relevant economic data as well as the individuals response to possible policy or infrastructure changes. The pilot accounts are the actual, feasible accounts given the available data for the 18 countries that UNITE covers. They can be used to assess the costs and revenues of transport per transport mode. The costs are reported and documented at the current level of transport demand for the reference years 1996, 1998 and for the forecast year 2005. Reported transport costs are allocated to user groups, where possible without arbitrary allocation methods.

## The accounts approach of UNITE

### Aims of the pilot accounts

The pilot accounts attempt to show the general relationship between the costs of transport and the revenues from transport pricing and charging in the country studied. The aims and role of the pilot accounts are discussed in detail in “The Accounts Approach” Link et al. (2000 b). It should be stressed that the accounts are aimed at providing the methodological and the empirical basis for in-depth policy analysis and monitoring rather than serving as a guide for immediate policy actions such as setting higher/lower prices and charges or opening up/shutting-down transport services/links in order to achieve cost coverage. The pilot Accounts are defined as follows:

The pilot accounts compare social costs and charges on a national level in order to monitor the development of costs, the structure and level of prices, taxes and subsidies. Accounts can therefore be seen as strategic and monitoring instruments at the same time. They have to consider the country-specific situation and the institutional frameworks.

The pilot accounts show the level of costs and charges as they were in 1998 and 1996 and provide a workable methodological framework to enable regular updating of transport accounts. Furthermore, an extrapolation for 2005 is given. The choices of additional accounting years (1996 and 2005) were motivated by the need to show a comparison between years and to give a good indication of trends in transport for the near future. Also, the inclusion of 1996 provides a double check on any major statistical anomalies that may occur in one year, for example very high infrastructure cost due to tunnelling operations or higher than average accident costs because of major accidents occurring in 1998. Note, however, that the base year of the pilot accounts is 1998. Both the results for 1996 and 2005 are derived from this base year.

### Core, supplementary and excluded data in the pilot accounts

The pilot accounts have been divided into the classes “core data” and “supplementary data”. Core data are those necessary to carry out a full basic review of the country accounts. Core data include the following categories: infrastructure costs, the external costs of transport accidents, the environmental categories air pollution, noise and global warming and supplier operating costs. Transport revenues and taxes are also documented here. Supplementary data fall into two categories. Firstly, for several cost categories being evaluated there is no standard methodology for the valuation of effects. An example of this is the valuation of loss of biodiversity due to transport infrastructure. Even though a valuation method has been developed for the UNITE Pilot Accounts, we feel that the level of uncertainty (due to lack of comparative studies) is high enough to warrant the information to be classified outside of the core data where efficient and well tried valuation methods have been utilised. Secondly, some costs, which can be estimated and valuated, are borne by the transport users themselves (for example delay costs). These costs and the methods used to valuate them present valuable further information to the reader, but are not part of the overall costs of transport as defined by UNITE (see Sansom et al, 2000). The second set of data includes: congestion costs, the internal part of accident costs including the risk value and the environmental costs risk due to the provision of nuclear power and the costs associated with nature and landscape, soil and water pollution. Subsidies also fall within the category supplementary data.

### The six UNITE pilot account cost categories

Data for the pilot accounts are collected within six cost and revenue categories that are described in Link et al. (2000b) and are summarised in the following section.

Infrastructure costs

For the pilot accounts, data for the assessment of infrastructure costs are structured to show the capital costs of transport infrastructure (including new investments and the replacement of assets) and the running costs of transport infrastructure (maintenance, operation and administration) for all modes of transport studied. As far as possible with the current methodological knowledge, infrastructure costs are allocated to user groups and types of transport. Where it is possible to quantify the share of joint costs they are separated out and are not allocated.

Supplier operating costs

All monetary costs incurred by transport operators for the provision of transport services are documented in the category supplier operating costs. Ideally, the data is structured to show what costs are incurred separately for vehicles, for personnel and for administration. However, this depends on data availability and differs from country to country. Since collecting and supplementing this data for all modes is extremely time consuming, the UNITE project focuses on estimating supplier operating costs only for those modes where significant state intervention and subsidisation is present. The main emphasis in this category is thus on rail transport and other public transport (tram, metro, bus). Whether other modes also have to be covered depends on the degree of state intervention in the respective countries. The corresponding revenues from the users of transport are included when supplier operating costs are estimated. The difference between such costs and revenues is the net public sector contribution (economic subsidy).

Delay costs due to congestion

In the European Commission’s White Paper “Fair payment for infrastructure use” (1998), costs caused by transport delays, accidents and environmental effects of transport are estimated to be the three major causes of external transport costs. In the category congestion costs, the costs of delay and delay-caused additional operating costs are estimated. The estimation of delay costs as defined here is carried out for all transport modes, provided data is available. This data is classified as supplementary data because the bulk of these costs are borne by transport users as a whole. (i.e. they are internal to the transport system).

Accident costs

The loss of lives and the reduction of health and prosperity through transport accidents are of major concern to all countries and to the European Commission. In this section of the accounts, the health related accident costs are calculated by assessing the loss of production, the risk value and the medical and non-medical rehabilitation of accident victims. Where the available data basis allows, the damage to property and the administrative costs of accidents are also considered. The external part of accident costs (defined in this report as accident costs imposed by transport users on the whole society) is included in the core section of the accounts. The internal part of accident costs however, is considered to be that part of accident costs imposed by one user on other users of the transport system and is therefore treated as supplementary costs.

Environmental costs

A wide range of transport related environmental impacts and effects, presently being hotly debated in all countries, is considered in this section of the accounts. Included in this cost category are: air pollution, global warming, noise, changes to nature and landscape, soil and water pollution and nuclear risks. The valuation of these environmental effects is carried out for all transport modes, provided adequate data is available.

Taxes, charges and subsidies

In this section, the level of charging and taxation for the transport sector is documented for each mode of transport. The comparison between taxes levied and the costs of infrastructure provision and use accrued per mode is central to this debate and holds a high level of political significance. Environmental taxes that apply to transportation are separately considered in this section. Taxes on transport activities such as VAT that do not differ from the standard rate of indirect taxes applied to the other sectors - and therefore do not influence the relative costs of transport - are excluded from this study.

A further part in this area is reporting on subsidies. The need to maintain free and undistorted competition is recognised as being one of the basic principles upon which the EU is built. State aid or subsidies are considered to distort free competition and eventually cause inefficiency. Subsidies to the transport sector provided by the member states are not exempted from the general provisions on state aid set out in the Amsterdam Treaty. There are, however, special provisions set out in the treaty in order to promote a common transport policy for the transport sectors of the member states (Treaty establishing the European Community: Articles 70 – 80). The subsidies of the transport sector are considered in this section. It should be noted that a complete reporting on subsidies would require an extremely time-consuming analyses of public budget expenditures at all administrative levels. Furthermore, the subsidies reported in the pilot accounts refer mainly to direct subsidies (e. g. monetary payments from the state to economic subjects) at the state level but generally not at the municipal level. Indirect subsidies (e. g. tax reductions and tax exemptions that cause lower revenues of state budgets) are quantified where possible.

### The transport modes covered in the pilot accounts

The modes covered in UNITE are road, rail, other public transport (tram, metro, trolley bus), aviation, inland waterway navigation and maritime shipping. The level of disaggregation into types of networks and nodes, means of transport and user groups depends on data availability and relevance per country. Table 1 summarises this disaggregation for the Italian pilot account. Section 2.1 provides in addition some indicators per mode in order to show the importance and relevance of each mode in the Italian transport system.

Table 1
The modes, network differentiation, transport means and 
user breakdown in the Italian pilot accounts

| Transport modes | Network and institutional differentiation | Means and user breakdown |
| --- | --- | --- |
| Road | Motorways Other non urban roads1) Urban roads | Motorcycles, mopeds Passenger cars Buses Light goods vehicles (LGV) Heavy goods vehicles (HGV) |
| Rail | National carrier Non national carrier | Passenger transport Freight transport |
| Other public transport | - | Metro Tram and trolley bus |
| Aviation | - | Passenger transport Cargo |
| Inland waterway shipping | - | - |
| Maritime shipping | - | - |
| 1) Includes state, provincial, municipal non-urban roads. Source: ISIS. |  |  |

## Results presentation and guidelines for interpretation

The goal of the data collection and estimation of costs and revenues in each category was a level of disaggregation that shows the pertinent costs and charges of the relevant transport mode. From the available, but very heterogeneous input data and results, a structure for reporting transport accounts has been developed. All results are documented separately for each cost category and are summarised in modal accounts covering all cost and revenue categories. Additionally, a set of macroeconomic indicators and basic transport data for Italy are shown in Table 2 and 3. These data form the common background to all cost evaluations, in order to ensure consistency between cost categories.

The categories studied present a comprehensive estimation of transport costs and revenues. They are however, not an estimation of total transport costs. Each cost category could include data in further areas and a definite border had to be drawn around the data to be collected for this project. For example, the estimation of environmental costs does not include the environmental costs incurred during the manufacturing of vehicles, even though these costs could be estimated. These costs would be included in an ideal account, but lie outside the scope of the pilot accounts. Further transport costs categories such as vibration – to be attributed to environmental costs - are not evaluated because no acceptable valuation method has been developed.

It should be noted that due to the separation into core and supplementary data with different levels of uncertainty and with different types (costs borne by transport users themselves versus external costs) care is needed when comparing costs and revenues.

## The structure of this annex report

This annex report contains four main parts. Chapter 2 briefly explains the structure of the Italian transport sector mode by mode in order to provide some background information for the interpretation of the pilot accounts. In addition, input data for each cost category are described here. Chapter 3 provides the methodological framework, focusing on departures from the overall UNITE methodology (see Link et al. 2000). The use of the recommended methodology is conditioned to the actual structure of the Italian transport system and to data availability. Results are presented and discussed in Chapter 4. Chapter 2, 3 and 4 are organised along the categories infrastructure costs, supplier operating costs, congestion costs, accident costs, environmental costs and taxes, charges and subsidies. Chapter 5 presents the summary tables by mode and cost category on the Italian pilot accounts and chapter 6 draws conclusions.

# Description of input data

## Overview on the Italian transport sector and basic input data used for all cost and revenue categories

This section aims at providing some basic information on the main features of the Italian transport sector, its organisational structure and the relative importance of transport modes as far as necessary for understanding and interpreting the pilot accounts. Table 2 therefore presents some basic Italian social and economic indicators, while Table 3 shows the main structural and performance indicators for each transport mode considered within the accounts.

Table 2
Basic indicators for Italy 1996 and 1998

|  | Unit | 1996 | 1998 |
| --- | --- | --- | --- |
| Land area | Km2 | 301 000 | 301 000 |
| Population | 1 000 | 57 460.98 | 57 612.62 |
| Population density | inhabitants/Km2 | 190.90 | 191.40 |
| Population employed | 1 000 | 20 125 | 20 435 |
| Employment Rate | % | 41.7 | 42.0 |
| GDP (at 1995 prices)1) | € billion | 933.142 | 969.130 |
| GDP per capita | € | 17 117 | 18 633 |
| GDP growth rate (change to previous year) | % (in prices of 1995) | 1.09 | 1.79 |
| Consumer price index 2) | 1995 = 100 | 104.0 | 108.1 |
| 1) At market prices. 2) Consumer price index referred to the entire population. Source: ISTAT |  |  |  |

Table 3
Basic transport related indicators for Italy 1998 per mode

| Indicator | Unit | Road | Rail | Public transport | Aviation | Inland waterway navigation | Maritime shipping | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Transport performance |  |  |  |  |  |  |  |  |
| Passengers carried | Million | : 1) | 577.0 | 4 4322) | 77 | 101 | 3 | : |
|  | % | : | : | : | : | : | : | : |
| Passenger-km | Billion P-km | 770.643) | 40.77 | 32.092) | 22.7 | 0.45 | : | 806 |
|  | % | 87.8 | 5.1 | 4.0 | 2.8 | 0.1 | : | 100 |
| Goods transported4) | Million t | 1230.3 | 90.1 | - | 0.695) | 1.05 | 266.4 | 1 588.5 |
|  | % | 77.4 | 5.7 | - | 0.0 | 0.1 | 16.8 | 100 |
| Tonne-km4) | Billion tkm | 191.48 | 25.44 | - | : | 0.13 | : | : |
|  | % | : | : | - | : | : | : | : |
| Network length | 1000 km | 836.07 | 19.47 | 0.56) | - | 1.48 | - | - |
| Employees7) | 1000 | 406.7 | 148.7 | -8) | 22.8 | 8.4 | 16.4 | 603.5 |
| Gross investments9) | € million | 6 25810) | 2 276 | 138 | 335 | 13 | 250 | 9 270 |
|  | % | 67,5 | 24,6 | 1,5 | 0,1 | 2,7 | 3,6 | 100 |
| Gross capital stock11) | € million | 7 25012) | 2 549 | 3413) | 427 | 32 | : | 10 292 |
|  | % | 70.4 | 24.8 | 0.3 | 4.1 | 0.3 | : | 100 |
| Accidents |  |  |  |  |  |  |  |  |
| Number of injuries14) | Casualties | 428 863 | 238 | 3 084 | 31 | 8 | : | 432 224 |
| Number of fatalities | Casualties | 7 333 | 83 | 37 | 20 | 4 | : | 7 477 |
| Environment |  |  |  |  |  |  |  |  |
| Direct transport emissions |  |  |  |  |  |  |  |  |
| CO2 | Million t | 100.2 | 2.8 | 0.9 | 10.8 | 0.1 | : | 114.8 |
| PM2.5 | 1 000 t (exhaust) | : | : | : | : | : | : | : |
| PM10 | 1000 t (non-exhaust) | 40.42 | 1.14 | 0.63 | : | : | : | 42.19 |
| NOx | 1 000 t | 725.02 | 14.35 | 8.55 | 4.76 | 0.91 | : | 753.59 |
| SO2 | 1 000 t | 29.17 | 11.55 | 0.86 | 0.39 | 0.09 | : | 42.06 |
| NMVOC | 1 000 t | 968.79 | 2.7 | 1.65 | 1.22 | 0.1 | : | 974.46 |
| 1) Includes passenger cars, powered two-wheelers (ISIS estimation); no estimations on passengers carried by coaches was available. - 2) Includes metro, tram, trolley and urban buses. - 3) Includes passenger cars, powered two-wheelers and coaches. – 4) Excluding goods transported in pipelines. – 5) Includes mail and goods. – 6) Including metro, tram. - 7) Excluding employees in other transport related auxiliary services; refer to 1996. – 8) Included in road account. - 9) At current prices. – 10) Includes gross investments for all municipal roads. – 11) At 1998 prices. – 12) Including land value. – 13) Excluding trolleys. - 14) Slight and severe injuries. - Source: ACI, ANIA, ANPA, ISTAT, Ministry of Transport, ISIS |  |  |  |  |  |  |  |  |

### Road transport

Road transport has a prominent role in the transport sector: in 1998, 65% of freight transport and 92% of passenger transport was made by road. The road network in operation was composed in 1998 by 6 478 km of motorways, 46 009 km of state roads, 114 909 km of provincial roads. The length of municipal roads network in 1999 was estimated to be 668 669 km by the Ministry of Transport.

The municipal, provincial, state road network is publicly owned and the access is free of charge. An exception is made for some urban areas, where the access to some road (mainly in city centres) might be subject to the payment of a fee during selected hours of the day. Concerning motorways and tunnels, only a part of the network (894 km) is publicly maintained through the Ente Nazionale per le Strade – ANAS, a company of the Ministry of Public Works, and therefore free of charge; the rest of the motorways and tunnel network is given under a concessionary regime to private companies, that impose an access charge (distance-based charge).

The concessionaries are in charge of infrastructure construction, road maintenance, toll collection and various transport-related services provision. Among these companies, the outstanding group is Autostrade Group, composed by the Group holding company, Autostrade S.p.A., and six motorway and tunnel subsidiaries, operating 3 120 km of motorways (on a total of 5 584 km of network privately operated). Table 4 gives an overview of mileages driven on the Italian road network in 1996 and 1998. Mileage forecasts to 2005 were not available.

### Rail transport

The Italian rail market is currently characterised by one dominating carrier, the State Railway Company (Ferrovie dello Stato S.p.A.- FS), whereas the Italian Railway infrastructure system consists of both FS and other 26 private railway companies (mainly operating in Southern Italy) which manage a portion (3 448 km 1998) of the Italian railway network (16 107,9 km) by means of a license issued by Italian State  (Ferrovie in concessione – licensed railways).

Table 4
Road mileage driven in Italy
– in million v-km

|  | Total road network | Motorways | Rural roads | Urban roads |
| --- | --- | --- | --- | --- |
| 1996 |  |  |  |  |
| Total | 477 857 | 100 623 | 231 696 | 145 538 |
| Mopeds, motorcycles | 43 186 | 874 | 13 830 | 28 482 |
| Passenger cars | 360 536 | 74 699 | 182 727 | 103 110 |
| Buses | 3 535 | 1 435 | 1 071 | 1 029 |
| Light goods vehicles | 32 759 | 6 552 | 18 018 | 8 190 |
| Heavy goods vehicles | 37 841 | 17 062 | 16 051 | 4 727 |
| Special vehicles | : | : | : | : |
| Agricultural vehicles | : | : | : | : |
| 1998 |  |  |  |  |
| Total | 493 207 | 107 441 | 241 971 | 143 796 |
| Mopeds, motorcycles | 46 876 | 940 | 15 003 | 30 933 |
| Passenger cars | 369 377 | 80 636 | 190 576 | 98 164 |
| Buses | 3 677 | 1 482 | 1 109 | 1 087 |
| Light goods vehicles | 37 183 | 7 437 | 20 450 | 9 296 |
| Heavy goods vehicles | 36 094 | 16 946 | 14 833 | 4 315 |
| Special vehicles | : | : | : | : |
| Agricultural vehicles | : | : | : | : |
| Source: ISIS calculations on ANPA, ACI data |  |  |  |  |

Since the years following the World War II until 1985, Italian railways were owned and managed by the Ministry of transport. In 1985 the company “Ente per le Ferrovie dello Stato” (a State entity charged with management tasks for rail network) was created, in order to improve both passengers and freight transport quality. In 1992, FS was privatised becoming a limited corporation whose shares could be sold to private stakeholders. So far, the Italian State still owns the whole of FS shares, thus hindering the real market competition and the improvement of rail network services. In 2001 FS has been restructured and is currently split into six main public limited companies in their own right (RFI – Rete Ferroviaria Italiana, responsible for equipment and trains circulation and safety whilst managing the whole of the rail network; Trenitalia, manages the sector of passengers and freight transport, Tav, devoted to high-speed lines, Italferr, an infrastructure engineering company, Metropolis, managing the FS real estate and Grandi Stazioni, devoted to the management of great railway stations).

In 1998 the network extension for FS railways was 16 080 km (82% of the total national network extension) while for the licensed railways this length amounted to 3 448 km (18%), of which 2 139 spreads through Southern Italy - mainly in Sicily, Basilicata and Sardinia (see Table 5).

Italian railway system suffers from several problems for a long time now, although FS implemented a concrete renewal and development policy for the whole of the rail infrastructure, services and equipment. First, the rail network has widely developed only in Northern Italy where most of electrified lines are located, whereas Southern Italy, where road infrastructure development has been favoured in the past decades, still possesses a fair share of non-electrified lines (33% of the total Southern rail network). Second, the equipment age is rather high (an average of 24.8 years for passenger coaches and 28.4 years for freight ones), albeit FS is endeavouring to renew its rail fleet in order to improve trains speed, comfort and safety. Third, in 1998 rail traffic accounted for a mere 49 846 million of passengers/km, whereas road traffic has reached as much as 818 140 million of passengers/km (within Italian borders). In 1999, 93% of total passengers traffic was carried out by road, a clear sign of a further shift from train to private transport and, to some extent, aviation (all data are drawn from Ministry of Transport, 2000).

Table 5
Rail Network Extension - Italy (km)

|  | 1996 | 1998 |
| --- | --- | --- |
| Total Length | 16 013.6 | 16 079.9 |
| Electrified lines (% on total length) | 10 318.5 (64.4%) | 10 487.7 (65.3%) |
| Non-electrified lines (% on total length) | 5 695.1 (35.6%) | 5 592.2 (34.7%) |
| Source: Ministry of Transport |  |  |

Table 6
Train-km of Italian National Rail (FS) 1996, 1998

|  | Unit | 1996 | 1998 |
| --- | --- | --- | --- |
| Train-km | million |  | 340.7 |
| Passenger transport (p-km) |  |  |  |
| Regional passenger transport | million | 19 340 | 17 994 |
| Long distance passenger transport | million | 25 442 | 23 398 |
| Freight transport (tkm) | million | 23 994 | 25 366 |
| Source: Ferrovie dello Stato |  |  |  |

### Public transport – tram, metro, bus

The Italian public transport system underwent an institutional modification that brought in 1997 to conferring all competences and duties concerning public transport to regions and local entities. From an organisational point of view, the most of the public transport is operated by bus. Rail borne transport in urban areas is operated in two cities by trolley bus (Milan, Genoa), while companies that run the passengers transport by tram are based in Turin, Milan, Rome and Naples, whereas in Genoa and Trieste there are tramway facilities. This service is mainly urban, and only Milan tram transport is operated out of urban areas. Underground transport service was present in 1998 in 4 cities (Milan, Rome, Genoa and Naples). The urban/interurban split is shown in table 7. Rail borne public transport account considers urban and extra-urban railways and the metro/subway service, excluding trolley buses, as they exist only in Genoa and Milan.

Table 7
Share of urban and interurban public transport in Italy 1996, 1998

| Mode | Vehicle-km or coach-km |  | Passenger km |  | Seats-km |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1996 | 1998 | 1996 | 1998 | 1996 | 1998 |
|  | Bus |  |  |  |  |  |
| Urban | 37,6% | 35,9% | 37.70% | 37.55% | 46,8% | 46,0% |
| Interurban | 62,4% | 64,1% | 62.30% | 62.45% | 53,2% | 54,0% |
| Total | 100% | 100% | 100% | 100% | 100% | 100% |
|  | Trolley |  |  |  |  |  |
| Urban | 94,8% | 94,3% | 96.49% | 95.45% | 95,9% | 95,7% |
| Interurban | 5,2% | 5,7% | 3.51% | 4.55% | 4,1% | 4,3% |
| Total | 100% | 100% | 100% | 100% | 100% | 100% |
|  | Metro |  |  |  |  |  |
| Urban | 100% | 100% | 100% | 100% | 100% | 100% |
| Total | 100% | 100% | 100% | 100% | 100% | 100% |
| Source: Ministry of Transport |  |  |  |  |  |  |

It should be noted that providing a clear and unambiguous definition of Public Transport and its current scope of operation proved far from straightforward. Ideally, the categories local/urban buses, tramways and trolley buses can be differentiated under this mode, but in some cases this separation is not feasible. The infrastructure costs of local/urban buses, for example, are included in the road infrastructure costs. Against this background, attention should be paid when the results between the different cost categories are interpreted for the urban public transport mode. A summary table of relevant public transport modes and their position within the accounts is given in table 9.

Table 8
Means of public transport per cost category 
and modal transport account for Italy

|  | Modal transport account |  |  |
| --- | --- | --- | --- |
| UNITE categories | Road account | Rail (Non-FS) account | Public Transport account |
| Infrastructure Costs | Buses (urban and coaches) | Non national rail carrier | Tram, metro |
| Supplier Operating Costs | - | Non national rail carrier | Buses (urban and extra-urban), tram, metro, trolley buses |
| Congestion Costs | - | - | - |
| Accident Costs | Buses (urban and coaches) | Non national rail carrier | - |
| Environmental Costs | Buses (coaches) |  | Urban buses, tram, trolley bus and metro |
| Taxes, Charges and Subsidies | - | - | Buses (urban and extra-urban), tram, metro, trolley buses |
| Source: ISIS |  |  |  |

### Aviation

In 1997, 102 Italian airports held passengers traffic, of which 80 are State-owned or owned by local administrations while 22 are private ones. In 1997, 41 airports have accounted a traffic over 1 000 passengers per day and 57 were also used by private planes and air-clubs, whereas 4 were shut down. The two hubs of Rome (Fiumicino) and Milan (Malpensa) account for more than 60% of total passengers traffic and 74% of cargo traffic. In late 1998, Alitalia (the biggest Italian air carrier) shifted most of the international and intercontinental flights from Fiumicino to Malpensa, causing a fair decrease in traffic for the former, although in early 2000 passengers increased by 10.7% and freight by 7.7%.

Italian airports can be ranked into the five categories shown in the table beneath:

|  | Intercontinental | Main Airports (over 1 million passengers) | Intermediate (from 1 million to 100,000 passengers) | Small (from 100.000 to 1.000 passengers) | General aviation or shut down | Total |
| --- | --- | --- | --- | --- | --- | --- |
| Number | 2 | 13 | 12 | 14 | 61 | 102 |

Source: Ministero dei Trasporti e della Navigazione (1998)

As a whole, Italian air passengers traffic followed a steadily increasing trend over time, from   58 566.6 thousand of passengers in 1995 to 76 539.7 thousand in 1998, whilst freight transport decreased in 1998 by 0.8% (compared to 1997).

Table 9
Italy’s Air Traffic Performance

|  | 1996 | 1998 |
| --- | --- | --- |
| Passengers (number) | 64 761 093 | 76 539 750 |
| - Domestic flights | n.a. | 39 110 887 |
| - - International flights | n.a. | 37 428 863 |
| Passengers aircraft / 1 000 km | 7 871 | 8 8741) |
| Freight (1000 t) | 65 558** | 72 3492) |
| 1) Ministry of Transport estimation 2) Mail and goods Source: Ministry of Transport, |  |  |

Nearly all Italian airports are completely managed by private companies  – through 40-year time contracts (as stated by Law 24/12/1993 n° 537, art. 10, comma 13 and D.M. 12/11/1997 n° 521 – the airport area is property of the State though), whereas some of them are “partially” managed (only for passengers and freight infrastructures) through 10-year time contracts – flight infrastructures are both managed and owned by the State. Only three Italian airports (Lampedusa, Pantelleria and Roma Urbe) are completely managed and owned by the Italian State.

Entities to control and regulate air transport in Italy are:

- ENAC (Italian Civil Aviation Authority), which is meant to monitor and guarantee the correct functioning of air transport activities, the quality of services, the implementation of new infrastructure investments, the air market competition (slots allocation etc.);

- ENAV (Italian Agency for Flight Assistance), which provides technical information and support (air traffic control and weather forecast by means of the GNSS technology) to air carriers’ fleet flying over the Italian territory.

### Waterborne transport: inland waterway navigation and maritime shipping

The importance of inland waterways transport in Italy is very limited: in fact it accounted in 1998 for as little as 0.053% of the total tons-km and 0.051% of the total passenger-km against 19% of total tons-km and 0.4% of total passenger-km recorded by maritime transport (short sea shipping).

## Input data per cost/revenue category

### Infrastructure costs

Basic data to calculate infrastructure costs for every single mode of transport (both capital and running costs) are of two kinds:

- physical data, as the length of the network or the area this network spreads through;

- financial data, as the investments expenditure.

As far as Italian data are concerned, we have to highlight that:

- The same kind of both data is not available for all of the modes of transport: indeed, for harbour infrastructure only a 20-year time series for both physical and financial data was available; these data were not enough to fulfil in the requirements of the methodology chosen for infrastructure capital value calculation (perpetual inventory method, see par. 3.1). For airport infrastructure the 40-year time series were available only for financial data and  therefore physical data were estimated;

- Time series taken into account are, as a whole, drawn from the State balance sheets published on the Transport National Account (CNT - Conto Nazionale dei Trasporti)  starting from 1966 (year of first publication of CNT) ;

- As far as road infrastructure is concerned, a distinction amongst motorways (autostrade), State roads (strade statali), provincial roads (strade provinciali) and municipal/local (extra-urban) roads (comunali) has been proposed. An evaluation for road infrastructure costs, including land costs, was carried out. The disaggregation of data by network type does not include land costs.

The basic financial data collected for each mode of transport is capital costs expenditure for new investments and goods replacement.

Road

Infrastructure data on network extension is a result of summing up the corresponding data referred to:

- Motorways;

- Highways;

- Provincial (country) road;

- Local road/extraurban.

Basic data for estimations are drawn from official evaluations mainly published by the Ministry of Transport in the CNT.

It was not possible to estimate urban roads infrastructure costs because of lack of data on urban road network length.

Rail

Time series of the rail network operated by the Italian national rail carrier (FS – Ferrovie dello Stato) were provided by Ferrovie dello Stato itself and published in Ferrovie dello Stato (1996b) for the time series 1950/1994 and in CNT for the time series 1995/1999. Despite the difference in the sources, all data are homogeneous, as CNT stems data from FS since 1995; slight discrepancies between the two sources are due to the fact that CNT takes into account also narrow gauge railway lines – that no longer exist since 1986. The account includes railway lines licensed to private companies starting from 1966.

Tramways and metro lines

Data on the whole network extension is a result of summing up all data referred to:

- Urban tramways;

- Extra-urban light rails;

- Metro underground.

Trolley buses network extension is excluded since its weight on the total is negligible; moreover, CNT includes capital expenditures for it in tramways account.

Time series are drawn from a variety of sources; a prospect is given below.

| Tramways Extra-urban light rails Subway and metro rails | - CISPEL - Ministry of Transport - MCTC - CISPEL - Ministry of Transport - CISPEL - Ministry of Transport | Until 1985; note that data from 1970 to 1973 are estimated by CISPEL After 1985 1966/1968 1968/1984 from 1985 on until 1980 after 1980 |
| --- | --- | --- |

Inland waterways

Data on the infrastructure length is based upon the “operating length”, whose main source is UNII (Union of Italian Inland Waterways). Most of this infrastructure is located in Northern Italy. In order to obtain a time series long enough to fulfil in the model requirements, only data on passengers transport have been taken into account, since data for freight transport were not available. This might lead to an underestimation that we consider negligible, as Italian inland waterways transport is mainly devoted to passenger transport.

Seaports

Data on this mode were included in the yearly report of the Ministry of Transport on commercial maritime transport (Relazione annuale Ministero della Marina Mercantile) only since 1981. Since data on total area of Italian harbours were not available, the Italian pilot account includes data on the total length of harbours quays, published in CNT.

Air

Infrastructure costs are calculated only for airports with an yearly accounted traffic over 1 000 passengers because national transport statistics reports (CNT) point out for these airports both the aprons and the land area, i.e. the total area included in the bordered airport surface (defined in the CNT as “area di sedime”). These two parameters – necessary for the calculation of the infrastructure quality indicator – are gathered albeit in a discontinuous manner only since the 80’s. Therefore, for the missing data we assume that both aprons and land area were those referred to the first year of working to the airport – or the latest known year.

Table 10 provides an overview of data sources and quality by mode.

Table 10
Sources and quality of input data for estimating infrastructure costs by transport mode

|  | Input data | Level of disaggregation | Quality of data, level of uncertainty |
| --- | --- | --- | --- |
| Road | Both physical and financial data come from the CNT, ISTAT and ACI. | For evaluations of running costs only global data are available, whereas a further disaggregation in maintenance costs, infrastructure costs and administrative costs have been carried out based on the State balance sheet, published by CNT for 1996 and 1998. | All data on infrastructure costs are from official sources, except for the land price that has been estimated. |
| Rail | Both physical and financial data come from the Ministry of Transport, CNT and ISTAT. | Total investment expenditure for the stations was not appraised, because of lack of specific data. For running costs a disaggregation in current expenditures for infrastructure and other services and current expenditure for transport services is not available: it was estimated by means of the parameter “current expenditure per kilometre of network” | High quality both for physical and financial data on infrastructure. To evaluate the “land price” some hurdle occurred, so estimation was carried out. |
| Public Transport (tram, metro) | Data sources are CNT and CISPEL (physical data), CNT and ISTAT (financial data) | See road account | See road account |
| Air | Only airports with a traffic over 1000 passengers have been taken into account | A data disaggregation in investments for construction and investments for equipment was not feasible. | The values of missing years have been estimated, assuming that “parking area” and “land area” were both those referred to the first working year to the airport – or the latest known year. |
| Inland waterway | Data sources for physical data are UNII and CNT, financial data sources are CNT and ISTAT | A data disaggregation in inland waterways and inland waterway harbours was not feasible. | Only data referred to passengers transport were taken into account – from most recent reports. |
| Maritime Shipping | No data availability on land area occupied by inland waterway harbours; Data on total length of quays were taken from the Relazione annuale Ministero della Marina Mercantile, i.e. the yearly report published by the Ministry of Transport | A data disaggregation in investments for construction and investments for equipment was not feasible. | Data are from official sources, but time series are published since 1981. |

### Supplier operating costs

All monetary costs incurred by transport operators for the provision of transport services are documented in the category supplier operating costs. Ideally, data are structured in order to allow the distinction between costs incurred for vehicles, for personnel and for administration. However, data availability differs from country to country. Since collecting these data for all modes is extremely time consuming, the UNITE project focuses on estimating supplier operating costs only for those modes where significant State intervention and subsidisation is present. The estimation of supplier operating costs for the Italian pilot accounts will therefore be made for rail transport (national and non national rail) and for public transport (bus and trolley bus, tram, metro).

Economic data for Italy are usually available in an aggregate form. The main sources are:

  - Rail sector: estimation of data provided by FS (Ferrovie dello Stato 2000a, containing 1998 and 1999 balance sheets), both for national and non national rail;

  - Public Transport (bus and trolley bus, tram, metro): detailed data from 1991 to 1997 are provided by CISPEL – Federtrasporti (published in CISPEL, 1998). Federtrasporti is an association that gathers almost 100% of Italian public transport operators and 70% of extra-urban transport operators.

### Delay costs due to congestion

We deem that the current situation of basic data availability for urban areas  does not allow to calculate a national figure for urban road congestion according to UNITE standards (basic data like time lost in congested conditions, individual trips, average speed of vehicles are available only for a few municipalities), and we will therefore not include it in this account. On the other side it is necessary to describe the studies that attempt to fill this gap.

The studies are produced by Amici della Terra (P.L. Lombard, A. Molocchi, 1998 and 2000) in collaboration with Ferrovie dello Stato, and by ACI and ANFIA (R. Giordano, A. Frondaroli, R. Amirante, I. Bastano, in ACI – ANFIA, 2001). The problem of congestion costs definition and calculation is tackled from very different perspectives.

The former adopts a top-down approach and is based on the idea that road congestion costs are external both with respect to the user and with respect to the transport system (i.e. the whole society would bear the consequences of congestion). The latter is a modelling exercise on the basis of traffic data (bottom-up approach) based on a welfare economic standard approach (dead-weight loss), and maintaining the idea that road congestion cost is internal to the road users club.

We have not taken into consideration these approaches to carry on congestion cost estimations, since they were not recommended by UNITE methodology (see Link H, Stewart L, Maibach M et al., 2000).

The following paragraphs briefly describe input data, methodology and results of both studies.

#### The study by Amici della Terra

The analysis of road congestion costs is part of a wider study on the environmental and social costs of mobility in Italy, contained in Lombard P. L., Molocchi A. (1998). The analysis refers to 1995 and focuses on urban areas congestion (municipalities with more than 20 000 inhabitants). It is worth mentioning that the authors themselves point out that the results are severely affected by the lack of data: this made it necessary to adopt simplifying hypotheses that might lead to an underestimation of the phenomenon.

This study adopts a top-down approach, starting from the available data on time spent for systematic and non-systematic trips in cities with more than 500 000 inhabitants, the population, the average speed. Table 11 shows the basic input data adopted in the study.

Considering that in 1995 the population of the selected municipalities was 7.34 million, and considering the quota of mobility due to motorcycles and mopeds white are assumed to be not affected by congestion, the daily amount of mobility affected by congestion is found to be 13.33 million of hours. Table 12 shows the results for the cities with more than 500 000 inhabitants.

Table 11
Congestion costs estimation - Basic input data
(Lombard, Molocchi 1998)

| Average daily per capita time spent for systematic and non systematic trips (in minutes) |  |
| --- | --- |
| Systematic trips | 45 m |
| Non systematic trips | 80 m |
| Total | 125 m (2.08 h) |
| Population | 7.34 million |
| Share of mobility due to mopeds and motorcycles 1) | 12.7% |
| Average speed |  |
| Congested conditions 2) | 18.4 km/h |
| Non congested conditions 3) | 25 km/h |
| 1) It is considered that the quota of mobility due to motorcycles and mopeds should not be affected by congestion; this quota is therefore subtracted from the total amount of mobility. 2) This data comes from a survey by CENSIS containing information on the average vehicle speed on the total of weekly trips in big urban areas, concerning the cities of Milan, Rome and Naples (CENSIS 1997b) 3) This is a conservative estimation made by the authors. Source: Lombard P. L., Molocchi A. (1998) on the basis of CENSIS and Ministry of Transport data. |  |

Table 12
Time lost in 1995 in the Italian cities with more than 500 000 inhabitants
due to congestion

|  | Millions of hours | Hours |
| --- | --- | --- |
| Weekly amount of mobility susceptible of congestion | 93.3 |  |
| Time lost due to congestion (weekly) | 24.6 |  |
| Time lost in 1995 by passenger and transport vehicles users due to congestion | 1 282.0 |  |
| Time lost in 1995 per inhabitant due to congestion |  | 174.6 |
| Source: Lombard P. L., Molocchi A. (1998) |  |  |

The analysis is extended in order to include congestion in medium (between 100 000 and 500 000 inhabitants) and small cities (between 20 000 and 100 000 inhabitants).

No data were available on time spent for systematic and non-systematic trips in medium sized and small cities; this amount is calculated on the basis of data available for large cities, adjusted through the vehicles density (ratio of number of vehicles and area occupied by the municipality).

Table 13 shows the total time lost in urban areas according to Lombard and Molocchi, per class of population.

Table 13
Time lost in 1995 in urban areas in Italy

| Municipalities per population classes | Millions of hours |
| --- | --- |
| Municipalities with > 500 000 inhabitants | 1 282.00 |
| Municipalities between 100 000 and 500 000 inhabitants | 349.00 |
| Municipalities between 20 000 and 500 000 inhabitants | 440.00 |
| Total municipalities > 20 000 inhabitants | 2 071.90 |
| Source: Lombard P. L., Molocchi A. (1998) |  |

The monetary value of time lost in congestion is defined as the value of lost production and consumption due to time lost by the society, respectively 43.2 € million (83.6 billions ITL) for employed and 7.7 € million (14.9 billion ITL) for unemployed (these values are expressed at 1995 prices and are provided by CENSIS, 1997a). Table 13 shows the results for the year 1995.

Table 14
External road congestion costs in 1995

|  | External congestion costs (€ million 1998) | Average congestion cost (€/1000 p-km, €/1000 t-km) |
| --- | --- | --- |
| Road | 7 745 |  |
| Passenger Transport | 2 594 | 11.9 |
| Private transport | 3 013 | 12.4 |
| Cars | 3 013 | 14.6 |
| Motorcycles, mopeds | - | - |
| Collective transport – buses and coaches | 95 | 5.6 |
| Freight transport | 4 637 | 65.3 |
| Light good vehicles | 1 380 | 275.9 |
| Heavy good vehicles | 3 257 | 49.4 |
| Source: Lombard P. L., Molocchi A. (1998) |  |  |

#### The study by ACI – ANFIA

This study, referred to the year 1998, is conducted by a group of researchers from CSST following the standard welfare economics approach to congestion (dead-weight loss approach, here called quantitative-economic approach), based on a contribution by R. Prud’homme (in ACI-ANFIA, 2001). Congestion costs are defined as those economic costs that society bears when road utilisation level is at current levels instead of being the optimal one.

This model is applied to a sample of 18 Italian municipalities, aggregated in 3 clusters according to the number of inhabitants. The unitary value of time applied is parameterised through factors allowing to take into account the variation of average and actual congestion levels for the central and the peripheral area and the presence of heavy vehicles on the network (transformed in vehicles equivalent).

The resulting values of time are the following:

- Large municipalities: 4.30 €/1 000 v-km

- Medium sized municipalities: 6.26 €/1 000 v-km

- Small municipalities: 4.69 €/1 000 v-km

The application and calibration of the model allows calculating the congestion costs related to the intra-municipal trips (2.32 € billions). The application of the model to a wider territorial area, in order to capture the longer trips (the territorial extension correspond to Italian Provinces), gives as a result a cost of 0.52 € billions. The total congestion costs for Italy in 1998 amounted therefore to 2.84 € billions.

### Accident costs

Basic data for estimating accident costs are related to the number of accidents, injuries and victims by transport modes, together with the estimation of the number of accidents involving damages to property only.

Some issues have to be underlined in order to provide a proper assessment of the data. They fall under the following headings:

- the estimation of the rate of underreporting;

- the insufficient coverage of injuries, i.e. the unavailability of the classification between slight and severe injuries;

- the restricted circulation of statistical information concerning damage to property, available only for road and with no reference to the damage of public property assets.

The issue of underreporting is strictly related to the characteristics of Italian statistical information system for accidents. Furthermore, it is only since 1999 that the time between the reported accident by police authorities and the health condition of the injured person has been extended to one month (a period allowing to include almost 99% of fatalities). Previously, only seven days were considered, leading to the underestimation of the number of fatalities.

Also, the insufficient classification of injuries depends on the characteristics of the accidents information system. In particular, the health national system doesn’t report the cause of the injury, which makes it impossible to relate the person injured with the accidents. This implies that sanitary information on the person injured in an accident, i.e. the degree of injury (slight or severe) and the number involving on rehabilitation therapies can only be estimated.  Regional and urban samples of collaboration between local health system agencies and local police authorities  can contribute to estimate the incidence of slight or severe injuries.

The National Institute of Statistics (ACI-ISTAT, 1998) suggests the following coefficients:

- 1.47 for injured persons

- 1.28 for fatalities

which have to be multiplied by the official reported number of accidents in order to offset the underestimation bias of official statistics on accidents.

The tables below show the official (Table 15) and estimated (Table 16) number of accidents by road type. The phenomenon of underreporting occurs only for road.

Table 15
Road accident cases by network type in Italy 1998 1

| Road type | Damages to property | Injuries | Fatalities |
| --- | --- | --- | --- |
| All Roads | 2 227 885 | 293 842 | 5 857 |
| Motorways | 150 649 | 24 356 | 660 |
| Other non urban roads 2 | 412 303 | 60 224 | 2 606 |
| Urban roads | 1 664 934 | 209 262 | 2 591 |
| 1) As reported to police (injuries and fatalities) and estimated from national insurance companies sources (damages to property) 2) State, Provincial and non-urban roads |  |  |  |
| Sources: ISTAT, ANIA |  |  |  |

Table 16
Total number of casualties in Italy 1998

|  | Casualties reported to police |  | Casualties estimated |  | Total number of casualties |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Injuries | Fatalities | Injuries | Fatalities | Injuries | Fatalities |
| Road / public transport |  |  |  |  |  |  |
|  | 293 842 | 5857 | 138 106 | 1 513 | 431 948 | 7 370 |
| Car | 196 620 | 3841 | 92 411 | 1 075 | 289 031 | 4 916 |
| Motorcycle | 72 086 | 1185 | 33 880 | 332 | 105 966 | 1 517 |
| Bus / tramway | 2 098 | 29 | 986 | 8 | 3 084 | 37 |
| Truck drivers | 10 226 | 349 | 4 806 | 0 | 15 032 | 349 |
| Others 3) | 12 812 | 453 | 6 022 | 98 | 18 834 | 551 |
| Rail 1) 2) | 238 | 83 | . | . | 238 | 83 |
| Aviation 2) | 31 | 20 | . | . | 31 | 20 |
| Inland navigation 2) | 8 | 4 | . | . | 8 | 4 |
| Maritime shipping | : | : | . | . | : | : |
| 1) Including FS and other railway companies. – 2) Five-year average including passengers and personnel, suicides excluded. 3) Coaches, Van, etc Sources: CNT, FS, ISIS |  |  |  |  |  |  |

The number of accidents for other transport modes is obtained as the average of the previous five years. No information is available for accidents in maritime shipping.

With reference to the accidents with material damage, the only estimation available from national insurance companies (ANIA, 2000) is related to road, with an allocation by vehicle type based on the number of accidents (Table 17).

Table 17
Material damages in Italy road transport 1998

| Damages to property | Total number of accidents |
| --- | --- |
| Road Accidents | 2 227 885 |
| Damage to vehicles | 2 227 885 |
| Passenger car | 1 696 379 |
| Motorcycle | 322 410 |
| Bus / Coach | 18 259 |
| Tramway | n.a. |
| Goods vehicle | 152 206 |
| Others | n.a. |
| Public property | n.a. |
| Other private property | n.a. |
| Source: ANIA |  |

An overview of statistical sources used for accident costs is presented in Table 18. Table 19 shows statistical sources for the five components of accidents costs:

Table 18
Sources and quality of input data for estimating accident costs by transport mode

|  | Input data | Level of disaggregation | Quality of data, level of uncertainty |
| --- | --- | --- | --- |
| Road | Statistics of accidents are taken from the Italian National Statistical Institute (ISTAT), integrated with the survey from the Association of National insurance Companies (ANIA) for the estimation of damages to property. No accidents resulting from suicide have been included. | 7 vehicle categories including public transport. Road types: motorways, urban roads and other roads (State and Provincial roads). | The difference between the number of accidents actually occurring and the number reported to the police and to insurance has been estimated using methodology from ISTAT |
| Rail | The number of accidents is taken from the Transportation National Account (CNT), integrated with the statistics from the National Railway Company (FS). A 5 year average is used to derive the yearly figure. | Passenger and freight only. | Good official statistics. No problems with underreporting. No division of statistics between Italian National Railways and Non-National Railways possible. |
| Public Transport | See road transport | See road transport | See road transport |
| Air | The number of accidents is taken from the Transportation National Account (CNT), Only passengers and transport staff are considered. A 5 year average figure is used. | One total for aviation. | Good official statistics. No problems with underreporting. |
| Inland waterway | The number of accidents is taken from the Transportation National Account (CNT), Only passengers and transport staff are considered. 5 year average figure is used. | One total for inland waterways. | Good official statistics. No problems with underreporting. |
| Maritime Shipping | No accident data for shipping available. | No data available | No data available |

Table 19
Sources and quality of input data for estimating accident costs by cost category

|  | Input data | Level of disaggregation | Quality of data, level of uncertainty |
| --- | --- | --- | --- |
| Costs of medical treatment | Costs and share of accident types with and without a steady reduction of working power estimated by ISIS using data from ISTAT and CENSIS (1997). | Injuries by severity class | Average length of illness: ISTAT estimates. Unitary Costs of lost production as proportion of GNP. |
| Valuation of administrative costs | Costs of police per accident from ISTAT. Administrative costs from national insurance companies are not available. Cost of justice are estimated from ISTAT (1998) | Costs of police per transport mode and severity of accident. | Costs estimated for road accident were transferred to other transport modes and are estimates only. |
| Valuation of material damage | Average cost of material damage to vehicles from ANIA for road transport only. | By vehicle category | Road estimation including public transport, not transferable to other modes. |
| Production losses | Net value of production losses from ISTAT | Estimation for slight and severe injuries | Estimation of production losses are estimated according to 1995 reference year and updated through indicators of GDP per capita and consumption expenditures |
| Risk Value | UNITE standard values (Nellthorp et al. 2000) | Risk values for accident victims only. No risk value for relatives and friends | Value is based on latest available studies and standardised for UNITE. The high risk value of € 1.14 million for Italy (at factor costs) represents the largest cost in this cost category |

### Environmental costs

The input data for the calculations were provided by the country account leaders. These data were used for cost calculations based on the ExternE methodology:

- with the EcoSense computer model for airborne pollutants,

- with shadow values for greenhouse gas emissions, and

- with new exposure-response functions and monetary values for noise.

#### General data for airborne emissions

The emissions due to the operation of internal combustion engines and the fuel production of electricity for electric traction were provided by the country account leaders.

Beside these emissions from the operation of a vehicle or vessel and the production of traction electricity the emissions due to the provision of the respective fuels was considered. The emission factors for crude oil extraction, refining and transport of petrol, diesel and kerosene are given in Table 20. The table gives as well the factors for extraction, transport and (where applicable) refinery of Coal, Lignite, Oil and Gas.

Table 20
Emissions caused by fuel production processes

| Type of emission | Unit | CO2 | PM10 | NOx | SO2 | NMVOC |
| --- | --- | --- | --- | --- | --- | --- |
| Emissions caused by the production of | g/kg fuel |  |  |  |  |  |
| Petrol |  | 560 | 0.105 | 1.10 | 1.90 | 1.80 |
| Diesel; Kerosene |  | 400 | 0.047 | 0.96 | 1.40 | 0.62 |
| Emissions caused by extraction, transport and refinery of | mg/kWhel |  |  |  |  |  |
| Coal |  | 34 000 | 3.5 | 44.4 | 38.1 | n.a. |
| Lignite |  | 31 900 | 3.1 | 50.6 | 13.8 | n.a. |
| Oil |  | 67 000 | 48.9 | 170.4 | 404.3 | n.a. |
| Gas |  | 14 800 | 17.9 | 69.3 | 3.25 | n.a. |
| Source: Production of petrol and diesel: Friedrich and Bickel (2001) for PM10, IFEU (1999) for other pollutants. Provision of power plant fuels: European Commission (1999b) |  |  |  |  |  |  |

#### General data for the calculation of costs due to air pollution

Besides the emissions of the transport modes in the different countries, a large number of additional information was required for the cost calculations. This includes data on the receptor distribution, meteorology, and on background emissions from all sources in all European countries. Such data is available in the EcoSense database and is briefly described in the following Table 21.

Table 21
Environmental data in the EcoSense database

|  | Resolution | Source |
| --- | --- | --- |
| Receptor distribution |  |  |
| Population | administrative units, EMEP 50 grid | EUROSTAT REGIO Database, The Global Demography Project |
| Production of wheat, barley, sugar beat, potato, oats, rye, rice, tobacco, sunflower | administrative units, EMEP 50 grid | EUROSTAT REGIO Database, FAO Statistical Database |
| Inventory of natural stone, zinc, galvanized steel, mortar, rendering, paint | administrative units, EMEP 50 grid | Extrapolation based on inventories of some European cities |
| Critical Loads/Levels for nitrogen-deposition for various ecosystems | EMEP 150 grid | UN-ECE |
| Meteorological data |  |  |
| Wind speed | EMEP 50 grid | European Monitoring and Evaluation Programme (EMEP) |
| Wind direction | EMEP 50 grid | European Monitoring and Evaluation Programme (EMEP) |
| Precipitation | EMEP 50 grid | European Monitoring and Evaluation Programme (EMEP) |
| Emissions |  |  |
| SO2, NOx, NH3, NMVOC, particles | administrative units, EMEP 50 grid | CORINAIR 1994/1990, EMEP 1998 TNO particulate matter inventory (Berdowski et al., 1997) |
| Source: IER. |  |  |

Receptor data

- Population data

Population data was taken from the EUROSTAT REGIO database (base year 1996), which provides data on administrative units (NUTS categories). For impact assessment, the receptor data is required in a format compatible with the output of the air quality models. Thus, population data was transferred from the respective administrative units to the 50 x 50 km2 EMEP grid by using the transfer routine implemented in EcoSense.

- Crop production

The following crop species were considered for impact assessment: barley, oats, potato, rice, rye, sunflower seed, tobacco, and wheat. Data on crop production were again taken from the EUROSTAT REGIO database (base year 1996). For impact assessment, crop production data were transferred from the administrative units to the EMEP 50 x 50 km2 grid.

- Material inventory

The following types of materials are considered for impact assessment: galvanised steel; limestone; mortar; natural stone; paint; rendering; sandstone; and, zinc. As there is no database available that provides a full inventory of materials, the stock at risk was extrapolated in ExternE from detailed studies carried out in several European cities.

Emission data

As the formation of secondary pollutants such as ozone or secondary particles depends heavily on the availability of precursors in the atmosphere, the EcoSense database provides a European wide emission inventory for SO2, NOx, NH3, NMVOC, and particles as an input to air quality modelling. The emission data are disaggregated both sectorally (‘Selected Nomenclature for Air Pollution’ - SNAP categories) and geographically (‘Nomenclature of Territorial Units for Statistics’ - NUTS categories). As far as available, EcoSense uses data from the EMEP 1998 emission inventory (Richardson 2000, Vestreng 2000, Vestreng and Støren 2000). Where required, data from the CORINAIR 1994 inventory and the CORINAIR 1990 inventory (McInnes 1996) are used. For Russia, national average emission data from the LOTOS inventory (Builtjes 1992) were included. Emission data for fine particles are taken from the European particle emission inventory established by Berdowski et al. (1997).

The emissions provided by the country account leaders were compared to the emissions in the EcoSense database. For road transport the primary particle emission data provided by the country account leaders were in general between 40 and 65% lower than the data in the EcoSense database. SO2 emissions compare well with the exceptions of France (data provided 20% lower) and the UK (data provided 300% higher). For NO2 as well the figures compare well with the exceptions of Spain (data provided 12% higher), France (data provided 55% lower), and Sweden (data provided 12% higher). For the NMVOC emissions, the data matched well for Denmark, Ireland, and the Netherlands; for Austria the data provided was 33% lower, for Spain 75% lower, for France 89% lower, for Sweden 8% lower, and for the UK 11% higher than in the EcoSense database. Among those differences, the differences in the primary particle emissions have the highest importance for the total costs.

This comparison of course cannot say which data are better, as the EcoSense emission inventory’s primary aim is to provide a complete set of “background” emission data for the whole of Europe and not to give the best available data for single countries. But the comparison gives a picture where discrepancies occur and further work is required.

The differences between emission data provided and the EcoSense database are higher for the modes other than road. This reflects the less advanced state of emission modelling in comparison to road transport.

Meteorological data

The Windrose Trajectory Model requires annual average data on wind speed, wind direction, and precipitation as an input. The EcoSense database provides data from the European Monitoring and Evaluation Programme (EMEP) for the base year 1998.

#### Basic data for airborne emissions in Italy

This paragraph provides the main input data concerning energy consumption and direct transport emissions in 1996 and 1998.

Energy consumption

The following tables 22 to 25 show the energy consumption input data used for environmental costs calculation.

Table 22
Energy consumption 1996 and 1998 (in tonnes)
- Road transport -

|  | Fuel | Motorways | Urban | Other non urban |
| --- | --- | --- | --- | --- |
|  |  | 1996 |  |  |
| Moped | Petrol | 0 | 449 746 | 192 748 |
| Motorcycles | Petrol | 31 765 | 310 158 | 165 499 |
| Passenger Cars | Petrol | 2 578 881 | 7 262 632 | 5 900 954 |
| Passenger Cars | Diesel | 1 330 329 | 892 784 | 1 681 384 |
| Buses (Urban & Coaches) | Diesel | 292 210 | 371 832 | 231 168 |
| Light Goods Vehicles (< 3.5 t) | Diesel | 461 375 | 777 358 | 958 562 |
| Heavy Goods Vehicles (> 3.5 t) | Diesel | 3 497 831 | 1 232 395 | 2 727 886 |
|  |  | 1998 |  |  |
| Moped | Petrol | 0 | 491 373 | 210 588 |
| Motorcycles | Petrol | 34 194 | 334 773 | 178 334 |
| Passenger Cars | Petrol | 2 823 985 | 7 051 921 | 6 165 752 |
| Passenger Cars | Diesel | 1 483 109 | 990 840 | 1 942 688 |
| Buses (Urban & Coaches) | Diesel | 301 673 | 392 834 | 239 299 |
| Light Goods Vehicles (< 3.5 t) | Diesel | 530 358 | 895 487 | 1 101 241 |
| Heavy Goods Vehicles (> 3.5 t) | Diesel | 3 679 197 | 1 245 662 | 2 780 602 |
| Source: ANPA Unita' censimento fonti di emissioni |  |  |  |  |

Table 23
Energy consumption 1996 and 1998 (in tonnes and kWh)
- Public transport -

|  | Diesel traction | Electric traction |
| --- | --- | --- |
|  | Fuel use tonnes | Electricity use kWh |
|  | 1996 |  |
| Urban Buses (Diesel) | 231 521 | - |
| Trolley Buses | - | : |
| Trams | - | 80 800 000 |
| Metro | - | 170 646 499 |
| Light Rail | - | 2 940 000 |
|  | 1998 |  |
| Urban Buses (Diesel) | 248 625 | - |
| Trolley Buses | - | : |
| Trams | - | 77 415 000 |
| Metro | - | 175 231 032 |
| Light Rail | - | 3 204 000 |
| Source: ANPA Unita' censimento fonti di emissioni, ISIS calculations |  |  |

Table 24
Energy consumption 1996 and 1998 (in tonnes and kWh)
- Rail transport -

|  | Diesel Traction (tonnes) | Electric Traction (kWh) |
| --- | --- | --- |
|  | 1996 |  |
| High Speed Trains | - | 105 089 137 |
| Passenger Trains | 172 371 | 3 618 310 389 |
| Freight Trains | 8 717 | 1 064 111 998 |
|  | 1998 |  |
| High Speed Trains | - | 229 006 187 |
| Passenger Trains | 179 414 | 3 261 794 736 |
| Freight Trains | 8 631 | 911 018 559 |
| Source: Annuario FS 1996, 1999 and ISIS calculations |  |  |

Table 25
Energy consumption 1996 and 1998 (in tonnes)
- Air transport -

|  | Fuel use |
| --- | --- |
|  | 1996 |
| Airports (LTO-cycle) | 741 163 |
| Flights | 2 397 017 |
|  | 1998 |
| Airports (LTO-cycle) | 645 645 |
| Flights | 2 692 356 |
| Source: ANPA Unita' censimento fonti di emissioni, ISIS |  |

###### Direct transport emissions 1998

For the calculation of the costs of vehicles direct emissions an emission inventory was produced. These input data are shown in table 17.

Table 26
Direct transport emissions in Italy 1998

|  | CO2 (Million Tonnes) | PM10 (tonnes) | NOx (tonnes) | SO2 (tonnes) | NMVOC (tonnes) |
| --- | --- | --- | --- | --- | --- |
| Road |  |  |  |  |  |
| Moped | 2.1 | : | 842 | 562 | 257 989 |
| Motorcycles | 1.7 | : | 2 666 | 438 | 36 985 |
| Passenger cars | 62.2 | 13 883 | 431 607 | 17 250 | 623 533 |
| Buses | 2.1 | 1 320 | 20 142 | 645 | 3 093 |
| Light goods vehicles 1) | 7.9 | 8 832 | 24 168 | 2 527 | 9 183 |
| Heavy goods vehicles 2) | 24.2 | 16 389 | 245 598 | 7 705 | 38 004 |
| Total | 100.2 | 40 423 | 725 023 | 29 167 | 968 786 |
| Public Transport |  |  |  |  |  |
| Urban Buses | 0.8 | 607 | 8 258 | 249 | 1 642 |
| Trams | 0 | 8 | 87 | 185 | 1 |
| Metro | 0.1 | 19 | 197 | 418 | 3 |
| Light rail | 0 | 0 | 4 | 8 | 0 |
| Total | 0.9 | 634 | 8 546 | 860 | 1 646 |
| Rail |  |  |  |  |  |
| Diesel traction | 0.6 | 658 | 9 402 | 1 034 | 2 633 |
| Electric traction | 2.2 | 478 | 4 952 | 10 511 | 65 |
| Total diesel and electric traction | 2.8 | 1 136 | 14 354 | 11 545 | 2 698 |
| Air | 10.8 | : | 4 762 | 387 | 1 221 |
| Inland waterways | 0.1 | : | 906 | 85 | 101 |
| 1) Up to 3.5 t max. GVW. – 2) Over 3.5 t max. GVW. Source: ANPA, ACI, CNT, Lombard and Molocchi, Hickman et al., ISIS. |  |  |  |  |  |

- a)	Road transport

The emissions factors of Italian road fleet were provided by National Environmental Protection Agency ANPA (2000): these factors were calculated through COPERT II model for the year 1997. Total emissions for 1998 were thus estimated on the basis of data on circulating fleet and total fuel consumption provided by ANPA (2001). COPERT classification of the rolling stock was calculated by ISIS for 1998, upgrading the existing one (1997) through own estimations based on the publications of ACI (various years, 1999, 2000). The emissions of SO2 were estimated through fuel consumption and sulphur content (50 ppm for diesel engine and 40 ppm for petrol engine). Emissions were furthermore differentiated by road type (motorways, urban roads and other extra-urban roads) and by vehicle’s category (moped, motorcycles, passenger car, LGV, HGV and buses). PM10 emissions from petrol vehicles were not considered.

- b)	Rail transport

Data on specific emissions of national rail carrier due to electric traction (electricity production in g/kWh) were taken from Lombard, Molocchi (2000); these specific emissions are based on data from the National Electric Energy Agency - ENEL. Data on specific emissions (g/kWh) due to diesel traction (fuel usage) were taken from Hickman et al. (1999). Total consumption of both electric and diesel traction were taken from Ferrovie dello Stato (1999) and Ministero dell’Industria (1997 and 1999). Pollutant’s emissions disaggregation to diesel traction and electric traction was based on fuel use applying the above mentioned emission factors. The allocation to passenger and freight trains was based on data on traffic volume published in Ferrovie dello Stato (1999). For non-national railways basic data on specific emission coefficients and specific consumptions were not available, therefore national coefficients were applied instead. The total traffic volume disaggregation (diesel / electric) of non-national railways was performed through the number of locomotives in use; data on total traffic volume were taken from CNT (1998).

- c)	Public transport

Urban road public transport emission account was built using the same sources and methodology as described above for road account.

Concerning railborne public transport, energy consumption of electric vehicles was calculated based on specific energy consumption factors in kWh/ km considered for rail transport. The national traffic volume of tramways, metro and light rail trains was taken from CNT (1998). Data on specific emissions are those used for rail transport.

- d)	Air transport

The emissions due to aviation were calculated for landing and take-off (LTO cycles) at airports in Italy, based on data taken from ANPA (2001). PM10 emissions were not available

- e)	Inland waterways

Data on total emissions (not allocated to passenger and freight) from inland waterway transport are based on data from ANPA. PM10 emissions were not available.

- f)	Shipping

Data on pollutant emissions of short sea shipping were not available.

###### Direct transport emissions 1996

For 1996 accounting year the same sources and methodology described in the above paragraph for 1998 were used. The table below shows direct transport emissions estimated for 1996.

Table 27
Direct transport emissions in Italy 1996

|  | CO2 (Million Tonnes) | PM10 (tonnes) | NOx (tonnes) | SO2 (tonnes) | NMVOC (tonnes) |
| --- | --- | --- | --- | --- | --- |
| Road |  |  |  |  |  |
| Moped | 1.9 | : | 771 | 514 | 236 134 |
| Motorcycles | 1.5 | : | 2 473 | 406 | 34 282 |
| Passenger cars | 59.7 | 15 561 | 497 835 | 16 498 | 738 264 |
| Buses | 2.1 | 1 358 | 20 020 | 663 | 3 097 |
| Light goods vehicles 1) | 6.9 | 8.619 | 23.874 | 2.197 | 8.109 |
| Heavy goods vehicles 2) | 23.4 | 17.893 | 260.378 | 7.458 | 40.991 |
| Total | 95.6 | 43 430 | 805 351 | 27 737 | 1 060 877 |
| Public |  |  |  |  |  |
| Urban Buses |  | 596 | 7 868 | 232 | 1 589 |
| Trams | 0 | 9 | 91 | 193 | 1 |
| Metro | 0.1 | 19 | 192 | 407 | 3 |
| Light rail | 0 | 0 | 3 | 7 | 0 |
| Total | 0.1 | 624 | 8154 | 839 | 1593 |
| Rail |  |  |  |  |  |
| Diesel traction | 0.6 | 634 | 9 054 | 996 | 2 535 |
| Electric traction | 2.4 | 519 | 5 386 | 11 432 | 71 |
| Total diesel and electric traction | 3.0 | 1 153 | 14 440 | 12 428 | 2 606 |
| Air | 9.6 | : | 4 212 | 342 | 1 083 |
| Inland waterway | 0.1 | : | 927 | 87 | 103 |
| 1) Up to 3.5 t max. GVW. – 2) Over 3.5 t max. GVW. Source: ANPA, ACI, CNT, ISIS, Hickman et al., Lombard and Molocchi, |  |  |  |  |  |

#### Global warming

The input data for the calculation of the costs of CO2 for 1996 and 1998 accounting years are directly based on the level of CO2 emissions described in the previous section for all modes of transport. The calculation was carried out through energy consumption data taken from the same source described in air pollution paragraph. The monetary values used for cost calculation are described in chapter 3.

#### Noise

Compared with the detailed information available on airborne emissions, the data quality on noise exposure is rather poor. Exposure estimates were taken from Lombard, Molocchi (2000) for road, rail and air transport (see table 28), and are referred to 1997. These data were based mainly on OCSE (1993), adjusted through INFRAS/IWW (1995) and Italian Ministry of Environment estimates (Ministero dell’Ambiente 1997). A further breakdown of the exposure by passenger or goods transport and by vehicle type was not possible, in order to avoid arbitrary cost allocation. For inland waterway and maritime transport no data were available, but the damage due to these transport modes can be expected to be negligible since receptor density is very low. Concerning public transport, it was not possible to isolate its contribution from road and rail accounts, therefore the respective accounts take into consideration respectively road public transport and rail public transport noise impacts.

The exposure estimates in table 28 were considered both for 1996 and 1998, because there is no evidence of significant variations between these two years.

Table 28
Italian population exposed to road and railway noise 1997

|  | Day & Night |  |  |
| --- | --- | --- | --- |
| Noise level (dB(A)) | Road (Mil. Pers) | Air (Mil. Pers) | Rail (Mil. Pers) |
| 55-60 | 18.49 | 1.42 | 3.50 |
| 60-65 | 12.14 | 1.02 | 2.50 |
| 65-70 | 7.03 | 0.44 | 1.30 |
| 70-75 | 2.11 | 0.20 | 0.40 |
| >75 | 0.60 | 0.12 | 0.10 |
| Source: Lombard, Molocchi (2000) |  |  |  |

#### Nature and landscape

The basic data for the application of the compensation approach - the share of roads and other infrastructures cutting through ecologically sensitive areas - was not available and therefore it was not possible to estimate nature and landscape costs.

#### Summary

Table 29 presents a summary of all input data used for the estimation of different types of environmental costs and remarks on data quality.

Table 29
Source and quality of input data for estimating environmental costs

|  | Input data | Level of disaggregation | Quality of data, level of uncertainty |
| --- | --- | --- | --- |
| Air Pollution | Vehicle emission data calculated from vehicle mileage (ANPA, FS, CNT) and emission factors (ANPA, Hickman et al. 1999). | The emissions of CO2, PM10, NOx, SO2 and NMVOC are estimated for road transport (5 vehicle types) and public transport. The emissions of CO2, PM10, NOx, SO2 and NMVOC are estimated for rail (passenger and freight) and for emissions due to diesel and electricity traction. The emissions of CO2, NOx, SO2 and NMVOC are estimated for aircraft and inland waterway. Emissions for shipping are not estimated. | Input data are of good quality. |
| Global warming | Vehicle emission data for CO2 as above (Air Pollution). | Road, rail (passenger and freight), public transport, aviation, inland waterways. | Data are of good quality. Uncertainty remains with the valuation of CO2. A shadow price of € 20 per ton CO2, has been used. This value is lower than presumed in previous studies, but reflects the latest estimates available. |
| Noise | Exposure estimates were taken from Lombard, Molocchi (1998); this estimate was based on OCSE (1993), INFRAS (1995) and Ministero dell’Ambiente (1997). Noise exposure for inland waterway and shipping is negligible. | Road, rail, aviation (not allocated to passenger and freight). Public transport: could not be isolated from road account and is included there. | Exposure data is quite good, but not disaggregated at the needed level. |
| Nature, landscape, soil and water pollution | The basic data for the application of the compensation approach - the share of roads and other infrastructures cutting through ecologically sensitive areas - was not available and therefore it was not possible to estimate nature and landscape costs. | - | - |
| Source: ISIS. |  |  |  |

### Taxes, charges, subsidies

The basic data for the estimation of transport revenues is the annual amount of taxes paid, charges collected and subsidies received by transport users and operators in the years 1996 and 2005. When it was not possible to have updated information, we reported existing surveys (such as for waterborne transport).

Table 30 gives an overview of the sources and level of disaggregation of the input data, with an assessment of quality and level of uncertainty.

Table 30
Sources and quality of input data for estimating taxes, charges and subsidies by cost category

|  | Input data | Level of disaggregation | Quality of data, level of uncertainty |
| --- | --- | --- | --- |
| Road | All data on tax revenues are provided by the Italian Ministry of Transport and from ACI. Revenues from highways and tunnel charges are provided by AISCAT No data were available for parking charges | No disaggregation is available for tax revenues Heavy and light vehicles | Good data for 1996 and 1998. |
| Rail | Data on charges and subsidies are provided by the national railways company (former FS, actually RFI and Trenitalia) Data on non national railways revenues are provided by the Ministry of Transport (CNT) | Data are disaggregated by type of revenue | Very good data for 1996 and 1998 |
| Public Transport | Data on tariff and other services revenues and on subsidies are drawn from CISPEL | Data are disaggregated by type of revenue | Last data refer to 1995; 1996 and 1998 data are ISIS estimations |
| Air | Data on aviation charges and subsidies provided by ENAC and Ministry of Infrastructures and Transport Data on air traffic control revenues come from ENAC and EUROCONTROL | Data are disaggregated by airport and by type of revenue Data are disaggregated by airport for terminal charges; only a total annual figure is available for en route charges | Data for 1996 are very good. Data for 1998 and 2005 were estimated |
| Inland waterways and maritime transport | Information is drawn from a survey by CER based on direct information from Ministry of Finance and Transport. This survey only includes taxes and subsidies. | Data are disaggregated by type of revenue and passenger/freight | Data quality is good but not updated (1994) |

Road transport

The road revenues account takes into consideration the following categories of levies:

- levies on fuel and lubricants

- levies on vehicle ownership (formerly on vehicle circulation)

- levies on vehicle related administrative acts (registration)

- other taxes (driving license, tax on insurance premium, MCTC)

- charges on infrastructure use (on motorways, tunnel and parking)

The main source of revenues for the State is certainly the fuel and lubricants excise duty: this levy is originally imposed on fuel producers, and shifted to fuel consumers.

Taxes on vehicle are yearly due by vehicle owners according to the vehicle technical specifications (weight, loading capacity, power – in kW), irrespective of the actual circulation of the vehicle. Until 1st January 1998 special taxes were due for LPG and methane fuelled vehicles; the diesel annual surtax on top of the ownership tax was also abolished at the same date, and currently the extra-charge only concerns “non-ecological” diesel cars.

All vehicle owners must pay a tax for the registration to the PRA – Pubblico Registro Automobilistico, and subsequently a charge for each formal act presented to the public authorities, including ownership transfer taxes (IET - imposta erariale di trascrizione and APIET - addizionale provinciale all’imposta di trascrizione). Prior to 1999 these taxes were collected by the State (IET) and the Provinces put an additional levy on top of it (APIET). Since 1st January 1999 these two taxes have been reduced to one, including registration and transfer, collected by the Provinces (IPT – imposta provinciale di trascrizione).

Other taxes included in road revenues accounts are yearly taxes on driving license (they are considered only for 1996, since are no longer due from 1998), and rights to MCTC.

VAT on vehicles purchase equals the standard rate of 20%, and is therefore not taken into account.

The owners of an electric vehicle are exempt form paying the ownership tax for five years from the first registration, and the rate of ownership tax is lower for “special usage” dedicated vehicles and for their trailers (if exclusively used): no data are available on the amount of these indirect subsidies.

The road network access is free of charge for the most of the urban and interurban roads: only motorways and some Alpine tunnels access are subject to the payment of a charge, set on a distance-related basis.

Currently no data are available concerning the amount of parking charges applied in several inner city areas (especially by larger cities, but also by some small tourist resorts).

Public transport

Revenues from public transport services taken into account as core data derive from services selling (tickets, season tickets, etc.). Direct subsidies to running expenditures are added as supplementary information.

Rail transport

Revenues of the rail sector come from the following sources:

- Taxes paid by the service provider and the final users (VAT)

- Charges on the final users of the rail transport service (tariff revenues)

- Charges for rail infrastructure access

- State contributions

Revenues from service tariffs are included to match the corresponding cost category (rail supplier operating costs): within this item, we consider revenues from passenger and freight transport and revenues from Public Service Contract (Contratto di Servizio Pubblico), that is revenues from the State purchasing from the Italian state rail operator the services that it would not have produced other ways.

Rail infrastructure access charges were introduced in Italy within the framework of the rail sector liberalisation process in order to regulate the relationship between infrastructure provider/manager (RFI) and the rail services providers (Trenitalia – former FS - and other passenger and freight carriers) and to guarantee a fair and efficient infrastructure use. Infrastructure access charges are relevant only for 2005 accounts, since they were introduced in 1998 and calculation procedures established in 2000. Nevertheless, no estimates on revenues from these charges were available.

Concerning non-national railways, the Ministry of Transport yearly publishes data on revenues for licensed railways. We took into account revenues from passenger and freight transport and State and/or local administration contributions to current expenditures.

Air transport

The income of air transport activity is linked to the following services:

  - essential operating services (landing and take-off assistance, parking, air assistance, maintenance, safety…)

  - handling (land-side assistance)

  - non aeronautic operations (commercial services)

Who is actually receiving these service revenues depends on the airport management structure: in the “full management” airports (one concessionary manages the whole airport, including the flight infrastructures, for a very long period) every aeronautic and commercial income is received by the airport manager; in the “partial management” airports (flight infrastructure ownership is public, while a concessionary is in charge of passenger and freight terminal services and possibly handling operations; other services can be entrusted to other concessionaries) the State gathers start / landing fees and parking fees for aircrafts, while the manager collects revenues from boarding and handling operations and from commercial activities; in the “public direct management” airports (the State directly manages all activities but handling and commercial activities, that are realised by concessionaires) the State takes all incomes stemming from aircrafts movements and parking, and also concessionary fees for the activities entrusted to other actors.

It is worth mentioning that the noise charge on aircrafts movement is quite the only example of environmental tax on the Italian transport sector: this levy, collected since 1990 by the State, became a regional tax in 1997. Revenues from this tax are earmarked to finance interventions to reduce noise emissions from aircrafts, and in particular to complete and improve acoustic pollution monitoring systems, to reduce noise pollution and to pay compensations to residents living near the airport area, that are the most damaged.

# Methodological issues

## Methodology for estimating infrastructure costs

Infrastructure costs calculation followed the methodology proposed by Link et al. (2000), whose main working steps are:

- Capital stock evaluation;

- Running costs estimation.

For the purpose of total infrastructure costs assessment, and following the UNITE standards, infrastructure expenditures are defined as follows:

  - capital costs are expenditures for new investments and renewals;

  - running costs are expenditures for maintenance, operational costs and administrative costs.

The capital costs evaluation is based upon the determination of the capital stock’s value, calculated through the PIM (Perpetual Inventory Method). PIM is briefly described in Box 1 below; a more detailed description is available in Link et al. (2000). Because of lack of data, a capital costs disaggregation into new investments and investments for substitution of property/assets was not carried out. Time series of capital costs for infrastructure – published by the Ministry of Transport in CNT – were based on the total amount of investments for every single mode of transport; these investments do not include renewals. For the Italian Pilot Accounts, a time series of the required length for PIM application was collected for each mode, except for maritime transport. Since the CNT does not publish, for every year, a disaggregation of capital costs into “infrastructure” and “operational means”, the values for years 1966-1973 were set by taking into account data for year 1974 which are available instead.

As far as running costs are concerned, the cost allocation and possible disaggregation by mode of transport were set as described under par. 2.2.1. Basic data for this cost category are current expenditures for infrastructure and other services. These are published by CNT, with the following running costs disaggregation:

- maintenance costs;

- infrastructure operator costs;

- administrative costs (including training and R&D costs) of which:

- control (port and airport authorities);

- traffic police;

- traffic control.

Box 1
The perpetual inventory model

The main idea of the perpetual inventory concept, a concept which is used by most OECD-countries for estimating the capital stock of industrial branches, is to capitalise time series of annual investment expenditures by cumulating the annual investments and by subtracting the value of those assets which exceeded their life-expectancy (written down assets) as expressed in the equations below: VG t+1 = VG t + It,t+1 - At,t+1 (1) VN t+1 = VN t + It,t+1 - Dt,t+1 (2) with: VG t : Gross value of assets at time t VN t : Net value of assets at time t It,t+1 : Investments during t, t+1 At,t+1 : Written down assets during t, t+1 (assets which exceeded life-expectancy) Dt,t+1 : Depreciation during t, t+1 As shown in these formulas the perpetual inventory method can be applied for estimating the gross value (gross concept) and the net value (net concept) of infrastructure assets. The gross value contains the value of all assets which still exist physically in the considered year, e.g. which have not yet exceeded their life expectancy. Thus, At,t+1 denotes those assets which could not be used any longer or which were shut down. It is assumed that the assets are properly maintained and can be used until they exceed their defined life-expectancy. Within the net-concept the annual depreciation Dt,t+1 are considered. The net value of assets describes the time-value of all assets which have not yet exceeded life-expectancy. According to the international conventions of the SNA, most countries use a linear depreciation method. The general principle as described above can be refined by more sophisticated approaches which use probability functions for the written down assets. This type of perpetual inventory model was used for the Italian pilot accounts. In contrast to simple perpetual inventory models, the refined models assume that the life expectancies of assets within an investment vintage are dispersed over the mean value. A probability function, the so-called survival function, is estimated, which describes the share of assets which are still in use. The inverse function which describes the written down assets At,t+1 was estimated as a polynomial of the third degree in Italian meaning that the probability function of the written-down assets has a right-skewed shape. This approach considers the fact that the investment spent for an asset group consists of parts with different life expectancies which are dispersed within an interval around the mean. Although also in the Italian method for all elements of the investment I1 - In a linear depreciation is applied, the overall asset group shows in fact a degressive depreciation due to the underlying type of probability function for the written-down assets. The perpetual inventory model requires in general long time series on annual investment expenditures, information on life expectancies of assets, and initial values of the capital stock (except the investment time series is as long as the life expectancy). Due to the fact that the use of probability functions in the refined concept implies that not single assets but technically homogeneous groups of assets (earthworks, bridges/tunnels, terminal buildings, pavement and equipment) are considered, investment time series for asset groups (for example pavement, tunnels/bridges, equipment) have to be available.

As far as rail, tramway and metro are concerned the disaggregation of total running costs between expenditures for transport services and expenditures for infrastructure and other services is not available for 1996 and 1998. The latest disaggregated data concern year 1983. Thus, we calculated the parameter “current expenditure per kilometre of network”, dividing the total expenditure by the network kilometres and subsequently applying this data to the existing network in 1996 and 1998, for both licensed railways and State-owned ones (FS). Hereby we restrictively assume that licensed railways and State-owned ones have the same unit cost per kilometre of network. Finally, total rail running costs are expressed at 1998 prices. Further disaggregation was calculated by distributing current expenditures of 1996 and 1998 according to accounting details contained in CNT for public direct expenditures and State contributions for all transport modes.

As far as air transport is concerned, time series of capital expenditure for infrastructure have been taken into account as the amount of investments for air transport in Italy. Only gross investments, excluding renewals, were considered - as published by the CNT. Expenditure for infrastructure to the transport services as well as other services were excluded. No further disaggregation into construction and equipment was feasible. The two major airport systems of Rome (Fiumicino and Ciampino) and Milan (Linate and Malpensa) account for more than 60% of total passengers traffic and 74% of cargo traffic.

To forecast data on 2005 we took into account the General Transport Plan for Italy by the Ministry of Transport: it provides data on infrastructure expenditure forecasts from 2000 to 2010, split by mode of transport and disaggregated by kind of intervention – all figures refer to the capital costs and are shown in Table 31.

Table 31
Forecast capital costs for transport mode in Italy 2005
– € million 1998 –

| Infrastructure | Capital costs forecast |
| --- | --- |
| Road | 2 011 |
| Rail | 2 869 |
| Sea and Inland | 316 |
| Air | 177 |
| Public Transport | 1 291 |
| Source: Ministero dei Trasporti e della Navigazione 2001 |  |

Figure  1
Forecast capital costs for transport mode in Italy 2005

Source: Ministero dei Trasporti e della Navigazione (2001)

## Methodology for estimating supplier operating costs

The estimation of supplier operating costs has followed the criteria hereafter:

- According to UNITE standards, supplier operating costs include supplier current expenditures, as they are reported by the Ministry of Transport in its Transport National Account (CNT), which is the main data source;

- Official sources like CNT provide data on ‘current expenditures for infrastructure and other services’, without disaggregation in expenditures for infrastructure running costs and other supplier operating costs. The quota attributed to infrastructure running cost has been estimated to be 33%, according to the procedure illustrated in par. 3.1. The remaining quota has been attributed to supplier operating costs.

- Cost disaggregation for Italy was influenced by availability of detailed data and therefore a slight diversion from UNITE standards was necessary. Therefore the above mentioned cost categorisation does not fully conform to the ideal methodology for supplier operating accounts within UNITE (based on Macario et al. 2000), even though some of the categories suggested have been used. Total costs can be disaggregated as follows:

        - material + fuel + other material costs

        - services

        - maintenance costs

        - wage costs

        - change stocks

        - depreciation

        - other supplier operating costs

The 2005 account for rail and public transport supplier operating costs is based on the SIMPT simulations published in Ministero dei Trasporti e della Navigazione (2001). SIMPT forecasts the yearly value added average increase rate in the service sector to increase in the period 1998-2005 to fall in a range within the 1.45% (low level scenario) and the 1.85% (high level scenario). Taking into consideration a yearly GDP growth estimated to be 2.16% from 1998 to 2020 (EUROSTAT), we assume that yearly average supplier operating costs increase from 1999 to 2005 will be 1.65%.

## Methodology for estimating delay costs due to congestion

Delay costs due to congestion were not estimated within this account, see par. 2.2.3.

## Methodology for estimating accident costs

Core year 1998: The main components of the accident costs to be analysed for the evaluation of external and internal costs are the following:

- material damages

- administrative costs

- medical treatments

- production losses

- risk values

The methodology for the evaluation of social costs of accidents at 1998 follows as far as possible the recommendations of the Unite project for accident costs assessment contained in Doll et al. (2000), integrated with national contributions in the field.

The evaluation of external costs includes the estimation of costs not covered by transport users, i.e. medical treatments costs exceeding the contribution from the transport users for funding the National Health System, that must here be paid for outside the transport sector.

On the other hand, the evaluation of internal costs is based on cost borne by the users of transport sector, i.e. the material damage covered by insurance companies, without affecting non-transport users.

In cases one of the accident costs items includes both an internal and an external component, such as medical costs, the internal component has been deducted from the estimation of external costs.

As result, the internal costs of accidents include material damages and risk value, considered internalised by the transport users decisions, while the external costs involve medical costs, production losses and administrative costs.

The departure from the methodological framework for estimating accident costs as defined in Doll et al. (2000) consists in the costs allocation procedure. Due to the data characteristics for the Italian case, the matrix of cost bearers and responsibility, which can only be built on the basis of a detailed level of information, cannot be presented.

The allocation of costs by users of a specific transport mode is considered to be directly proportional to the costs borne by the specific transport modes, through the number of accidents recorded by each mode.

- a)	The costs of medical treatment

Table 32 shows the unitary costs per degree of casualty and type of action. The methodological background for the estimation of medical costs for day-hospital treatment, transport (including medical treatment) and rehabilitation therapies is drawn from ISTAT (1998).

As explained in the description of input data for estimating accident costs, the information concerning medical costs and the number of injured persons requiring rehabilitation therapies are gathered by means of statistical averages based on sample surveys at the urban and regional level.

The share of persons involved in accidents with slight and severe injuries are estimated respectively in 80% and 20%. Rehabilitation therapies affecting people with severe injuries is required by 30% for slight, temporary, reduction of working power and by 70% for severe, permanent cases.

Table 32
Average costs for medical treatment in Italy 1998
- in € per casualty -

| Type of action | Slight injuries | Severe injuries | Fatalities |
| --- | --- | --- | --- |
| Day-hospital treatment | 86.49 | 2 257.8 | 0 |
| Transport | 87.64 | 87.64 | 87.64 |
| Rehabilitation | 575.44 | 5 178.93 | 0 |
| TOTAL per casualty | 749.56 | 7 524.40 | 87.64 |
| Source: ISTAT |  |  |  |

The coverage of medical costs internal to the transport users is estimated to be € 929 000 000 in 1998 corresponding to the contribution to the National Health System in the measure of a percentage on insurance fees (8.5%).

- b)	Production losses

The methodology adopted for estimating the cost category of production losses include three components:

- the loss of the production power of chronically disabled

- the net loss of production deriving from traffic fatalities.

- the temporary costs for the victim’s employer.

The following table shows the corresponding unitary values.

Table 33
Average costs for the calculation of production losses
- in € per casualty -

| Category of treatment | Temporary inability |  | Permanent inability | Fatality |
| --- | --- | --- | --- | --- |
|  | Slight | Severe |  |  |
| Disability to work (days) | 10 | 25 |  |  |
| Share of victims in employable age |  |  |  |  |
| Occupied | 35.00% | 35.00% | 35.00% |  |
| Non occupied | 65.00% | 65.00% | 65.00% |  |
| Losses of production of one day for occupied people | 107.88 | 107.88 |  |  |
| Losses of consumption of one day for non-occupied people | 26.25 | 26.25 |  |  |
| Losses of production of occupied people (year) |  |  | 124 599.54 |  |
| Losses of consumption for non-occupied people (year) |  |  | 20 734.09 |  |
| Net losses of production for fatalities (year) |  |  |  | 98 294.88 |
| Source: ISTAT |  |  |  |  |

Estimations on the share of victims in employable age and unitary values are based on ISTAT and CENSIS (1997) surveys. The net value for production losses refers to gross production loss per lost year. In order to avoid double counting with the Risk Value the share of future consumption was subtracted from the gross production. We estimated this share as the final consumption expenditure of households at 1998 on the gross domestic output at market prices (equal to 80%).

c)	Valuation of administrative costs

The estimation of administrative costs is made of the costs for police and justice with reference to road. No clear and reliable information on administrative and legal costs of insurance sector are provided.

Table 34 shows the unitary costs borne by police and justice systems by type of accidents. These costs are external to the transport system because they are covered by transfer payments from State budget. In the case of costs for traffic police, ACI-ISTAT (1998) estimated the unitary costs at 1997 from a case study that has examined unitary costs by type of accident. Unitary costs at 1997 have been updated to 1998 prices at factor costs through the Unite Convention guide.

Table 34
Administrative costs: unit costs per accident 1998 
– € million 1998 –

| Unit Costs 1998 | Material damages | Injuries | Fatalities |
| --- | --- | --- | --- |
| Police | 114.92 | 130.59 | 260.38 |
| Justice | 24.29 | 24.29 | 24.29 |
| Source: ISTAT |  |  |  |

The evaluation of unitary costs for justice (ACI-ISTAT, 1998) derives from a study analysing the incidence of judiciary cases caused by accidents on the total of judiciary cases. In order to derive costs, the national expenditure for justice has been allocated to accidents in proportion of the share of judiciary cases.

- d)	Valuation of material damages

The estimation of material damages is based on data of the national association of insurance companies (ANIA, 2000), only for road and for three vehicle types. Estimations of damages to public property and to other private properties are not available.

Table 35
Average unit costs per material damage of road accidents 1998
- € per v-km -

| Mode of transport & Damage category | Unit costs per case reported to liability insurance |
| --- | --- |
| Damage to vehicles |  |
| Passenger car | 2 540 |
| Motorcycle | 1 650 |
| Goods vehicles 1 | 2 461 |
| Damage to public property | : |
| Damage to other private property | : |
| 1 Estimated value Source: ANIA |  |

- The unitary costs related to damage generated by goods vehicles are estimated through the overall amount of the insurance premium for recovering material damages of goods vehicles divided by the number of accidents caused by goods vehicle.

All costs of material damages are considered to be totally internal to the transport sector.

- e)	The risk value

The Risk Value has been taken from the recommendations of the UNITE valuation conventions for Italy (Nellthorp et al. 2001) and amounts to € 1.51 million for fatalities expressed at factor costs, i.e. the risk value has been divided by the average indirect taxation index in Italy (1.320 as per Nellthorp et al. 2001).

As results, the following values have been taken into account:

- € 1.14 million for fatalities,

- 13% of € 1.14 million = € 148 712 for severe injuries.

- 1% of the value of statistical life = € 11 439 for slight injuries.

With reference to slight and severe injuries, due to the unavailability of official data the estimated share of 80% (slights) and 20% (severe) has been applied. Risk values for relatives and friends were not considered. For the UNITE accounts, risk value is considered to be fully internal.

Year 1996: The evaluation of accident cost accounts 1996 is based on accidents and casualties reported by ISTAT (1996), both for the number of physical units and for the corresponding costs. For each cost category, the methodology previously outlined for 1998  has also been followed for 1996.

Forecast methodology: The methodology for deriving accidents accounts at 2005 is based on the use of two tools:

- a model for the forecast of the key variables related to accidents, i.e. mileage by vehicle types, and transport modes

- statistical analysis, i.e. regression analysis, for deriving the number of accidents and fatalities.

The regression analysis has been carried out for road accidents, where sufficient long time series are available, in order to estimate the relationship between mileage and number of accidents. The resulting number of accidents at 2005 is 105 000, with a multiple regression correlation coefficient R2 = 0.80.

For the other transport modes, where data availability does not allow for a statistical analysis, future trends concerning the number of accidents are set assuming that recent (ten years) trends can be confirmed.

## Methodology for estimating environmental costs

### Air pollution

#### General approach

In order to quantify the costs due to airborne pollutants, the Impact Pathway Approach developed in the ExternE project series, was applied. A detailed description of the approach can be found in European Commission (1999). The impact pathway approach is composed of the following steps: emission estimation, dispersion and chemical conversion modelling, calculation of physical impacts and monetary valuation of these impacts.

The ideal approach is to use emission inventories in spatial disaggregation (i.e. a geo-coded data set for the different air pollutants) for the calculation of the costs of direct emissions from vehicle operation. As such detailed data was not available for most of the countries, a simplified approach was used instead. Country-specific damage costs per tonne of pollutant emitted were calculated based on the emission inventory included in EcoSense, which contains information on the spatial distribution of emissions. For this, emissions were modified compared to the reference inventory and Europe-wide impacts were calculated and subtracted from impacts resulting from the reference inventory without unchanged emissions. This procedure using a reference inventory is required, because of air chemistry processes where “background” emissions play an important role. Then the resulting costs were divided by the difference in the amount of pollutants emitted to obtain the costs per tonne of pollutant. A description of the computer model EcoSense, which was used for the calculations, including exposure-response functions and monetary values is given below.

In addition to these regional scale calculations, damages on the local scale – up to about 20 km to each side of a line emission source (e.g. road) – were quantified. Due to the lack of detailed geo-coded emission data, specific local-scale costs for the categories “urban roads”, “extra-urban roads” and “motorways” were transferred from detailed calculations for Germany in Tranche A.

Then the emissions provided by the country account leaders were multiplied by the respective damage factor to obtain the costs caused by the different modes and vehicle categories.

Note that primary particle emissions from internal combustion engines were treated as PM2.5, while primary particles from fossil power plants were treated as PM10 (higher deposition rate and lower impact compared to PM2.5).

a)	Description of the EcoSense computer model for assessment of costs due to airborne emissions

The EcoSense model has been developed within the series of ExternE Projects on ‘External Costs of Energy’ funded by the European Commission (see e.g. European Commission 1999a). The model supports the quantification of environmental impacts by following a detailed site-specific ‘impact pathway’ (or damage function) approach, in which the causal relationships from the release of pollutants through their interactions with the environment to a physical measure of impact are modelled and, where possible, valued monetarily. A schematic flowchart of the EcoSense model is shown in Figure 1. EcoSense provides harmonised air quality and impact assessment models together with a comprehensive set of relevant input data for the whole of Europe, which allow a site-specific bottom-up impact analysis.

In ExternE, EcoSense was used to calculate external costs from individual power plants in a large number of case studies in all EU countries. While the first generation of the EcoSense model was focused on the analysis of single emission sources, the new ‘multi-source’ version of the model provides a link to the CORINAIR database, which allows the analysis of environmental impacts from more complex emission scenarios. The CORINAIR database provides emission data for a wide range of pollutants according to both a sectoral (‘Selected Nomenclature for Air Pollution’ - SNAP categories) and geographic (‘Nomenclature of Territorial Units for Statistics’ - NUTS categories) disaggregation scheme (McInnes, 1996). A transformation module implemented in EcoSense supports the transformation of emission data between the NUTS administrative units (country, state, municipality) and the grid system required for air quality modelling (EMEP 50 x 50 km2 grid). Based on this functionality, EcoSense allows to modify emissions from a selected sector (e.g. road transport) within a specific administrative unit, creates a new gridded European-wide emission scenario for air quality modelling, and compares environmental impacts and resulting damage costs between different emission scenarios. In other words, environmental damage costs are calculated by comparing the results of two model runs:

- A model run using the ‘full’ European emission scenario as an input to air quality and damage modelling, including emissions from all emission sources in Europe, as well as the emissions from the transport sector considered.

- A second model run in which the emissions from the transport sector considered were set modified.

The difference in impacts and costs resulting from the two model runs represents the damages due to modified emissions.

Figure  2
Flowchart of the EcoSense model

Source: IER

b)	Air quality models

Within the UNITE project two air quality models were used from the three available within the Eco-Sense system. The model for local scale effects was not required as they were covered based on GIS-based calculations.

- The Windrose Trajectory Model (WTM) (Trukenmüller et al. 1995) is used in EcoSense to estimate the concentration and deposition of acid species on a regional scale.

- The Source-Receptor Ozone Model (SROM), based on the EMEP country-to-grid matrices (Simpson et al. 1997), is used to estimate ozone concentrations on a European scale.

c)	Dose-effect models

The dose-response functions used within UNITE are the final recommendations of the expert groups in the final phase of the ExternE Core/Transport project (Friedrich and Bickel 2001). The following table gives a summary of the dose-response functions as they are implemented in the EcoSense version used for this study.

Table 36
Health and environmental effects included in the analysis of air pollution costs

| Impact category | Pollutant | Effects included |
| --- | --- | --- |
| Public health – mortality | PM2.5 , PM10 1) SO2, O3 | Reduction in life expectancy due to acute and chronic mortality Reduction in life expectancy due to acute mortality |
| Public health – morbidity | PM2.5 , PM10, O3 | respiratory hospital admissions |
|  |  | restricted activity days |
|  | PM2.5 , PM10 only | cerebrovascular hospital admissions |
|  |  | congestive heart failure |
|  |  | cases of bronchodilator usage |
|  |  | cases of chronic bronchitis |
|  |  | cases of chronic cough in children |
|  |  | cough in asthmatics |
|  |  | lower respiratory symptoms |
|  | O3 only | asthma attacks |
|  |  | symptom days |
| Material damage | SO2, acid deposition | Ageing of galvanised steel, limestone, natural stone, mortar, sandstone, paint, rendering, zinc |
| Crops | SO2 | Yield change for wheat, barley, rye, oats, potato, sugar beet |
|  | O3 | Yield loss for wheat, potato, rice, rye, oats, tobacco, barley, wheat |
|  | Acid deposition | increased need for liming |
|  | N, S | fertilisational effects |
| 1) Including secondary particles (sulphate and nitrate aerosols). Source: IER. |  |  |

d)	Exposure-response functions for the quantification of health effects

Table 37 lists the exposure response functions used for the assessment of health effects. The exposure response functions are taken from the 2nd edition of the ExternE Methodology report (European Commission 1999a), with some small modifications resulting from recent recommendations of the health experts in the final phase of the ExternE Core/ Transport project (Friedrich and Bickel 2001).

e)	Exposure-response functions for the quantification of impacts on crops

Functions are used within the model to quantify changes in crop yields due to the emissions of SO2, nitrates, ozone and acids.

f)	Exposure-response functions for the quantification of material damage

Functions were developed to quantify and value damages to limestone, sandstone, natural stone, mortar, rendering, zinc and galvanised steel and paint due to the effects of air pollution.

#### Monetary values

Table 38 summarises the monetary values used for valuation of transboundary air pollution. According to Nellthorp et al. (2001) average European values should be used for transboundary air pollution costs, except for the source country, where country specific values were used. These were calculated according to the benefit transfer rules given in Nellthorp et al. (2001). The values for the single countries are given in the Annex.

Table 37
Quantification of human health impacts due to air pollution1)

| Receptor | Impact Category | Reference | Pollutant | fer |
| --- | --- | --- | --- | --- |
| ASTHMATICS (3.5% of population) |  |  |  |  |
| Adults | Bronchodilator usage | Dusseldorp et al., 1995 | PM10 Nitrates PM2.5 Sulphates | 0.163 0.163 0.272 0.272 |
|  | Cough | Dusseldorp et al., 1995 | PM10, Nitrates PM2.5 Sulphates | 0.168 0.280 0.280 |
|  | Lower respiratory symptoms (wheeze) | Dusseldorp et al., 1995 | PM10 Nitrates PM2.5 Sulphates | 0.061 0.061 0.101 0.101 |
| Children | Bronchodilator usage | Roemer et al., 1993 | PM10 Nitrates PM2.5 Sulphates | 0.078 0.078 0.129 0.129 |
|  | Cough | Pope and Dockery, 1992 | PM10 Nitrates PM2.5 Sulphates | 0.133 0.133 0.223 0.223 |
|  | Lower respiratory symptoms (wheeze) | Roemer et al., 1993 | PM10 Nitrates PM2.5 Sulphates | 0.103 0.103 0.172 0.172 |
| All | Asthma attacks (AA) | Whittemore and Korn, 1980 | O3 | 4.29E-3 |
| ELDERLY 65+ (14% of population) |  |  |  |  |
|  | Congestive heart failure | Schwartz and Morris, 1995 | PM10 Nitrates PM2.5 Sulphates CO | 1.85E-5 1.85E-5 3.09E-5 3.09E-5 5.55E-7 |
| CHILDREN (20% of population) |  |  |  |  |
|  | Chronic cough | Dockery et al., 1989 | PM10 Nitrates PM2.5 Sulphates | 2.07E-3 2.07E-3 3.46E-3 3.46E-3 |
| ADULTS (80% of population) |  |  |  |  |
|  | Restricted activity days (RAD) | Ostro, 1987 | PM10 Nitrates PM2.5 Sulphates | 0.025 0.025 0.042 0.042 |
|  | Minor restricted activity days (MRAD) | Ostro and Rothschild, 1989 | O3 | 9.76E-3 |
|  | Chronic bronchitis | Abbey et al., 1995 | PM10 Nitrates PM2.5 Sulphates | 2.45E-5 2.45E-5 3.9E-5 3.9E-5 |
| ENTIRE POPULATION |  |  |  |  |
|  | Chronic Mortality (CM) | Pope et al., 1995 | PM10 Nitrates PM2.5 Sulphates | 0.129% 0.129% 0.214% 0.214% |
|  | Respiratory hospital admissions (RHA) | Dab et al., 1996 | PM10 Nitrates PM2.5 Sulphates | 2.07E-6 2.07E-6 3.46E-6 3.46E-6 |
|  |  | Ponce de Leon, 1996 | SO2 O3 | 2.04E-6 3.54E-6 |
|  | Cerebrovascular hospital admissions | Wordley et al., 1997 | PM10 Nitrates PM2.5 Sulphates | 5.04E-6 5.04E-6 8.42E-6 8.42E-6 |
|  | Symptom days | Krupnick et al., 1990 | O3 | 0.033 |
|  | Cancer risk estimates | Pilkington et al., 1997; based on US EPA evaluations | Benzene Benzo-[a]-Pyrene 1,3-buta-diene Diesel particles | 1.14E-7 1.43E-3 4.29E-6 4.86E-7 |
|  | Acute Mortality (AM) | Spix et al. / Verhoeff et al., 1996 | PM10 Nitrates PM2.5 Sulphates | 0.040% 0.040% 0.068% 0.068% |
|  |  | Anderson et al. / Touloumi et al., 1996 | SO2 | 0.072% |
|  |  | Sunyer et al., 1996 | O3 | 0.059% |
| 1) The exposure response slope, fer, has units of [cases/(yr-person-µg/m3)] for morbidity, and [%change in annual mortality rate/(µg/m3)] for mortality. Concentrations of SO2, PM10 , PM10, sulphates and nitrates as annual mean concentration, concentration of ozone as seasonal 6-h average concentration. Source: Friedrich and Bickel 2001. |  |  |  |  |

#### Discussion of uncertainties

In spite of considerable progress made in recent years the quantification and valuation of environmental damage is still linked to significant uncertainty. This is the case for the Impact Pathway Methodology as well as for any other approach. While the basic assumptions underlying the work in ExternE are discussed in detail in (European Commission 1999), below an indication of the uncertainty of the results is given as well as the sensitivity to some of the key assumptions.

Table 38
Monetary values for health impacts (factor costs, European average)
- € million at 1998 prices -

| Impact | Monetary value (rounded) |  |
| --- | --- | --- |
| Year of life lost (chronic effects) | 74 700 | € per YOLL |
| Year of life lost (acute effects) | 128 500 | € per YOLL |
| Chronic bronchitis | 137 600 | € per new case |
| Cerebrovascular hospital admission | 13 900 | € per case |
| Respiratory hospital admission | 3 610 | € per case |
| Congestive heart failure | 2 730 | € per case |
| Chronic cough in children | 200 | € per episode |
| Restricted activity day | 100 | € per day |
| Asthma attack | 69 | € per day |
| Cough | 34 | € per day |
| Minor restricted activity day | 34 | € per day |
| Symptom day | 34 | € per day |
| Bronchodilator usage | 32 | € per day |
| Lower respiratory symptoms | 7 | € per day |
| Source: Own calculations based on Friedrich and Bickel 2001 and Nellthorp et al. (2001). |  |  |

Within ExternE, Rabl and Spadaro (1999) made an attempt to quantify the statistical uncertainty of the damage estimates, taking into account uncertainties resulting from all steps of the impact pathway, i.e. the quantification of emissions, air quality modelling, dose-effect modelling, and valuation. Rabl and Spadaro show that - due to the multiplicative nature of the impact pathway analysis - the distribution of results is likely to be approximately lognormal, thus it is determined by its geometric mean and the geometric standard deviation g. In ExternE, uncertainties are reported by using uncertainty labels, which can be used to make a meaningful distinction between different levels of confidence, but at the same time do not give a false sense of precision, which seems to be unjustified in view of the need to use subjective judgement to compensate the lack of information about sources of uncertainty and probability distributions (Rabl and Spadaro 1999). The uncertainty labels are:

A = high confidence, corresponding to g = 2.5 to 4;

B = medium confidence, corresponding to g = 4 to 6;

C = low confidence, corresponding to g = 6 to 12.

According to ExternE recommendations, the following uncertainty labels are used to characterise the impact categories addressed in this report:

Mortality:		B

Morbidity:		A

Crop losses:		A

Material damage:	B.

Beside the statistical uncertainty indicated by these uncertainty labels, there is however a remaining systematic uncertainty arising from a lack of knowledge, and value choices that influence the results. Some of the most important assumptions and their implications for the results are briefly discussed in the following.

- Effects of particles on human health

The dose-response models used in the analysis are based on results from epidemiological studies which have established a statistical relationship between the mass concentration of particles and various health effects. However, at present it is still not known whether it is the number of particles, their mass concentration or their chemical composition which is the driving force. The uncertainty resulting from this lack of knowledge is difficult to estimate.

- Effects of nitrate aerosols on health

We treat nitrate aerosols as a component of particulate matter, which we know cause damage to human health. However, in contrast to sulphate aerosol (but similar to many other particulate matter compounds) there is no direct epidemiological evidence supporting the harmfulness of nitrate aerosols, which partly are neutral and soluble.

- Valuation of mortality

While ExternE recommends using the Value of a Life Year Lost rather than the Value of Statistical Life for the valuation of increased mortality risks from air pollution (see European Commission 1999 for a detailed discussion), this approach is still controversially discussed in the literature. The main problem for the Value of a Life Year Lost approach is that up to now there is a lack of empirical studies supporting this valuation approach.

- Impacts from ozone

As the EMEP ozone model, which is the basis for the Source-Receptor Ozone Model (SROM) included in EcoSense does not cover the full EcoSense modelling domain, some of the ozone effects in Eastern Europe are omitted. As effects from ozone are small compared to those from other pollutants, the resulting error is expected to be small compared to the overall uncertainties.

- Omission of effects

The present report is limited to the analysis of impacts that have shown to result in major damage costs in previous ExternE studies. Impacts on e.g. change in biodiversity, potential effects of chronic exposure to ozone, cultural monuments, direct and indirect economic effects of change in forest productivity, fishery performance, and so forth, are omitted because they currently cannot be quantified.

### Global warming

The method of calculating costs of CO2 emissions basically consists of multiplying the amount of CO2 emitted by a cost factor. Due to the global scale of the damage caused, there is no difference how and where the emissions take place.

A European average shadow value of €20 per tonne of CO2 emitted was used for valuing CO2 emissions. This value represents a central estimate of the range of values for meeting the Kyoto targets in 2010 in the EU based on estimates by Capros and Mantzos (2000). They report a value of €5 per tonne of CO2 avoided for reaching the Kyoto targets for the EU, assuming a full trade flexibility scheme involving all regions of the world. For the case that no trading of CO2 emissions with countries outside the EU is permitted, they calculate a value of €38 per tonne of CO2 avoided. It is assumed that measures for a reduction in CO2 emissions are taken in a cost effective way. This implies that reduction targets are not set per sector, but that the cheapest measures are implemented, no matter in which sector.

Looking further into the future, more stringent reductions than the Kyoto aims are assumed to be necessary to reach sustainability. Based on a reduction target of 50% in 2030 compared to 1990, INFRAS/IWW (2000) use avoidance costs of € 135 per t of CO2; however one could argue that this reduction target has not yet been accepted.

A valuation based on the damage cost approach, as e.g. presented by ExternE (Friedrich and Bickel 2001), would result in substantially lower costs. Due to the enormous uncertainties involved in the estimation process, such values have to be used very cautiously.

For those country, where emissions of methane (CH4) and nitrous oxide (N2O) were available, the shadow value for CO2 was multiplied by the global warming potential of 21 and 310 respectively, leading to values of 420 €/t CH4 and 6 200 €/t N2O.

### Noise

Noise costs were quantified for a number of health impacts calculated with new exposure-response functions, plus amenity losses estimated by hedonic pricing.

The methodology for quantifying noise costs was extended to the calculation of physical impacts. Costs for the following endpoints were quantified:

- Myocardial infarction (fatal, non-fatal)

- Angina pectoris

- Hypertension

- Subjective sleep quality

In addition, the willingness-to-pay for avoiding amenity losses were quantified based on hedonic pricing studies. A large number of such studies have been conducted, giving NSDI values (Noise Sensitivity Depreciation Index – the value of the percentage change in the logarithm of house price arising from a unit increase in noise) ranging from 0.08% to 2.22% for road traffic noise. Soguel (1994) conducted a hedonic pricing study in the town of Neuchatel in Switzerland. Rather than using housing prices, the dependent variable was monthly rent, net of charges. The coefficient on the noise variable in this study suggested a NSDI of 0.9. This value is similar to the average derived from European studies and was taken for our calculations.

The following table presents the monetary values used for valuing the health effects. The values for the single countries are given in the Annex.

Table 39
Valuation of health effects (factor costs, European average) from noise exposure
- € million at 1998 prices -

| Endpoint | Value | Unit |
| --- | --- | --- |
| Myocardial infarction (fatal, 7 YOLL) | 522 900 | € per case |
| Myocardial infarction (non-fatal, 8 days in hospital, 24 days at home) | 22 600 | € per case |
| Angina pectoris (severe, non-fatal, 5 days in hospital, 15 days at home) | 14 160 | € per case |
| Hypertension (hospital treatment, 6 days in hospital, 12 days at home) | 3 960 | € per case |
| Medical costs due to sleep disturbance (per year) | 197 | € per year |
| YOLL = Year of life lost. Source: IER calculations based on Metroeconomica (2001) and Nellthorp et al. (2001). |  |  |

As railway noise is perceived as less annoying than road noise, a bonus of 5 dB(A) was applied. This is in line with noise regulations in a number of European countries (e.g. Switzerland, France, Denmark, Germany; see INFRAS/IWW 2000).

For the quantification of the WTP for avoiding amenity losses a threshold value of 55 dB(A) was applied. It is assumed that noise levels equal to and over this value cause disamenity. The average rents which were the basis for the calculations are given in the Annex.

### Methodology for 1996 and for the forecast to 2005

Concerning environmental costs, the quantifiable differences between the account years 1996 and 1998 are quite small. Firstly, the activities (vehicle mileage, number of starts and landings of aircraft) and emission factors do not change considerably within two years. Secondly, the actual changes are difficult to detect, as much of the required data is not available in sufficient detail. It has to be born in mind that the estimated changes from 1996 to 1998 are comparably rough and thus have to be interpreted with caution. This is even more the case for the forecast to the year 2005, as the estimation of future developments is even more uncertain.

## Methodology for estimating taxes, charges and subsidies

### General issues

The estimation of revenues from the transport sector is based as far as possible on direct data: rough estimations and arbitrary allocations are avoided.

It seems necessary to make some general methodological remarks:

- Following the Italian Pilot accounts approach, we have not made an exhaustive list of all taxes, charges and subsidies paid by/to the transport sector, but only considered those taxes and charges paid by infrastructure users (individual passengers as well as transport operators) in some way specific to the transport sector, as opposed to an item of general taxation or when the rate applied is significantly different from the standard one.

- To stick to UNITE purposes, all revenues are considered disregarding their origin, since irrespective of who pays and who receives, all these financial flows represent transport actors contribution to cost coverage.

- According to the information collected both from the Ministry of Finance and the Ministry of Transport, the revenues coming from the different actors in the transport sector (end user, service providers, infrastructure providers) to the state (government, local entities) are an item of general taxation and are not specifically earmarked for use within the transport sector. The only exception is the noise emission tax for aircrafts, whose yield is destined to activities aimed at reducing noise level and damages. A revenue is therefore considered as directly allocatable to a given cost category when it is partly or totally used to cover that cost. If it is not the case, revenues are treated as transport specific, but not specifically allocated.

- VAT is only included when information on net VAT was available. When it was not possible to give a net accounting, possible exemptions are described in the text but not quantified.

- The treatment of subsidies within this account is certainly not exhaustive, since a complete investigation of all direct and indirect subsidies provided by the government at each level (state, regions, provinces, municipalities) is out of scope. For the sake of consistence with the other national accounts, we adopt the definition of direct and indirect subsidies in Macario et al. (2000), reported in the box below. Subsidies for infrastructure provision are accounted for in the cost side for each mode of transport as far as data were available (this is to avoid double counting of this item, since the methodology used for capital value estimation in the infrastructure costs account – the perpetual inventory method - considers all investments irrespective of the source).

Direct subsidies can be defined as a government expenditure that is directly paid to the economic subject in question without any market-based return-service of the recipient. It decreases the cost of providing a specific good or service and thus supports the production sale or purchase of a good or service. Indirect subsidies are all governmental interventions and regulations that favour selected economic agents by reducing their costs or by guaranteeing purchases of their products. To these belong tax subsidies, price reducing subsidies, purchase subsidies, regulatory subsidies and guarantees.

- Revenues from service tariffs are included in the accounts only for those modes for which the matching cost category (supplier operating costs) is calculated, namely rail transport and public transport.

- All data are expressed at factor costs and 1998 prices; the adjustment parameters are taken from Nellthorp et al. (2001).

### Methodological issues per mode

a) Road transport

Revenues from road transport in 1996 and 1998 are published by ACI and Ministry of Transport (CNT). It was not possible to split revenues yielded by vehicle tax, registration tax and fuel tax between private road users and public transport in order to isolate the contribution of road public transport. Consequently this amount is entirely allocated to road and not repeated in public transport accounts.

Concerning revenues allocation to vehicle types, the allocation of vehicle taxes and insurance taxes was done according to the number of vehicles. The allocation of registration taxes was made through national statistics on vehicles registrations for the years 1996 and 1998, provided by ACI; these data do not disaggregate between light and heavy goods vehicles, therefore the results for goods vehicles are presented as aggregated.

It does not seem appropriate to allocate registration tax to network types, since this levy is not related to infrastructure usage.

The amount of revenues coming from motorways charging refers only to the part of the network managed by Società Autostrade. For this revenue item it was not possible to obtain information on the monetary contribution or on mileages detailed by vehicles category. Therefore we disaggregated the total amount of motorways and tunnel tolls through the split of mileages provided by ANPA (Unità Censimento Fonti di Emissione) for the whole motorways network (including both those privately owned and those public), assuming unvaried the contribution of each vehicle category to the total of mileages.

Fuel tax and VAT on fuel consumption are allocated to vehicle and network types through total fuel consumption per vehicle category, provided by ANPA (Unità Censimento Fonti di Emissione) for the years 1996 and 1998.

b) Rail transport – Italian National Railways (FS)

The State contributions to the rail sector involve both the infrastructure side and the service provision side. Concerning infrastructure, public financing for new investments is already considered at the cost side.

On the service provision side, compensation payments for reduced tariffs are included (the State integrates the loss of yield of the rail operator due to transport services with a social aim, that do not pay back their cost).

Other contributions such as transfer payments for debt service payment and Pension Fund deficit coverage are added as additional information.

The split of tariff revenues in passenger and freight transport services was available from Ferrovie dello Stato (2000b).

c) Non-FS railways

Data on non-FS railways revenues for 1996 and 1998 are directly available from Ministry of Transport statistics (CNT), and were expressed at 1998 prices.

d) Public transport

Vehicle taxes, registration taxes and fuel taxes are included in road account.

The set of data available for assessing public transport revenues refers to 1995, and are published by CISPEL. CISPEL also provides a parameter of tariffs and other services revenues per passenger, which was applied to 1996 and 1998 number of passengers to estimate the amount of revenues for the same years.

The amount of subsidies received by the public transport sector was not available for 1996 and 1998: the data we present in 1996 accounts in par. 4.6.4 were obtained by adjusting 1995 amount to the 1998 constant prices.

e) Aviation

In order to be consistent with the air infrastructure account and to allow a proper assessment of air sector costs and revenues, only revenue data related to airports with an annual traffic of more than 1 000 yearly passengers are taken into account. The most recent set of data available from ENAC dates back to 1996: therefore both data for 1998 and for 2005 were estimated.

Landing and take-off charges for 1998 estimation is based on 1996 data: a parameter of €/movement was calculated for 1996, applied to the number of movements in 1998 and inflation adjusted. This estimation is made without considering any change in the level of the unitary charge and probably leads to an underestimation.

By the same way landing and take-off charges increase for night activities were estimated assuming constant the weight of night activity earnings on the total landing and take-off charges and applying this percentage to 1998 amount.

The amount of passenger boarding rights & freight loading/unloading taxes for 1998 was calculated applying the 1998 unitary rate for passenger to the number of passengers departing from Italian airports (source ISTAT). The price applied to passengers flying in national and international flights is differentiated between national flights (3.06 €) and international flights (7.83 €).

Concerning freight loading/unloading taxes, 1998 data is a rough estimate based on 1996 data: the amount of taxes per ton 1996 is applied to freight carried in 1998.

Revenues from handling and ticket emission revenues were estimated as well applying the parameter ‘revenues 1996/passenger-km’ to the passenger-km carried in 1998.

The actual nature of revenues items is not always clear: for instance take-off and landing charges for General Aviation aircrafts are doubled respect to commercial aviation aircrafts, in the same operating conditions. According to UNITE definition of tax and charge (Macario et al., 2000), this fee is to be considered more as a tax, since the service provided is in this case not proportionally remunerated. This does not apply to the increase in fares applied for night flights, since this kind of operation requires additional services (for instance lighting).

All revenues generated by the use of airport infrastructure are allocated to infrastructure costs, in particular airport charges; all revenues from noise emission charge are allocated to environmental costs.

It was not possible to estimate 2005 revenues because of lack of forecasting on air traffic performance.

f) Maritime and inland waterways transport

There is a shortage of statistics concerning maritime and inland waterways transport related revenues: the only set of data available is contained in a survey by CER (1996), and refers to the year 1994. Updated values were not available, nor data on port charges. It was not possible to produce robust extrapolations of 1996 and 1998 revenues, therefore the paragraph 4.6.6 will limit to show 1994 values at 1998 prices.

## Forecasting methodology

The 2005 Pilot Accounts are calculated on the basis of the prognosis made by the MURE model. MURE has three main components:

- a qualitative database of measures undertaken by the 15 Member States of the EU to promote energy conservation in four end-use sectors: Households, Transport, Industry and Tertiary (the services sector). Measures may be legislative, normative, fiscal and financial, but also information campaigns, energy audits, etc.

- a quantitative database of energy related statistics covering the 15 EU Countries and desegregated by end-use sector

- a simulation tool to carry out calculations of energy savings and emissions reduction potentials in each of the four sectors.

Based on the disaggregated data stored in the quantitative database, MURE National – Transport generates the energy demand by mode and fuel and provides information on energy savings and avoided pollutant emissions for any given RUE scenario. It also provides information on traffic volumes. This tool was used to forecast fuel consumption and passenger-km to the year 2005.

### MURE National - Transport scenario description

The 2005 scenario was built by making a series of assumptions on transport performance and consumption indicators trend, starting from a reference scenario (1996). These assumptions are based on:

- official international forecast database (AUTOIL II);

- own forecasts, based on Eurostat or national time series through Volterra equations (see paragraph 3.1.2).

The tables below show the basic data used for the reference scenario description (1996) and the main assumptions made to build the 2005 scenario. A further column describes the references take as a basis for the forecasting.

Passenger Private Transport

|  | Reference year 1996 | Forecast year 2005 | Source of information for forecasting scenario |
| --- | --- | --- | --- |
| Cars stock growth rates (respect to previous year, %) | 1.40 | 1.40 | Calculated on the basis of AUTOIL II growth rates |
| Average specific consumption – new cars (L/100 km) |  |  |  |
| Petrol | 7.77 | 7.00 | (cfr above) |
| Diesel | 6.64 | 5.20 | (cfr above) |
| LPG | 9.24 | 9.24 | (cfr above) |
| CNG | 8.70 | 8.70 | (cfr above) |
| Electricity | 47.12 | 47.12 | (cfr above) |
| Other | 6.64 | 6.64 | (cfr above) |
| Average distance travelled (km) |  |  |  |
| Petrol | 9 962 | 10 885 | (cfr above) |
| Diesel | 20 351 | 22 238 | (cfr above) |
| LPG | 12 000 | 12 000 | (cfr above) |
| CNG | 12 000 | 12 000 | (cfr above) |
| Electricity | : | : | (cfr above) |
| Other | : | : | (cfr above) |
| Car registration split by fuel (%) |  |  |  |
| Petrol | 84 | 60 |  |
| Diesel | 16 | 40 |  |

Passenger Collective Transport

|  | Reference year 1996 | Forecast year 2005 | Source of information for forecasting scenario |
| --- | --- | --- | --- |
| Urban mobility growth rates (respect to previous year, %) | 0.71 (1997 respect to 1996) | 0.80 | Calculated on the basis of AUTOIL II growth rates |
| Non urban mobility growth rates (yearly, %) | 0.44 | 1.97 | Own forecasts based on EUROSTAT time series (Volterra equations) |
| Specific consumption diesel (L/100 km) | 41.70 | 40.50 | Calculated on the basis of AUTOIL II growth rates |
| Modal split - Urban transport (%) |  |  |  |
| Bus | 66.31 | 65.20 | Own forecasts based on EUROSTAT time series (Volterra equations) |
| Tram and metro | 33.69 | 34.80 | (cfr. above) |
| Modal split – Non urban transport (%) |  |  |  |
| Coach | 52.80 | 49.43 | (cfr. above) |
| Train | 32.10 | 28.24 | (cfr. above) |
| Airplane | 13.09 | 19.95 | (cfr. above) |
| Ship | 2.01 | 2.38 | (cfr. above) |
| Urban bus – traffic share by fuel (%) |  |  |  |
| Diesel | 100 | 98 | (cfr. above) |
| LPG | 0 | 0 | (cfr. above) |
| CNG | 0 | 2 | (cfr. above) |
| Electricity | 0 | 0 | (cfr. above) |
| Biofuel | 0 | 0 | (cfr. above) |
| Hybrid | 0 | 0 | (cfr. above) |
| Other | 0 | 0 | (cfr. above) |
| Coach – traffic share by fuel (%) |  |  |  |
| Diesel | 100 | 100 | Own assumption |
| Train – traffic share by fuel (%) |  |  |  |
| Diesel | 25.80 | 25.80 | Own assumption |
| Electric | 74.20 | 74.20 | Own assumption |

Goods Transport

|  | Reference year 1996 | Forecast year 2005 | Source of information for forecasting scenario |
| --- | --- | --- | --- |
| Traffic growth rates (yearly, %) | 1.67 | 1.67 | Own estimation, based on the General Transport Plan (Ministero dei Trasporti e della Navigazione, 2001) |
| Modal split – Non urban transport (%) |  |  |  |
| Truck | 41.84 | 42.5 | Own assumption |
| Trailer | 31.03 | 33 | Own assumption |
| Train | 10.30 | 11 | Own assumption |
| Airplane | 0 | 0 | Own assumption |
| Ship | 16.83 | 13.50 | Own assumption |

### Logistic Substitution Models

The logistic substitution model considers separately each variable (e.g.: populations of passenger cars, busses, coaches, passenger miles on cars, aircraft, trains, etc.) and determines for each the most probable equation governing the development process. The model is agile, can be applied rapidly and easily expanded.

The model is based on the application of Volterra's equations. These were derived to model the development of biological populations and are represented by S shaped logistic curves. They depict accurately the mutual influences of two or more species competing in the same habitat for the same type of food. These equations describe accurately also the growth and the decline of populations of human products and artefacts. When historical data (time series) are available, the model determines the equation of the process and permits then to compute future developments.

If we call   x  the number of units belonging to a population, the equation is:

dx/dt =  k x (N - x)

where the derivative is made with respect to time t. The solution is:

x = N/(1 + exp(A t + B))

where  N  is the asymptote or final constant measure of the population.

The LOGI5000 software used fits an equation to a historical time series and measures the standard error of the measured data with respect to the equation. The fit can be considered adequate when the standard error is less than .01. When the standard error is much larger, we cannot affirm that the process can be described by means of  a Volterra equation.

Volterra equations were used with great success in the past to describe development of a variable to fill an available ecological niche, but in some cases the fit is not satisfactory due to the presence of noise in the data.

The software used - and the mathematics on which it is founded - also allows giving a judgement on the quality of the data used. Consequently the model provides also an indication of the credibility of the forecasts and projections it produces.

The model was applied to data published in Eurostat (2001). It was satisfactorily checked that standard error and other parameters used to establish the unicity and credibility of the curves, were within adequate limits.

# Results

## Infrastructure costs

The aim of this section is to present capital costs of infrastructure and the operating costs for infrastructure for all modes of transport. This is possible for road, rail, tram-metro, air and inland transport, where figures can be collected from annual reports. These figures were the input data for the Perpetual Inventory Model (PIM). The infrastructure asset value for the different modes is stated for years 1996 and 1998.

The results for 2005 are calculated on the basis of the General Transport Plan and are reported under paragraph 3.1 (Table 31).

### Road transport.

In 1998 the Italian road network had a gross capital value of € 144.35 billions and a net value of € 84.37 billions, with total capital costs of € 7.25 billions. The running costs had an € 6.4 billions value.

Table 40
Capital value, capital costs and running costs for road transport in Italy 1996-1998
– € million 1998 –

|  | All roads 1) | Motorways | State roads 1) | Provincial 1) | Local 1) |
| --- | --- | --- | --- | --- | --- |
|  | 1996 |  |  |  |  |
| Gross capital value | 143 981 | 35 107 | 52 070 | 23 523 | 17 179 |
| Net capital value | 83 879 | 16 912 | 29 345 | 11 498 | 10 021 |
| Capital costs | 7 321 | 1 807 | 2 865 | 1 213 | 953 |
| Running costs | 6 565 | : | : | : | : |
| Total infrastructure costs | 13 886 | : | : | : | : |
|  | 1998 |  |  |  |  |
| Gross capital value | 144 354 | 33 761 | 52 887 | 23 103 | 18 427 |
| Net capital value | 83 369 | 15 905 | 29 115 | 11 323 | 10 850 |
| Capital costs | 7 250 | 1 706 | 2 861 | 1 189 | 1 008 |
| Running costs | 6 395 | : | : | : | : |
| Total infrastructure costs | 13 645 | : | : | : | : |
| 1) Including land value. Source: DIW, ISIS elaboration on Ministry of Transport, ISTAT data. |  |  |  |  |  |

### Rail transport

The gross value of capital stock of the network rail amounted in 1998 to € 36.05 billions and the net value was € 19.88 billions. The capital costs were calculated with the perpetual inventory approach (PIM) and amounted to € 2.55 billions. Total infrastructure costs of rail network amounted in 1998 to € 5.61 billions. The running costs of rail network were estimated to be about  € 3.06 billions in 1998.

Table 41
Capital value, capital costs and running costs for rail, tram-metro, inland waterways and air transport in Italy 1996 and 1998
- € million at 1998 prices -

|  | Rail | Public Transport (tram, metro) | Air | Inland Waterways |
| --- | --- | --- | --- | --- |
|  | 1996 |  |  |  |
| Gross capital value | 35 079 | 561 | 6241 | 506 |
| Net capital value | 19 797 | 254 | 3606 | 239 |
| Capital costs | 2 548 | 39 | 429 | 32 |
| Running costs | 3 093 | : | 254 | 4 |
| Total infrastructure costs | 5 641 | 39 | 683 | 36 |
|  | 1998 |  |  |  |
| Gross capital value | 36 045 | 501 | 6226 | 478 |
| Net capital value | 19 877 | 221 | 3653 | 215 |
| Capital costs | 2 549 | 34 | 427 | 29 |
| Running costs | 3 056 | : | 144 | 29 |
| Total infrastructure costs | 5 605 | 34 | 571 | 58 |
| Source: DIW, ISIS elaboration on Ministry of Transport, ISTAT data. |  |  |  |  |

### Public Transport (tram and metro)

Metro service, although it is 20% of the total network extension, accounts for 70% of the total service supply. For this mode, only capital stock was calculated, whose amount was € 0.56 billions in 1996 (gross value) and € 0.25 billions (net value), which leads to a capital costs value of  € 0.04 billion. Several hurdles, already pointed out, have not allowed us to properly quantify and evaluate all the running costs for tramway and metro infrastructures.

### Inland waterways

Data on network extension is based upon the operating length, whose main source is the UNII (Union of Italian Inland Waterways). These infrastructures are gathered in Northern Italy in a sparse and limited way though.

Inland waterway transport is not taken as an alternative mode of transport, mainly for lack of infrastructures. Furthermore, several river ports are not equipped enough and have no infrastructure links to road and rail networks, since poor investments for the network construction and maintenance have been carried out. The CNT suggest this a major factor hindering the economic development of this mode (CNT 2000).

In order to create a time series long enough, only data on passengers transport were taken into account. The inland waterway capital stocks amounted in 1998 to € 478 million (gross value) and € 215 million (net value), which implied a capital cost amount of € 29 million. Total running costs reach  € 29 million.

Figure  3
Capital value, capital costs and running costs for inland waterways transport in Italy 1996 and 1998

Source: DIW, ISIS calculations on ISTAT, Ministry of transport data

### Aviation

The air transport capital stock amounted in 1998 to € 6.23 billions (gross value) and € 3.65 billions (net value), which implied a capital cost of € 0.43 billions. Total running costs reach  € 0.14 billions. Total infrastructure costs for aviation in Italy in 1998 were therefore € 571 million. Figure 4 compares infrastructure costs in 1996 and 1998.

Figure  4
Capital value, capital costs and running cost for air transport in Italy 1996 and 1998

Source: DIW, ISIS calculations on ISTAT, Ministry of transport data

### Infrastructure running costs

The summary table below shows running costs of all of the mode of transports for the year 1998. It includes running costs values for harbour infrastructure. Moreover, running costs are disaggregated into:

- maintenance costs;

- infrastructure operator costs;

- administrative costs (including training and R&D costs) of which:

      - control (port and airport authorities);

      - traffic police;

      - traffic control.

Expenditure for transport infrastructure investments in Italy is characterised by a substantial decrease – in both absolute terms and GDP terms – whereas traffic is steadily growing, thus leading to possible network saturation. Moreover, this expenditure mainly focuses on the road mode, followed by rail whereas harbour and airport infrastructures are granted a very low share of investments – close to nil for inland waterways. In terms of the absolute value of infrastructure length, and its deployment throughout the peninsula, Italy thus performs well. When translated into per capita values, (i.e. availability of infrastructure units/km per inhabitants) the performance is much poorer anyway (Ministero dei Trasporti e della Navigazione, 2001).

Table 42
Infrastructure running costs in Italy 1998
- € million at 1998 prices -

|  | Road | Rail | Tram-Metro | Inland waterways | Maritime | Air |
| --- | --- | --- | --- | --- | --- | --- |
| RUNNING COSTS | 6 395 | 3 056 | : | 29 | 369 | 144 |
| Maintenance Costs | 527 | 0 | : | 28 | 16 | 37 |
| Infrastructure operator costs | 492 | 275 | : | 0 | 100 | 41 |
| Administrative costs | 5 376 | 2 781 | : | 1 | 253 | 66 |
| of which: |  |  | : |  |  |  |
| Control (port and airport authorities) | 0 | 0 | : | 0 | 40 | 0 |
| Traffic police | 5 252 | 0 | : | 1 | 0 | 0 |
| Traffic control | 0 | 0 | : | 0 | 0 | 66 |
| Source: CNT 2001 |  |  |  |  |  |  |

## Supplier operating costs

The operating costs for both rail and public transport services suppliers for the years 1996 and 1998 are showed below. An estimation of supplier operating costs in 2005 is also provided.

### National and non national rail sector

Table 43 and 44 show the operating costs of the rail sector. In 1996 the national rail carrier (FS) costs of the operations amounted to 7 049 € million: more than a half (52.8%) of these costs are due to personnel costs (wages), and in 1998 the share undergoes only a slight decrease (51.7%). The total rail sector, including therefore also licensed railways (non national rail carriers), borne in 1996 a total cost of 7 611 € million for passenger and freight transport, while in 1998 the total cost decreased to 6 673 € million.

Table 43
Supplier operating costs for Italian rail sector 1996
- € million at 1998 prices -

| Cost category | National rail | Non national rail | Total Italian rail |
| --- | --- | --- | --- |
| Material + fuel + other material costs | 394 | 31 | 426 |
| Services | 698 | 56 | 753 |
| Maintenance costs | 150 | 12 | 162 |
| Wage costs | 3 722 | 297 | 4 019 |
| Change stocks | -65 | -5 | -70 |
| Depreciation | 1 196 | 95 | 1 291 |
| Other | 954 | 76 | 1 030 |
| Total | 7 049 | 562 | 7 611 |
| Source: ISIS calculations on FS and CNT data |  |  |  |

Table 44
Supplier operating cost for Italian rail sector 1998 
- € million at 1998 prices -

| Cost category | National Rail | Non national rail | Total Italian rail |
| --- | --- | --- | --- |
| Material + fuel + other material costs | 430 | 45 | 475 |
| Services | 683 | 71 | 754 |
| Maintenance costs | 142 | 15 | 157 |
| Wage costs | 3 126 | 326 | 3 452 |
| Change stocks | 14 | 1 | 15 |
| Depreciation | 1 169 | 122 | 1 291 |
| Other | 480 | 50 | 530 |
| Total | 6 043 | 630 | 6 673 |
| Source: ISIS calculations on FS and CNT data |  |  |  |

A forecast to 2005 of rail sector supplier operating costs is provided without disaggregation in cost items, since 2005 cost structure compared to 1998 one will be greatly affected by the organisational changes that FS - Ferrovie dello Stato underwent during 2000 (see par. 2.1.2). A cautious estimate is provided on the basis of 1996 and 1998 data and on the methodology described in the paragraph 3.2; the amount results to be 6 777 € million for national rail carriers and 706 € million for licensed railways, that result in 7 483 € million as a total supplier operating costs for 2005.

### Public Transport

Public transport sector operating costs for the years 1996 and 1998 are showed in Table 45 and 46. The total costs for 1996 amounted to 6 271 € million, of which a 90.8% is due to busses and trolley buses, and a 9.2% to rail borne public transport (namely, 5.6% to tram and 3.6% to underground transport). In 1998 the weight of buses and trolley related costs on the total is slightly higher, 91.6%, tram related costs are 5.3% and metro related costs are 3.2%.

Table 45
Supplier Operating Costs- Public Transport 1996 
- € million at 1998 prices -

| Cost Category | Busses & trolley bus | Tram | Metro | Total Public Transport |
| --- | --- | --- | --- | --- |
| Material + fuel + other material costs | 742 | 47 | 30 | 820 |
| Services | 412 | 26 | 17 | 455 |
| Maintenance costs | 148 | 9 | 6 | 164 |
| Wage costs | 3 929 | 249 | 161 | 4 339 |
| Change stocks | - | - | - | - |
| Depreciation | 302 | 17 | 11 | 330 |
| Other | 163 | - | - | 163 |
| Total | 5 697 | 349 | 225 | 6 271 |
| Source: ISIS calculations on CNT and CISPEL data |  |  |  |  |

Table 46
Supplier Operating Costs for Public Transport 1998 
- € million at 1998 prices -

| Cost category | Busses and Trolley bus | Tram | Metro | Total Public Transport |
| --- | --- | --- | --- | --- |
| Material + fuel + other material costs | 653 | 38 | 23 | 713 |
| Services | 385 | 22 | 13 | 421 |
| Maintenance costs | 138 | 8 | 5 | 151 |
| Wage costs | 3 656 | 210 | 127 | 3 992 |
| Change stocks | - | - | - | - |
| Depreciation | 283 | 16 | 10 | 309 |
| Other |  | - | - | - |
| Total | 5 115 | 294 | 177 | 5 586 |
| Source: ISIS calculations on CNT and CISPEL data |  |  |  |  |

Table 47 shows the public transport supplier operating costs forecast for 2005, whose total amount is 6 264 € million.

Table 47
Supplier Operating Costs for Public Transport 2005
- € million at 1998 prices -

| Cost category | Busses and Trolley bus | Tram | Metro | Total Public Transport |
| --- | --- | --- | --- | --- |
| Material + fuel + other material costs | 732 | 42 | 25 | 799 |
| Services | 432 | 25 | 15 | 472 |
| Maintenance costs | 155 | 9 | 5 | 169 |
| Wage costs | 4 099 | 236 | 142 | 4 477 |
| Change stocks | - | - | - | - |
| Depreciation | 317 | 18 | 11 | 347 |
| Other | - | - | - | - |
| Total | 5 736 | 330 | 198 | 6 264 |
| Source: ISIS calculations on CNT and CISPEL data |  |  |  |  |

## Delay costs due to congestion

Delay costs due to congestion were not estimated within this account, see par. 2.2.3.

## Accident costs

### Results for 1998- total costs by category and main cost bearer

Table 48 shows the total internal and external accident costs for Italy by mode in 1998. Total social costs of accidents (internal and external) amount to € 23,9 billion. The external part accounts for 17% of social costs (€ 4,1 billion).

With reference to the external costs, the most significant cost component is the estimated production losses (80% of the total), followed by health and medical costs (€ 450 million, corresponding to 11% of the total) and administrative costs (0.9% of the total).

On the other side, the risk value represents the most significant item within internal costs, which accounts for 70% of total internal costs.

Table 48
Total internal and external accident costs in Italy – 1998
- € million at 1998 prices -

|  | Internal costs |  | External costs |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Material damages | Risk value | Administrative costs | Health costs | Production loss | Total costs 1998 | Total external costs |
| Road 1) | 6 179.65 | 13 497.71 | 379.18 | 449.50 | 3 316.32 | 23 822.36 | 4 145.00 |
| Rail |  | 104.44 | 0.01 | 0.77 | 9.60 | 114.82 | 10.38 |
| Aviation |  | 23.85 |  | 0.10 | 2.13 | 26.08 | 2.23 |
| Inland waterway |  | 5.12 |  | 0.03 | 0.46 | 5.60 | 0.49 |
| Maritime shipping |  |  |  |  |  |  |  |
| Total | 6 179.7 | 13 631.1 | 379.2 | 450.4 | 3 328.5 | 23 968.9 | 4158.1 |
| 1) Passenger cars, motorcycles, goods vehicles, buses and coaches. Source: ISIS calculations on ANIA, CNT, FS, ISTAT data |  |  |  |  |  |  |  |

By transport mode road is largely prevailing, accounting for 99% of costs, whether external or total.

### Allocation of total costs to modes and types of infrastructure

The methodology adopted for the allocation of external accidents costs by mode and type of infrastructure would require detailed information in order to assess the responsibility to be assigned to transport users.  Since such data are not available, the allocation procedure is based on the following assumptions:

- the allocation of accident costs by transport mode relies on the costs borne by the transport mode by means of the number of accidents and related costs which it generates;

- in order to allocate costs by vehicle type (road), the methodology considers as key variable the traffic volumes, i.e. vehicle-km;

- in aviation, only aggregated calculations were possible,

- in rail transport passengers and on-board staff killed or injured are allocated to passenger transport, while accident occurring to other staff are allocated to all types of service by train-km. The distinction between high-speed passenger and conventional passenger services has not been possible.

The tables 49 and 50 show the results of this cost allocation procedure for road transport and other transport modes.

Table 49
Total external costs in Italy by vehicle type – road transport
- € million at 1998 prices -

| Road accidents | Motorways | Other roads | Urban roads | All roads |
| --- | --- | --- | --- | --- |
| Private vehicles 1) | 300 | 2 129 | 1 357 | 3 786 |
| Bus and coaches 2) | 0 | 15 | 15 | 29 |
| LGV 3) | 28 | 120 | 55 | 203 |
| HGV 4) | 18 | 84 | 25 | 127 |
| 1) Passenger cars, motorcycles, mopeds - 2) Cost allocation by vehicle kilometres. - 3) Cost allocation with coaches, vans, agricultural vehicles by vehicle kilometres. 4) Cost allocation between vehicle types via vehicle kilometres. Source: ISIS calculations on ISTAT data |  |  |  |  |

Table 50
Total external costs in other transport modes
- € million at 1998 prices -

|  | All network |
| --- | --- |
| Rail transport | 10 |
| Passenger traffic | 8 |
| Freight traffic | 2 |
| Aviation 1 | 2 |
| Passenger traffic | : |
| Freight traffic | : |
| Inland navigation 1) | 0.5 |
| Maritime shipping | : |
| 1) Aviation and Inland waterways: no criteria available for the allocation of external costs to freight and passengers Source: ISIS calculations on ISTAT, CNT data |  |

### Average costs in 1998

Average costs have been calculated for vehicle-km (road), train-km, and vessel-km. Data expressed in aircraft-km are not available. The results of the average cost estimates are presented in Table 51 and 52.

Table 51
Average accident costs in Italy 1998 – road transport
- €/1000 v-km -

| Road accidents | Motorways | Other roads | Total network |
| --- | --- | --- | --- |
| Private vehicles 1) | 23.02 | 64.45 | 56.40 |
| Bus and coaches | 0.00 | 77.01 | 45.98 |
| LGV 2) | 26.19 | 40.45 | 37.60 |
| HGV 3) | 5.99 | 32.75 | 20.19 |
| 1) Cars, station wagons, motorcycles - 2) Rigid and articulated goods vehicles with a gross weight > 3,5t. - 3) Goods vehicles / vans <3.5 t, agricultural vehicles and other use vehicles. Source: ISTAT, CNT |  |  |  |

Table 52
Average accident costs in Italy 1998 – other transport modes
- €/1000 v-km -

|  | Unit | Total network |
| --- | --- | --- |
| Rail transport | € / 1000 train-km | 326.42 |
| Aviation | € / 1000 aircraft-km | : |
| Inland waterways | € / 1000 vessel-km | 867.52 |
| Source: ISTAT |  |  |

### Results for 1996

Compared to 1996, the total accident costs in 1998 decreased by 31.7%. Such pronounced decreasing trend was mainly due to the reduction of injuries and fatalities in the road sector (-7%, compared with the corresponding figure for 1998).

Table 53
Total accident costs in 1996
- € million at 1998 prices -

|  | Internal costs |  |  | External costs |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Material damages | Risk value | Production loss | Administrative costs | Health costs | Total costs | Total costs 1996 | Relative to 1998 (%) |
| Road 1) | 5 097.58 | 9 407.90 | 2 714.25 | 297.31 | 566.08 | 3 577.64 | 18 083.13 | -31.7 |
| Rail |  | 138.06 | 10.51 | 0.01 | 1.44 | 11.97 | 150.03 | 23.5 |
| Aviation |  | 23.78 | 1.63 | 0.001 | 0.09 | 1.73 | 25.51 | -2.2 |
| Inland waterway |  | 10.76 | 0.78 |  | 0.08 | 0.86 | 11.62 | 51.8 |
| Maritime shipping |  |  |  |  |  |  |  |  |
| Total | 5 097.6 | 9 580.5 | 2 727.2 | 297.3 | 567.7 | 3 592.2 | 18 270.3 | -31.2 |
| 1) Passenger cars, motorcycles, goods vehicles, buses and coaches. Source: ISIS calculations on ANIA, CNT, FS, ISTAT data |  |  |  |  |  |  |  |  |

By transport mode, it can be observed that both rail and inland waterways show an opposite trend, with increasing rates, i.e. respectively +23.5% and + 51.8%. However, it should be considered that in 1996 the number of reported accidents in rail and inland waterways could be influenced by random events.

Table 54 and table 55 show the estimated average values per type of vehicle (road transport) and for the other transport modes in 1996.

Table 54
Average accident costs in Italy 1996 – road transport
- €/1000 v-km -

| Road accidents | Motorways | Other roads | Total network | Relative to 1998 (%) |
| --- | --- | --- | --- | --- |
| Private vehicles 1) | 12.60 | 51.88 | 44.60 | -26.5 |
| Bus and coaches | 0.00 | 62.01 | 36.83 | -24.8 |
| LGV 2) | 19.31 | 38.43 | 34.60 | -8.65 |
| HGV 3) | 3.77 | 24.66 | 15.24 | -32.4 |
| 1) Cars, station wagons, motorcycles - 2) Rigid and articulated goods vehicles with a gross weight > 3,5t. - 3) Goods vehicles / vans <3,5t, agricultural vehicles and other use vehicles. Source: ISIS calculations on ISTAT data |  |  |  |  |

Table 55
Average accident costs in Italy 1996 – other transport modes 
- €/1000 v-km -

|  | Unit | Total network | Relative to 1998 (%) |
| --- | --- | --- | --- |
| Rail transport | € / 1000 train-km | 410.36 | 20.5 |
| Aviation | € / 1000 aircraft-km | : | : |
| Inland waterways | € / 1000 vessel-km | 1 886.90 | 54.0 |
| Source: ISIS calculations on ISTAT data |  |  |  |

### Results for 2005

The estimation of total accident costs in Italy in 2005 is presented in the following table.

Table 56
Total accident costs in Italy 2005
- € million at 1998 prices -

|  | Internal costs |  | External costs |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Material damages | Risk value | Production loss | Administrative costs | Health costs | Total costs | Total costs 2005 | Relative to 1998 (%) |
| Road 1) | 8 309.37 | 9 288.90 | 2 969.57 | 299.52 | 89.82 | 3 358.90 | 20 957.17 | -13.7 |
| Rail |  | 140.06 | 11.67 | 0.02 | 1.12 | 12.80 | 152.86 | 24.9 |
| Aviation |  | 24.14 | 1.99 | : | 0.17 | 2.17 | 26.31 | 0.9 |
| Inland waterway |  | 8.06 | 0.65 |  | 0.04 | 0.69 | 8.75 | 36.0 |
| Maritime shipping |  |  |  |  |  |  |  |  |
| Total | 8 309.4 | 9 461.2 | 2 983.9 | 299.5 | 91.1 | 3 374.6 | 21 145.1 | -13.4 |
| 1) Passenger cars, motorcycles, goods vehicles, buses and coaches Source: ISIS calculations on ISTAT, CNT data |  |  |  |  |  |  |  |  |

Compared to the 1998 accident accounts, the overall decreasing rate is about -13.4%, in particular for road (-13.7%). A growing rate concerns instead, rail (+24.9%) and inland water (+36%).

Table 57
Average accident costs in Italy 2005 – road transport
- €/1000 v-km -

| Road accidents | Motorways | Other roads | Total network | Relative to 1998 |
| --- | --- | --- | --- | --- |
| Private vehicles 1) | 23.39 | 42.60 | 38.87 | -31.09 |
| Bus and coaches | 0.00 | 31.60 | 18.87 | -58.96 |
| LGV 2) | 28.7 | 27.95 | 28.11 | -25.22 |
| HGV 3) | 6.13 | 21.10 | 14.07 | -30.29 |
| 1) Cars, station wagons, motorcycles. - 2) Rigid and articulated goods vehicles with a gross weight > 3,5t. - 3) Goods vehicles / vans <3,5t, agricultural vehicles and other use vehicles. Source: ISTAT, CNT |  |  |  |  |

With reference to road the average costs per v-km show a marked decreasing trend (-31%, private cars), due both to the reduction in the number of accidents rates and a general improvement of safety.

Table 58
Average accident costs in Italy 2005 – other transport modes
- €/1000 v-km -

|  | Unit | Total network | Relative to 1998 |
| --- | --- | --- | --- |
| Rail transport | € / 1000 train-km | 379.90 | 20.37 |
| Aviation | € / 1000 aircraft-km | : | : |
| Inland waterways | € / 1000 vessel-km | 1 323.32 | 52.54 |
| Source: ISIS elaboration on ISTAT data |  |  |  |

With reference to the other transport modes, in the case of rail and inland navigation, the growth in volumes is mainly responsible for the increasing trend in the average costs.

## Environmental Costs

### Results for 1998

Table 59 presents the environmental costs of transport in Italy for the year 1998. The highest share of costs, 56.4%, stems from the emission of air pollutants, followed by noise with 24.1%. Global warming is responsible for 19.6% of total costs,  the only external noise cost assessed is the health impact due mainly to ischaemic heart disease and hypertension and the subjective impairment of sleep quality. Nature and Landscape costs were not estimated (see par. 2.2.5.4.)

The sector responsible of the highest environmental costs is road transport, reflecting its dominating role in transport performance. Road transport is responsible for 92.3% of the total transport sector costs. Air pollution is the most important cost category, for both passenger and freight transport. Costs are dominated by impacts due to primary and secondary particles, above all loss of life expectancy and increased morbidity rates. . Noise, the second important cost category, is dominated by amenity losses. Further cost components here are health impacts due to ischaemic heart disease and hypertension and the subjective impairment of sleep quality.

Table 59
Environmental costs for Italy 1998
- € million at 1998 prices -

|  | Air Pollution | Global Warming | Noise | Total |
| --- | --- | --- | --- | --- |
| Road | 7 228.5 | 2 323.7 | 2 784.0 | 12 336.2 |
| Passenger Transport | 4 760.4 | 1 599.7 | 1 911.7 | 8 271.7 |
| Freight Transport | 2 468.2 | 724.0 | 872.3 | 4 064.4 |
| Rail | 144.6 | 61.3 | 242.7 | 448.6 |
| Passenger transport | 126.3 | 50.8 | 168.4 | 345.5 |
| Freight Transport | 18.3 | 10.5 | 74.3 | 103.1 |
| Public Transport | 93.9 | 18.2 | : | 112.1 |
| Urban Bus | 89.4 | 15.6 | : | 105.0 |
| Tram & Trolley bus | 1.3 | 0.8 | : | 2.1 |
| Metro and other | 3.1 | 1.8 | : | 4.9 |
| Aviation1) | 76.7 | 197.0 | 193.7 | 467.4 |
| Airports | 31.8 | 46.0 | : | 77.7 |
| Flights | 44.9 | 151.0 | : | 196.0 |
| Inland Waterways1) | 4.9 | 1.3 | : | 6.2 |
| Maritime Shipping | : | : | : |  |
| Total | 7 548.6 | 2 601.4 | 3220.4 | 13 370.5 |
| Note: 1) The lack of data about PM10 emissions causes underestimation of air pollution costs. Source: IER, ISIS. |  |  |  |  |

Noise costs of rail transport are the most significant cost category. Costs due to air pollution and global warming are comparably (in percentage respect to road transport mode) low due to a high share of electric traction which leads to much lower emissions of air pollutants and CO2 than traction based on fossil fuels.

The costs of public transport are mainly due to diesel buses. The costs of aviation are dominated by global warming. The category “flights” covers the costs due to emissions of CO2 and indirect emissions of air pollutants (due to fuel production) based on the civil aviation fuel taken in Italy. For technical reasons CO2 emissions at airports are included in this category. “Airports” contains costs of pollutant emissions (except CO2) during the Landing and Take-off (LTO) cycles at Italian airports.

The costs of inland waterway transport are extremely low and stem mainly from air pollution. Noise costs are virtually negligible, as it can be assumed that the threshold of 55 dB(A) is hardly exceeded and thus population exposure is not significant.

In general, it has to be noted that the costs given in Table 59 are only the costs which are currently quantifiable. The lack of some cost figure is due to the fact that for some modes or cost categories no appropriate data was available (e.g. population exposure due to public transport noise); in addition, there are effects for which currently no consistent monetary values exist (e.g. costs of ecosystem impairment due to nitrogen deposition).

Table 60
Environmental costs road transport Italy 1998 - Disaggregation by vehicle type
- € million at 1998 prices -

|  | Air Pollution | Global Warming | Noise | Total |
| --- | --- | --- | --- | --- |
| Moped | 507.5 | 50.2 | 747.7 | 1305.4 |
| Motorcycles | 92.2 | 39.1 | 429.1 | 560.4 |
| Passenger Cars (petrol) | 3 193.4 | 1 147.4 | 598.3 | 4939.1 |
| Passenger Cars (diesel) | 812.5 | 312.5 | 86.9 | 1211.9 |
| Buses | 154.9 | 50.5 | 49.6 | 255.0 |
| Light Goods Vehicles | 562.6 | 178.8 | 413.6 | 1155.0 |
| Heavy Goods Vehicles | 1 905.6 | 545.2 | 458.7 | 2909.5 |
| Total | 7 228.6 | 2 339.2 | 2 784.0 | 12 336.2 |
| Source: IER, ISIS. |  |  |  |  |

Table 60 shows the environmental costs of road transport for different types of vehicles.  Passenger cars cause the highest total costs, followed by heavy goods vehicles, light goods vehicles mopeds and light goods vehicles. The noise cost split by vehicle categories has to be cautiously interpreted, because it represents only a very rough estimation; this uncertainty is due to the fact that noise exposure estimates and vehicle mileages stem from different sources, and the procedure of splitting the total costs does not necessarily represent the vehicle categories’ true share in causing noise exposure.

Table 61
Environmental costs road transport Italy 1998
– Disaggregation by vehicle and road type - € million at 1998 prices -

|  | All roads1) |  | Motorways2) | Other Extra-Urban roads2) | Urban Roads2) |
| --- | --- | --- | --- | --- | --- |
| Moped | 1 305.4 | 0.0 |  | 162.7 | 395.0 |
| Motorcycles | 560.4 | 8.4 |  | 36.5 | 86.4 |
| Passenger Cars (petrol) | 4 939.1 | 794.2 |  | 1 564.3 | 1 982.3 |
| Passenger Cars (diesel) | 1 211.9 | 329.9 |  | 398.4 | 396.6 |
| Buses | 205.5 | 86.7 |  | 62,4 | 56,2 |
| Light Goods Vehicles | 1 155.0 | 112.1 |  | 264.5 | 364.8 |
| Heavy Goods Vehicles | 2 909.5 | 1 020.1 |  | 810.4 | 620.2 |
| Total | 12 336.2 | 2 351.5 |  | 3 299.3 | 3 901.5 |
| Note: 1) Includes air pollution, global warming, noise costs. - 2) Noise costs are not included, in order to avoid arbitrary cost allocation. Source: IER, ISIS. |  |  |  |  |  |

In table 61 the costs are split per vehicle type and road type. Noise costs have been included only in “all roads” category in order to avoid arbitrary cost allocations.

The average costs per vehicle km are given in Table 62. The costs were given per vehicle category only, because the different vehicle types are too different to be aggregated. Because the values are presented in vehicle kilometres, vehicles with a high capacity (ships, trains) show a higher value than vehicles with a low capacity (LGV, HGV). Noise costs are not included in order to avoid an arbitrary cost allocation.

Table 62
Average environmental costs for Italy 1998 
- €/1000 v-km -

|  | Air Pollution | Global Warming | Noise | Total |
| --- | --- | --- | --- | --- |
| Road |  |  |  |  |
| Moped | 18.1 | 1.8 | : | 19.9 |
| Motorcycles | 4.9 | 2.1 | : | 7.0 |
| Passenger Cars (Petrol) | 12.2 | 4.4 | : | 16.5 |
| Passenger Cars (Diesel) | 10.7 | 4.1 | : | 14.8 |
| Buses | 66.4 | 18.0 | : | 84.4 |
| Light Goods Vehicles | 18.1 | 5.8 | : | 23.9 |
| Heavy Goods Vehicles | 52.9 | 15.1 | : | 68.0 |
| Rail |  |  |  |  |
| Passenger Transport (electric traction) | 257.0 | 170.0 | : | 427.0 |
| Passenger Transport ( Diesel traction ) | 894.6 | 165.0 | : | 1 059.6 |
| Freight Transport (electric traction) | 239.0 | 158.0 | : | 397.0 |
| Freight Transport ( Diesel traction ) | 894.6 | 165.0 | : | 1 059.6 |
| Public Transport |  |  |  |  |
| Urban Buses (Diesel) | 125.3 | 21.9 | : | 147.2 |
| Tram & Trolley bus | 39.1 | 22.9 | : | 62.0 |
| Metro and other | 37.7 | 22.1 | : | 59.8 |
| Aviation | : | : | : | : |
| Inland Waterways | : | : | 0 | : |
| Maritime Shipping | : | : | 0 | : |
| Source: IER, ISIS. |  |  |  |  |

### Results for 1996 and 2005

Table 63 shows the costs for the 1996 account. Changes compared to the account year 1998 are small, as the impact parameters for environmental costs only changed little within the two years. For road and rail transport, costs were similar to those of 1998, reflecting lower mileage and higher emission factors. For public transport and aviation, costs were lower in 1996 than in 1998, reflecting a slower activity. Costs due to inland waterways transport remain constant.

Table 63
Environmental costs for Italy 1996 
- € million at 1998 prices -

|  | Air Pollution | Global Warming | Noise | Total |
| --- | --- | --- | --- | --- |
| Road | 7 469.6 | 2 107.8 | 2.647,6 | 12 718.5 |
| Passenger Transport | 5 019.0 | 1 458.1 | 1.818,0 | 8 628.9 |
| Freight Transport | 2 450.7 | 649.7 | 829,6 | 4 089.6 |
| Rail | 141.9 | 62.0 | 230,8 | 445.1 |
| Passenger Transport | 121.8 | 50.3 | 160,1 | 341.2 |
| Freight Transport | 20.0 | 11.7 | 70,7 | 104.0 |
| Public Transport | 91.0 | 17.0 | : | 108.0 |
| Urban Bus | 86.5 | 14.5 | : | 101.0 |
| Tram & Trolley bus | 1.4 | 0.8 | : | 2.2 |
| Metro and other | 3.1 | 1.7 | : | 4.8 |
| Aviation1) | 69.5 | 181.7 | 184.2 | 448.3 |
| Airports | 26.7 | 38.1 | : | 68.2 |
| Flights | 42.7 | 143.6 | : | 196.0 |
| Inland Waterways1) | 4.7 | 1.3 | 0 | 6.3 |
| Maritime Shipping | : | : | 0 | : |
| Total | 7 772.2 | 2 369.0 | 2 912.5 | 13 576,3 |
| Note: 1) The lack of data about PM10 emissions causes underestimation of air pollution costs. Source: IER, ISIS. |  |  |  |  |

The results for 2005 are presented in table 64. Total costs are not comparable with 1996 and 1998 costs because of lack of estimations about some transport sector’s emissions. The only mode comparable is rail transport, which shows a considerable decrease of air pollution costs. This decrease is linked to the better performances in electricity production’s emissions.

Table 64
Environmental Costs for Italy 2005
- € million at 1998 prices -

|  | Air Pollution | Global Warming | Noise | Total |
| --- | --- | --- | --- | --- |
| Road | 4 465. 5 | 2 820. 1 | : | 7 285. 7 |
| Passenger Transport1) | 2 482. 0 | 1 932. 5 | : | 4 414. 5 |
| Freight Transport2) | 1 983. 5 | 887. 6 | : | 2 871. 1 |
| Rail | 123. 9 | 81. 2 | : | 205. 2 |
| Passenger transport | 108. 4 | 62. 8 | : | 171. 2 |
| Freight Transport | 15. 5 | 18. 4 | : | 34. 0 |
| Public Transport | 112. 8 | 21. 0 | : | 133. 8 |
| Urban Bus | 110. 5 | 17. 6 | : | 128. 1 |
| Tram & Metro | 2. 3 | 3. 4 | : | 5. 7 |
| Aviation | : | : | : | : |
| Airports | : | : | : | : |
| Flights | : | : | : | : |
| Inland Waterways | : | : | 0 | : |
| Maritime Shipping | : | : | 0 | : |
| Total | 4 702. 3 | 2 922. 3 | : | 7 624 6 |
| Note: 1) Mopeds and motorcycles are not included. - 2) Light goods vehicles are not included Source: IER, ISIS |  |  |  |  |

## Taxes, charges, subsidies

This section reports on the transport related taxes and charges that can be compared with the related costs. Furthermore, as far as the available data did allow, subsidies were quantified.

### Road transport

Table 65 quantifies the amount of revenues generated by the road sector in the years 1996 and 1998. revenues for 2005 are not estimated.

Table 65
Road Transport Revenues in Italy 1996, 1998 and 2005
- € million at 1998 prices -

|  | All roads | Motorways | Other non urban roads | Urban roads |
| --- | --- | --- | --- | --- |
|  | - 1996- |  |  |  |
| Mopeds, Motorcycles | 1 519.40 | - | - | - |
| Passenger Cars | 21 666.73 | - | - | - |
| Buses 1) | 745.84 | - | - | - |
| Light Goods Vehicles 2) | 7 001.51 | - | - | - |
| Heavy Goods Vehicles 3) |  | - | - | - |
| Total | 30 933.48 | - | - | - |
| Structure of Revenues: | 30 933.48 | - | - | - |
| Annual Registration Tax | 826.69 | - | - | - |
| Annual Vehicle Tax | 2 676.48 | 563.59 | 1 297.73 | 815.16 |
| Fuel Tax | 16 503.20 | 3 961.73 | 6 280.51 | 6 260.96 |
| Insurance Taxes | 1 099.54 | : | : | : |
| Other Taxes 4) | 945.29 | : | : | : |
| Motorway and Tunnel tolls | 2 066.62 | 2 066.62 | - | - |
| Additional Information VAT on fuel tax | 6 298.59 | 1 550.17 | 2 408.56 | 2 339.86 |
| VAT on motorway and tunnel tolls | 517.08 | 517.08 | - | - |
|  | - 1998 - |  |  |  |
| Mopeds, Motorcycles | 1 820.98 | - | - | - |
| Passenger Cars | 25 143.72 | - | - | - |
| Buses 1) | 814.25 | - | - | - |
| Light Goods Vehicles 2) | 8 405.39 | - | - | - |
| Heavy Goods Vehicles 3) |  | - | - | - |
| Total | 36 184.33 | - | - | - |
| Structure of Revenues: | 36 184.33 | - | - | - |
| Annual Registration Tax | 865.15 | - | - | - |
| Annual Vehicle Tax | 3 324.57 | 724.24 | 1 631.05 | 969.29 |
| Fuel Tax | 21 993.69 | 5 449.10 | 8 500.09 | 8 044.50 |
| Insurance Taxes | 934.32 | : | : | : |
| Other Taxes 4) | : | : | : | : |
| Motorway and Tunnel tolls | 2 221.67 | 2 221.67 | - | - |
| Additional Information VAT on fuel tax | 6 406.89 | 1 614.29 | 2 486.93 | 2 305.68 |
| VAT on motorway and tunnel tolls | 438.05 | 438.05 | - | - |
| *) See par. 3.6.2 for allocation methods. 1) This includes urban buses and coaches. 2) Up to 3,5 t GVW. 3) Over 3,5 t max GVW. 4) Includes taxes on driving licenses and rights for MCTC technical operations. Sources: ACI, ANPA, Ministry of Transport, ISIS calculations |  |  |  |  |

### Rail transport – FS

Revenues from taxes and charges levied on rail sector in the years 1996, 1998 and 2005 are shown in Table 66.

Table 66
Revenues from taxes and charges in rail transport – FS Spa
- € million at 1998 prices -

| Type of revenue/ type of transport | Tariff revenues 1) | Tariff revenues (PSC)2) | Charge on infrastructure access |
| --- | --- | --- | --- |
| – 1996 – |  |  |  |
| Passenger transport | 1 420.77 | 1 503.25 | - |
| Freight transport | 577.65 | - | - |
| Total | 1 998.41 | 1 503.25 | -3) |
| – 1998 – |  |  |  |
| Passenger transport | 1 359.77 | 1 517.30 | - |
| Freight transport | 564.54 | - | - |
| Total | 1 924.31 | 1 517.30 | -3) |
| – 2005 – |  |  |  |
| Passenger transport | 1423.69 | : | : |
| Freight transport | 735.51 | : | : |
| Total2) | 2159.20 | : | : 3) |
| 1) Excluding subsidies and VAT. 2) Public Service Contract, see par. 2.2.6. 3) Infrastructure access charges are relevant only for 2005, but no estimation was possible. Sources: ISIS elaborations on FS data |  |  |  |

Table 68 summarises the subsidies paid to the national rail carrier (FS) in the years 1996 and 1998. We did not estimate subsidies for 2005.

Table 67
Subsidies for FS 1996 and 1998
- € million at 1998 prices -

| Items | 1996 | 1998 |
| --- | --- | --- |
| Core data |  |  |
| Compensation payments for reduced tariffs | 1 828.31 | 1 700.02 |
| Total core data | 1 828.31 | 1 700.02 |
| Additional information |  |  |
| Transfer payments for debt service payment (ministry of Treasure) | 1 723.73 | - |
| Extraordinary contributions | 92.14 | 15.70 |
| Pension Fund deficit coverage (ministry of Treasure) | 855.77 | 1 724.00 |
| Total additional information | 2 671.63 | 1 739.69 |
| Total | 4 499.94 | 3 439.71 |
| Sources: ISIS elaborations on FS data |  |  |

### Non-national railways

Data on non-national rail carriers are available at a more aggregated level from the Ministry of Transport (CNT). Table 69 contains revenues from passenger and freight transport operations for the years 1996 and 1998. No forecast was possible for non-national railways revenues.

Table 68
Tariff revenues and subsidies of non-national railways
- € million at 1998 prices -

| Type of revenue | 1996 | 1998 |
| --- | --- | --- |
| Revenues from traffic |  |  |
| Tariff revenues (charges on transport services) | 82.03 | 100.90 |
| Subsidies |  |  |
| State contributions to running expenditures | 619.33 | : |
| Local administrations direct financing | : | 0.13 |
| Local administrations contributions | : | 0.17 |
| Total subsidies | 619.33 | 0.30 |
| Total | 701.36 | 101.20 |
| Sources: ISIS elaborations on Ministry of Transport data |  |  |

### Public transport

The calculation of revenues for public transport is affected by the lack of recent published data. It is worth keeping in mend that the estimation of tariff revenues for 1996 and 1998 was made on the basis of 1995 data assuming constant the level of tariffs and according to the trend of the number of passengers (for further details and for the treatment of subsidies, see par. 3.6.2).

Table 69
Tariff revenues and subsidies of public transport
- € million at 1998 prices -

| Type of revenue | 1996 | 1998 |
| --- | --- | --- |
| Tariff and other services revenues 1) | 1 310.00 | 1 382.55 |
| State contributions to running expenditures | 2 069.202) | : |
| Other contributions | 203.652) | : |
| Total | 3 582.85 | 1 382.55 |
| 1) Includes bus, tram and metro. – 2) This data refers to 1995, expressed at 1998 prices. Sources: ISIS elaborations on CISPEL, CNT data |  |  |

### Aviation

The revenues taken into consideration for air transport accounts refer to aircraft-related taxes (due by airlines), airport charges and air traffic control charges. Data for 1996 were directly available from ENAC, while ISIS estimated data for 1998, where possible, on the basis of 1996 data and of traffic performance.

Table 70
Taxes and charges of air transport in Italy 1996 and 1998
- € million at 1998 prices -

| Type of revenue | 1996 | 1998 | 2005 |
| --- | --- | --- | --- |
| Taxes |  |  |  |
| Aircrafts registration taxes | 0.091) |  | n.a. |
| Aircrafts insurance taxes | 6.991) | 12.412) | n.a |
| Noise tax for aircrafts | 5.74 |  | n.a |
| Total taxes | 12.82 |  | n.a |
| Airport charges |  |  | n.a |
| Landing and take-off charges | 108.93 | 129.79 | n.a |
| Landing and take-off charges increase for night activities (+50%) | 2.56 | 3.17 | n.a |
| Passenger boarding & freight loading/unloading rights | 136.62 | 153.65 | n.a |
| Revenues from handling | 437.09 | 498.35 | n.a |
| Ticket emission revenues | 8.52 | 9.71 | n.a |
| General aviation activities rights doubling | 0.55 | : | n.a |
| Air traffic control charges |  |  | n.a |
| Route charges | 73.62 | 200.00 | n.a |
| Terminal charges | 49.74 |  | n.a |
| Total charges | 817.62 |  | n.a |
| Non transport related revenues (revenues from commercial activities) | 287.70 | : | n.a |
| TOTAL REVENUES | 1 118.15 |  | n.a |
| 1) These data refer to 1994 2) This data refers to 1997 Sources: ISIS elaboration on ENAC, Eurocontrol data |  |  |  |

### Maritime and inland waterways transport

Table 72 shows the available statistics on maritime and inland waterways transport related revenues. As previously mentioned (see par. 3.6.2), the only set of data available refers to the year 1994. It was not possible to produce robust extrapolations of 1996 and 1998 revenues, therefore Table 72 shows 1994 values at 1998 prices.

Table 71
Taxes of maritime transport sector in Italy 1994
- € million at 1998 prices -

| Type of revenue | 1994 |
| --- | --- |
| Vessel related taxes (inscription and others) | 6.87 |
| Insurance tax for vessel and freight | 110.58 |
| Freight loading and unloading charges | n.a. |
| Channel crossing charges | 6.98 |
| VAT (net for maritime transport companies) | 8.14 |
| VAT on maritime transport services paid by end users | 113.95 |
| Total | 203.49 |
| Source: CER (1996) |  |

# Summary of results for Italy

In order to provide a clear overview of the transport situation in Italy, basic social and economic indicators are presented before the detailed results of the Italian pilot accounts are discussed.

## Road transport

In Table 73 and following tables up to Table 78 the total costs of road transport documented within the Italian pilot account are presented.

In 1998, the core year of the pilot accounts, the largest cost block was accident costs. Total accident costs amounted to € 19 677 million, out of these € 13 497 million were risk value accident costs. Infrastructure costs were the second largest cost block (€ 13 645 million). As concerns about congestion costs, which in the UNITE accounts refer to costs of delay (e.g. time and fuel costs) were not estimated within this account because of lack of data (please refer to par. 2.2.3 for more explanations). For 2005, we have forecast cost increases for infrastructure up to some € 2 011 million (as already reported in the General Transport Plan 2001 released by the Ministry of Transport). External accident costs will increase by € 3 358 million in 2005, whereas internal ones will amount to € 17 598.3 million, of which € 9 288.9 will relate to the risk value. On the revenue side we have estimated road transport-related revenues amounting to some € 2 221.7 million in 1998, directly related to charges for motorways and tunnels usage. A further disaggregation into fixed and variable revenues was not feasible as well as forecasts for year 2005. Other relevant transport specific revenues are those drawn by the annual registration tax (€ 865.2 million) and the fuel tax (€ 21 993.7 million), which was the most substantial source of revenue together with VAT on fuel tax (€ 6 406.9 million).

Table 72
Italian road account for 1996, 1998 and 2005 
- € million at 1998 prices -

| Costs |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Core information | 1996 | 1998 |  | 2005 |  |
| Infrastructure Costs | 13 886 | 13 645 |  | 1) |  |
| Fixed | : | : |  | : |  |
| Variable | : | : |  | : |  |
| Accident costs (user external)2) 3) | 3 577.6 | 4 145.0 |  | 3 358.9 |  |
| Environmental costs | 12 225.0 | 12 336.2 |  | 7285.74) |  |
| Air pollution | 7 469.6 | 7 228.5 |  | 4465.54) |  |
| Global warming | 2 107.8 | 2 323.7 |  | 2820.14) |  |
| Noise | 2 647.6 | 2 784.0 |  | : |  |
| Total | 29 688.6 | 30 126.2 |  | : |  |
| Additional information |  |  |  |  |  |
| Congestion costs5) | : | : |  | : |  |
| Time costs | : | : |  | : |  |
| Fuel costs | : | : |  | : |  |
| Accident costs (user internal)6) | 14 505.5 | 19 677.4 |  | 17 598.3 |  |
| From this: risk value | 9 407.9 | 13 497.7 |  | 9 288.9 |  |
| Environmental costs | : | : |  | : |  |
| Nature and landscape, soil and water pollution7) | : | : |  | : |  |
| Nuclear risk7) | - | - |  | - |  |
| Revenues |  |  |  |  |  |
| Directly related to a specific cost category |  |  |  |  |  |
| Charges for motorways and tunnels usage | 2 066.6 | 2 221.7 |  | : |  |
| Fixed | : | : |  | : |  |
| Variable | : | : |  | : |  |
| Total | 2 066.6 | 2 221.7 |  | : |  |
| Other transport specific revenues |  |  |  |  |  |
| Annual registration tax | 826.7 | 865.2 |  | : |  |
| Annual vehicle tax | 2 676.5 | 3 324.6 |  | : |  |
| Fuel tax | 16 503.2 | 21 993.7 |  | : |  |
| Insurance tax | 1 099.5 | 934.3 |  | : |  |
| Other taxes | 945.3 | : |  | : |  |
| VAT on fuel tax | 6 298.6 | 6 406.9 |  | : |  |
| VAT on motorways and tunnels usage | 517.1 | 438.1 |  | : |  |
| Total | 28 866.9 | 33 962.7 |  | : |  |
| Subsidies | : | : |  | : |  |
| 1) The General Transport Plan 2001 by the Ministry of Transport estimates the infrastructure capital cost to be 2 011 € millions in 2005; no forecasts for running costs available. - 2) Refers to the part of road accident costs not borne by road users and insurance companies, but by the public sector and third parties. - 3) Includes road public transport accident costs. – 4) Excluding mopeds and motorcycles - 5) No estimation for congestion costs was carried out, see Annex 1, par. 2.2.3. - 6) Refers to those parts of accident costs which are caused by and borne by road users and insurance companies. - 7) Because there is no standardised methodology for the calculation of these costs, the figures given here are to be regarded only as approximate indications that may change greatly over time with the development of a standard methodology. – Sources: ISIS. |  |  |  |  |  |

Comments on specific cost categories

Infrastructure costs

Infrastructure costs calculation followed the method proposed by Link et al. (2000), whose main working steps are the capital stock evaluation and the running costs estimation. For the purpose of total infrastructure costs assessment, and following the UNITE standards, basic data for capital costs estimation were expenditures for new investments and renewals, while running costs were calculated on the basis of expenditures for maintenance, operational costs and administrative costs. These data were provided by the Ministry of Transport. The capital costs evaluation is based upon the determination of the capital stock’s value, calculated through the PIM (Perpetual Inventory Method - a more detailed description is available in Link et al. (2000).

In 1998 the Italian road network had a gross capital value of € 144.35 billion and a net value of € 84.37 billion, with total capital costs of € 7.25 billion. The running costs had an € 6.4 billion value. To forecast data on 2005 we took into account the General Transport Plan for Italy by the Ministry of Transport, that provides data on infrastructure expenditure forecasts from 2000 to 2010, split by mode of transport and disaggregated by kind of intervention. Thus, road transport forecasts for year 2005 point out a value of  € 2 011 million

Congestion costs

Delay costs due to congestion were not estimated within the pilot accounts for Italy (see par. 2.2.3).

Accident costs

The methodology for the evaluation of social costs of accidents at 1998 follows as far as possible the recommendations of the Unite project for accident costs assessment contained in Doll et al. (2000), integrated with national contributions in the field The evaluation of external costs includes the estimation of costs not covered by transport users, i.e. medical treatments costs exceeding the contribution from the transport users for funding the National Health System. The evaluation of internal costs is based on cost borne by the users of transport sector, i.e. the material damage covered by insurance companies, without affecting non-transport users. As result, the internal costs of accidents include material damages and risk value, considered internalised by the transport users decisions, while the external costs involve medical costs, production losses and administrative costs.

The departure from the methodological framework for estimating accident costs as defined in Doll et al. (2000) consists in the costs allocation procedure. Due to the data characteristics for the Italian case, the matrix of cost bearers and responsibility, which can only be built on the basis of a detailed level of information, cannot be presented. Data source for the evaluation of costs of medical treatments and production losses was ISTAT, whereas CENSIS provided data on administrative costs and ANIA (the national association of insurance companies) supplied data to carry out an estimation of material damages.

Environmental costs

The input data for the calculations were provided by the country account leaders. These data were used for cost calculations based on the ExternE methodology:

- with the EcoSense computer model for airborne pollutants,

- with shadow values for greenhouse gas emissions, and

- with new exposure-response functions and monetary values for noise.

For further details on these calculation tools to estimate environmental costs (stemming from noise, global warming and air pollution, please refer to par. 3.5.1, 3.5.2 and 3.5.3).

The emissions factors of Italian road fleet were provided by National Environmental Protection Agency ANPA (2000): these factors were calculated through COPERT II model for the year 1997. Total emissions for 1998 were thus estimated on the basis of data on circulating fleet and total fuel consumption provided by ANPA (2001). COPERT classification of the rolling stock was calculated by ISIS for 1998, upgrading the existing one (1997) through own estimations based on the publications of ACI (various years, 1999, 2000). The emissions of SO2 were estimated through fuel consumption and sulphur content (50 ppm for diesel engine and 40 ppm for petrol engine). Emissions were furthermore differentiated by road type (motorways, urban roads, and other extra-urban roads) and by vehicle’s category (moped, motorcycles, passenger car, LGV, HGV and buses). PM10 emissions from petrol vehicles were not considered.

The sector responsible of the highest environmental costs in Italy in 1998 is road transport, thus reflecting its dominating role in transport performance. Road transport is responsible for 93.7% (€ 10 341.4 million in 1998) of the total transport sector costs. Air pollution is the most important cost category, for both passenger (€ 4 760.4 million) and freight transport (€ 2 468.2 million). Costs are dominated by impacts due to primary and secondary particles, above all loss of life expectancy and increased morbidity rates. Passenger cars cause the highest total costs, followed by heavy goods vehicles, light goods vehicles and mopeds. For more figures on average environmental costs in 1998 (per vehicle/km) for Italy split by vehicle type (for road transport and for the other means of transport, too), please check Table 82. For figures related to 2005 forecasts (only air pollution and global warming were calculated) check Table 84 instead.

Taxes, charges and subsidies

Revenues from road transport in 1996 and 1998 are published. It was not possible to split revenues yielded by vehicle tax, registration tax and fuel tax between private road users and public transport in order to isolate the contribution of road public transport. Consequently this amount is entirely allocated to road and not repeated in public transport accounts.

The amount of revenues coming from motorways charging refers only to the part of the network managed by Autostrade S.p.A.

Table 73
Average variable costs of road transport per vehicle km: Italy
- €/km at 1998 prices -

| All Roads |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1998 |  |  |  |  |  |  |
|  | Motor-cycles |  | Passenger cars |  | Buses | LGV | HGV |
| Core information |  |  |  |  |  |  |  |
| Infrastructure costs | : |  | : |  | : | : | : |
| Fixed | : |  | : |  | : | : | : |
| Variable | : |  | : |  | : | : | : |
| External accident costs1) | 0.009 |  |  |  | 0.008 | 0.005 | 0.003 |
| Environmental costs | 0.015 |  | 0.016 |  | 0.084 | 0.024 | 0,068 |
| Air pollution | 0.013 |  | 0.012 |  | 0.066 | 0.018 | 0,053 |
| Global warming | 0.002 |  | 0.004 |  | 0.018 | 0.006 | 0,015 |
| Noise2) | : |  | : |  | : | : | : |
| Total I | 0.039 |  | 0.041 |  | 0.176 | 0.053 | 0.139 |
| Additional information |  |  |  |  |  |  |  |
| Delay costs3) | : |  | : |  | : | : | : |
| Internal accident costs | 0.043 |  | 0.038 |  | 0.026 | 0.015 | 0.043 |
| Material damages | 0.014 |  | 0.012 |  | 0.008 | 0.005 | 0.014 |
| Risk value | 0.030 |  | 0.026 |  | 0.018 | 0.011 | 0.030 |
| Environmental costs | : |  | : |  | : | : | : |
| Nature, landscape, soil and water pollution4) | : |  | : |  | : | : | : |
| Total II | 0.086 |  | 0.076 |  | 0.052 | 0.031 | 0.086 |
| Revenues |  |  |  |  |  |  |  |
| Fixed |  |  |  |  |  |  |  |
| Annual registration tax | : |  | : |  | : | : | : |
| Annual vehicle tax | : |  | : |  | : | : | : |
| Variable |  |  |  |  |  |  |  |
| Fuel tax | : |  | : |  | : | : | : |
| Charges for motorways and tunnels usage | : |  | : |  | : | : | : |
| VAT on fuel tax | : |  | : |  | : | : | : |
| VAT on motorways and tunnels usage charges | : |  | : |  | : | : | : |
| Insurance taxes | : |  | : |  | : | : | : |
| Total | : |  | : |  | : | : | : |
| Basic data |  |  |  |  |  |  |  |
| Million vehicle km | 46 876 |  | 369 377 |  | 3 677 | 37 183 | 39 065 |
| Million passenger km | 64 000 | 643 600 |  | 90 900 |  | - | - |
| Million tonne km | - |  | - |  | - | 191 481.7 |  |
| 1) Both external and internal accident costs. – 2) No sound allocation methodologies were available for noise costs. - 3) No estimation for congestion costs was carried out, see Annex 1, par. 2.2.3. - 4) Because there is no standardised methodology for the calculation of these costs, the figures given here are to be regarded only as approximate indications that may change greatly over time with the development of a standard methodology. – Source: ISIS. |  |  |  |  |  |  |  |

Table 74
Total costs of road transport: Italy - € million at 1998 prices -

| All Roads |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1998 |  |  |  |  |  |  |
|  | Motor-cycles | Passenger cars |  | Buses | LGV | HGV | Total |
| Core information |  |  |  |  |  |  |  |
| Infrastructure costs1) | : | : |  | : | : | : | 13 645 |
| Fixed | : | : |  | : | : | : | : |
| Variable | : | : |  | : | : | : | : |
| External accident costs | 3 785.86 |  |  | 29.42 | 203.22 | 126.50 | 4 145.00 |
| Administrative | 346.33 |  |  | 2.69 | 18.59 | 11.57 | 379.18 |
| Health costs | 410.55 |  |  | 3.19 | 22.04 | 13.72 | 449.50 |
| Production loss | 3 028.98 |  |  | 23.54 | 162.59 | 101.21 | 3 316.32 |
| Environmental costs | 1 865.82) | 6 151.0 |  | 255.0 | 1 155.0 | 2 909.5 | 12 336.3 |
| Air pollution | 599.7 | 4 005.9 |  | 154.9 | 562.6 | 1 905.6 | 7 228.7 |
| Global warming | 89.3 | 1 459.9 |  | 50.5 | 178.8 | 545.2 | 2 323.7 |
| Noise | 1 176.8 | 685.2 |  | 49.6 | 413.6 | 458.7 | 2 783.9 |
| Total I | : |  |  | : | : | : | 30 126.3 |
| Additional information |  |  |  |  |  |  |  |
| Delay costs3) | : |  |  | : | : | : | : |
| Internal accident costs | 17 972.45 |  |  | 139.65 | 964.71 | 600.55 | 19 677.36 |
| Material damages | 5 644.23 |  |  | 43.86 | 302.97 | 188.60 | 6 179.66 |
| Risk value | 12 328.22 |  |  | 95.79 | 661.75 | 411.95 | 13 497.71 |
| Environmental | : | : |  | : | : | : | : |
| Nature, landscape, soil and water pollution4) | : | : |  | : | : | : | : |
| Total II | 17 972.45 |  |  | 139.65 | 964.71 | 600.55 | 19 677.36 |
| Revenues |  |  |  |  |  |  |  |
| Fixed |  |  |  |  |  |  |  |
| Annual registration tax | 75.98 | 727.34 |  | 1.29 | 60.55 |  | 865.15 |
| Annual vehicle tax | 402.61 | 2 614.10 |  | 114.53 | 37.74 | 155.60 | 3 324.57 |
| Variable |  |  |  |  |  |  |  |
| Fuel tax | 943.63 | 14 812.35 |  | 482.68 | 1 757.39 | 3 997.65 | 21 993.69 |
| Charges for motorways and tunnels usage | 19.45 | 1 668.85 |  | 30.56 | 153.35 | 349.46 | 2 221.67 |
| VAT on fuel tax | 262.32 | 4 257.38 |  | 146.99 | 523.20 | 1 217.00 | 6 406.89 |
| VAT on motorways and tunnels usage charges | 3.84 | 329.05 |  | 6.02 | 30.24 | 68.90 | 438.05 |
| Insurance taxes | 113.15 | 734.65 |  | 32.19 | 10.60 | 43.73 | 934.32 |
| Total | 1 820.98 | 25 143.72 |  | 814.26 | 8 405.41 |  | 36 184.34 |
| Basic data |  |  |  |  |  |  |  |
| Number of vehicles (thousand) | 5 105.17 | 33 147.67 |  | 1 452.28 | 478.5 | 1 973.03 | 42 156.64 |
| Million vehicle km | 46 876 | 369 377 |  | 3 677 | 37 183 | 39 065 | 493 208 |
| Million passenger km | 64 000 |  | 643 600 | 90 9005) | - | - | 798 500 |
| Million tonne km | - | - |  | - | 191 481.7 |  | 191 481.7 |
| 1) It was not possible to allocate infrastructure capital costs to vehicle types; an allocation of running variable costs is proposed in the account, see par. 4.1.6. – 2) Including mopeds. - 3) No estimation for congestion costs was carried out, see Annex 1, par. 2.2.3. – 4) Because there is no standardised methodology for the calculation of these costs, the figures given here are to be regarded only as approximate indications that may change greatly over time with the development of a standard methodology. – 5) Includes buses and coaches. Source: ISIS. |  |  |  |  |  |  |  |

Table 75
Total costs of road transport: Italy
- € million at 1998 prices -

| Motorways |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1998 |  |  |  |  |  |  |
|  | Motor-cycles | Passenger cars |  | Buses | LGV | HGV | Total |
| Core information |  |  |  |  |  |  |  |
| Infrastructure costs1) | : | : |  | : | : | : | 1 7062) |
| Fixed | : | : |  | : | : | : | : |
| Variable | : | : |  | : | : | : | : |
| External accident costs | 300.06 |  |  | 0 | 28.31 | 17.62 | 345.98 |
| Administrative | 27.45 |  |  | 0 | 2.59 | 1.61 | 31.65 |
| Health costs | 32.54 |  |  | 0 | 3.07 | 1.91 | 37.52 |
| Production loss | 240.07 |  |  | 0 | 22.65 | 14.1 | 276.81 |
| Environmental costs | 8.4 | 1 124.1 |  | 86.7 | 112.1 | 1 020.1 | 2 351.5 |
| Air pollution | 5.9 | 817.2 |  | 65.4 | 74.6 | 759.8 | 1 722.9 |
| Global warming | 2.4 | 306.9 |  | 21.3 | 37.5 | 260.3 | 628.5 |
| Noise3) | : | : |  | : | : | : | : |
| Total I | : |  | : | : | : | : | 4 403.38 |
| Additional information |  |  |  |  |  |  |  |
| Delay costs4) | : |  |  | : | : | : | : |
| Internal accident costs | 1 424.43 |  |  | 0.00 | 134.39 | 83.66 | 1 642.48 |
| Material damages | 447.34 |  |  | 0.00 | 42.20 | 26.27 | 515.82 |
| Risk value | 977.09 |  |  | 0.00 | 92.18 | 57.39 | 1 126.66 |
| Environmental costs | : | : |  | : | : | : | : |
| Nature, landscape, soil and water pollution5) | : | : |  | : | : | : | : |
| Total II | 1 424.43 |  |  | 0.00 | 134.39 | 83.66 | 1 642.48 |
| Revenues |  |  |  |  |  |  |  |
| Fixed |  |  |  |  |  |  |  |
| Annual registration tax 6) | : | : |  | : | : | : | : |
| Annual vehicle tax | 6.347) | 543.55 |  | 9.99 | 50.13 | 114.24 | 724.24 |
| Variable |  |  |  |  |  |  |  |
| Fuel tax | 25.83 | 2 989.72 |  | 155.93 | 368.82 | 1 908.79 | 5 449.10 |
| Charges for motorways and tunnels usage | 19.45 | 1 668.85 |  | 30.56 | 153.35 | 349.46 | 2 221.67 |
| VAT on fuel tax | 7.18 | 868.73 |  | 47.49 | 109.80 | 581.09 | 1 614.29 |
| VAT on motorways and tunnels usage charges | 3.84 | 329.05 |  | 6.02 | 30.24 | 68.90 | 438.05 |
| Insurance taxes 6) | : | : |  | : | : | : | : |
| Total | 56.3 | 5 856.35 |  | 240 | 662.21 | 2 908.24 | 10 477.35 |
| Basic data |  |  |  |  |  |  |  |
| Million vehicle km | 940 | 80 636 |  | 1 482 | 7 437 | 16 948 | 107 442 |
| 1) It was not possible to allocate infrastructure capital costs to vehicle types; an allocation of running variable costs is proposed in the account, see par. 4.1.6. - 2) It was not possible to split running infrastructure costs by network types; only the amount of capital infrastructure costs is shown. – 3) No sound allocation methodologies were available for noise costs. - 4) No estimation for congestion costs was carried out, see Annex 1, par. 2.2.3. – 5) Because there is no standardised methodology for the calculation of these costs, the figures given here are to be regarded only as approximate indications that may change greatly over time with the development of a standard methodology. – 6) No allocation to network types was possible; see Annex 1, par. 4.1.6. – 7) This data refers only to motorcycles. - Source: ISIS elaboration on various sources. |  |  |  |  |  |  |  |

Table 76
Total costs of road transport: Italy
- € million at 1998 prices -

| Other non urban roads 1) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1998 |  |  |  |  |  |
|  | Motor-cycles | Passenger cars | Buses | LGV | HGV | Total |
| Core information |  |  |  |  |  |  |
| Infrastructure costs2) | : | : | : | : | : | 5 0583) |
| Fixed | : | : | : | : | : | : |
| Variable | : | : | : | : | : | : |
| External accident costs | 2 128.96 |  | 14.85 | 120.25 | 84.34 | 2 348.41 |
| Administrative | 194.76 |  | 1.36 | 11.00 | 7.72 | 214.83 |
| Health costs | 230.87 |  | 1.61 | 13.04 | 9.15 | 254.67 |
| Production loss | 1 703.33 |  | 11.88 | 96.21 | 67.48 | 1 878.91 |
| Environmental costs | 199.3 | 1962.7 | 62.4 | 264.5 | 810.4 | 3 299.3 |
| Air pollution | 171.44) | 1 384.3 | 45.5 | 186.6 | 613.7 | 2 401.4 |
| Global warming | 27.94) | 578.4 | 16.9 | 77.9 | 196.7 | 897.8 |
| Noise5) | : | : | : | : | : | : |
| Total I | : | : | : | : | : | 10 705.71 |
| Additional information |  |  |  |  |  |  |
| Delay costs6) | : |  | : | : | : | : |
| Internal accident costs | 10 106.72 |  | 70.52 | 570.85 | 400.40 | 11 148.48 |
| Material damages | 3 174.00 |  | 22.15 | 179.27 | 125.74 | 3 501.17 |
| Risk value | 6 932.72 |  | 48.37 | 391.57 | 274.65 | 7 647.31 |
| Environmental costs | : | : | : | : | : | : |
| Nature, landscape, soil and water pollution7) | : | : | : | : | : | : |
| Total II | 10 106.72 |  | 70.52 | 570.85 | 400.40 | 11 148.48 |
| Revenues |  |  |  |  |  |  |
| Fixed |  |  |  |  |  |  |
| Annual registration tax 8) | : | : | : | : | : | : |
| Annual vehicle tax | 101.13 | 1 284.62 | 7.47 | 137.85 | 99.98 | 1 631.05 |
| Variable |  |  |  |  |  |  |
| Fuel tax | 293.77 | 5 874.21 | 123.69 | 765.82 | 1 442.60 | 8 500.09 |
| Charges for motorways and tunnels usage | - | - | - | - | - | - |
| VAT on fuel tax | 81.67 | 1 700.43 | 37.67 | 228 | 439.17 | 2 486.93 |
| VAT on motorways and tunnels usage charges | - | - | - | - | - | - |
| Insurance taxes 8) | : | : | : | : | : | : |
| Total | 476.57 | 8 859.26 | 168.83 | 1 131.67 | 1 981.75 | 12 618.08 |
| Basic data |  |  |  |  |  |  |
| Million vehicle km | 15 003 | 190 576 | 1 109 | 20 450 | 14 832 | 241 970 |
| 1) State, Provincial, Municipal extra-urban roads. - 2) It was not possible to allocate infrastructure capital costs to vehicle types; an allocation of running variable costs is proposed in the account, see par. 4.1.6. - 3) It was not possible to split running infrastructure costs by network types, only the amount of capital infrastructure costs is shown. – 4) Including mopeds. – 5) No sound allocation methodologies were available for noise costs. - 6) No estimation for congestion costs was carried out, see Annex 1, par. 2.2.3. – 7) Because there is no standardised methodology for the calculation of these costs, the figures given here are to be regarded only as approximate indications that may change greatly over time with the development of a standard methodology. – 8) No allocation to network types was possible, see Annex 1, par. 4.1.6 for further details. – Source: ISIS elaboration on various sources. |  |  |  |  |  |  |

Table 77
Total costs of road transport: Italy
- € million at 1998 prices -

| Urban roads |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1998 |  |  |  |  |  |  |
|  | Motor-cycles |  | Passenger cars | Buses | LGV | HGV | Total |
| Core information |  |  |  |  |  |  |  |
| Infrastructure costs 1) | : |  | : | : | : | : | : |
| Fixed | : |  | : | : | : | : | : |
| Variable | : |  | : | : | : | : | : |
| External accident costs | 1 356.85 |  |  | 14.56 | 54.66 | 24.54 | 1 450.61 |
| Administrative | 124.12 |  |  | 1.33 | 5.00 | 2.24 | 132.70 |
| Health costs | 147.14 |  |  | 1.58 | 5.93 | 2.66 | 157.31 |
| Production loss | 1 085.58 |  |  | 11.65 | 43.73 | 19.63 | 1 160.60 |
| Environmental costs | 481.42) |  | 2 378.9 | 56.1 | 364.8 | 620.2 | 3 901.4 |
| Air pollution | 422.4 |  | 1 804.4 | 43.9 | 301.4 | 532.1 | 3 104.2 |
| Global warming | 59.0 |  | 574.5 | 12.2 | 63.4 | 88.1 | 797.2 |
| Noise3) | : |  | : | : | : | : | : |
| Total I |  |  |  |  |  |  |  |
| Additional information |  |  |  |  |  |  |  |
| Delay costs 4) | : |  |  | : | : | : | : |
| Internal accident costs | 6 441.29 |  |  | 69.13 | 259.48 | 116.50 | 6 886.40 |
| Material damages | 2 022.88 |  |  | 21.71 | 81.49 | 36.59 | 2 162.67 |
| Risk value | 4 418.41 |  |  | 47.42 | 177.99 | 79.91 | 4 723.73 |
| Environmental costs | : |  | : | : | : | : | : |
| Nature, landscape, soil and water pollution5) | : |  | : | : | : | : | : |
| Total II | : | : |  | : | : | : | : |
| Revenues |  |  |  |  |  |  |  |
| Fixed |  |  |  |  |  |  |  |
| Annual registration tax 6) | : |  | : | : | : | : | : |
| Annual vehicle tax | 208.51 |  | 661.70 | 7.33 | 62.66 | 29.09 | 969.29 |
| Variable |  |  |  |  |  |  |  |
| Fuel tax | 624.03 |  | 5 948.42 | 203.05 | 622.74 | 646.26 | 8 044.50 |
| Charges for motorways and tunnels usage | - |  | - | - | - | - | - |
| VAT on fuel tax | 173.48 |  | 1 688.23 | 61.83 | 185.40 | 196.74 | 2 305.68 |
| VAT on motorways and tunnels usage charges | - |  | - | - | - | - | - |
| Insurance taxes 6) | : |  | : | : | : | : | : |
| Total | 1 006.02 |  | 8 298.35 | 272.21 | 870.8 | 872.09 | 11 319.47 |
| Basic data |  |  |  |  |  |  |  |
| Million vehicle km | 30 933 |  | 98 164 | 1 087 | 9 296 | 4 315 | 143 796 |
| 1) No estimation possible, see par. 2.2.1. – 2) Including mopeds. – 3) No sound allocation methodologies were available for noise costs. - 4) No estimation for congestion costs was carried out, see Annex 1, par. 2.2.3. – 5) Because there is no standardised methodology for the calculation of these costs, the figures given here are to be regarded only as approximate indications that may change greatly over time with the development of a standard methodology. – 6) No allocation to network types was possible, see Annex 1, par. 4.1.6. – Source: ISIS elaboration on various sources. |  |  |  |  |  |  |  |

## Rail transport – national rail carrier Ferrovie dello Stato S.p.A. – FS and other Italian railway companies

As highlighted in table 78, the largest cost blocks in the rail account for both FS (State-owned carrier) and the other Italian rail companies, are infrastructure and external accident costs which are in a different order of magnitude though (the former are € 5 605 million and the latter are € 10.38 million respectively). For 2005, both external and internal accident costs (risk value) are estimated to be those costs with the highest increases compared to 1998 (both categories show a growth by respectively € 12.80 million and some € 140.06 million for the latter, from € 104.44 million in 1998). Total rail transport-related revenues stemming from subsidies for concessionary fares amounted in 1998 to € 1 700.02 million. User tariffs (excluding both subsidies and VAT) amounted to € 1 924.31 million. Forecasts for this cost driver in 2005 envisage a growth up to € 2 159.20 million. Revenues from Public Service Contract (see par. 2.2.7) amounted to € 1 517.30 million in 1998 but no forecast was feasible for 2005.

As a whole, revenues from taxes and charges in rail transport were split (for the Italian Pilot Account) into Passenger Transport and Freight Transport, thanks to the fair amount of reliable data, mainly drawn from the CNT (Transport National Account). In particular, tariff revenues (excluding subsidies and VAT) amounted to € 1 359.77 million in 1998 for passenger transport and to € 564.54 million for freight one. On the other hand, total subsidies for FS rose in 1998 by € 3 439.71 million. In the Italian Pilot Account, moreover, a revenues calculation for tariffs revenues and subsidies for non-national railway companies was carried out (see par. 4.6.3). Non-national railway companies network is 18% of the whole Italian rail network extension (16 079.9 km in year 1998), of which 82% belongs to FS. The former are 26 overall and mainly operate in Southern Italy.

Table 78
Italian rail account 1996, 1998 and 2005 
– € million at 1998 prices –

| Costs |  |  |  |
| --- | --- | --- | --- |
| Core information | 1996 | 1998 | 2005 |
| Infrastructure Costs1) | 5 641 | 5 605 | : 2) |
| Fixed | : | : | : |
| Variable | : | : | : |
| Supplier operating costs | 7 611 | 6 673 | : 3) |
| Accident costs (external) | 11.97 | 10.38 | 12.80 |
| Environmental costs | 445.2 | 448.6 | 169.7 |
| Air pollution | 149.2 | 144.6 | 102.5 |
| Global warming | 65.2 | 61.3 | 67.2 |
| Noise4) | 230.8 | 242.7 | : |
| Total core social costs | 13 709.1 | 12 737.0 | : |
| Additional information |  |  |  |
| Congestion costs5) | : | : | : |
| Accident costs (internal) | : | : | : |
| From this: risk value | 138.06 | 104.44 | 140.06 |
| Environmental costs | : | : | : |
| Nature and landscape, soil and water pollution6) | : | : | : |
| Nuclear risk | - | - | - |
| Revenues |  |  |  |
| Directly related to Supplier Operating Costs |  |  |  |
| Subsidies for concessionary fares | 1 828.31 | 1 700.02 | : |
| User Tariffs 7) | 1 998.41 | 1 924.31 | 2 159.20 |
| Revenues from Public Service Contract8) | 1 503.25 | 1 517.30 | : |
| Additional Information |  |  |  |
| Revenues directly related to infrastructure costs |  |  |  |
| Infrastructure access charges | - | - | : |
| Fixed | - | - | : |
| Variable | - | - | : |
| Subsidies 9) | 2 671.63 | 1 739.69 | : |
| Non-transport related revenues of rail companies | : | : | : |
| 1) Totals for Italian National and other Italian rail companies. - 2) The General Transport Plan 2001 by the Ministry of Transport estimates the infrastructure capital cost to be 2 869 € million in 2005; no forecasts for running costs available. – 3) See Annex 1, par. 4.2.1. – 4) No sound allocation methodologies were available for noise costs. - 5) No estimation for congestion costs was carried out, see Annex 1, par. 2.2.3. – 6) Because there is no standardised methodology for the calculation of these costs, the figures given here are to be regarded only as approximate indications that may change greatly over time with the development of a standard methodology. – 7) Subsidies and VAT are excluded – 8) See Annex 1, par. 2.2.7. – 9) Subsidies included here refer to transfer payment for debt service (Ministry of Treasure), extraordinary contributions, Pension Fund deficit coverage (Ministry of Treasure). Sources: ISIS, 2001 |  |  |  |

Comments on specific cost categories

Infrastructure costs

The gross value of capital stock of the network rail amounted in 1998 to € 36.05 billions and the net value was € 19.88 billions. The capital costs were calculated with the perpetual inventory approach (PIM) and amounted to € 2.55 billions. Total infrastructure costs of rail network amounted in 1998 to € 5.61 billion. The running costs of rail network were estimated to be about  € 3.06 billion in 1998.

Supplier operating costs

In 1996 the national rail carrier (FS) costs of the operations amounted to 7 049 € million: more than a half (52.8%) of these costs are due to personnel costs (wages), and in 1998 the share undergoes only a slight decrease (51.7%). The total rail sector, including therefore also licensed railways (non national rail carriers), borne in 1996 a total cost of 7 611 € million for passenger and freight transport, while in 1998 the total cost decreased to 6 673 € million. A forecasting to 2005 of rail sector supplier operating costs is provided without disaggregation in cost items, since 2005 cost structure compared to 1998 one will be greatly affected by the organisational changes that FS underwent during 2000. A cautious estimate is provided on the basis of 1996 and 1998 data. The amount results to be 6 777 € million for national rail carriers and 706 € million for licensed railways, that result in 7 483 € million as a total supplier operating costs for 2005.

Congestion costs

Delay costs due to congestion were not estimated within this account (see par. 2.2.3).

Accident costs

The methodology adopted for the allocation of external accidents costs by mode and type of infrastructure would require detailed information in order to assess the responsibility to be assigned to transport users.  Since such data are not available, the allocation procedure is based on the following assumption  for rail transport, namely that passengers and on-board staff killed or injured are allocated to passenger transport, while accident occurring to other staff are allocated to all types of service by train-km. The distinction between high-speed passenger and conventional passenger services has not been possible.

Total internal and external accident costs in Italy for 1998 were estimated to be € 114.82 million. Average costs have been calculated for train-km (€ / 1000 train-km) and amounted for rail transport to € 326.42 in year 1998. The estimation of total accident costs (internal and external) for rail in Italy in 2005 shows a growing rate of +24.9% compared with 1998 value. Moreover the value for average costs will rise up to 379.90 € / 1000 train-km.

Environmental costs

Data on specific emissions of national rail carrier (FS) due to electric traction (electricity production in g/kWh) were taken from Lombard, Molocchi (2000); these specific emissions are based on data from the former National Electricity Utility - ENEL. Data on specific emissions (g/kWh) due to diesel traction (fuel usage) were taken from Hickman et al. (1999). Total consumption of both electric and diesel traction were taken from Ferrovie dello Stato (1999) and Ministero dell’Industria (1997 and 1999). Pollutant’s emissions disaggregation to diesel traction and electric traction was based on fuel use applying emission factors before mentioned. The allocation to passenger and freight trains was based on data on traffic volume published in Ferrovie dello Stato (1999). For non-national railways basic data on specific emission coefficients and specific consumptions were not available, therefore national coefficients were applied instead. The total traffic volume disaggregation (diesel / electric) of non-national railways was performed through the number of locomotive in use; data on total volume traffic were taken from CNT (1998).

Total environmental costs stemming from rail transport (both freight and passengers) amounted up to Euro 282.6 million in 1998 (with passengers transport rising by some Euro 230.3 million) ranking third after aviation and road transport. Air pollution of rail transport is the most relevant cost category (Euro 144.6 million). Costs due to air pollution and global warming are comparably (in percentage respect to road transport mode) low due to a high share of electric traction which leads to much lower emissions of air pollutants and CO2 than traction based on fossil fuels. On the other hand, noise costs reached about Euro 76.7 million, although amenity losses costs were not evaluated, thus leading to a significant underestimation of total noise costs.

Taxes, charges and subsidies

Revenues from taxes and charges levied on rail sector were split into passenger transport and freight transport. Data on non-national rail carriers were available at a more aggregated level from the Ministry of Transport (CNT) and also from FS. No forecast was feasible for non-national railways revenues as well as for FS. Detailed figures are reported in par. 4.6.2 for tariff revenues and subsidies concerning FS and par. 4.6.3 for those concerning non-national railway companies.

The average variable costs of rail transport are shown in table 79:

Table 79
Average variable costs of rail transport per vehicle km
National and non national rail
- €/train km at 1998 prices -

| National and non national rail |  |  |
| --- | --- | --- |
|  | 1998 |  |
|  | Passenger | Freight |
| Core information |  |  |
| Infrastructure costs | : | : |
| Fixed | : | : |
| Variable | : | : |
| External accident costs | 0,0315 | 0,0230 |
| Administrative | : | : |
| Health costs | : | : |
| Production loss | : | : |
| Environmental costs | 1.15 | 1.56 |
| Air pollution | 0.42 | 0.28 |
| Global warming | 0.17 | 0.16 |
| Noise | 0.56 | 1.12 |
| Total I | : | : |
| Additional Information |  |  |
| Delay costs1) | : | : |
| Internal accident cost | 0,307 |  |
| Material damages | : | : |
| Risk value | 0,307 |  |
| Environmental costs | : | : |
| Nature, landscape, soil and water pollution2) | : | : |
| Nuclear risk2) | - | - |
| Total II | : | : |
| Revenues |  |  |
| User tariffs | : | : |
| Revenues from Public Service Contract3) | : | : |
| Infrastructure access charges4) | : | : |
| Total | : | : |
| Subsidies | : | : |
| Basic data |  |  |
| Passenger km (bill) | 41.39 | . |
| Tonne km (bill) | . | 25.44 |
| 1) No estimation for congestion costs was carried out, see Annex 1, par. 2.2.3. -2) Because there is no standardised methodology for the calculation of these costs, the figures given here are to be regarded only as approximate indications that may change greatly over time with the development of a standard methodology. – 3) See par. 2.2.7. - 4) No allocation to freight/passenger transport possible. Source: ISIS elaboration on various sources. |  |  |

Table 80 shows the total costs of rail transport for passenger and freight transport.

Table 80
Total costs of rail transport (National and non national rail)
- € million at 1998 prices -

| National and non national rail |  |  |  |
| --- | --- | --- | --- |
|  | 1998 |  |  |
|  | Passenger | Freight | Total |
| Core information |  |  |  |
| Infrastructure costs | : | : | 5 605 |
| Tracks |  |  |  |
| Fixed | : | : | : |
| Variable | : | : | : |
| Stations |  |  |  |
| Fixed | : | : | : |
| Variable | : | : | : |
| Supplier operating costs | : | : | 6 673 |
| Out of these: track + station charges |  |  |  |
| External accident costs | 8 | 2 | 10.38 |
| Administrative | : | : | 0.01 |
| Health costs | : | : | 0.77 |
| Production loss | : | : | 9.60 |
| Environmental costs | 345.5 | 103.1 | 448.6 |
| Air pollution | 126.3 | 18.3 | 144.6 |
| Global warming | 50.8 | 10.5 | 61.3 |
| Noise | 168.4 | 74.3 | 242.7 |
| Total I | : | : | 12 737.0 |
| Additional information |  |  |  |
| Delay costs1) | : | : | : |
| Internal accident costs | : | : | 104.44 |
| Material damages | : | : | : |
| Risk value | : | : | 104.44 |
| Environmental costs | : | : | : |
| Nature, landscape, soil and water pollution2) | : | : | : |
| Nuclear risk2) | - | - | - |
| Total II | : | : | : |
| Revenues |  |  |  |
| User tariffs | 1 359.77 | 564.54 | 1 924.31 |
| Revenues from Public Service Contract3) | : | : | 1 517.30 |
| Infrastructure access charges4) | - | - | : |
| Total | : | : | : |
| Subsidies | : | : | 3 439.71 |
| Basic data |  |  |  |
| Passenger km (bill) | 40.77 | . | 40.77 |
| Tonne km (bill) | . | 25.44 | 25.44 |
| 1) No estimation for congestion costs was carried out, see Annex 1, par. 2.2.3. – 2) Because there is no standardised methodology for the calculation of these costs, the figures given here are to be regarded only as approximate indications that may change greatly over time with the development of a standard methodology. - 3) See par. 2.2.7. - 4) Charges due since 2000, a forecasting to 2005 was not possible. Source: ISIS elaboration on various sources. |  |  |  |

## Public transport: tram, metro and trolley bus

It was not possible to elaborate a complete pilot account for this segment of the Italian transport system. Environmental costs, noise, congestion and accidents costs could not be quantified due to methodological and/or data availability problems. Note furthermore, that buses are included in the road account for the category of the supplier operating costs and tariffs and other services revenues (where VAT and subsidies are not included anyway).

Supplier operating costs were estimated to € 5 586 million with an increase by 6 264 in year 2005. User tariffs and other services revenues form, at  € 1 382.55 million, the most important component on the revenue side that we were able to estimate from available data. Subsidies were finally estimated for 1996 only, namely € 2 272.85 million. This figure is expressed at 1998 prices though.

Table 81
Italian account for metro, tram, trolley bus 1996, 1998 and 2005 
- € million at 1998 prices -

| Costs |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Core information | 1996 | 1998 |  | 2005 |  |
| Infrastructure Costs | 39 | 34 |  | : 1) |  |
| Fixed | : | : |  | : |  |
| Variable | : | : |  | : |  |
| Services |  |  |  |  |  |
| Supplier operating costs 2) | 6 271 | 5 586 |  | 6 264 |  |
| Accident costs (external) | : | : |  | : |  |
| Environmental costs | 108.0 | 112.1 |  | 110.7 |  |
| Air pollution | 91.0 | 93.9 |  | 93.3 |  |
| Global warming | 17.0 | 18.2 |  | 17.3 |  |
| Noise | : | : |  | : |  |
| Additional information |  |  |  |  |  |
| Congestion costs3) | : | : |  | : |  |
| Accident costs (internal) | : | : |  | : |  |
| From this: risk value | : | : |  | : |  |
| Environmental costs | : | : |  | : |  |
| Nature and landscape, soil and water pollution 4) | : | : |  | : |  |
| Nuclear risk4) | - | - |  | - |  |
| Revenues |  |  |  |  |  |
| Directly related to a specific cost category |  |  |  |  |  |
| Charges for infrastructure usage | : | : |  | : |  |
| Fixed | : | : |  | : |  |
| Variable | : | : |  | : |  |
| Subsidies for concessionary fares 2) |  |  |  |  |  |
| User Tariffs and other service revenues 2) 5) | 1 310 | 1 382.55 |  |  |  |
| Other transport specific revenues | : |  | : |  | : |
| Subsidies 6) | 2 272.857) | : |  | : |  |
| 1) The General Transport Plan 2001 by the Ministry of Transport estimates the infrastructure capital cost to be 1291 € millions in 2005; no forecasts for running costs available. – 2) Including buses. – 3) No estimation for congestion costs was carried out, see Annex1, par. 2.2.3. – 4) Because there is no standardised methodology for the calculation of these costs, the figures given here are to be regarded only as approximate indications that may change greatly over time with the development of a standard methodology. – 5) Subsidies and VAT are excluded. – 6) Includes subsidies given as a contribution to running expenditures and other contributions. – 7) This data refers to 1995 at 1998 prices. Sources: ISIS elaboration on various sources. |  |  |  |  |  |

Comments on specific cost categories

Infrastructure costs

Metro service, although it is 20% of the total network extension, accounts for 70% of the total service supply. For this mode, only capital stock was calculated, whose amount was € 0.56 billions in 1996 (gross value) and € 0.25 billions (net value), which leads to a capital costs value of  € 0.04 billion. Several hurdles, already pointed out, have not allowed us to properly quantify and evaluate all the running costs for tramway and metro infrastructures. These figures were the input data for the Perpetual Inventory Model (PIM). The infrastructure asset value for the different modes is stated for years 1996 and 1998. The results for 2005 are calculated on the basis of the General Transport Plan and are reported under paragraph 3.1.

Supplier operating costs

The total costs for public transport sector in 1996 amounted to 6 271 € million, of which a 90.8% is due to buses and trolley buses, and a 9.2% to rail borne public transport (namely, 5.6% to tram and 3.6% to underground transport). In 1998 the weight of buses and trolley related costs on the total is slightly higher, 91.6%, tram related costs are 5.3% and metro related costs are 3.2%. Forecasts for 2005 show a value of  € 6 264 million, of which € 5 736 million are referred to buses, trolley buses, € 330 million to tram and € 198 to subway transport. All estimations and forecasts were carried out from data supplied by CNT and CISPEL.

Congestion costs

Delay costs due to congestion were not estimated within this account (see par. 2.2.3).

Accident costs

Accident costs for metro, tram and trolley bus were not estimated. For more detailed explanations, check par. 2.2.4, 3.4 and 4.4.

Environmental costs

Urban road public transport emission account was built using the same sources and methodology as described for road account. Concerning rail-borne public transport, energy consumption of electric vehicles was calculated based on specific energy consumption factors in kWh/ km considered for rail transport. The national traffic volume of tramways, metro and light rail trains was taken from CNT (1998). Data on specific emissions are those used for rail transport. As concerns about Public transport costs, they accounted for a mere € 112.1 million in 1998 (€ 102.7 million in 1996), with diesel urban bus (€ 105.0 million) overwhelming tram and trolley bus (€ 2.1 million) and metro (€ 4.9 million). Noise costs were not estimated due to a lack of consistent data (refer to Table 79) Average costs per vehicle-km for 1998 are reported in Table 82 and split by sector and vehicle type.  Forecasts to 2005 highlight an overall environmental costs rise by € 133.8 million, where air pollution would still be the most substantial cost driver (€ 112.8 million)

Taxes, charges and subsidies

The calculation of revenues for public transport is affected by the lack of recent published data. It is worth keeping in mend that the estimation of tariff revenues for 1996 and 1998 was made on the basis of 1995 data assuming constant the level of tariffs and according to the trend of the number of passengers.

In table 12 we have attempted to show the fully allocated costs of metro, tram and trolley bus services. As can be seen from the table more research is needed in this area. In table 13, the total costs of public transport (metro, tram and trolley bus) are shown disaggregated by vehicle type.

Table 82
Average variable costs of metro, tram, trolley bus per vehicle km: Italy
- €/km at 1998 prices –

|  | 1998 |  |  |
| --- | --- | --- | --- |
|  | Metro and other |  | Tram and trolley bus |
| Core information |  |  |  |
| Infrastructure costs | : |  | : |
| Fixed | : |  | : |
| Variable | : |  | : |
| Supplier operating costs | 5.09 |  | 3.08 |
| External accident costs | : |  | : |
| Administrative | : |  | : |
| Health costs | : |  | : |
| Production loss | : |  | : |
| Environmental costs | : |  | : |
| Air pollution | 0.38 |  | 0.39 |
| Global warming | 0.22 |  | 0.23 |
| Noise | : |  | : |
| Total I | : |  | : |
| Additional information |  |  |  |
| Delay costs1) | : |  | : |
| Internal accident costs | : |  | : |
| Material damages | : |  | : |
| Risk value | : |  | : |
| Environmental costs | : |  | : |
| Nature, landscape, soil and water pollution2) | : |  | : |
| Nuclear risk2) | - |  | - |
| Total II | : |  | : |
| Revenues |  |  |  |
| User tariffs3) | : |  | : |
| Subsidies | : |  | : |
| Basic data |  |  |  |
| Passengers carried (million) | 569.52 | 3 566.072) |  |
| Passenger km (billion) | 4.13 | 0.001 |  |
| 1) No estimation for congestion costs was carried out, see Annex 1, par. 2.2.3. – 2) Because there is no standardised methodology for the calculation of these costs, the figures given here are to be regarded only as approximate indications that may change greatly over time with the development of a standard methodology. – 3) This data includes tram, trolley and buses. Source: ISIS elaboration on various sources. |  |  |  |

Table 83
Total costs of metro, tram, trolley bus: Italy
- € million at 1998 prices -

|  | 1998 |  |  |
| --- | --- | --- | --- |
|  | Metro and other | Tram and trolley bus | Total |
| Core information |  |  |  |
| Infrastructure costs | : | : | : |
| Fixed | : | : | : |
| Variable | : | : | : |
| Supplier operating costs | 177 | 5 4091) | 5 586 |
| External accident costs | : | : | : |
| Administrative | : | : | : |
| Health costs | : | : | : |
| Production loss | : | : | : |
| Environmental costs | 4.9 | 107.12) | 112.1 |
| Air pollution | 3.1 | 90.72) | 93.9 |
| Global warming | 1.8 | 16.42) | 18.2 |
| Noise | : | : | : |
| Total I | : | : | : |
| Additional information |  |  |  |
| Delay costs3) | : | : | : |
| Internal accident costs | : | : | : |
| Material damages | : | : | : |
| Risk value | : | : | : |
| Environmental costs | : | : | : |
| Nature, landscape, soil and water pollution | : | : | : |
| Nuclear risk | - | - | - |
| Total II | : | : | : |
| Revenues |  |  |  |
| User tariffs | : | : | 1 382.55 |
| Subsidies | : | : | : |
| Basic data |  |  |  |
| Passengers carried (million) | 569.52 | 3 566.071) | 4 431.91 |
| Passenger km (billion) | 4.13 | 0.001 | 4.13 |
| 1) This data includes tram, trolley and buses. 2) This data includes tram, trolley and urban buses. – 3) No estimation for congestion costs was carried out, see Annex 1, par. 2.2.3 – Source: ISIS elaboration on various sources. |  |  |  |

## Aviation

Infrastructure costs represented € 571 million, however. Total social costs of accidents (risk value) amounted to € 23.85 million, whereas external accidents costs were merely  € 2.23 million. Other relevant figures were drawn from the aircraft insurance taxes, which albeit referred to 1997, reached € 12.41 million at 1998 prices. The income of air transport activity is linked to services such as essential operating services (landing and take-off assistance, parking, air assistance, maintenance, safety), handling (land-side assistance) and non aeronautic operations (commercial services). Who is actually receiving these service revenues depends on the airport management structure: in the “full management” airports (one concessionary manages the whole airport, including the flight infrastructures, for a very long period) every aeronautic and commercial income is perceived by the airport manager; in the “partial management” airports (flight infrastructure ownership is public, while a concessionary is in charge of passenger and freight terminal services and possibly handling operations; other services can be entrusted to other concessionaries) the State gathers start / landing fees and parking fees for aircrafts, while the manager collects revenues from boarding and handling operations and from commercial activities; in the “public direct management” airports (the State directly manages all activities but handling and commercial activities, that are realised by concessionaires) the State takes all incomes stemming from aircrafts movements and parking, and also concessionary fees for the activities entrusted to other actors. It is worth mentioning that the noise charge on aircrafts movement is quite the only example of environmental tax on the Italian transport sector: this levy, collected since 1990 by the State, became a regional tax in 1997. Revenues from this tax are earmarked to finance interventions to reduce noise emissions from aircrafts, and in particular to complete and improve acoustic pollution monitoring systems, to reduce noise pollution and to pay compensations to residents living near the airport area, that are the most damaged.

Table 84
Italian air transport account for 1996, 1998 and 2005 
- € million at 1998 prices -

| Costs |  |  |  |
| --- | --- | --- | --- |
| Core information | 1996 | 1998 | 2005 |
| Infrastructure Costs1) | 683 | 571 | : |
| Fixed | : | : | : |
| Variable | : | : | : |
| Accident costs (external) | 1.7 | 2.2 | 2.2 |
| Environmental costs | 448.3 | 467.4 | : |
| Air pollution | 73.1 | 76.7 | : |
| Global warming | 191.0 | 197.0 | : |
| Noise2) | 184.2 | 193.7 | : |
| Total | 1 133.0 | 1 040.6 | : |
| Additional information |  |  |  |
| Congestion costs3) | : | : | : |
| Accident costs (internal) | : | : | : |
| From this: risk value | 23.78 | 23.85 | 24.14 |
| Environmental costs | : | : | : |
| Nature and landscape, soil and water pollution4) | : | : | : |
| Nuclear risk4) | - | - | - |
| Revenues |  |  |  |
| Directly related to a specific cost category |  |  |  |
| Charges for infrastructure usage |  |  |  |
| Airport revenues | 1 380.54 | 794.675) | : |
| ATM charges6) | 123.36 | 2007) | : |
| Charges for aircraft noise emissions | 5.74 | : | : |
| Total | : | : | : |
| Other transport specific revenues |  |  |  |
| Aircraft registration taxes | 0.098) | : | : |
| Aircraft insurance taxes | 6.998) | 12.419) | : |
| Subsidies | : | : | : |
| Non-transport related revenues of airports | 287.70 | : | : |
| 1) The General Transport Plan 2001 by the Ministry of Transport estimates the infrastructure capital cost to be 177 € millions in 2005; no forecasts for running costs available. – 2) No sound allocation methodologies were available for noise costs. - 3) No estimation for congestion costs was carried out, see Annex 1, par. 2.2.3. - 4) Because there is no standardised methodology for the calculation of these costs, the figures given here are to be regarded only as approximate indications that may change greatly over time with the development of a standard methodology. – 5) Revenues from general aviation activity rights doubling are not included. – 6) Includes terminal charges and en-route charges. - 7) Revenues from terminal charges not included due to lack of data. - 8) This data refers to 1994, expressed at 1998 prices. - 9) This data refers to 1997, expressed at 1998 prices. Source: ISIS elaboration on various sources. |  |  |  |

Forecasts for 2005 were carried out only for external accidents costs (decreasing by € 2.17 million from € 2.23 million in 1998) and internal ones (increasing up to € 24.14 million instead).

Comments on specific cost categories

Infrastructure costs

Infrastructure costs are calculated only for airports with an yearly accounted traffic over 1 000 passengers because national transport statistics (CNT) provides only for these airports the parameters necessary for the calculation of the infrastructure quality indicator (total area included in the bordered airport surface – aprons and land area), albeit in a discontinuous manner only since the 80’s. The air transport capital stock amounted in 1998 to € 6.23 billion (gross value) and € 3.65 billion (net value), which implied a capital cost of € 0.43 billion. Total running costs reach  € 0.14 billions. Total infrastructure costs for aviation in Italy in 1998 were therefore € 571 million. Running costs amounted to € 144 million in 1998, of which € 37 belonging to maintenance costs, € 41 million to infrastructure and € 66 million to administrative costs.

Congestion costs

Delay costs due to congestion were not estimated within this account (see par. 2.2.3).

Accident costs

The major accident cost for the aviation sector is the risk value, thus highlighting the importance of air safety to society. Its value amounted to € 23.85 million in 1998. In particular, external costs amounted to € 2.23 million in 1998, whereas total costs for this mode of transport were as high as € 26.08 million.

It is to be remarked that, as concerns about allocation of total costs to modes and types of infrastructure, in aviation only aggregated calculations were feasible. Average accidents costs in 1998 were not calculated for aviation as well as forecasts for year 2005 were not carried out, due to a lack of consistent data concerning this mode of transport.

Environmental costs

The emissions due to aviation were calculated for landing and take-off (LTO) at airports in Italy, based on data taken from ANPA (2001). Data on PM10 emissions were not available, thus causing underestimation of air pollution costs.

The costs of aviation in 1998 were dominated by global warming (€ 197.0 million). Flights environmental costs accounted by € 196.0 million, double than airports ones (€ 77.0 million). The category “flights” covers the costs due to emissions of CO2 and indirect emissions of air pollutants (due to fuel production) based on the civil aviation fuel taken in Italy. For technical reasons CO2 emissions at airports are included in this category. “Airports” contains costs of pollutant emissions (except CO2) during the Landing and Take-off (LTO) cycles at Italian airports. Compared to 1996 figures, total costs of aviation rose from € 276.5 million to € 300.3 million in 1998. No average cost calculation per vehicle-km was carried out as well as no forecasts to 2005, since data were not available but existing anyway.

Taxes, charges and subsidies

The revenues taken into consideration for air transport accounts refer to aircraft-related taxes (due by airlines), airport charges and air traffic control charges. In order to be consistent with the air infrastructure account and to allow a proper assessment of air sector costs and revenues, only revenue data related to airports with an annual traffic of more than 1 000 yearly passengers are taken into account. The most recent set of data available from ENAC dates back to 1996: therefore both data for 1998 were estimated on the basis of 1996 data and of traffic performance. It was not possible to estimate 2005 revenues because of lack of forecasting on air traffic performance. All revenues generated by the use of airport infrastructure are allocated to infrastructure costs, in particular airport charges; all revenues from noise emission charge are allocated to environmental costs.

In table 15 the fully allocated costs of passenger and cargo air transport per movement are shown.

Table 85
Average variable costs of Aviation per LTO cycle: Italy
– €/LTO cycle at 1998 prices –

|  | 1998 |  |
| --- | --- | --- |
|  | Passenger | Cargo |
| Core information |  |  |
| Infrastructure costs | : | : |
| Fixed | : | : |
| Variable | : | : |
| External accident costs | 4.4 |  |
| Administrative | : |  |
| Health costs | 0.2 |  |
| Production loss | 4.2 |  |
| Environmental costs | 931.7 |  |
| Air pollution | 152.9 |  |
| Global warming | 392.7 |  |
| Noise | 386.1 |  |
| Total I | 940.3 |  |
| Additional information |  |  |
| Delay costs: per arriving flight | : | : |
| Internal accident costs | : | : |
| Material damages | : | : |
| Risk value | 47.5 |  |
| Environmental costs | : | : |
| Nature, landscape, soil and water pollution1) | : | : |
| Nuclear risk1) | : | : |
| Total II | : | : |
| Revenues |  |  |
| Charges for infrastructure usage | : | : |
| Airport revenues | : | : |
| ATM charges | : | : |
| Charges for aircraft noise emissions | : | : |
| Aircraft registration taxes | : | : |
| Aircraft insurance taxes | : | : |
| Total | : | : |
| Basic data |  |  |
| Passenger km (bill) | 101 | . |
| Tonne km (bill) | . | 2.4 |
| 1) Because there is no standardised methodology for the calculation of these costs, the figures given here are to be regarded only as approximate indications that may change greatly over time with the development of a standard methodology. Source: ISIS elaboration on various sources. |  |  |

In table 86 the total costs are disaggregated between passenger and freight transport.

Table 86
Total costs of Aviation: Italy
- € million at 1998 prices -

|  | 1998 |  |  |
| --- | --- | --- | --- |
|  | Passenger | Cargo | Total |
| Core information |  |  |  |
| Infrastructure costs | : | : | 571 |
| Fixed | : | : | : |
| Variable | : | : | : |
| External accident costs | : | : | 2.23 |
| Administrative | : | : | : |
| Health costs | : | : | 0.12 |
| Production loss | : | : | 2.13 |
| Environmental costs1) | : | : | 467.4 |
| Air pollution | : | : | 76.7 |
| Global warming | : | : | 197.0 |
| Noise1) | : | : | 193.7 |
| Total I | : | : | 1 040.6 |
| Additional information |  |  |  |
| Delay costs2) | : | : | : |
| Internal accident costs | : | : | : |
| Material damages | : | : | : |
| Risk value | : | : | 23.85 |
| Environmental costs |  |  |  |
| Nature, landscape, soil and water pollution3) | : | : | : |
| Nuclear risk | - | - | - |
| Total II | : | : | : |
| Revenues |  |  |  |
| Charges for infrastructure usage |  |  |  |
| Airport revenues | : | : | 799.784) |
| ATM charges | : | : | 2005) |
| Charges for aircraft noise emissions | - | - | : |
| Aircraft registration taxes | : | : | : |
| Aircraft insurance taxes | : | : | 12.416) |
| Total | : | : | 1 012.19 |
| Subsidies | : | : | : |
| Total | : | : | : |
| Basic data |  |  |  |
| Passenger km (bill) | 101 | . | 101 |
| Tonne km (bill) | . | 2.4 | 2.4 |
| 1) No sound allocation methodologies were available for noise costs. - 2) No estimation for congestion costs was carried out, see Annex 1, par. 2.2.3. – 3) Because there is no standardised methodology for the calculation of these costs, the figures given here are to be regarded only as approximate indications that may change greatly over time with the development of a standard methodology. – 4) Revenues from general aviation activity rights doubling are not included. – 5) Revenues from terminal charges not included due to lack of data. – 6) This data refers to 1997, expressed at 1998 prices. Source: ISIS elaboration on various sources. |  |  |  |

## Inland waterways transport

Infrastructure costs play the major role in inland waterway transport. The figure reported in the above table (€ 58 million for inland waterways) confirms this. For 2005, an increase of infrastructure costs up to € 316 million was carried out by the Ministry of Transport, as well as internal accidents costs, that will rise by € 8.06 million. No further costs categories were appraised for this mode of transport, except for infrastructure and accident costs and revenues. As concerns about capital value (gross value) and running costs for inland waterways, the former amounted to € 478 million in 1998 whereas the latter rose by  € 29 million. This value was also estimated for capital costs. It’s remarkable the fact that capital costs decreased from more than 80% in 1996 to about 50% of total infrastructure costs in 1998.

Comments on specific cost categories

Infrastructure costs

Data on network extension is based upon the operating length, whose main source is the UNII (Union of Italian Inland Waterways). In order to create a time series long enough, only data on passengers transport were taken into account. The inland waterway capital stocks amounted in 1998 to € 478 million (gross value) and € 215 million (net value), which implied a capital cost amount of € 29 million. Total running costs reach  € 29 million.

Congestion costs

Delay costs due to congestion were not estimated within this account (see par. 2.2.3).

Accident costs

This cost driver does not play a significant role for total accidents costs of transport. Values in 1998 referred to inland waterways and maritime transport amounted to € 5.60 million out of some € 23 968.9 million of total costs for all modes. Internal costs amounted to € 5.12 million, about 96% of total accidents costs for inland waterway. Compared to the 1998 accident accounts, forecasts in 2005 show a growing rate for total accidents costs by 36% (namely € 8.75 million), whereas the average will rise up to 1 323.32 €/1000 vessel-km. As shown in the par. 4.4, Table 44, for inland waterway transport the growth in volumes is mainly responsible for the increasing trend in the average costs

Table 87
Italian inland waterways account for 1996, 1998 and 2005 
- € million at 1998 prices -

| Costs |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Core information | 1996 |  | 1998 |  | 2005 |  |
| Infrastructure costs – inland waterway harbours | 36 |  | 58 |  | :1) |  |
| Fixed | : |  | : |  | : |  |
| Variable | : |  | : |  | : |  |
| Infrastructure costs – inland waterways | : |  | : |  | : |  |
| Fixed | : |  | : |  | : |  |
| Variable | : |  | : |  | : |  |
| Accident costs (external) | 0.86 |  | 0.49 |  | 0.69 |  |
| Environmental costs | 6.3 |  | 6.2 |  | : |  |
| Air pollution | 5.0 |  | 4.9 |  | : |  |
| Global warming | 1.4 |  | 1.3 |  | : |  |
| Noise | : |  | : |  | : |  |
| Total | : |  | : |  | : |  |
| Additional information |  |  |  |  |  |  |
| Congestion costs2) | : |  | : |  | : |  |
| Accident costs (internal) | : |  | : |  | : |  |
| From this: risk value | 10.76 |  | 5.12 |  | 8.06 |  |
| Environmental costs |  |  |  |  |  |  |
| Nature and landscape, soil and water pollution3) | : |  | : |  | : |  |
| Nuclear risk3) | - |  | - |  | - |  |
| Revenues |  |  |  |  |  |  |
| Directly allocatable |  |  |  |  |  |  |
| Charges for infrastructure usage | : |  | : |  | : |  |
| Fixed | : |  | : |  | : |  |
| Variable | : |  | : |  | : |  |
| Total | : |  | : |  | : |  |
| Other transport specific revenues |  | : |  | : |  | : |
| Subsidies | : |  | : |  | : |  |
| Non-transport related revenues of ports | : |  | : |  | : |  |
| 1) The General Transport Plan 2001 by the Ministry of Transport estimates the infrastructure capital cost for inland waterways and maritime transport to be 316 € millions in 2005; no forecasts for running costs available. – 2) No estimation for congestion costs was carried out, see Annex 1, par. 2.2.3. – 3) Because there is no standardised methodology for the calculation of these costs, the figures given here are to be regarded only as approximate indications that may change greatly over time with the development of a standard methodology. Source: ISIS elaboration on various sources. |  |  |  |  |  |  |

Environmental costs

Data on total emissions (not allocated to passenger and freight) from inland waterway transport are based on data provided by ANPA. PM10 emissions were not available. Data on pollutant emissions of short sea shipping were not available as well.

Environmental costs stemming from inland waterways in 1998 accounted for a mere € 6.2 million (€ 6.0 in 1996) namely less than 0.06% of total environmental costs for Italy in 1998. Air pollution reached € 4.9 million, whereas global warming cost driver rose up to € 1.3 million. No calculation was feasible for noise costs, as well as for the average costs for vehicle-km and forecasts to 2005 (where noise is reported to be nil).

Taxes, charges and subsidies

There is a shortage of statistics concerning maritime and inland waterways transport related revenues: the only set of data available is contained in a survey by CER (1996), and refers to the year 1994. Updated values were not available, nor data on port charges. It was not possible to produce robust extrapolations of 1996 and 1998 revenues. Data for 1994 show significant figures for insurance tax for vessel and freight which amounted to € 100.58 million and VAT on maritime transport services paid by end users equal to € 113.95. Total of taxes for only maritime transport thus amounted to € 203.49 million in 1994 at 1998 prices

All figures previously reported can be checked in the two following tables 88 and 89.

Table 88
Average variable costs of inland waterways and maritime shipping
per vehicle km
- €/km at 1998 prices -

|  | 1998 |  |
| --- | --- | --- |
|  | Inland waterways | Maritime shipping |
| Core information |  |  |
| Infrastructure costs | : | : |
| Inland waterway harbours | : | . |
| Fixed | : | . |
| Variable | : | . |
| Inland waterways | : | . |
| Fixed | : | . |
| Variable | : | . |
| Sea harbours | . | : |
| Fixed | . | : |
| Variable | . | : |
| External accident costs | : | : |
| Administrative | : | : |
| Health costs | : | : |
| Production loss | : | : |
| Environmental costs | : | : |
| Air pollution | : | : |
| Global warming | : | : |
| Noise | : | : |
| Total I | : | : |
| Additional information |  |  |
| Delay costs1) | : | : |
| Internal accident costs | : | : |
| Material damages | : | : |
| Risk value | : | : |
| Environmental costs | : | : |
| Nature, landscape, soil and water pollution | : | : |
| Nuclear risk | - | - |
| Total II | : | : |
| Revenues |  |  |
| Charges for infrastructure usage | : | : |
| fixed | : | : |
| variable | : | : |
| Other transport specific revenues | : | : |
| Subsidies | : | : |
| Basic data |  |  |
| Vehicle Kilometres | : | : |
| Tonne km | 126.14 | : |
| 1) No estimation for congestion costs was carried out, see Annex 1, par. 2.2.3. - Source: ISIS elaboration on various sources. |  |  |

Table 89
Total costs of inland waterways and maritime shipping: Italy
- € million at 1998 prices -

|  | 1998 |  |
| --- | --- | --- |
|  | Inland waterways | Maritime shipping |
| Core information |  |  |
| Infrastructure costs |  |  |
| Harbours |  |  |
| Inland waterways | 58 | - |
| Fixed | : | - |
| Variable | : | - |
| Sea harbours | - | : |
| Fixed | - | : |
| Variable | - | : |
| Waterways |  |  |
| Inland waterways | : | - |
| Fixed | : | - |
| Variable | : | - |
| Maritime shipping |  | : |
| Fixed | - | : |
| Variable | - | : |
| External accident costs | 0.49 | : |
| Administrative | : | : |
| Health costs | 0.03 | : |
| Production loss | 0.46 | : |
| Environmental costs | 6.2 | : |
| Air pollution | 4.9 | : |
| Global warming | 1.3 | : |
| Noise | : | : |
| Total I | : | : |
| Additional information |  |  |
| Delay costs1) |  |  |
| Internal accident costs |  |  |
| Material damages | : | : |
| Risk value | 5.12 | : |
| Environmental costs |  |  |
| Nature, landscape, soil and water pollution2) | : | : |
| Nuclear risk2) | : | : |
| Total II |  |  |
| Revenues |  |  |
| Charges for infrastructure usage | : | : |
| fixed | : | : |
| variable | : | : |
| Other transport specific revenues | : | : |
| Subsidies | : | : |
| Basic data |  |  |
| Tonne km (bill) | 126.14 | : |
| 1) No estimation for congestion costs was carried out, see Annex 1, par. 2.2.3. – 2) Because there is no standardised methodology for the calculation of these costs, the figures given here are to be regarded only as approximate indications that may change greatly over time with the development of a standard methodology. Source: ISIS elaboration on various sources. |  |  |

# Conclusions

The Italian pilot accounts show the level of transport costs and revenues for the year 1998 and 1996 and provide a workable methodological framework to enable regular updating of transport accounts. Where possible, an extrapolation for 2005 is given.

Given the current availability of data, the Pilot accounts contain the following:

- Total infrastructure costs were estimated for road, rail (national and non national carrier), public transport, air, inland waterways, maritime transport;

- Supplier operating costs were estimated for the modes where State subsidisation was relevant, i.e. rail transport and public transport, with more relevant problems concerning public transport costs estimations, due to data availability;

- Delay costs due to congestion were not estimated, since we deem the current situation of data availability for urban areas does not allow calculating national figures for this cost category. The available studies attempting to fill this gap are described within the account.

- Accident costs are estimated for all modes with the exception of maritime transport. For all the other modes both the internal (material damages and risk value) and the external part (administrative costs, health costs and production losses) were estimated on the basis of national statistics.

- The various categories of the environmental costs were estimated as follows: air pollution costs could be calculated at the base year for road transport, rail and public transport; an estimation of air pollution costs for aviation and inland waterways was provided for 1996, but not for 2005. The same applies to global warming. Concerning noise costs, this category is clearly underestimated when compared to other countries, since amenity losses costs could not be calculated due to the lack of the necessary hedonic price indicator.

- The account provides a compendium of taxes and charges paid by the transport sector actors, which however only considers those levies somewhat specific to the transport sector, paid by infrastructure users (individual passengers as well as operators). Taxes and charges for road, rail and public transport were presented, and when available, information on subsidies is also provided. The information on revenues from the inland waterways and maritime transport sector is rather poor.

## Relevance of the accounts for the transport policy

The role that transport accounts can play in informing policy decisions is manifold and is discussed within other UNITE deliverables at the overall European level.

Concerning Italy, it is worthwhile stressing the following considerations. Social external costs, and environmental costs in particular, are increasingly recognised as a major component of total transport costs. However, the lack of sufficient quantitative evidence, associated to the uncertainty that currently characterises monetary valuation processes, often lead to simply ignoring external costs in decision taking processes, whereby policy makers experience difficulties in justifying decisions based on hazy and insufficiently robust data. As a meaningful example, one can refer to the recent elaboration of the National Transport Plan for Italy, where neither for what concerns infrastructure provision and the associated investments, nor in the area of transport pricing, external cost considerations have been visibly incorporated. As a result, transport planning decisions rely on incomplete evidence, with the recourse to quali-quantitative techniques, such as MCA, as the only practicable option.

Transport accounts as those elaborated within UNITE can play a dramatic role in redressing such a situation.

Transport statistics in Italy suffer from a variety of problems, most of which can be directly appraised by looking at the numerous gaps unveiled in the course of this UNITE work. The following section reports on the most relevant among those. In general, uneven statistical availability across modes and irregular updating of basic figures can be highlighted as two major areas where a systematic follow-up of the UNITE work could produce considerable policy relevant improvements.

## Open questions and future improvements

The completion of the Pilot Accounts raised some issues that would need further research and efforts. The main points are reported hereafter:

- Forecasts - It is worth noting that a major problem was found when attempting a forecast of the costs to 2005, due to the lack of official national transport forecasts on such basic variables as: number of vehicles, passengers and freight carried, mileages. Unofficial forecasts were not made available due to privacy reasons.

- Public transport - Due to the current state of data availability and data ownership, it is hardly feasible to systematically isolate the share of public transport costs and revenues from the respective road and rail accounts. In most cases, it is not possible to allocate the available data without making strong assumptions and arbitrary allocations (this is the case of infrastructure costs accident costs and environmental costs for road, that obviously include buses – urban and coaches); therefore a cautious interpretation of the existing results is suggested.

- The estimation of the supplier operating costs was hindered by the lack of detailed data: in fact, the only effective solution would have been to analyse the individual balance sheets of all public transport operators, which was obviously beyond the scope and resources available within the project (data produced from professional associations were not updated).

- Concerning accident costs, the following issues are critical to possibly improve both the reporting activity and the evaluation:

- improving the quality of information for injuries: at present, neither the health national system nor police authorities provide evidence on such issues as e.g. the differentiation between slight and severe injuries caused by accidents;

- reducing the rate of accidents underreporting, through a better co-ordination and standardisation of reporting activity among the different public authorities involved, i.e. police authorities, municipal police, Carabinieri;

- achieving a more consistent and robust evaluation of accident costs through a better estimation of damages to public property and to other private properties (damage costs).

- Environmental costs – some gaps in the data remain, especially for what concerns PM10 emissions for mopeds and motorcycles, as well as for aircrafts and the environmental impacts of the inland waterways and maritime transport.

- Maritime transport and inland waterways –knowledge about the costs and revenues connected to these modes of transport is scarce

- The study of the transport sector subsidisation was not exhaustive, and results were presented only when easily accessible due to the fact that they were considered as additional information within the UNITE project. Therefore the attempt of comparing figures for costs and for revenues within this account might be misleading in certain cases. The knowledge about direct and indirect subsidies for all modes of transport is certainly an area needing further efforts in order to correctly compare costs and revenues.

# References

ACI, Annuario Statistico, various years.

ACI (1999), Cessazione dalla circolazione dei veicoli, 1997.

ACI (2000), Cessazione dalla circolazione dei veicoli, anno 1998.

ACI – ANFIA (2001), I costi e i benefici esterni del trasporto, CSST and ANFIA, Torino.

ACI - ISTAT (1998) Il costo sociale degli incidenti stradali. Roma

ACTT - Azienda Consorziale Trasporti Trieste, balance sheets 1991, 1992, 1993, 1994:

AISCAT, “Informazioni”, n. 3/4 1996 and 1998

ANIA (2000) National Association of Insurance Companies, Position paper, ANIA web site http://www.ania.it

ANPA, Serie Storiche dal 1980 al 1999 delle emissioni dei principali inquinanti in Italia organizzati per settore, http://www.sinanet.anpa.it/aree/atmosfera/emissioni

ANPA (2000), Le emissioni in atmosfera da trasporto stradale, Roma.

ANPA (2001), direct information provided by Unita' censimento fonti di emissioni.

Autostrade SpA (1998), Balance Sheet 1998

Berdowski, J., Mulder, W., Veldt, C., Visschedijk, A., Zandveld, P. (1997), Particulate matter (PM10 - PM2.5 - PM0.1) emissions in Europe. TNO-report TNO-MEP-R 96/472, TNO, Appeldoorn, The Netherlands.

Bickel P, Schmid S, Friedrich R Maibach M, Doll C, Tervonen J, Enei R (2000): Accounts Approach for Environmental Costs, Version 0.6, Stuttgart, UNITE (UNIfication of accounts and marginal costs for Transport Efficiency). Contained in Link et al. (2000), Accounts Approach. Funded by 5th Framework RTD Programme. ITS, University of Leeds, Leeds October.

Builtjes, P. (1992), The LOTOS-Long Term Ozone Simulation project, Summary report, TNO-report, IMW-R 92/240, 1992.

Capros, P. and Mantzos, L. (2000), Kyoto and technology at the European Union: costs of emission reduction under flexibility mechanisms and technology progress, Int. J. Global Energy Issues, 14, pp. 169-183.

CEC (1995) Green Paper, Towards fair and efficient pricing in transport. COM(95)691, European Commission, Brussels.

CEC (1998) White Paper, Fair Payment for Infrastructure Use: A phased approach to a common transport infrastructure charging framework in the EU. COM(98) 466 final. European Commission, Brussels.

CENSIS (1997a), La sicurezza possibile. Rischi stradali e tecnologie dei veicoli. Sintesi della ricerca, Roma

CENSIS (1997b), Traffico e metropoli: un programma per il monitoraggio. Sintesi della ricerca, Roma

Centro Europa Ricerche (1996), Il fisco e i trasporti, Veutro Editore, Roma

CISPEL (1998), Annuario ufficiale economico-statistico, Cispel, D’Anselmi, Hoepli. Milano, Roma

CORINAIR 1994 inventory. (http://www.aeat.co.uk/netcen/corinair/94/)

CNT - Conto Nazionale Trasporti, Ministero dei Trasporti e della Navigazione, Roma, various years

Doll C, Lindberg G, Niskanen, E. (2000) Accounts Approach for Accidents. UNITE (UNIfication of accounts and marginal costs for Transport Efficiency). Contained in Link et al. (2000), Accounts Approach. Funded by 5th Framework RTD Programme. ITS, University of Leeds, Leeds October.

Doll C. (2001) Pilot accounts nature and landscape Germany 1998 (and 1996), Draft. UNITE (UNIfication of accounts and marginal costs for transport efficiency). Funded by 5th Framework RTD Programme.

European Commission (1995), ExternE - Externalities of Energy. Voll. 1-6. Report produced for the EC – DG XII, Office of Publications for the European Communities, Luxembourg.

European Commission (1999), ExternE - Externalities of Energy. Voll. 7-10. Report produced for the EC – DG XII, Office of Publications for the European Communities, Luxembourg.

Eurostat (2001), EU Energy and Transport in Figures 2001

Ferrovie dello Stato (1996a), Annuario Statistico delle Ferrovie dello Stato, Funzione Strategia e Controllo Servizi Studi e Statistica, Roma

Ferrovie dello Stato (1996b), Le Ferrovie Italiane tra Stato e Mercato, Funzione Strategia e Controllo Servizi Studi e Statistica, Roma

Ferrovie dello Stato (1999), Annuario Statistico delle Ferrovie dello Stato, Direzione Business Development – Statistica, Roma

Ferrovie dello Stato (2000a), Bilancio consolidato al 31.12.1999, Roma

Ferrovie dello Stato (2000b), Ferrovie dello Stato: i risultati della gestione economica delle ferrovie italiane dal 1906 al 1998, Spirito P., P. Buratta eds, coordinamento Strategia e Innovazione, Roma.

Ferrovie dello Stato (2000c), Il piano di ristrutturazione delle Ferrovie in gestione commissariale governativa. Rapporto conclusivo, Roma

Friedrich, R. and Bickel, P. (eds.) (2001), Environmental External Costs of Transport. Springer Verlag, Heidelberg.

Hickman J., D. Hassel, R. Joumard, Z. Samaras & S. Sorenson (1999), Methodology for calculating transport emissions and energy consumption. TRL report, n° PR/SE/491/98, Crowthorne, U.K., and European Commission, DG VII, ISBN 92-828-6785-4, Luxembourg.

INFRAS/IWW (1995) External Effects of Transport, UIC

INFRAS/IWW (2000) External Costs of Transport, Study for the International Union of Railways, Paris.

ISTAT, Annuario Statistico Italiano, various years.

ISTAT (1998), Statistica degli incidenti stradali, Roma

Link, H. (2000a) Accounts Approach for Infrastructure Costs. UNITE (UNIfication of accounts and marginal costs for Transport Efficiency). Contained in Link et al. (2000), Accounts Approach. Funded by 5th Framework RTD Programme. ITS, University of Leeds, Leeds October.

Link H, Stewart L, Maibach M et al. (2000b) Accounts Approach. UNITE (UNIfication of accounts and marginal costs for Transport Efficiency) Deliverable 2 Funded by 5th Framework RTD Programme. ITS, University of Leeds, Leeds October.

Lombard P.L., Molocchi A. (1998), I costi ambientali e sociali della mobilità in Italia, Franco Angeli, Roma

Lombard P.L., Molocchi A. (2000), Produzione, esercizio e smaltimento dei mezzi di trasporto: i costi sociali e ambientali, Franco Angeli, Roma

Macario et al. (2000) Accounts approach for Taxes, Charges and Subsidies. UNITE (UNIfication of accounts and marginal costs for Transport Efficiency). Contained in Link et al. (2000), Accounts Approach. Funded by 5th Framework RTD Programme. ITS, University of Leeds, Leeds, October.

Metroeconomica (2001), Monetary valuation of noise effects. Draft Final Report prepared for the EC UNITE Project, subcontracted to IER, Bath (UK).

McInnes, G. (1996), Atmospheric Emission Inventory Guidebook, a joint EMEP/CORINAIR Production, First Edition. EMEP Task Force on Emission Inventories, European Environment Agency, Copenhagen.

Ministero dei Trasporti e della Navigazione, Relazione Annuale Ministero della Marina Mercantile, various years

Ministero dei Trasporti e della Navigazione (1998), L’impresa Aeroporto in Italia, Istituto Poligrafico e Zecca dello stato Roma.

Ministero dei Trasporti e della Navigazione (2001), Piano Generale dei Trasporti, downloaded at the site http://www.infrastrutturetrasporti.it.

Ministero dei Trasporti e della Navigazione – DG POC (1994), Le ferrovie in concessione e in gestione governativa, Roma.

Ministero dei Trasporti e della Navigazione – DG POC (1996), 30 anni di Trasporto in Italia, Istituto Poligrafico e Zecca dello Stato, Roma

Ministero dell’Ambiente (1997), Relazione sullo stato dell’ambiente, Istituto Poligrafico e Zecca dello Stato, Roma.

Ministero dell’Industria (1997), Bilancio Energetico Nazionale 1996, Direzione Generale delle fonti di energia e delle risorse minerarie, Roma.

Ministero dell’Industria (1999), Bilancio Energetico Nazionale 1998, Direzione Generale delle fonti di energia e delle risorse minerarie, Roma.

Nellthorp J, Sansom T, Bickel P, Doll C and Lindberg G (2001). Valuation Conventions for UNITE. UNITE (UNIfication of accounts and marginal costs for Transport Efficiency) Working Funded by 5th Framework RTD Programme. ITS, University of Leeds, April 2001.

Ntziachristos L., Samaras Z., COPERT II: Computer Programme to Calculate Emissions from Road Traffic – Methodology and Emission Factors, European Topic Centre on Air Emissions, Thessaloniki, 1998.

OCSE (1993), Environmental Data Compendium, Parigi.

Posch M, Hettelingh JP, de Smet P, Downing RJ (1997) Calculation and Mapping of Critical Thresholds in Europe: Status Report 1997. RIVM Report No. 259101007. Coordination Center for Effects, National Institute of Public Health and the Environment (RIVM), Bilthoven, Netherlands.

Rabl, A, Spadaro, J (1999), Environmental Damages and Costs: an analysis of Uncertainties. Paper accepted for publication in Environment International.

Richardson, S. (2000), Atmospheric Emission Inventory Guidebook, A joint EMEP/CORINAIR Production, Second Edition. European Environment Agency, Technical Report, No. 30, Copenhagen.

Sansom, T., Nellthorp, J., Proost, S., Mayers., et al. (2000) The Overall UNITE Methodology UNITE (UNIfication of accounts and marginal costs for Transport Efficiency). Funded by 5th Framework RTD Programme. ITS, University of Leeds, Leeds, May.

Simpson, D, Olendrzynski, K, Semb, A, Storen, E, Unger, S (1997), Photochemical oxidant modelling in Europe: multi-annual modelling and source-receptor relationships. EMEP/MSC-W Report 3/97, Oslo.

Soguel, N. (1994), Evaluation monétaire des atteintes a l'environnement: Une étude hédoniste et contingente sur l'impact des transports, Imprimerie de L'evolve SA Neuchatel.

RECORDIT (2001), D6 - Imbalances and inefficiencies of the current pricing system. Project supported by the Commission of the European Communities – DG TREN Key Action 2: Sustainable Mobility and Intermodality.

Trukenmüller, A, Friedrich, R (1995) Die Abbildung der großräumigen Verteilung, chemischen Umwandlung und Deposition von Luftschadstoffen mit dem Trajektorienmodell WTM, in: Jahresbericht 1995, ALS Universität Stuttgart, pp. 93 - 108.

UNII - Unione Navigazione Interna Italiana, personal communication.

UPI - Unione Petrolifera (2001), Previsioni di domanda energetica e petrolifera italiana 2001-2015, Roma.

Vestreng, V. (2000), Tables of 1998 sector emissions in the ECE region. Pollutants: sulphur, nitrogen oxides, ammonia, NMVOCs, carbon monoxide, methane, carbon dioxide. Co-operative programme for monitoring and evaluation of the long range transmission of air pollutants in Europe (EMEP). http://projects.dnmi.no/~emep/emis_tables/ stab1.html, Last Update: 9/2000, Status from: 3/2001

Vestreng, V. and Støren, E. (2000), Analysis of UNECE/EMEP Emission Data, MSC-W Status Report 2000. Co-operative programme for monitoring and evaluation of long range transmission of air pollutants in Europe (EMEP), EMEP MSC-W Note, 1/00, Blindern, Norway.

# Glossary

| Accident Costs | Costs caused by transport accidents. These costs are directly related to material damage costs and medical costs, the administrative costs of police and insurance companies, the costs associated with production loss through accident related illness and fatalities and the costs of “suffering” associated with accidents (risk value). |
| --- | --- |
| Capital costs | The capital costs comprise the consumption of fixed capital and interest. Capital costs represent a high share of total infrastructure costs and are different to the annual capital expenditures. |
| Capital value | The capital value is the value of fixed capital measured either as a gross or a net value. The gross value represents the capital value of all assets still physically existing in the capital stock. It can thus be considered as an equivalent of production capacity. The net value represents the value of assets minus the meanwhile consumed fixed capital. The difference to the gross value is thus the loss of value due to foreseen obsolescence and the normal amount of accidental damage that is not made good by normal repair, as well as normal wear and tear. Methods for estimating capital values are the direct method (synthetic method) and the indirect method (perpetual inventory concept). |
| Congestion | Congestion arises when traffic exceeds road capacity so that the travelling speed of vehicles is slowed down. It can be defined as a situation where traffic is slower than it would be if traffic flows were at low levels. The definition of these „low levels„ (reference level) is complicated and varies from country to country (e.g. six service levels in the American HCM). |
| CORINAIR | Programme to establish an inventory of emissions of air pollutants in Europe. It was initiated by the European Environment Agency Task Force and was part of CORINE (COoRdination d’Information Environmentale) work programme set up by the European Council of Ministers in 1985. End of 1994 the EEA’s European Topic Centre on Air Emissions (ETC/AEM) took over the CORINAIR programme. |
| Earmarking | Direct connections between the financial source and the financial purpose, in order to secure financial resources. In practice, specific funds are used therefore (e.g. earmarking road pricing revenues and financing of road infrastructure or environmental measures). |
| Gross Domestic Product (GDP) | The GDP is the sum of all goods and services produced within a country and a year. GDP per capita can be regarded as the relative economic power of a country per inhabitant. |
| GVW | GVW is the gross vehicle weight and contains the weight of the vehicle itself and the weight of the payload. |
| HGV | HGV means heavy goods vehicles. Within this study they are defined as all goods vehicles with a maximum GVW equal or more than 3,5 tons. |
| Impact Pathway Approach (IPA) | Methodology for externality quantification developed in the ExternE project series. It follows the chain of causal relationships from pollutant emission via dispersion (including chemical transformation processes), leading to changes in ambient air concentrations from which impacts can be quantified using exposure-response functions. Damages are then calculated using monetary values based on the WTP approach. |
| Individual transport | Transport performed on the own account of users with their own vehicle for private reasons. |
| Infrastructure Cost | Cost category which comprises capital costs (depreciation and interests) and running costs for maintenance and repair, operation and administration, overheads and traffic police. |
| Infrastructure suppliers | are defined as the totality of public and private enterprises which are financing the provision and maintenance of the transport infrastructure for all modes (road, rail and water) within the urban area analysed. |
| LGV | LGV means light goods vehicles. Within this study they are defined as all goods vehicles with a maximum GVW equal or less than 3,5 tons. |
| NUTS | Nomenclature of territorial units for statistics; level 0 = countries, level III = départements, Kreise, etc. (depending on country considered). |
| Opportunity costs | The expressions "opportunity costs" and "shadow prices" are used synonymously within the Real Cost Scheme. They determine the value added for an individual in the case a good would not have been bought or built or in case negative effects of transport would not be present. Opportunity values are used for the evaluation of investments (capital costs), lost lives (statistical value of human life) or for the assessment of noise nuisance. |
| Passenger car unit (PCU) | PCU is used in order to standardise vehicles in relation to a passenger car. Speed and lengths differentials are most common. |
| Perpetual inventory method (PIM) | Perpetual inventory model: This is a method to estimate the asset value from a time series of annual investment expenditures. Annual new investments are cumulated and - according to their remaining life time - a depreciation will be calculated. The sum of these annual remaining asset values is equal to the total amount of the asset value. |
| PPP | PPP means purchasing power parity. PPPs are the rates of currency conversions, which equalise the purchasing power of different countries. This means that a given sum of money, when converted into different currencies at the PPP rates, will buy the same basket of goods and services in all countries. In particular, PPPs are applied if figures for specific products or branches shall be expressed in foreign currency (for example in ECU or in US $) because in these cases the use of official exchange rates is not appropriate. |
| Primary particles | Particles that are directly emitted. |
| Public Transport | PT subsumes all services that are supplied according to a pre-defined timetable in passenger and freight transport. The final user here pays an average fare. Typical PT is rail, bus, air and ferry services. The transport of an additional person or unit of goods does not cause in the short run additional vehicle kilometres, as scheduled vehicles are used, which are running anyway. In the long run, due to increased capacity use, additional or larger vehicles have to be scheduled. In the former case the marginal costs are zero, in the latter case the marginal costs are the costs per vehicle kilometre divided by the capacity use. |
| Replacement value/cost | The cost of replacing a particular asset of a particular quality with an asset of equivalent quality. Replacement cost may exceed the original purchase cost because of changes in the prices of the assets. |
| Risk value | The risk value represents the society’s willingness to pay for avoiding death casualties or injuries in transport. It reflects the decrease in social welfare due to the suffering and grief of the victims and their relatives and friends. The relevant cost elements are: Own risk value and suffering and grief of relatives and friends |
| Secondary particles | Particles, such as nitrates and sulphates that are formed in the atmosphere through atmospheric chemical reactions. |
| Supplier Operating Cost | Costs mainly related to costs incurred by supplier in its operations. |
| Survival function | Survival functions are used in rather refined perpetual inventory models. The survival function g (i) is based on the assumption that the service lives of assets within an investment vintage are dispersed around the mean. g (i) explains then which share of investments within an investment-vintage still exists in the capital stock after i years. The survival function is characterised by a downward slope of shares between 100 % (in the first year of investment) and 0 % (after exceeding the maximal lifetime of all assets in the investment vintage). |
| Synthetic method | One of the two main methods to value the existing road network (see also: perpetual inventory method). The synthetic method values the road network by estimating what it would cost to replace the road network with assets of equivalent quality. The method therefore involves measuring the existing physical assets, in terms of road length of particular types, bridges, etc, and then multiplying these measures of physical assets by unit replacement costs, such as the cost of constructing a motorway with the same physical characteristics as the existing one. |
| Vehicle category | Road: passenger car, motorcycle, bus, goods transport vehicles. Public transport: bus, tram, trolley bus, metro. Rail: electric passenger train, diesel passenger train, electric goods train, diesel goods train. Inland Waterways / Marine: Goods transport. Air: passenger, goods transport |
| VOSL | Value of statistical life: A unit often used to express individuals willingness-to-pay (WTP) for safety. The individual state (or reveal) a WTP for a small reduction in risk (dz) for a fatal accident; he is never asked the question about the value of life per se. If this risk change is summed over (n) individuals so that statistical the risk reduction will save one life we can also sum their WTP; this sum of the WTP then becomes the Value of statistical life (VOSL). VOSL = WTP*n = WTP/dz if n*dz = 1 |
| VOT | Value of time. The value of time is standardised within the UNITE accounts. |
| WTP | Willingness to pay: The direct or indirect response to questionnaire about individuals willingness-to-pay for a good. For example the WTP for higher safety. |
| YOLL | Year of life lost |

# Abbreviations

| Btkm | Billion tonne kilometres |
| --- | --- |
| CISPEL | Confederazione Italiana Servizi Pubblici Enti Locali |
| CNT | Transport National Account - Conto Nazionale dei Trasporti |
| CO2 | Carbon dioxide |
| COI | Cost of illness |
| dB(A) | Decibel, weighted with the A-filter. Logarithmic unit of sound pressure level. |
| ISTAT | Istituto Centrale di Statistica |
| GDP | Gross Domestic Product |
| GIS | Geographical Information System |
| GVW | Gross vehicle weight (weight of the vehicle itself and the weight of the payload) |
| HGV | Heavy goods vehicles |
| Kph | Kilometres per hour |
| KWh | Kilowatt hour |
| Laeq | Energy equivalent noise level |
| LGV | Light goods vehicles |
| LTO | Landing and take-off cycle |
| MCTC | Directorate-General for Road Traffic and Privately Operated Transport Services |
| MWh | Megawatt hour |
| n.a. | No data available |
| NMHC | Hydrocarbon |
| NMVOC | Non-methane volatile organic compounds |
| Nox | Nitrogen oxides (mix of NO and NO2) |
| NUTS | Nomenclature of territorial units for statistics; level 0 = countries, level III = départements, Kreise, etc. (depending on country considered) |
| PCU | Passenger car unit |
| PIM | Perpetual Inventory Model |
| p-km | Passenger kilometres |
| PM10 | Fine particles with a diameter of 10 µm and less |
| PM2.5 | Fine particles with a diameter of 2.5 µm and less |
| PPP | Purchasing power parity |
| PT | Public transport |
| SOC | Supplier operating costs |
| SO2 | Sulphur dioxide |
| UPT | Urban public transport |
| v-hours | Vehicle hours |
| v-km | Vehicle kilometres |
| VOC | Volatile organic compounds |
| VOT | Value of time |
| WTP | Willingness to pay |
| YOLL | Years of life lost |

# Abbreviations used in data tables

| – | No existing data category (for example sea ports in Switzerland) |
| --- | --- |
| 0 | Zero or approximately zero when compared to other data entries |
| . | Not applicable (for example the length of a sea harbour) |
| : | No data available |

The concession is the administrative tool by means of which the Italian State, implementing specific legal standards, entrusts a company with the construction and operation of motorway infrastructures for a set period of time (the period of the concession). The activities are regulated by special agreements between ANAS and the concessionaires, which fix the duration of the concession, and establish how tolls are to be adjusted.

Law 59/97 and DL 422/97: “Conferimento alle Regioni ed agli Enti Locali di Funzioni e Compiti in Materia di Trasporto Pubblico Locale” (“Conferring Local Public Transport Functions and Duties to Regions and Local Entities”).

Public airports are those built by the Italian State and are part of State Property for Air (demanio aeronautico), while private airports are built on lands outside the State Property and are managed by private stakeholders with a State licence.

Time series for physical data (network extension, etc.) are also available from 1950.

Local extra urban roads (comunali) are roads outside towns/villages linking the municipality with its own hamlets. CNT reckons their length (141 666 kilometres) since 1977 (last ISTAT – Italian Statistic Institute’s estimation) up to 1998 (last Ministry of transport’s estimation). This data about their length from 1978 to 1997 has been thus estimated by means a polynomial regression equation of 2nd order.

The last ISTAT statistic on the extension of municipal urban roads dates back to 1977. The Ministry of Transport provides an estimation for 1999 on the basis of a survey on Municipalities expenditures for municipal roads, and gives a total municipal roads network extension of 668 669 km, of which 171 779 km are urban, 312 149 extra urban and 187 745 are other extra urban (strade vicinali) (CNT 2000).

For urban light rails, until 1980 data included the length of the elevators in the towns of Genoa and Bergamo, too.

Rome’s subway has been working since February 9th 1955 and Milan’s one since November 1st 1964.

The aprons are aircraft parking areas within the airport surface.

The study focuses on urban areas congestion, aiming to capture a share of the total volume of the phenomenon in the order of 60 to 80%. These shares are quoted in the EC Green Paper (CEC 1995), on the basis of a study by Newbery (Reforming Road Taxation, The Automobile Association, Hampshire, 1995), and are confirmed by a study by British Telecom (CERTE, Estimating the external costs of UK passenger transport: the first step towards an efficient transport market, Canterbury, 1994).

The underlying assumption is that in smaller cities the lower population density together with a reduced time dedicated to individual trips results in a lower time lost per inhabitant.

Even though the application of the model results in a wide variety of behaviours among the municipalities involved, the study shows a certain behavioural homogeneity of the road networks if the municipalities are divided in 3 clusters, namely large municipalities (more than 500 000 inhabitants), medium sized municipalities (from 500 000 to 100 000 inhabitants), small municipalities (less than 100 000 inhabitants) (ACI-ANFIA, 2001).

In Italy, the reporting of accidents involves at least three different levels ed institutions: police authorities, Carabinieri, and municipal police. While the information gathered by police authorities and Carabinieri is integrated at the national level, the municipal police operates under different regulations depending on local circumstances and geographical locations, i.e. the rate of accident reported in North regions is higher than in South of Italy. As result, a loss of information in the overall amount of accidents can occur.

For instance, the Province of Modena, the Milan municipality, the Region of Emilia-Romagna

A significant exception is made for vehicles for the disabled, where VAT rate is 4%, but it was not possible to quantify this exemption.

The nature of these contributions might be controversial, since they could be assimilated to subsidies. We prefer to consider them as tariff revenues and not as subsidies, thus following the FS accounting practice (see Ferrovie dello Stato, 2000b), since the State acts within this relationship as a real customer, and buys services that FS would not have produced in other cases (social not remunerative services).

Deliberation by CIPE published in G.U. no 17, 22 January 2000 and decree by the Ministry of Transports 43T/2000 published in G.U. 94, 21 April 2000, which develops principles and criteria contained in the DPR 277/98.

SIMPT (Sistema Informativo per il Monitoraggio e la Pianificazione del Sistema dei Trasporti) is the system used by the Ministry of Transport for the monitoring of the transport system (demand and supply), the definition of short and long term policies and the support to the General Transport Plan implementation, the supply of information to transport operators.

In particular, the estimation from ACI-ISTAT (1998), with updating at 2001, and the evaluation from Molocchi, Lombard (1995, 2000) must be mentioned.

Estimation from CENSIS (1997)

The amount of aeronautic charges for passengers, freight and airlines is determined yearly by the Ministry of Transport. The tariffs considered within this account for the year 1998 are drawn from the decree of the Ministry of Transport of January the 20th 1997,  published in GU n. 59, serie generale, March the 12th 1997.

The category “General Aviation” includes all the traffic excluding the commercial one (scheduled air traffic, charter and aero taxi), more specifically aero club, aviation schools, little private aircrafts and the air working services.

A charge is a levy that requires a direct and clear service in proportion of the payment on the part of the government. A tax is a levy that must be paid with no discernible service required from the government or a service that is not in proportion to the payments

MURE (Mesures d’Utilisation Rationnelle de l’Energie) has been designed and developed by a team of European experts, led and co-ordinated by ISIS (Istituto di Studi per l’Integrazione dei Sistemi - Rome). The MURE team further includes INESTENE (F), ISI-Fraunhofer (G) and March Consulting Group (UK).

An allocation of running infrastructure variable costs (maintenance costs) to vehicle types (motorcycles, passenger cars, buses, LGV, HGV) can be made applying the marginal infrastructure costs for heavy goods vehicles in 1998 (cfr. RECORDIT, 2001) to the HGV vehicles-km for 1998. We calculated maintenance infrastructure costs due to HGV (3 248 € million) that represents the 55% of total maintenance costs (the total running costs net of infrastructure operator costs are 5 903 € million, see Table 42).