COMPETITIVE AND SUSTAINABLE GROWTH

(GROWTH)

PROGRAMME

Deliverable 12: The Pilot Accounts of UNITE The Pilot Accounts of Sweden - Annex 8a

Final Version 4.7

June 2003

Authors: 
Ola Nääs, Gunnar Lindberg

Contract: 1999-AM.11157 Project Coordinator: ITS, University of Leeds Funded by the European Commission 5th Framework – Transport RTD

UNITE Partner Organisations ITS/UNIVLEEDS (UK), DIW (De), NEI (Nl), CES/KUL (Be), TIS.PT (Pt), IWW/UNIKARL (De), VTI (Se), IER/USTUTT (De), CERAS/ENPC (Fr), HERRY (Au), EIET/ULPGC (Es), ISIS (It), STRATEC (Be), SYSTEMA (Gr), JP-TRANSPLAN (Fi), VATT (Fi), ECOPLAN (Ch), INFRAS (Ch), EKONO (Fi), EKI (Se)

UNITE

1999-AM.11157

UNIfication of accounts and marginal costs for Transport Efficiency

Deliverable 8:  The Pilot Accounts of Sweden - Appendix Report

This document should be referenced as:

Nääs, O. Lindberg, G. ‘The Pilot Accounts of Sweden - Appendix Report’

Version 4.5

Authors:	as above, with contributions from other UNITE partners

PROJECT  INFORMATION

Contract no: 1999-AM.11157:

UNIfication of accounts and marginal costs for Transport Efficiency

Website:	www.its.leeds.ac.uk/unite

Commissioned by: European Commission – DG TREN; Fifth Framework Programme

Lead Partner:   Institute for Transport Studies, University of Leeds (UK)

Partners:  ITS/UNIVLEEDS (UK), DIW (De), NEI (Nl), CES/KUL (Be), TIS.PT (Pt), IWW/UNIKARL (De), VTI (Se), IER/USTUTT (De), CERAS/ENPC (Fr), HERRY (Au), EIET/ULPGC (Es), ISIS (It), STRATEC (Be), SYSTEMA (Gr), JP-TRANSPLAN (Fi) VATT (Fi), ECOPLAN (Ch), INFRAS (Ch), EKONO (Fi), EKI (Se)

DOCUMENT  CONTROL  INFORMATION

Status: 	Accepted

Distribution:	Commission and UNITE partner

Availability:	Public, once accepted as D12 Annex

Filename:	UNITE D12 Annex 8a

Quality assurance:

Coordinator’s review:

Signed:          			   			Date:

Table of Contents	Page:

Table of contents	i

Abbreviations	ii

Sammanfattning/Executive summary	v

1	Introduction	1

1.1	Study Context and Objectives	1

1.2	The Accounts Approach of UNITE	1

1.2.1	Aims of the Pilot Accounts	1

1.2.2	Core, Supplementary and Excluded Data in the Pilot Accounts	2

1.2.3	The six UNITE Pilot Account Cost Categories	2

1.2.4	The Transport Modes of the Pilot Accounts	4

1.3	Results presentation and guidelines for interpretation	4

1.4	The Structure of this Report	5

2	Input Data	6

2.1	General Input Data	6

2.1.1	Basic Economic data	6

2.1.2	Transport volumes	7

2.2	Infrastructure Costs	7

2.3	Supplier Operating Costs	8

2.4	User Cost/Delays	9

2.5	Accident Costs	9

2.6	Environmental Costs	11

2.6.1	Air Pollution and global warming	11

2.6.2	Noise	12

2.7	Taxes, Charges and Subsidies	12

3	Methodological Issues	14

3.1	General data	14

3.2	Infrastructure Costs	14

3.2.1	Road	15

3.2.2	Rail	15

3.2.3	Aviation	16

3.2.4	Maritime transport	16

3.3	Supplier Operating Costs	17

3.4	User Cost / Delay	18

3.4.1	Road	18

3.4.2	Rail	19

3.4.3	Aviation	19

3.5	Accident Cost	19

3.5.1	Road	20

3.5.2	Rail	21

3.5.3	Public transport	21

3.5.4	Aviation	21

3.5.5	Maritime	22

3.6	Environmental Cost	23

3.6.1	Air pollution and global warming	23

3.6.2	Noise	23

3.7	Taxes, Charges and Subsidies	24

4	The Results in Detail	25

4.1	Basic Economic Data	25

4.2	Transport volumes	25

4.2.1	Road	26

4.2.2	Rail	27

4.2.3	Public transport	27

4.2.4	Aviation	28

4.2.5	Maritime transport	29

4.3	Infrastructure Costs	29

4.3.1 Road	30

4.3.1	Rail	31

4.3.2	Aviation	34

4.3.3	Maritime transport	35

4.4	Supplier Operating Costs	35

4.5	Delay costs due to congestion	38

4.5.1	Road	38

4.5.2	Rail	38

4.5.3	Aviation	38

4.6	Accident Cost	38

4.6.1	Road	40

4.6.2	Rail	44

4.6.3	Public transport	45

4.6.4	Aviation	46

4.6.5	Maritime	47

4.7	Environmental Cost	48

4.7.1	Road	49

4.7.2	Rail	50

4.7.3	Aviation	51

4.8	Taxes, Charges and Subsidies	51

4.8.1	Road	52

4.8.2	Rail	52

4.8.3	Public Transport	52

4.8.4	Aviation	53

4.8.5	Maritime transport	53

5	Final Presentation of the Results per Mode	54

5.1	The Swedish result in perspective	54

5.2	Basic indicators	58

5.3	Road transport	58

5.4	Rail transport	65

5.5	Public transport	70

5.6	Aviation	71

5.7	Maritime transport	75

6	Conclusions	80

7	References	82

Abbreviations

| ATS | Flygtrafikledning (Air Traffic Services) |
| --- | --- |
| BV | Banverket (The Swedish National Rail Administration) |
| DIW | Deutsches Institut für Wirtschaftsforschung e.V. |
| GDP | Brutto national produkt (Gross Domestic Product) |
| IBAB | Inlandsbanan AB (a 1 053 km railway track belonging to the municipal authorities along the track) |
| KF | Svenska kommunförbundet (The Swedish Municipality association) |
| KI | Konjunkturinstitutet (The National Institute of Economic Research) |
| LFV | Luftfartsverket (The Swedish Civil Aviation Administration) |
| PIM | Perpetual Inventory Model |
| Pkm | Personkilometer (Passenger kilometres) |
| SCB | Statistiska centralbyrån (Statistics Sweden) |
| SFV | Sjöfartsverket (The Swedish Maritime Administration) |
| SHSF | Sveriges Hamn- och Stuveriförbund (The Swedish Harbour- and stevedore association) |
| SIKA | Statens Institut för kommunikationsanalys (The Swedish Institute for Transport and Communications Analysis) |
| Skm | Utbudskilometer (Supply kilometres) |
| SL | Stockholms lokaltrafik (Stockholm public transport authority) |
| SLTF | Svenska lokaltrafikföreningen (The Swedish Public Transport Union) |
| SNI | Svensk näringsindelning (Swedish Standard Industrial Classification) |
| SVEDAB | Svensk-Danska broförbindelsen AB (The company responsible for the Swedish land connection from the bridge between Denmark and Sweden. |
| Tkm | Tonkilometer (Tonne kilometres) |
| VV | Vägverket ( The Swedish National Road Administration) |
| Vkm | Fordomskilometer (Vehicle kílometres) |
| WD/RS | Av-nedskrivning/återföring (Write downs/reversals) |
| ÖSK | Öresundsbro konsortiet (The company responsible for the bridge between Denmark and Sweden) |

Sammanfattning / Executive Summary/

UNITE projektet är ett stort europeiskt forskningsprojekt med ett 20-tal inblandade universitet, institut och konsultföretag över hela Europa och finansieras av EU. Projektet täcker alla transportslag och 18 länder i Europa. Det innehåller tre huvudblock; Marginalkostnader, Bokföring, samt Integration. Det första blocket ’Marginalkostnader’ strävar efter att ta fram en teori över trafikens marginalkostnader samt att skatta marginalkostnader i ett antal utvalda fall. ’Bokföring’ strävar efter att samla in kostnadsinformation för samtliga trafikslag inom respektive land. Denna insamling, inklusive viss metodbeskrivning, finns redovisat i denna rapport. Slutligen, ’Integration’, söker sammanföra det bokföringsmässiga tänkandet med marginalkostnadsansatsen.

För en person någorlunda bevandrad i svensk trafikpolitik kan projektets delsyfte med bokföring kännas föråldrat. Sverige hade ju tidigare en trafikpolitik som betonade både ett totalkostnadsansvar och ett marginalkostnadsansvar för trafikslagen, detta dubbla krav togs ju bort 1998 och kvar blev bara ett marginalkostnadsansvar. Man ska dock vara medveten om att många stora och inflytelserika medlemsländer i Unionen har svårt att ta till sig ett renodlat marginalkostnads resonemang. Förhoppningsvis kommer slutsatserna från detta projekt bringa olika ståndpunkter närmare varandra.

Kostnadsansvarsutredningar inom trafiksektorn har genomförts ett flertal i Sverige. Den senaste vi har överblick över är Dsk 1992:44 ’Trafikavgifter på samhällsekonomiska villkor’ (Hansson och Lindberg). Jämfört med dessa tidigare utredningar innehåller denna rapport ett antal nya dimensioner:

- Infrastrukturkostnaden är beräknad som en kostnad och inte som årliga utgifter

- I vissa fall har vi sökt täcka trafikoperatörens kostnader

- Däremot är trängselkostnaden styvmoderligt behandlat i denna utredning liksom tidigare

- Olyckskostnaden är baserad på teorier nära kopplade till marginalkostnadstänkandet

- Miljökostnaden är beräknat med en avancerad ’Impact Pathway model’ (ExternE) vid Universitetet i Stuttgart.

- Liksom tidigare finns skatter och avgifter med men här inkluderas även Momsen som ligger ’ovanpå skatterna’.

Vi sammanfattar inte här de enskilda resultaten utan hänvisar till avsnitt 5 för en sammanfattning per trafikslag och avsnitt 4 för en sammanfattning per kostnadsslag. Alla resultat är uttryckta i Euro (€) i prisnivå 1998. Konvertering från kronor har skett med kursen 8.92 kr/€. Några kortare metoddiskussioner finns i avsnitt 3 och definition av vad vår information kommer ifrån i avsnitt 2. Strukturen följer en given mall som alla länder följer.

Erfarenheten från detta arbete är att det är ett mycket tidskrävande jobb som ställer stora krav på omdömet hos den som sammanställer informationen. Det är snarare regel än undantag att sifferuppgifter som borde vara lika skiljer sig åt mellan olika källor. Sådant är frustrerande och tidskrävande. Vi kan alltså konstatera att databaser och statistik källor sällen är organiserade så att det underlättar insamlandet av den här typen av information.

Samtidigt måste vi vara självkritiska mot projektet. Möjligen på grund av de ’kultur'-skillnader som finns mellan medlemsländer, vilket vi antydde ovan, har inte bokföringens syfte klart uttalats. Det talas i vissa fall om ett ’open information system’. Även om det låter bra vet vi att ramverket kring ett öppet system som tillåter många olika användningsområden ger lite styrning vid de många val som måste göras. Ska man fortsätta med denna typ an arbete måste ramverket stramas upp betydligt.

Trots detta kan vi dra några slutsatser rörande data tillgången som är värda att ta med sig. Ska vi sammanfatta de två svåraste områdena så är det att sammanställa kommunala kostnader (tex vägar eller flygplatser) samt transportslaget ’Public transport’ som skär tvärs över olika transportslag. I övrigt har vi observerat:

- Infrastrukturkostnader har vi ganska bra överblick över statliga vägar och järnvägar där vi nyttjat 40 åriga tidsserier. För statliga flygplatser har vi också bra data även om det är svårt att härleda kostnaderna till olika funktioner. För kommunala vägar och flygplatser har vi enbart några stickprov som räknats upp till Sverige nivå; för flygplatser är det ett mindre problem eftersom de största är statliga medan problemet är större för vägarna.

- För operatörens kostnader (supplier operating costs) har vi nyttjat detaljerad data från Nationalräkenskaperna. Däremot saknas ju här företagens egna transport verksamhet; dvs. om företag A köper transporter av B finns det med men om A sköter transporterna med en egen transportavdelning finns det inte med.

- Vi har bra information, dock med bortfall, över personskador men betydligt sämre för materiella kostnader. För transportslag där person olyckor är dominerande, t.ex. biltrafik, blir kostnadsuppskattningen bra om vi accepterar officiella värderingar per personskada. För andra transportslag, t.ex. sjöfarten har det visat sig vara svårt att finna information om kostnaden för skadorna på fartygen; de kostnader som redovisas utgörs av personskadekostnader.

- Miljökostnaden har beräknats med en Impact Pathway modell. Ansatsen är så pass intressant att vi för närvarande håller på att genomföra en mer detaljerad analys för Sverige i samarbete med universitetet i Stuttgart.

- Skatter och avgifter är ett svårt område. Skatterna är någorlunda lätta att få information om även om disaggregeringar är svåra att göra. Däremot är subventioner en ’djungel’ som vi inte lyckats bena ut på ett tillfredställande sätt i denna rapport.

Med dessa ord har vi gett in ingång till rapporten och rekommenderar läsare att börja med kapitel 5.

Ola Nääs

Gunnar Lindberg

Borlänge i april 2002

# Introduction

## Study Context and Objectives

This Annex Report contains the full version of the Swedish pilot accounts developed within the UNITE project. It serves as background report for the results presented in UNITE report deliverable 8, which covers transport cost for a number of European countries. The Annex Report gives detailed descriptions the input data and their reliability, quality and on the methodology used. However, the general and detailed discussion of the accounts approach has been presented in Link et al. 2000 and will only be summarised in this document. In addition to the core accounts for 1998 the Annex Report also presents results for 1996 and a forecast for 2005. A statistical annex is accompanying the Swedish Annex Report.

The UNITE project endeavours to provide accurate information about the costs, benefits and revenues of all transport modes including the underlying economic, financial, environmental and social factors. To achieve this goal, three main areas of research are carried out, known as “transport accounts”, “marginal costs” and “integration of approaches”.

This Appendix Report belongs to the research area “transport accounts”. The pilot accounts are the actual, feasible accounts given the available data for the 18 countries that UNITE covers. They can be used to assess the costs and revenues of transport per transport mode.

## The Accounts Approach of UNITE

### Aims of the Pilot Accounts

The pilot accounts attempt to show the general relationship between costs of transport and the revenues from transport pricing and charging in the country studied. The aims and role of the pilot accounts are discussed in detail in Link et al. (2000). It should be stressed that the accounts are aimed at providing the methodological and the empirical basis for in-depth policy analysis rather than serving as a guide for immediate policy actions such as setting higher/ lower prices and charges or shutting-down transport services/ links in order to achieve cost coverage. The pilot accounts are defined as stated in the box below.

The pilot accounts to be elaborated in UNITE compare social costs and / charges on a national level in order to monitor the development of costs, the financial taxes balance and the structure and level of prices. Accounts can therefore be seen as monitoring and strategic instruments at the same time. They have to consider the country-specific situation and the institutional frameworks.

The pilot accounts show the level of costs and charges as they were in 1998 (and 1996 respectively) and provide a workable methodological framework to enable regular updating of transport accounts. Furthermore, an extrapolation for 2005 is given. The choices of additional accounting years (1996 and 2005) were motivated to show a comparison between years and to give a good indication of trends in transport for the near future. Furthermore, the inclusion of 1996 enables to rule out any major statistical abnormalities that may occur only in one year, for example very high infrastructure cost due to tunnelling operations or higher than average accident costs because of major accidents occurring in 1998.

### Core, Supplementary and Excluded Data in the Pilot Accounts

The pilot accounts have been divided into the classes “core data” and “supplementary data”. Core data is the data necessary to do a full basic review of the country accounts. Supplementary data falls into three categories:

- Firstly, data that adds additional information to the core accounts is described as supplementary data.

- Secondly, for several cost categories being evaluated there is no standard methodology for the valuation of effects. An example of this is the valuation of loss of biodiversity due to transport infrastructure. Even though a valuation method has been developed for the UNITE pilot accounts, we feel that the level uncertainty (due to the lack of comparative studies) is high enough to warrant the information to be classified outside of the core data where efficient and well tried valuation methods have been utilised.

- Thirdly, some costs, which can be estimated and valuated, are borne by the transport users themselves (for example delay costs). These costs and the methods used to valuate them present valuable further information to the reader, but can not be considered to be part of the overall costs of transport as defined by UNITE.

### The six UNITE Pilot Account Cost Categories

Data for the pilot accounts have been collected within six cost and revenue categories that are described in  (Link et al., 2000) and are summarised in the following.

Infrastructure Costs

For the pilot accounts, data for the assessment of infrastructure costs are structured to show the capital costs of transport infrastructure (including new investments and the replacement of assets) and the running costs of transport infrastructure (maintenance, operation and administration) for all modes of transport studied.

Supplier Operating Costs

All monetary costs incurred at transport operators for the provision of transport services are documented in the category supplier operating costs. Ideally, the data is structured to show what costs are incurred for vehicles, for personnel and for administration. However, this depends on data availability and will differ from country to country. Since the official categorisation in Sweden differs from the proposed categorisation in UNITE and data from official sources are easy available, we use the official categories.

Transport User Costs

In the category transport user costs, the costs of delay and delay-caused additional operating costs are estimates. This estimation is intended to provide supplementary data for the accounts and is carried out for all transport modes, provided data are available. This data is classified as supplementary data because the bulk of these costs are borne by transport users as a whole (transport system internal costs).

Accident Costs

The loss of lives and the reduction of health and prosperity through transport accidents are of major concern to all countries and to the European Commission. In this section of the accounts, the health related accident costs are calculated by assessing the loss of production, the risk value and the medical and non-medical rehabilitation of accident victims. Where the available data allows, the damage to property and the administrative costs of accidents are considered, too. The external part of accident costs (defined in this report as accident costs imposed by transport users on the rest of society) is included in the core section of the accounts. Total accident costs however, include a substantial proportion of costs imposed by one user on others and are therefore treated as supplementary costs (transport system internal costs).

Environmental Costs

A wide range of transport related environmental impacts and effects is considered in this section of the accounts. Included in this cost category are: air pollution, global warming, noise, changes to nature and landscape and nuclear risks. The valuation of these environmental effects is carried out for all transport modes, provided adequate data is available.

It is the aim of UNITE to calculate these costs according to a harmonised methodology. Against this background, it was decided to carry out all calculations for air pollution and noise with the EcoSense model running at the University of Stuttgart. The model was developed within the series of ExternE Projects funded by the European Commission. A description is given in a separate report.

Taxes, Charges and Subsidies

Transport taxes and charges are exceptionally heterogeneous throughout Europe. In this section, the level of charging and taxation for the transport sector is documented for each mode of transport. In accordance with the UNITE methodology, VAT on taxes are included. Wherever possible, the revenues from taxes and charges are shown for fixed taxes and charges and variable ones.

A further part in this area is reporting on subsidies. Subsidies can take the form of direct monetary transfers to infrastructure provider and transport services. In addition, we include tax exemptions in subsidies, i.e. when some modes have a reduced tax level for some commodity, for example lower diesel tax. However, it should be noted that a complete reporting on subsidies would require extremely time-consuming analyses of public budget expenditures at all administrative levels. The subsidies reported in the pilot accounts refer mainly to direct subsidies (e. g. monetary payments from the state to economic subjects).

### The Transport Modes of the Pilot Accounts

The modes covered in the Swedish accounts are road, rail, public transport, maritime transport and aviation. The level of disaggregation into types of networks and nodes, means of transport and user groups depends on data availability and relevance. Table 1 summarises this disaggregation for the Sweden Pilot Account. The disaggregation is not the same through the whole report. It differs due to differences in the sources. For all modes, except for the mode public transport where it was not possible to calculate infrastructure cost, the following items are estimated: infrastructure cost, supplier operating cost, user cost, accident cost, environmental cost, taxes, charges and subsidies.

The modes road, rail, aviation and maritime are treated as infrastructure. In addition, for rail, aviation and maritime we look at the supplier of transport services. Public transport is only treated as supplier of transport services and can be seen in the Swedish accounts as additional information where we try to allocate some of cost originally presented in road or rail as belonging to public transport.

Table 1-1 The modes, network differentiation, transport means and user breakdown

| Transport modes | Infra-structure | Supplier of transport services | Network differentiation | Means user breakdown (and unit of measurement) |
| --- | --- | --- | --- | --- |
| Road | Yes | No | State roads Municipal roads Private roads In some cases the categories are: Urban roads Regional roads | Motorcycles Passenger cars Buses Light goods vehicles LGV Heavy goods vehicles HGV (Pkm, vkm, tkm, t) |
| Rail | Yes | Yes | Single track Double track Electrified track | Passenger transport Freight transport (Pkm, tkm, t) |
| Public transport* | No | Yes | Tramways Underground Buses Train County train | Passenger transport Mill. passenger (Skm, pkm,) |
| Aviation | Yes | Partly | State owned airports Military with civil traffic Municipal airports | Passenger transport Freight and mail transport Number of starts Number of passengers (Pkm, tkm, t) |
| Maritime transport | Yes | Partly | Fairways Harbours | Freight transport Passenger transport (Pkm, tkm, t) |

* The transport mode "urban public transport" offers difficulties. Ideally, the categories urban/local buses, tramways and underground are summarised under this mode. In some cases this separation is not feasible. The infrastructure costs of local/urban buses, for example, are included in the road infrastructure costs. Besides that, most of the statistics in Sweden does not separate urban public transport from overall public transport. Against this background we use the term public transport. In the mode public transport the category infrastructure cost is excluded because of double accounting within the modes road and rail.

## Results presentation and guidelines for interpretation

All results are documented separately for each cost category and are summarised in modal accounts covering all cost and revenue categories (chapter 5 of this report). These results present a comprehensive estimation of transport costs and revenues. They are however, not a total estimation of transport costs. Each cost category could include data in further areas and a definite border had to be drawn around the data to be collected for this project. It should be noted that due to the separation into core and supplementary data with different levels of uncertainty and with different types (costs borne by transport users themselves versus external costs) a simple summing up of the different cost and revenue categories to totals and the calculation of a cost recovery rate is not sensible. As an example - the capital cost in municipal airports is not fully comparable with the capital costs in the state owned airports due to differences in the definitions of fixed assets. Another problem is the definition of public transport.

## The Structure of this Report

This report contains four major parts:

- In chapter 2, the input data used to calculate the figures given in the UNITE Pilot Accounts is discussed. The tables with data are reproduced in a statistical annex.

- Chapter 3 deals with methodological issues. In some cases, it was not possible to apply the methodology developed in Link et al. (2000) and very briefly summarised in section 1.2.3 above. Chapter 3 concentrates on the methodology that was feasible to apply to the Swedish pilot accounts.

- The fourth chapter of the report finally deals with the results of the calculations and their interpretation in detail. The descriptions in these chapters are organised along the categories stated in section 1.2.3 above.

- In the last chapter, the detailed results of chapter 4 are finally brought together per mode. This final presentation of the results is the same for the different UNITE countries.

# Input Data

- This chapter is divided into six sections where we make a presentation of the most important specific input data on which the assessment of the cost and revenue categories of UNITE is based. The categories are:

- Infrastructure costs (2.2),

- Supplier operating costs (2.3),

- Delay costs due to congestion (2.4),

- Accident costs (2.5),

- Environmental costs (2.6), and,

- Taxes, charges and subsidies (2.7).

- Each of the six sections shows the sources of the input data and its availability. Because of differences in the sources trustworthiness, we also make a judgement about its reliability in order to make the information transparent. The table below summarises our judgement. The presentation of the input data is restricted to the most important values and figures but does not aim to reproduce all the data that has been collected for the calculations.

Table 2-2 Reliability of input data

|  | Road | Rail | Urban Public Transport | Aviation | Maritime / Inland waterway |
| --- | --- | --- | --- | --- | --- |
| Infrastructure Cost | Good on state road, poor on municipal roads and very poor on private roads | Good (Poor on private railways, which are a minor part of Swedish railways) | No data collected | Good on state owned airports Acceptable on municipal airports | Good on fairways and harbours |
| Supplier Operating Cost | Very good. Based on official structural business statistics from Statistics Sweden. Non existing for in-house transport, i.e. transport cost within a company that is not a registered transport company. |  |  |  |  |
| User Cost | Not available | Rough estimate | Not available | Rough estimate | Not available |
| Accident Cost | Good, based on police reported accidents | Good for state owned railways | Uncertain, covers metro and tram only. | Good Difficulties with the cost of material damages | Good Difficulties with the cost of material damages |
| Environmental Cost | Good, but a high level of aggregation is used |  | No data collected | Good, but a high level of aggregation is used | Not available |
| Taxes, charges and subsidies | The result in this category must be seen as a survey on the surface. The area is too big and too time consuming for the Pilot Accounts. Despite that the figures themselves are reliable, but the aggregation and summing up is very unreliable. |  |  |  |  |

## General Input Data

### Basic Economic data

All the figures are adjusted to prices 1998 and converted into Euro at the exchange rate SEK 8,92.

Table 2-3 Input data, availability and reliability

|  | Input data: | Data availability and reliability: |
| --- | --- | --- |
|  | The basic economic data is based on figures from Statistics Sweden (SCB), the National Institute of Economic Research (KI) and from valuation conventions for UNITE (Nellthorp et.al 2001) | The data is recorded on yearly basis and there is no uncertainty about the figures, except for the possibility of getting revised figures, which can cause discrepancies. |

### Transport volumes

There are a number of sources for transport volumes where SIKA and SCB and trade organisations offer many figures. Due to different definitions between different sources, one can get different figures. The way to describe transport volumes includes a huge amount of different methods. However, we found the figures from the sources below suitable to put together into the same tables.

Table 2-4 Input data, availability and reliability

|  | Input data: | Data availability and reliability: |
| --- | --- | --- |
| Road | Appendix 5.2 vehicle kilometres and fuel consumption 1994-1999 (VV) | The availability and reliability is good. |
| Rail | SIKA report, railways 1996-1997, SLTF trade statistics 1998. | The report from SIKA is of good reliability but the figures for 1998 is not available until the end of 2001.The figures from SLTF are easy to get but the reliability is not total, because mistakes might have been done during the data collection. |
| PT | SLTF trade statistics 1996 and 1998, SIKA communications nr 4-2000. | The report from SIKA have a good reliability and the characteristics of the figures from SLTF is the same for PT as for rail. |
| Aviation | SIKA air traffic 1999, SLTF trade statistics 1998, SIKA. | The same as for public transport. |
| Maritime | SIKA | The same as for public transport.The data on passenger kilometre (pkm) has a big confidence interval. |

## Infrastructure Costs

Table 2-5 Input data, availability and reliability

|  | Input data: | Data availability and reliability: |
| --- | --- | --- |
| Road | The infrastructure costs for road is based on 1) the business accounts from the Swedish National Road Administration (VV). The VV is responsible for all the state roads (national trunk roads, other national roads and county roads). 2) A yearly report (municipal road maintenance and park attention) from the municipal association (Kommuförbundet) which covers the municipal roads and give some figures for the private roads. 3) Statistical Yearbook of Sweden published by Statistics Sweden (SCB). Forecast data stems from National Plan for Road Transport System 1998-2007. | All the data except the forecast data can easily be obtained each year. The data on state roads are very reliable, whereas the data on municipal roads is little less reliable. The reason is that municipal authorities have different software accounting system. The figures also originate from a survey where the authorities have answered a questionnaire; missing answers have been extrapolated and there might be some mistakes done in that procedure. The data on private roads are even more unreliable since they stem from approximations rather than accounting. The forecast data must be seen as very unreliable since the economic development has changed after the forecast was made and the state budget with appropriations have has revised. |

| Rail | The infrastructure costs for rail stems from 1) the annual report from the Swedish National Rail Administration (BV). The BV accounts for about 75 % of total length in track. 2) Statistical Yearbook of Sweden published by SCB. Forecast data stems from BV, ‘stomnätsplan 1998-200’7 and ‘länstransportplan 1998-2007’. | The data is easy to get and, except for the forecast data, it is published each year. The data for the years 1996 and 1998 is of very high quality since it comes from business accounts and from SCB. The forecast data must be seen as very unreliable since the economic development has changed after the forecast was made and the state budget with appropriations has been revised. |
| --- | --- | --- |
| PT | Infrastructure costs for public transport cannot be separated from the modes road and rail. Therefore, no infrastructure costs is calculated within this mode. |  |
| Aviation | The accounts for aviation infrastructure in the Swedish pilot account cover the 19 State owned airports and 8 out of 28 municipal, military and private airports. These eight airports stand for about 70 % of passenger traffic and about 44 % of the landings out of the 28 airports. Out of the scope are two private airports, one of which becomes private in 1997, and 18 municipal and military airports. The information stems from annual financial reports from the Swedish National Civil Aviation Administration (LFV) and a survey of the eight municipal airports from the same authority, (airport comparison- state owned and municipal airports). | The data is easy to get and it is published each year. The data covering the state owned airports stems from annual reports from LFV and is very reliable. The figures that describe the municipal airports must be regarded as less reliable since, in the survey, the airports themselves delivered the figures, and there might be some mistakes made. |
| Maritime | In the case of maritime transport, the sources are the annual reports from the Swedish Maritime Administration (SFV) and a yearly aggregated report about the Swedish harbours (harbour traffic and economy) from the association Swedish Harbours. Forecast data stems from strategic report to SAMPLAN / harbour structure and shipping. The report concentrates on harbours and waterways used by commercial freight traffic. Out of the scope are the fishing ports and some inland waterways used by pleasure traffic and passenger traffic for pleasure purpose. The inland waterways with freight traffic are under the SFV control and their asset value and running costs included in SFV annual report. Inland waterways do not appear as a category in the Swedish accounts because it would be to time consuming to separate inland waterway traffic and harbours from overall waterway traffic and harbours. | The data is easy to get and except for the forecast data, it is published each year. The data from SFV is of very high quality. But the running costs also include costs, which do not fit into the infrastructure running costs, e.g. environmental efforts and research. It might be possible to distinguish such items from the pure infrastructure running costs, but such work is very time consuming. Regarding the harbours, one must be aware that the figures contain municipal harbour administration, harbour and longshore companies as well as pure longshore companies. There is no way to separate those categories from each other with the available data from Swedish Harbours. To examine each harbour would be too time consuming and perhaps impossible. The forecast data must be seen as very unreliable since the economic development have changed after the forecast was made, and the state budget with appropriations have been revised. |

## Supplier Operating Costs

Table 2-6 Input data, availability and reliability

|  | Input data: | Data availability and reliability: |
| --- | --- | --- |
| PT | For the supplier operating cost, the official structural business statistics was used. The structural business statistics have a different categorisation than suggested in UNITE. The structural business statistics is made by the SCB and it shows the overall income statement and the balance sheet of the Swedish companies excluding small private enterprises (only the corporation sector is included). See the statistical annex for an example. | The data is easy to get and it is published each year. The data is of high quality but a minor uncertainty occurs because the data include only the corporate sector. However, it should be remembered that the cost refers only to the transport sector. Companies that do not belong to the transport sector do transport services on their own account but will not be recorded here. Vehicle ownership has been used to give an idea of the magnitude of the problem. |

## User Cost/Delays

Table 2-7 Input data, availability and reliability

|  | Input data: | Data availability and reliability: |
| --- | --- | --- |
| Road | Traffic count for different road types from Vägverket. Volume Delay functions from the EVA-model. | The data is available in the VV databases (VDB) but needs some manipulation. The data is reliable. However, the estimate of the delay is very uncertain. Urban areas are not included. |
| Rail | Information on net delays (70 416 train hours) as a summary of a thousand measure points from Tågtrafikledningen by passenger (27%) and freight train (73%). Number of delayed arriving passenger trains to the final destination is also available. For 1998, 5% were delayed between 5 – 15 minutes and 2% delayed more than 15 minutes. | Monthly reported data but a very rough measure of delays. |
| PT | No delays estimated. |  |
| Aviation | Information on proportion of delayed departure (19% above 15 minutes) and average delay time (39 minutes) for Arlanda airport for the year 1999 from LFV. | The better dataset will soon be available and reported annually. We have only measured delays for Arlanda airport. The number estimated for 1999 is consistent with an estimate for 2000 on a subset of the new dataset (12% smaller). This is a very rough measure of delays but for 2001 it is possible to do estimates that are more reliable. |
| Maritime | No delays estimated. |  |

## Accident Costs

We chose an approach where we start with physical information on accidents with different degree of severity. For each type of severity, we apply unit values. The argument for this approach is that it would be too time consuming to survey all hospitals and insurance companies, and significant resources have been put into the task to update the Swedish unit values. The approach also has the advantage to be easy to update. However, where possible, we have cross-checked the result with other sources. For some modes, notably aviation and maritime transport (and to some degree rail), the number of accidents is small and very different in magnitude. Therefore, we have tried to find the actual accident cost of these accidents from insurance companies.

We have not made any ‘moving average’ over a period of years. We report the actual number of accidents for 1996 and 1998. It could be argued that for some of the modes, where usually the number of accidents is small but where sometimes huge crashes occur, a ‘moving average’ over three or five years are preferable. However, the UNITE data is easy to collect and presented for two years. In the future, it can easily be updated annually; therefore, we do not attempt any averaging.

The monitoring scheme set out in UNITE is based on a victim/injurer perspective. The table below summarises the number of fatalities in transport for 1998. The victim/injurer perspective gives us a possibility to understand how the physical burden is split between different user groups. We take as an example HGVs; from column ‘HGV’ we can see that they have been involved in accidents where 101 persons have been killed during 1998 and from the row ‘HGV’ that 6 fatalities where HGV users. In this case all of them where killed in single-accidents. We can also see from column ‘HGV’ that the majority of the victims in HGV accidents were car users followed by unprotected users.

The physical burden is not the same as the legal outcome; a person crossing the road illegally and hit by a car driven legally , will in our scheme be presented as: 1 unprotected road user as a victim with a car as injurer.

Table 2-8: Fatalities in Swedish transport (all modes) 1998 by victim and injurer

|  | INJURER |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VICTIM | Single | Unprotected | MC | Car | Bus/Coach | LGV | HGV | Other road | Tram | Train | Plane | Ship | Game | Other | Total |
| Unprotected road user | 11 | 2 | 5 | 70 | 2 | 5 | 25 | 1 | 0 | 1 | 0 | 0 | 0 | 5 | 127 |
| Motorcycle(incl moped) | 20 | 2 | 0 | 20 | 3 | 0 | 3 | 0 | 0 | 2 | 0 | 0 | 2 | 0 | 52 |
| Car | 135 | 1 | 0 | 76 | 6 | 10 | 72 | 4 | 0 | 3 | 0 | 0 | 13 | 7 | 327 |
| Bus/Coach | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 3 |
| LGV | 3 | 0 | 0 | 3 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 9 |
| HGV | 6 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 6 |
| Other road users | 5 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 7 |
| Metro passenger | 4 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 4 |
| Tram passenger | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| Railway passengers | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| Railway employees | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| Other persons | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 16 | 0 | 0 | 0 | 0 | 16 |
| (Suicide) | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 46 | 0 | 0 | 0 | 0 | 46 |
| All commercial air traffic | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Private air traffic | 4 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 4 |
| Commercial maritime transp. | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Fishing vessel | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Pleasure boats | 33 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 33 |
| (Suicide) | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Total | 225 | 5 | 5 | 169 | 12 | 16 | 101 | 5 | 1 | 69 | 0 | 0 | 16 | 13 | 637 |

Table 2-9 Input data, availability and reliability

|  | Input data: | Data availability and reliability: |
| --- | --- | --- |
| Road | The accident data is taken directly from Vägverkets accident database, VITS, which includes all police-reported road traffic accidents in Sweden. The level of disaggregating is almost unlimited; the result can be presented by axle-weight, vehicle type, engine size, or even colour of the vehicle. However, this level of disaggregation requires some manipulation of huge databases. An annual summary report, which is easily available, issued by Statistics Sweden and SIKA, can be found at: http://www.sika-institute.se/statistik_fr.html (Vägtrafiksskador 1998). The annex gives a detailed presentation of this data set for fatalities, severe injuries, and slight injuries 1996 and 1998. In addition to the information for the whole of Sweden, we have made dissagregation for urban and non-urban areas. The forecast for year 2005 is based on the preliminary road plan for the period 1998 to 2007 (Vägverket; Förslag till Nationell plan för vägtransportsystemet 1998 – 2007, mars 1998). The expected decrease in accidents is interpolated for year 2005, and only applied on the total number of road accidents. | The quality of the data is high, given that it includes all accidents reported to the police . However, underreporting of less severe accidents exists. This is corrected with factors for underreporting used officially in Sweden. However, the factor (2.3 for severe and slight injuries) is not actually reliable on a very detailed level. |

| Rail | Banverket presents information on railway accidents, which include accidents on all state railways, for all railway companies (Banverket; Statistik över olyckor på statens spåranläggningar 1998). The source does not distinguish between freight and passenger trains. Some older data sources are limited to only the state carrier SJ; this means that care has to be taken if comparison is made with some older data sources. The forecast for year 2005 is based on the policy for level crossings in the Transport Bill and a constant risk per train kilometre for other accidents. | Very reliable source but excludes accidents on private railways. |
| --- | --- | --- |
| PT | Buses are covered within the mode road but can be distinguished in the result both as urban and non-urban. Urban (commuter) trains are covered in the mode rail and cannot be distinguished. Only Tram and Metro are presented in Public Transport (Source: SIKA, Transporter och Kommunikationer, årsbok 2000/2001, tab 6.10). We have not made any forecast for these accidents. | The quality of the data is uncertain. Some of the tram accidents, where a road user is victim, are reported in the road sector. |
| Aviation | The data is taken from Statistics Sweden and can be found at: http://www.sika-institute.se/statistik_fr.html (Luftfart 1999 and 1996). In aviation, we include all powered aircraft, i.e. also small private aircraft. We do not include balloons etc (although the data exists). We have used data for aviation of all nationality in accidents on Swedish ground. The cost for material damages is based on information from insurance companies. The forecast for year 2005 is based on an extrapolation of Luftfartsverket forecast separately for flight hour and accidents per flight hour (PM Säkerhetsutveckling för civil luftfart I Sverige, 2000). | The quality of the data is high. |
| Maritime | The data is from Sjöfartsverket (Sjöfartsverkets meddelanden nr 2 1999 resp. nr 4 1997) and covers accident on Swedish water irrespective of nationality of the ship. We have included also pleasure boats. A problem is accidents on international water, which will not be covered in the accounts with the territorial principle. The cost for material damages is not included. The forecast for year 2005 is based on an extrapolation of the trend since the past 10 years. | The quality of the data is good but it is difficult to assign costs to the different accident types. |

## Environmental Costs

- The cost category environmental costs contains five different types, namely; i) Air pollution, ii) Global warming, iii) Noise, iv) Nature, landscape and further environmental effects and v) Nuclear risk. The Swedish report includes only the first three types. The discussion of the input data below is divided according to the different categories instead of the modal division used in other sections.

### Air Pollution and global warming

- For quantifying the costs due to airborne pollutants the Impact Pathway Approach, the methodology developed in the ExternE project series has been applied. It comprises the followingsteps:

- emission estimation,

- dispersion and chemical conversion modelling,

- calculation of physical impacts, and

- monetary valuation of these impacts.

- For the calculation of the costs of direct emissions from vehicle operation emission inventories in spatial disaggregation are needed, i.e. a geo-coded data set for the different air pollutants. Besides the emission data, the distribution of the population over the space is the second central input for the calculations of the most important costs of air pollution, i.e. the health costs.

Table 2-10 Input data, availability and reliability

|  | Input data: | Data availability and reliability: |
| --- | --- | --- |
|  | For the Pilot Accounts it was only possible to apply the impact pathway model on an aggregate level due to time constraints. Therefore, only aggregate data has been collected. See the Annex for more detailed information on the input data provided. The estimates on emissions from each mode originate from respectively Administration. For rail and aviation, PM was not included. No emission was estimated for the maritime sector. | The data can on an aggregate level be found in environmental reports from the administrations. The data is of the same quality as the data of the annual environmental reports. |

### Noise

Table 2-11 Input data, availability and reliability

|  | Input data: | Data availability and reliability: |
| --- | --- | --- |
|  | Input data in the form of persons exposed to transport noise at different levels are collected from each administration. | Estimate of exposed persons are very uncertain. No estimate of day and night time noise was available. |

## Taxes, Charges and Subsidies

The data on taxes charges and subsidies originates from different sources, which are being shown under each mode below. This is a very complex area and to collect and put together a complete list of this area of the accounts need efforts far behind what is practicable within the Pilot Accounts. For instance, it is difficult to distinguish transport subsidies from other subsidies like regional policy subsidies. In view of this fact, the category taxes, charges, and subsidies must be regarded as a survey on the surface for all modes.

Table 2-12 Input data, availability and reliability

|  | Input data: | Data availability and reliability: |
| --- | --- | --- |
| Road | Information from The Ministry of Finance with data on taxes and infrastructure user charges. Annual reports VV | Annual reports from VV are published each year and easy to get. Other information must be search for from different sources. The figures contain no uncertainty. |
| Rail | Annual reports BV State budget proposal, expenditure area 22 communications The official structural business statistics | All the data is published each year. The reports from BV and the state budget proposal are of high quality with no uncertainty. The official structural business statistics shows the net sales excluding excise duties. This means that there can be some sales covered in that item that does not come from charges. |
| PT | Trade association statistics from the Swedish Public Transport Union (SLTF). | The data is easy to get and it is published each year. The data is collected from about 30 ‘public transport principals’ from each county who is responsible for the figures, and there might be some improper information within the schedules. |
| Aviation | Annual reports from the National Civil Aviation Administration (LFV). A survey of eight out of twenty-eight municipal airports from LFV. A report from the Swedish Environmental Protection Agency, environmental taxes in Sweden | The data is easy to get and it is published each year. The reports from LFV are of very good quality and contain no uncertainty. The figures that describe the municipal airports must be regarded as less reliable since, in the survey, the airports themselves delivered the figures, and there might be some mistakes done. |
| Maritime | State budget proposal, expenditure area 22 communications. Annual reports from the National Maritime Administration (SFV) Reports (Harbour Traffic and Economy) from the trade association Swedish Harbour and stevedore association (SHSF). | The data is easy to get and it is published each year. The state budget proposals and annual reports contain no uncertainty and are of high quality. The figures in the reports from Swedish Harbours are aggregated figures and less reliable. |

# Methodological Issues

- The preferred methodology is described in detail in Link et al. (2000) and very briefly summarised in section 1.2 of this Appendix Report. Against this background, we refrain from repeating the extensive methodological discussion and concentrate on what has been feasible in the Swedish Pilot Accounts.

The reader of this report should be aware that between years and between different modes, the figures are not fully comparable due to different methodologies for different sources. For example, for some years it is impossible to separate deprecation from financial income/ losses because of changes in the way that available data are reported.

All the figures are adjusted to prices 1998 and converted into Euro at the exchange rate SEK 8,92. In the tables, we will use (.) for Data not applicable, (..) Data not available and (-) Magnitude nil.

## General data

The aim has been to collect available data and, as far as possible, avoid own estimations. Both basic economic data and transport volumes stem from sources described in the chapter input data. All forecasts are based on interpolation and extrapolation from available data (from SCB, KI, SIKA, LFV, and SAMPLAN) with assumptions of linearity. Where the forecasts from different sources differ, we have made a judgement on a reasonable figure somewhere in between.

## Infrastructure Costs

- The infrastructure cost calculation follows as far as possible the method proposed by Link et al. (2000) where the main working steps are:

- The valuation of the capital stock.

- The derivation of capital cost out of the asset value.

- The estimation of running cost.

- The allocation of costs to transport types user groups and vehicle types.

- However, the last step, allocation of costs, has not been done in the Swedish accounts. In Sweden, there is no acceptable method available for such calculations.

- For all modes, except for public transport, the business account was used to calculate the capital costs. It was impossible to calculate infrastructure cost for the mode public transport because double counting occur with the modes road and rail. In addition, we collect information of 40 years investment cost for the modes road, rail and aviation. These time series was the input data for the perpetual inventory model (PIM) and the result from those calculations are shown together with the business data. A comparison can be made between the two methods. All the figures are adjusted to prices 1998 with the GDP deflator.

### Road

Capital cost:

To calculate the asset value for the state administrated roads we use the business accounts from the Swedish National Road Administration (VV). To calculate an alternative asset value we use data from Statistics Sweden (SCB), with time series of 40 years for the items yearly investment in national and country roads and the length of the state administrated roads, motor roads, national roads, and country roads. This time series was input data for the PIM where the index of overall building costs was used in order to get fixed prices. In the case of municipal and private roads, no suitable data was available for such calculations. The municipal road capital cost, imputed interest cost and deprecation is approximated through the assumption that the length of the roads of state and municipal roads is a mirror of the costs in the respective area. The bridge costs are approximated through the asset value. Deprecation and imputed interest rate (3%) on the asset value add up to the capital cost.

Running cost:

For the state administrated roads, running costs stems from VV annual reports and consists of personnel expenses, premises expenses, and other operating expenses. For the municipal and private roads the figures come from the report, Municipal Road Maintenance and Park Keeping by the Swedish municipality association Kommunförbundet (KF), where the cost categories are operating and maintenance costs.

Capital cost and running cost add up to total infrastructure cost.

The Öresund bridge (Öresundsbron)

When calculating the value of the bridge, we took the Swedish part of the whole bridge cost, which is 50% of total cost. Then we added the cost of Swedish land connections to get the Swedish part of the cost. Due to recommendations from experts (while there is not a complete consensus), we allocated the cost between road and rail with 25% of the cost to road and 75% of the cost to rail.

### Rail

The Swedish National Rail Administration (BV) accounts for about 75 % of total rail infrastructure. Because of difficulties in obtaining figures for the last 25 % we omit them in the Pilot Accounts for running costs. The private railways have a minor part of the traffic. In view of this fact, the running cost contains only figures from BV. In the case of capital costs, SCB who provides figures for long time series, provides only figures for total railway, e.g both state owned and private. This makes the running costs underestimated but in the view of the fact that the private railway is a minor part, the underestimation seems to be small.

Capital cost:

To calculate the asset value for the state administrated railways we use the business accounts from BV. To calculate an alternative asset value we use data from SCB, with time series of 40 years for the items investment in state owned railway infrastructure, electrified double track normal width, electrified single track normal width, not electrified double track normal width and not electrified single track normal width. This time series was input data for the PIM where the index of overall building costs was used in order to get fixed prices. Deprecation and imputed interest rate (3%) on the asset value add up to the capital cost.

Running cost:

The running costs stem from BV annual reports and consist of operating expenses including personnel and premises expenses.

Capital cost and running cost add up to total infrastructure cost.

The Öresunds bridge (Öresundsbron)

See road.

Forecast methodology:

The figures in the report are interpolated and extrapolated due to available forecast data with an assumption of linearity.

### Aviation

Capital cost:

There are two sources of asset valuation. The first one is the business accounts from the Swedish National Civil Aviation Administration (LFV) and the second is a survey of eight out of 28 municipal, military, and private airports. These eight airports stand for the major part of the traffic within the municipal airports. The fixed asset total excluding financial assets from those sources is the input data for asset valuation. We also collected data for the PIM; monetary investment divided into the two categories airfield and other. Deprecation and imputed interest rate (3%) add up to total capital cost.

Running cost:

The same source as for capital cost was used. The running costs for LFV consist of personnel expenses and other expenses while for the municipal airports the running costs consist of running expenses, other expenses and ATS- agreements. There are different categorisations of cost items but they all belong to the category running costs.

Capital and running costs add up to total infrastructure cost.

### Maritime transport

Capital cost:

When calculating asset value for maritime transport no time series of investment and asset accounting was available. Therefore, asset valuation is built on business accounts only and no perpetual inventory concept was used. The sources for the asset valuation for the mode Maritime transport is the Swedish National Maritime Administrations (SFV) annual reports and reports of the harbours aggregated financial situation from the trade organisation Swedish Harbours. The fixed asset total excluding financial assets is the input data for asset valuation. Deprecation and imputed interest rate (3%) on the asset value add up to the capital cost.

Running cost:

The calculation of running cost stems from the same sources that was used for the capital cost calculation. The cost items are wages, social security contribution and other operating costs for the harbours while SFV annual report only gives figures for total operating cost which besides fairway maintenance also includes environmental efforts and maritime safety.

Capital and running costs add up to total infrastructure cost.

Forecast methodology:

The figures in the report are interpolated and extrapolated due to the available forecast data with an assumption of linearity. The forecast figures for volume transport stem from SAMPLAN. The categorisations and forecasts in SAMPLAN is Cargo shipping + 7.6% (tkm) and ton carried + 14.6% (t). These figures are overall figures and say nothing about domestic or international traffic, which is the categorisation in our table. However, we transfer them into our table and the reader should be aware that the figures are very unreliable.

## Supplier Operating Costs

The Official statistics in Sweden provide aggregated business accounts with income statement with specifications and balance sheet by industry, the Swedish Standard Industrial Classification 1992 (SNI 1992). This classification differs from the suggested in UNITE but it is possible to aggregate them into the cost categories in UNITE. These accounts cover the whole body of enterprises belonging to the corporation category in the transport sector.

The supplier operating cost items that were available for 1996 were: operating cost, financial profit, financial loss and deprecation. For the year 1998 there was an allocation that did not show up in the 1996 accounts. The cost items for the year 1998 were: goods for resale, raw materials and consumables, other external costs, personnel costs and other operating costs. On the other hand, the cost items financial profit and losses were compressed to profit/loss from financial investment. This made it impossible to separate financial profits (incomes) from losses (costs). In view of this fact, we show the costs as aggregated operating cost and deprecation for all modes

- The categories in supplier operating cost following SNI 1992 are rail, public transport, coach, air, taxi, other land transport of passenger, freight transport on road, ferry at sea, other sea/coast transport and inland waterway transport.

The cost only refers to the ‘transport sector’. An idea of the magnitude of transport on own account can be found from vehicle ownership by sector (SCB). The transport sector owns 7% of the LGV (0-3.5 tonne) and 46% of the HGV above 3.51 tonne. More than half of the trucking can be performed by transport on own account. For buses, most of them are registered in the transport sector (87%) and only a minor part is used on own account.

All the figures are adjusted to prices 1998 with the GDP deflator and converted into Euro at the exchange rate SEK 8,92.

## User Cost / Delay

The cost of delay is based on estimation of the ‘extra’ time a trip takes. ‘Extra’ time is here defined as travel time above scheduled travel time for scheduled traffic and travel time above travel time at free flow speed for road traffic. This extra time is valued by the value of time (VOT).

The values of time (VOT) per passenger-hour are from the UNITE valuation paper (Nellthorp et al., 2001), PPP-adjusted and converted into factor costs (commuting and leisure values only). According to the valuation paper (Nellthorp et al. (2001) it was assumed that Values of Time grow over time in line with real incomes (elasticity of 1.0 to the country's real GDP per capita).

Table 3-13:	VOT-Values for Sweden, passengers (PPP-adjusted values, Nellthorp et al. (2001), in € per hour, 1998 prices

|  | 1996 |  |  | 1998 |  |  | 2005 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Business | Commuting | Leisure | Business | Commuting | Leisure | Business | Commuting | Leisure |
| Car | 19.14 | 4.40 | 2.94 | 20.66 | 4.75 | 3.17 | 24.25 | 5.58 | 3.72 |
| Inter-urban rail | 19.14 | 4.70 | 3.45 | 20.66 | 5.07 | 3.72 | 24.25 | 5.95 | 4.37 |
| Coach | 19.14 | 4.40 | 2.94 | 20.66 | 4.75 | 3.17 | 24.25 | 5.58 | 3.72 |
| Air | 25.98 | 7.34 | 7.34 | 28.04 | 7.92 | 7.92 | 32.90 | 9.30 | 9.30 |
| Urban bus/ Tramway | 19.14 | 4.40 | 2.35 | 20.66 | 4.75 | 2.54 | 24.25 | 5.58 | 2.97 |

The values proposed for UNITE are different from the Swedish values. First, the Swedish VOT for business travel reduces the VOT when the passenger can work during the trip. This explains the comparably low Swedish values for rail, bus and air travel. For cars the Swedsih and UNITE values are equal. Secondly, the Swedish VOT for non-business travel distinguish only between regional and inter regional trips with a VOT of 3.92 €/h for regional and 7.85 €/h for interregional (>100km).

Time spent in delay is valued higher than the travel time presented above. The factor used is 1.5. (see Nellthorp et al. (2001))

Table 3-14:	VOT-Values for Sweden, freight (PPP-adjusted values, Nellthorp et al. (2001), in € per hour, 1998 prices

|  | Freight | 1996 | 1998 | 2005 |
| --- | --- | --- | --- | --- |
| Road | LGV | 36.46 | 39.36 | 46.18 |
|  | HGV | 39.20 | 42.31 | 49.64 |
| Rail | Full train load | 660.85 | 713.40 | 837.04 |
|  | wagon load | 27.35 | 29.52 | 34.64 |
|  | average per tonne | 0.69 | 0.75 | 0.88 |
| Maritime shipping | Full ship load | 182.30 | 196.80 | 230.91 |
|  | Average per tonne | 0.16 | 0.18 | 0.21 |
| Air transport | Average per tonne | 3.65 | 3.94 | 4.62 |

### Road

- We have not made any estimate of this delay cost due to lack of information from Swedish National Road Administration.

### Rail

The net delay is measured at 1 300 measurement points and summarised into an aggregate number of delayed hours for passenger and freight trains. We assume that an average train with 100 passengers passes each point. The delayed hours for passengers (H) are around 2 million hours. The delayed hours (H) is multiplied by the value of time (VOT) adjusted for the inconvenience of delays (1.5), which result in the delay cost (DC); (DC=H*VOT*1.5). For freight the observed delay (H) is multiplied by the average UNITE VOT per freight train (DC=VOT*H).

### Aviation

We have information of the proportion (p) of delayed (above 15 min) arrival flights and average delay time (ADH) for arrivals at Arlanda airport in 1999. We have applied this proportion on arriving passengers (PASS) at Arlanda airport 1998. Multiplied by the average delay time we have estimated the number of delayed hours (H) to around 1 million hours (H=p*PASS*ADH). The delayed hours (H) is multiplied by the value of time (VOT) adjusted for the inconvenience of delays (1.5), which result in the delay cost (DC); (DC=H*VOT*1.5).

## Accident Cost

Official Swedish unit values  (SIKA report 1999:6) have been used for material costs and the UNITE value for the risk value.

The Swedish official risk value for fatality is 1.45 M€, compared with the UNITE value 1.50 M€, which adjusted for purchase power is 1.53 M€ for Sweden, i.e. 5% above the Swedish official value. The UNITE relationships between the cost of fatalities and severe and slight injuries will be used, the values are close to the Swedish values. The UNITE value should be expressed as factor cost. All Swedish values are expressed as consumer values; we have adjusted them to factor cost in the table below.

The following cost categories are considered: net lost production, hospital care, administration, and property damage. In the questionnaire, where the user is asked to respond on his willingness-to-pay for increased safety, which is the background to the risk value, it is usually assumed that the user includes his lost consumption as a part of this risk value. Consequently, from his ‘Gross lost production’, i.e. the present value of his future production capacity, the present value of his future consumption needs to be subtracted; therefore the Net lost production is used in this analysis. However, we also include the ‘gross lost production’ as additional information. The split between different cost categories are taken from Vägverket (1997) and are presented in the table A below.

The assumed proportion of costs born by the user/victim, including costs covered by private insurances, is presented in Table 3.5B. The cost born by user/victim is presented in Table 3.5C and the cost born by the rest of society in Table 3.5D. The values are adjusted with GDP/capita when used for 1996 or 2005. For non-road accidents, the cost category ‘property damage’ is excluded and mode specific estimates have been used.

Table 3-15: Unite accident unit values 1998 (Euro per case)

|  | TABLE A UNITE accident unit value |  |  | TABLE B Proportion internal to user |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| € | Fatality | Severe Injured | Slight Injured | Fatality | Severe Injured | Slight Injured |
| Willingness to pay | 1 231 884 | 160 145 | 12 319 | 1.00 | 1.00 | 1.00 |
| Net lost production | 87 125 | 22 960 | 1 257 | 0.00 | 0.00 | 0.00 |
| Hospital care | 3 179 | 17 220 | 967 | 0.13 | 0.13 | 0.13 |
| Administration | 5 166 | 1 052 | 484 | 0.85 | 0.85 | 0.85 |
| Property damage | 19 174 | 5 070 | 2 611 | 0.95 | 0.95 | 0.95 |
| Total cost | 1 346 528 | 274 669 | 15 529 | . | . | . |
| Gross lost production | 416 254 | 25 352 | 1 257 | 0.40 | 0.40 | 0.40 |
|  | TABLE C Cost born by victim |  |  | TABLE D Cost born by rest of society |  |  |
| € |  |  |  |  |  |  |
| Willingness to pay | 1 231 884 | 160 145 | 12 319 | 0 | 0 | 0 |
| Net lost production | 0 | 0 | 0 | 87 125 | 22 960 | 1 257 |
| Hospital care | 401 | 2 170 | 122 | 2 778 | 15 050 | 845 |
| Administration | 4 391 | 894 | 411 | 775 | 158 | 73 |
| Property damage | 18 215 | 4 817 | 2 481 | 959 | 254 | 131 |
| Total cost | 1 254 891 | 168 026 | 15 332 | 91 637 | 38 421 | 2 306 |
| Gross lost production | 166 577 | 10 145 | 503 | 249 678 | 15 206 | 754 |

### Road

The unit values above have been used on a disaggregated dataset. It is not certain that the costs are applicable on a very detailed level. A uniform official rate of underreporting has been used. Data has been collected for both 1996 and 1998.

The definition of external and internal accident cost is discussed at length in Link et al (2000). As follows from the table above, it is assumed that each road user bears the risk value and a part of the hospital, administrative and property damage cost. For an aggregate of road users, these costs become internal cost. The external cost is only the cost born by the rest of society (Table D above). In the following, we will use the term ‘cost born by user’ and ‘cost born by the rest of society’ for this aggregate statement.

However, for a disaggregation of the data, the definition becomes more complicated. Assume that we want to examine the category ‘car’. The total accident cost in accidents involving cars consists of the cost both when the car user is a victim (the shadowed row in Table 4-53) and the cost in accidents with other users (the shadowed column, excluding double counting of the car/car category). The ‘cost born by user’ is the cost born by car users as victims (the shadowed row in Table 4-55). With this definition, we may talk about external and internal cost.

The ‘real’ cost of the insurance system, i.e. the administrative cost, is included in the unit values above. However, if we have compensation from a guilty user to an un-guilty victim the structure of the matrix will be altered, but the total cost for the whole mode will be constant.

Forecast methodology

We use the forecast made by Vägverket in the preliminary road plan 1998-2007. Until 2007 it is estimated that the number of fatalities will be reduced by 270, severe injuries 1 670 and slight injuries 5 210. From 1998 to 2005 this means a reduction of 216 fatalities, 1 336 severe injuries and 4 168 slight injuries.

### Rail

The unit values, excluding property damages, have been used on casualties in railway accidents. For level crossing accidents the property damages unit values has been used to estimate the cost on the road vehicles. For all train accidents, Banverket’s unit value for costs on train has been used (25 785 Euro/accident - source ‘Banverkets beräkningshandledning’ 2001-03-30). The cost of level-crossing accidents and suicides are reported separately. Data has been collected for both 1998 and 1996.

Forecast methodology

It is forecasted that the accidents in level crossings will decline with 50% between 1996 and 2007. An annual decrease of 5% is used. For other railway accidents a constant risk per train kilometre is employed, which means that the cost will increase with the traffic volume.

### Public transport

The unit values, excluding property damages, have been used on casualties in tram and metro accidents. Data has been collected for both 1998 and 1996.

Forecast methodology

We have not made any forecast for this limited number of accidents.

### Aviation

The unit values, excluding property damages, have been used on casualties in aviation accidents. Property damage has been estimated based on cost information from insurance companies. Data has been collected for both 1998 and 1996.

Forecast methodology

The forecast for aviation accident is based on two trends, flighthour and accident risk, made by Luftfartsverket (PM Säkerhetsutveckling för civil luftfart i Sverige, 2000). However, we use the trends beyond the period that it has been used by Luftfartsverket. Therefore, we assume a constant risk for private operations instead of the 2% increase in risk indicated by past trends.

Based on the number of flighthours 1998 (source: Luftfart 1999, tab 4.16) we have estimated flighthour 2005 (seeTable below). The number of accidents per flighthour is based on a ten year average up to 1999 and the trend assumed by Luftfartsverket. The expected number of accidents year 2005 is compared with the expected number of accidents 1998 and a change (%) in the number of accidents 1998 to 2005 is estimated. The expected number of accidents 1998 is compared with the actual number of accidents 1998 in the last rows of the table below

Table 3-16: Forecast methodology for aviation accidents

|  |  | Scheduled and non-scheduled traffic | Miscellaneous commercial operations | Instructional operations | Private operations |
| --- | --- | --- | --- | --- | --- |
| Flighthour | Year 1998 | 362 879 | 68 004 | 35 368 | 88 579 |
|  | Trend per annum (%) | 8.2 | 7.1 | 10.2 | 13.1 |
|  | Forecast 2005 | 630 018 | 109 916 | 69 804 | 209 686 |
| Accidents per 100 000 h | 10 year average | 0.49 | 6.6 | 9.8 | 27 |
|  | Trend per annum (%) | -10 | -14 | -7 | 0 |
|  | Expected 2005 | 1.48 | 2.53 | 4.12 | 56.6 |
| Changed number of accidents from 1998 (%) |  | -17% | -44% | 19% | 137% |
| Model test 1998 | Accidents | 2 | 4 | 2 | 25 |
|  | Expected number | 1.78 | 4.49 | 3.47 | 23.92 |
|  | Diff | -0.11 | 0.12 | 0.73 | -0.04 |

### Maritime

The unit values, excluding property damages, have been used on casualties in maritime accidents. Property damage has not been estimated but could possible be based on cost information from insurance companies. Data has been collected for both 1998 and 1996. The cost of work related accidents and suicide/illness of passengers are reported separately. However, accidents with pleasure boats are included.

Forecast methodology

We have not made any forecast for accident with commercial ships assuming the number of accidents to be constant. From the figure below, we see an unclear trend over the last 10 years. The peak during 1997 is caused by the number of accidents, and incidents, during the construction of the Öresund bridge.

For fatalities on pleasure boat traffic, a long term decreasing trend can be observed. We assume an annual decrease of 10%, which will reduce the number of fatalities by 47% between 1998 and year 2005.

Figure 3-1: Trends in maritime accidents and fatalities in traffic with pleasure boats

## Environmental Cost

### Air pollution and global warming

The employed impact pathway approach is described in more detail in a separate report (Bickel et al 2001 ‘Approach for estimating costs due to air pollution, global warming and noise). To compare the result from the impact pathway approach in UNITE we present below the official Swedish unit values. These values are used as a comparison between the UNITE approach and older Swedish methods. The local cost is based on a valuation per exposed person and transferred to a cost per kilogram for one city as an example (Falun). The model takes the population and ventilation into account.

Table 3-17 Swedish unit values (€/kg)

|  | PM10 | NOx | SO2 | NMVOC | CO2 |
| --- | --- | --- | --- | --- | --- |
| Regional | . | 6.73 | 2.24 | 3.36 | 0.17 |
| Local (FALUN) | 291.48 | 5.49 | 7.96 | 1.68 | . |

### Noise

Also, in the case of noise a more detailed description is available. In addition to information on exposed persons for Sweden average rent is used. The average rent used for the calculations is estimated at 2160 €/year and person. This is based on net cost per household as an average for flat and houses from SCB (46743 SEK 1997) and an average of 2.49 persons per household. The same figure is used for both 1996 and 1998. In the ExternE model, a bonus of 5 dB(A) is applied for railways.

## Taxes, Charges and Subsidies

- A complete summary of the area taxes, charges and subsidies in the transport sector is far beyond the available time in the Pilot Accounts. The results that we show in this report are not a result from a complete investigation, and must be seen as a picture of the surface. However, the sources that we used and the figures that we show in the tables are very reliable. The method used was not to do calculation by ourselves but to collect available data. Because of data availability the mode urban public transport has become public transport which includes all the public transport and not only the urban. All the figures are adjusted to prices 1998 with the GDP deflator and converted into Euro at the exchange rate SEK 8,92. The following lines show the different sources for the different incomes/costs.

Taxes:

The method used to estimate the extent of taxes was to contact and collect data from the Ministry of Finance.

Charges:

The method used to estimate the extent of charges was to collect data from the Ministry of Finance, Statistics Sweden (SCB) where the structural business statistics shows the net sales excluding excise duties. The Swedish Public Transport Association (SLTF), the Swedish National Civil Aviation Administration (LFV), the Swedish National Rail Administration (BV), and the Swedish National Maritime Administration (SFV).

When trying to separate the charges in rail between private and state owned rail we failed because the net sales which are shown in the annual reports from the Swedish State Railway Company (SJ) contains income from catering and income from road and ferry freight. It could be messy and need a lot of time to clean those figures from these incomes. Therefore, we present the total figures which stems from the structural business accounts.

Subsidies:

The term subsidies in this report cover appropriation and subsidy as well as tax relief. The method used to estimate the extent of subsidies was to collect data from the Ministry of Finance, SLTF, the Swedish National Road Administration, LFV, BV, and SFV. When the subsidy refers to tax relief the amount is expressed in net tax relief.

# The Results in Detail

- This chapter starts with a presentation of some basic economic data of interest followed by figures of transport volumes for the different modes (4.2). After that, we show monetary figures of the categories:

- Infrastructure cost (4.3) ,

- Supplier operating cost (4.4) ,

- Delay costs due to congestion cost (4.5) ,

- Accident cost (4.6),

- Environmental cost (4.7),

- Taxes, charges and subsidies (4.8).

These categories are shown for the different modes:

- Road,

- Rail,

- Aviation,

- Maritime transport,

- Public transport.

## Basic Economic Data

The table below shows the basic economic and structural data for Sweden in the years 1996 and 1998 with forecasts for the year 2005.

Table 4-18: Basic economic and structural data for Sweden

| Data | Unit | 1996 | 1998 | 2005* | Change % 1996-1998 | Change % 1998-2005 |
| --- | --- | --- | --- | --- | --- | --- |
| Land area | sqkm | 449 964 | 449 964 | 449 964 | 0 | 0 |
| Population | 1000 | 8 845 | 8 854 | 9 044 | 0.001 | 2.2 |
| GDP (in current prices) | Euro mill. | 196 901 | 213 604 | 263 000 | 8.5 | 23.4 |
| GDP (in 1998 prices) | Euro mill. | 197 670 | 213 604 | 256 000 | 8 | 20.2 |
| GDP / capita (in 1998 prices) | Euro | 22 348 | 24 125 | 28 306 | 8 | 17.3 |
| Consumer price index (1980=100) | Index | 256 | 257 | 284 | 0 | 10.5 |
| GDP deflator | Index | 100 | 102.5 | (113)** | 2.5 | na |

- * The figures for the year 2005 are calculated with interpolation and extrapolation from available forecasts.

- ** The GDP deflator for 2005 is based on the development of CPI – own estimation

- (Sources: www.konjunkrurinstitutet.se, www.scb.se)

In the case of the growth of GDP between 1996 and 1998, we can see that there is a growth rate of about eight percent, which is in line with the most of the European countries. We also see a difference in the price indexes. Consumer Price Index rose about 0.4 percent while the GDP deflator rose about 2.5 percent. When calculating fixed prices this is an important issue since the two methods will give different results. In the UNITE accounts we use the GDP deflator.

## Transport volumes

- In this chapter, we show the transport volumes for the different modes for the years 1996 and 1998. In the case where reliable data could be found, we allocate the transport volumes to different categories. Besides that, in the cases where we could find relevant forecasts we show the expected volumes for the year 2005. The first table is a summary of the following tables for the year 1998.

Table 4-19: Transport volumes in Sweden 1998

| Mode | Mill. pkm | Mill. tkm | Mill. t |
| --- | --- | --- | --- |
| Road | 80 842 | 32 674 | 336 |
| Rail | 7 144 | 19 086 | 56 |
| Public transport | 12 373 | .. | .. |
| Aviation | 14 209/4 885* | 347 | 0.23 |
| Maritime transport | 270 | 8 634 | 1.4 |

- *Pkm/domestic pkm

### Road

- The transport volume for road is presented as vehicle kilometre, passenger kilometre and finally, tonnes and tonne kilometre.

Table 4-20: Transport volumes of road transport in Sweden, in mill. Vkm.

| . Mode | Road infrastructure | 1996 | 1998 | 2005 | Change % 1996-1998 | Change % 1998-2005 |
| --- | --- | --- | --- | --- | --- | --- |
| Motorcycle, mopeds | Total Urban roads Regional roads | 700 280 420 | 750 290 460 | .. | 7.1 3.6 9.5 | . |
| Car | Total Urban roads Regional roads | 56 010 20 140 35 870 | 56 680 20 430 36 250 | 63 368 | 1.2 1.4 1.1 | +12** |
| Bus* | Total Urban roads Regional roads | 1 090 470 620 | 1 130 480 650 | 1 163 | 3.7 2.1 4.8 | +3** |
| LGV and HGV total | Total Urban roads Regional roads | 8 820 2 520 6 300 | 9 010 2 550 6460 | 11 217 | 2.1 1.2 2.5 | +25 |
| LGV | Total Urban roads Regional roads | 4 890 1 760 3 130 | 4 960 1 790 3 170 | .. | 1.4 1.7 1.3 | . |
| HGV | Total Urban roads Regional roads | 3 930 760 3 170 | 4 050 760 3 290 | .. | 3.1 0 3.8 | . |

- *	Public buses, urban and non-urban

- ** The trend is supposed to be linear.

- (Sources: VV Bilaga 5.2 Trafikarbete och bänsleförbrukning, SAMPLAN Rapport 99:2, SIKA Rapport 2000:7)

Table 4-21: Transport volumes of road transport in Sweden, in mill. pkm

| Mode | 1996 | 1998 | 2005 | Change in % 1996-1998 | Change in % 1998-2005 |
| --- | --- | --- | --- | --- | --- |
| Total road | 83 650 | 80 842 | .. | - 3.3 | . |
| Mopeds/MC | 439 | 359 | .. | - 18 | . |
| Car | 83 211 | 80 483 | 90 141 | - 3.3 | + 12 % |

- (Sources: SIKA Inrikes transportarbete efter färdsätt 1994-1998, SAMPLAN Rapport 99:2, SIKA Rapport 2000:7)

Table 4-22: Transport volumes of road transport and freight, in Sweden, in mill. tonnes and tonne- kilometres

|  | Unit | 1996 | 1998 | 2005 | Change % 1996-1998 | Change % 1998-2005 |
| --- | --- | --- | --- | --- | --- | --- |
| Tonnes | Million | 322 | 336 | .. | 4.3 | . |
| Tonne- kilometres | Million | 31 186 | 32 674 | 38 849 | 4.8 | +19 |

- (Sources: SIKA inrikes godstransportarbete efter färdsätt 1975-1999, SIKA inrikes transporterad godsmängd efter transportsätt 1975-1999, SAMPLAN Rapport 99:2)

- Lorries and cars are expected to have a substantial increase in traffic performance between 1998-2005, 25% and 12% respectively, while buses are supposed to increase with three percent. When we look at the measurement tonne-kilometres we also see a substantial increase. The figures confirm each other, and freight transport on road seems to grow by approximately three percent annually.

### Rail

- In the table below we summarises the situation and development for rail transport. The table distinguishes between:

- The National Railway Company (SJ), and,

- 13 private companies whose figures are aggregated.

Table 4-23: Transport volumes of rail transport in Sweden, in mill. passenger-km for passenger traffic and mill. tonne-kilometres for freight traffic.

| Railway companies | 1996 | 1998 | 2005 | Change % 1996-1998 | Change % 1998-2005 |
| --- | --- | --- | --- | --- | --- |
| National company (SJ) Passenger transport (pkm) Freight transport (tkm) Tonne | 6 218 16 479 42 | 6 997 14 781 29 | .. .. . | 12.5 10.3 - 31 | . . . |
| Other companies Passenger transport (pkm) Freight transport (tkm) Tonne | 147 2 361 13 | 147 4 305 27 | .. .. .. | 0 82.3 107 | . . . |
| Total rail Passenger transport (pkm) Freight transport (tkm) Tonne | 6 365 18 840 55 | 7 144 19 086 56 | 8 451 20 536 | 12.2 1.3 1.8 | Na +18.3 + 7.6 |

- (Sources: SCB statistisk årsbok, SIKA inrikes godstransportarbete efter transportsätt 1975-1999, SAMPLAN 99:2)

- Passenger transport stands for the major part of the increase in rail transport. In contrast to road, freight transport is not expected to have a substantial increase during the years 1998-2005. For rail, SIKA 2000:7 forecasts an increase for vehicle kilometres by 5% during the period 1998- 2005. Another striking result is that between the years 1996 and 1998 the private companies raise their freight transport with 1 944 tkm (82%) approximately the same amount 1 698 tkm (10%) that SJ looses.

### Public transport

- Rail transport and road transport include some public transport, which covers urban as well as inter urban traffic. In Sweden, urban public transport is not separated from the overall public transport. The interurban scheduled traffic, like urban public transport, is partly financed with taxes. Because of this, and the fact that there was no possibility to separate the two categories from each other in the available data we show the overall public transport separated into the modes bus/car, underground, tram and train. The tables below show the transport volumes in public transport. Be aware that double accounting occurs with road and rail.

Table 4-24: Transport volumes of public transport in Sweden, in mill. supply km

| Mode | 1996 | 1998 | 2005 | Change in % 1996-1998 | Change in % 1998-2005 |
| --- | --- | --- | --- | --- | --- |
| Total | 628. 5 | 643. 1 | .. | 2.3 | . |
| Bus/car | 456. 1 | 462. 5 | .. | 1.4 | . |
| Underground | 85. 4 | 85. 6 | .. | 0.3 | . |
| Tram | 16. 3 | 24.4 | .. | 49.4 | . |
| Train | 70. 8 | 70. 7 | .. | - 0.2 | . |

- (Source: SLTF branschstatistik)

Table 4-25: Transport volumes of public transport in Sweden, in mill. pkm

| Mode | 1996 | 1998 | 2005 | Change in % 1996-1998 | Change in % 1998-2005 |
| --- | --- | --- | --- | --- | --- |
| Total | 11 818 | 12 373 | .. | 4.7 | . |
| Bus | 9 032 | 9 005 | .. | 0 | . |
| Tram/U-ground/Commuter train | 2 786 | 3 368 | .. | 21 | . |

- (Source: SIKA inrikes transportarbete efter färdsätt 1994-1998)

Table 4-26: Transport volumes of public transport in Sweden, in mill. passenger

| Mode | 1996 | 1998 | 2005 | Change in % 1996-1998 | Change in % 1998-2005 |
| --- | --- | --- | --- | --- | --- |
| Total | 1 051 | 1 055 | .. | 0.4 | . |
| Bus/car | 599 | 593 | .. | -1 | . |
| Underground | 257 | 269 | .. | 4.7 | . |
| Tram | 76 | 66 | .. | -13.2 | . |
| County train* | 94 | 99 | .. | 5.3 | . |
| Train SJ* | 25 | 28 | .. | 12 | . |

*Due to definitions in SIKA communications nr 4-2000 (Source: SIKA kommunikationer nr 4-2000)

Train had the largest increase followed by county (regional) train and underground. Tram had a great increase in skm and a big decrease in number of passengers. Another striking result is that the total percentage increase in pkm is much bigger than the total percentage increase in number of passengers.

### Aviation

- In the case of aviation, we see that the forecast figures make an assumption of a great increase in the traffic volumes in the period 1998-2005. The table down below shows the number of starts and the number of passengers and passenger kilometres together with tonnes of freight and mail. In addition, there are also figures of total passenger kilometres.

Table 4-27: Transport volumes of aviation in Sweden

|  | Unit | 1996 | 1998 | 2005 | Change % 1996-1998 | Change % 1998-2005 |
| --- | --- | --- | --- | --- | --- | --- |
| Total number of starts | Number | 452 200 | 468 600 | 571 692 | 3.6 | +22% |
| Passengers | 1000 | 18486 | 21 723 | 27 718 | 17.5 | +27.6% |
| Freight and mail | Tonnes | 199 600 | 232 700 | .. | 16.6 | . |
| Freight and mail | Mill. tkm | 269 | 347 | .. | 29 | . |
| Domestic passenger kilometre | Mill. pkm | 2 706 | 4 885 | .. | 80.5 | . |
| Passenger kilometres | Mill. pkm | 13 975* | 14 209 | 15 629 | 1.7 | +10% |

- * Figures for the year 1997

### Maritime transport

- The transport volume in maritime transport, measured as tonnes and tonne-kilometres, are expected to have an increase during the years 1998-2005. This is opposite to the period 1996-1998 when the growth was almost zero.

Table 4-28: Transport volumes of maritime transport in Sweden, tonnes, tonne- kilometres and passenger kilometres.

|  | Unit | 1996 | 1998 | 2005* | Change % 1996-1998 | Change %* 1998-2005 |
| --- | --- | --- | --- | --- | --- | --- |
| Quantity goods in domestic traffic | 100 t | 13 700 | 13 800 | 15 815 | 0.7 | 14.6 |
| Domestic traffic | Milj. tkm | 8 334 | 8 334 | 9 290 | 0 | 7.6 |
| International traffic along the Swedish coast | Milj. tkm | 22 097 | 22 097 | 24 683 | 0 | 7.6 |
| Maritime transport | Mill. pkm | 457 | 270 | .. | - 41 | . |
| Number of calls** | Number | .. | 120 658 | .. | . | . |

- *The forecast figures stems from SAMPLAN. The categorisation and forecasts in SAMPLAN is Cargo shipping + 7.6% (tkm) and ton carried + 14.6% (t). These figures are overall figures and say nothing about domestic or international traffic. However we transfer them into our table and the reader should be aware that this makes the figures are very unreliable. (Sources: SIKA inrikes transporterad godsmängd efter transportsätt 1975-1999, SIKA inrikes transportarbete efter färdsätt 1994-1998)

- **Cargo 38 086 and passenger 82 572

## Infrastructure Costs

The aim is to present capital cost of infrastructure and the operating cost for infrastructure for all modes of transport. This is possible for road, rail, air and maritime transport, where figures can be collected from annual reports. In addition, it was also possible to get information about investment for 40 years for the modes road, rail and aviation. These figures were the input data for the Perpetual Inventory Model (PIM). In the mode public transport it was not possible to calculate infrastructure costs because of double accounting occur with the modes road and rail. The infrastructure asset value for the different modes for the year 1998 is presented in the table below where we compare three different methods for asset calculation.

Table 4-29: Infrastructure asset value 1998, in thousands of Euro.

|  | Value with |  |  |
| --- | --- | --- | --- |
| Mode | Business accounts | PIM* | Replacement cost** |
| Road | 8 127 840*** | 6 745 000 | 20 724 215 |
| Rail | 6 954 882*** | 6 752 000 | 7 761 323 |
| Public transport | .. | .. | 1 414 574 |
| Aviation | 845 853 | 795 000**** | 1 227 242 |
| Maritime transport | 695 273 | .. | 1 959 978 |

*Net capital value

**The figures stems from SIKA 1998 and refer to the year 1995 with 1995 prices.

** *Without the bridge between Denmark and Sweden. The Swedish part of the bridge with land connections add 286 935 Euro to road and 860 804 Euro to rail.

****Only state owned airports.

### 4.3.1 Road

The Swedish road infrastructure is described in the scheme below where we show the length in kilometre and the transport volume (vkm) in percent. (Source: SIKA 1998).

Figure 4-1 Roads in Sweden 1998.

State roads

98 000 km

70 %

Municipal roads

38 500 km

26 %

Private roads

284 000 km

4 %

National

Trunk

Roads

8000 km

32 %

Other

National

Roads

6 600 km

12 %

County

Roads

83 400 km

26 %

With

Subsidy

74 000 km

3 %

Without

Subsidy

210 000 km

1 %

We see that the private roads, mostly forest roads in bad condition, have a major part of the length but only a minor part of the transport volume. The infrastructure costs for road is calculated with two different methods in the table below. The two methods used are calculation with business accounts and calculation with perpetual inventory model.

Table 4-30: Infrastructure costs road, in thousand € 1998 prices

|  | State (VV) | Municipal | Bridge* | Total |
| --- | --- | --- | --- | --- |
| Total cost business accounts |  |  |  |  |
| 1996 | 1 327 391 | 585 829 | 5 010 | 1 918 230 |
| 1998 | 1 539 098 | 619 906 | 13 486 | 2 172 490 |
| Total cost with perpetual inventory model |  |  |  |  |
| 1996 | 1 504 799 |  |  |  |
| 1998 | 1 704 826 |  |  |  |
| Running costs |  |  |  |  |
| 1996 | 851 799 | 400 348 | 0 | 1 252 147 |
| 1998 | 1 025 826 | 419 731 | 0 | 1 445 557 |
| Deprecation |  |  |  |  |
| 1996 | 256 436 | 100 010 | 1 812 | 358 258 |
| 1998 | 269 437 | 105 080 | 4 878 | 379 395 |
| Imputed interest cost, business accounts, exclude fin. assets |  |  |  |  |
| 1996 | 219 156 | 85 471 | 3 198 | 307 825 |
| 1998 | 243 835 | 95 095 | 8 608 | 347 538 |
| Imputed interest cost and deprecation perpetual inventory model |  |  |  |  |
| 1996 | 653 000 |  |  |  |
| 1998 | 679 000 |  |  |  |
| Asset value, business accounts, exclude fin. assets |  |  |  |  |
| 1996 | 7 305 212 | 2 849 032 | 106 595 | 10 260 839 |
| 1998 | 8 127 840 | 3 169 858 | 286 935 | 11 584 633 |
| Asset value perpetual inventory model** |  |  |  |  |
| 1996 | 6 467 000 |  |  |  |
| 1998 | 6 745 000 |  |  |  |

*The figures represent the Swedish part of the bridge between Denmark and Sweden with Swedish land connections.

**Net capital value.

(Sources: DIW,  VV årsrapport, KF kommunernas väghållning och parkskötsel, årsrapporter från ÖSK och SVEDAB  and own calculations)

When we calculated the imputed interest cost for the state own roads we used the asset value in the business accounts excluding financial assets. If we had used the asset value from the business accounts, only road capital, the picture had been only slightly different. For the year 1998 the imputed interest rate had been 239 005 instead of 243 835 and the total cost would have been 1 534 268 instead of 1 539 098, a difference of about 0,3 percent. The reason that we use the business accounts excluding financial assets is that those figures are available for all modes. In the case of municipal roads we failed to collect data about asset value and therefore made our own calculations of asset value, imputed interest rate and depreciation. In the case of the bridge we only had data about asset value and did our own calculations of imputed interest rate and deprecation. The bridge had no running costs for the years 1996 and 1998. Imputed interest cost, deprecation and running cost sum up to total cost.

The next table shows forecast for road costs where the asset value are supposed to increase while the running costs decrease and total costs are supposed to be lower in 2005 than it was in 1998.

Table 4-31: Forecast road for the year 2005, only state roads*

| Asset value | Imputed interest cost | Deprecation | Running cost | Total cost |
| --- | --- | --- | --- | --- |
| 12 433 196 | 372 996 | 310 830 | 627 803 | 1 311 629 |

*Due to national plan for road transport system 1998-2007 and own calculations.

*In chapter 5 we have assumed the cost for municipal roads to be constant between 1998 and 2005.

### Rail

The Swedish Railway infrastructure system consists of BV (state) and others (private). The track length for BV was 11 600 km (75%) while for the others the track length was 3 852 km (25%). The figures of infrastructure length may differ between different sources due to different methods used when calculating the length. In the scheme below, we show the structure of the Swedish railway system 1998 with track length in km and the share in percent (Source: BV annual report 1998).

Figure 4-2 Rail tracks in Sweden 1998.

Track total 15 452 km (100%)

State (BV) 11 600 km (75%)

Others 3 852 km (25%)

Frame track

8 422 km

73 %

County

track

3 073 km

26 %

Not in use

105 km

1 %

IBAB, SL m.fl.

1 255 km

33 %

Museal

track

297 km

8 %

Capillary track

2 300 km

59 %

Infrastructure costs for rail could not be divided into passenger transport related and freight transport related costs. The privately owned railways, which account for 25 % of the track length 1998 are excluded because of the difficulties in obtaining relevant data. Be aware that that most of the private owned track is capillary track. In the next table, we show the composition of infrastructure cost for rail.

Table 4-32: Infrastructure costs rail, thousand € 1998 prices

|  | State (BV) | Bridge* | Total |
| --- | --- | --- | --- |
| Total cost business accounts |  |  |  |
| 1996 | 745 814 | 18 740 | 764 554 |
| 1998 | 825 382 | 30 907 | 856 289 |
| Total cost perpetual inventory model |  |  |  |
| 1996 | 853 852 |  |  |
| 1998 | 949 853 |  |  |
| Running cost |  |  |  |
| 1996 | 406 852 | 0 | 406 852 |
| 1998 | 397 853 | 0 | 397 853 |
| Deprecation |  |  |  |
| 1996 | 161 274 | 5 330 | 166 604 |
| 1998 | 218 883 | 14 347 | 233 230 |
| Imputed interest cost, business accounts, exclude fin. assets |  |  |  |
| 1996 | 177 688 | 13 410 | 191 098 |
| 1998 | 208 646 | 16 560 | 225 206 |
| Imputed interest cost and deprecation perpetual inventory model |  |  |  |
| 1996 | 447 000 |  |  |
| 1998 | 552 000 |  |  |
| Asset value perpetual inventory model (net capital value) |  |  |  |
| 1996 | 5 989 000 |  |  |
| 1998 | 6 752 000 |  |  |
| Asset value business account, exclude financial assets |  |  |  |
| 1996 | 5 922 946 | 319 785 | 6 242 731 |
| 1998 | 6 954 882 | 860 804 | 7 815 686 |

* The figures refer to the Swedish part of the bridge between Denmark and Sweden and the land connections. The bridge was opened in the year 2000 and no running cost exists the years 1996 and 1998.  Imputed interest cost and deprecation (60year) add up to total cost.

(Sources: DIW, annual reports from BV, ÖSK, SVEDAB and own calculations)

The Swedish National Road Administration reports two different kinds of asset value. If we had based the imputed interest cost on the "business accounts only rail capital" instead of "business accounts exclude financial assets" we would get a difference of 4 986 which is about 0,6 percent of total cost. Imputed interest cost, deprecation and running cost sum up to total cost.

The asset value is supposed to increase by about 43 percent and total costs are supposed to increase by about 14 percent. The forecast is shown in the table below.

Table 4-33: Forecast rail for the year 2005, only state railways (BV)

| Asset value | Imputed interest cost | Deprecation | Running cost* | Total cost |
| --- | --- | --- | --- | --- |
| 9 928 664 | 297 860 | 248 217 | 397 853 | 943 930 |

*With assumption of zero change between the years 1998-2005

(Sources: BV, stomnätsplan 1998-2007 and länstransportplan 1998-2007 and own calculations).

### Aviation

The Swedish airports can be divided into five different categories are shown in the table below.

Table 4-34: Number of airports in Sweden 1998

|  | State owned (LFV) | Military, run by LFV | Military with municipal principal | Municipal | Private | Total |
| --- | --- | --- | --- | --- | --- | --- |
| Number | 14 | 5 | 8 | 18 | 2 | 47 |

(Source: SIKA luftfart 99)

LFV was running 19 airports, municipal authorities 26 and private principals ran two airports. The table below shows the number of landings and passengers for the different categories.

Table 4-35: Number of landings and passengers and the percent share of the total figures for the year1998.

|  | LFV | The other 28 | Total | 20 out of 28 the excluded | Eight out of 28 |
| --- | --- | --- | --- | --- | --- |
| Landings | 349 853 | 118 786 | 468 639 | 66 928 | 51 858 |
|  |  |  |  | 14.3 % | 11 % |
| Passenger | 27 508 143 | 1 453 045 | 28 961 188 | 453 416 | 999 629 |
|  |  |  |  | 1.6 % | 3.5 % |

(Source: SIKA luftfart 99)

When we exclude 20 minor airports we loose data on 1.6 % of the passengers and 14 % of the landings. Therefore we can make a good approximation by doing this. The airports under state administration  count for about 95% of the total number of passenger and about 73% of the total landings (year 2000,www.lfv.se). Infrastructure costs for aviation (all of the state administrated and eight municipal airports and national flight control) in Sweden are shown in the table below.

Table 4-36: Infrastructure costs aviation, state-administrated airports and eight municipal airports, in thousand € 1998 prices

|  | State (LFV) | Municipal* | Total |
| --- | --- | --- | --- |
| Total cost |  |  |  |
| 1996 | 417 265 | 20 711 | 437 976 |
| 1998 | 421 906 | 25 482 | 447 388 |
| Running costs |  |  |  |
| 1996 | 330 942 | 19 521 | 350 463 |
| 1998 | 332 399 | 20 280 | 352 679 |
| Deprecation |  |  |  |
| 1996 | 61 247 | 3 335** | 64 582 |
| 1998 | 65 022 | 4 311 | 69 333 |
| Imputed interest cost, business accounts |  |  |  |
| 1996 | 25 076 | 855 | 25 931 |
| 1998 | 24 484 | 891 | 25 375 |
| Asset value excluded financial assets, business accounts |  |  |  |
| 1996 | 835 858 | 28 498 | 864 356 |
| 1998 | 816 144 | 29 709 | 845 853 |

* The figures represent the eight municipal airports with the largest traffic volume.

**The figures show financial result

(Sources: LFV årsrapport, LFV flygplatsjämförelse 1996 och 1998 statliga och kommunala flygplatser and own calculations)

We could not get data for the perpetual inventory model (PIM) for municipal airports. But for the state owned airports we could. Therefore, we show the PIM results in a separate table. Be aware that the figures refer to the state owned airports and not to the total.

Table 4-37: Asset value and capital cost for state owned airports with perpetual inventory model

| Year | Net capital value | Imputed interest rate and deprecation |
| --- | --- | --- |
| 1996 | 771 000 | 103 000 |
| 1998 | 795 000 | 106 000 |

(Source: DIW)

### Maritime transport

The Swedish maritime infrastructure consists of 50 harbours, all members of the trade organisation Swedish Harbour, and fairways for which SFV is responsible.

Table 4-38: Infrastructure costs maritime transport, municipal harbours and fairways in thousand € 1998 prices

|  |  | State (SFV) fairways etc. |  | Municipal harbours* |  | Total |
| --- | --- | --- | --- | --- | --- | --- |
| Total cost |  |  |  |  |  |  |
| 1996 |  | 162 032 |  | 423 578 |  | 585 610 |
| 1998 |  | 125 726 |  | 453 864 |  | 579590 |
| Running costs |  |  |  |  |  |  |
| 1996 |  | 149 371 |  | 376 476 |  | 525 847 |
| 1998 |  | 113 236 |  | 392 436 |  | 505 672 |
| Deprecation |  |  |  |  |  |  |
| 1996 |  | 8 967 |  | 30 104 |  | 39 071 |
| 1998 |  | 8 988 |  | 44 071 |  | 53 059 |
| Imputed interest cost |  |  |  |  |  |  |
| 1996 |  | 3 694 |  | 16 997 |  | 20 691 |
| 1998 |  | 3 501 |  | 17 357 |  | 20 858 |
| Asset value exclude financial assets, business accounts |  |  |  |  |  |  |
| 1996 | 123 136 |  | 566 584 |  | 689 720 |  |
| 1998 | 116 711 |  | 578 562 |  | 695273 |  |

* For the municipal harbours the figures include harbour companies as well as harbour-and-stevedore companies and stevedore companies. Fixed assets total do not exclude financial assets. For the year 1996 there was 49 harbours while for 1998 three harbours are added. These are Delta Terminal, Landskrona and Umeå.

** (Sources: SFV årsrapport, Sveriges Hamnar hamntrafik och ekonomi)

The major part of the infrastructure cost stems from the harbours and the harbours asset value are expected to rise. The investment plan for all harbours is 1 000 Million € during the years 1999-2005, and this investment includes all infrastructure investments. That means that double accounting may occur with the modes road and rail. Investment in cargo handling and storing only during the same period is 300 Million €. With extrapolation for the years 2004 and 2005, the asset value end up to be 881 252 thousands € at the end of 2005. (SFV Strategisk Analys Underlagsrapport till Samplan om Hamnstruktur och Sjöfart).

## Supplier Operating Costs

All the information in this chapter is not necessary for the pilot accounts, but they might be interesting as a part of a "stand alone report" for Sweden. For the supplier operating cost, the official structural business statistics was used. The Statistics Sweden (SCB) makes the structural business statistics and it shows the overall income statement and the balance sheet of the Swedish companies belonging to the corporation sector. The structural business statistics have a categorisation that differs from the categorisation suggested in UNITE. No distinction is made between urban public transport and public transport because no such data was available without double counting. Therefore, we use the term public transport only. The Swedish Industry Classification (SNI) code 62 (AIR) is not divided into minor parts because of secrecy safeguards. From the overall income statement in the structural business accounts, the item costs in the tables below are for the year 1998 subdivided into goods for resale, other external cost, personnel cost, other operating costs.

For the year 1996 no such allocation was reported. For 1996 the cost category in the overall income statement was operating costs. Therefore, in the tables below, we show just the item costs. However, disaggregation is feasible for the year 1998. The first table in this section shows the financial items plus the number of firms and employees in each mode for the year 1998.

Table 4-39: Financial items in thousand € and number of firms and employees for supplier operating cost

| Item | Road | Rail | Public Transport | Aviation | Maritime transport | Total |
| --- | --- | --- | --- | --- | --- | --- |
| Number of firms | 11 823 | 22 | 49 | 194 | 606 | 12 694 |
| Number of employed | 72 345 | 11 482 | 9 708 | 12 785 | 14 437 | 120 757 |
| Net sales | 7 199 103 | 1 324 888 | 1 065 471 | 2 496 413 | 3 393 946 | 15 479 821 |
| Other operating income | 173 655 | 12 332 | 665 807 | 211 099 | 80 269 | 1 143 162 |
| Costs | 6 564 910 | 1 269 619 | 1 632 511 | 2 481 390 | 3 280 045 | 15 228 475 |
| Deprication and WD/RS | 477 802 | 88 789 | 88 453 | 104 709 | 129 260 | 889 013 |
| Operating profit/loss | 304 955 | -21 076 | 10 314 | 121 188 | 72 759 | 488 140 |
| Net profit/loss | 145 627 | 15 022 | 3 700 | 70 291 | 43 386 | 278 026 |
| Tangible assets | 2 399 103 | 1 426 457 | 1 228 139 | 1 174 776 | 1 537 108 | 7 765 583 |

- The tables below follow the categorisation in SNI 92 and show financial data for the different categories under each mode. SNI codes are in brackets. (Source: SCB ekonomisk redogörelse för företagen). Underreporting occur due to transport on own account (see section 3.3.).

Table 4-40: Income statement for coach and freight transport on road in thousands € (1998 prices)

|  | Coach (60212) |  |  | Freight transport on road (60240) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1996 | 1998 | 2005 | 1996 | 1998 | 2005 |
| Net sales | 888 164 | 839 798 |  | 5 319 031 | 5 352 803 |  |
| Other operating income | .. | 27 915 |  | .. | 121 973 |  |
| Costs | 853 440 | 758 408 |  | 4 755 560 | 4 850 336 |  |
| Deprication and WD/RS | 57 584 | 66 480 |  | 322 354 | 366 031 |  |
| Operating profit/loss | -22 859 | 4 283 |  | 241 142 | 258 744 |  |
| Net profit/loss | 6 752 | 3 363 |  | 113 124 | 116 704 |  |
| Tangible assets | 405 193 | 485 314 |  | 1 461 894 | 1 696 525 |  |

Table 4-41: Income statement for taxi and other land transport of passenger, in thousands € (1998 prices)

|  | Taxi (60220) |  |  | Other land transport of passengers (60230) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1996 | 1998 | 2005 | 1996 | 1998 | 2005 |
| Net sales | 797 189 | 847 309 |  | 132 168 | 159 193 |  |
| Other operating income | .. | 18 498 |  | .. | 5 269 |  |
| Costs | 750 788 | 802 242 |  | 115 194 | 153 924 |  |
| Deprication and WD/RS | 27 738 | 31 502 |  | 10 581 | 13 789 |  |
| Operating profit/loss | 18 672 | 31 502 |  | 6 393 | 10 426 |  |
| Net profit/loss | 16 832 | 23 094 |  | 2 275 | 2 466 |  |
| Tangible assets | 113 711 | 131 502 |  | 61 751 | 85 762 |  |

Table 4-42: Income statement for rail, in thousands € (1998 prices)

|  | Rail (60100) |  |  |
| --- | --- | --- | --- |
|  | 1996* | 1998 | 2005 |
| Net sales | 51 098 | 1 324 888 |  |
| Other operating income | .. | 12 332 |  |
| Costs | 45 193 | 1 269 619 |  |
| Deprication and WD/RS | 2 923 | 88 789 |  |
| Operating profit/loss | 2 981 | -21 076 |  |
| Net profit/loss | -771 | 15 022 |  |
| Tangible assets | 49 853 | 1 426 457 |  |

- *The state owned company SJ was not included in the statistics 1996

Table 4-43: Income statement for public transport, in thousands € (1998 prices)

|  | Public transport (60211) |  |  |
| --- | --- | --- | --- |
|  | 1996 | 1998 | 2005 |
| Net sales | 992 949 | 1 065 471 |  |
| Other operating income | .. | 665 807 |  |
| Costs | 1 488 076 | 1 632 511 |  |
| Deprication and WD/RS | 71 139 | 88 453 |  |
| Operating profit/loss | -566 266 | 10 314 |  |
| Net profit/loss | 3 585 | 3 700 |  |
| Tangible assets | 801 700 | 1 228 139 |  |

Table 4-44: Income statement for aviation, in thousands € (1998 prices)

|  | Aviation (62100) |  |  |
| --- | --- | --- | --- |
|  | 1996 | 1998 | 2005 |
| Net sales | 2 214 015 | 2 496 413 |  |
| Other operating income | .. | 211 099 |  |
| Costs | .. | 2 481 390 |  |
| Deprication and WD/RS | 86 417 | 104 709 |  |
| Operating profit/loss | 87 884 | 121 188 |  |
| Net profit/loss | 71 098 | 70 291 |  |
| Tangible assets | 938 905 | 1 174 776 |  |

Table 4-45: Income statement for ferry at sea and other sea/coast transport, in thousands € (1998 prices)

|  | Ferry at sea (61101) |  |  | Other sea/coast transport (61102) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1996 | 1998 | 2005 | 1996 | 1998 | 2005 |
| Net sales | 942 965 | 891 928 |  | 2 167 647 | 2 407 960 |  |
| Other operating income | .. | 18722 |  | .. | 56 054 |  |
| Costs | 845 070 | 857 511 |  | 2 020 487 | 2 322 870 |  |
| Deprecation and WD/RS | 50 102 | 43 834 |  | 93 963 | 79 148 |  |
| Operating profit/loss | 47 792 | 17 265 |  | 53 197 | 61 996 |  |
| Net profit/loss | -1 758 | 13 229 |  | 108 833 | 29 372 |  |
| Tangible assets | 774 055 | 361 211 |  | 1 198 471 | 1 115 247 |  |

Table 4-46: Income statement for inland waterway transport, in thousands € (1998 prices)

|  | Inland waterway transport (61200) |  |  |
| --- | --- | --- | --- |
|  | 1996 | 1998 | 2005 |
| Net sales | 66 847 | 94 058 |  |
| Other operating income | .. | 5 493 |  |
| Costs | 68 680 | 99 664 |  |
| Deprecation and WD/RS | 5 007 | 6 278 |  |
| Operating profit/loss | -6 839 | -6 502 |  |
| Net profit/loss | 738 | 785 |  |
| Tangible assets | 54 628 | 60 650 |  |

## Delay costs due to congestion

### Road

Not data available.

### Rail

With an assumed passenger mix (25% business, 37.5% commuting and 37.5% leisure) the total delay cost for passengers is 26 900 thousand€. For freight trains the cost is 36 700 thousand€.

### Aviation

With an assumed passenger mix (25% business, 75% leisure) the total delay cost is 20 Million € in 1998 at Arlanda airport. If all airports in Sweden had the same proportion delays, the total cost would be 35 Million €. This is an upper limit and not used in the rest of this report.

## Accident Cost

The total cost of traffic accidents was 3 407 million € in Sweden 1998. It is 3% higher than the cost in 1996 but it is forecasted that the cost will decline until year 2005. This positive trend is an effect of the decreasing number of road accidents due to a strong road safety policy. It is also forecasted that fatalities in pleasure boat accidents will decline, following a long-term trend.

The majority of total accident cost are in the road sector (96%). The other transport modes have relatively low accident costs, the maritime sector 2.4%, rail 1.1%, public transport (tram and metro) 0.4% and aviation 0.2%. In maritime, 85% of the cost is the result of fatalities while using pleasure boats. If this cost is ignored the share of the maritime sector falls to 0.3%. However, leisure travel is included in all modes so it seems reasonable to include it also for the maritime sector. Private aircraft causes the majority of the cost in the aviation sector.

Table 4-47: Total accident cost by mode (M€)

|  |  | Road | Rail | PT | Aviation | Maritime | Total |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Total Cost | 1996 | 3195.62 | 26.10 | 9.77 | 33.05 | 52.11 | 3 317 | 97% |
|  | 1998 | 3268.90 | 37.93 | 12.36 | 7.06 | 81.06 | 3 407 | 100% |
|  | 2005 | 2174.60 | 40.90 | 12.36 | 17.99 | 39.14 | 2 285 | 67% |
| Percentage | 1998 | 96% | 1.1% | 0.4% | 0.2% | 2.4% | 100% | - |

Rail – excluding suicide (61 M€)

PT – only metro and tram.

Aviation – includes private powered aircraft

Maritime – includes pleasure boats, excludes work related accidents and suicide

Approximately ¾ of the total accident cost consists of the so called ‘risk value’, i.e. the users willingness to pay for a risk reduction ex ante. Ex post, this value can be interpreted as the willingness-to-pay of all users to avoid all accidents during one year. Hospital care is estimated to cost 196 M€ in 1998, property damage 164 M€ and administrative cost 32 M€. The gross lost production is 2 772 M€. However, a part of this is assumed to be included in the risk value and, when the risk value is included separately, only the net lost production is appropriate to add. The net lost production is 311 M€.

Table 4-48: Total accident cost by cost category and year (M€)

| MEuro |  | All Modes | Percentage |
| --- | --- | --- | --- |
| Risk value | 1996 | 2 610 | 77% |
|  | 1998 | 2 705 | 79% |
| Net lost production | 1996 | 300 | 9% |
|  | 1998 | 311 | 9% |
| Hospital care | 1996 | 189 | 6% |
|  | 1998 | 196 | 6% |
| Administration | 1996 | 31 | 1% |
|  | 1998 | 32 | 1% |
| Property damage | 1996 | 186 | 5% |
|  | 1998 | 164 | 5% |
| Total Cost | 1996 | 3 317 | 98% |
|  | 1998 | 3 407 | 100% |
| Gross lost production | 1996 | 2 669 | 78% |
|  | 1998 | 2 772 | 81% |

The cost per mode in Table 4-47 is based on an allocation by victim. This is a straightforward allocation of the cost, and applicable to most countries as accident statistics, at a minimum, reports if the injured person is using road, public transport, rail, or other modes. However, this allocation will underestimate the total cost of accidents that a ‘mode’ is involved in. In Table 4-49 below, the total accident cost is allocated by injurer and victim for all modes.

Table 4-49: Total Accident Cost by victim and injurer per mode, Sweden 1998 (M€)

| VICTIM / INJURER | Non-Collision | Unprotected | Road | PT | Rail | Aviation | Maritime | Other | TOTAL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unprotected | 92 | 42 | 573 | 2 | 24 | 0 | 0 | 28 | 762 |
| Road vehicles | 1 038 | 15 | 1 279 | 5 | 9 | 0 | 0 | 156 | 2 502 |
| PT | 5 | 0 | 7 | 0 | 0 | 0 | 0 | 0 | 12 |
| Rail | 6 | 22 | 0 | 0 | 0 | 0 | 0 | 0 | 28 |
| Aviation | 7 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 7 |
| Maritime | 81 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 81 |
| TOTAL | 1 230 | 56 | 1 860 | 7 | 33 | 0 | 0 | 185 |  |

Rail – excluding suicide (61 M€)

PT – only metro and tram.

Aviation – includes private powered aircraft

Maritime – includes pleasure boats, excludes work related accidents and suicide

Looking at the mode ‘road’ we see that users of motorized vehicles (road vehicles) as victim generates a cost of 2 502 M€. The majority of the cost is created in collisions between two, or more, vehicle users (1 279 M€) with non-collisions as the single largest part (1 038 M€). But they are also involved in accidents, which generate costs for other modes/users. The total cost of accidents where road users are involved is 3 083 M€, which includes the cost for unprotected users (2 502+1 860-1 279). The total cost by mode with this method is summarised in Table 4-50. A part of this total cost is born by users, notable the risk value and a minor part of the hospital care and administrative costs.

Of the total accident cost for road vehicle users (3 083 M€) they bear 2 132 M€ themselves. The ‘external’ part is 31%. For the mode rail the ‘external’ part is high (85%), as a large part of these accidents fall outside the ‘own’ mode. The observation on public transport is too uncertain for this type of estimation. For aviation and maritime transport, the ‘external’ cost is around 10%, which is the hospital cost and net lost production costs imposed on the rest of society.

Table 4-50: External cost in transport by mode, Sweden 1998 (M€)

|  | Internal | Total Cost | Percentage external |
| --- | --- | --- | --- |
| Unprotected | 629 | 776 | 19% |
| Road vehicles | 2132 | 3 082 | 31% |
| PT | 11 | 19 | .. |
| Rail | 6 | 39 | 85% |
| Aviation | 7 | 7 | 8% |
| Maritime | 76 | 81 | 7% |
| All mode | 2861 | 3 371 | 15% |

Rail – excluding suicide

PT – only metro and tram.

Aviation – includes private powered aircraft

Maritime – includes pleasure boats, excludes work related accidents and suicide

1) The total cost by one mode, includes cost of accident with other modes. When the total cost is summarised (last row) for all modes, these inter-modal accident costs could be counted twice, if this is not noticed. As a result, the total cost by each modeb can not simple be summarised.

### Road

In the remaining parts of section 4.6 we return to the unit ‘1000 Euro’ (k€). The higher level of detail in the figures does not suggest that the precision in the result is higher.

Of the total road accident cost in 1998, almost 2/3 was cost for car user victims (63%), and as much as (23%) was cost for unprotected road user victims. Victims using motorcycles, including mopeds, generated 9% of the cost, Light Goods vehicles 2%, and bus/coach, HGV generated 1% each.

Table 4-51: Road accident cost by user as victim and cost category, all Sweden (kEuro)

| kEuro |  | Unprotected | Motorcycle (incl moped) | Car | Bus/Coach | LGV | HGV | Other | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Risk value | 1996 | 624 631 | 242 897 | 1 525 461 | 23 796 | 63 359 | 24 336 | 21 474 | 2 525 954 |
|  | 1998 | 588 363 | 227 598 | 1 618 383 | 31 434 | 62 285 | 30 455 | 21 940 | 2 580 457 |
| Net lost production | 1996 | 74 414 | 28 685 | 175 327 | 2 669 | 7 425 | 2 847 | 2 570 | 293 937 |
|  | 1998 | 69 479 | 26 821 | 188 382 | 3 884 | 7 501 | 3 485 | 2 441 | 301 992 |
| Hospital care | 1996 | 48 228 | 18 219 | 111 506 | 1 638 | 4 784 | 1 900 | 1 686 | 187 961 |
|  | 1998 | 44 381 | 16 938 | 121 642 | 2 743 | 5 096 | 2 257 | 1 399 | 194 456 |
| Administration | 1996 | 6 865 | 2 471 | 19 863 | 302 | 770 | 373 | 249 | 30 894 |
|  | 1998 | 6 270 | 2 258 | 21 055 | 478 | 876 | 455 | 186 | 31 578 |
| Property damage | 1996 | 34 550 | 12 321 | 101 363 | 1 534 | 3 909 | 1 933 | 1 260 | 156 869 |
|  | 1998 | 31 426 | 11 222 | 107 526 | 2 484 | 4 516 | 2 347 | 898 | 160 419 |
| Total Cost | 1996 | 788 688 | 304 593 | 1 933 521 | 29 939 | 80 248 | 31 388 | 27 238 | 3 195 615 |
|  | 1998 | 739 919 | 284 837 | 2 056 987 | 41 023 | 80 274 | 38 999 | 26 863 | 3 268 902 |
| Percentage | 1996 | 25% | 10% | 61% | 1% | 3% | 1% | 1% | 100% |
| Percentage | 1998 | 23% | 9% | 63% | 1% | 2% | 1% | 1% | 100% |

The forecast suggests that the total accident cost will decline by 33% until year 2005 due to an assumption of a rather successful implementation of the current Zero-accident policy. We have assumed that the cost per casualty is constant and have not made any forecast by vehicle type.

Table 4-52: Forecasted road accident cost 2005 (kEuro)

| kEuro | Total |
| --- | --- |
| Risk value | 1 728 906 |
| Net lost production | 202 334 |
| Hospital care | 130 285 |
| Administration | 21 158 |
| Property damage | 107 481 |
| Total Cost | 2 190 164 |

The table below presents the total accident cost by victim and injurer for the year 1998. This monitoring scheme gives an almost unlimited number of observations. Total accidents cost where car users are victim was 2 056 M€. This cost is generated by car-car collisions (769 M€) and single accidents (773 M€) as the dominant accident type. In addition, cars are involved in accidents with other modes, with a total cost of 627 M€ (1 396-769). The majority of this cost falls on unprotected road user (457 M€) and motorcycles including mopeds (130 M€). The total cost for accidents where cars are involved is 2 677 M€ (2050+627).

Table 4-53: Total accident cost by victim and injurer, Sweden 1998 (kEuro)

| INJURER/ VICTIM | Single | Unprotected | MC | Car | Bus/ Coach | LGV | HGV | Other transport | Train | Tram | Game | Other | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unprotected | 91 545 | 41 616 | 24 125 | 456 728 | 19 708 | 29 904 | 42 773 | 8 071 | 2 296 | 2 143 | 597 | 20 412 | 739 919 |
| Motorcycle (incl moped) | 105 336 | 8 589 | 4 581 | 130 452 | 4 839 | 7 240 | 9 993 | 1 627 | 2 693 | 515 | 7 384 | 1 587 | 284 837 |
| Car | 772 664 | 5 720 | 5 121 | 768 791 | 34 033 | 59 034 | 195 609 | 25 413 | 5 273 | 4 228 | 70 082 | 111 019 | 2 056 987 |
| Bus/Coach | 25 471 | 81 | 81 | 3 181 | 487 | 203 | 2 195 | 475 | 0 | 0 | 243 | 8 605 | 41 023 |
| LGV | 31 881 | 284 | 0 | 24 823 | 2 389 | 4 585 | 7 118 | 162 | 637 | 41 | 2 824 | 5 531 | 80 274 |
| HGV | 22 289 | 0 | 81 | 6 991 | 637 | 637 | 6 371 | 556 | 0 | 0 | 0 | 1 436 | 38 999 |
| Other | 13 798 | 41 | 0 | 5 044 | 0 | 41 | 4 479 | 637 | 1 862 | 162 | 0 | 799 | 26 863 |
| TOTAL | 1 062 983 | 56 331 | 33 990 | 1 396 011 | 62 094 | 101 643 | 268 539 | 36 941 | 12 761 | 7 089 | 81 129 | 149 390 | 3 268 902 |

We can also see how the total accident cost for a certain type of collision is distributed between the two parts involved. In collisions between unprotected road users and MCs, 74% of the total cost falls on the unprotected road user. When unprotected road users are involved in accidents with cars and heavier vehicles almost 100% of the total cost falls on the unprotected road user.

Table 4-54: The distribution of total cost in accidents between two vehicle types, Sweden 1998

| Bearer of cost: | In accidents with: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Unprotected | MC | Car | Bus/Coach | LGV | HGV |
| Unprotected | 100% | 74% | 99% | 100% | 99% | 100% |
| Motorcycle (incl moped) | . | 100% | 96% | 98% | 100% | 99% |
| Car | . | . | 100% | 91% | 70% | 96% |
| Bus/Coach | . | . | . | 100% | 8% | 78% |
| LGV | . | . | . | . | 100% | 92% |
| HGV | . | . | . | . | . | 100% |

Of the total accident cost, the user bears a part himself (see section 3.5), mainly the risk value. The table below presents the users own cost, distributed by victim and injurer.

Table 4-55: Users own cost by victim and injurer, Sweden 1998 (kEuro)

| INJURER/ VICTIM | Single | Unprotected | MC | Car | Bus/Coach | LGV | HGV | Other transport | Train | Tram | Game | Other | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unprotected | 77 166 | 34 872 | 20 657 | 386 994 | 16 538 | 25 365 | 38 897 | 6 811 | 2 028 | 1 757 | 492 | 17 561 | 629 139 |
| Motorcycle(incl moped) | 89 756 | 7 398 | 3 798 | 110 353 | 4 433 | 5 978 | 8 650 | 1 336 | 2 510 | 422 | 6 377 | 1 300 | 242 313 |
| Car | 658 068 | 4 952 | 4 294 | 651 407 | 29 091 | 50 583 | 171 749 | 21 508 | 4 784 | 3 518 | 60 150 | 93 652 | 1 753 756 |
| Bus/Coach | 21 284 | 71 | 71 | 2 712 | 423 | 176 | 1 829 | 386 | 0 | 0 | 212 | 7 381 | 34 546 |
| LGV | 26 836 | 247 | 0 | 21 148 | 2 135 | 3 964 | 5 872 | 141 | 528 | 35 | 2 486 | 4 571 | 67 963 |
| HGV | 19 461 | 0 | 71 | 5 841 | 528 | 528 | 5 275 | 457 | 0 | 0 | 0 | 1 196 | 33 355 |
| Other | 12 074 | 35 | 0 | 4 148 | 0 | 35 | 3 820 | 528 | 1 677 | 141 | 0 | 669 | 23 127 |
| TOTAL | 904 646 | 47 575 | 28 890 | 1 182 603 | 53 149 | 86 629 | 236 093 | 31 166 | 11 526 | 5 873 | 69 718 | 126 330 | 2 784 199 |

Based on the total costs (Table 4-53) and the users own cost (Table 4-55) we can estimate the external cost by vehicle category (Table 4-56). The external cost is the total cost where a vehicle category is involved as victim or injurer with the users own cost subtracted. The proportion of external cost is the lowest for the unprotected users and increases with the level of ‘protection’. For car users the external cost is 35% of the total accident cost where cars are involved and for HGV the external cost is 89% in 1998.

The importance to record all accidents where a vehicle category is involved is obvious. Assume we only have recorded victims for a category, say HGVs. The total cost is 39 M€ of which 5.6 M€ is born by the rest of society and thus external. However, other users suffer from HGV accidents. The total cost for these non-HGV victims are 262 M€ (31+231). The external cost is 687% of the total cost as victim.

Table 4-56: Example of external accident cost for different vehicle categories, Sweden 1998 and 1996 (kEuro)

| 1998 | VICTIM |  |  | INJURER |  | VICT & INJ. | EXTERNAL COST |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Born by Rest of soc | Born by user | Total | Born by rest of soc | Born by other user | Total | Born by Rest of soc | Born by other user | Total | % |
| Unprotected | 110 780 | 629 139 | 739 919 | 2 012 | 12 703 | 754 634 | 112 792 | 12 703 | 125 495 | 17% |
| Motorcycle (incl moped) | 42 524 | 242 313 | 284 837 | 4 317 | 25 092 | 314 245 | 46 841 | 25 092 | 71 932 | 23% |
| Car | 303 231 | 1 753 756 | 2 056 987 | 96 024 | 531 196 | 2 684 207 | 399 255 | 531 196 | 930 451 | 35% |
| Bus/Coach | 6 477 | 34 546 | 41 023 | 8 882 | 52 725 | 102 630 | 15 359 | 52 725 | 68 084 | 66% |
| LGV | 12 311 | 67 963 | 80 274 | 14 394 | 82 665 | 177 332 | 26 705 | 82 665 | 109 369 | 62% |
| HGV | 5 644 | 33 355 | 38 999 | 31 350 | 230 818 | 301 167 | 36 994 | 230 818 | 267 812 | 89% |
| Other | 3 736 | 23 127 | 26 863 | 5 665 | 30 639 | 63 167 | 9 401 | 30 639 | 40 040 | 63% |
| Motor vehicles. | 370 187 | 2 131 933 | 2 502 120 | 86 348 | 496 455 | 3 084 924 | 456 536 | 496 455 | 952 991 | 31% |

| 1996 | VICTIM |  |  |  | INJURER |  |  | VICT & INJ. | EXTERNAL COST |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Born by Rest of soc | Born by user |  | Total | Born by rest of soc |  | Born by other user | Total | Born by Rest of soc | Born by other user | Total | % |
| Unprotected | 119 322 |  | 669 365 | 788 688 | 1 196 | 7 595 |  | 797 479 | 120 519 | 7 595 | 128 114 | 16% |
| Motorcycle(incl moped) | 45 596 |  | 258 998 | 304 593 | 3 877 | 20 838 |  | 329 308 | 49 473 | 20 838 | 70 310 | 21% |
| Car | 280 831 |  | 1 652 690 | 1 933 521 | 96 435 | 532 959 |  | 2 562 915 | 377 266 | 532 959 | 910 226 | 36% |
| Bus/Coach | 4 223 |  | 25 716 | 29 939 | 11 513 | 74 795 |  | 116 247 | 15 736 | 74 795 | 90 531 | 78% |
| LGV | 11 918 |  | 68 331 | 80 248 | 14 447 | 86 650 |  | 181 345 | 26 365 | 86 650 | 113 015 | 62% |
| HGV | 4 660 |  | 26 728 | 31 388 | 25 282 | 186 211 |  | 242 881 | 29 942 | 186 211 | 216 153 | 89% |
| Other | 4 059 |  | 22 531 | 26 589 | 6 290 | 37 173 |  | 70 052 | 10 348 | 37 173 | 47 522 | 68% |
| Motor vehciles | 347 227 |  | 2 032 463 | 2 379 690 | 87 403 | 499 840 |  | 2 966 933 | 434 630 | 499 840 | 934 471 | 31% |

For an aggregated of motorized vehicles the cost as victim was 2 502 M€ (1998) and the cost imposed on other road users was 496 M€, with an addition 86 M€ imposed on the rest of society for these accidents. The total cost of accidents where motorized vehicles where involved was thus 3 084 M€. Of this only 952 M€ was external, i.e. 31% of the total cost.

We have also analysed the cost, based on the accident matrix, for urban and non-urban areas. The information is presented in Table 4-57.

Table 4-57: Total accident cost by victim and injurer, non-urban and urban area, Sweden 1998 (kEuro)

| NON-URBAN | VICTIM |  |  |  | INJURER |  |  |  | VICT & INJ. |  | EXTERNAL COST |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Born by Rest of soc | Born by user |  | Total | Born by rest of soc |  | Born by other user |  | Total |  | Born by Rest of soc |  | Born by other user | Total | % |
| Unprotected | 18 114 |  | 126 060 | 144 175 |  | 482 |  | 4 094 |  | 148 750 | 18 596 | 4 094 |  | 22 690 | 15% |
| Motorcycle(incl moped) | 18 672 |  | 117 267 | 135 939 |  | 1 321 |  | 9 556 |  | 146 815 | 19 993 | 9 556 |  | 29 548 | 20% |
| Car | 205 371 |  | 1 193 437 | 1 398 808 |  | 22 048 |  | 141 790 |  | 1 562 646 | 227 419 | 141 790 |  | 369 209 | 24% |
| Bus/Coach | 3 667 |  | 19 640 | 23 307 |  | 3 294 |  | 21 195 |  | 47 796 | 6 961 | 21 195 |  | 28 156 | 59% |
| LGV | 8 388 |  | 46 417 | 54 805 |  | 5 967 |  | 38 374 |  | 99 146 | 14 355 | 38 374 |  | 52 729 | 53% |
| HGV | 4 785 |  | 28 850 | 33 635 |  | 20 773 |  | 159 637 |  | 214 045 | 25 558 | 159 637 |  | 185 194 | 87% |
| Other | 2 800 |  | 17 864 | 20 663 |  | 4 230 |  | 22 664 |  | 47 558 | 7 030 | 22 664 |  | 29 694 | 62% |
| Motor vehciles | 240 882 |  | 1 405 611 | 1 646 494 |  | 15 539 |  | 110 775 |  | 1 772 807 | 256 421 | 110 775 |  | 367 196 | 21% |

| URBAN | VICTIM |  |  |  |  |  | INJURER |  |  |  | VICT & INJ. |  | EXTERNAL COST |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Born by Rest of soc |  | Born by user |  | Total |  | Born by rest of soc |  | Born by other user |  | Total |  | Born by Rest of soc |  |  | Born by other user | Total |  | % |  |
| Unprotected |  | 92 561 |  | 502 586 |  | 595 147 |  | 1 530 |  | 8 609 |  | 605 287 |  | 94 091 | 8 609 |  |  | 102 701 |  | 17% |
| Motorcycle(incl moped) |  | 23 853 |  | 125 046 |  | 148 898 |  | 2 996 |  | 15 536 |  | 167 430 |  | 26 848 | 15 536 |  |  | 42 384 |  | 25% |
| Car |  | 97 860 |  | 560 319 |  | 658 179 |  | 73 971 |  | 389 371 |  | 1 121 521 |  | 171 831 | 389 371 |  |  | 561 202 |  | 50% |
| Bus/Coach |  | 2 810 |  | 14 906 |  | 17 716 |  | 5 587 |  | 31 531 |  | 54 833 |  | 8 397 | 31 531 |  |  | 39 928 |  | 73% |
| LGV |  | 3 923 |  | 21 546 |  | 25 469 |  | 8 426 |  | 44 291 |  | 78 186 |  | 12 350 | 44 291 |  |  | 56 640 |  | 72% |
| HGV |  | 859 |  | 4 505 |  | 5 364 |  | 10 577 |  | 71 181 |  | 87 122 |  | 11 436 | 71 181 |  |  | 82 617 |  | 95% |
| Other |  | 936 |  | 5 263 |  | 6 200 |  | 1 435 |  | 7 974 |  | 15 609 |  | 2 371 | 7 974 |  |  | 10 346 |  | 66% |
| Motor vehciles |  | 129 305 |  | 726 321 |  | 855 626 |  | 70 805 |  | 385 645 |  | 1 312 076 |  | 200 110 | 385 645 |  |  | 585 754 |  | 45% |

### Rail

The total accident cost for railways is estimated to 28 M€. This amount excludes accidents at level crossings. If these costs are included, which mainly are born by the road user as a victim, the cost increases to 38 M€. In addition, since 1998 the number of suicides is reported separately, and if these are valued as other fatalities the total accident cost will increase by 61 M€ to almost 100 M€. In the following, we will ignore the cost of suicides. However, when accident costs are compared between modes it should be remembered that suicides are included in other modes. The main part of the total accident cost, including road users cost at level crossings, is the cost of accidents with non-rail users outside level crossings (50%), at level crossings (26%) and with electricity (8%). The passenger-accidents cost 4% and the employee-accidents 7%. The remaining 5% is material cost for the railway companies.

Table 4-58: Railway accident cost by cost category, all Sweden (kEuro)

| keuro |  | Passenger | Railway employ. | Non-rail users at level crossings | Other Non-rail users excl suicide | Other Non-rail users electricity | Railway material cost | TOTAL | Total excl road users cost at level crossing | Suicide |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Risk value | 1996 | 320 | 961 | 17 727 | 2 624 | 0 | . | 21 632 | 3 905 | .. |
|  | 1998 | 1 392 | 2 353 | 8 943 | 17 567 | 2 624 | . | 32 879 | 23 936 | 56 667 |
| Net lost production | 1996 | 46 | 138 | 1 289 | 197 | 0 | . | 1 670 | 381 | .. |
|  | 1998 | 110 | 248 | 656 | 1 266 | 197 | . | 2 477 | 1 821 | 4 008 |
| Hospital care | 1996 | 34 | 103 | 96 | 24 | 0 | . | 258 | 161 | .. |
|  | 1998 | 20 | 124 | 57 | 79 | 24 | . | 303 | 247 | 146 |
| Administration | 1996 | 2 | 6 | 75 | 11 | 0 | . | 95 | 20 | .. |
|  | 1998 | 6 | 13 | 38 | 74 | 11 | . | 143 | 105 | 238 |
| Property damage | 1996 | .. | .. | 284 |  | .. | 2 166 | 2 450 | 2 166 | .. |
|  | 1998 | .. | .. | 144 | .. | .. | 1 985 | 2 130 | 1 985 | .. |
| Total Cost | 1996 | 403 | 1 208 | 19 471 | 2 856 | 0 | 2 166 | 26 104 | 6 633 | .. |
|  | 1998 | 1 529 | 2 737 | 9 839 | 18 986 | 2 856 | 1 985 | 37 932 | 28 093 | 61 058 |

It is forecasted that the level crossings accidents will decrease to 7 M€ in 2005 while other accidents will increase with the traffic volume to 33 M€ which results in a total accident cost on rail of 40 M€ for 2005.

We examine ‘cost born by user’ and ‘cost born by the rest of society’ for railway accidents in Table 4-59. We assume that the costs imposed on passengers, employees and railway material is internal. The external cost was in 1998 32 M€ if we include level crossing accidents. The external cost is as high as 85% (1998).

Table 4-59: Railway accident cost by users cost and cost for the rest of society (R o S), all Sweden (k€)

| VICTIMS where RAIL is INJURER |  |  |  |  |  |  | Internal | External cost |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1996 |  |  | 1998 |  |  | to | 1996 | 1998 |
|  | R o S | User | TC | R o S | User | TC | Rail |  |  |
| Passenger | 76 | 327 | 403 | 129 | 1400 | 1529 | Yes | 76 | 129 |
| Railway employees | 229 | 980 | 1208 | 357 | 2380 | 2737 | Yes | 229 | 357 |
| Non-rail users at level crossings | 1398 | 18073 | 19471 | 718 | 9121 | 9839 | No | 19471 | 9839 |
| Other excl. suicde | 219 | 2637 | 2856 | 1346 | 17640 | 18986 | No | 2856 | 18986 |
| Other electricity | 0 | 0 | 0 | 219 | 2637 | 2856 | No | 0 | 2856 |
| Railway material cost | 108 | 2058 | 2166 | 99 | 1886 | 1985 | Yes | 108 | 99 |
| TOTAL | 2030 | 24073 | 26104 | 2868 | 35063 | 37932 |  | 22740 | 32266 |
| Total excl level crossing | 633 | 6001 | 6633 | 2150 | 25943 | 28093 |  | 3269 | 22427 |
| Suicide | .. | .. | .. | 4171 | 56888 | 61058 | . | . | . |

### Public transport

The total accident costs where metro or tram passengers are victims are estimated to 12 M€. The number is uncertain, and the disaggregation into cost categories in the table below is only indicative. We assume the cost constant until 2005.

Table 4-60: Tram and Metro accident cost by cost category (kEuro)

| kEuro |  | TOTAL |
| --- | --- | --- |
| Risk value | 1996 | 8 660 |
|  | 1998 | 10 730 |
| Net lost production | 1996 | 777 |
|  | 1998 | 1 067 |
| Hospital care | 1996 | 273 |
|  | 1998 | 491 |
| Administration | 1996 | 56 |
|  | 1998 | 74 |
| Property damage | 1996 | .. |
|  | 1998 | .. |
| Total Cost | 1996 | 9 766 |
|  | 1998 | 12 362 |

In addition to the cost born by metro users and tram users, we have reported cost born by road users in accidents with tram (7.0 M€) in the road mode. We know also the total accident cost for buses in urban and non-urban areas, and if the bus passenger or other road users is the victim. This is presented in the road mode.

Taking this wider definition of public transport, the total accident cost where public transport is involved increases to 122 M€ in 1998. The majority of this cost is bus accidents where other road users are killed or injured (see section 4.6.1).

Table 4-61: Total accident cost by victim and injurer, Sweden 1998 (k€)

|  | Other | Metro | Tram | Bus urban | Bus non urban | TOTAL |
| --- | --- | --- | --- | --- | --- | --- |
| Metro | 6 920 |  |  |  |  | 6 920 |
| Tram | 5 442 |  |  |  |  | 5 442 |
| Bus urban | 17 310 |  |  | 406 |  | 17 716 |
| Bus-non urban | 23 226 |  |  |  | 81 | 23 307 |
| Other road | .. |  | 7 089 | 37 118 | 24 489 | 68 696 |
| TOTAL | 52 898 | 0 | 7 089 | 37 524 | 24 570 | 122 081 |

Based on the information of cost born by user from section 4.6.1 for buses and the split of total accident cost between cost born by users and the rest of society presented in section 3 we can estimate the external cost for public transport at large. As expected, metro has a low share of external cost. The highest share can be found for tram and urban buses. These two categories have in common that they are mixing with many unprotected road users. The information on tram accidents is uncertain while the statistics for buses has a higher degree of certainty.

Table 4-62: Example of external accident cost for different public transport categories, Sweden 1998 (k€).

|  | TOTAL COST | Born by user | Rest of soicety | Born by other users | Proportion external (%) |
| --- | --- | --- | --- | --- | --- |
| Metro | 6 920 | 5916 | 1 004 | 0 | 15% |
| Tram | 12 531 | 4653 | 1 817 | 6061 | 66% |
| Bus urban | 54 834 | 14 906 | 8397 | 31 531 | 73% |
| Bus-non urban | 47 796 | 19 640 | 6961 | 21 195 | 59% |

### Aviation

The total accident cost in Swedish air was 7 M€ in the aviation sector year 1998. The majority of the cost is caused by private operations (93%). In aviation, only ‘single’ accidents occurred during the observed two years. We have estimated that 6.5 M€ is born by users and 0.6 M€ born by the rest of society. The total cost in 2005 is estimated to 18 M€, due to a strong increase in the number of flight hours for private operations.

Table 4-63: Aviation accident cost by cost category (kEuro)

| kEuro |  | Scheduled and non-scheduled traffic | Miscellaneous commercial operations | Instructional operations | Private operations | Non profit aerial work | TOTAL |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Risk value | 1996 | 0 | 2 784 | 1 552 | 1 417 | 0 | 5 753 |
|  | 1998 | 160 | 160 | 0 | 5 088 | 0 | 5 408 |
| Net lost production | 1996 | 0 | 220 | 133 | 113 | 0 | 466 |
|  | 1998 | 23 | 23 | 0 | 371 | 0 | 417 |
| Hospital care | 1996 | 0 | 41 | 38 | 22 | 0 | 101 |
|  | 1998 | 17 | 17 | 0 | 30 | 0 | 64 |
| Administration | 1996 | 0 | 12 | 7 | 7 | 0 | 27 |
|  | 1998 | 1 | 1 | 0 | 22 | 0 | 24 |
| Property damage | 1996 | 25 224 | 213 | 191 | 1 076 | 0 | 26 704 |
|  | 1998 | 0 | 135 | 67 | 942 | 0 | 1 143 |
| Total Cost | 1996 | 25 224 | 3 270 | 1 921 | 2 635 | 0 | 33 050 |
|  | 1998 | 201 | 336 | 67 | 6 452 | 0 | 7 057 |

### Maritime

For the Maritime sector we have detailed information for Swedish ships on all water and less detailed information for ships registered in all countries in Swedish water. None of the definitions will assure that an aggregate of UNITE accounts will cover all maritime accidents in Europe.

The total accident cost of maritime accidents is 81 M€ in Swedish water 1998 of which 55 M€ is due to pleasure boats (in this version we have not included the material damages). Accidents with Swedish ships costs 65 M€ including pleasure boats. Excluding pleasure boats the cost is 9.5 M€ of which 86% are accidents with fishing vessels and 14% with passengers on bulk ship cost. The total accident cost born by the rest of society is estimated to be 8%.

Table 4-64: Maritime accident cost, by cost category (kEuro)

|  |  | Swedish ships on all water |  |  |  |  |  |  |  |  | All ships on Swedish water |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Passenger ship |  | Tanker | Bulk |  | Fishing vessel | Other | Pleasure boat | Total | Commercial ship | D.O incl pleasure boats | Emp. all Com.ship | (illness and suicide) |
| kEuro |  | Emp. | Pass. | Emp. | Emp. | Pass. | Emp. |  |  |  | Total | Total |  |  |
| Risk value | 1996 | 0 | 12 | 0 | 0 | 0 | 1 404 | 0 | 40 652 | 42 069 | 7 675 | 48 327 | 6 566 | 8 623 |
|  | 1998 | 0 | 25 | 0 | 0 | 1 232 | 7 613 | 0 | 51 739 | 60 609 | 23 480 | 75 219 | 4 312 | 8 623 |
| Net lost production | 1996 | 0 | 1 | 0 | 0 | 0 | 105 | 0 | 2 875 | 2 981 | 552 | 3 427 | 593 | 610 |
|  | 1998 | 0 | 3 | 0 | 0 | 87 | 545 | 0 | 3 659 | 4 294 | 1 663 | 5 322 | 401 | 610 |
| Hospital care | 1996 | 0 | 1 | 0 | 0 | 0 | 17 | 0 | 105 | 123 | 41 | 146 | 328 | 22 |
|  | 1998 | 0 | 2 | 0 | 0 | 3 | 36 | 0 | 134 | 175 | 66 | 200 | 245 | 22 |
| Administration | 1996 | 0 | 0 | 0 | 0 | 0 | 12 | 0 | 170 | 183 | 42 | 213 | 171 | 36 |
|  | 1998 | 0 | 1 | 0 | 0 | 5 | 40 | 0 | 217 | 263 | 101 | 318 | 126 | 36 |
| Property damage | 1996 |  |  |  |  |  |  |  |  | 0 | 0 | 0 |  |  |
|  | 1998 |  |  |  |  |  |  |  |  | 0 | 0 | 0 |  |  |
| Total Cost | 1996 | 0 | 15 | 0 | 0 | 0 | 1 538 | 0 | 43 803 | 45 355 | 8 310 | 52 112 | 7 659 | 9 291 |
|  | 1998 | 0 | 30 | 0 | 0 | 1 327 | 8 235 | 0 | 55 749 | 65 341 | 25 310 | 81 059 | 5 084 | 9 291 |
| Gross lost production | 1996 | 0 | 1 | 0 | 0 | 0 | 434 | 0 | 13 736 | 14 172 | 2 526 | 16 263 | 1 251 | 2 914 |
|  | 1998 | 0 | 3 | 0 | 0 | 416 | 2 520 | 0 | 17 483 | 20 422 | 7 916 | 25 399 | 731 | 2 914 |

For other modes, we have not included suicides where it has been possible to distinguish them from other accidents (rail). In addition, work related accidents are partly included and partly excluded in the statistics for other modes. These two components are 9.3 and 5.0 M€ respectively, which makes the total cost 14.3 M€. The number of accidents involving pleasure boats is forecasted to decline while other the total cost for other accidents are assumed to be constant. The total cost for 2005 becomes 39 M€.

## Environmental Cost

The total environmental cost is 1 104 Million € which is 0.5% of the Swedish GDP. The majority of the cost is estimated for road transport (89%) and only 5% is the result of railway traffic (including production of electricity) and 6% from aviation. No estimate is made for public transport (within road) or the maritime sector. Due to missing information on PM-emissions the environmental costs of the aviation sector are underestimated.

The largest part of the environmental cost in this estimate is global warming (45%) followed by air pollution (38%) and noise (17%).

Table 4-65: Environmental cost (k€)

|  | Road | % | Rail | % | PT | % | Aviation | % | Maritime | % | Total | % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Air pollution | 410 000 | 42% | 4 900 | 10% | (27) | 51% | 2 100 | 3% | .. | . | 417 000 | 38% |
| Global warming | 430 000 | 44% | 3 200 | 6% | (18) | 34% | 64 600 | 96% | .. | . | 497 800 | 45% |
| Noise | 143 026 | 15% | 42 850 | 84% | (8) | 15% | 415 | 1% | .. | . | 189 291 | 17% |
| Total | 983 026 | 100% | 50 950 | 100% | (53) | 100% | 67 115 | 100% | .. | . | 1 104 091 | 100% |

This is a low estimate compared to what has been achieved in Sweden previously. With the same input data and the Swedish values presented in chapter 3 we can estimate an environmental cost of 3 817 Million €. The UNITE values is approximately ¼ of this very rough estimate. This suggests very huge differences between the two methods.

### Road

The total cost of air pollution and global warming is presented for the years 1996, 1998 and 2005 in the table below divided into highway, urban and rural areas.

Table 4-66: Total cost for air pollution and green house gases in Swedish road sector (k€)

|  | Air pollution |  | Green house |  | Indirect |  | Total |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1996 |  |  |  |  |  |  |  |  |
| Highway | 50 000 |  | 48 000 |  | 11 000 |  | 109 000 |  |
| Urban | 186 000 |  | 147 000 |  | 35 000 |  | 368 000 |  |
| Rural | 170 000 |  | 170 000 |  | 38 000 |  | 379 000 |  |
| Total | 407 000 |  | 366 000 |  | 84 000 |  | 856 000 |  |
| 1998 |  |  |  |  |  |  |  |  |
| Highway | 45 000 |  | 51 000 |  | 11 000 |  | 107 000 |  |
| Urban | 172 000 |  | 152 000 |  | 35 000 |  | 359 000 |  |
| Rural | 154 000 |  | 180 000 |  | 40 000 |  | 374 000 |  |
| Total | 370 000 |  | 383 000 |  | 87 000 |  | 839 000 |  |
| 2005 |  |  |  |  |  |  |  |  |
| Highway |  | 28 000 |  | 61 000 |  | 14 000 |  | 103 000 |
| Urban |  | 108 000 |  | 160 000 |  | 42 000 |  | 310 000 |
| Rural |  | 97 000 |  | 216 000 |  | 48 000 |  | 362 000 |
| Total |  | 233 000 |  | 438 000 |  | 103 000 |  | 774 000 |

The cost is in the table below expressed as an average cost per kilometre by vehicle type for the whole of Sweden. In the statistical annex, a more detailed break down by area is presented.

Table 4-67: Cost by vehicle type (€/100vkm)

|  |  | 1996 | 1998 | 2005 |
| --- | --- | --- | --- | --- |
| Motorcycles | Petrol | 0.87 | 0.89 | 0.86 |
| Passenger Cars | Petrol | 0.92 | 0.86 | 0.59 |
| Passenger Cars | Diesel | 1.61 | 1.14 | 0.71 |
| Buses | Diesel | 4.36 | 4.00 | 3.17 |
| Light Goods Vehicles (< 3.5 t) | Diesel | 1.44 | 1.35 | 1.02 |
| Heavy Goods Vehicles (> 3.5 t) | Diesel | 5.23 | 4.85 | 3.52 |

The result from the noise cost estimate (in k€) is presented in Table 4-68.

Table 4-68: Total cost for noise in Swedish road sector (k€)

| k€ | Road |
| --- | --- |
| Myocardial infarction | 2 013.9 |
| Angina pectoris | 20.1 |
| Hypertension | 1.4 |
| Sleep disturbance | 16 467 |
| Amenity losses | 124 524 |
| Total | 143 026 |

### Rail

The majority of the cost for air pollution and global warming in the railway sector origins from the diesel trains (70%). The forecast for 2005 suggests that all costs will origin from diesel trains (note: the table below is in Million €).

Table 4-69: Total cost for air pollution and green house gases in Swedish rail sector (M€)

|  | Diesel |  |  |  | Electrical |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Air pollution | Green house | Indirect | Total | Air pollution | Green house | Indirect | Total | TOTAL |
| 1996 |  |  |  |  |  |  |  |  |  |
| High Speed Trains | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 | 0.1 | 0.1 |
| Passenger Trains | 1.6 | 0.4 | 0.1 | 2.0 | 0.1 | 0.4 | 0.0 | 0.5 | 2.6 |
| Freight Trains | 3.1 | 0.9 | 0.1 | 4.1 | 0.1 | 0.6 | 0.0 | 0.7 | 4.8 |
| Total | 4.6 | 1.3 | 0.2 | 6.1 | 0.2 | 1.1 | 0.0 | 1.4 | 7.5 |
| 1998 |  |  |  |  |  |  |  |  |  |
| High Speed Trains | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 | 0.2 | 0.2 |
| Passenger Trains | 1.5 | 0.4 | 0.1 | 2.0 | 0.2 | 0.5 | 0.1 | 0.8 | 2.7 |
| Freight Trains | 2.7 | 0.9 | 0.1 | 3.7 | 0.5 | 1.0 | 0.1 | 1.5 | 5.2 |
| Total | 4.2 | 1.3 | 0.2 | 5.7 | 0.7 | 1.6 | 0.2 | 2.4 | 8.1 |
| 2005 |  |  |  |  |  |  |  |  |  |
| High Speed Trains | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Passenger Trains | 2.0 | 0.7 | 0.1 | 2.8 | 0.0 | 0.0 | 0.0 | 0.0 | 2.8 |
| Freight Trains | 4.1 | 1.4 | 0.2 | 5.7 | 0.0 | 0.0 | 0.0 | 0.0 | 5.7 |
| Total | 6.1 | 2.1 | 0.3 | 8.5 | 0.0 | 0.0 | 0.0 | 0.0 | 8.5 |

The cost is in the table below expressed as an average cost per kilometre by vehicle type for Sweden.

Table 4-70: Cost by vehicle type (€/100vkm)

|  | Diesel | Electrical |
| --- | --- | --- |
| 1996 |  |  |
| High Speed Trains | 0.00 | 1.02 |
| Passenger Trains | 25.62 | 1.07 |
| Freight Trains | 131.58 | 2.09 |
| 1998 |  |  |
| High Speed Trains | 0.00 | 1.04 |
| Passenger Trains | 21.68 | 1.64 |
| Freight Trains | 118.27 | 4.22 |
| 2005 |  |  |
| High Speed Trains | 0.00 | 0.00 |
| Passenger Trains | 23.41 | 0.00 |
| Freight Trains | 126.26 | 0.01 |

Table 4-71: Total cost for noise in Swedish rail sector 1998 (k€)

|  | Rail |
| --- | --- |
| Myocardial infarction | 1 092 |
| Angina pectoris | 10.9 |
| Hypertension | 0.8 |
| Sleep disturbance | 4 809 |
| Amenity losses | 36 936 |
| Total | 42 850 |

### Aviation

The total cost for air pollution and global warming for the Swedish aviation sector is presented in the table below (note: the table below is in Million €).

Table 4-72: Total cost for air pollution and green house gases in Swedish aviation sector (M€)

|  | Air pollution | Green House | Indirect | TOTAL |
| --- | --- | --- | --- | --- |
| 1996 |  |  |  |  |
| Airports | 2.1 | 5.4 | 1.0 | 8.5 |
| Flights | 0.0 | 40.6 | 9.4 | 49.9 |
| Total | 2.1 | 46.0 | 10.4 | 58.5 |
| 1998 |  |  |  |  |
| Airports | 2.1 | 6.0 | 1.1 | 9.2 |
| Flights | 0.0 | 46.8 | 10.7 | 57.5 |
| Total | 2.1 | 52.8 | 11.8 | 66.7 |
| 2005 |  |  |  |  |
| Airports | 3.1 | 7.6 | 1.4 | 12.2 |
| Flights | 0.0 | 59.5 | 13.6 | 73.1 |
| Total | 3.1 | 67.1 | 15.0 | 85.3 |

Table 4-73: Total cost for noise in Swedish aviation sector 1998 (k€)

|  | Aviation |
| --- | --- |
| Myocardial infarction | * |
| Angina pectoris | * |
| Hypertension | * |
| Sleep disturbance | ** |
| Amenity losses | 414.8 |
| Total | 414.8 |

* exposure does not exceed threshold value of exposure-response function

** no data for night time noise; assumption: no flights during the night, therefore no sleep disturbance

## Taxes, Charges and Subsidies

To make a complete summary of the area taxes, charges and subsidies in the transport sector is a huge challenge that is far beyond the available resources in the Pilot Account project. The results we show in this report, are not a result from a complete investigation, and must be seen as a skimming of the surface. However, the sources that we use and the figures we present are reliable. The method used was not to do calculation by our selves but to collect available data. Because of data availability, the mode urban public transport becomes public transport, which includes all the public transport and not only the urban. The following tables show available figures of taxes, charges, and subsidies for the different modes road, rail, public transport, aviation and maritime transport.

### Road

Table 4-74: Taxes, charges and subsidies road, thousand € 1998 prices

|  | 1996 | 1998 | 2005 | From | To |
| --- | --- | --- | --- | --- | --- |
| Road charges | 0 | 59 000 | .. | Freight cars | State |
| Vehicle tax | 629 000 | 684 000 | .. | Vehicle owner | State |
| Sales tax | 161 000 | 30 000 | .. | Vehicle owner | State |
| Fuel tax petrol | 2 730 000 | 2 709 000 | 2 730 000 | Owner of petrol vehicles | State |
| Fuel tax diesel | 729 000 | 838 000 | 963 000 | Owner of diesel vehicles | State |
| VAT on fuel (25%) | 865 000 | 887 000 | 924 000 | Vehicle owner | State |
| Appropriation* | 749 689 | 914 896 | .. | State | VV |
| Tax relief on diesel | 643 498 | 747 758 | .. | State | Owners of diesel vehicles |
| Tax relief according to travel between home and workplace | 386 000 | 480 000 | .. | State | Individual |

*A minor part belong to rail and public transport and it is difficult to separate it from the road appropriation. (Sources: VV årsred, Brevsvar från finansdepartementet,  Prop. 1997/1998:150 bilaga 6 skatteavvikelser)

The only charge on road use in Sweden in 1998 was the eurovinjett (Eurovignette). In the year 1996 no such charge existed. The forecast for 2005 is based on the forecasted change in fuel consumption of petrol and diesel (See 4.7.1).

### Rail

Table 4-75: Taxes charges and subsidies rail, thousand € 1998 prices

|  | 1996 | 1998 | 2005 | From | To |
| --- | --- | --- | --- | --- | --- |
| Subsidies* | 109165 | 127 803 | .. | State | BV |
| Appropriation | 314 256 | 364 361 | .. | State | BV |
| Subsidy | 4 171 | 8 125 | .. | Eu | BV( bv årsrapp not 9.) |
| Infrastructure charge | .. | 80 726 | .. | Infrastructure user | State |
| Infrastructure charge | .. | 17 457 | .. | Infrastructure user | BV |
| Charges** | 51 098 | 1 324 888 | .. | Personnel and freight charge | Railway companies |

*Tax relief for track operation and electricity and fuel use in track operation.

**Figures from supplier operating cost

(Sources: Prop. 1997/98:150 appendix 6 tax deviations, BV annual report)

Looking at the development of the charges/revenues, it stands out that there is a substantial increase between 1996 and 1998. The reason is that for the year 1996 the Swedish rail company SJ was not included in the structural business accounts from Statistics Sweden while for 1998 it was.

### Public Transport

Table 4-76: Taxes charges and subsidies public transport thousand € 1998 prices

|  | 1996 | 1998 | 2005 | From | To |
| --- | --- | --- | --- | --- | --- |
| Subsidies total | 780 309 | 752 462 | .. | State, county council, local authorities | PT companies |
| Subsidy | 28 146 | 27 789 | .. | State | PT companies |
| Subsidy | 501 973 | 480 776 | .. | County council | PT companies |
| Subsidy | 250 190 | 243 897 | .. | Local authorities | PT companies |
| Tax relief * | 227 522 | 232 063 | .. | State | PT companies |
| Charges** | 712 388 | 729 924 | ... | Individual | PT companies |

*Due to different VAT rate on public transport; **Figures from supplier operating cost.

(Sources: SLTF trade statistics, Prop. 1997/98:150 appendix 6 tax deviations)

The major part of the subsidies to public transport stems from county council followed by local authorities while the state stands for a minor part.

### Aviation

Table 4-77: Taxes charges and subsidies aviation, in thousand € 1998 prices

|  | 1996 | 1998 | 2005 | From | To |
| --- | --- | --- | --- | --- | --- |
| Taxes | 13 444* |  | .. | Airline companies | State |
| Subsidies |  |  | .. |  |  |
| Infrastructure income total | 279 806 | 327 995 | .. | Airline companies | LFV and municipal companies |
| State | 273 318** | 319 955 | .. | Airline companies | LFV |
| Airport revenues | .. | 183.6 | .. | Airline companies | LFV |
| ATM charges | .. | 20.4 | .. | Airline companies | LFV |
| En route | .. | 98.9 | .. | Airline companies | LFV |
| Capacity agreement, security | .. | 17.0 | .. | Airline companies | LFV |
| Municipal | 6 488** | 8 040 | .. | Airline companies | Municipal companies |
| Charges | 2 214 015*** | 2 496 412*** | .. | Buyer of air services | Airline companies |

*Tax on domestic flight

**Traffic income

*** Net sales excluding excise duties

(Sources: LFV flygplatsjämförelse 1996 och 1998 statliga och kommunala flygplatser samt egna beräkningar, Charges från supplier operating cost)

The tax on domestic flight disappeared in 1997 and we could not find any other taxes on air that would be in line with the pilot accounts approach. Neither could we find any subsidies but the picture would perhaps be different if we had investigated the accounts of the local authorities.

### Maritime transport

Table 4-78: Taxes charges and subsidies maritime transport, in thousand € 1998 prices

|  | 1996 | 1998 | 2005 | From | To |
| --- | --- | --- | --- | --- | --- |
| Pilot age | 14 408 | 0 | .. | Shipping company | SFV |
| Fairway charge | 57 109 | 100 391 | .. | Shipping company | SFV |
| Light charge | 33 401 | 0 | .. | Shipping company | SFV |
| Sum. fairway charges | 104 919 | 100 391 | .. | Shipping company | SFV |
| Ship harbour charge | 49 444 | 57 693 | .. | Shipping company | Harbour authority |
| Goods harbour charge | 70 733 | 79 550 | .. | Shipping company | Harbour authority |
| Passenger charge. | 7 069 | 5 751 | .. | Shipping company | Harbour authority |
| Gods handling income | 208 169 | 233 229 | .. | Shipping company | Harbour authority |
| Crane charge | 11 968 | 12 550 | .. | Shipping company | Harbour authority |
| Sum. port charges | 347 383 | 388 773 | .. | Shipping company | Harbour authority |
| Appropriation | 11 652 | 11 885 | .. | State | SFV |
| Subsidies* | 47 113 | 82 960 | .. | State | Shipping company |
| Sum. appropriation and subsidies | 58 765 | 94 845 | .. | State | SFV, shipping company |
| Charges Ferry at sea** | 942 965 | 891 928 | .. | Individual and Freight companies | Shipping company |
| Charges Other sea coast transport** | 2 167 647 | 2 407 960 | .. | Individual and Freight companies | Shipping company |
| Charges Inland waterway transport** | 66 847 | 9 406 | .. | Individual and Freight companies | Shipping company |
| Sum. charges | 3 177 459 | 3 309 294 | .. | Individual and Freight companies | Shipping company |

*Tax relief according to domestic water transport

* *Net sales excluding excise duties

(Sources: SFV årsredovisning, Prop.1997/98:150 bilaga 6 skatteavvikelser, Charges från supplier operating cost)

# Final Presentation of the Results per Mode

In this chapter we present first the result for all modes together (5.1), then we summarise some UNITE common basic indicators (5.2) before the result by mode are presented (5.2-5.7). The discussion around the cost components by mode is short, as the methods used are presented in Chapter 3 and  detailed information Chapter 4 of the Swedish report.

## The Swedish result in perspective

We can define four levels of cost bearers in the transportation sector. First, we have the user or customer, who pays the ticket, tariff, or own vehicle-operating cost (VOC) including fuel taxes. In addition to the monetary cost, the user bears the ‘cost’ of travel time and accident risks. Secondly, the supplier of transport services, such as rail and bus companies or trucking companies, receives ticket revenues from the user, has costs for their vehicles, and employees (VOC), pays charges to infrastructure provider and taxes to the State or receives subventions. Thirdly, the infrastructure provider maintains and builds infrastructure and receives revenues from charges or appropriation from the state. Finally, the rest of the society, including the state, receives general taxes and pays subventions and appropriation to the infrastructure provider. In addition, the rest of the society bears costs for environmental damages and hospital care in the case of accidents.

Figure 5-2: Structure of costs and revenues

The UNITE account concentrates on the infrastructure providers cost and the cost for accidents and environment born by the rest of the society. The UNITE account also includes supplier operating cost for rail and public transport. As additional information own accident cost and delay time costs are included for some modes.

The total cost for infrastructure, supplier operation, accidents and environment is estimated to be 24 000 Million € in 1998 (11% of GDP). We have not been able to estimate the cost for the users own resources, time spent while travelling and VOT for own account transport. Of the estimated cost, 63% is supplier operating cost, 17% infrastructure cost, 15% accident cost and 5% environmental cost.

The core costs, infrastructure, external accidents and environmental costs are estimated to be 6 265 Million € in 1998. If we also include the internal accident cost and the small part of delay cost we have estimated, the core cost rises to 8 986 Million €. With this broader definition of core costs we nevertheless only cover 37% of the total cost we have estimated.

- While it is important to ensure efficient provision and use of the infrastructure - which includes pricing for infrastructure damages, external accident cost, environment, and congestion - it is also important to ensure that the transportation market, where the majority of the costs are generated, is working properly.

Figure 5-3: Structure of costs in Swedish transport sector 1998, M€

The total infrastructure cost is 4 042 Million € (1.9% of GDP). The total environmental cost is estimated to 1 154 Million € or 0.5% of GDP with 38% for air pollution, 45% green house gases and 17% noise cost. The UNITE methodology assumes that a large part of the accident cost is internal to the user. In addition to the accident cost born by the rest of society, the users bears 2 638 Million € in own risk valuation. The total sum of the accident cost is 3 707 Million € or 1.7% of the GDP. The total time cost is not estimated in UNITE. Delay cost is estimated for aviation and rail transport.

- We have estimated infrastructure costs to 1.9% of GDP, total accident cost to 1.7% and environmental cost to 0.5% of GDP.

Figure 5-4: Annual infrastructure, accidents and environmental cost for the Swedish infrastructure sector 1998 (M€)

The users, including firms, pay 21 266 Million € in taxes, tariffs and charges which is 10 % of GDP. The majority is payment to supplier of infrastructure services but 29% is taxes and charges. In addition, users pay own vehicle costs, both private and firms on own account.

Total taxes and charges, including VAT on the tax and charge, was 6 122 Million € in 1998. The inclusion of VAT follows the UNITE methodology. More than half of the taxes and charges (59%) are fuel tax (3 547 Million €) paid by road users, 14% VAT on taxes and charges, 11% vehicle tax and only 16% direct allocated revenues. In fact, this is also an over estimation as neither the Eurovignette for road nor rail track charges is directly allocated to the infrastructure provider.

We have found 2 274 Million € in state financing of infrastructure. In addition, the state can support the transport sector through subventions. The subvention can take the form of subvention per se or tax relief. We have found 1 943 Million € in subventions. The majority of the subvention is tax relief, both from fuel tax (45%) and VAT (12%). Subvention per se to companies is only 4% and 39% is concessionaire fares.

Table 5-79: Summary of costs, tariffs, charges and taxes in Swedish transport sector 1998, M€.

| MODEL | Infrastructure cost | External accident cost | Environmental cost | Internal accident cost | Delay | Supplier operating cost | User tariff | Infr Charges | Subsidy | Fuel tax | Other tax | VAT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ROAD | 2 159 | 953 | 982 | 2 502 |  | 6 564 | 7 198 | 59 |  | 3547 | 714 | 887 |
| RAIL | 856 | 32 | 51 | 6 | 63 | 1269 | 1 325 | 98 |  | 0 | 0 | 0 |
| PT | 0 | 77 | 53 | 49 |  | 1720.9 | 729.9 |  | 752.5 |  |  |  |
| AVIATION | 447 | 1 | 67 | 7 | 21 | 2481 | 2496 | 328 |  |  |  |  |
| MARITIME | 580 | 6 | 0 | 75 |  | 3279 | 3394 | 489 | 83 |  |  |  |
| TOTAL | 4 042 | 1 068 | 1 154 | 2 638 | 84 | 15 315 | 15 143 | 974 | 835 | 3 547 | 714 | 887 |

We have also made forecasts for 2005 and in the absence of any forecast we have assumed the cost to be constant. Our observed costs for 1996 and 1998 describe an increasing trend (+7%) while our forecast for 2005 suggests that the cost will fall (-9%). The sharp decline in the costs is thanks to the positive forecasted development (-31%) in road safety due to the ‘Zero-accident vision’ (which is not zero accidents by 2005). In addition, the environmental cost will decline (-6%). We have not made any forecasts for the delay costs.

- If the ‘Zero-accident vision’ is successful in the future without increasing the infrastructure cost or the non monitored time cost of users, the core cost of Swedish transport infrastructure has reached a peak and will start to decline.

Figure 5-5 Core cost (Infrastructure, external accident and environment) by mode, 1996, 1998 and 2005 (M€)

## Basic indicators

In order to obtain a clear picture of the transport situation in Sweden, basic social, economic and transport indicators are presented in Table 5-80 and Table 5-81.

Table 5-80: Basic indicators for Sweden 1996 and 1998

|  | unit | 1996 | 1998 |
| --- | --- | --- | --- |
| Land area | sqkm | 449 964 | 449 964 |
| Population | 1 000 | 8 845 | 8 854 |
| Population density | inhabitants/sqkm | 19.7 | 19.7 |
| Population employed | 1 000 | 4065 | 4071 |
| Employment Rate | % | 91.9 | 93.5 |
| GDP | € million 1998 prices | 197 670 | 213 604 |
| GDP per capita | € million 1998 prices | 22.348 | 24 125 |
| GDP growth rate(change to previous year) | % (in prices of 1995) | 1.1 | 3.6 |
| Consumer price index | 1995 = 100 | 100.5 | 100.9 |

Sources: See Section 4.1

Table 5-81 Basic transport related indicators for Sweden 1998 per mode

| Indicator |  | Unit |  | Road |  | Rail |  |  | Public transport |  | Aviation |  | Maritime shipping |  | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Transport performance1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Passenger Carried 2) |  | Mill. |  | 4 722 |  | 471 |  |  | 1 0773) |  | 22 |  | 7.3 |  | 6299 |
|  |  | % |  | 75.0 |  | 7.5 |  |  | 17.0 |  | 0.3 |  | 0.1 |  | 100 |
| Passenger-km |  | Mill. Pkm |  | 80 842 |  | 7 144 |  |  | 12 373 |  | 48854) |  | 270 |  | 105 514 |
|  |  | % |  | 70.2 |  | 6.3 |  |  | 10.9 |  | 12.5 |  | 0.1 |  | 100 |
| Goods transported |  | Mill. t |  | 336 |  | 56 |  |  | - |  | 0.23 |  | 1.4 |  | 393.6 |
|  |  | % |  | 85.4 |  | 14.2 |  |  | - |  | 0 |  | 0.4 |  | 100 |
| Tonne-km |  | Mill. tkm |  | 32 674 |  | 19 086 |  |  | - |  | 347 |  | 30 4315) |  | 82 538 |
|  |  | % |  | 54 |  | 31.6 |  |  | - |  | 0.6 |  | 13.8 |  | 100 |
| Network length6) |  | 1000 km |  | 136.5 |  | 15.1 |  |  | .. |  | . |  | . |  | 151.6 |
| Employees |  | 1000 |  | 72 345 |  | 11 482 |  |  | 9 708 |  | 12 785 |  | 14 437 |  | 120 757 |
| Investments |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Gross investments7) |  | € mill. |  | 831.6 |  | 755 |  |  | .. |  | 24 |  | 10.7 |  | 1621.3 |
|  |  | % |  | 51.3 |  | 46.6 |  |  | .. |  | 1.5 |  | 0.6 |  | 100 |
| Gross capital stock8) | € mill. |  | 8 128 |  | 6 956 |  |  | .. |  | 846 |  | 695 |  | 16 625 |  |
|  | % |  | 48.9 |  | 41.8 |  |  | . |  | 5.1 |  | 4.2 |  | 100 |  |
| Accidents |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Number of injuries | Persons |  | 51 2549) |  | 1310) |  |  | 25 (882) |  | 3 |  | .. |  | 51 295 |  |
| Number of fatalities | Persons |  | 531 |  | 25 |  |  | 5 (8) |  | 4 |  | 4311) |  | 608 |  |
| Direct transport emissions12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CO2 | Mill. t |  | 19.1 |  | 0.15 |  |  | .. (0.90)13) |  | 2.6 |  | .. |  | 21.85 |  |
| PM2.5 | t |  | .. |  | .. |  |  | .. |  | .. |  | .. |  | .. |  |
| PM10 | t |  | 3 267 |  | 90 |  |  | .. (226) 13) |  | .. |  | .. |  | 3 357 |  |
| NOx | t |  | 125 728 |  | 1 661 |  |  | .. (9113) 13) |  | 105514) |  | .. |  | 128 444 |  |
| SO2 | t |  | 1 101 |  | 201 |  |  | .. (48) 13) |  | 9514) |  | .. |  | 1 397 |  |
| NMVOC | t |  | 99 129 |  | 95 |  |  | .. (702) 13) |  | 23314) |  | .. |  | 101 555 |  |
| - See section 4.2. - The year 1999 number of trips (SIKA 2000). - Double accounting occurs with other modes. - Domestic. - Domestic and international traffic along the Swedish coast. - In the case of road the private roads, 284 000 km mostly forest roads, which accounts for 4 % of transport volumes are excluded. Regarding rail the museal tracks are excluded. See Statistical annex section 1. - Regarding rail, only state owned railways. Regarding road, national and country roads. Regarding air, airfields. Regarding maritime shipping, due to SFV annual report. |  |  |  |  |  |  | - Business account; see section 4.3. - Adjusted for underreporting with factor 2.4 . 18% of the number of injuries is severe injuries. - Only severe injury. (PT) In parenthesis has bus passengers as victims added. Adjusted for underreporting 2.4. - Including 33 fatalities in pleasure boat use. - Includes both emissions from diesel and electrical trains. - Emission refers to emissions from bus traffic - Emissions only at airports. |  |  |  |  |  |  |  |  |

## Road transport

The total cost for infrastructure provider, environmental cost and accident cost not born by the user is 4 094 Million € or 1.9% of the Swedish GDP in 1998. Approximately half of the cost is infrastructure cost (53%), 23% accident cost not born by the user, and 24% environmental cost.

Figure 5-6: Swedish road account, Core information 1996, 1998 and 2005 (M€).

Infrastructure cost

Of Swedish roads 98 000 km are state roads, 38 500 km municipal roads and 284 000 km private roads. However, the last category is mainly small roads. We have collected information for state road investments 40 year back and made an estimate with the PIM model. The result is compared with the business account of the road administration. The asset value based on the business account for the state roads are 8 128 Million € and with the PIM model 6 745 Million €. The number presented here is based on the business account, as that information is available for all modes. For municipal roads, we have neither access to business account nor any long time series for investment costs. A rough estimate of 3 170 Million € has been made for the asset value.  No private roads are included.

Accidents

Looking at the mode ‘road’ we see that users of motorized vehicles (road vehicles) as victim generate a cost of 2 502 M€. The majority of the cost is created in collisions between two or more vehicle users (1 279 M€) with non-collisions as the single largest part (1 038 M€). But they are also involved in accidents, which generate costs for other modes/users. The total cost of accidents where road users are involved is 3 455 M€, which includes the cost for unprotected users. A part of this total cost is born by users, notable the risk value and a minor part of the hospital care and administrative costs. Of the total accident cost for road vehicles, users bear 69% themselves. The total accident cost is 3 455 Million € or 1.6% of the GDP.

Environment

The total environmental cost is 982 M€ of which 42% air pollution, 44% green house gases and 15% noise.

Revenues

The revenues from road users to the state was 5 207 Million €. Only 1% can possibly be regarded as allocated direct to the sector, the Eurovignette. The majority of the revenues are fuel tax (68%), VAT on taxes (17%) and vehicle tax (13%).

Additional information

In addition we have estimated 6 564 Million € in supplier operating costs for the road transport sector (excluding public transport and transport on own account). Adding this cost to the infrastructure provider, environment cost and total accident cost the cost in the road sector is approximately 6.2% of GDP. No delay costs could be estimated.

Table 5-82:Swedish road account for 1996, 1998 and 2005 - € million at 1998 prices –

| Costs |  |  | 1996 |  |  | 1998 |  | 2005 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Core information |  |  |  |  |  |  |  |  |
| Infrastructure Costs |  |  | 1 918 |  |  | 2 172 |  | 1 9321) |
| Fixed |  |  | 666 |  |  | 726 |  | 884 |
| Variable |  |  | 1 252 |  |  | 1 446 |  | 1 048 |
| Accident costs (external)2) |  |  | 935 |  |  | 953 |  | 638 |
| Environmental costs |  |  | 954 |  |  | 982 |  | 917 |
| Air pollution |  |  | 445 |  |  | 456 |  | 281 |
| Global warming |  |  | 366 |  |  | 383 |  | 493 |
| Noise |  |  | 1433) |  |  | 143 |  | 1433) |
| Total I |  |  | 3 807 |  |  | 4 108 |  | 3 487 |
| Additional information |  |  |  |  |  |  |  |  |
| Congestion costs |  |  | .. |  |  | .. |  | .. |
| Time costs |  |  | .. |  |  | .. |  | .. |
| Fuel costs |  |  | .. |  |  | .. |  | .. |
| Accident costs (internal)4) |  |  | 2 380 |  |  | 2 502 |  | 1 676 |
| From this: risk value |  |  | 2 213 |  |  | 2 327 |  | 1 559 |
| Environmental costs |  |  |  |  |  | .. |  |  |
| Nature and landscape, soil and water pollution |  |  | .. |  |  | .. |  | .. |
| Nuclear risk |  |  | .. |  |  | .. |  | .. |
| Revenues |  |  |  |  |  |  |  |  |
| Directly related to a specific cost category |  |  |  |  |  |  |  |  |
| Charges for infrastructure usage |  |  |  |  |  |  |  |  |
| Fixed |  |  | 0 |  |  | - |  | - |
| Variable |  |  | 0 |  |  | 59 |  | 593) |
| Other transport specific revenues |  |  |  |  |  |  |  |  |
| Annual circulation tax |  |  | 629 |  |  | 684.0 |  | 6843) |
| Fuel tax |  |  | 3459 |  |  | 3 547 |  | 3 693 |
| Car import tax |  |  | 161 |  |  | 30 |  | 303) |
| VAT5) |  |  | 865 |  |  | 887 |  | 924 |
| Total |  |  | 5 114 |  |  | 5 207 |  | 5 390 |
| Subsidies |  |  |  |  |  |  |  |  |
| Appropriation from state to VV |  |  | 749.7 |  |  | 914.9 |  | .. |
| Tax relief on diesel |  |  | 643.5 |  |  | 747.8 |  | .. |
| - Includes both municipal and state roads but with assumption of constant cost for municipal roads. - We here consider road as motorized vehicles. External cost is cost born by the rest of society, mainly medical cost, and net lost production and cost by other users, mainly the risk value of unprotected road users. - With assumption of zero change from 1998 |  |  |  | - Transport system internal cost is cost born by users of motorized vehicles and consists mainly of the ‘risk value’. - Revenues from VAT refer on VAT raised on fuel tax, in Sweden there is no VAT on car import tax and annual vehicle tax |  |  |  |  |

Trend

The costs (including users accident cost but excluding supplier operating costs) have increased by 5% between 1996 and 1998 but are forecasted to decline by 25% until 2005. This positive trend is an effect of a forecasted very positive trend in traffic safety. The total cost information is presented in Table 5-82 for 1996, 1998 and 2005.

Cost per vehicle kilometre

The total cost for 1998 is presented by vehicle category in Table 5-85 and by vehicle category and road category in Table 5-86 and Table 5-87. However, following the UNITE structure we first presents the result per vehicle kilometre, Table 5-83 and Table 5-84. Comments to be found in the last three tables are also applicable in the first two tables.

In Sweden, we have not any officially accepted cost allocation method to distribute joint costs between different vehicle categories. The infrastructure cost, seen as a joint cost for all road traffic, is therefore only presented as an average for all vehicle kilometres (vkm). For accident cost, we have made detailed surveys in databases and for environmental cost we relay on the common UNITE methodology.

The following table only includes variable costs and revenues.

Table 5-83: Average (variable) costs of road transport per vehicle km: Sweden
- €/km at 1998 prices –

| All Swedish roads |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1998 |  |  |  |  |  |  |  |  |  |
|  | Motor-cycles |  | Passenger cars |  | Buses |  | LGV |  | HGV3) | Total |
| Core information |  |  |  |  |  |  |  |  |  |  |
| Infrastructure costs |  |  |  |  |  |  |  |  |  |  |
| Variable |  |  |  |  |  |  |  |  |  | 0.21 |
| External accident costs | 0.096 |  | 0.016 |  | 0.060 |  | 0.022 |  | 0.066 | 0.014 |
| Material costs | 0.062 |  | 0.007 |  | 0.014 |  | 0.005 |  | 0.009 | 0.007 |
| Risk value of non-motorized vehicle or ‘other’ vehicle category1) | 0.033 |  | 0.009 |  | 0.047 |  | 0.017 |  | 0.057 | 0.007 |
| Environmental costs | 0.011 |  | 0.009 |  | 0.047 |  | 0.021 |  | 0.069 | 0.015 |
| Air pollution | 0.005 |  | 0.004 |  | 0.024 |  | 0.008 |  | 0.037 | 0.007 |
| Global warming | 0.002 |  | 0.004 |  | 0.016 |  | 0.006 |  | 0.023 | 0.006 |
| Noise | 0.004 |  | 0.001 |  | 0.007 |  | 0.007 |  | 0.009 | 0.002 |
| Total I | 0.107 |  | 0.026 |  | 0.108 |  | 0.043 |  | 0.135 | 0.050 |
| Additional information |  |  |  |  |  |  |  |  |  |  |
| Delay costs | .. |  | .. |  | .. |  | .. |  | .. | .. |
| Internal accident costs | 0.348 |  | 0.035 |  | 0.034 |  | 0.015 |  | 0.009 | 0.037 |
| Material damages | 0.024 |  | 0.002 |  | 0.002 |  | 0.001 |  | 0.001 | 0.003 |
| Risk value | 0.324 |  | 0.032 |  | 0.031 |  | 0.014 |  | 0.009 | 0.034 |
| Environmental costs | .. |  | .. |  | .. |  | .. |  | .. | .. |
| Nature, landscape, soil and water pollution | .. |  | .. |  | .. |  | .. |  | .. | .. |
| Total II | 0.348 |  | 0.035 |  | 0.034 |  | 0.015 |  | 0.009 | 0.037 |
| Revenues/ variable |  |  |  |  |  |  |  |  |  |  |
| Fuel tax | 0.028 |  | 0.049 |  | 0.088 |  | 0.031 |  | 0.127 | 0.052 |
| Eco tax | . |  | . |  | . |  | . |  | . | . |
| Distance related infrastructure charges | 0.000 |  | 0.000 |  | 0.000 |  | 0.000 |  | 0.000 | 0.000 |
| VAT | 0.007 |  | 0.012 |  | 0.022 |  | 0.008 |  | 0.032 | 0.013 |
| Total | 0.036 |  | 0.061 |  | 0.111 |  | 0.039 |  | 0.159 | 0.066 |

NOTE: see Tables below for comments

In the two tables that follow, we present the cost for urban and non-urban roads. We have not been able to disaggregate the information into motorways and other road sections (except for environmental costs). Sweden does not have any specific motorway network and the need for a motorway section specific cost estimate is not clear as we expect the average cost is much higher in urban areas than in non-urban areas.

Table 5-84: Average (variable) costs of road transport per vehicle km: Non-urban and urban roads Sweden - €/km at 1998 prices –

| Non-urban roads including motorways |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1998 |  |  |  |  |  |
|  | Motor-cycles | Passenger cars | Buses | LGV | HGV3) | Total |
| Core information |  |  |  |  |  |  |
| Infrastructure costs / variable |  |  |  |  |  | 0.023 |
| External accident costs | 0.064 | 0.010 | 0.043 | 0.017 | 0.056 | 0.008 |
| Environmental costs | 0.006 | 0.007 | 0.028 | 0.011 | 0.056 | 0.011 |
| Total I | 0.070 | 0.017 | 0.072 | 0.027 | 0.112 | 0.043 |
| Additional information |  |  |  |  |  |  |
| Delay costs | .. | .. | .. | .. | .. | .. |
| Internal accident costs | 0.295 | 0.039 | 0.036 | 0.017 | 0.010 | 0.038 |
| Environmental costs | .. | .. | .. | .. | .. | .. |
| Nature, landscape, soil and water pollution | .. | .. | .. | .. | .. | .. |
| Total II | 0.295 | 0.039 | 0.036 | 0.017 | 0.010 | 0.038 |
| Revenues / variable |  |  |  |  |  |  |
| Fuel tax1) | 0.030 | 0.041 | 0.066 | 0.027 | 0.129 | 0.047 |
| Eco tax | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Distance related infrastructure charges | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| VAT1) | 0.007 | 0.010 | 0.017 | 0.007 | 0.032 | 0.012 |
| Total | 0.037 | 0.051 | 0.083 | 0.033 | 0.161 | 0.058 |

Continue on next page.

| Urban roads |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1998 |  |  |  |  |  |
|  | Motor-cycles | Passenger cars | Buses | LGV | HGV3) | Total |
| Core information |  |  |  |  |  |  |
| Infrastructure costs/variable |  |  |  |  |  | 0.018 |
| External accident costs | 0.146 | 0.027 | 0.083 | 0.032 | 0.109 | 0.025 |
| Environmental costs | 0.019 | 0.014 | 0.073 | 0.040 | 0.124 | 0.021 |
| Total I | 0.166 | 0.042 | 0.156 | 0.071 | 0.232 | 0.063 |
| Additional information |  |  |  |  |  |  |
| Delay costs | .. | .. | .. | .. | .. | .. |
| Internal accident costs | 0.431 | 0.027 | 0.031 | 0.012 | 0.006 | 0.036 |
| Environmental costs | .. | .. | .. | .. | .. | .. |
| Total II | 0.431 | 0.027 | 0.031 | 0.012 | 0.006 | 0.036 |
| Revenues / variable |  |  |  |  |  |  |
| Fuel tax1) | 0.026 | 0.063 | 0.118 | 0.038 | 0.119 | 0.064 |
| Distance related infrastructure charges | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| VAT1) | 0.007 | 0.016 | 0.030 | 0.010 | 0.030 | 0.016 |
| Total | 0.033 | 0.079 | 0.148 | 0.048 | 0.149 | 0.079 |

The costs per vehicle kilometre are based on the following three final road cost tables, where the total cost is disaggregated into vehicle categories for all Swedish roads, for non-urban roads and for urban roads.

Table 5-85:Total costs of road transport: Sweden- € million at 1998 prices –

| All roads |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1998 |  |  |  |  |  |
|  | Motor-cycles | Passenger cars | Buses | LGV | HGV3) | Total |
| Core information |  |  |  |  |  |  |
| Infrastructure costs1) | .. | .. | .. | .. | .. | 2 172 |
| Fixed | .. | .. | .. | .. | .. | 726 |
| Variable | .. | .. | .. | .. | .. | 1446 |
| External accident costs | 72 | 930 | 68 | 109 | 268 | 953 |
| Material costs | 47 | 399 | 15 | 27 | 37 | 457 |
| Risk value of non-motorized vehicle or ‘other’ vehicle category2) | 25 | 531 | 53 | 83 | 231 | 496 |
| Environmental costs | 8 | 538 | 53 | 105 | 278 | 982 |
| Air pollution | 3 | 239 | 27 | 38 | 148 | 456 |
| Global warming | 2 | 240 | 18 | 30 | 93 | 383 |
| Noise3) | 3 | 58 | 8 | 37 | 37 | 143 |
| Total I | 80 | 1468 | 121 | 214 | 546 | 4 108 |
| Additional information | .. | .. | .. | .. | .. | .. |
| Delay costs | .. | .. | .. | .. | .. | .. |
| Internal accident costs | 261 | 1959 | 38 | 76 | 38 | 2502 |
| Material damages | 18 | 137 | 3 | 5 | 3 | 175 |
| Risk value4) | 243 | 1822 | 36 | 71 | 35 | 2327 |
| Environmental costs | .. | .. | .. | .. | .. | .. |
| Nature, landscape, soil and water pollution | .. | .. | .. | .. | .. | .. |
| Total II | 261 | 1959 | 38 | 76 | 38 | 2502 |
| Revenues |  |  |  |  |  |  |
| Fixed5) | .. | .. | .. | .. | 59 | 773 |
| Vignette | 0 | 0 | 0 | 0 | 59 | 59 |
| Annual circulation tax and car import tax5) | 0 | 0 | 0 | 0 | 0 | 714 |
| Variable | 22 | 2 758 | 100 | 153 | 514 | 3547 |
| Fuel tax6) | 22 | 2 758 | 100 | 153 | 514 | 3547 |
| Eco tax7) | 0 | 0 | 0 | 0 | 0 | 0 |
| Distance related infrastructure charges | 0 | 0 | 0 | 0 | 0 | 0 |
| VAT8) | 5 | 639 | 25 | 38 | 128 | 887 |
| Total | 28 | 3 448 | 125 | 191 | 701 | 5207 |
| Basic data |  |  |  |  |  |  |
| Number of vehicles thousand9) | 101/137 | 3 792 | 15 | 338 |  | 4 246/4 282 |
| Million vehicle km | 750 | 56 680 | 1 130 | 4 960 | 4050 | 67 570.0 |
| Million passenger km | 359 | 80 483 | 9 005 | . | . | 89 847 |
| Million tonne km | . | . | . | 32 674 |  | 32 674 |
| - We do not allocate joint costs as Sweden does not have any official cost allocation method for infrastructure cost. - When motorized vehicles are disaggregated into HGVs etc. the definition of external includes the risk value of other categories, including other motorized vehicles such as cars, see section 4.6. - The noise cost is allocated to vehicle type by the common UNITE methodology; vehicle kilometre in urban areas by category is weighted with the following; MC=5, cars=1, Buses=6, LGV=7, HGV=14. - Internal within the category in question, see section 4.6. - We have not allocated annual circulation tax (684 M€) or car import tax (30 M€). - Allocation due to own calculations where the number of LGV due to SIKA consists of 66 % petrol and 34% diesel vehicles. The total diesel consumption of 456 576 tonnes in the statistical annex is devided into 155 236 tonnes of diesel and 301 340 tonnes of petrol. The figures should be regarded as approximations. - The Swedish fuel tax consists of energy and CO2 tax. - The allocation of VAT on fuel tax is due to the figures of fuel tax. - Different dates,12.31 and 06.30 respectively |  |  |  |  |  |  |

Table 5-86: Total costs of non-urban road transport: Sweden - € million at 1998 prices –

| State owned roads/ non-urban roads incl motorways |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1998 |  |  |  |  |  |
|  | Motor-cycles | Passenger cars | Buses | LGV | HGV | Total |
| Core information |  |  |  |  |  |  |
| Infrastructure costs1) | .. | .. | .. | .. | .. | 1552 |
| Fixed | .. | .. | .. | .. | .. | 526 |
| Variable | .. | .. | .. | .. | .. | 1026 |
| External accident costs | 30 | 369 | 28 | 53 | 185 | 367 |
| Material costs | 20 | 227 | 7 | 14 | 26 | 256 |
| Risk value of non-motorized vehicle or ‘other’ vehicle category2) | 10 | 142 | 21 | 38 | 160 | 111 |
| Environmental costs | 3 | 242 | 18 | 34 | 184 | 481 |
| Air pollution | 2 | 113 | 11 | 17 | 107 | 250 |
| Global warming | 1 | 129 | 8 | 17 | 77 | 231 |
| Noise3) | 0 | 0 | 0 | 0 | 0 | 0 |
| Total I | 32 | 611 | 47 | 86 | 369 | 2387 |
| Additional information |  |  |  |  |  |  |
| Delay costs | .. | .. | .. | .. | .. | .. |
| Internal accident costs | 136 | 1399 | 23 | 55 | 34 | 1647 |
| Material damages | 10 | 98 | 2 | 4 | 2 | 115 |
| Risk value4) | 126 | 1301 | 22 | 51 | 31 | 1531 |
| Environmental costs | 0 | 0 | 0 | 0 | 0 | 0 |
| Nature, landscape, soil and water pollution | 0 | 0 | 0 | 0 | 0 | 0 |
| Total II | 136 | 1399 | 23 | 55 | 34 | 1647 |
| Revenues |  |  |  |  |  |  |
| Fixed5)A) |  |  |  |  |  |  |
| Vignette |  |  |  |  | 59 | 59 |
| Annual circulation tax and car import tax5) |  |  |  |  |  |  |
| Variable |  |  |  |  |  |  |
| Fuel tax6)A) | 14 | 1472 | 43 | 85 | 424 | 2038 |
| Eco tax7) |  |  |  |  |  |  |
| Distance related infrastructure charges |  |  |  |  |  |  |
| VAT8)A) | 3 | 368 | 11 | 21 | 106 | 510 |
| Total | 17 | 1840 | 54 | 106 | 589 | 2607 |
| Basic data |  |  |  |  |  |  |
| Million vehicle km | 460 | 36 250 | 650 | 3 170 | 3 290 | 43 820 |
| Notes: for notes with numbers see Table 5-85. For notes with letters, see below: |  |  |  |  |  |  |
| A) Allocation due to fuel consumption at state owned roads and municipal roads respectively. |  |  |  |  |  |  |

Table 5-87: Total costs of urban road transport: Sweden - € million at 1998 prices -

| Urban roads A) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1998 |  |  |  |  |  |
|  | Motor-cycles | Passenger cars | Buses | LGV | HGV | Total |
| Core information |  |  |  |  |  |  |
| Infrastructure costsB) | .. | .. | .. | .. | .. | 620 |
| Fixed | .. | .. | .. | .. | .. | 200 |
| Variable | .. | .. | .. | .. | .. | 420 |
| External accident costs | 42 | 561 | 40 | 57 | 83 | 586 |
| Material costs | 27 | 172 | 8 | 12 | 11 | 200 |
| Risk value of non-motorized vehicle or ‘other’ vehicle category2) | 16 | 389 | 32 | 44 | 71 | 386 |
| Environmental costs | 6 | 296 | 35 | 71 | 94 | 502 |
| Air pollution | 2 | 126 | 17 | 21 | 41 | 207 |
| Global warming | 1 | 112 | 10 | 13 | 16 | 152 |
| Noise3) | 3 | 58 | 8 | 37 | 37 | 143 |
| Total I | 48 | 857 | 75 | 128 | 177 | 1707 |
| Additional information |  |  |  |  |  |  |
| Delay costs | .. | .. | .. | .. | .. | .. |
| Internal accident costs | 125 | 560 | 15 | 22 | 5 | 856 |
| Material damages | 9 | 39 | 1 | 2 | 0 | 60 |
| Risk value4) | 116 | 521 | 14 | 20 | 4 | 796 |
| Environmental costs |  |  |  |  |  |  |
| Nature, landscape, soil and water pollution |  |  |  |  |  |  |
| Total II | 125 | 560 | 15 | 22 | 5 | 856 |
| Revenues |  |  |  |  |  |  |
| Fixed5)C) |  |  |  |  |  |  |
| Vignette |  |  |  |  |  |  |
| Annual circulation tax and car import tax5) |  |  |  |  |  |  |
| Variable |  |  |  |  |  |  |
| Fuel tax6)C) | 8 | 1286 | 57 | 68 | 90 | 1509 |
| Eco tax7) |  |  |  |  |  |  |
| Distance related infrastructure charges |  |  |  |  |  |  |
| VAT8)C) | 2 | 322 | 14 | 17 | 23 | 377 |
| Total | 10 | 1608 | 71 | 85 | 113 | 1886 |
| Basic data |  |  |  |  |  |  |
| Million vehicle km | 290 | 20430 | 480 | 1790 | 760 | 23750 |
| Notes: for notes with numbers see Table 5-85. For notes with letters, see below: |  |  |  |  |  |  |
| A) Urban roads and municipal roads are not the same. Some state owned roads goes through urban areas. B) Rough estimates, see section 4.3.1. C) Allocation due to fuel consumption at state owned roads and municipal roads respectively. |  |  |  |  |  |  |

## Rail transport

The total cost for infrastructure provider, environmental cost, accident cost not born by the user and delays is 1 003 Million € or 0.5% of the Swedish GDP in 1998. The largest part of the cost is infrastructure cost (85%), with only 3% accident cost not born by the user, and 5% environmental cost and 6% roughly estimated delay cost.

Figure 5-7: Swedish rail account, core information 1996, 1998 and 2005 (M€)

Infrastructure

The Swedish railway system consists of 11 600 km state owned tracks and 3 852 million private tracks. We have not collected any information for the private tracks. The asset value based on the business account is 6 955 Million € and with the PIM model 6 752 Million €.

Environmental

The total environmental cost is 51 M€. The noise cost is by far the largest environmental cost (84%). The cost for air pollution (10%) and global warming (6%) includes the cost of electricity production. However, 70% of the cost for air pollution and green house gases origins from diesel trains.

Accidents

It is obvious that the highest risk with rail transport is accidents with other user categories, road users at level crossings and pedestrians. The rail internal cost is only 15% of total accident costs compared to an average of 70% for all modes. The total accident cost for railways is estimated to 38 M€ or 0.02% of GDP. In addition, since 1998 the number of suicides is reported separately, and if these are valued as other fatalities the total accident cost will increase with 61 M€ to almost 100 M€. In the following, we will ignore the cost of suicides. However, when accident costs are compared between modes it should be remembered that suicides are included in other modes. The main part of the total accident cost, including road users cost at level crossings, is the cost of accidents with non-rail users outside level crossings (50%), at level crossings (26%) and with electricity (10%). The passenger-accidents cost 4% and the employee-accidents 7%. The remaining 8% is material cost for the railway companies.

Delays

A rough estimate on delay cost shows a cost of 63 Million € of which 37 Million € on freight trains.

Supplier operation

Supplier have costs of 1 269 Million €, of which 98 Million €, is charges for the use of infrastructure. The total cost of the rail sector, including internal accident cost, is thus 2 278 Million €, or 1.1 % of GDP.

Table 5-88: Swedish rail account 1996, 1998 and 2005 
- € million at 1998 prices –

| Costs | 1996 |  | 1998 |  | 2005 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Core information |  |  |  |  |  |  |
| Infrastructure Costs | 764.6 |  | 856.3 |  | 943.9 |  |
| Fixed | 357.7 |  | 458.4 |  | 546.0 |  |
| Variable | 406.9 |  | 397.9 |  | 397.9 |  |
| Supplier operating costs1) | 45.2 |  | 1 269.6 |  | 1 269.6 |  |
| Accident costs (external) | 22.7 |  | 32.3 |  | 34.9 |  |
| Environmental costs | 50.3 |  | 51 |  | 51.4 |  |
| Air pollution | 4.8 |  | 4.9 |  | 6.1 |  |
| Global warming | 2.6 |  | 3.2 |  | 2.4 |  |
| Noise | 42.9 |  | 42.9 |  | 42.9 |  |
| Total I | 882.8 |  | 2 209.2 |  | 2 299.8 |  |
| Additional information |  |  |  |  |  |  |
| Congestion costs | .. |  | 63 |  | .. |  |
| Accident costs (internal) | 3.4 |  | 5.7 |  | 6.1 |  |
| From this: risk value | 1.3 |  | 3.8 |  | 4.1 |  |
| Environmental costs | .. |  | .. |  | .. |  |
| Nature and landscape, soil and water pollution | .. |  | .. |  | .. |  |
| Nuclear risk | .. |  | .. |  | .. |  |
| Revenues |  |  |  |  |  |  |
| Directly related to Supplier Operating Costs |  |  |  |  |  |  |
| Subsidies for concessionary fares2) | . |  | . |  | . |  |
| User Tariffs 3) | 51.1 |  | 1 324.9 |  | .. |  |
| Revenues directly related to infrastructure costs |  |  |  |  |  |  |
| Track charges4) | .. |  | 98.2 |  | .. |  |
| Fixed | .. |  | - |  | .. |  |
| Variable | .. |  | 98.2 |  | .. |  |
| Station charges4) |  |  |  |  |  |  |
| Other transport specific revenues |  |  |  |  |  |  |
| Fuel tax | - |  | - |  | - |  |
| Eco tax | . |  | . |  | . |  |
| VAT |  | - |  | - |  | - |
| Subsidies |  |  |  |  |  |  |
| Tax relief5) |  | 109.2 |  | 127.8 |  | .. |
| Subsidy from Eu to BV |  | 4.2 |  | 8.1 |  | .. |
| Appropriation from state to BV |  | 314.3 |  | 364.4 |  | .. |
| Non-transport related revenues of rail companies |  | .. |  | 5.5 |  | .. |
| - Excluded from these costs and revenues are infrastructure charges. For 1996 the Swedish state owned rail company SJ is not included in the statistics . For 2005 we assume zero change. - Subsidies for concessionary fares are found under public transport. - Net turnover excluding excise duties.For 1996 SJ is excluded. - No distinction is made between track charges and station charges. In the Swedish accounts the conception is infrastructure charges. - For track operation, electricity and fuel use, from state to BV. |  |  |  |  |  |  |

Revenues

The revenue from rail companies to the state was 98 Million €. This is the rail track charge. Users pays 1 325 Million € in tariffs to the supplier.

Trend

The costs (including users accident cost but excluding noise and supplier operating costs) have increased with 13% between 1996 and 1998 and are forecasted to increase with additional 11% until 2005.

Cost per kilometre

As for the road sector, no official cost allocation method exists in Sweden. Costs that we consider joint cost for all rail transport are not arbitrarily allocated to any category of trains. Unfortunately, the official statistics we used for accidents do not allow us to distinguish between passenger and freight train. This information exists of course but requires that we rely on other sources. The environmental costs are  based on the UNITE methodology and estimated with ‘models’ which makes it possible to describe the result rather detailed by train type (see further section 4 above).

Table 5-89: Average (variable) costs of rail transport per vehicle km: Sweden
€/train km at 1998 prices

|  | 1998 |  |
| --- | --- | --- |
|  | Passenger | Freight |
| Core information |  |  |
| Infrastructure costs1) |  |  |
| Variable | 3.6 |  |
| External accident costs | 0.3 |  |
| Material cost | 0.0 |  |
| Risk value | 0.3 |  |
| Environmental costs | 0.33 | 0.80 |
| Air pollution | 0.02 | 0.08 |
| Global warming | 0.02 | 0.14 |
| Noise 2) | 0.29 | 0.58 |
| Total I | 4.40 |  |
| Additional information |  |  |
| Delay costs | 0.38 | 0.96 |
| Internal accident costs | 0.05 |  |
| Material damages | 0.02 |  |
| Risk value | 0.03 |  |
| Environmental costs | .. | .. |
| Nature, landscape, soil and water pollution | .. | .. |
| Nuclear risk | .. | .. |
| Total II | 0.63 |  |
| Revenues |  |  |
| User tariffs | 12.09 |  |
| Track charges | 0.90 |  |
| Station charges | . |  |
| Fuel tax | . |  |
| Eco tax | . |  |
| VAT | . |  |
| Subsidies |  |  |
| Basic data |  |  |
| Train kilometre (million) | 71.3 | 38.4 |
|  | 109.7 |  |
| Passenger km (million) | 7 144 | - |
| Tonne km (million) | - | 19 086 |
| - Separation into freight and passenger not possible in Sweden - The noise cost is allocate to vehicle type by the common UNITE methodology; vehicle kilometre in urban areas by category is weighted with the following; Passenger train 1; Freight train 2. |  |  |

Table 5-90 below shows the total costs of rail transport for passenger and freight transport.

Table 5-90: Total costs of rail transport: - € million at 1998 prices -

| All Swedish railways |  |  |  |
| --- | --- | --- | --- |
|  |  | 1998 |  |
|  | Passenger | Freight | Total |
| Core information |  |  |  |
| Infrastructure costs1) | . | . | 856.3 |
| Tracks | . | . |  |
| Fixed | . | . | 458.4 |
| Variable | . | . | 397.9 |
| Stations | . | . |  |
| Fixed | . | . |  |
| Variable | . | . |  |
| Supplier operating cost2) | .. | .. | 1 269.6 |
| Out of these: track + station charges | .. | .. | 98.2 |
| External accident costs | .. | .. | 32.3 |
| Material costs | .. | .. | 2.8 |
| Risk value | .. | .. | 29.4 |
| Environmental costs | 20.5 | 33.7 | 51.0 |
| Air pollution | 1.7 | 3.2 | 4.9 |
| Global warming | 1.1 | 5.3 | 3.2 |
| Noise | 17.7 | 25.2 | 42.9 |
| Total I | . | . | 2 209.2 |
| Additional information |  |  |  |
| Delay costs | 27 | 37 | 63 |
| Internal accident costs | .. | .. | 5.7 |
| Material damages | .. | .. | 1.9 |
| Risk value | .. | .. | 3.8 |
| Environmental costs | .. | .. | .. |
| Nature, landscape, soil and water pollution | .. | .. | .. |
| Nuclear risk |  |  | .. |
| Total II | . | . | 68.7 |
| Revenues |  |  |  |
| User tariffs |  |  | 1 324.9 |
| Track charges |  |  | 98.2 |
| Station charges |  |  | 0 |
| Fuel tax |  |  | 0 |
| Eco tax |  |  |  |
| VAT |  |  | 0 |
| Total |  |  | 1 423.1 |
| Subsidies |  |  |  |
| Basic data |  |  |  |
| Train kilometre (mill) | 71.3 | 38.4 | 109.7 |
| Passenger km (mill) | 7.144 | - | 7.144 |
| Tonne km (mill) | - | 19.086 | 19.086 |
| - Due to BV annual report, separation is not possible. - Separation not possible. |  |  |  |

## Public transport

Public transport is a ‘mode’ of a different kind. The core of public transport is services run on road or rail infrastructure. Therefore, no infrastructure cost is estimated for this mode. The majority of the cost is cost for running the service, the supplier operating cost is 1 721 Million € (0.81% of GDP). The remaining (estimated) core costs are only 130 M€ and consists of more or less equal share accidents and environmental costs, mainly for buses.

Table 5-91: Swedish account for public transport 1996, 1998 and 2005 
- € million at 1998 prices –

| Costs | Public transport incl. buses |  |  |  |
| --- | --- | --- | --- | --- |
| Core information | 1996 | 1998 | 2005 |  |
| Infrastructure Costs 1) | .. | .. | .. |  |
| Fixed | .. | .. | .. |  |
| Variable | .. | .. | .. |  |
| Services |  |  |  |  |
| Supplier operating costs 2) | 1 559.2 | 1 720.9 | 1 720.9 |  |
| Accident costs (external) | 99.3 | 76.8 | 54.4 |  |
| Environmental costs | 54.9 | 53.5 | 51.9 |  |
| Air pollution | 27.2 | 27.2 | 21.2 |  |
| Global warming ) | 19.4 | 18.1 | 22.4 |  |
| Noise | 8.3 | 8.3 | 8.3 |  |
| Total core social cost | 1 713.4 | 1 851.2 | 1827.2 |  |
| Additional information |  |  |  |  |
| Congestion costs | .. | .. | .. |  |
| Accident costs (internal) | 36.2 | 48.7 | 36.1 |  |
| From this: risk value | 34.2 | 46.0 | 34.3 |  |
| Environmental costs | .. | .. | .. |  |
| Nature and landscape, soil and water pollution | .. | .. | .. |  |
| Nuclear risk | .. | .. | .. |  |
| Revenues |  |  |  |  |
| Directly related to a specific cost category |  |  |  |  |
| Charges for infrastructure usage |  |  |  |  |
| Fixed | .. | .. | .. |  |
| Variable | .. | .. | .. |  |
| Subsidies for concessionary fares 3) | 780.3 | 752.5 | .. |  |
| User Tariffs 4) | 712.4 | 729.9 | .. |  |
| Other transport specific revenues |  |  |  |  |
| Fuel tax | .. | .. | .. |  |
| Eco tax | . | . | . |  |
| VAT | .. | .. | .. |  |
| Subsidies |  |  |  |  |
| - No calculation of infrastructure cost is made. Double accounting occurs with road and rail. - Due to the categorisation in the structural business accounts (SCB). Cost and depreciation. - Subsidies total see section 4.8.3 - Subsidies and VAT are excluded. |  |  |  |  |

Only for accidents can we find costs unique for public transport. These are the  accident cost for trams and for metro. The costs low compared to other transport modes. In the table above also the cost for bus accidents are included. This cost is a part of the accident cost reported in the road mode. In addition, the environmental cost for buses has been duplicated in this table. Compared to the supplier operating cost also these costs are small.

Table 5-92: Total costs of public transport Sweden - € million at 1998 prices –

|  | 19981) |  |  |
| --- | --- | --- | --- |
|  | Bus | Other | Total |
| Core information |  |  |  |
| Infrastructure costs | . | .. | . |
| Fixed | . | .. | . |
| Variable | . | .. | . |
| Supplier operating costs | .. | .. | 1 720.9 |
| External accident costs | 68.0 | 8.8 | 76.8 |
| Material Cost | 15.3 | 2.8 | 18.1 |
| Risk Value | 52.7 | 6.0 | 58.7 |
| Environmental costs | 53.5 | .. | 53.5 |
| Air pollution | 27.2 | .. | 27.2 |
| Global warming | 18.1 | .. | 18.1 |
| Noise | 8.3 | .. | 8.3 |
| Total I | 121.5 | 8.8 | 1 851.2 |
| Additional information |  |  |  |
| Delay costs | .. | .. | .. |
| Internal accident costs | 38.2 | 10.5 | 48.7 |
| Material damages | 2.7 | 0.0 | 2.7 |
| Risk value | 35.5 | 10.5 | 46.0 |
| Environmental costs | .. | .. | .. |
| Nature, landscape, soil and water pollution | .. | .. | .. |
| Nuclear risk | .. | .. | .. |
| Total II | 38.2 | 10.5 | 48.7 |
| Revenues |  |  |  |
| User tariffs |  |  | 729.9 |
| Subsidies |  |  | 752.52) |
| Basic data |  |  |  |
| Passengers carried | 593 | 4843) | 1 077 |
| Passenger km Million | 9 005 | 3 3684) | 12 373 |
| 1) In the Swedish accounts the categorisation is public transport divided into buses and other due to data availability 2) Subsidies from state, county council and local authorities to public transport companies plus tax relief due to different VAT rate on public transport. 3) Underground, tram, train, air. 4) Underground, tram, commuter train. Source: VTI. |  |  |  |

## Aviation

The total cost for infrastructure provider, environmental cost and accident cost not born by the user is 515 Million € or 0.2% of the Swedish GDP in 1998. The largest part of the cost is infrastructure cost (87%), and environmental cost (13%) with only 0.1% accident cost not born by the user.

Figure 5-8: Swedish aviation account, Core information 1996, 1998 and 2005 (M€)

Infrastructure cost

The state run 19 airports, municipal 26 airports and two were private in 1998. The state administered airports accounts for 95% of the passengers and 73% of the landings. For these airports, we have cost information. In addition, we have information for the 8 largest municipal airports. The asset value based on the business account is 845 Million € and with the PIM model 795 Million €.

Accidents

The total accident cost is 7.2 Million €. The majority of the cost (93%) is caused by private operations. The users bear an accident cost of 6.5 Million € and consequently, the external part is very small.

Environment

The majority of the environmental cost (67 M€) is global warming (96%).

Delay cost

We have also roughly estimated a delay cost of at least 20 Million € due to late arrivals.

Revenues

The revenue from companies to the infrastructure provider was 328 Million €. The revenues are 64% of the cost for infrastructure providers, environment, and accident cost born by non-users.

Supplier operator

In addition, users pay 2 496 Million € in tariffs to (Swedish) airline companies. Their costs were 2 481 Million € (not included in the table below). If we add the suppliers cost to the infrastructure, environmental and accident cost for non-users the aviation sector has a cost of 2 668 Million € or 1.2 % of GDP.

Trend

The environmental and accident cost is forecasted to increase. The accident cost will increase due to a strong increase in private operations, which are responsible for the majority of the accidents. The cost for these categories can increase with as much as 30% until 2005.

Table 5-93: Swedish air transport account for 1996, 1998 and 2005 - € million at 1998 prices –

| Costs |  |  |  |
| --- | --- | --- | --- |
|  | 1996 | 1998 | 2005 |
| Core information |  |  |  |
| Infrastructure Costs 1) | 438 | 447.4 | (447.4) |
| Fixed | 87.5 | 94.7 | .. |
| Variable | 350.5 | 352.7 | .. |
| Accident costs (external) 2) | 1.6 | 0.6 | 1.5 |
| Environmental costs | 58.9 | 67.1 | 73.7 |
| Air pollution | 2.1 | 2.1 | 3.1 |
| Global warming | 56.4 | 64.6 | 70.2 |
| Noise | 0.4 | 0.4 | 0.4 |
| Total | 499 | 515 | 523 |
| Additional information |  |  |  |
| Congestion costs | .. | 20.5 | .. |
| Accident costs (internal) | 31.4 | 6.5 | 16.5 |
| From this: risk value | 5.7 | 5.4 | .. |
| Environmental costs | .. | .. | .. |
| Nature and landscape, soil and water pollution | .. | .. | .. |
| Nuclear risk | .. | .. | .. |
| Revenues |  |  |  |
| Directly related to a specific cost category |  |  |  |
| Charges for infrastructure usage 3) | 279.8 | 328.0 | .. |
| Airport revenues 4) | 155.3 | 183.6 | .. |
| ATM charges 5) | 20.1 | 20.4 | .. |
| En route 5) | 79.3 | 98.9 | .. |
| Capacity agreement and security 5) | 18.7 | 17.0 | .. |
| Total | 280 | 328 | .. |
| Loss of revenues due to tax exemptions |  |  |  |
| Kerosene tax |  |  |  |
| VAT on ticket price |  |  |  |
| Other transport specific revenues |  |  |  |
| Fuel tax |  |  |  |
| Eco tax |  |  |  |
| VAT |  |  |  |
| Subsidies |  |  |  |
| Non-transport related revenues of airports 6) |  | 211.1 |  |
| Charges from passenger to airline companies | 2 214 | 2 496.4 |  |
| - LFV and the most important municipal airports. - The cost is allocated to the mode in which the victim belongs. Of the total cost for aviation 5.6 M€ occur in private operations. The external cost is cost born by the rest of society, mainly medical cost, and net lost production. Transport system internal cost is cost born by road users and consists mainly of the ‘risk value’. - LFV and the most important municipal airports (which accounts for 6.5 and 8.0 for the years 1996 and 1998 respectively) see section 4.8.4. - LFV only, landing and passenger charges. - LFV only. - See section 4.4. |  |  |  |

Cost per start

In the following table, the cost is expressed per start. As we have not been able to allocate the cost between passenger and freight flights the figures presented below are averages. We also know that the majority of the accident cost is due to private operations.

Table 5-94: Average (variable) costs of Aviation per start: Sweden
- €/start at 1998 prices -

|  | 1998 |  |
| --- | --- | --- |
|  | Passenger | Cargo |
| Core information |  |  |
| Infrastructure costs 1) |  |  |
| Variable | 753 |  |
| External accident costs | 1.3 |  |
| Material cost | 1.3 |  |
| Risk value | 0 |  |
| Environmental costs | 143 |  |
| Air pollution | 4.5 |  |
| Global warming | 138 |  |
| Noise | 0.9 |  |
| Total I | 897 |  |
| Additional information |  |  |
| Delay costs (per arriving flight) | 44 |  |
| Internal accident costs | 13.9 |  |
| Material damages | 2.3 |  |
| Risk value | 11.5 |  |
| Environmental costs | .. | .. |
| Nature, landscape, soil and water pollution | .. | .. |
| Nuclear risk | .. | .. |
| Total II | 57.6 |  |
| Revenues |  |  |
| Charges for infrastructure usage 2) | 700 |  |
| Airport revenues 3) | 392 |  |
| ATM charges 3) | 44 |  |
| En route 3) | 211 |  |
| Capacity agreement and security 3) | 36 |  |
| Fuel tax | .. | .. |
| Eco tax | . | . |
| VAT | .. | .. |
| Subsidies |  |  |
| Exemption for kerosene tax |  |  |
| Exemption of VAT on ticket price |  |  |
| Basic data |  |  |
| Number of starts | 468 600 |  |
| Passenger km (bill) dom/tot | 4.9/14.2 | . |
| Tonne km (bill) | . | 0.347 |
| - LFV and the most important municipal airports. - LFV and the most important municipal airports. - LFV only |  |  |

As the standard format of the following table is slightly different from the Table 5-93 above, we nevertheless include that table. However, the information is the same as we have not been able to allocate the cost between freight and passengers.

Table 5-95: Total costs of Aviation: Sweden- € million at 1998 prices –

|  | 1998 1) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Passenger | Cargo |  | Total |
| Core information |  |  |  |  |
| Infrastructure costs 2) | 447.4 |  |  | 447.4 |
| Fixed | 94.7 |  |  | 94.7 |
| Variable | 352.7 |  |  | 352.7 |
| External accident costs | 0.6 |  |  | 0.6 |
| Material Cost | 0.6 |  |  | 0.6 |
| Risk value | 0 |  |  | 0 |
| Environmental costs | 67.1 |  |  | 67.1 |
| Air pollution | 2.1 |  |  | 2.1 |
| Global warming | 64.6 |  |  | 64.6 |
| Noise | 0.4 |  |  | 0.4 |
| Total I | 515.2 |  |  | 515.2 |
| Additional information |  |  |  |  |
| Delay costs | 20.5 |  |  | 20.5 |
| Internal accident costs | 6.5 |  |  | 6.5 |
| Material damages | .. |  |  | 1.1 |
| Risk value | .. |  |  | 5.4 |
| Environmental costs |  |  |  |  |
| Nature, landscape, soil and water pollution | .. |  | .. | .. |
| Nuclear risk | .. |  | .. | .. |
| Total II | 27.0 |  |  | 27.0 |
| Revenues 3) |  |  |  |  |
| Charges for infrastructure usage | 328.0 |  |  | 328.0 |
| Airport revenues | 183.6 |  |  | 183.6 |
| ATM charges | 20.4 |  |  | 20.4 |
| En route | 98.9 |  |  | 98.9 |
| Capacity agreement and security | 17.0 |  |  | 17.0 |
| Fuel tax |  |  |  |  |
| VAT on tax |  |  |  |  |
| Total | 328.0 |  |  | 328.0 |
| Subsidies |  |  |  |  |
| Exemption for kerosene tax |  |  |  |  |
| Exemption of VAT on ticket price |  |  |  |  |
| Total |  |  |  |  |
| Basic data |  |  |  |  |
| Number of starts | 468 600 |  |  | 468 600 |
| Passenger km (bill) dom/tot | 4.9/14.2 |  | - | 4.9/14.2 |
| Tonne km (bill) | - |  | 0.347 | 0.347 |
| - Separation of total into passenger and cargo was not possible. 2) LFV and the most important municipal airports. 3) The total charges of 328.0 includes LFV (320) and the most important municipal airports (8.0), the subdivision refer to LFV only. Source: VTI. |  |  |  |  |

## Maritime transport

The total cost for infrastructure provider, environmental cost and total accident costs is 586 Million € or 0.3% of the Swedish GDP in 1998. The largest part of the cost is infrastructure cost (99%), with only 1% accident cost not born by the user. No environmental cost is estimated.

Table 5-96: Swedish inland waterway and maritime shipping 1996, 1998 and 2005  -€ million at 1998 prices

| Costs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Core information |  |  | 1996 |  |  | 1998 |  |  | 2005 |
| Infrastructure costs - waterways1) |  |  | 162 |  |  | 125.7 |  |  | .. |
| Fixed |  |  | 12.6 |  |  | 12.5 |  |  | .. |
| Variable |  |  | 149.4 |  |  | 113.2 |  |  | .. |
| Infrastructure costs – harbours2) |  |  | 423.6 |  |  | 453.9 |  |  | .. |
| Fixed |  |  | 47.1 |  |  | 61.5 |  |  | .. |
| Variable |  |  | 376.5 |  |  | 392.4 |  |  | .. |
| Accident costs (external)3) |  |  | 3.9 |  |  | 5.8 |  |  | 3.3 |
| Environmental costs |  |  | .. |  |  | .. |  |  | .. |
| Air pollution |  |  | .. |  |  | .. |  |  | .. |
| Global warming |  |  | .. |  |  | .. |  |  | .. |
| Noise |  |  | .. |  |  | .. |  |  | .. |
| Total |  |  |  |  |  | 585.4 |  |  |  |
| Additional information |  |  |  |  |  |  |  |  |  |
| Congestion costs |  |  | .. |  |  | .. |  |  | .. |
| Accident costs (internal) |  |  | 42.3 |  |  | 75.2 |  |  | 36.4 |
| From this: risk value |  |  | .. |  |  | .. |  |  | .. |
| Environmental costs |  |  | .. |  |  | .. |  |  | .. |
| Nature and landscape, soil and water pollution |  |  | .. |  |  | .. |  |  | .. |
| Nuclear risk |  |  | .. |  |  | .. |  |  | .. |
| Revenues |  |  |  |  |  |  |  |  |  |
| Directly allocatable |  | 452.3 |  |  | 489.2 |  |  | .. |  |
| Charges for infrastructure usage |  |  | 452.3 |  |  | 489.2 |  |  | .. |
| Fixed |  |  |  |  |  |  |  |  |  |
| Variable |  |  |  |  |  |  |  |  |  |
| Total4) |  |  | 452.3 |  |  | 489.2 |  |  |  |
| Other transport specific revenues |  |  |  |  |  |  |  |  |  |
| Fuel tax |  |  |  |  |  |  |  |  |  |
| Eco tax |  |  |  |  |  |  |  |  |  |
| VAT |  |  |  |  |  |  |  |  |  |
| Subsidies |  |  |  |  |  |  |  |  |  |
| From State to shipping company5) |  |  | 47.1 |  |  | 83.0 |  |  | .. |
| Appropriation from State to SFV |  |  | 11.7 |  |  | 11.9 |  |  | .. |
| Charges from buyers of sea transport to shipping companies |  |  | 3 177.5 |  |  | 3 309.2 |  |  | .. |
| - Due to SFV. - Due to SHSF - The cost is allocated to the mode in which the victim belongs. Of the total cost for maritime accidents, 56 M€ are fatalities in pleasure boat traffic. Excluded in the cost are work related accidents (5.2 M€) and suicide or illness among passengers (9 M€). The external cost is cost born by the rest of society, mainly medical cost, and net lost production. Transport system internal cost is cost born by road users and consists mainly of the ‘risk value’. - Fairway charge due to gross weight and the weight of loaded/unloaded goods, harbour charge due to ship, goods and passenger charge, goods handle income and crane charge. Pilot charge about 15 million euro is not included. Regarding the charges to SFV the charges are variable due to gross weight and the weight of goods loaded and/or unloaded. Regarding the charges to harbours 80 % of the charges are commercial agreements and 20 % are due to fees. The only charge that is variable due to transport length is the pilot charge. This makes it difficult to subdivide the total into variable and fixed charges. See section 4.8.5. - Tax relief according to domestic water transport. |  |  |  |  |  |  |  |  |  |

Infrastructure

The Swedish maritime infrastructure consists of 50 harbours and fairways, both at sea and at inland water. The majority of the cost is harbour infrastructure cost. Only asset value from business account is included.

Accidents

For the Maritime sector we have detailed information by Swedish ships on all water and less detailed information for all ships in Swedish water. None of the definitions will assure that an aggregate of UNITE accounts will cover all Maritime accidents in Europe.The total accident cost of maritime accidents is 81 M€ in Swedish water 1998 of which 55 M€ is due to pleasure boats (in this version we have not included the material damages). Accidents with Swedish ships costs 65 M€ including pleasure boats. Excluding pleasure boats the cost is 9.5 M€ of which 86% is accidents with fishing vessels and 14% accident with passengers on bulk ship cost. The total accident cost born by the rest of society is estimated to 8%.

Revenues and supplier operators

The revenue from companies to the infrastructure provider was 489 Million €. The revenues are 84% of the cost for infrastructure providers and accident cost born by non-users. In addition, users pay 3 309 Million € in tariffs to (Swedish) shipping companies. For these services they have costs of 3 280 Million €. If we add the suppliers cost to the infrastructure, environmental and accident cost for non-users the shipping sector has a cost of 3 436 Million € or 1.6 % of GDP.

Trend

The costs (including users accident cost but environmental cost and supplier operating cost) have increased with 2% between 1996 and 1998 and are forecasted to increase with additional 20% until 2005. This strong trend is due to an optimistic investment plan in harbours. Accident costs are forecasted to decline.

Cost per call

To find reliable estimates on vehicle kilometres has been difficult. In the following, we express the cost per call instead. The table does only include variable costs and revenues. The accident cost is, as noted above, 93% due to accidents with pleasure boats. The total accident cost per call is presented in parenthesis below.

Table 5-97: Average (variable) costs of Inland Waterways and maritime shipping per number of calls: Sweden - euro/call at 1998 prices –

|  | 1998 |  |  |
| --- | --- | --- | --- |
|  | Inland waterways1) |  | Maritime shipping1) |
| Core information |  |  |  |
| Infrastructure costs / variable | 4 190 |  |  |
| Inland waterway harbours and sea harbours |  |  |  |
| Variable | 3 252 |  |  |
| Inland waterways2) |  |  |  |
| Variable | 938 |  |  |
| External accident costs | (48) |  |  |
| Material cost | (48) |  |  |
| Risk value | - |  |  |
| Environmental costs | .. |  | .. |
| Air pollution | .. |  | .. |
| Global warming | .. |  | .. |
| Noise | .. |  | .. |
| Total I | 4 286 |  |  |
| Additional information |  |  |  |
| Delay costs |  |  |  |
| Internal accident costs | (623) |  |  |
| Material damages | ..- |  |  |
| Risk value | (623) |  |  |
| Environmental costs |  |  |  |
| Nature, landscape, soil and Water pollution | .. | .. |  |
| Nuclear risk | . | . |  |
| Total II |  |  |  |
| Revenues |  |  |  |
| Charges for infrastructure usage3) | 4 054 |  |  |
| Fixed |  |  |  |
| Variable |  |  |  |
| Fuel tax |  |  |  |
| Eco tax |  |  |  |
| VAT |  |  |  |
| Subsidies | 6884) |  |  |
| Basic data |  |  |  |
| Vehicle Kilometres |  |  |  |
| Tonne km (mill)5 | 30 431 |  |  |
| Number of calls6) | 120 658 |  |  |
| - In the Swedich accounts it was not possible to distinguish between Inland waterway and Maritime shipping. - Fairways etc. Inland waterway and maritime shipping. - Fairway charges and port charges. - Tax relief according to domestic watertransport. - Domestic 8.3 and international traffic along the Swedish coast 22,1. - Cargo 38 086 and passenger 82 572. |  |  |  |

Total costs

For Sweden, we have not distinguished between inland waters and maritime shipping. The following table contains mainly therefore the same information as the previous tables. However, following the UNITE standard, Table 5-98 has a slightly different form than Table 5-96.

Table 5-98: Total costs of Inland Waterways and maritime shipping: Sweden
- € million at 1998 prices –

|  | 1998 |  |
| --- | --- | --- |
|  | Inland waterways1) | Maritime shipping1) |
| Core information |  |  |
| Infrastructure costs | 579.6 |  |
| Harbours | 453.92) |  |
| Inland waterways |  |  |
| Fixed | 61.52) |  |
| Variable | 392.42) |  |
| Sea harbours | .. | .. |
| Fixed | Included above |  |
| Variable | Included above |  |
| Waterways | 125.73) |  |
| Inland waterways |  |  |
| Fixed | 12.53) |  |
| Variable | 113.23) |  |
| Maritime shipping |  |  |
| Fixed | Included above |  |
| Variable | Included above |  |
| External accident costs | 5.8 |  |
| Material cost | 5.8 |  |
| Risk value | 0 |  |
| Environmental costs | .. | .. |
| Air pollution | .. | .. |
| Global warming | .. | .. |
| Noise | .. | .. |
| Total I | 585.4 |  |
| Additional information |  |  |
| Delay costs |  |  |
| Internal accident costs | 75.2 |  |
| Material damages | .. |  |
| Risk value | 75.2 |  |
| Environmental costs | .. | .. |
| Nature, landscape, soil and water pollution | .. |  |
| Nuclear risk | .. |  |
| Total II | 75.2 |  |
| Revenues |  |  |
| Charges for infrastructure usage | 489.24) |  |
| Fixed |  |  |
| Variable |  |  |
| Fuel tax |  |  |
| Eco tax |  |  |
| VAT |  |  |
| Subsidies | 82.95) |  |
| Basic data |  |  |
| Tonne km (mill) | 30 4316) |  |
| - In the Swedish accounts there was not possible to distinguish between Inland waterways and maritime shipping. - Inland waterway harbours and sea harbours. - Fairways etc. - Fairway charges (100) and port charges (388.8) - Tax relief according to domestic water transport. - Domestic and international traffic along the Swedish coast |  |  |

# Conclusions

The Swedish transport sector had a cost in 1998 equivalent to 11% of GDP. Excluded from this cost estimate are the cost for transport on own account and the cost of travel time for passengers of all modes and drivers of passenger cars. Of the cost, 63% is supplier operating cost, 17% infrastructure cost, 15% accident cost and 5% environmental cost. Our conclusions from the Swedish pilot accounts are in summary:

- While efficient use of infrastructure is important, it is also important to ensure a well functioning transport market as supplier operating cost is a dominant element.

- The past years policies to curb transport externalities will bear fruit in the near future, the total cost of the transport sector will decline.

- The pilot accounts shows that it is difficult to make comprehensive transport account, the information is often lacking, difficult to find and of a very uncertain quality.

- However, the work with the pilot accounts could have been more efficient if the purpose of the account was clearly stated in the project. All the subjective judgemenst about the ‘correct number’ has to be made with an unclear purpose as the leading star.

The magnitude of the supplier operating cost compared to the infrastructure related cost is an interesting observation with policy implications. While it is important to ensure efficient provision and use of the infrastructure - which includes pricing for infrastructure damages, external accident cost, environment, and congestion - it is also important to ensure that the transportation market is working properly.

Our observed costs for 1996 and 1998 describe an increasing trend (+7%) while our forecast for 2005 suggests that the cost will fall (-9%). The sharp decline in the costs is thanks to the positive forecasted development in road safety due to the ‘Zero-accident vision’ and in the environmental cost. If the ‘Zero-accident vision’ is successful in the future, without increasing the infrastructure cost or the non monitored time cost of users, the core cost of Swedish transport infrastructure has reached a peak and will start to decline. The past years policies to curb transport externalities will start to bear fruit in the near future.

From the Swedish perspective, with a clear marginal cost based transport policy, the aim of the accounts is rather unclear. However, looking around the European Union it can be noticed that accounting practice is a more common ‘science’ than marginal cost estimates. The final stage of the UNITE project, when all the accounts are summarised and integrated with the marginal cost work, can hopefully close the gap between the current transportation pricing practices in Europe.

Transport accounts have been carried out previously in Sweden and were a part of an older transport policy. Compared to the last official account (Hansson, Lindberg 1992) the UNITE pilot account includes some new dimensions:

- The infrastructure cost is calculated as a cost and not as annual expenditure

- The supplier operating cost is included

- However, congestion cost is badly treated in the pilot account as well as in previous accounts

- The accident cost is presented in a way that is closely linked to the marginal cost theory

- The environmental cost is calculated with the European ‘standard’ Impact Pathway model

- As before, taxes and charges are included but in the Pilot account also the VAT on top of taxes are included

It is a time consuming work to collect all the information for the Pilot accounts. Many figures are not collected centrally and they have thus to be found at single firms or municipalities. While other data is centrally stored, numbers differs often between different sources; a very difficult subjective judgement has to be made. To make these judgements, and to decide on the proper resources to allocate on detailed data finding, a clearer purpose of the Pilot account would have been helpful.

We have had problems with especially two areas; Public transport and Municipalities. Public transport is a ‘strange mode’. It contains road and rail transport, private firms and public owned entities. To disentangle all this information and ensure that costs do not appear twice has been a difficult job and probably not always successful. The other problem is the cost of municipalities. Some of the costs needed for the pilot accounts are not centrally collected and barely understood at the municipality level. To spend huge resources to contact each municipality has not been possible within the project.

We do often rely on information collected by other organisations, as the Swedish Municipality association. If these organisations do not find it worthwhile to continue their work, the production of accounts will be even more difficult in the future. If transportation accounts shall be produced on a regularly basis a restructuring of the data collection process has to be made. However, it is probably more about asking the right question and store the right information than to collect new information. A small restructuring of the data collection structure could make production of the transport account a simply task in the future.

# References

BV, databasen BIS

BV, Information on net delays  from Tågtrafikledningen

BV, länstransportplan 1998-2007

BV, Statistik över olyckor på statens spåranläggningar 1998

BV, stomnätsplan 1998-2007

BV, Årsredovisning 1996, 1998

European Commission, 1999. ExternE  Externalities of Energy

KF, Kommunernas väghållning och parkskötsel 1998

KI, www.konj.se

LFV, Flygplatsjämförelse 1996 Statliga och kommunala flygplatser

LFV, Flygplatsjämförelse 1998 Statliga och kommunala flygplatser

LFV, PM Säkerhetsutveckling för civil luftfart I Sverige, 2000

LFV, Årsredovisning 1996, 1998

Link et al. 2000. The account approach

Naturvårdsverket, Miljöskatter i Sverige 1997

Nellthorp, Valuation conventions for UNITE (Nellthorp et.al 2001)

Regeringen, Prop. 1996/97, 1997/98, 2000/2001 Utgiftsområde 22 kommunikationer

Regeringen, Prop. 1997/1998:150 bilaga 6 skatteavvikelser

SAMPLAN Rapport 99:2.

SCB, Ekonomisk redogörelse för företagen 1996,1998

SCB, Statistisk årsbok 1955-1999

SCB, www.scb.se

SFV, Sjöfartsverkets meddelanden nr 2 1999 resp. nr 4 1997

SFV, Strategisk analys, underlagsrapport till samplan om Hamnstruktur och Sjöfart 1999

SFV, Årsredovisning 1996,1998

SHSF, Hamntrafik och ekonomi 1996, 1998

SIKA Årsbok 1998, 2000/2001

SIKA,  Järnvägar 1996-1997

SIKA,  Kommunikationer nr 4-2000

SIKA, Inrikes godstransportarbete efter färdsätt 1975-1999

SIKA, Inrikes transportarbete efter färdsätt 1994-1998

SIKA, Inrikes transporterad godsmängd efter transportsätt 1975-1979

SIKA, Luftfart 1996, 1999

SIKA, Luftfart 1999

SIKA, Rapport 2000:7

SIKA, Transporter och Kommunikationer, årsbok 2000/2001

SIKA, www.sika-institute.se

SIKA, Vägtrafiksskador 1998

SLTF, Branschstatistik 1996, 1998

SVEDAB Årsrapport 1998

VV, Bilaga 5.2 Trafikarbete och bränsleförbrukning 1994-1999

VV, Nationell plan för vägtransportsystemet 1998-2007

VV, Traffic count for different road types, volume delay functions from the EVA-model.

VV, Vägverkets accident database, VITS

VV, Årsrapport 1996, 1998

ÖSK, Årsrapport 1998

"External Costs of Energy"

SIKA Report 1998:6 gives many different definitions and discuss the problem more in detail.

Arrivals 4Q 2000 at Arlanda; no-delay 57%, I) 26%, II) 9%, III) 5%, IV) 3%. Flygets utveckling 2000, sektorsrapport. LFV, Norrköping (2001).

If we had used the original database (BIS at Banverket), which we have access to, we could have made a distinction between freight and passenger train involved. However, given the limited number of accidents we did not make this a priority.

- European Commission (1999) ExternE Externalities of Energy.

Car 21.30 €/h, rail 15.70 €/h, air 16.82 €/h and bus/coach 12.33 €/h.

Consistent with information on passenger kilometre and train kilometre

The relationship between the risk value for fatality and severe injury is 0.16 and for light injury 0.007 for the official Swedish values. UNITE proposes to use the factor 0.13 and 0.01 respectively (see UNITE valuation note).

The 1.53 M€ is divided with the average indirect taxation 1.242 (see UNITE valuation note).

Urban public transport is in the Swedish accounts named public transport and covers urban and interurban public transport.

In the statistical annex the composition of the structural business statistics is presented in detail.

The total cost by one mode, includes cost of accidents with other modes. When the total cost is summarised (last row) for all modes, these inter-modal accident costs could be counted twice, if this is not noticed. As a result, the total cost by each mode can not simple be summarised.

Includes MC, moped, Car, Bus/Coach, LGV and HGV

Refers to emissions from bus traffic which is also reported in the column road