- COMPETITIVE AND SUSTAINABLE GROWTH

(GROWTH)

PROGRAMME

Deliverable 14 Annex 2 Costs due to air pollution from maritime shipping

- Version 1.0

- 25 September 2002

Authors: Peter Bickel, Stephan Schmid (IER),

with contributions from partners

Contract: 1999-AM.11157 Project Co-ordinator: ITS, University of Leeds Funded by the European Commission 5th Framework – Transport RTD

UNITE Partner Organisations ITS/UNIVLEEDS (UK), DIW (De), NEI (Nl), CES/KUL (Be), TIS.PT (Pt), IWW/UNIKARL (De), VTI (Se), IER/USTUTT (De), CERAS/ENPC (Fr), HERRY (Au), EIET/ULPGC (Es), ISIS (It), STRATEC (Be), SYSTEMA (Gr), JP-TRANSPLAN (Fi), VATT (Fi), ECOPLAN (Ch), INFRAS (Ch), EKI (Se)

- UNITE

1999-AM.11157

- UNIfication of accounts and marginal costs for Transport Efficiency

- Approach for estimating costs due to air pollution, global warming and noise

- This document should be referenced as:

Peter Bickel, Stephan Schmid (IER) Costs due to air pollution from maritime shipping, UNITE (UNIfication of accounts and marginal costs for Transport Efficiency) Working Funded by 5th Framework RTD Programme. ITS, University of Leeds, Leeds, September 2002.

25 September 2002

Version No: 1.0

Authors: as above

- PROJECT  INFORMATION

- Contract no: 1999-AM.11157:

- UNIfication of accounts and marginal costs for Transport Efficiency

- Website: www.its.leeds.ac.uk/unite

- Commissioned by: European Commission – DG TREN; Fifth Framework Programme

- Lead Partner:	Institute for Transport Studies, University of Leeds (UK)

Partners: 	ITS/UNIVLEEDS (UK), DIW (De), NEI (Nl), CES/KUL (Be), TIS.PT (Pt), IWW/UNIKARL (De), VTI (Se), IER/USTUTT (De), CERAS/ENPC (Fr), HERRY (Au), EIET/ULPGC (Es), ISIS (It), STRATEC (Be), SYSTEMA (Gr), JP-TRANSPLAN (Fi), VATT (Fi), ECOPLAN (Ch), INFRAS (Ch), EKONO (Fi), EKI (Se)

- DOCUMENT  CONTROL  INFORMATION

- Status: 	Accepted

Distribution:	UNITE partners

Availability:	Public (only once status above is “Accepted” as part of Deliv. 14)

Filename:	UNITE D14 Annex 2

Quality assurance:

Co-ordinator’s review:	C A Nash

Signed: 	Date:

- Table of contents

1	Introduction	4

2	Description of input data	4

2.1	Emissions from Maritime Shipping	4

2.2	General data for the calculation of costs due to air pollution	5

2.2.1	Receptor data	6

2.2.2	Emission data	7

2.2.3	Meteorological data	7

3	Methodological issues	8

3.1	Air pollution	8

3.1.1	General approach	8

3.1.2	Monetary values	13

3.1.3	Discussion of uncertainties	13

3.2	Global warming	15

4	Results	16

References	17

Glossary	19

Abbreviations	20

ANNEX: Illustration of emission areas	Error! Bookmark not defined.

# 1	Introduction

In the phase of collecting data for the country accounts it soon showed that reliable and consistent data for calculating emissions of air pollutants and CO2 is hardly available in most countries. Furthermore the allocation of emissions to the different countries is not straightforward. As a consequence it was decided that environmental costs  of maritime shipping should be treated as a separate part of the pilot accounts. Costs due to air pollution and global warming are calculated consistently to the other modes, based on an emission inventory of shipping emissions provided by EMEP. Noise costs are assumed to be of very minor importance and are therefore neglected. The following chapters describe input data, methodology and results.

# 2	Description of input data

## 2.1	Emissions from Maritime Shipping

Starting point of the calculations was a spatially disaggregated emission inventory published by EMEP, based on estimations from Lloyd’s Register of Shipping. Base year for the emissions is 1990, data for more recent base years are not available. Relevant pollutants available were SO2 and NOx. Emissions of PM2.5 were derived from the NOx emissions using an average NOx/PM2.5 ratio of 28 which was estimated based on data from Alexandersson (1990) and Marintek (1989). CO2 emissions were derived from SO2 emissions, assuming an average sulphur content of 45 g SO2 per kg of fuel (MEPC 1989) and an emission factor of 3.18 kg CO2 per kg of fuel.

EMEP/MSC-W (2000a) and EMEP/MSC-W (2000b) give emission from international ship traffic in all sea areas. To get a more differentiated picture, seven geographical areas were distinguished:

- Atlantic Ocean,

- Baltic Sea,

- Black Sea,

- The Channel,

- North Sea,

- Mediterranean Sea (east), and

- Mediterranean Sea (west).

These areas are illustrated in the Annex. The emission data presented in Table 1 were used for the calculations

- with the EcoSense computer model for airborne pollutants, and

- with shadow values for greenhouse gas emissions.

- Table 1
Shipping emissions in different geographical areas (1000 tonnes)

|  | SO2 | NOx | PM2.5 | CO2 |
| --- | --- | --- | --- | --- |
| Atlantic Ocean | 584 | 832 | 29.7 | 41 221 |
| Baltic Sea | 230 | 355 | 12.7 | 16 237 |
| Black Sea | 80 | 121 | 4.3 | 5 657 |
| The Channel | 212 | 298 | 10.7 | 14 960 |
| North Sea | 260 | 378 | 13.5 | 18 323 |
| Mediterranean Sea (east) | 531 | 738 | 26.4 | 37 447 |
| Mediterranean Sea (west) | 487 | 669 | 23.9 | 34 344 |
| Source: IER based on EMEP/MSC-W (2000a,b) |  |  |  |  |

## 2.2	General data for the calculation of costs due to air pollution

Besides the emissions from maritime shipping, a large number of additional information was required for the cost calculations. This includes data on the receptor distribution, meteorology, and on the background emissions from all sources in all European countries. Such data is available in the EcoSense database and is briefly described in the following.

- Table 2
Environmental data in the EcoSense database

|  | Resolution | Source |
| --- | --- | --- |
| Receptor distribution |  |  |
| Population | administrative units, EMEP 50 grid | EUROSTAT REGIO Database, The Global Demography Project |
| Production of wheat, barley, sugar beat, potato, oats, rye, rice, tobacco, sunflower | administrative units, EMEP 50 grid | EUROSTAT REGIO Database, FAO Statistical Database |
| Inventory of natural stone, zinc, galvanized steel, mortar, rendering, paint | administrative units, EMEP 50 grid | Extrapolation based on inventories of some European cities |
| Meteorological data |  |  |
| Wind speed | EMEP 50 grid | European Monitoring and Evaluation Programme (EMEP) |
| Wind direction | EMEP 50 grid | European Monitoring and Evaluation Programme (EMEP) |
| Precipitation | EMEP 50 grid | European Monitoring and Evaluation Programme (EMEP) |
| Emissions |  |  |
| SO2, NOx, NH3, NMVOC, particles | administrative units, EMEP 50 grid | CORINAIR 1994/1990, EMEP 1998 TNO particulate matter inventory (Berdowski et al., 1997) |
| Source: IER. |  |  |

### 2.2.1	Receptor data

- Population data

Population data was taken from the EUROSTAT REGIO database (base year 1996), which provides data on administrative units (NUTS categories). For impact assessment, the receptor data is required in a format compatible with the output of the air quality models. Thus, population data was transferred from the respective administrative units to the 50 x 50 km2 EMEP grid by using the transfer routine implemented in EcoSense.

- Crop production

The following crop species were considered for impact assessment: barley, oats, potato, rice, rye, sunflower seed, tobacco, and wheat. Data on crop production were again taken from the EUROSTAT REGIO database (base year 1996). For impact assessment, crop production data were transferred from the administrative units to the EMEP 50 x 50 km2 grid.

- Material inventory

The following types of materials are considered for impact assessment: galvanised steel; limestone; mortar; natural stone; paint; rendering; sandstone; and, zinc. As there is no database available that provides a full inventory of materials, the stock at risk was extrapolated in ExternE from detailed studies carried out in several European cities.

### 2.2.2	Emission data

As the formation of secondary pollutants such as ozone or secondary particles depends heavily on the availability of precursors in the atmosphere, the EcoSense database provides a European wide emission inventory for SO2, NOx, NH3, NMVOC, and particles as an input to air quality modelling. The emission data are disaggregated both sectorally (‘Selected Nomenclature for Air Pollution’ - SNAP categories) and geographically (‘Nomenclature of Territorial Units for Statistics’ - NUTS categories). As far as available, EcoSense uses data from the EMEP 1998 emission inventory (Richardson 2000, Vestreng 2000, Vestreng and Støren 2000). Where required, data from the CORINAIR 1994 inventory. (http://www.aeat.co.uk/netcen/corinair/94/) and the CORINAIR 1990 inventory (McInnes 1996) are used. For Russia, national average emission data from the LOTOS inventory (Builtjes 1992) were included. Emission data for fine particles are taken from the European particle emission inventory established by Berdowski et al. (1997).

### 2.2.3	Meteorological data

The Windrose Trajectory Model requires annual average data on wind speed, wind direction, and precipitation as an input. The EcoSense database provides data from the European Monitoring and Evaluation Programme (EMEP) for the base year 1998.

# 3	Methodological issues

## 3.1	Air pollution

### 3.1.1	General approach

For quantifying the costs due to airborne pollutants the Impact Pathway Approach, the methodology developed in the ExternE project series was applied. A detailed description of the approach can be found in European Commission (1999). The impact pathway approach utilises the following steps: emission estimation, dispersion and chemical conversion modelling, calculation of physical impacts and monetary valuation of these impacts.

For the calculation of the costs of direct emissions from vessel operation emission inventories in spatial disaggregation are needed, i.e. a geo-coded data set for the different air pollutants. As illustrated in the Annex such an emission inventory was produced for the different geographical areas, giving total emissions in spatial disaggregation. For each of these emission inventories, Europe-wide impacts were calculated and subtracted from impacts resulting from a reference inventory without these emissions. This procedure using a reference inventory is required, because of air chemistry processes where “background” emissions play an important role. A description of the computer model EcoSense, which was used for the calculations, including exposure-response functions and monetary values is given below.

As most of the pollutants are emitted on open sea, only regional scale emissions were calculated, local scale damages were assumed to be negligible. Damages due to ozone formation could not be considered due to limitations of the SROM-Model.

a)	Description of the EcoSense computer model for assessment of costs due to airborne emissions

The EcoSense model has been developed within the series of ExternE Projects on ‘External Costs of Energy’ funded by the European Commission (see e.g. European Commission 1999). The model supports the quantification of environmental impacts by following a detailed site-specific ‘impact pathway’ (or damage function) approach, in which the causal relationships from the release of pollutants through their interactions with the environment to a physical measure of impact are modelled and, where possible, valued monetarily. A schematic flowchart of the EcoSense model is shown in Figure 1. EcoSense provides harmonised air quality and impact assessment models together with a comprehensive set of relevant input data for the whole of Europe, which allow a site specific bottom-up impact analysis.

- Figure 1:	Flowchart of the EcoSense model

In ExternE, EcoSense was used to calculate external costs from individual power plants in a large number of case studies in all EU countries. While the first generation of the EcoSense model was focused on the analysis of single emission sources, the new ‘multi-source’ version of the model provides a link to the CORINAIR database, which allows the analysis of environmental impacts from more complex emission scenarios. The CORINAIR database provides emission data for a wide range of pollutants according to both a sectoral (‘Selected Nomenclature for Air Pollution’ - SNAP categories) and geographic (‘Nomenclature of Territorial Units for Statistics’ - NUTS categories) disaggregation scheme (McInnes, 1996). A transformation module implemented in EcoSense supports the transformation of emission data between the NUTS administrative units (country, state, municipality) and the grid system required for air quality modelling (EMEP 50 x 50 km2 grid). Based on this functionality, EcoSense allows to modify emissions from a selected sector (e.g. road transport) within a specific administrative unit, creates a new gridded European-wide emission scenario for air quality modelling, and compares environmental impacts and resulting damage costs between different emission scenarios. In other words, environmental damage costs are calculated by comparing the results of two model runs:

- A model run using the ‘full’ European emission scenario as an input to air quality and damage modelling, including emissions from all emission sources in Europe.

- A second model run in which the shipping emissions were modified.

The difference in impacts and costs resulting from the two model runs represents the damages due to modified emissions.

b)	Air quality models

Within the UNITE project two air quality models were used from the three available within the Eco-Sense system. The model for local scale effects was not required as they were covered based on GIS-based calculations. The SROM model could not be applied for shipping emissions, because it is based on so called country-to-grid matrices, but the emissions take place over sea.

- The Windrose Trajectory Model (WTM) (Trukenmüller and Friedrich 1995) is used in EcoSense to estimate the concentration and deposition of acid species on a regional scale.

- The Source-Receptor Ozone Model (SROM), based on the EMEP country-to-grid matrices (Simpson et al. 1997), is used  to estimate ozone concentrations on a European scale.

c)	Dose-effect models

The dose-response functions used within UNITE are the final recommendations of the expert groups in the final phase of the ExternE Core/Transport project (Friedrich and Bickel 2001). The following table gives a summary of the dose-response functions as they are implemented in the EcoSense version used for this study.

- Table 3
Health and environmental effects included in the analysis of air pollution costs

| Impact category | Pollutant | Effects included |
| --- | --- | --- |
| Public health – mortality | PM2.5 , PM10 1) SO2, O3 | Reduction in life expectancy due to acute and chronic mortality Reduction in life expectancy due to acute mortality |
| Public health – morbidity | PM2.5 , PM10, O3 | respiratory hospital admissions |
|  |  | restricted activity days |
|  | PM2.5 , PM10 only | cerebrovascular hospital admissions |
|  |  | congestive heart failure |
|  |  | cases of bronchodilator usage |
|  |  | cases of chronic bronchitis |
|  |  | cases of chronic cough in children |
|  |  | cough in asthmatics |
|  |  | lower respiratory symptoms |
|  | O3 only | asthma attacks |
|  |  | symptom days |
| Material damage | SO2, acid deposition | Ageing of galvanised steel, limestone, natural stone, mortar, sandstone, paint, rendering, zinc |
| Crops | SO2 | Yield change for wheat, barley, rye, oats, potato, sugar beet |
|  | O3 | Yield loss for wheat, potato, rice, rye, oats, tobacco, barley, wheat |
|  | Acid deposition | increased need for liming |
|  | N, S | fertilisational effects |
| 1) including secondary particles (sulphate and nitrate aerosols). Source: IER. |  |  |

d)	Exposure-response functions for the quantification of health effects

Table 4 lists the exposure response functions used for the assessment of health effects. The exposure response functions are taken from the 2nd edition of the ExternE Methodology report (European Commission 1999), with some small modifications resulting from recent recommendations of the health experts in the final phase of the ExternE Core/ Transport project (Friedrich and Bickel 2001).

Table 4
Quantification of human health impacts due to air pollution1)

| Receptor | Impact Category | Reference | Pollutant | fer |
| --- | --- | --- | --- | --- |
| ASTHMATICS (3.5% of population) |  |  |  |  |
| Adults | Bronchodilator usage | Dusseldorp et al., 1995 | PM10 Nitrates PM2.5 Sulphates | 0.163 0.163 0.272 0.272 |
|  | Cough | Dusseldorp et al., 1995 | PM10, Nitrates PM2.5 Sulphates | 0.168 0.280 0.280 |
|  | Lower respiratory symptoms (wheeze) | Dusseldorp et al., 1995 | PM10 Nitrates PM2.5 Sulphates | 0.061 0.061 0.101 0.101 |
| Children | Bronchodilator usage | Roemer et al., 1993 | PM10 Nitrates PM2.5 Sulphates | 0.078 0.078 0.129 0.129 |
|  | Cough | Pope and Dockery, 1992 | PM10 Nitrates PM2.5 Sulphates | 0.133 0.133 0.223 0.223 |
|  | Lower respiratory symptoms (wheeze) | Roemer et al., 1993 | PM10 Nitrates PM2.5 Sulphates | 0.103 0.103 0.172 0.172 |
| All | Asthma attacks (AA) | Whittemore and Korn, 1980 | O3 | 4.29E-3 |
| ELDERLY 65+ (14% of population) |  |  |  |  |
|  | Congestive heart failure | Schwartz and Morris, 1995 | PM10 Nitrates PM2.5 Sulphates CO | 1.85E-5 1.85E-5 3.09E-5 3.09E-5 5.55E-7 |
| CHILDREN (20% of population) |  |  |  |  |
|  | Chronic cough | Dockery et al., 1989 | PM10 Nitrates PM2.5 Sulphates | 2.07E-3 2.07E-3 3.46E-3 3.46E-3 |
| ADULTS (80% of population) |  |  |  |  |
|  | Restricted activity days (RAD) | Ostro, 1987 | PM10 Nitrates PM2.5 Sulphates | 0.025 0.025 0.042 0.042 |
|  | Minor restricted activity days (MRAD) | Ostro and Rothschild, 1989 | O3 | 9.76E-3 |
|  | Chronic bronchitis | Abbey et al., 1995 | PM10 Nitrates PM2.5 Sulphates | 2.45E-5 2.45E-5 3.9E-5 3.9E-5 |
| ENTIRE POPULATION |  |  |  |  |
|  | Chronic Mortality (CM) | Pope et al., 1995 | PM10 Nitrates PM2.5 Sulphates | 0.129% 0.129% 0.214% 0.214% |
|  | Respiratory hospital admissions (RHA) | Dab et al., 1996 | PM10 Nitrates PM2.5 Sulphates | 2.07E-6 2.07E-6 3.46E-6 3.46E-6 |
|  |  | Ponce de Leon, 1996 | SO2 O3 | 2.04E-6 3.54E-6 |
|  | Cerebrovascular hospital admissions | Wordley et al., 1997 | PM10 Nitrates PM2.5 Sulphates | 5.04E-6 5.04E-6 8.42E-6 8.42E-6 |
|  | Symptom days | Krupnick et al., 1990 | O3 | 0.033 |
|  | Cancer risk estimates | Pilkington et al., 1997; based on US EPA evaluations | Benzene Benzo-[a]-Pyrene 1,3-buta-diene Diesel particles | 1.14E-7 1.43E-3 4.29E-6 4.86E-7 |
|  | Acute Mortality (AM) | Spix et al. / Verhoeff et al., 1996 | PM10 Nitrates PM2.5 Sulphates | 0.040% 0.040% 0.068% 0.068% |
|  |  | Anderson et al. / Touloumi et al., 1996 | SO2 | 0.072% |
|  |  | Sunyer et al., 1996 | O3 | 0.059% |
| 1) The exposure response slope, fer, has units of [cases/(yr-person-µg/m3)] for morbidity, and [%change in annual mortality rate/(µg/m3)] for mortality. Concentrations of SO2, PM10 , PM10, sulphates and nitrates as annual mean concentration, concentration of ozone as seasonal 6-h average concentration. Source: Friedrich and Bickel 2001. |  |  |  |  |

e)	Exposure-response functions for the quantification of impacts on crops

Functions are used within the model to quantify changes in crop yields due to the emissions of SO2, nitrates, ozone and acids.

f)	Exposure-response functions for the quantification of material damage

Functions were developed to quantify and value damages to limestone, sandstone, natural stone, mortar, rendering, zinc and galvanised steel and paint due to the effects of air pollution.

### 3.1.2	Monetary values

Table 5 summarises the monetary values used for valuation of transboundary air pollution.

- Table 5
Monetary values (factor costs; European average) for health impacts (€1998)

| Impact | Monetary value (rounded) |  |
| --- | --- | --- |
| Year of life lost (chronic effects) | 74 700 | € per YOLL |
| Year of life lost (acute effects) | 128 500 | € per YOLL |
| Chronic bronchitis | 137 600 | € per new case |
| Cerebrovascular hospital admission | 13 900 | € per case |
| Respiratory hospital admission | 3 610 | € per case |
| Congestive heart failure | 2 730 | € per case |
| Chronic cough in children | 200 | € per episode |
| Restricted activity day | 100 | € per day |
| Asthma attack | 69 | € per day |
| Cough | 34 | € per day |
| Minor restricted activity day | 34 | € per day |
| Symptom day | 34 | € per day |
| Bronchodilator usage | 32 | € per day |
| Lower respiratory symptoms | 7 | € per day |
| Source: Own calculations based on Friedrich and Bickel (2001) and Nellthorp et al. (2001). |  |  |

### 3.1.3	Discussion of uncertainties

In spite of considerable progress made in recent years the quantification and valuation of environmental damage is still linked to significant uncertainty. This is the case for the Impact Pathway Methodology as well as for any other approach. While the basic assumptions underlying the work in ExternE are discussed in detail in (European Commission 1999), below an indication of the uncertainty of the results is given as well as the sensitivity to some of the key assumptions.

Within ExternE, Rabl and Spadaro (1999) made an attempt to quantify the statistical uncertainty of the damage estimates, taking into account uncertainties resulting from all steps of the impact pathway, i.e. the quantification of emissions, air quality modelling, dose-effect modelling, and valuation. Rabl and Spadaro show that - due to the multiplicative nature of the impact pathway analysis - the distribution of results is likely to be approximately lognormal, thus it is determined by its geometric mean and the geometric standard deviation g. In ExternE, uncertainties are reported by using uncertainty labels, which can be used to make a meaningful distinction between different levels of confidence, but at the same time do not give a false sense of precision, which seems to be unjustified in view of the need to use subjective judgement to compensate the lack of information about sources of uncertainty and probability distributions (Rabl and Spadaro 1999). The uncertainty labels are:

A = high confidence, corresponding to g = 2.5 to 4;

B = medium confidence, corresponding to g = 4 to 6;

C = low confidence, corresponding to g = 6 to 12.

According to ExternE recommendations, the following uncertainty labels are used to characterise the impact categories addressed in this report:

Mortality:		B

Morbidity:		A

Crop losses:		A

Material damage:	B.

Beside the statistical uncertainty indicated by these uncertainty labels, there is however a remaining systematic uncertainty arising from a lack of knowledge, and value choices that influence the results. Some of the most important assumptions and their implications for the results are briefly discussed in the following.

- Effects of particles on human health

The dose-response models used in the analysis are based on results from epidemiological studies which have established a statistical relationship between the mass concentration of particles and various health effects. However, at present it is still not known whether it is the number of particles, their mass concentration or their chemical composition which is the driving force. The uncertainty resulting from this lack of knowledge is difficult to estimate.

- Effects of nitrate aerosols on health

We treat nitrate aerosols as a component of particulate matter, which we know cause damage to human health. However, in contrast to sulphate aerosol (but similar to many other particulate matter compounds) there is no direct epidemiological evidence supporting the harmfulness of nitrate aerosols, which partly are neutral and soluble.

- Valuation of mortality

While ExternE recommends to use the Value of a Life Year Lost rather than the Value of Statistical Life for the valuation of increased mortality risks from air pollution (see European Commission, (1999) for a detailed discussion), this approach is still controversially discussed in the literature. The main problem for the Value of a Life Year Lost approach is that up to now there is a lack of empirical studies supporting this valuation approach.

- Omission of effects

The present report is limited to the analysis of impacts that have shown to result in major damage costs in previous ExternE studies. Impacts on e.g. change in biodiversity, potential effects of chronic exposure to ozone, cultural monuments, direct and indirect economic effects of change in forest productivity, fishery performance, and so forth, are omitted because they currently cannot be quantified.

## 3.2	Global warming

The method of calculating costs of CO2 emissions basically consists of multiplying the amount of CO2 emitted by a cost factor. Due to the global scale of the damage caused, there is no difference how and where the emissions take place.

A European average shadow value of €20 per tonne of CO2 emitted was used for valuing CO2 emissions. This value represents a central estimate of the range of values for meeting the Kyoto targets in 2010 in the EU based on estimates by Capros and Mantzos (2000). They report a value of €5 per tonne of CO2 avoided for reaching the Kyoto targets for the EU, assuming a full trade flexibility scheme involving all regions of the world. For the case that no trading of CO2 emissions with countries outside the EU is permitted, they calculate a value of €38 per tonne of CO2 avoided. It is assumed that measures for a reduction in CO2 emissions are taken in a cost effective way. This implies that reduction targets are not set per sector, but that the cheapest measures are implemented, no matter in which sector.

Looking further into the future, more stringent reductions than the Kyoto aims are assumed to be necessary to reach sustainability. Based on a reduction target of 50% in 2030 compared to 1990, INFRAS/IWW (2000) use avoidance costs of € 135 per t of CO2; however one could argue that this reduction target has not yet been accepted.

A valuation based on the damage cost approach, as e.g. presented by ExternE (Friedrich and Bickel 2001), would result in substantially lower costs. Due to the enormous uncertainties involved in the estimation process, such values have to be used very cautiously.

# 4	Results

Table 6 presents the quantifiable environmental costs due to maritime shipping in European waters. Costs due to air pollution are not only determined by the amount of pollutants emitted, but as well by the geographical location and thus the number of receptors affected. A good example for the importance of the location of the emission is the Channel, which has second lowest emissions for all pollutants (see Table 1), but shows the highest cost due to airborne pollutants. In contrast to air pollution costs, costs due to global warming are independent of the location of emission, but are directly proportional to the amount of CO2 emitted.

- Table 6
Quantifiable environmental costs due to maritime shipping in Europe (costs 1998, emissions 1990)

| [Million €] | Air Pollution1,2) | Global Warming1) | Noise | Total |
| --- | --- | --- | --- | --- |
| Atlantic Ocean | 1 733 | 824 | 0 | 2 557 |
| Baltic Sea | 1 245 | 325 | 0 | 1 570 |
| Black Sea | 536 | 113 | 0 | 649 |
| The Channel | 2 632 | 299 | 0 | 2 932 |
| North Sea | 2 178 | 367 | 0 | 2 544 |
| Mediterranean Sea (east) | 1 572 | 749 | 0 | 2 321 |
| Mediterranean Sea (west) | 2 363 | 687 | 0 | 3 050 |
| Total | 12 258 | 3 364 | 0 | 15 622 |
| 1) direct emissions from fuel combustion; 2) no effects due to ozone formation included due to model restrictions |  |  |  |  |

The results should be interpreted as an indication of the possible costs, because emission estimates show a higher degree of uncertainty than estimates for other modes, above all road transport. This is underlined by the fact that the most current estimates available refer to the year 1990. In spite of the uncertainties it has become clear that maritime shipping causes considerable costs due to airborne emissions. However, it is important to improve the availability of data, above all on emission of pollutants and greenhouse gases.

# References

Alexandersson, A. (1990) Presentation of the swedish investigation - Exhaust emissions from ships, Gothenburg, cited after Brandt und Höth (1999).

Berdowski, J., Mulder, W., Veldt, C., Visschedijk, A., Zandveld, P. (1997) Particulate matter (PM10 - PM2.5 - PM0.1) emissions in Europe. TNO-report TNO-MEP-R 96/472, TNO, Appeldoorn, The Netherlands.

Brandt, M. und Höth, Th. (1999) MARION - Umweltrelevantes Informations- und Analysesystem für den Seeverkehr, Hansestadt Bremisches Hafenamt (HBH) im Auftrag des Umweltbundesamtes, Bremerhaven.

Builtjes, P. (1992) The LOTOS-Long Term Ozone Simulation project, Summary report, TNO-report, IMW-R 92/240, 1992.

Capros, P. and Mantzos, L. (2000) Kyoto and technology at the European Union: costs of emission reduction under flexibility mechanisms and technology progress, Int. J. Global Energy Issues, 14, pp. 169-183.

EMEP/MSC-W (2000a) SOx Emission from International Ship traffic in All Sea Areas. Produced by EMEP/MSC-W on September 30 2000. http://www.emep.int/emis_data/allsea_sox.txt

EMEP/MSC-W (2000b) NOx Emission from International Ship traffic in All Sea Areas. Produced by EMEP/MSC-W on September 30 2000. http://www.emep.int/emis_data/allsea_nox.txt

European Commission (1999) ExternE Externalities of Energy. Vol 7 – Methodology 1998 update. A Report produced for the EC – DG XII, Luxembourg, Office of Publications for the European Communities, Luxembourg.

Friedrich, R. and Bickel, P. (eds.) (2001) Environmental External Costs of Transport. Springer Verlag, Heidelberg.

INFRAS/IWW (2000) External Costs of Transport, Study for the International Union of Railways, Paris.

Jonson, J.E., Tarrasón, L. and Bartnicki, J. (2000) Effects of international shipping on European pollution levels. EMEP/MSC-W Note 5/2000, Oslo.

MARINTEK (1989) Emission from ships - a global view, Oslo, cited after Brandt und Höth (1999).

McInnes, G. (1996) Atmospheric Emission Inventory Guidebook, A joint EMEP/CORINAIR Production, First Edition. EMEP Task Force on Emission Inventories, European Environment Agency, Copenhagen.

MEPC (1989) Exhaust emissions from ships, a global view, Marine Environment Protection Committee (MEPC) 29/18, London, cited after Brandt und Höth (1999).

Nellthorp, J., Sansom, T., Bickel, P., Doll, C. and Lindberg, G. (2001) Valuation Conventions for UNITE, UNITE (UNIfication of accounts and marginal costs for Transport Efficiency). Contained in Link et al. (2001), Pilot Accounts – Results for Germany and Switzerland. Funded by 5th Framework RTD Programme. ITS, University of Leeds, Leeds, April.

Rabl, A, Spadaro, J (1999) Environmental Damages and Costs: an analysis of Uncertainties. Paper accepted for publication in Environment International.

Richardson, S. (2000) Atmospheric Emission Inventory Guidebook, A joint EMEP/CORINAIR Production, Second Edition. European Environment Agency, Technical Report, No. 30, Copenhagen.

Simpson, D, Olendrzynski, K, Semb, A, Storen, E, Unger, S (1997) Photochemical oxidant modelling in Europe: multi-annual modelling and source-receptor relationships. EMEP/MSC-W Report 3/97, Oslo.

Trukenmüller, A, Friedrich, R (1995) Die Abbildung der großräumigen Verteilung, chemischen Umwandlung und Deposition von Luftschadstoffen mit dem Trajektorienmodell WTM, in: Jahresbericht 1995, ALS Universität Stuttgart, pp. 93 - 108.

Vestreng, V. (2000) Tables of 1998 sector emissions in the ECE region. Pollutants: sulphur, nitrogen oxides, ammonia, NMVOCs, carbon monoxide, methane, carbon dioxide. Co-operative programme for monitoring and evaluation of the long range transmission of air pollutants in Europe (EMEP). http://projects.dnmi.no/~emep/emis_tables/ stab1.html, Last Update: 9/2000, Status from: 3/2001

Vestreng, V. and Støren, E. (2000) Analysis of UNECE/EMEP Emission Data, MSC-W Status Report 2000. Co-operative programme for monitoring and evaluation of long range transmission of air pollutants in Europe (EMEP), EMEP MSC-W Note, 1/00, Blindern, Norway.

# Glossary

| CORINAIR | Programme to establish an inventory of emissions of air pollutants in Europe. It was initiated by the European Environment Agency Task Force and was part of CORINE (COoRdination d’Information Environmentale) work programme set up by the European Council of Ministers in 1985. End of 1994 the EEA’s European Topic Centre on Air Emissions (ETC/AEM) took over the CORINAIR programme. |
| --- | --- |
| GDP | (= Gross Domestic Product). The GDP is the sum of all goods and services produced within a country and a year. GDP per capita can be regarded as the relative economic power of a country per inhabitant. |
| Impact Pathway Approach (IPA) | Methodology for externality quantification developed in the ExternE project series. It follows the chain of causal relationships from pollutant emission via dispersion (including chemical transformation processes), leading to changes in ambient air concentrations from which impacts can be quantified using exposure-response functions. Damages are then calculated using monetary values based on the WTP approach. |
| NUTS | Nomenclature of territorial units for statistics; level 0 = countries, level III = départements, Kreise, etc. (depending on country considered). |
| PPP | PPP means purchasing power parity. PPPs are the rates of currency conversions which equalise the purchasing power of different countries. This means that a given sum of money, when converted into different currencies at the PPP rates, will buy the same basket of goods and services in all countries. In particular, PPPs are applied if figures for specific products or branches shall be expressed in foreign currency (for example in ECU or in US $) because in these cases the use of official exchange rates is not appropriate. |
| Primary particles | Particles, that are directly emitted. |
| Secondary particles | Particles, such as nitrates and sulphates, that are formed in the atmosphere through atmospheric chemical reactions. |
| WTP | Willingness to pay: The explicit or implicit willingness-to-pay for a good, reflecting the individual’s preferences. For example the WTP for higher safety. |
| YOLL | Year of life lost |

# Abbreviations

| CH4 | Methane (greenhouse gas) |
| --- | --- |
| CO2 | Carbon dioxide (greenhouse gas) |
| COI | Cost of illness |
| dB(A) | Decibel, weighted with the A-filter. Logarithmic unit of sound pressure level. |
| EMEP | European Monitoring and Evaluation Programme |
| GDP | Gross Domestic Product |
| GIS | Geographical Information System |
| GWP | Global warming potential |
| kWh | Kilowatt hour |
| LAeq | Energy equivalent noise level |
| LTO | Landing and take-off cycle |
| mill. | Million |
| MWh | Megawatt hour |
| N2O | Nitrous oxide (greenhouse gas) |
| n.a. | No data available |
| NMVOC | Non-methane volatile organic compounds |
| NOx | Nitrogen oxides (mix of NO and NO2) |
| NUTS | Nomenclature of territorial units for statistics; level 0 = countries, level III = départements, Kreise, etc. (depending on country considered) |
| PM10 | Fine particles with a diameter of 10 µm and less |
| PM2.5 | Fine particles with a diameter of 2.5 µm and less |
| PPP | Purchasing power parity |
| PT | Public transport |
| SO2 | Sulphur dioxide |
| UPT | Urban public transport |
| VOC | Volatile organic compounds |
| WTP | Willingness to pay |
| YOLL | Years of life lost |