- COMPETITIVE AND SUSTAINABLE GROWTH

(GROWTH)

PROGRAMME

Deliverable 14 Annex 3 The Pilot Accounts for Estonia

- Final Draft

- 16 September 2002

Authors: Tarmo Loog, Kaur Lass (Entec Ltd), Mall Villemi (Tallinn Technical University), Veli Himanen, Tiina Idstrom, (JP-Transplan Ltd),

with contributions from partners

Contract: 1999-AM.11157 Project Co-ordinator: ITS, University of Leeds Funded by the European Commission 5th Framework – Transport RTD

UNITE Partner Organisations ITS/UNIVLEEDS (UK), DIW (De), NEI (Nl), CES/KUL (Be), TIS.PT (Pt), IWW/UNIKARL (De), VTI (Se), IER/USTUTT (De), CERAS/ENPC (Fr), HERRY (Au), EIET/ULPGC (Es), ISIS (It), STRATEC (Be), SYSTEMA (Gr), JP-TRANSPLAN (Fi), VATT (Fi), ECOPLAN (Ch), INFRAS (Ch), EKONO (Fi), EKI (Se)

- UNITE

1999-AM.11157

- UNIfication of accounts and marginal costs for Transport Efficiency

- Pilot Accounts – Results for Estonia

- This document should be referenced as:

Tarmo Loog, Kaur Lass (Entec Ltd), Mall Villemi (Tallinn Technical University), Veli Himanen, Tiina Idstrom, (JP-Transplan Ltd). UNITE (UNIfication of accounts and marginal costs for Transport Efficiency) Deliverable 14. Working Funded by 5th Framework RTD Programme. ITS, University of Leeds, Leeds, September 2002.

16 September 2002

Final Draft

Authors: as above

- PROJECT  INFORMATION

- Contract no: 1999-AM.11157:

- UNIfication of accounts and marginal costs for Transport Efficiency

- Website: www.its.leeds.ac.uk/unite

- Commissioned by: European Commission – DG TREN; Fifth Framework Programme

- Lead Partner:	Institute for Transport Studies, University of Leeds (UK)

Partners: 	ITS/UNIVLEEDS (UK), DIW (De), NEI (Nl), CES/KUL (Be), TIS.PT (Pt), IWW/UNIKARL (De), VTI (Se), IER/USTUTT (De), CERAS/ENPC (Fr), HERRY (Au), EIET/ULPGC (Es), ISIS (It), STRATEC (Be), SYSTEMA (Gr), JP-TRANSPLAN (Fi), VATT (Fi), ECOPLAN (Ch), INFRAS (Ch), EKONO (Fi), EKI (Se)

- DOCUMENT  CONTROL  INFORMATION

- Status: 	Accepted

Distribution:	UNITE partners

Availability:	Public (only once status above is “Accepted” as part of Deliverable 14)

Filename:	UNITE D14 Annex 3

Quality assurance: 	ITS

Co-ordinator’s review:

Signed: 	Date:

Table of contents

1	Introduction	5

1.1	Study context and objectives of this annex report	5

1.2	The accounts approach of UNITE	5

1.2.1	Aims of the pilot accounts	5

1.2.2	Core, supplementary and excluded data in the pilot accounts	6

1.2.3	The six UNITE pilot account cost categories	6

1.2.4	The transport modes covered in the pilot accounts	8

1.3	Results presentation and guidelines for interpretation	9

1.4	The structure of this annex report	9

2	Description of input data	10

2.1	Overview on the Estonian transport sector and basic input data	10

2.1.1 Basic economic and structural data	10

2.1.2 Basic Transport Data	10

2.2	Infrastructure costs	13

2.3	Supplier operating costs	14

2.4	Delay costs due to congestion	14

2.5	Accident costs	15

2.6	Environmental costs	17

2.6.1	Environmental costs	17

2.6.2	Summary	21

2.7	Taxes, charges, subsidies	22

3	Methodological issues	23

3.1	Infrastructure costs	23

3.1.1	Road	23

3.1.2	Rail	23

3.1.3	Public transport infrastructure – tram, trolley buses	23

3.1.4	Aviation infrastructure	24

3.1.5	Inland waterways and harbours	24

3.1.6	Maritime shipping – coastal channels, harbours and icebreakers	24

3.2	Methodology for estimating supplier operating costs	24

3.2.1	Rail carriers (Estonian Railways Ltd and Edelaraudtee Ltd)	24

3.2.3	Public transport	25

3.3	Methodology for estimating delay costs due to congestion	25

3.4	Methodology for estimating accident costs	25

3.5	Methodology for estimating environmental costs	27

3.5.1	Air pollution	28

3.5.2	Global warming	33

3.5.3	Noise	34

3.5.4	Costs due to impacts on nature, landscape, soil and groundwater	34

3.5.5	Costs due to nuclear risks	34

3.5.6	Methodology for 1996 and for the forecast to 2005	34

3.6	Methodology for estimating taxes, charges and subsidies	35

3.6.1	General issues	35

3.6.2	Methodological issues per mode	35

4	Results	38

4.1	Infrastructure costs	38

4.1.1	Road transport	38

4.1.2	Railways	38

4.1.4	Other public transport infrastructure (tram, trolley buses)	38

4.1.5	Aviation infrastructure	38

4.1.6	Waterborne transport infrastructure	38

4.2	Supplier operating costs	39

4.2.1	National rail carriers – Estonian Railways Ltd and Edelaraudtee Ltd	39

4.2.2	Public transport	39

4.3	Delay costs due to congestion	39

4.4	Accident costs	39

4.4.1	Results for 1998- total costs by category	39

4.4.2	Allocation of total costs to modes and types of infrastructure	40

4.4.3	Average costs in 1998	40

4.5 Environmental Costs	41

4.5.1	Results for 1998	41

4.5.2	Account years 1996 and 2005	44

4.6	Taxes, charges, subsidies	46

4.6.1	Road transport	46

4.6.2	Rail transport	46

4.6.4	Public transport excluding rail	47

4.6.5	Aviation	47

4.6.6	Waterborne transport	48

5	Pilot accounts for Estonia	49

5.1	Road transport	51

5.2	Rail transport	58

5.3	Public transport	62

5.4	Maritime shipping	66

Abbreviations used in data tables	72

List of Tables

Table 1. The modes, network differentiation, transport means and user breakdown in the Estonian pilot accounts	8

Table 2. Basic indicators for Estonia 1996 and 1998	10

Table 3. Road mileage driven in Estonia	11

Table 4. Passengers and freight in road transport in Estonia	11

Table 5. Hauled train-kilometres in 1996 and 1998 in Estonia	12

Table 6. Passenger and freight volumes and kilometres in Estonia´s railways in 1996 and 1998	12

Table 7. Transport volumes of trolley buses and tramways in Estonia in 1996 and 1998	12

Table 8. Passengers in scheduled and charter flights in Estonia in 1996 and 1998	13

Table 9. Estonian maritime transport in 1996 and 1998	13

Table 10. Sources and quality of input data for estimating infrastructure costs	14

Table 11. Sources and quality of input data for estimating supplier operating costs	14

Table 12. Total number of accidents by mode in Estonia in 1996 and 1998	15

Table 13. Basic input data for estimating accident costs: Total number of injuries and fatalities by mode in Estonia in 1998	15

Table 14. Source and quality of data for estimating accident costs by transport mode	16

Table 15. Source and quality of data for estimating accident costs by cost category	16

Table 16. Environmental data in the EcoSense database	17

Table 17. Direct transport emissions in Estonia in 1998	19

Table 18. Emission factors caused by fuel production processes	20

Table 19. Source and quality of input data for estimating environmental costs	21

Table 20. Input data for taxes, charges and subsidies	22

Table 21. Average costs for medical treatment per degree of injury in Estonia 1998	26

Table 22. Police costs in road accidents in Estonia in 1998	27

Table 23. Health and environmental effects included in the analysis of air pollution costs	30

Table 24. Quantification of human health impacts due to air pollution	31

Table 25. Monetary values (factor costs) for valuing health impacts for Estonia	32

Table 26. Total internal and external accident costs in Estonia 1998 by cost category and mode	40

Table 27. Total internal and external accident costs in Estonia 1998 by mode	40

Table 28. Average external accident costs in Estonia 1998	41

Table 29. Environmental costs for Estonia 1998	42

Table 30. Environmental costs for road transport in Estonia in 1998 (excluding noise costs, nature, landscape, soil and water pollution) by vehicle type	42

Table 31. Environmental costs (direct and indirect emissions) by vehicle and road type for road transport in Estonia in 1998 (excluding noise costs, nature, landscape, soil and water pollution)	43

Table 32. Average environmental costs for Estonia 1998	43

Table 33. Environmental costs (excluding noise costs, nature, landscape, soil and water pollution) for Estonia in 1996	45

Table 34. Environmental costs for Estonia in 2005	46

Table 35. Road Transport Revenues in Estonia 1996 and 1998	46

Table 36. Revenues from tariffs in rail transport in 1998	47

Table 37. Tariff revenues and subsidies of public transport (excl. rail)  in 1996 and 1998	47

Table 38. Revenues and subsidies of the Estonian government in aviation in 1998	48

Table 39. Basic indicators for Estonia in 1996 and 1998	49

Table 40. Basic transport related indicators for Estonia in 1998 per mode	50

Table 41. Estonian road account for 1996, 1998 and 2005  - € million at 1998 prices -	51

Table 42. Fully allocated costs of road transport per vehicle km in Estonia in 1998 - € per 1000 vehicle km at 1998 prices	54

Table 43. Total costs of road transport in Estonia in 1998 by vehicle category - € million at 1998 prices	55

Table 44. Total costs of road transport in Estonian inter-urban roads in 1998 - € million at 1998 prices	56

Table 45. Total costs of road transport in Estonian urban roads in 1998 - € million at 1998 prices	57

Table 46. Estonian rail account for Estonian Railway Administration, Estonian Railways Ltd and Edelaraudtee Ltd in 1996 and1998 - € million at 1998 prices -	58

Table 47. Fully allocated costs of rail transport per train km in Estonia in 1998: Estonian Railway Administration, Estonian Railways Ltd and Edelaraudtee Ltd - € per train-km at 1998 prices	60

Table 48. Total costs of rail transport in Estonia in 1998: Estonian Railway Administration, Estonian Railways Ltd and Edelaraudtee Ltd - € million at 1998 prices	61

Table 49. Estonian account for public transport in 1996 and 1998 - € million at 1998 prices	62

Table 50. Estonian air transport account for 1996 and 1998 - € million at 1998 prices	64

Table 51. Estonian maritime shipping transport account for 1996 and 1998 - € million at 1998 prices	66

# 1	Introduction

## 1.1	Study context and objectives of this annex report

This annex report contains the full version of the Estonian pilot account developed within the UNITE project. The general and detailed discussion of the accounts approach was presented in Link et al. (2000) and will be only summarised in this document. This report discusses methodologies only in so far as they are necessary background information for understanding the results and describes rather the application of methodology to the Estonian case. Furthermore, in addition to the core accounts for 1998 this annex report also presents the results for 1996 and a forecast for 2005. This report was produced jointly by Entec Ltd and Tallinn Technical University (data gathering) and JP-Transplan Ltd (report writing).

In order to put this annex report into the context of the UNITE project a summary of the aims and research areas of UNITE is given here. The UNITE project endeavours to provide accurate information about the costs, benefits and revenues of all transport modes including the underlying economic, financial, environmental and social factors. To achieve this goal, three main areas of research are carried out, known as “transport accounts”, “marginal costs” and “integration of approaches”. This report belongs to the research area “transport accounts”. For a better understanding of the results presented here it has to be borne in mind that the UNITE project distinguishes between ideal accounts on the one hand and the pilot accounts on the other hand. The ideal accounts reflect the perfect situation with the utmost disaggregation, showing factors such as the time and location and duration of individual trips, all the relevant economic data as well as the individuals´ response to possible policy or infrastructure changes. The pilot accounts are the actual, feasible accounts given the available data for the 18 countries that UNITE covers. They can be used to assess the costs and revenues of transport per transport mode. The costs are reported and documented at the current level of transport demand for the reference years 1996, 1998 and for the forecast year 2005. Reported transport costs are allocated to user groups, where possible without arbitrary allocation methods.

## 1.2	The accounts approach of UNITE

### 1.2.1	Aims of the pilot accounts

The pilot accounts attempt to show the general relationship between costs of transport and the revenues from transport pricing and charging in the country studied (for a detailed discussion on the aims and role of the pilot accounts see Link et al. (2000)). It should be stressed that the accounts are aimed at providing the methodological and the empirical basis for in-depth policy analysis and monitoring rather than serving as a guide for immediate policy actions such as setting higher/lower prices and charges or opening up/shutting-down transport services/links in order to achieve cost coverage. The pilot Accounts are defined as follows:

The pilot accounts compare social costs and / charges on a national level in order to monitor the development of costs, the financial taxes balance and the structure and level of prices. Accounts can therefore be seen as monitoring and strategic instruments at the same time. They have to consider the country-specific situation and the institutional frameworks.

The pilot accounts show the level of costs and charges as they were in 1998 (and 1996 respectively) and provide a workable methodological framework to enable regular updating of transport accounts. Furthermore, an extrapolation for 2005 is given. The choices of additional accounting years (1996 and 2005) were motivated by the need to show a comparison between years and to give a good indication of trends in transport for the near future. Also, the inclusion of 1996 provides a double check on any major statistical abnormalities that may occur in one year, for example very high infrastructure cost due to tunnelling operations or higher than average accident costs because of major accidents occurring in 1998. Note, however, that the core year of the pilot accounts is 1998.

### 1.2.2	Core, supplementary and excluded data in the pilot accounts

The pilot accounts have been divided into the classes “core data” and “supplementary data”. Core data is the data necessary to do a full basic review of the country accounts. Core data is data within the following categories; infrastructure costs; the external costs of transport accidents; the environmental categories air pollution, noise and global warming and supplier operating costs. Transport revenues and taxes are also documented here. Supplementary data falls into two categories. Firstly, for several cost categories being evaluated there is no standard methodology for the valuation of effects. An example of this is the valuation of loss of biodiversity due to transport infrastructure. Even though a valuation method has been developed for the UNITE Pilot Accounts, we feel that the level of uncertainty (due to lack of comparative studies) is high enough to warrant the information to be classified outside of the core data where efficient and well tried valuation methods have been utilised. Secondly, some costs which can be estimated and valuated are borne by the transport users themselves (for example delay costs). These costs and the methods used to valuate them present valuable further information to the reader, but can not be considered to be part of the overall costs of transport as defined by UNITE. Supplementary data is data within the following categories: congestion costs; the internal part of accident costs including the risk value; and, the environmental costs risk due to the provision of nuclear power and the costs associated with nature and landscape, soil and water pollution. Subsidies also fall within the category supplementary data.

### 1.2.3	The six UNITE pilot account cost categories

Data for the pilot accounts are collected within six cost and revenue categories that are described in Link et al. (2000) and are summarised in the following section.

- Infrastructure costs

For the pilot accounts, data for the assessment of infrastructure costs are structured to show the capital costs of transport infrastructure (including new investments and the replacement of assets) and the running costs of transport infrastructure (maintenance, operation and administration) for all modes of transport studied. As far as possible with current methodological knowledge, infrastructure costs are allocated to user groups and types of transport. Where it is possible to quantify the share of joint costs they are separated out and are not allocated.

- Supplier operating costs

All monetary costs incurred by transport operators for the provision of transport services are documented in the category supplier operating costs. Ideally, the data is structured to show what costs are incurred for vehicles, for personnel and for administration. However, this depends on data availability and will differ from country to country. Since collecting and supplementing this data for all modes is extremely time consuming, the UNITE project focuses on estimating supplier operating costs only for those modes where significant state intervention and subsidisation is present. The main emphasis in this category is thus on rail transport and other public transport (tram, metro, bus). Whether other modes also have to be covered depends on the degree of state intervention in the respective countries. The corresponding revenues from the users of transport are included when supplier operating costs are estimated. The difference between such costs and revenues is the net public sector contribution (economic subsidy).

- Delay costs due to congestion

In the European Commission’s White Paper “Fair payment for infrastructure use” (1998), costs caused by transport delays, accidents and environmental effects of transport are estimated to be the three major causes of external transport costs. In the category congestion costs, the costs of delay and delay-caused additional operating costs are estimated. Note, within the pilot accounts the term congestion costs is used even though delay costs only were calculated. The name of the cost category “user costs” (Link et al. 2000, Doll et al. 2000) signifies that we are aware that this category does not cover all aspects of costs related to congestion. The estimation of delay costs as defined here is carried out for all transport modes, provided data is available. This data is classified as supplementary data because the bulk of these costs are borne by transport users as a whole.

- Accident costs

The loss of lives and the reduction of health and prosperity through transport accidents are of major concern to all countries and to the European Commission. In this section of the accounts, the health related accident costs are calculated by assessing the loss of production, the risk value and the medical and non-medical rehabilitation of accident victims. Where the available data basis allows, the damage to property and the administrative costs of accidents are also considered. The external part of accident costs (defined in this report as accident costs imposed by transport users on the whole society) is included in the core section of the accounts. The internal part of accident costs - costs borne by transport users - are treated as supplementary costs.

- Environmental costs

A wide range of transport related environmental impacts and effects, presently being hotly debated in all countries, is considered in this section of the accounts. Included in this cost category are: air pollution, global warming, noise, changes to nature and landscape, soil and water pollution and nuclear risks. The valuation of these environmental effects is carried out for all transport modes, provided adequate data is available.

- Taxes, charges and subsidies

In this section, the level of charging and taxation for the transport sector is documented for each mode of transport. Wherever possible, the revenues from taxes and charges are shown for fixed taxes and charges and variable ones. This information plays an important part in the ongoing discussions about the level of taxation between transport modes and countries. The comparison between taxes levied and the costs of infrastructure provision and use accrued per mode is central to this debate and holds a high level of political significance. Environmental taxes that apply to transportation are separately considered in this section. Taxes such as VAT that do not differ from the standard rate of indirect taxes are excluded from this study.

A further part in this area is reporting on subsidies. The need to maintain free and undistorted competition is recognised as being one of the basic principles upon which the EU is built. State aid or subsidies are considered to distort free competition and eventually cause inefficiency. Subsidies to the transport sector provided by the member states are not exempted from the general provisions on state aid set out in the Amsterdam Treaty. There are, however, special provisions set out in the treaty in order to promote a common transport policy for the transport sectors of the member states (Treaty establishing the European Community : Articles 70 – 80). The subsidies of the transport sector are considered in this section. It should be noted that a complete reporting on subsidies would require an extremely time-consuming analyses of public budget expenditures at all administrative levels. Furthermore, the subsidies reported in the pilot accounts refer mainly to direct subsidies, e.g. monetary payments from the state to economic subjects. In the Estonian accounts also municipal subsidies for public transport are estimated. Indirect subsidies (e. g. tax reductions and tax exemptions that cause lower revenues of state budgets) are quantified where possible.

### 1.2.4	The transport modes covered in the pilot accounts

The modes covered in UNITE are road, rail, other public transport (tram, trolley buses), aviation, inland waterway navigation and maritime shipping. The level of disaggregation into types of networks and nodes, means of transport and user groups depends on data availability and relevance per country. Table 1 summarises this disaggregation for the Estonian pilot account. Section 2.1 provides in addition some indicators per mode in order to show the importance and relevance of each mode in the Estonian transport system.

Table 1. The modes, network differentiation, transport means and user breakdown in the Estonian pilot accounts

| Transport modes | Network and institutional differentiation | Means and user breakdown |
| --- | --- | --- |
| Road | Motorways State roads Regional roads Urban roads | Mopeds, Motorcycles Passenger cars Buses Light goods vehicles Heavy goods vehicles rigid goods vehicles articulated vehicles Agricultural vehicles Trailers |
| Rail | National Railways Other Railways | Passenger transport domestic passenger transport international passenger transport Freight transport |
| Other public transport | – | Tram, trolley buses |
| Aviation | Airports Air transport | – |
| Inland waterway shipping | Inland waterways Inland waterways harbours | Used mainly for leisure purposes |
| Maritime shipping | Seaports Harbours for domestic ferry boats | Cargo ships Passenger ships Domestic ferry boats |
| Source: JP-Transplan Ltd |  |  |

## 1.3	Results presentation and guidelines for interpretation

The goal of the data collection and estimation of costs and revenues in each category was a level of disaggregation that shows the pertinent costs and charges of the relevant transport mode. From the available, but very heterogeneous input data and results, a structure for reporting transport accounts has been developed. All results are documented separately for each cost category and are summarised in modal accounts covering all cost and revenue categories. Additionally, a set of data needed as basic data for all cost categories was collected to ensure that commonly used data have consistency between the cost categories. Minor discrepancies in the basic data used between cost categories are due to the fact that the level of disaggregation in the input data required for each cost category differed. However every effort was used to consolidate the basic data used by partners to ensure consistent results for all cost categories.

The categories studied present a comprehensive estimation of transport costs and revenues. They are however, not a total estimation of transport costs. Each cost category could include data in further areas and a definite border had to be drawn around the data to be collected for this project. For example, the estimation of environmental costs does not include the environmental costs incurred during the manufacturing of vehicles, even though these costs could be estimated. These costs would be included in an ideal account, but lie outside the scope of the pilot accounts. Further transport costs categories such as vibration as attributing to environmental costs are not evaluated because no acceptable valuation method has been developed.

It should be noted that due to the separation into core and supplementary data with different levels of uncertainty and with different types (costs borne by transport users themselves versus external costs) care is needed when comparing costs and revenues.

## 1.4	The structure of this annex report

This report contains four major parts. Chapter 2 briefly explains firstly the organisation of the Estonian transport sector and the importance of each mode in order to provide some background information for the interpretation of the pilot accounts. Secondly, the input data that was used in the accounts is described here. The main methodological issues which have arisen during the elaboration of the accounts for Estonia are discussed in chapter 3. The results are presented and discussed in chapter 4. The descriptions in these chapters are organised along the categories infrastructure costs, supplier operating costs, congestion costs, accident costs, environmental costs, and taxes, charges and subsidies. Chapter 5 presents the summary tables on the Estonian pilot accounts.

# 2	Description of input data

## 2.1	Overview on the Estonian transport sector and basic input data

This section aims at providing some basic information on the features of the Estonian transport sector, the organisational structure and the importance of transport modes as far as necessary for understanding and interpreting the pilot accounts.

### 2.1.1 Basic economic and structural data

Estonia is a small country with rather low GDP per capita - if compared to the EU countries - but the annual growth in GDP has recently been quite high and the inflation was already stabilising in 1998 (see Table 2).

Table 2. Basic indicators for Estonia 1996 and 1998

|  | unit | 1996 | 1998 |
| --- | --- | --- | --- |
| Land area | sqkm | 45 227 | 45 227 |
| Population | 1 000 | 1 469 | 1 449 |
| Population density | inhabitants/sqkm | 32 | 32 |
| Population employed | % | 61.3 | 58.1 |
| GDP per capita1) | € | : | 9 193 |
| GDP growth rate (change to previous year) | % | 4.0 | 5.5 |
| Annual inflation | % | 23.1 | 8.2 |
| 1) OECD GDP per Capita, PPP-adjusted (Nelthorp et al, 2001). Sources: Statistical yearbook for Estonia 1999, 2000, COWI and TTU (2001). |  |  |  |

### 2.1.2 Basic Transport Data

- Most of the transport data used for the calculation of the different costs and revenues is summarised in the following sections 2.2 to 2.7. Next, we concentrate on the basic information about transport volumes in the three UNITE years. These figures are used to derive cost rates per unit of performance in the chapters with the results for each cost category (chapter 4).

- Estonia´s total road network had a length of 43 825 km in 1998. Passenger cars made 77 per cent of mileage in road transport (see Table 3). The figures are differentiated according to the road infrastructure type. For this mode, detailed information is available about past, current and future transport volumes. According to Table 3 it is only passenger car kilometres which are expected to grow between the UNITE base year 1998 and the forecast year 2005. In total 12 million tons were transported on Estonian road network in 1998 (see Table 4). It has to be noticed that because of lacking surveys there is great uncertainty about the amount of car trips and their lengths. Also considerably lower figures have been presented by MTC (2000). The figures used here are based on data provided by Tallinn Technical University to Entec Ltd and they correspond well with data in DHV and LT Consultants (1999).

Table 3. Road mileage driven in Estonia

|  | Million vehicle-km |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | All Roads | Motorways | State roads | Regional roads | Urban roads |
| 1996 |  |  |  |  |  |
| Mopeds, motorcycles | : | : | : | : | : |
| Passenger cars | 4 339 | 151 | 2173 | 449 | 1 566 |
| Buses | 159 | 7 | 94 | 7 | 51 |
| Light goods vehicles | 884 | 36 | 574 | 140 | 134 |
| Heavy goods vehicles | 540 | 27 | 337 | 94 | 82 |
| Total | 5 922 | 221 | 3 178 | 690 | 1 833 |
| 1998 |  |  |  |  |  |
| Mopeds, motorcycles | : | : | : | : | : |
| Passenger cars | 4 763 | 188 | 2 421 | 386 | 1 768 |
| Buses | 169 | 8 | 100 | 6 | 55 |
| Light goods vehicles | 840 | 37 | 538 | 115 | 150 |
| Heavy goods vehicles | 510 | 28 | 317 | 76 | 89 |
| Total | 6 282 | 261 | 3 376 | 583 | 2 062 |
| 2005 |  |  |  |  |  |
| Mopeds, motorcycles | : | : | : | : | : |
| Passenger cars | 5 860 | 290 | 2 800 | 460 | 2 310 |
| Buses | 162 | 9 | 91 | 6 | 56 |
| Light goods vehicles | 710 | 42 | 428 | 95 | 145 |
| Heavy goods vehicles | 425 | 33 | 247 | 60 | 85 |
| Total | 7 157 | 374 | 3566 | 621 | 2 596 |
| Source: Tallinn Technical University |  |  |  |  |  |

Table 4. Passengers and freight in road transport in Estonia

|  | Unit | 1996 | 1998 |
| --- | --- | --- | --- |
| Passengers | Million passenger-km1) | 9 842 | 10 760 |
|  | Millions2) | 203 | 189 |
| Freight | Million ton-km | : | 1 230 |
|  | Million tons | 11 | 12 |
| 1) Private cars, buses, and taxis. 2) Buses and taxis only. Source: MTC (2000), DHV and LT Consultants (1999), JP-Transplan Ltd. |  |  |  |

- In 1996 Estonian rail network was managed and rail transport organised by a state enterprise, called Estonian Railways. It was reorganised in 1977 and divided into two state owned companies: Estonian Railways Ltd and Edelaraudtee Ltd. Later on, new operators – 17 in September 1999 - did receive a licence to manage the railway infrastructure and to provide rail transport services. The length of the track network has been decreasing but the remaining network – 800 km of public and some 600 km of private track in 1998 - is supposed to stay in use. Railway passenger transport has been decreasing, but freight transport increasing from 1996 to 1998 (Table 5 and Table 6).

Table 5. Hauled train-kilometres in 1996 and 1998 in Estonia

|  | Million kilometres |  |
| --- | --- | --- |
|  | 1996 | 1998 |
| Passenger trains Freight trains | 6.0 2.7 | 2.7 4.0 |
| Total | 8.7 | 6.7 |
| Source: Entec Ltd |  |  |

Table 6. Passenger and freight volumes and kilometres in Estonia´s railways in 1996 and 1998

|  |  | Unit | 1996 | 1998 |
| --- | --- | --- | --- | --- |
| Passenger | Domestic | Million | 6.7 | 6.7 |
|  | International | Million | 0.5 | 0.3 |
|  | Total | Million | 6.7 | 6.7 |
|  | Domestic | Million passenger-km | 231 | 189 |
|  | International | Million passenger-km | 78 | 48 |
|  | Total | Million passenger-km | 309 | 237 |
| Freight1) | Domestic2) | Million tons | 13.5 | 13.4 |
|  | Transit | Million tons | 11.3 | 18.6 |
|  | Total | Million tons | 24.8 | 32.0 |
|  | Domestic2) | Million ton-km | 1 213 | 1 272 |
|  | Transit | Million ton-km | 2 681 | 4 516 |
|  | Total | Million ton km | 3 894 | 5 788 |
| 1) Public railways only. - 2) Includes also Estonian export and import. Source: MTC (2000) |  |  |  |  |

- As mentioned in section 1.2, the UNITE mode category "Urban public transport" covers modes which are normally contained in other mode categories: Buses are part of road transport and urban rail services are included in the mode rail transport. Therefore, the table below only contains figures on trolley buses and trams in Tallinn. These  modes do not exist in other Estonian cities. The length of tramways in 1998 was 39 km and the length of trolley bus lines 142 km. Against this background, attention should be paid when the results between the different cost categories are interpreted for the mode urban public transport. Passengers travelled 268  million kilometres by tram and trolley buses in 1998 (see Table 7).

Table 7. Transport volumes of trolley buses and tramways in Estonia in 1996 and 1998

| Mode | Unit | 1996 | 1998 |
| --- | --- | --- | --- |
| Trolley buses | Million passengers | 31.8 | 38.9 |
| Tram | Million passengers | 35.6 | 30.6 |
| Total | Million passengers | 67.4 | 69.5 |
| Trolley buses | Million passenger-km | 146 | 179 |
| Tram | Million passenger-km | 103 | 89 |
| Total | Million passenger-km | 249 | 268 |
| Source: Tallinn City Government Statistics |  |  |  |

The infrastructure services in the airports are produced (since 1998) by airport companies – like Tallinn Airport Ltd - and the air traffic control is provided by Estonian Air Navigation Services Ltd. Domestic air transport is - because of the size of Estonia - quite modest. The number of air passengers has increased rapidly between 1996 and 1998 (see ).

Table 8. Passengers in scheduled and charter flights in Estonia in 1996 and 1998

|  | Unit | 1996 | 1998 |
| --- | --- | --- | --- |
| Domestic flights | Thousand passengers | : | 19 |
| International flights | Million passengers | : | 0.56 |
| Total | Million passengers | 0.44 | 0.58 |
| Source: MTC (2000) |  |  |  |

Estonia has 320 km of navigable inland waterways, but they are used mainly for leisure purposes with only a minimal amount of freight and passenger transport.

Maritime shipping is important for Estonia. There exist 5 seaports with channel depth of 9 m or more. For wintertime operations the aid of icebreakers is needed. Transit traffic – mainly to/from Russia – was 18.4 million tonnes in 1998 and formed the major part of maritime freight. International passenger traffic had main destinations in Finland and Sweden (Table 9).

Table 9. Estonian maritime transport in 1996 and 1998

|  | Unit | 1996 | 1998 |
| --- | --- | --- | --- |
| Freight | Million tonnes | 17,7 | 27,4 |
| Passengers | Million passengers | 4.5 | 5.6 |
| Source: Statistical Office of Estonia, Yearbook |  |  |  |

## 2.2	Infrastructure costs

The main required input data was a long and disaggregated investment time series per mode, needed for the perpetual inventory model. This would then be used to calculate the value of the capital stock and the capital costs. Unfortunately this kind of data could not be obtained in Estonia because of the administrative changes related to the re-establishment of independence in 1991. However, data for running costs was available from 1998 and partly also from 1996. The input data and an evaluation of their quality are summarised in Table 10.

Table 10. Sources and quality of input data for estimating infrastructure costs

|  | Input data | Level of disaggregation | Quality of data, level of uncertainty |
| --- | --- | --- | --- |
| Road | Financial and infrastructure data from DHV and LT Consultants (1999) and MTC (2000). | Current investments and running costs for inter-city roads and urban roads. | The data is in general of good quality, but time series are too short for capital stock calculations. |
| Rail | Infrastructure and financial data from DHV and LT Consultants (1999) and MTC (2000). | Tracks and stations together. Current investments and running costs known. | The overall quality of data is good, but time series too short for capital stock calculations. |
| Public Transport | Information provided by the Tallinn Technical University. | Tram and trolley buses. Buses are included in the road section as well as taxis and rental cars. Only total costs were available. No capital stock could be estimated neither running costs. | Limited use because of missing disaggregation and short time series. |
| Aviation | Data for infrastructure costs from DHV and LT Consultants (1999) and MTC (2000). | All 7 airports included. Running costs presented. | Good quality data for investment and running costs but time series too short for capital stock calculations. |
| Inland waterway | Inland waterways used mainly for leisure transport. | No financial data available. | No financial data available. |
| Maritime shipping | All waterways information from DHV and LT Consultants (1999) and MTC (2000). | Current investments and running costs were known for marked channels and domestic ferry harbours. No data for other sea harbours or icebreakers. | Good quality investment and running cost data. Time series too short for capital stock calculations. |
| Source: JP-Transplan Ltd |  |  |  |

## 2.3	Supplier operating costs

As stated in the previous chapter, supplier operating costs are calculated only for public transport and rail services. Table 11 summarises the input data used.

Table 11. Sources and quality of input data for estimating supplier operating costs

|  | Input data | Level of disaggregation | Quality of data, level of uncertainty |
| --- | --- | --- | --- |
| Rail | All data from DHV and LT Consultants (1999) and MTC (2000). | For two railway companies quite detailed costs. | Good data quality. |
| Public transport | All data from DHV and LT Consultants (1999), MTC (2000) and Tallinn City Government. | Disaggregation between buses and trams/trolley buses. | Good quality data. |
| Source: JP-Transplan Ltd |  |  |  |

## 2.4	Delay costs due to congestion

There are very few congestion problems in Estonia outside Tallinn. There are no data on possible delays.

## 2.5	Accident costs

Input data for estimating accident costs refer firstly to input data per transport mode such as number of accidents, number of injuries, fatalities and material damages. This input data is shown in Table 12 and Table 13, and remarks on their quality are given in Table 14. Secondly, accident costs have five components: medical costs; material damage costs; administrative costs; costs due to production losses; and, the costs of suffering and grief (risk value). The input data for these cost components refer to valuations and unit costs and are summarised in Table 15.

A few remarks seem to be necessary for a proper interpretation of Table 12 and Table 13. For the calculation of medical costs and material damage in road transport, the specific problem of underreporting must be addressed. Here we have used a recent study of COWI and TTU (2001). Statistics reported for rail, aviation, inland waterways and maritime shipping can be considered to be correct. The estimation of material damage is made only for road transport (COWI and TTU, 2001).

Table 12. Total number of accidents by mode in Estonia in 1996 and 1998

| Mode | 1996 | 1998 |
| --- | --- | --- |
| Road | : | 20 187 |
| Rail | 51 | 44 |
| Aviation | 1 | 0 |
| Inland waterway1 | 0 | 2 |
| Maritime shipping2 | 0 | 4 |
| - The number of casualties in boat – motor and sailing boats - accidents in inland and coastal waterways - Shipwrecks in the Estonian territory |  |  |
| Source: COWI and TTU (2001), Railway Administration, Civil Aviation Administration and Maritime Board. |  |  |

Table 13. Basic input data for estimating accident costs: Total number of injuries and fatalities by mode in Estonia in 1998

| Mode | Injuries | Fatalities |
| --- | --- | --- |
| Road Rail | 1 990 23 | 284 21 |
| Aviation | 0 | 0 |
| Inland navigation 1) | 0 | 2 |
| Maritime shipping | 0 | 4 |
| 1) Includes boat traffic accidents both in inland and coastal waterways |  |  |
| Source: COWI and TTU (2001), Railway Administration, Civil Aviation Administration and Maritime Board. |  |  |

Table 14. Source and quality of data for estimating accident costs by transport mode

|  | Input data | Level of disaggregation | Quality of data, level of uncertainty |
| --- | --- | --- | --- |
| Road | Data from a recent study by COWI and Tallinn Technical University. | No disaggregation according to road type or vehicle gategory. | In principle good input data. |
| Rail | Number of accidents received from Railway Administration. | No disaggregation. | Good official statistics. No problems with underreporting. |
| Public Transport | see road transport | see road transport | see road transport |
| Air | Number of accidents from Civil Aviation Administration. | No disaggregation. | Good official statistics. No problems with underreporting. |
| Inland waterway | Number of fatalities from Maritime Board. | One total for inland and coastal boat accidents. | Official statistics but there can be problems with underreporting. Problems with allocating between motor and sailing boats and rowing boats. |
| Maritime Shipping | Number of shipwrecks in Estonian territory from Maritime Board. | No disaggregation. | Good official statistics. No problems with underreporting |

Table 15. Source and quality of data for estimating accident costs by cost category

|  | Input data | Level of disaggregation | Quality of data, level of uncertainty |
| --- | --- | --- | --- |
| Costs of medical treatment | Data from a recent study by COWI and Tallinn Technical University. | Number of fatalities and injuries, severity of injuries estimated. | Costs are based on information from one hospital and they do not include the costs in other medical organisations. Therefore a low approximation of total costs. |
| Valuation of administrative costs | Data from a recent study by COWI and Tallinn Technical University. | Costs of police per road accident. | Based on Danish practice. Therefore only indicative. |
| Valuation of material damage | Data from a recent study by COWI and Tallinn Technical University on road transport. No estimations for other modes. | No disaggregation. | A rough estimate for road including public transport, not transferable to other modes. |
| Production losses | Data for age distribution from a recent study by COWI and Tallinn Technical University. No data on the length of temporary absence from the labour market, therefore used available German values. Data on average income and consumption from Statistical Office of Estonia. It has been assumed that there are no extra costs covered by employers because of their employees accidents. | Number of fatalities and injuries, no disaggregation for severity class for injuries | A rough estimate. |
| Risk Value | UNITE standard values (Nellthorp et al.2001). It has been assumed that gratification and transfer payments do not exist. | Risk values for accident victims only. No risk value for relatives and friends. | Value is based on latest available studies and standardised for UNITE. |

## 2.6	Environmental costs

### 2.6.1	Environmental costs

The input data for the calculations was provided by Entec Ltd to  IER. These data were used for cost calculations based on the ExternE methodology with the EcoSense computer model for airborne pollutants and with shadow values for greenhouse gas emissions. Noise costs were not estimated.

#### Air pollution

The emission calculations are based on fuel consumption, annual performance and emission factors by quality of fuel and vehicle type. This data is used to calculate the costs of air pollution and global warming.

Other statistical sources and research results were used for assessing the emission costs of the fuel chain. There are no nuclear power stations in Estonia and all electricity is produced from oil shale.

The EcoSense database used in UNITE for estimating national environmental costs of air pollution has integrated basic data on receptors, meteorology and emissions as shown in Table 16.

Table 16. Environmental data in the EcoSense database

|  | Resolution | Source |
| --- | --- | --- |
| Receptor data |  |  |
| Population | administrative units, EMEP 50 grid | EUROSTAT REGIO Database, The Global Demography Project |
| Natural resources: Production of wheat, barley, sugar beat, potato, oats, rye, rice, tobacco, sunflower | administrative units, EMEP 50 grid | EUROSTAT REGIO Database, FAO Statistical Database |
| Materials: Inventory of natural stone, zinc, galvanized steel, mortar, rendering, paint | administrative units, EMEP 50 grid | Extrapolation based on inventories of some European cities |
| Ecosystems: Critical Loads/Levels for nitrogen-deposition for various ecosystems | EMEP 150 grid | UN-ECE |
| Meteorological data |  |  |
| Wind speed | EMEP 50 grid | European Monitoring and Evaluation Programme (EMEP) |
| Wind direction | EMEP 50 grid | European Monitoring and Evaluation Programme (EMEP) |
| Precipitation | EMEP 50 grid | European Monitoring and Evaluation Programme (EMEP) |
| Emissions |  |  |
| SO2, NOx, NH3, NMVOC, particles | administrative units, EMEP 50 grid | CORINAIR 1994/1990, EMEP 1998 TNO particulate matter inventory (Berdowski et al., 1997) |
| Source: IER |  |  |

Receptor data

- Population data

Population data for Estonia was taken from the EUROSTAT REGIO database (base year 1996), which provides data on administrative units (NUTS categories). For impact assessment, the receptor data is required in a format compatible with the output of the air quality models. Thus, population data was transferred from the respective administrative units to the 50 x 50 km2 EMEP grid by the transfer routine in EcoSense.

- Crop production

The following crops were considered for impact assessment in Estonia: barley, oats, potato, rye and wheat. Data on crop production were again taken from the EUROSTAT REGIO database for Estonia (base year 1996). For impact assessment, crop production data were transferred from the administrative units to the EMEP 50 x 50 km2 grid.

- Material inventory

The following types of materials are considered for impact assessment: galvanised steel, limestone, mortar, natural stone, paint, rendering, sandstone and zinc. As there is no database available that provides a full inventory of materials for Estonia, the stock at risk was extrapolated in ExternE from detailed studies carried out in several European cities.

- Critical loads for ecosystems

The EcoSense database provides critical load data on acidification and eutrophication for a wide range of ecosystems from the UN-ECE Co-ordination Centre for Effects for the year 1997 (Posch et al., 1997). The spatial resolution of critical load data is 150 x 150 km2.

Emission data

As the formation of secondary pollutants such as ozone or secondary particles depends heavily on the availability of precursors in the atmosphere, the EcoSense database provides a European wide emission inventory for SO2, NOx, NH3, NMVOC, and particles as an input to air quality modelling. It has used data from the EMEP 1998 emission inventory (Richardson, 2000, Vestreng, 2000, Vestreng and Støren, 2000). Also data from the CORINAIR 1994 inventory (http://www.aeat.co.uk/netcen/corinair/94/) and the CORINAIR 1990 inventory (McInnes, 1996) are used as supplements. For Russia, national average emission data from the LOTOS inventory (Builtjes, 1992) is included. Emission data for fine particles are taken from the European particle emission inventory established by Berdowski et al. (1997).

Meteorological data

The Windrose Trajectory Model requires annual average data on wind speed, wind direction, and precipitation as an input. The EcoSense database provides data from the European Monitoring and Evaluation Programme (EMEP) for the base year 1998.

For the calculation of the national costs of direct emissions from vehicle operation, inventories in spatial disaggregation are needed, i.e. a geo-coded data set for the different air pollutants. For each mode or vehicle category (e.g. road passenger transport, motorcycles, heavy goods vehicles) an emission inventory, giving total vehicle emissions in spatial disaggregation, was produced for Estonia. This input data is shown in Table 17.

Table 17. Direct transport emissions in Estonia in 1998

|  | CO2 (1000) tonnes | PM10 tonnes exhaust | NOx tonnes | SO2 tonnes | NMVOC tonnes4) |
| --- | --- | --- | --- | --- | --- |
| Road transport |  |  |  |  |  |
| Motor cycles1) | - | - | - | - | - |
| Passenger cars | 963 | 445 | 12701 | 109 | 7477 |
| Buses | 139 | 80 | 1765 | 49 | 949 |
| Goods vehicles | 621 | 1312 | 13199 | 340 | 13075 |
| Total | 1724 | 1837 | 27665 | 498 | 21502 |
| Rail transport |  |  |  |  |  |
| Diesel and electric traction |  |  |  |  |  |
| Diesel traction | 79 | 147 | 1527 | 99 | 265 |
| Electric traction2) | : | : | : | : | : |
| Air transport3) | 42 | : | 456 | 4 | 132 |
| Maritime shipping5) | 33 | 18 | 763 | 120 | 286) |
| 1) No data available – 2) Power plant emissions 3) Scheduled commercial flights within the Estonian flight information zone. Over flights excluded. – 4) Primarily HC. – 5) Route and berth emissions for maritime shipping on the Baltic Sea within the Estonian economic region. – 6) Including methane. Source: Entec Ltd |  |  |  |  |  |

- a)	Road transport

Direct road transport emissions were calculated according to annual vehicle performance and emission factors by the statistical vehicle fleet of Estonia. Emission factors for different vehicle categories consider the engine technology of the vehicles, emission abatement technology and fuel quality. The emission inventory is disaggregated into rural roads and urban roads (streets). Particle emissions presented as PM10 consist of particles with the diameter of 10 micrograms or less. Non-exhaust emissions, stemming from tyre and break wear, are not considered.

- b)	Rail transport

Data on emissions from diesel traction (fuel usage) includes shunting and other train yard operations.

The emissions of diesel traction are spatially distributed as in the CORINAIR 1990 emission inventories (SNAP sector 8/2/0 Other Mobile Sources and Machinery / Rail).

- c)	Public transport

The emissions from electrified trams and trolley buses could not be estimated due to lack of data.

Direct emissions from buses - consists of both urban and non-urban performance - are integrated into the road accounts. Local trains are included in the emission inventory of rail transport, and are considered within the rail account.

- d)	Air transport

Data on emissions from air transport covers all scheduled commercial aircraft movements in Estonia. Emissions are calculated by total fuel consumption, allocating it to aircraft mainly according to landing-takeoff -cycles. Over flights and general aviation are not included in the inventory of UNITE. The inventory of emissions covers operations within the Estonian flight control zone, an area equivalent to the Estonian economic region.

- e)	Maritime shipping

Vessel emissions are estimated within the Estonian economic region, an area larger than territorial waters.

Emissions are spatially distributed as in the CORINAIR (1990) emission inventories according to SNAP sector 8/3/0 Other Mobile Sources and Machinery/Inland Waterways.

f) Indirect emissions

With regard to costs due to air pollution, not only the operation of a vehicle is relevant, but also the provision of fuel or electricity. These indirect emissions sources are due to the process steps of average primary source of energy in electricity, crude oil extraction, refining and transport. Electricity production is considered explicitly in the emission inventories presented above. Emissions due to fuel provision have to be quantified separately. The respective emission factors for electricity production and provision of petrol and diesel are given in Table 18.

Table 18. Emission factors caused by fuel production processes

| Type of emission | Unit | CO2 | PM10 | NOx | SO2 | NMVOC |
| --- | --- | --- | --- | --- | --- | --- |
| Emissions caused by the production of | g/kg fuel |  |  |  |  |  |
| Petrol |  | 560 | 0.105 | 1.10 | 1.90 | 1.80 |
| Diesel; Kerosene |  | 400 | 0.047 | 0.96 | 1.40 | 0.62 |
| Source: Production of petrol and diesel: Friedrich and Bickel (2001) for PM10, IFEU (1999) for other pollutants. |  |  |  |  |  |  |

#### Global warming

- The calculation of the costs of CO2 are based directly on the level of CO2 emission for all modes of transport.

#### Noise

There do not exist data on population exposed to traffic noise in Estonia. Therefore noise costs could not be estimated.

#### Nature and landscape

The main input data for fixed environmental costs is the area taken by transport infrastructure. This cost is assessed based on assumed land consumption in different categories of infrastructure, along with the annual growth in infrastructure length and width respectively. A detailed background paper on cost estimation within this cost category is available from the UNITE consortium upon request (Doll, 2000). The needed data could not be obtained from Estonia and therefore fixed environmental costs are not calculated for Estonia.

#### Nuclear risk

The electricity consumption per transport mode is used for the estimation of risk due to the production of electricity in nuclear power stations. However, this is not relevant in Estonia where all electricity is produced from oil shale.

### 2.6.2	Summary

Table 19 presents a summary of input data used for the estimation of different types of environmental costs. Some remarks are made on the methodology and data quality.

Table 19. Source and quality of input data for estimating environmental costs

|  | Input data | Level of disaggregation | Quality of data, level of uncertainty |
| --- | --- | --- | --- |
| Air Pollution | Vehicle emission data provided by the Tallinn Technical University. | The emissions of CO, CO2, PM10, NOx, SO2 and NMVOC are estimated for road transport (4 vehicle types). The emissions of CO, CO2, PM10, NOx, SO2 and NMVOC are estimated for rail (passenger and freight) and for emissions due to the production of petrol diesel. The emissions of CO, CO2, PM10, NOx, SO2 and NMVOC are estimated for aircraft and maritime transport. Emissions for inland waterways transport are not estimated. | Data of reasonable quality. A sophisticated model (IPM) used for measuring dispersion and chemical conversion of emissions, the calculation of physical impacts and a valuation of these effects. Even though the model reflects the state-of-the-art knowledge and has been previously used for estimating the costs of emissions of power production and transport in Europe, it is like all models accompanied by uncertainties and the results are best estimates only. |
| Global warming | Vehicle emission data for CO2 as above. | Road, rail (passenger and freight), aviation and maritime transport. | Data is of reasonable quality. Uncertainty remains with the valuation of CO2. A shadow value of €20 per tonne CO2, has been used. This value is lower than presumed in previous studies, but reflects the latest estimates available. |
| Noise | Data on the number of people exposed to transport noise is not available. Therefore noise costs are not estimated. |  |  |
| Nature, landscape, soil and water pollution | Necessary data could not be obtained for any mode. Therefore respective costs are not estimated. |  |  |
| Nuclear risk | Nuclear energy is not used. All electricity is produced from oil shale. |  |  |

## 2.7	Taxes, charges, subsidies

Table 20 gives an overview of the data used. In general the data quality is considered to be good.

Table 20. Input data for taxes, charges and subsidies

|  | Input data | Level of disaggregation | Quality of data, level of uncertainty |
| --- | --- | --- | --- |
| Road | Revenues from taxes from MTC (2000), DHV and LT Consultants (1999). | No disaggregation to vehicle categories. | In general good data, but no disaggregation to vehicle categories. |
| Rail | Data on revenues from MTC (2000), DHV and LT Consultants (1999). | Tariff revenues divided between passenger and freight transport. | Good data. |
| Public Transport | Data on revenues from MTC (2000), DHV and LT Consultants (1999). | Total tariff revenues only. Subsidies are broken down into: state and municipal ones. | Good data. |
| Air | Data on revenues from MTC (2000), DHV and LT Consultants (1999). | Revenues for navigation fee and fuel tax, and also subsidies are given. | Good data. Indirect subsidies (for example no fuel tax on kerosene on international flights) were not quantified. |
| Inland waterway | Not considered. |  |  |
| Maritime shipping | Data on revenues from MTC (2000), DHV and LT Consultants (1999). | Revenues are given for navigation fees and fuel tax, and also subsidies. | Good data. |
| Source: JP-Transplan Ltd |  |  |  |

# 3	Methodological issues

The methodology used in developing the UNITE pilot accounts has been documented in the publication “The Accounts Approach” by Link et al. (2000). In this annex report on the Estonian pilot accounts we will only summarise the methodology as far as it is necessary to understand and interpret the accounting results. We will focus on new methodology or deviations from the general methodology developed in Link et al. (2000) and on the methods used to compile the results for 1996 and 2005.

## 3.1	Infrastructure costs

Infrastructure costs contain capital costs (depreciation and interests) for new investments and for replacement of assets on the one hand and running costs for maintenance, operation and administration/ overheads on the other hand. The basis for estimating capital costs is the value of the capital stock. Several methods to quantify the capital stock are described in Link et al. (2000). The perpetual inventory method has usually been applied in UNITE for all modes, which had long enough time series for investments. The necessary length of time series is related to the life expectancies of infrastructure asset groups.

Estonia gained back its independence in 1991. The following change in administration has made it impossible to provide long enough time series for investments. Therefore the value of the capital stock could not be calculated for any mode.

### 3.1.1	Road

Core year 1998: Capital stock and capital costs could not be calculated. Data on current investments and running costs was obtained from MTC (2000) and DHV and LT Consultants (1999). Available data did not allow cost allocation per urban and rural roads neither per vehicle category.

- Year 1996: The same methodology as for 1998 was applied.

Forecast methodology: Forecasts were not made due to the lack of data.

### 3.1.2	Rail

Core year 1998: Tracks and stations could not be separated. Current investments and running costs were obtained from MTC (2000) and DHV and LT Consultants (1999). Some uncertainty remained regarding the share of investments and running costs. Infrastructure costs could not be divided between passenger and freight transport according to wagon-axle kilometres, because of missing data.

- Year 1996: The same methodology as for 1998 was applied.

Forecast methodology: Forecasts were not made due to the lack of data.

### 3.1.3	Public transport infrastructure – tram, trolley buses

Core year 1998: We excluded buses from the estimation of infrastructure costs due to the fact that infrastructure costs caused by buses are included in the road account. The necessary time series - on investments in tram and trolley buses - for estimating capital stock could not be obtained, nor the infrastructure running costs could be estimated.

Year 1996: The same methodology as for 1998 was applied. The same data problems did not allow the estimation of capital stock.

Forecast methodology: Forecasts were not made due to the lack of data.

### 3.1.4	Aviation infrastructure

Core year 1998: Capital stock and capital costs for Estonian airports could not be calculated with the perpetual inventory model because of too short time series. Data for investments and running costs was available from MTC (2000) and DHV and LT Consultants (1999). The running costs cover air traffic control services, aviation administration, education and one tenth of runway repairing costs, but exclude other airport services. It is supposed that runway repairing lasts for 10 years; therefore one tenth of the costs is allocated for the running costs and the rest to the capital costs.

Year 1996: The same methodology as for 1998 was applied.

Forecast methodology: Forecasts were not made due to the lack of data.

### 3.1.5	Inland waterways and harbours

Inland waterways are used mainly for leisure purposes and therefore left out of the pilot accounts.

### Maritime shipping – coastal channels, harbours and icebreakers

Core year 1998: Time series for investments were too short for the use of the perpetual inventory model. Data on investments and running costs for coastal waterways and for ferry harbours for domestic transport were available from MTC (2000) and DHV and LT Consultants (1999).

Year 1996: The same methodology was applied as for the core year 1998.

Forecast methodology: Forecasts were not made because of missing data.

## 3.2	Methodology for estimating supplier operating costs

For the UNITE pilot accounts it was decided to calculate supplier operating costs only for transport modes where the revenues from the transport users do not cover the costs of the supplier. This is mainly true for public transport and rail transport and is considered to be core data for these transport modes. This means for Estonia to analyse first of all the rail carriers and public transport companies (tram, trolley buses, buses). It has to be noticed that in Estonia also air and ferry transport to the islands is subsidized.

### 3.2.1	Rail carriers (Estonian Railways Ltd and Edelaraudtee Ltd)

Core year 1998: Data was obtained from MTC (2000) and DHV and LT Consultants (1999). No disaggregation to different cost categories was made.

Year 1996: The same methodology as for 1998 was applied.

Forecast methodology: Forecasts were not made because of missing data.

### 3.2.3	Public transport

Core year 1998: Ideally, supplier operating costs would have separately to be estimated for companies with tram and trolley bus operations (or their respective business units) and for companies operating bus services (or their respective business units). Furthermore, an analysis of supplier operating costs would require a separate treatment of municipal companies with (at least partly) public ownership on the one hand and private companies on the other hand. However, only total figures for all companies were available from MTC (2000) and DHV and LT Consultants (1999).

Year 1996: We refer to the discussion on the core year 1998.

Forecast methodology: No forecast was made due to the lack of reliable information.

## 3.3	Methodology for estimating delay costs due to congestion

Core year 1998: The UNITE methodology defines congestion costs as the sum of those time and operating costs perceived by transport users which exceed average time and operating costs.

Estonian transport network has a few congestion problems outside the capital city Tallinn. Data on possible delays is not available.

## 3.4	Methodology for estimating accident costs

Core year 1998: Material damages, administration costs, medical costs, production losses and the valuation of the risk associated with using transportation are the subcategories used for the evaluation of accident costs. Each of these subcategories is valued through the use of the number of incidents and the costs arising from the incident. The numbers and costs from material damages, administration and medical subcategories were obtained from COWI and TTU (2001). Production losses represent an estimation of the losses to the national economy due to replacement costs, lost output of employed persons and lost non-market production (e.g. domestic work) resulting from accidents. The emphasis within this cost category was placed on medical costs and the cost arising from transport related fatalities. All valuations are documented in Nellthorp et al. (2001).

Accident costs are divided into internal and external accident costs. External accident costs are those costs imposed by the transport user on those outside the transport sector. Hence “internal  costs” embrace all costs borne by the individual transport users (e.g. damages to property not covered by insurance companies and the risk associated with using transport) and costs borne by the community of transport users (including all costs covered by traffic insurance companies). Explicitly external costs are administrative costs for police or the legal system, the costs of medical treatment not covered by traffic insurance companies and production losses. Due to the present data situation it was not possible to divide medical costs into internal and external costs, and thus in a simplified approach this cost component was considered to be totally external. The remaining internal costs therefore comprise only of the costs of material damages and the risk value. Risk value is considered to be internal for the purpose of UNITE. This means that we implicitly assume that accident risks are fully anticipated by individuals when they decide to take part in transport. External accident costs are considered to be core data while internal accident costs, because the costs are borne by the transport users and not society as a whole, are considered to be additional information only.

The methodology applied here followed as much as possible the recommendations of Doll et al. (2000). The focus of the present accounts is clearly on external accident costs. Because of the data situation the presentation of a matrix of cost responsibility and cost bearers as proposed in Doll et al. (2000) respectively could not be presented.

- The costs of medical treatment

The costs of medical treatment of traffic casualties can be broken down into a number of different activities. Injuries can be further divided into severely and lightly injured. Costs in Table 21 are based on information from one hospital and they do not include the costs in other medical organisations. Therefore they must be considered to represent a low approximation of total costs, especially for severe injuries.

Table 21. Average costs for medical treatment per degree of injury in Estonia 1998

| Severity of Accident | € per casualty |
| --- | --- |
| Slight injuries | 25 |
| Severe injuries | 1 458 |
| Fatalities | 1 695 |
| Source: COWI and TTU (2001) |  |

- b)	Production losses

According to the methodology described in Doll et al. (2000) the cost category “Production Loss” comprises of two elements:

- The loss of the production power of steadily disabled or traffic fatalities.

- The temporary costs for the victim’s employer.

The lost production time per victim takes into consideration the duration of various medical actions and the duration of partial disability preventing the victim taking part in the production process. The effective loss of productive time further considers the degree of disability to work and the share of victims of employable age. In Estonia information about ages of victims was obtained from COWI  and TTU (2001).

The gross production loss per lost year of working time refers to the production potential of the national economy rather than to the actual GNP. Thus the gross production loss is composed of the GNP per capita in employable age and the relation between GNP and the production potential (1.04 according to the Cochrane-Orcutt production function). In order to avoid double counting with the Risk Value the future consumption was subtracted from the gross production potential. The resulting net production potential then was discounted to 1998 using a social interest rate of 3%.

Ages of people killed in accidents were obtained from yearly statistics from insurance companies. We used similar average length of illness as IWW. Statistical office of Estonia (www.stat.ee) provided information of average income per capita, of private consumption per capita and GNP. We did not include temporary costs at the victim's former working place.

c)	Valuation of administrative costs

Administrative costs are composed of the costs for police, justice and for the insurance sector. For Estonia, data provided by COWI and TTU (2001) included only costs for traffic police (Table 22).

This information is valid for road transport only. It cannot be assumed that the time required by police to deal with accidents in other modes is similar to the road case.

Table 22. Police costs in road accidents in Estonia in 1998

| Type of accident | € per casualty |
| --- | --- |
| Slight injuries | 75 |
| Severe injuries | 68 |
| Fatalities | 54 |
| Source: COWI and TTU, 2001 |  |

Any estimates of costs to the legal system and to the administration of insurance companies could not be obtained. The administrative costs for traffic police, as considered here, are totally external to the transport sector as they are covered by the general budget and are the only administrative costs evaluated.

- d)	Valuation of material damages

Information on the average costs of accidents was only available for road accidents from COWI and TTU (2001). However, they did not include any differentiation by vehicle category. In 1998 material damages were estimated to € 2 124 per road accident.

e)	The risk value

The Risk Value was set according to the recommendations of the UNITE valuation conventions.:

- € 0.65 million for fatalities,

- 15% of € 0.65 million = € 97 500 for severe injuries.

- 1% of the value of statistical life = € 6 500 for slight injuries.

Risk values for relatives and friends were not considered. For the UNITE accounts, risk value is considered to be fully internal.

Year 1996: No estimations for the year 1996 were made.

Forecast methodology: No forecasts were made.

## Methodology for estimating environmental costs

This section is organized along the subcategories of environmental costs for the core year 1998 and the procedures applied for the estimates for 1996 and the forecast 2005.

For the estimation of environmental costs, four sub cost categories have been developed. These are: air pollution, noise, environmental impacts on nature, landscape, soil and ground water, and the risk associated with nuclear energy production. The first two of these subcategories (air pollution and noise) are considered core cost data with emphasis on cost assessment, and the latter categories are considered additional cost data with less emphasis on cost assessment due to methodological uncertainties.

### 3.5.1	Air pollution

#### General Approach

For quantifying the damage costs of airborne pollutants the Impact Pathway Approach, a methodology developed in the ExternE project series was applied. A detailed description can be found in European Commission (1999). The methodology consists of the following steps: emission estimation, dispersion and chemical conversion modelling, calculation of physical impacts of pollution concentrations, and monetary valuation of these impacts.

For calculating the damage costs of direct emissions from vehicle operation at regional level, emission inventories in spatial disaggregation, i.e. a geo-coded data set for the different pollutants, are needed. For each mode or vehicle category (e.g. road passenger transport, heavy goods vehicles), an emission inventory, giving total vehicle emissions in spatial disaggregation, was produced. For each of these emission inventories, Europe-wide impacts were calculated and subtracted from impacts resulting from a reference inventory without these emissions. This procedure using a reference inventory is required because of air chemistry processes where “background” emissions play an important role.

Damage costs on the local scale – up to about 20 km to each side of a line emission source (e.g. road) – were quantified using a Geographical Information System. Detailed, location specific calculations were carried out for emissions from the operation of vehicles, aircrafts or vessels, and for emissions from power plants. For fuel production (comprising the processes fuel extraction, transportation and refining), no information was available on the location of emissions. In this case, an average damage cost factor per tonne of pollutant emitted in Germany was used.

A description of the computer model EcoSense, which was used for the cost calculations, including exposure-response functions and monetary unit values of damages, is given below.

a)	Description of the EcoSense computer model for assessment of costs due to airborne emissions

The EcoSense model has been developed within the series of ExternE Projects on ‘External Costs of Energy’ funded by the European Commission (European Commission, 1999). The model allows the quantification of environmental impacts by following a detailed site-specific ‘impact pathway’ (or damage function) approach, in which the causal relationships from the release of pollutants through their interactions with the environment to a physical measure of impact are modelled and, where possible, valued monetarily. A schematic flowchart of the EcoSense model is shown in figure 1. EcoSense provides integrated air quality and impact assessment models together with a comprehensive set of input data for the whole of Europe, which allow a site-specific bottom-up impact analysis.

In ExternE, EcoSense was used to calculate external costs from individual power plants in a large number of case studies in all EU countries. While the first generation of the EcoSense model was focused on the analysis of single emission sources, the new ‘multi-source’ version of the model provides a link to the CORINAIR database, which allows the analysis of environmental impacts from more complex emission scenarios. The CORINAIR database provides emission data for a wide range of pollutants according to both a sector (‘Selected Nomenclature for Air Pollution’ - SNAP categories) and geographic (‘Nomenclature of Territorial Units for Statistics’ - NUTS categories) disaggregation scheme (McInnes, 1996).

A transformation module implemented in EcoSense supports the transformation of emission data between the NUTS administrative units (country, state, municipality) and the grid system required for air quality modelling (EMEP 50 x 50 km2 grid). Based on this functionality, EcoSense allows a user to change emissions from a selected sector (e.g. road transport) within a specific administrative unit, creates a new European-wide emission grid scenario for air quality modelling, and compares environmental impacts and resulting damage costs between different emission scenarios. In other words, environmental damage costs are calculated by comparing the results of two model runs:

- A model run using the ‘full’ European emission scenario as an input to air quality and damage modelling, including emissions from all emission sources in Europe, as well as the emissions from the transport sector considered.

- A second model run in which the emissions from the transport sector are set to zero.

The difference in impacts and costs resulting from the two model runs represents the damages due to the transport sector.

- Figure 1. Flowchart of the EcoSense model.

In addition to these Europe-wide impacts, local scale impacts are quantified using a Geographical Information System and spatially highly disaggregated data (see section 2.2.5).

b)	Air quality models

Within the UNITE project two air quality models were used from the three available within the EcoSense. The model for local scale effects was not required as they were covered with the GIS environment. Thus, the following models are required for the tasks of UNITE:

- The Windrose Trajectory Model (WTM) (Trukenmüller et al., 1995) is used in EcoSense to estimate the concentration and deposition of acid species on a regional scale.

- The Source-Receptor Ozone Model (SROM), based on the EMEP country-to-grid matrices (Simpson et al., 1997), is used to estimate ozone concentrations on a European scale.

c)	Dose-effect models

The dose-response functions used within UNITE are based on the recommendations of the expert groups in the final phase of the ExternE Core/Transport project (Friedrich and Bickel, 2001). Table 23 presents a summary of the dose-response functions as they are implemented in the EcoSense version used for this study.

Table 23. Health and environmental effects included in the analysis of air pollution costs

| Impact category | Pollutant | Effects included |
| --- | --- | --- |
| Public health – mortality | PM2.5 , PM10 1) SO2, O3 | Reduction in life expectancy due to acute and chronic mortality Reduction in life expectancy due to acute mortality |
| Public health – morbidity | PM2.5 , PM10, O3 | Respiratory hospital admissions |
|  |  | Restricted activity days |
|  | PM2.5 , PM10 only | Cerebrovascular hospital admissions |
|  |  | Congestive heart failure |
|  |  | Cases of bronchodilator usage |
|  |  | Cases of chronic bronchitis |
|  |  | Cases of chronic cough in children |
|  |  | Cough in asthmatics |
|  |  | Lower respiratory symptoms |
|  | O3 only | Asthma attacks |
|  |  | Symptom days |
| Material damage | SO2, acid deposition | Ageing of galvanised steel, limestone, natural stone, mortar, sandstone, paint, rendering, zinc |
| Crops | SO2 | Yield change for wheat, barley, rye, oats, potato, sugar beet |
|  | O3 | Yield loss for wheat, potato, rice, rye, oats, tobacco, barley, wheat |
|  | Acid deposition | Increased need for liming |
|  | N, S | Fertilisational effects |
| 1) including secondary particles (sulphate and nitrate aerosols) Source: IER |  |  |

d)	Exposure-response functions for the quantification of health effects

Table 24 lists the exposure response functions used for the assessment of health effects. The exposure response functions are taken from the 2nd edition of the ExternE Methodology report (European Commission, 1999), with some modifications according to recent recommendations of the health experts in the ExternE Core/Transport project (Friedrich and Bickel, 2001).

Table 24. Quantification of human health impacts due to air pollution

| Receptor | Impact Category | Reference | Pollutant | fer |
| --- | --- | --- | --- | --- |
| ASTHMATICS (3.5% of population) |  |  |  |  |
| Adults | Bronchodilator usage | Dusseldorp et al., 1995 | PM10 Nitrates PM2.5 Sulphates | 0.163 0.163 0.272 0.272 |
|  | Cough | Dusseldorp et al., 1995 | PM10, Nitrates PM2.5 Sulphates | 0.168 0.168 0.280 0.280 |
|  | Lower respiratory symptoms (wheeze) | Dusseldorp et al., 1995 | PM10 Nitrates PM2.5 Sulphates | 0.061 0.061 0.101 0.101 |
| Children | Bronchodilator usage | Roemer et al., 1993 | PM10 Nitrates PM2.5 Sulphates | 0.078 0.078 0.129 0.129 |
|  | Cough | Pope and Dockery, 1992 | PM10 Nitrates PM2.5 Sulphates | 0.133 0.133 0.223 0.223 |
|  | Lower respiratory symptoms (wheeze) | Roemer et al., 1993 | PM10 Nitrates PM2.5 Sulphates | 0.103 0.103 0.172 0.172 |
| All | Asthma attacks (AA) | Whittemore and Korn, 1980 | O3 | 4.29E-3 |
| ELDERLY 65+ (14% of population) |  |  |  |  |
|  | Congestive heart failure | Schwartz and Morris, 1995 | PM10 Nitrates PM2.5 Sulphates CO | 1.85E-5 1.85E-5 3.09E-5 3.09E-5 5.55E-7 |
| CHILDREN (20% of population) |  |  |  |  |
|  | Chronic cough | Dockery et al., 1989 | PM10 Nitrates PM2.5 Sulphates | 2.07E-3 2.07E-3 3.46E-3 3.46E-3 |
| ADULTS (80% of population) |  |  |  |  |
|  | Restricted activity days (RAD) | Ostro, 1987 | PM10 Nitrates PM2.5 Sulphates | 0.025 0.025 0.042 0.042 |
|  | Minor restricted activity days (MRAD) | Ostro and Rothschild, 1989 | O3 | 9.76E-3 |
|  | Chronic bronchitis | Abbey et al., 1995 | PM10 Nitrates PM2.5 Sulphates | 2.45E-5 2.45E-5 3.9E-5 3.9E-5 |
| ENTIRE POPULATION |  |  |  |  |
|  | Chronic Mortality (CM) | Pope et al., 1995 | PM10 Nitrates PM2.5 Sulphates | 0.129% 0.129% 0.214% 0.214% |
|  | Respiratory hospital admissions (RHA) | Dab et al., 1996 | PM10 Nitrates PM2.5 Sulphates | 2.07E-6 2.07E-6 3.46E-6 3.46E-6 |
|  |  | Ponce de Leon et al., 1996 | SO2 O3 | 2.04E-6 3.54E-6 |
|  | Cerebrovascular hospital admissions | Wordley et al., 1997 | PM10 Nitrates PM2.5 Sulphates | 5.04E-6 5.04E-6 8.42E-6 8.42E-6 |
|  | Symptom days | Krupnick et al., 1990 | O3 | 0.033 |
|  | Cancer risk estimates | Pilkington et al., 1997; based on US EPA evaluations | Benzene Benzo-[a]-Pyrene 1.3-buta-diene Diesel particles | 1.14E-7 1.43E-3 4.29E-6 4.86E-7 |
|  | Acute Mortality (AM) | Spix et al. / Verhoeff et al.,1996 | PM10 Nitrates PM2.5 Sulphates | 0.040% 0.040% 0.068% 0.068% |
|  |  | Anderson et al. / Touloumi et al., 1996 | SO2 | 0.072% |
|  |  | Sunyer et al., 1996 | O3 | 0.059% |
| The exposure response slope, fer, has units of [cases/(yr-person-µg/m3)] for morbidity, and [%change in annual mortality rate/(µg/m3)] for mortality. Concentrations of SO2, PM10 , PM10, sulphates and nitrates as annual mean concentration, concentration of ozone as seasonal 6-h average concentration. Source: Friedrich and Bickel 2001. |  |  |  |  |

e)	Exposure-response functions for the quantification of impacts on crops

Functions are used to quantify changes in crop yields due to the emissions of SO2, nitrates, ozone and acids.

f)	Exposure-response functions for the quantification of material damage

Functions are used to quantify and value damages to limestone, sandstone, natural stone, mortar, rendering, zinc and galvanized steel and paint due to the effects of air pollution.

g)	Acidification and eutrophication of ecosystems

There are no effect models available to quantify the expected damage to ecosystem resulting from exceeding of critical loads. Therefore, such effects are not quantified in the present study.

#### Monetary values

Table 25  summarizes the monetary values used for the valuation air pollution impacts in Estonia in UNITE. According to Nellthorp et al. (2001) average European values should be used for trans-boundary air pollution costs, except for the source country, where country specific values were used. These values were calculated for Estonia according to the benefit transfer rules given in Nellthorp et al. (2001).

Table 25. Monetary values (factor costs) for valuing health impacts for Estonia

| Impact | € (rounded) |
| --- | --- |
| Year of life lost (chronic effects) | 32,200 |
| Year of life lost (acute effects) | 55,400 |
| Chronic bronchitis | 59,300 |
| Cerebrovascular hospital admission | 5,990 |
| Respiratory hospital admission | 1,560 |
| Congestive heart failure | 1,180 |
| Chronic cough in children | 80 |
| Restricted activity day | 40 |
| Asthma attack | 30 |
| Cough | 15 |
| Minor restricted activity day | 15 |
| Symptom day | 15 |
| Bronchodilator usage | 14 |
| Lower respiratory symptom | 3 |
| Source: Calculations based on Friedrich and Bickel (2001) and Nellthorp et al. (2001) |  |

#### Discussion of uncertainties

In spite of considerable progress made in recent years, the quantification and valuation of environmental damage is still linked to significant uncertainty. This is the case for the Impact Pathway Methodology as well as for any other approach. While the basic assumptions underlying the work in ExternE are discussed in detail in (European Commission, 1999), below an indication is given of the uncertainty of the results, as well as the sensitivity of the key assumptions.

Within ExternE, Rabl and Spadaro (1999) made an attempt to quantify the statistical uncertainty of the damage estimates, taking into account uncertainties resulting from all steps of the impact pathway. Rabl and Spadaro (1999) show that - due to the multiplicative nature of the impact pathway analysis - the distribution of results is likely to be approximately lognormal, thus it is determined by its geometric mean and the geometric standard deviation g.

In ExternE, uncertainties are reported by using uncertainty labels, which can be used to make a meaningful distinction between different levels of confidence, but at the same time do not give a false sense of precision, which seems to be unjustified in view of the need to use subjective judgement to compensate the lack of information about sources of uncertainty and probability distributions (Rabl and Spadaro, 1999). The uncertainty labels are:

A = high confidence, corresponding to g = 2.5 to 4;

B = medium confidence, corresponding to g = 4 to 6;

C = low confidence, corresponding to g = 6 to 12.

According to ExternE recommendations, the following uncertainty labels are used to characterize the impact categories addressed in this report:

Mortality:		B

Morbidity:		A

Crop losses:		A

Material damage:	B.

Beside the statistical uncertainty indicated by these uncertainty labels, there is however a remaining systematic uncertainty arising from a lack of knowledge, and value choices that influence the results. Some of the most important assumptions and their implications for the results are briefly discussed in the following.

- Effects of particles on human health

The dose-response models used in the analysis are based on results from epidemiological studies, which have established a statistical relationship between the mass concentration of particles and various health effects. However, at present it is still not known whether it is the number of particles, their mass concentration or their chemical composition, which is the driving force. The uncertainty resulting from this lack of knowledge is difficult to estimate.

- Effects of nitrate aerosols on health

We treat nitrate aerosols as a component of particulate matter, which we know cause damage to human health. However, in contrast to sulphate aerosol (but similar to many other particulate matter compounds) there is no direct epidemiological evidence supporting the harmfulness of nitrate aerosols, which partly are neutral and soluble.

- Valuation of mortality

While ExternE recommends using the Value of a Life Year Lost rather than the Value of Statistical Life for the valuation of increased mortality risks from air pollution (see European Commission, 1999) for a detailed discussion), this approach is still controversially discussed in the literature. The main problem for the Value of a Life Year Lost approach is that up to now there is a lack of empirical studies supporting this valuation approach.

- Impacts from ozone

As the EMEP ozone model, which is the basis for the Source-Receptor Ozone Model (SROM) included in EcoSense does not cover the full EcoSense modelling domain, some of the ozone effects in Eastern Europe are omitted. As effects from ozone are small compared to those from other pollutants, the resulting error is expected to be small compared to the overall uncertainties.

- Omission of effects

The present report is limited to the analysis of impacts that have shown to result in major damage costs in previous ExternE studies. Impacts on e.g. change in biodiversity, potential effects of chronic exposure to ozone, cultural monuments, direct and indirect economic effects of change in forest productivity, fishery performance, and so forth, are omitted because currently they cannot be quantified.

### 3.5.2	Global warming

The method of calculating the costs of CO2 emissions basically consists of multiplying the amount of CO2 emitted by a cost factor. Due to the global scale of the damage caused, there is no difference how and where the emissions take place.

A shadow value of € 20 per tonne of CO2 emitted is used for valuing CO2 emissions, which reflects the costs of meeting the Kyoto targets in Germany (Fahl et al.,1999) and Belgium (Duerinck, 2000). This value lies within a range of values of € 5 to € 38 per tonne of CO2 avoided presented by Capros and Mantzos (2000). These authors calculated shadow prices for the EU to meet the Kyoto targets with and without emission trading.

Looking further into the future, more stringent reductions than the Kyoto aims are assumed to be necessary for reaching a sustainable emission level. Based on a reduction target of 50% in 2030 compared to 1990, INFRAS/IWW (2000) use an avoidance cost of € 135 per tonne of CO2; however one can argue that this reduction target has not yet been accepted.

A valuation based on the damage cost approach, as e.g. presented by Friedrich and Bickel (2001), would result in substantially lower costs. Due to the evident uncertainties involved in the estimation process, such values have to be used very cautiously.

### 3.5.3	Noise

Noise costs in UNITE have been quantified for a number of health impacts calculated with new exposure-response functions, plus amenity losses estimated by hedonic pricing. The methodology for quantifying noise costs was extended to the calculation of physical impacts. However, the lack of input data hindered the use of this methodology for Estonian transport.

### 3.5.4	Costs due to impacts on nature, landscape, soil and groundwater

The methodology applied to determine the annual fixed environmental costs for the year of investigation (here 1998) follows the approach of INFRAS/IWW (2000). According to this methodology, the costs of nature and landscape are defined as the share of the accounting period at the total loss of ecological resources caused by the construction of transport infrastructure from a defined base year until the year of accounting. However, the lack of input data hindered the use of this methodology for Estonian transport.

### 3.5.5	Costs due to nuclear risks

The estimate for the costs due to nuclear risks is based on the damage cost approach. Nuclear risk does not exist in Estonia where all electricity is produced from oil shale.

### 3.5.6	Methodology for 1996 and for the forecast to 2005

The forecasted environmental costs of airborne pollutants for road, rail, air and waterway transport are based on the future projections of transport emissions by the Tallinn Technical University. These projections are based on expected changes in mileage and emission factors by mode.

Concerning the monetary values for 2005, the UNITE valuation convention has been applied. According to Nellthorp et al. (2001) values change proportionally to real incomes. Hence, values are adjusted according to changes in real GDP per capita. This results in a factor of 0.86 for the 1996 values relating to 1998 values (based on Nellthorp et al. (2001) - Annex 2) and a factor of 1.28 for 2005 values relating to 1998 values (assuming that the average growth rate - 4,87 % per annum - from 1995 to 2001 continues up to 2005). These factors are applied for all cost categories and modes.

## 3.6	Methodology for estimating taxes, charges and subsidies

### 3.6.1	General issues

The general methodology for collecting, supplementing and estimating transport related taxes, charges and subsidies was as far as practically possible based on “Accounts Approach for Taxes, Charges and Subsidies”, by Macario et al. (2000).

Before discussing the methodology in detail per mode it seems to be necessary to describe the following methodological issues and problems:

- The aim of the UNITE accounts was not to compile a complete data set of all taxes, charges and subsidies of the transport sector. The aim was rather to define properly those taxes and charges paid by infrastructure users (individual passengers as well as transport operators) which can be seen as revenues corresponding to the cost side of the accounts.

- Although the scope of taxes and charges included in the analysis was defined along their relationship to the different cost categories (infrastructure costs, accident costs, environmental costs, supplier operating costs) they can hardly be directly compared with the respective cost category. The reason for this is, first of all, the historical evolution of national taxation systems with different and from time to time changing justification of taxation purposes, levels, structures and (eventually existing) earmarking procedures (see “The Accounts Approach” Link et al. (2000) for a more detailed discussion).

- In the philosophy of the UNITE transport accounts - with a cost side and a revenue side -subsidies have to be treated at both sides of the account: Subsidies paid for infrastructure financing have to be considered as costs of infrastructure provision. The input data on investments - for capital stock valuation – ought to contain all investments spent per mode, independent of their financial source. On the other hand, direct subsidies paid to transport operators (for example for public service obligations but also as compensation payments for reduced tariffs for certain social groups) increase the revenues of the respective companies and are often contained in the item “tariff revenues” in their business accounts. As far as possible the subsidies contained there are reported as additional information outside the main body of the accounts.

- Indirect subsidies such as tax exemptions/reductions were quantified whenever possible and reported separately. It should be noted, however, that due to the fact that certain modes or user groups are exempted from taxes the accounts show at the revenue side either no entries or lower numbers (in case of tax reduction). Thus, indirectly these tax exemptions are considered even when not quantitatively reported. This data is additional information only.

- VAT is reported as an additional information if and only if VAT rates in transport differ from those paid in other sectors of the economy. Note, that the basic principle for the UNITE accounts is a net principle, e.g. a reporting on a factor cost basis (see Nellthorp et al., 2001).

### 3.6.2	Methodological issues per mode

a) Road transport

In road transport taxes fall into the following categories:

- fuel tax,

- motor vehicle tax,

- registration tax,

- annual vehicle tax .

Motor fuel tax has been collected from July 1993. The technical base of the fuel tax is the fuel type.

Motor vehicle tax was enforced on April 1995. It is paid when motor vehicles are taken in use in Estonia. From April 1999 tax for cars with cylinder volume over 2 litre and for older imported passenger cars was increased.

From January 1999 all fees for services provided by the Motor Vehicle Registration Centre are included as well as the above taxes - in the national budget.

Annual vehicle tax was enforced on January 1996 in Tallinn and on April 1996 in Alajõe municipality, but later on abolished in Alajõe municipality. Taxation of all types of vehicles is based on engine capacity (kW) and age. Annual vehicle tax is collected to the budget of the local government.

The total revenues from taxation were taken from MTC (2000) and DHV and LT Consultants (1999).

b) Rail transport

Starting from the year 2000 Estonian Railways Ltd received railway user fee from Edelaraudtee Ltd and Elektriraudtee Ltd as well as from other carriers who also paid the transport costs of their rail cars or train assemblage. The tariff revenues of railway companies were included in the analysis of UNITE as this revenue category directly corresponds to the supplier operating costs.

Other revenues (revenues that do not relate directly to costs) include the fuel tax paid for diesel consumption in the rail sector, which was obtained from DHV and LT Consultants (1999). It has to be noticed that diesel oil used in road transport has almost six times as high tax per ton as engine oil used in railway locomotives (MTC, 2000).

A special problem when discussing rail is to quantify the level of subsidisation. Subsidies to railways are granted for several purposes. First capital repairs or a construction of railways can get partial support from the national budget. These subsidies are already reflected at the cost side - in infrastructure costs. Second, there are subsidies for passenger transport including also compensation payments for reduced tariffs for certain social groups. Values for subsidies are from TCM (2000).

c) Public transport

In public transport (excluding rail) taxes and charges, which could be directly compared with infrastructure costs do not exist for this mode of transport.

Fuel tax paid by bus transport was included in the road account.

Public transport subsidies are not earmarked for any cost categories but are used for covering general deficits of public transport companies. In Estonia VAT is 18 per cent for public transport as well as for other items.

Infrastructure subsidies are already considered at the cost side like in all other modes due to the principle of including all investments independent of their origin and/ or financial source.

d) Aviation

The Government of Estonia fixes the rate of the navigation fee paid for aircraft passing through Estonian air space. From January 1998 all taxes - except fuel tax for inland flights - in air transport are collected by Estonian Air Navigation Services Ltd. This company then pays income tax on regular basis.

Airport management establishes the rates of airport charges for start, landing and parking of aircrafts in cooperation with the Ministry of Transport and Communication. Charges for ground services of aircraft as well as passenger and cargo services are fixed independently by the airport management.

International aviation is exempted from fuel taxes. The tax loss due to this exemption can be considered to be an indirect subsidy. VAT is not charged on the price of international tickets. These indirect subsidies could not be estimated due to the lack of data.

Navigation fees received by the Estonian government in 1996 exceeded the costs included in the state budget - probably covering also the capital costs.

e) Maritime transport

Fuel tax is paid only for the fuel oil used in domestic transport, consisting mainly coastal passenger ferries and cargo ships.

Ministry of Transport and Communication establishes the rates of user charges: piloting, lighthouse and ice-breaking fees.

Estonian ports are state owned, municipal or private. The owner or holder of the port establishes the rates of port charges.

# Results

## 4.1	Infrastructure costs

Estonia gained back its independence in 1991. The following change in administration has made

it impossible to provide long time series for investments. Therefore the value of the capital stock could not be calculated for any mode.

In the following we present the results for the core year 1998 and the year 1996 both at 1998 prices. Estimates of infrastructure costs for the forecast year 2005 do not exist for any mode.

### 4.1.1	Road transport

The Estonian state and municipalities used €57.4 million in 1998 for the running costs of the Estonian road network compared to €57.9 million in 1996. The investments for the state road network were €7.9 million in 1998 and €9.8 million in 1996. The Estonian municipalities used €24.3 million in 1998 and €34.1 million in 1996 for municipal road network. The municipal spending included also some investments, which could not be separated from the total.

### Railways

The Estonian Railways Ltd and Edelaraudtee Ltd used €13.1 million in 1998 for infrastructure running costs of the Estonian rail network. Corresponding investments were €16.4 million in 1998. Infrastructure costs could not be divided between the track network and the stations.

### 4.1.4	Other public transport infrastructure (tram, trolley buses)

As discussed in chapter 3 we excluded buses from estimating infrastructure costs due to the fact that infrastructure costs caused by buses are included in the road account. Within the available investment time series it was not possible to calculate the capital stock value for tram and trolley bus transport. Nor running costs could be estimated.

### 4.1.5	Aviation infrastructure

Due to data problems it was not possible to estimate the capital stock and capital costs. The infrastructure running costs for the Estonian airports were €1.4 million in 1998 and €3.5 million in 1996 excluding the non-transport related running costs. The corresponding investments were €1.9 million in 1998 and €3.0 million in 1996.

### 4.1.6	Waterborne transport infrastructure

Due to data problems it was not possible to estimate the capital stock and capital costs. The infrastructure running costs for the Estonian maritime transport - without ports - were €9.2 million in 1998 and €6.2 million in 1996.  The corresponding investments were €5.7 million in 1998 and € 4.8 million in 1996. The Maritime Board used also €1.2 million in 1998 and €1.1 million in 1996 for the running costs of ferry ports for domestic transport.

## 4.2	Supplier operating costs

For the Estonian pilot account we analysed the national rail carriers Estonian Railways Ltd, Edelaraudtee Ltd and public transport companies (tram, trolley buses, and buses).

### 4.2.1	National rail carriers – Estonian Railways Ltd and Edelaraudtee Ltd

Supplier operating costs for Estonian Railways Ltd and Edelaraudtee Ltd were €88.7 million in 1998. This figure does not include any compensation for the use of tracks and stations.

### 4.2.2	Public transport

Total transport costs of local passenger transport by buses, tram and trolley buses in Estonia were €41.8 million in 1998 and €35.8 million in 1996. In principle these figures can be considered to represent actual supplier operating costs (including costs for vehicles, personnel and administration). However, it is possible that also infrastructure costs for tram and trolley bus networks are included.

## 4.3	Delay costs due to congestion

These costs could not be estimated.

## 4.4	Accident costs

### 4.4.1	Results for 1998- total costs by category

Table 26 and Table 27 present total internal and external accident costs for Estonia by transport mode. Total social costs of accidents amounted in 1998 to € 519 million. Total internal accident costs were € 484 million, of which 96 per cent were due to road transport. Because internal accident costs are carried by the transport user or the community of transport users the interpretation of the results for accident costs should be based on the total external accident costs that are carried by society as a whole. Total external accident costs for Estonia amounted to €34 million in 1998 with 99 per cent to be allocated to road transport.

The most important cost driver is the risk value, which accounted in 1998 for 91% of total costs, followed by production losses (6%) and material damages (2%). The costs arising from medical treatment and administration were of minor importance.

Table 26. Total internal and external accident costs in Estonia 1998 by cost category and mode

| Mode | Total internal and external accident costs by category in € million |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Total | Material damage | Administra- tive costs | Medical treatment | Production losses | Risk Value |
| Road | 498.4 | 12.7 | 0.1 | 3.9 | 29.7 | 452.0 |
| Rail | 15.4 | : | : | : | 0.3 | 15.1 |
| Aviation | 0 | 0 | 0 | 0 | 0 | 0 |
| Inland navigation | 1.3 | : | : | : | : | 1.3 |
| Maritime shipping | 2.7 | : | : | : | 0.1 | 2.6 |
| Total | 518.5 | 12.7 | 0.1 | 3.9 | 30.1 | 471.7 |
| Source: JP-Transplan Ltd |  |  |  |  |  |  |

As already mentioned, the risk value was responsible for 91% of total accident costs in 1998. This seemingly high ratio is explained mainly by the high priority society places on improved traffic safety. The value of a statistical life of around € 0.65 million per fatality lies more at the lower end of possible values (Nellthorp et al. 2001). On the other hand it has to be noticed that a part of data is missing (cf. Table 1) and some unit costs are probably too low (cf. section 3.4).

Table 27. Total internal and external accident costs in Estonia 1998 by mode

| Mode | € million |  |
| --- | --- | --- |
|  | Internal | External |
| Road transport | 464.7 | 33.7 |
| Rail transport | 15.1 | 0.3 |
| Aviation | 0 | 0 |
| Inland navigation | 1.3 | 0 |
| Maritime shipping | 2.6 | 0.1 |
| Total | 483.7 | 34.1 |
| Source: JP-Transplan Ltd |  |  |

### 4.4.2	Allocation of total costs to modes and types of infrastructure

The question of responsibility in the field of traffic accidents is a very complex one. Official records from traffic police and insurance companies naturally relate the definition of “responsibility” under the current national legislation framework. However, suitable data on the distribution of accident responsibilities are not available. Whilst a principle of UNITE is to avoid arbitrary cost allocation, we were not able to produce a responsibility - coverage matrix as proposed in “Accounts Approach for Accidents”, Doll et al. (2000) for Estonia. Neither couldwe allocated accident costs to vehicle types as suggested in “The Accounts Approach” (Link et al. 2000).

### 4.4.3	Average costs in 1998

Average costs were calculated based on vehicle-km (road), train-km, and aircraft movements (arriving or departing). Vessel-kilometres were not available, nor necessary data for public transport. A breakdown to pkm / tkm was not made. The results of the average cost estimates are presented in Table 28.

Table 28. Average external accident costs in Estonia 1998

|  | Unit | Average external accident costs |
| --- | --- | --- |
| Road transport | €/ 1000 vehicle-km | 5.4 |
| Rail transport | € / 1000 train-km | 44.8 |
| Source: JP-Transplan Ltd |  |  |

## 4.5 Environmental Costs

### 4.5.1	Results for 1998

Table 29 presents the environmental costs of transport in Estonia for the year 1998. The costs of air pollution and global warming include both direct transport emissions and indirect emissions from fuel production and fuel supply chain. The indirect emissions of Estonian electricity production are not included due to missing emission data from power plant operation. Noise costs are not assessed neither the costs of nature, landscape and soil and water pollution.

Air pollution and global warming are the major environmental costs of transport in Estonia, with road transport as the dominant source of the costs. Air pollution and its local health effects (morbidity and mortality) in particular, is a significant detriment. The costs of global warming are also high, but the impacts are not experienced as directly as the impacts of air pollution.

Freight transport by road causes about half of the total environmental costs of the transport sector, but the share of passenger transport by road is almost as big. In comparison, the costs of rail transport as well as aviation are low.

Maritime shipping is not considered in the country accounts of UNITE.

In general, the cost information produced in UNITE is subject to uncertainty due to methodological imperfections, as well as variation in the empirical results on environmental costs in the various cost categories. The valuation of local and regional air pollution is relatively well established. Concerning global warming, the damage and avoidance cost estimates per a ton of CO2 vary considerably both below and above the unit cost (€ 20/ton) applied in UNITE.

Table 29. Environmental costs for Estonia 1998

|  | € million |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Mode | Air Pollution | Global Warming1) | Noise | Nuclear Risks | Total (excl. fixed environmental costs) | Nature, Landscape, Soil and Water pollution |
| Road transport |  |  |  |  |  |  |
| Passenger | 62.0 | 25.7 | : | - | 87.7 | : |
| Freight | 68.9 | 24.3 | : | - | 93.2 | : |
| Rail transport |  |  |  |  |  |  |
| Passenger | 0.9 | 0.3 | : | - | 1.2 | : |
| Freight | 5.1 | 1.5 | : | - | 6.6 | : |
| Aviation 2) |  |  |  |  |  |  |
| Airports & Flights3) | 1.5 | 0.8 | : | - | 2.3 | : |
| Total | 138.4 | 52.6 | : | - | 191.0 | : |
| - Direct emissions + emissions from fuel chain. – 2) For the “Estonian flight information region”. - 3) Underestimation due to missing PM. Source: IER, JP-Transplan Ltd |  |  |  |  |  |  |

As presented in Table 30, major part of the cost of air pollution and global warming from road transport is attributed to goods transport. Within the road transport sector, 71 % of all emission related costs are attributed to air pollution (mainly health impacts) and 29 % to the impacts of global warming.

Table 30. Environmental costs for road transport in Estonia in 1998 (excluding noise costs, nature, landscape, soil and water pollution) by vehicle type

| Vehicle type | € million |  |  |
| --- | --- | --- | --- |
|  | Air Pollution | Global Warming3 | Total |
| Motorcycles1 | : | : | : |
| Passenger Cars | 53.8 | 22.6 | 76.4 |
| Buses2 | 8.2 | 3.1 | 11.4 |
| Goods Vehicles | 68.9 | 24.3 | 93.2 |
| Total | 130.9 | 50.1 | 181.0 |
| 1 No data available. 2 Including urban and inter-urban buses. 3 Direct emissions + emissions from fuel chain. Source: IER, JP-Transplan Ltd |  |  |  |

According to Table 31, two thirds of the emission costs of road transport are caused on rural roads.

Table 31. Environmental costs (direct and indirect emissions) by vehicle and road type for road transport in Estonia in 1998 (excluding noise costs, nature, landscape, soil and water pollution)

| Vehicle category | € million |  |  |
| --- | --- | --- | --- |
|  | Urban roads | Rural roads | All roads |
| Motorcycles | : | : | : |
| Passenger Cars | 31.9 | 44,5 | 76,4 |
| Buses | 5,7 | 5,7 | 11,3 |
| Goods Vehicles | 23,5 | 69,8 | 93,2 |
| Total | 61,1 | 119,9 | 181,0 |
| Source: IER, JP-Transplan Ltd |  |  |  |

In Table 32, the environmental costs (excluding nature, landscape and soil and water pollution) of transport are averaged by vehicle performance. The average passenger car causes the lowest environmental cost in all cost categories. The emission costs of the average bus and the average goods vehicle are of the same magnitude.

In rail transport, the environmental costs of freight trains are more than double compared to the costs of passenger trains.

It should be noted, that this presentation is an average of all vehicle and fuel types, and urban and non-urban environments. Furthermore, the averaging conceals differences in vehicle occupancy rates or payload factors. Thus, comparisons between modes must be considered only as generalizations, not exact ratios of environmental performance. It must be noted further, that road vehicle performance and train performance are not directly comparable.

Table 32. Average environmental costs for Estonia 1998

| Mode | € per 1000 vehicle/train-km |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Air Pollution | Global Warming | Noise | Nuclear Risks | Total |
| Road |  |  |  |  |  |
| Motorcycles | : | : | : | - | : |
| Passenger Cars1 | 11.3 | 4.7 | : | - | 16.0 |
| Buses1 | 48.5 | 18.3 | : | - | 66.9 |
| Goods Vehicles1 | 51.0 | 18.0 | : | - | 69.0 |
| Rail2 |  |  |  |  |  |
| Passenger Transport | 333.3 | 111.1 | : | - | 444.4 |
| Freight Transport | 1225.0 | 375.0 | : | - | 1600.0 |
| 1 Average for petrol and diesel & urban and non-urban. 2 Per train-km. Only diesel trains included. Source: IER, JP-Transplan Ltd |  |  |  |  |  |

### 4.5.2	Account years 1996 and 2005

The costs of air pollution and global warming risks for the accounting years 1996 and 2005 are presented in Table 33 and Table 34. The impact of economic growth on the environmental costs is taken into consideration as stated in section 3.5.6.

For road, rail and aviation, the environmental costs presented for 2005 are based on projections of growth in emissions. Besides assuming changes in the volume of transport, these projections take into consideration also the effect of expected improvements in fuel quality, engine technology and emission abatement technology (fleet composition).

The environmental costs for Estonia for 1996 are generally somewhat lower compared to 1998. This is explained by less traffic. Between 1996 and 1998 very little technical development, which would have reduced (slowed down) the growth of emission, has taken place.

Table 33. Environmental costs (excluding noise costs, nature, landscape, soil and water pollution) for Estonia in 1996

|  | € million |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Air Pollution | Global Warming | Noise | Nuclear Risks | Total |
| Road |  |  |  |  |  |
| Passenger Transport | 45.9 | 20.1 | : | - | 66.0 |
| Freight Transport | 60.4 | 21.8 | : | - | 82.2 |
| Rail |  |  |  |  |  |
| Passenger Transport | 2.31 | 0.71 | : | - | 3.02 |
| Freight Transport | 2.89 | 0.89 | : | - | 3.78 |
| Aviation |  |  |  |  |  |
| Airports & Flights | 1.28 | 0.73 | - | - | 2.01 |
| Total | 112.8 | 44.3 |  |  | 157.1 |
| Source: IER, JP-Transplan Ltd |  |  |  |  |  |

For the year 2005, there are increases in environmental costs in almost all cost categories. Besides attributing to a change in the value of money, the actual cost changes result mainly on the increase of person transport.

Table 34. Environmental costs for Estonia in 2005

| Mode | € million |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Air Pollution | Global Warming | Noise | Nuclear Risks | Total |
| Road transport |  |  |  |  |  |
| Passenger | 120.0 | 59.1 | : | - | 179.1 |
| Freight | 50.3 | 28.1 | : | - | 78.4 |
| Rail | : | : | : | - | : |
| Aviation Airports & Flights |  |  |  |  |  |
|  | 2.15 | 1.23 | : | - | 3.39 |
| Total | 172.5 | 88.32 | : | - | 260.8 |
| Source: IER, JP-Transplan Ltd |  |  |  |  |  |

## 4.6	Taxes, charges, subsidies

This section reports on the transport related taxes and charges, which can be compared with the related costs. Furthermore, as far as the available data did allow to do so, subsidies were quantified. All revenues are expressed in 1998 prices.

### 4.6.1	Road transport

Table 35 shows the revenues related to road infrastructure costs for 1996 and 1998. Revenues from fuel tax, motor vehicle tax, registration tax and annual vehicle tax amounted in 1998 to € 82.6 million. If VAT to be paid on top of the fuel and motor vehicle tax is considered too, revenues amounted to € 97.5 million. Compared to 1996 revenues increased by 67 per cent.

Table 35. Road Transport Revenues in Estonia 1996 and 1998

| Tax category | € million at 1998 prices |  |
| --- | --- | --- |
|  | 1996 | 1998 |
| Fuel tax | 38.6 | 72.6 |
| Motor vehicle tax | 7.8 | 6.5 |
| Registration tax | 1.1 | 1.9 |
| Annual vehicle tax | 1.9 | 1.6 |
| Total | 49.5 | 82.6 |
| Source: DHV and LT Consultants (1999) |  |  |

Annual vehicle tax is gathered by the municipalities and all other taxes by the national government.

### 4.6.2	Rail transport

Estonian Railways Ltd and Edelaraudtee Ltd received the major part of their tariff revenues from freight transport (Table 36). During the 90s Estonian railways went through many reorganising phases.  In 1996 the national railway organisation - Estonian Railways - received €8 million from passenger and €44 million (at 1998 price level) from freight transport. So, the incomes from freight transport had increased almost by half from 1996 to 1998.

Table 36. Revenues from tariffs in rail transport in 1998

| Type of revenue/ type of transport | Railway use fee2) | Tariff revenues1) | Fuel tax | VAT on fuel tax |
| --- | --- | --- | --- | --- |
| Passenger transport | 0 | 7.1 | : | : |
| Freight transport | 0 | 64.2 | : | : |
| Total | 0 | 71.3 | 0.43 | 0.078 |
| 1) Excluding subsidies and VAT. 2) Taken in use from the year 2000. |  |  |  |  |
| Source: MTC (2000), DHV and LT Consultants (1999), JP-Transplan Ltd |  |  |  |  |

Estonian Railways Ltd and Edelaraudtee Ltd received €6.3 million in 1998 and €2.8 million in 1996 (at 1998 price level) subsidies for passenger transport. Possible subsidies for infrastructure financing are – like in all modes – considered at the cost side.

### 4.6.4	Public transport excluding rail

Table 37 summarises the revenues (including reimbursements for concessionary fares) for public transport excluding rail.

Table 37. Tariff revenues and subsidies of public transport (excl. rail)  in 1996 and 1998

| Type of revenue | € million at 1998 prices |  |
| --- | --- | --- |
|  | 1996 | 1998 |
| Tariff revenues | : | 19.6 |
| Subsidies from state | 9.0 | 7.0 |
| Subsidies from municipalities | 9.5 | 14.5 |
| Total subsidies | 18.5 | 21.5 |
| Sources: MTC (2000). |  |  |

### 4.6.5	Aviation

Estonian Air Navigation Services Ltd received €7,3 million as navigation fees in 1998.

Tallinn Airport Ltd had net sales and revenues of €7.9 million in 1998.

Direct subsidies in aviation are modest (Table 38), but indirect subsidies due to tax exemptions in international transport are bigger. However the last ones could not be estimated due to the lack of data.  Possible subsidies for infrastructure financing are – like in all modes – considered at the cost side.

Table 38. Revenues and subsidies of the Estonian government in aviation in 1998

| Type of revenue or subsidy | € million |
| --- | --- |
|  | 1998 |
| Income tax from Estonian Air Navigation Services Ltd | 0.07 |
| Owners profit from Estonian Air Navigation Services Ltd | 0.19 |
| Income tax from Tallinn Airport Ltd | 0.07 |
| Fuel tax on kerosene in domestic transport | 0.076 |
| VAT on fuel tax on kerosene in domestic transport | 0.014 |
| Total revenues | 0.42 |
| Subsidies on domestic passenger transport | 0.49 |
| Source: MTC (2000), DHV and LT Consultants (1999) |  |

### Waterborne transport

Taxes on fuel oil used in waterborne transport were € 0.8 million in 1998.

The pilotage-, beacon- and icebreaking fees were €11.5 million in 1998.

The ferry companies received a subvention of €2.8 million in 1998.

# 5	Pilot accounts for Estonia

In order to obtain a clear picture of the transport situation in Estonia, basic social and economic indicators are presented in Table 39 before the detailed results of the Estonian pilot accounts are discussed.

Table 39. Basic indicators for Estonia in 1996 and 1998

|  | Unit | 1996 | 1998 |
| --- | --- | --- | --- |
| Land area | Sqkm | 45 227 | 45 227 |
| Population | 1 000 | 1 469 | 1 449 |
| Population density | Inhabitants/sqkm | 32 | 32 |
| Population employed | % | 61.3 | 58.1 |
| GDP per capita1) | € | : | 9 193 |
| GDP growth rate (change to previous year) | % | 4.0 | 5.5 |
| Annual inflation | % | 23.1 | 8.2 |
| 1) OECD GDP per Capita, PPP-adjusted (Nettthorp et al, 2001). Sources: Statistical yearbook for Estonia 1999, 2000, DHV and LT Consultants (1999). |  |  |  |

In Table 40, basic transport indicators used within the Estonian pilot accounts are presented.

Table 40. Basic transport related indicators for Estonia in 1998 per mode

| Indicator | Unit | Road | Rail | Public transport1) | Aviation | Maritime shipping | Total |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Transport performance |  |  |  |  |  |  |  |
| Passengers carried | Mill. | 1892) | 7 | 69.5 | 0.58 | 5.6 | 272 |
|  | % | 70 | 3 | 26 | 0.2 | 2 | 100 |
| Passenger-km | Mill. Pkm | 10 7603) | 237 | 268 | 227 | 359 | 11 851 |
|  | % | 91 | 2 | 2 | 2 | 3 | 100 |
| Goods transported | Mill. t | 12 | 32 | • | 0 | 27 | 71 |
|  | % | 17 | 45 | • | 0 | 38 | 100 |
| Tonne-km | Mill. tkm | 1 230 | 5 788 | • | : | : | : |
|  | % | : | : | • | : | : | : |
| Network length | 1000 km | 43.8 | 0.84) | 0.18 | • | • | • |
| Employees | 1000 | : | 6.45) | : | : | : | : |
| Gross investments | € mill. | 7.96) | 16.45) | : | 1.9 | 6.97) | 33.1 |
|  | % | 24 | 50 | : | 6 | 21 | 100 |
| Gross capital stock | € mill. | : | : | : | : | : | : |
|  | % | : | : | : | : | : | : |
| Accidents |  |  |  |  |  |  |  |
| Number of injuries | Casualties | 1 990 | 23 | : | 0 | 0 | 2013 |
| Number of fatalities | Fatalities | 284 | 21 | : | 0 | 4 | 309 |
| Environment |  |  |  |  |  |  |  |
| Direct transport emissions |  |  |  |  |  |  |  |
| CO2 | 1000 t | 1 724 | 79 | : | 42 | 33 | 1 878 |
| PM10 or below | tons | 1 837 | 147 | : | : | 18 | 2 002 |
| NOx | tons | 27 665 | 1 527 | : | 456 | 763 | 30 411 |
| SO2 | tons | 498 | 99 |  | 4 | 120 | 721 |
| NMVOC | tons | 21 502 | 265 |  | 132 | 28 | 21 927 |
| 1) Tram and trolley buses only. – 2) Buses and taxis only. – 3) Private cars, buses and taxis. – 4) Public tracks only. – 5) Estonian Railways Ltd and Edelaraudtee Ltd. - 6) Estonian state road network (16 400 km) only. - 7) Coastal waterways and ferry harbours for domestic transport. Source: Statistical yearbook for Estonia 1999 and 2000, MTC (2000), TTU, IER, JP-Transplan Ltd |  |  |  |  |  |  |  |

## 5.1	Road transport

In Table 41 the total costs of road transport documented within the Estonian pilot account are presented.

Table 41. Estonian road account for 1996, 1998 and 2005 - € million at 1998 prices

| Costs |  |  |  |
| --- | --- | --- | --- |
| Core information | 1996 | 1998 | 2005 |
| Infrastructure Costs 1) | 58 | 57 | : |
| Fixed | : | : | : |
| Variable | : | : | : |
| Accident costs (external)2) | : | 34 | : |
| Environmental costs |  |  |  |
| Air pollution | 106 | 131 | 170 |
| Global warming | 42 | 50 | 87 |
| Noise | : | : | : |
| Total | : | 272 | : |
| Additional information |  |  |  |
| Congestion costs | : | : | : |
| Time costs |  |  |  |
| Fuel costs |  |  |  |
| Accident costs (internal)3) | : | 465 | : |
| From this: risk value | : | 452 | : |
| Environmental costs |  |  |  |
| Nature and landscape, soil and water pollution | : | : | : |
| Nuclear risk | - | - | - |
| Revenues |  |  |  |
| Directly related to a specific cost category |  |  |  |
| Charges for infrastructure usage | 0 | 0 | 0 |
| Fixed |  |  |  |
| Variable |  |  |  |
| Total | 0 | 0 | 0 |
| Other transport specific revenues |  |  |  |
| Fuel tax | 38.6 | 72.6 | : |
| Motor vehicle tax | 7.8 | 6.5 | : |
| Registration tax | 1.1 | 1.9 | : |
| Annual vehicle tax | 1.9 | 1.6 | : |
| VAT 4) | 8.4 | 14.2 | : |
| Total | 57.8 | 96.8 | : |
| Subsidies5) | 0 | 0 | 0 |
| 1) All roads. Running costs only, including VAT. - 2) Refers to those parts of accident costs which are not borne by road users and insurance companies but by the public sector and third parties. - 3) ) Refers to those parts of accident costs which are borne by road users and insurance companies. - 4) VAT levied on fuel and motor vehicle tax. Sources: JP-Transplan Ltd |  |  |  |

In 1998, the core year of the pilot accounts, the largest cost block was accident costs. Total accident costs amounted to €498 million, out of these 7 % (€34 million) were external accident costs, i.e. those parts of accident costs which are not borne by road users themselves or by transport insurance companies. Environmental costs (excluding costs for noise, nature, landscape, soil and water pollution) were the second largest cost block (€181 million). Infrastructure running costs amounted to €57 million.

For 2005, we have estimated that environmental costs increase heavily (42%).

On the revenue side we have estimated road transport related revenues of €97 million in 1998. This is almost double compared to the year 1996. Charges, which relate directly to infrastructure usage were not in use.

Comments on specific cost categories

Infrastructure costs

Road capital stock could not be calculated because of missing data on investment time series. This is due to administrative changes in 1991 when Estonia gained back its independence. Running costs were obtained from DHV and LT Consultants (1999). Cost allocation to vehicle types could not be made.

Congestion costs

Congestion costs, which in the UNITE accounts refer to costs of delay (e.g. time and fuel costs) and not to the deadweight welfare loss of congestion, are a minor issue in Estonia. National estimates could not be made, because necessary data was not available.

Accident costs

The input data for estimating road accident costs was obtained from a recent study (COWI and TTU, 2001). The costs are extremely dependant on the valuation of risk which was standardised within the UNITE project. For Estonia we used a risk value for fatalities of € 0.65 million. 15% of this was applied for severe injuries and 1% of this for minor injuries.

The estimated external road accident costs of €34 million are composed as follows: 88 % are attributed to production losses and 12 % to medical treatment, costs arising for administration are of minor importance only. Risk value accounted for 97 % of internal accident costs and material damage 3 %. Material damage was only estimated for damage to vehicles, making this cost component wholly internal to the transport sector. Due to the lack of input data no further valuation of material damages could be estimated.

Environmental costs

Costs of air pollution and global warming have been estimated. Costs of noise, nature, landscape and soil and water pollution could not be estimated due to the lack of data.

Environmental costs were calculated using the methodology suggested in Link et al. (2000) and summarised in chapter 3. For road transport, the total environmental costs of emissions amounted to €181 million in 1998. These costs represent 95 % of the respective environmental costs of all transport modes and reflect the dominant role of road transport within Estonia.

Air pollution is the major environmental cost component for road transport (€132 million), with the costs of nitrogen oxides being the major cost driver. The second highest environmental cost for Estonia, costs related to global warming (€50 million), reflects the currently accepted avoidance costs of CO2 emissions. For UNITE, a shadow value of a € 20/emitted tonne of CO2 was used. This reflects the costs of meeting the Kyoto targets (Fahl et. al., 1999, Duerinck, 2000) and lies within a range of values of € 5 to € 38 per tonne of CO2 avoided as presented by Capros and Mantzos (2000). This value could rise in the future if, e.g. a 50% CO2 reduction target for 2030 (Intergovernmental Panel on Climate Change - IPCC) is accepted.

The costs of nuclear risk from the use of vehicles driven by electricity do not exist in Estonia, because all electricity is produced from oil shale.

Taxes, charges and subsidies

The input data was obtained from a recent study by DHV and LT Consultants (1999).

In the Table 42 the fully allocated costs of road transport for all roads and vehicle types is presented. From Table 43 to Table 45 the total costs of road transport are shown per road type (all roads, inter-urban roads and urban roads) and disaggregated by vehicle type (motorcycles, passenger cars, buses, goods vehicles). Only the full allocation of environmental costs (emissions) was possible in all presentations.

Table 42. Fully allocated costs of road transport per vehicle km in Estonia in 1998 - € per 1000 vehicle km at 1998 prices

| All Roads |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1998 |  |  |  |  |  |  |
|  | Motor-cycles | Passenger cars |  | Buses |  | LGV | HGV |
| Core information |  |  |  |  |  |  |  |
| Infrastructure costs | : | : |  | : |  | : | : |
| Fixed | : | : |  | : |  | : | : |
| Variable | : | : |  | : |  | : | : |
| External accident costs | : | : |  | : |  | : | : |
| Environmental costs |  |  |  |  |  |  |  |
| Air pollution | : | 11.3 |  | 59.7 |  | 51.0 |  |
| Global warming | : | 4.7 |  | 18.3 |  | 18.0 |  |
| Noise | : |  |  |  |  |  |  |
| Total I |  |  |  |  |  |  |  |
| Additional information |  |  |  |  |  |  |  |
| Delay costs |  |  |  |  |  |  |  |
| Internal accident costs |  |  |  |  |  |  |  |
| Material damages |  |  |  |  |  |  |  |
| Risk value |  |  |  |  |  |  |  |
| Environmental costs |  |  |  |  |  |  |  |
| Nature, landscape, soil and water pollution | : | : |  | : |  | : | : |
| Nuclear risk | 0 | 0 |  | 0 |  | 0 | 0 |
| Total II |  |  |  |  |  |  |  |
| Revenues |  |  |  |  |  |  |  |
| Fixed |  |  |  |  |  |  |  |
| Motor vehicle tax |  |  |  |  |  |  |  |
| Registration tax |  |  |  |  |  |  |  |
| Annual vehicle tax |  |  |  |  |  |  |  |
| VAT1) |  |  |  |  |  |  |  |
| Variable |  |  |  |  |  |  |  |
| Fuel tax |  |  |  |  |  |  |  |
| VAT2) |  |  |  |  |  |  |  |
| Basic data |  |  |  |  |  |  |  |
| Million vehicle km | : | 4 763 |  | 169 |  | 840 | 510 |
| Million passenger km |  | 8 335 |  |  | 2 425 |  |  |
| Million tonne km |  |  |  |  |  | 1230 |  |
| 1) VAT on motor vehicle tax. - 2) VAT on fuel tax Sources: JP-Transplan Ltd |  |  |  |  |  |  |  |

Table 43. Total costs of road transport in Estonia in 1998 by vehicle category - € million at 1998 prices

| All Roads |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1998 |  |  |  |  |  |
|  | Motor-cycles | Passenger cars | Buses | LGV | HGV | Total |
| Core information |  |  |  |  |  |  |
| Infrastructure costs | : | : | : | : | : | 57 |
| Fixed | : | : | : | : | : | : |
| Variable | : | : | : | : | : | : |
| External accident costs1) |  |  |  |  |  |  |
| Administrative |  |  |  |  |  | 0.1 |
| Health costs |  |  |  |  |  | 3.9 |
| Production loss |  |  |  |  |  | 29.7 |
| Environmental costs |  |  |  |  |  |  |
| Air pollution | : | 53.8 | 8.2 | 68.9 |  | 131 |
| Global warming | : | 22.6 | 3.1 | 24.3 |  | 50 |
| Noise | : |  |  |  |  |  |
| Total I |  |  |  |  |  | 272 |
| Additional information |  |  |  |  |  |  |
| Delay costs | : |  | : | : | : | : |
| Internal accident costs 2) |  |  |  |  |  |  |
| Material damages |  |  |  |  |  | 12.7 |
| Risk value |  |  |  |  |  | 452.0 |
| Environmental |  |  |  |  |  |  |
| Nature, landscape, soil and water pollution |  |  |  |  |  |  |
| Total II |  |  |  |  |  | 465 |
| Revenues |  |  |  |  |  |  |
| Fixed |  |  |  |  |  |  |
| Motor vehicle tax |  |  |  |  |  | 6.5 |
| Registration tax |  |  |  |  |  | 1.9 |
| Annual vehicle tax |  |  |  |  |  | 1.6 |
| VAT1) |  |  |  |  |  | 1.2 |
| Variable |  |  |  |  |  |  |
| Fuel tax |  |  |  |  |  | 72.6 |
| VAT2) |  |  |  |  |  | 13.1 |
| Total |  |  |  |  |  | 97 |
| Basic data |  |  |  |  |  |  |
| Number of vehicles (1000) | : | 448 700 | 6 300 | 80 400 |  | 535 400 |
| Million vehicle km | : | 4 763 | 169 | 840 | 510 | 6 282 |
| Million passenger km |  | 8 335 | 2 425 | . | . | 10 760 |
| Million tonne km | • | • | • | 1 230 |  | 1 230 |
| 1) VAT levied on motor vehicle tax. – 2) VAT levied on fuel tax. Source: JP-Transplan Ltd |  |  |  |  |  |  |

Table 44. Total costs of road transport in Estonian inter-urban roads in 1998 - € million at 1998 prices

| Inter-urban roads |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1998 |  |  |  |  |  |
|  | Motor-cycles | Passenger cars | Buses | LGV | HGV3) | #### Total |
| Core information |  |  |  |  |  |  |
| Infrastructure costs |  |  |  |  |  |  |
| Fixed | : | : | : | : | : | : |
| Variable | : | : | : | : | : | : |
| External accident costs |  |  |  |  |  |  |
| Administrative |  |  |  |  |  |  |
| Health costs |  |  |  |  |  |  |
| Production loss |  |  |  |  |  |  |
| Environmental costs |  |  |  |  |  |  |
| Air pollution | : | 31.5 | 3.8 | 50.8 |  | 86 |
| Global warming2) | : | 13.0 | 1.9 | 19.0 |  | 34 |
| Noise4) | : | : | : | : | : | : |
| Total I |  |  |  |  |  |  |
| Additional information |  |  |  |  |  |  |
| Delay costs1) |  |  |  |  |  |  |
| Internal accident costs |  |  |  |  |  |  |
| Material damages |  |  |  |  |  |  |
| Risk value |  |  |  |  |  |  |
| Environmental costs |  |  |  |  |  |  |
| Nature, landscape, soil and water pollution6) | : | : | : | : | : | : |
| Total II |  |  |  |  |  |  |
| Revenues |  |  |  |  |  |  |
| Fixed |  |  |  |  |  |  |
| Motor car and motorcycle tax | : | : | • | • | • |  |
| Motor vehicle tax | : | : | : | : | : |  |
| Vehicle tax |  |  |  |  |  |  |
| Variable |  |  |  |  |  |  |
| Fuel tax | : | : | : | : | : | : |
| VAT | : | : | : | : | : |  |
| Total |  |  |  |  |  |  |
| Basic data |  |  |  |  |  |  |
| Million vehicle km | : | 2 995 | 114 | 1 111 |  | 4 220 |
| Source: JP-Transplan Ltd |  |  |  |  |  |  |

Table 45. Total costs of road transport in Estonian urban roads in 1998 - € million at 1998 prices

| Urban roads |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1998 |  |  |  |  |  |
|  | Motor-cycles | Passenger cars | Buses | LGV | HGV | Total |
| Core information |  |  |  |  |  |  |
| Infrastructure costs | : | : | : | : | : | : |
| Fixed |  |  |  |  |  |  |
| Variable |  |  |  |  |  |  |
| External accident costs | : |  | : | : | : | : |
| Administrative |  |  |  |  |  |  |
| Health costs |  |  |  |  |  |  |
| Production loss |  |  |  |  |  |  |
| Environmental costs |  |  |  |  |  |  |
| Air pollution | : | 22.3 | 4.4 | 18.1 |  | 45 |
| Global warming | : | 9.6 | 1.3 | 5.4 |  | 16 |
| Noise) | : |  |  |  |  |  |
| Total I |  |  |  |  |  |  |
| Additional information |  |  |  |  |  |  |
| Delay costs | : |  | : | : | : | : |
| Internal accident costs | : |  | : | : | : | : |
| Material damages |  |  |  |  |  |  |
| Risk value |  |  |  |  |  |  |
| Environmental costs |  |  |  |  |  |  |
| Nature, landscape, soil and water pollution | : | : | : | : | : | : |
| Total II |  |  |  |  |  |  |
| Revenues |  |  |  |  |  |  |
| Fixed |  |  |  |  |  |  |
| Motor car and motorcycle tax |  |  |  |  |  |  |
| Motor vehicle tax | : | : | : | : | : |  |
| Vehicle tax | : | : | : | : | : |  |
| Variable |  |  |  |  |  |  |
| Fuel tax | : | : | : | : | : |  |
| VAT | : | : | : | : | : |  |
| Total |  |  |  |  |  |  |
| Basic data |  |  |  |  |  |  |
| Million vehicle km | : | 1 768 | 55 | 239 |  | 2 062 |
| Source: JP-Transplan Ltd |  |  |  |  |  |  |

## Rail transport

In Table 46 the total costs of rail transport documented within the Estonian pilot account are presented.

Table 46. Estonian rail account for Estonian Railway Administration, Estonian Railways Ltd and Edelaraudtee Ltd in 1996 and1998 - € million at 1998 prices

| Costs |  |  |
| --- | --- | --- |
| Core information | 1996 | 1998 |
| Infrastructure costs 1) | : | 13.1 |
| Fixed | : | : |
| Variable | : | : |
| Supplier operating costs | : | 88.7 |
| Out of these: Track & station charges2) | 0 | 0 |
| Accident costs (external) | : | 0.3 |
| Environmental costs3) |  |  |
| Air pollution | 5.2 | 6.0 |
| Global warming | 1.6 | 1.8 |
| Noise | : | : |
| Total | : | 109.7 |
| Additional information |  |  |
| Congestion costs | : | : |
| Accident costs (internal) | : | 15.1 |
| From this: risk value | : | 15.1 |
| Environmental costs3) |  |  |
| Nature and landscape, soil and water pollution | : | : |
| Nuclear risk | - | - |
| Revenues |  |  |
| Directly related to a specific cost category |  |  |
| Track & station charges2) | 0 | 0 |
| Fixed |  |  |
| Variable |  |  |
| User tariffs4) | 52 | 71.3 |
| Total | 52 | 71.3 |
| Other transport specific revenues |  |  |
| Fuel tax | : | 0.4 |
| VAT 5) | : | 0.1 |
| Total | : | 0.5 |
| Subsidies, for passenger transport 6) | 2.8 | 6.3 |
| Non-transport related revenues of rail companies | : | 2.3 |
| 1) Running costs only. – 2) Track and station charges were applied from the year 2000 onwards. – 3) Only diesel pull. - 4) Subsidies and VAT are excluded. 5) VAT levied on fuel tax– 6) Subsidies included here refer to subsidies given for the provision of passenger rail services. These subsidies can clearly not be allocated to either the cost or to the revenue side of this table. Subsidies are in cash flow terms and are not on the same basis as the economic costs. Sources: JP-Transplan Ltd |  |  |

As can be seen from Table 46, the largest cost blocks in the rail account in 1998 are supplier operating costs of Estonian Railways Ltd and Edelaraudtee Ltd (€88.7 million) and accident costs (€15.4 million respectively). Infrastructure costs amounted to €13.1 million. However, these include only running costs, because capital costs could not be estimated. Environmental costs (emissions and global warming) were at €7.6 million.

Total rail transport related revenues – including actually only user tariffs and excluding subsidies amounted in 1998 to €71.3 million. Subsidies for passenger transport amounted to €6.3 million.

Revenues from user tariffs increased from €52 million in 1996 to €71 million in 1998. This increase was totally due to freight transport. In passenger transport revenues decreased from €8 million in 1996 to €7 million in 1998, but governmental subsidies more than doubled from €2.8 million to €6.3 million.

Due to the lack of reliable data we were not able to estimate costs, revenues or subsidies for 2005.

Comments on specific cost categories

Infrastructure costs

Rail infrastructure costs include only running costs, because capital costs could not be estimated due to the lack of data on investment time series. Infrastructure costs could not be divided between passenger and freight transport (see Table 48).

Supplier operating costs

Data for supplier operating costs was received from MTC (2000).

Congestion costs

The delay costs for rail transport could not be estimated due to missing data.

Accident costs

The methodology explained in chapter 3 was used. As in road transport, the major accident component is the risk value. Production losses are the second major component. It should be remembered however, that because of the lack of data, material damages could not be estimated.

Environmental costs

The methodology used is as described in chapter 3. The costs related to noise pollution as well as to nature, landscape and soil and water pollution could not be estimated because of the lack of data. The costs of air pollution and global warming, are much lower in comparison to road transport. Only diesel pull is included. Electricity is produced in Estonia from oil shale, but no data was available about production emissions.

Taxes, charges and subsidies

Charges for access to tracks and stations were applied from the year 2000 onwards, so they were not in use in 1998.  Fuel tax paid on engine fuel is the only relevant tax for this mode.

The fully allocated costs of rail transport are shown in Table 47.

Table 47. Fully allocated costs of rail transport per train km in Estonia in 1998: Estonian Railway Administration, Estonian Railways Ltd and Edelaraudtee Ltd - € per train-km at 1998 prices

| Estonian Railway Adminstration, Estonian Railways Ltd and Edelaraudtee Ltd |  |  |
| --- | --- | --- |
|  | 1998 |  |
|  | Passenger | Freight |
| Core information |  |  |
| Infrastructure costs 1) | 1.96 |  |
| Fixed | : | : |
| Variable | : | : |
| External accident costs |  |  |
| Administrative | : |  |
| Health costs | : |  |
| Production loss | 0.045 |  |
| Environmental costs2) |  |  |
| Air pollution | 0.33 | 1.29 |
| Global warming | 0.11 | 0.38 |
| Noise | : | : |
| Total I |  |  |
| Additional Information |  |  |
| Delay costs |  |  |
| Internal accident costs |  |  |
| Material damages |  |  |
| Risk value | 2.26 |  |
| Environmental costs |  |  |
| Nature, landscape, soil and water pollution |  |  |
| Nuclear risk | - | - |
| Total II |  |  |
| Revenues |  |  |
| User tariffs | 2.60 | 16.21 |
| Track charges | 0 | 0 |
| Fuel tax |  | 0,064 |
| VAT3) | 0,012 |  |
| Subsidies |  | : |
| Direct funding | 2.30 | 0 |
| Concessionary VAT on tickets | - | - |
| Basic data |  |  |
| Train kilometre (million) only diesel pull | 1.63 | 3.96 |
| Train kilometre (million) diesel and electric pull | 2.73 | 3.96 |
| Passenger km (million) | 237 |  |
| Tonne km (million) |  | 5 788 |
| 1) Only running costs. – 2) Only diesel pull. - 3) VAT on fuel tax. Source: JP-Transplan Ltd |  |  |

Table 48 shows the total costs of rail transport disaggregated into passenger and freight transport.

Table 48. Total costs of rail transport in Estonia in 1998: Estonian Railway Administration, Estonian Railways Ltd and Edelaraudtee Ltd - € million at 1998 prices

|  | 1998 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Passenger |  | Freight | Total |
| Core information |  |  |  |  |
| Infrastructure costs 1) |  |  |  | 13.1 |
| Tracks |  |  |  |  |
| Fixed | : |  | : |  |
| Variable | : |  | : |  |
| Stations |  |  |  |  |
| Fixed | : |  | : |  |
| Variable | : |  | : |  |
| Supplier operating costs | : |  | : | 88.7 |
| Out of these: track + station charges | : |  | : | 0 |
| External accident costs |  |  |  |  |
| Administrative |  |  |  |  |
| Health costs |  |  |  |  |
| Production loss |  |  |  | 0.3 |
| Environmental costs2) |  |  |  |  |
| Air pollution | 0.9 |  | 5.1 | 6.0 |
| Global warming | 0.3 |  | 1.5 | 1.8 |
| Noise | : | : |  |  |
| Total I |  |  |  | 109.9 |
| Additional information |  |  |  |  |
| Delay costs | : |  | : | : |
| Internal accident costs |  |  |  |  |
| Material damages | : |  | : | : |
| Risk value | : |  | : | 15.1 |
| Environmental costs |  |  |  |  |
| Nature, landscape, soil and water pollution | : | : |  | : |
| Nuclear risk | - |  | - | - |
| Total II |  |  |  | 15.1 |
| Revenues |  |  |  |  |
| User tariffs | 7.1 |  | 64.2 | 71.3 |
| Track charges | 0 |  | 0 | 0 |
| Station charges | 0 |  | 0 | 0 |
| Fuel tax | : |  | : | 0.4 |
| VAT | : |  | : | 0.1 |
| Total | : |  | : | 71.8 |
| Subsidies 3) |  |  |  |  |
| Direct funding | 6.3 |  | 0 | 6.3 |
| Concessionary VAT on tickets |  |  |  |  |
| Basic data |  |  |  |  |
| Train kilometre (million) diesel pull | 1.63 |  | 3.96 | 5.59 |
| Train kilometre (million) diesel and electric pull | 2.73 |  | 3.96 | 6.69 |
| Passenger km (million) | 237 |  |  | 237 |
| Tonne km (million) |  |  | 5 788 | 5 788 |
| 1) Only running costs. – 2) Only diesel pull. - 3) VAT on fuel tax. Source: JP-Transplan Ltd |  |  |  |  |

## 5.3	Public transport

Table 49 shows the total costs of public transport.

Table 49. Estonian account for public transport in 1996 and 1998 - € million at 1998 prices

| Core information | Costs |  |
| --- | --- | --- |
|  | 1996 | 1998 |
| Infrastructure costs | : | : |
| Fixed | : | : |
| Variable | : | : |
| Supplier operating costs1) | 35.8 | 41.8 |
| Accident costs (external) | : | : |
| Environmental costs |  |  |
| Air pollution | : | : |
| Global warming | : | : |
| Noise | : | : |
| Additional information |  |  |
| Congestion costs | : | : |
| Accident costs (internal) | : | : |
| From this: risk value | : | : |
| Environmental costs |  |  |
| Nature and landscape, soil and water pollution | : | : |
| Nuclear risk | - | - |
| Revenues |  |  |
| Directly related to a specific cost category |  |  |
| Charges for infrastructure usage | 0 | 0 |
| User tariffs1) 2) | : | 19.6 |
| Other transport specific revenues |  |  |
| Fuel tax | : | : |
| VAT 3) | : | : |
| Subsidies, for passenger transport 4) | 18.5 | 21.5 |
| 1) Buses, trams and trolley buses. – 2) Subsidies and VAT are excluded. - 3) VAT levied on fuel tax– 4) Subsidies included here refer to subsidies given for the provision of public transport services. These subsidies can clearly not be allocated to either the cost or to the revenue side of this table. Subsidies are in cash flow terms and are not on the same basis as the economic costs. Sources: JP-Transplan Ltd |  |  |

It was not possible to elaborate a complete pilot account for this segment of the Estonian transport system. Infrastructure, congestion and accident costs, as well as environmental costs could not be quantified due to methodological and/or data problems. Note furthermore, that buses are included in the road account except for supplier operating costs, user tariffs and subsidies. It has to be remembered that tram and trolley bus services exist only in Tallinn city.

Charges for infrastructure use do not exist for tram and trolley bus infrastructure since these companies are vertically integrated.

Comments on specific cost categories

Infrastructure costs

Infrastructure costs for bus transport are included in the road account. Urban railway lines are included in the rail account. The estimation of tram and trolley bus lines only is considered in this account. No capital costs could be estimated for them.

Supplier operating costs

Data on supplier operating costs was received from MTC (2000).

Congestion costs

Congestion costs could not be estimated because of limited data.

Accident costs

Accident costs are included in the road account.

Environmental costs

Environmental costs for buses are included in the road account. The environmental costs associated with trams and trolley buses are only related to the production of electricity, and for this reason remain low. They could not be estimated because the emissions from electricity production are not known.

Taxes, charges and subsidies

Taxes and charges which can be directly allocated to infrastructure use do not exist in Estonia for public transport. Fuel taxation is included in the road account for buses and in the rail account for rail transport. Subsidies paid for the provision of public transport services are included.

The costs of public transport services could not be allocated per vehicle-km nor by vehicle type due to lack of data.

  - Aviation

Table 50 shows the total costs of air transport.

Table 50. Estonian air transport account for 1996 and 1998 - € million at 1998 prices

| Costs |  |  |  |
| --- | --- | --- | --- |
| Core information | 1996 | 1998 |  |
| Infrastructure costs1) | 3.5 | 1.4 |  |
| Fixed | : | : |  |
| Variable | : | : |  |
| Accident costs (external) | : | : |  |
| Environmental costs |  |  |  |
| Air pollution | 1.3 | 1.5 |  |
| Global warming | 0.7 | 0.8 |  |
| Noise | : | : |  |
| Additional information |  |  |  |
| Congestion costs | : | : |  |
| Accident costs (internal) | 0.7 | 0 |  |
| From this: risk value | 0.7 | 0 |  |
| Environmental costs |  |  |  |
| Nature and landscape, soil and water pollution | : | : |  |
| Nuclear risk | - | - |  |
| Revenues |  |  |  |
| Directly related to a specific cost category |  |  |  |
| Charges for infrastructure usage |  |  |  |
| Navigation fees2) | : | 7.3 |  |
| Airport revenues3) | : | 7.9 |  |
| Loss of revenues due to tax exemptions |  |  |  |
| Kerosene tax4) | : | : |  |
| VAT on ticket price4) | : | : |  |
| Other transport specific revenues |  |  |  |
| Fuel tax5) | : | 0.08 |  |
| VAT 6) | : | 0.01 |  |
| Subsidies |  |  |  |
| Subsidies for passenger transport7) | : |  | 0.5 |
| Non-transport related revenues of airports | : |  | : |
| 1) Running costs only. No capital cost estimates available. – 2) For Estonian Air Navigation Services Ltd. – 3) For Tallinn Airport Ltd only. Including revenues from non-transport related business.- 4) International transport. – 5) Domestic transport. - 6) VAT levied on fuel tax – 7) Subsidies included here refer to subsidies given for the provision of public transport services. These subsidies can clearly not be allocated to either the cost or to the revenue side of this table. Subsidies are in cash flow terms and are not on the same basis as the economic costs. Sources: JP-Transplan Ltd |  |  |  |

Running costs as part of total infrastructure costs amounted to € 1.4 million in 1996. Environmental costs represented €2.3 million, however, this figure excludes noise costs, which could not be quantified. Note that the picture of environmental costs without noise costs is biased since noise plays a considerable role for air traffic. Total social costs of accidents were zero because there were no serious accidents in 1998. Congestion costs could not be estimated due to missing data.

Infrastructure related revenues were estimated at € 15.2 million in 1998.

According to the conventions set for the UNITE accounts we can report on indirect subsidies as a supplementary data. Indirect subsidies play a major role in the aviation sector. Commercial aviation is exempted from paying kerosene tax as well as VAT on the ticket price for international flights.

Comments on specific cost categories

Infrastructure costs

Calculation of the capital stock could not be executed due to missing data. Running costs were available from MTC (2000). Estonian air traffic control was also included in the calculations.

Congestion costs

Congestion costs could not be estimated due to missing data.

Accident costs

Accident costs were very low because only one accident with one fatality was reported in 1996 and none in 1998. Data needed for the estimation of other cost segments except risk value was  not available and they were not included in the evaluation.

Environmental costs

Environmental costs for the aviation sector are based on Landing and Take-off cycles at Estonian airports and the civil aviation fuel tanked in Estonia. However, the emission costs covered here do not include the costs of impacts caused by PM10. Although noise emissions from aviation are considered to be a major environmental problem they could not be evaluated for this account, as no basic emission data was available.

The environmental costs of aviation cannot be allocated to performance nor disaggregated between passenger and fright transport because of missing data.

Taxes, charges and subsidies

Charges for the use of airports and air corridors as well as for the aid of air traffic control services are gathered by respective companies. The total revenues of navigation fees were available from MTC (2000), but the revenues from the use of airports were available only for the major airport – Tallinn. It was not, however possible to exclude non-transport related revenues from the account.

## Maritime shipping

Estonia has 320 km of navigable inland waterways, but they are used mainly for leisure purposes with only a minimal amount of freight and passenger transport.  Therefore only maritime shipping is discussed next. Table 51 shows the total costs of maritime shipping transport.

Table 51. Estonian maritime shipping transport account for 1996 and 1998 - € million at 1998 prices

|  | Costs |  |  |
| --- | --- | --- | --- |
| Core information | 1996 | 1998 |  |
| Infrastructure costs1) |  |  |  |
| Running costs for maritime navigation2) | 6.2 | 9.2 |  |
| Running costs for ferry ports3) | 1.1 | 1.2 |  |
| Accident costs (external) | : | 0.1 |  |
| Environmental costs |  |  |  |
| Air pollution | : | : |  |
| Global warming | : | : |  |
| Noise | : | : |  |
| Additional information |  |  |  |
| Congestion costs | : | : |  |
| Accident costs (internal) |  | 2.6 |  |
| From this: risk value | : | 2.6 |  |
| Environmental costs |  |  |  |
| Nature and landscape, soil and water pollution | : | : |  |
| Nuclear risk | - | - |  |
| Revenues |  |  |  |
| Directly related to a specific cost category Charges for infrastructure usage2) | : | 11.5 |  |
| Other transport specific revenues |  |  |  |
| Fuel tax3) | : | 0.83 |  |
| VAT 4) | : | 0.15 |  |
| Subsidies |  |  |  |
| Subsidies for passenger transport5) | : |  | 2.8 |
| Non-transport related revenues of ports | : |  | : |
| 1) Running costs only. No capital cost estimates available. – 2) Coastal waterways only. No harbours. – 3) Domestic transport. - 4) VAT levied on fuel tax – 5) Subsidies included here refer to subsidies given for the provision of public transport services. These subsidies can clearly not be allocated to either the cost or to the revenue side of this table. Subsidies are in cash flow terms and are not on the same basis as the economic costs. Sources: JP-Transplan Ltd |  |  |  |

Infrastructure costs play the major role in coastal waterway transport. This can be stated even though we were not able to estimate capital costs neither the infrastructure running costs of harbours – except ferry ports for domestic transport - due to the lack of data. The available figures - €9.2 million for coastal waterways and €1.2 million for ferry ports in 1998 - confirm this.  In Estonian coastal waterways there were shipwrecks with 4 fatalities resulting to €2.7 million accident costs.

Charges for the use of coastal waterways amounted in 1998 to €11.5 million. There are many big islands on the Estonian cost and ferry transport to them was subsidised by €2.8 million.

Comments on specific cost categories

Infrastructure costs

Data was available only for running costs of coastal waterways and ferry ports. These ferry ports are used for domestic transport. Calculation of capital stock could not be executed.

Congestion costs

No formal statistics on delay costs for this transport mode are kept in Estonia and no estimation of the associated costs could be made.

Accident costs

In Estonian coastal waterways there were some shipwrecks with fatalities. In addition it has to be noticed that due to the use of leisure boats in inland and coastal waterways there were boat accidents with two fatalities resulting to €1.3 million accident costs. Therefore accident costs form a big cost block of the total waterway costs.

Environmental costs

The environmental costs of maritime shipping are not considered in the UNITE country accounts.

References:

Abbey, DE., Lebowitz, MD., Mills, PK., Petersen, FF., Lawrence Beeson, W., Burchette, RJ. (1995). Long-term ambient concentrations of particulates and oxidants and development of chronic disease in a cohort of nonsmoking California residents. Inhalation Toxicology 7, 19-34.

Anderson, HR., Ponce de Leon, A., Bland, JM., Bower, JS., Strachan, DP. (1996). Air pollution and daily mortality in London: 1987-92. BMJ 312: 665-9.

BASt (2001). International Road Traffic and Accident Database (IRTAD). Online Access via Bundesanstalt für Straßenwesen , April 2001. (http://www.bast.de/htdocs/fachthemen/irtad/index.htm)

Berdowski, J., Mulder, W., Veldt, C., Visschedijk, A., Zandveld, P. (1997). Particulate matter (PM10 - PM2.5 - PM0.1) emissions in Europe. TNO-report TNO-MEP-R 96/472, TNO, Appeldoorn, The Netherlands.

Capros, P. and Mantzos, L. (2000). Kyoto and technology at the European Union: costs of emission reduction under flexibility mechanisms and technology progress, Int. J. Global Energy Issues, 14, pp. 169-183.

COWI  and Tallinn Technical University (2001). External costs of transport in Estonia, Summary report. Estonian Ministry of Transport and Communications and Danish Ministry of Transport. Tallinn.

Dab, W., Quenel, SMP., Le Moullec, Y., Le Tertre, A., Thelot, B., Monteil, C., Lameloise, P., Pirard, P., Momas, I., Ferry, R., Festy, B. (1996). Short term respiratory health effects of ambient air pollution: results of the APHEA project in Paris. J Epidem Comm Health 50 (suppl 1): S42-46.

DHV and LT Consultants (1999). Transport and the Environment: A Multy-Country Approach. Final Report. Funded by the European Commission, PHARE-programme. Brussels.

Dockery, DW., Speizer, FE., Stram, DO., Ware, JH., Spengler, JD., Ferries, BG. (1989). Effects of inhalable particles on respiratory health of children. Am Rev Respir Dis 139, 587-594.

Doll, C., Lindberg, G., Niskanen, E. (2000). Accounts Approach for Accidents. UNITE (UNIfication of accounts and marginal costs for Transport Efficiency) Interim Report 82. Funded by 5th Framework RTD Programme. ITS, University of Leeds, Leeds, June 2000.

Duerinck, J. (2000). Personal communication based on: Duerinck, J. et al. (1999). Prospective study of emissions in Belgium until 2008/2012 of the greenhouse gases included in the Kyoto Protocol. Costs and potential measures and policy instruments to reduce GHG emissions, Vito (Mol) & KU-Leuven.

Dusseldorp, A., Kruize, H., Brunekreef, B., Hofschreuder, P., de Meer, G., van Oudvorst, AB. (1995). Associations of PM10 and airborne iron with respiratory health of adults near a steel factory. Am J Respir Crit Care Med 152, 1932-1939.

European Commission (1999). ExternE Externalities of Energy. Vol 7 – Methodology 1998 update. A Report produced for the EC – DG XII, Luxembourg, Office of Publications for the European Communities, Luxembourg.

Fahl, U., Läge, E., Remme, U., Schaumann, P. (1999) E3Net. In: Forum für Energiemodelle und Energiewirtschaftliche Systemanalysen in Deutschland (Hrsg.) (1999) Energiemodelle zum Klimaschutz in Deutschland. Physica-Verlag, Heidelberg.

Friedrich, R. and Bickel, P. (2001). Environmental External Costs of Transport. Springer Verlag. Heidelberg.

IFEU (Institut für Energie- und Umweltforschung) (1999). Wissenschaftlicher Grundlagenbericht zur "Mobilitäts-Bilanz" und zum Softwaretool "Reisen und Umwelt in Deutschland 1999". Im Auftrag der Deutschen Bahn AG und der Umweltstiftung WWF-Deutschland, Heidelberg.

INFRAS/IWW (2000). External Costs of Transport. Study for the International Union of Railways. Paris.

Krupnick, AJ., Harrington, W., Ostro, B. (1990). Ambient ozone and acute health effects: Evidence from daily data. J. Environ Econ Manage 18, 1-18.

Link, H., Stewart, L., Maibach, M. (2000). Accounts Approach UNITE (UNIfication of accounts and marginal costs for Transport Efficiency). Funded by 5th Framework RTD Programme.  ITS, University of Leeds, Leeds October.

McInnes, G. (1996). Atmospheric Emission Inventory Guidebook, A joint EMEP/CORINAIR Production, First Edition. EMEP Task Force on Emission Inventories, European Environment Agency, Copenhagen.

Metroeconomica (2001). Monetary valuation of noise effects. Draft Final Report prepared for the EC UNITE Project, subcontracted to IER, Bath (UK).

Ministry of Transport and Communications (2000). Consolidated statistical-informative summary. Publication 98/98. Tallinn.

Nellthorp, J., Sansom, T., Bickel, P., Doll, C. and Lindberg, G. (2001). Valuation Conventions for UNITE, Leeds, April 2001.

Ostro, BD.. (1987). Air pollution and morbidity revisited: A specification test. J Environ Econ Manage 14, 87-98.

Pilkington A, Hurley JF, Donnan P. (1997). Health effects in ExternE Transport: assesment and exposure-response functions. Working Paper, ExternE Project, DGXII, Euiropean Commission.

Ponce de Leon A, Anderson HR, Bland JM, Strachan DP, Bower J. (1996). Effects of air pollution on daily hospital admissions for respiratory disease in London between 1987-88 and 1991-92. J Epidem Comm Health 50 (suppl 1): S63-70.

Pope, CA. III, Thun, MJ., Namboodiri, MM., Dockery, DW., Evans, JS., Speizer, FE., Heath, CW. Jr. (1995). Particulate air pollution as predictor of mortality in a prospective study of US adults. Am J Resp Crit Care Med 151: 669-674.

Pope, CA. and Dockery, DW. (1992). Acute health effects of PM10 pollution on symptomatic and asymptomatic children. Am Rev Respir Dis 145, 1123-1126.

Posch, M., Hettelingh, J., de Smet, P. and Downing, R. (1997). Calculation and Mapping of Critical Thresholds in Europe: Status Report 1997. RIVM Report No. 259101007. Coordination Center for Effects, National Institute of Public Health and the Environment (RIVM), Bilthoven, Netherlands.Statistics Estonia (1999). Energy statistics. Energy 1998:2.

Rabl, A. & Spadaro, J. (1999). Environmental Damages and Costs: an analysis of Uncertainties. Paper accepted for publication in Environment International.

Richardson, S. (2000). Atmospheric Emission Inventory Guidebook, A joint EMEP/CORINAIR Production, Second Edition. European Environment Agency, Technical Report, No. 30, Copenhagen.

Roemer, W., Hoek, G., Brunekreef, B. (1993). Effect of ambient winter air pollution on respiratory health of children with chronic respiratory symptoms. Am Rev Respir Dis 147, 118-124.

Schwartz, J. and Morris, R. (1995). Air pollution and hospital admissions for cardiovascular disease in Detroit, Michigan. Am J Epidem 142, 23-35. Am J Epidem 137, 701-705.

Simpson, D., Olendrzynski, K., Semb, A., Storen, E., Unger, S. (1997). Photochemical oxidant modelling in Europe: multi-annual modelling and source-receptor relationships. EMEP/MSC-W Report 3/97, Oslo.

Spix, C., Wichmann, HE. (1996). Daily mortality and air pollutants: findings from Köln, Germany. J Epidem Comm Health 50 (suppl 1): S52-S58.

Sunyer, J., Castellsague, J., Saez, M., Tobias, A., Anto, JM. (1996). Air pollution and mortality in Barcelona. J Epidem Comm Health 50 (suppl 1): S76-S80.

Trukenmüller, A., Friedrich, R. (1995). Die Abbildung der großräumigen Verteilung, chemischen Umwandlung und Deposition von Luftschadstoffen mit dem Trajektorienmodell WTM, in: Jahresbericht 1995, ALS Universität Stuttgart, pp. 93 - 108.

Verhoeff, AP., Hoek, G., Schwartz, J., van Wijnen, JH. (1996). Air pollution and daily mortality in Amsterdam. Epidemiology 7, 225-230.

Vestreng, V. (2000). Tables of 1998 sector emissions in the ECE region. Pollutants: sulphur, nitrogen oxides, ammonia, NMVOCs, carbon monoxide, methane, carbon dioxide. Co-operative programme for monitoring and evaluation of the long range transmission of air pollutants in Europe (EMEP). http://projects.dnmi.no/~emep/emis_tables/ stab1.html, Last Update: 9/2000, Status from: 3/2001.

Vestreng, V. and Støren, E. (2000) Analysis of UNECE/EMEP Emission Data, MSC-W Status Report 2000. Co-operative programme for monitoring and evaluation of long range transmission of air pollutants in Europe (EMEP), EMEP MSC-W Note, 1/00, Blindern, Norway.

Whittemore, AS., Korn, EL. (1980). Asthma and air pollution in the Los Angeles area. Am J Public Health 70, 687-696.

Wordley, J., Walters, S., Ayres, JG. (1997). Short term variations in hospital admissions and mortality and particulate air pollution. Occup Environ Med; 54: 108-116.

# Abbreviations used in data tables

| – | No existing data category (for example sea ports in Switzerland) |
| --- | --- |
| 0 | Zero or approximately zero when compared to other data entries |
| . | Not applicable (for example the length of a sea harbour) |
| : | No data available |

From BASt (2001) an average gratification payment € 730 per fatality from the liability insurance to the victim’s relatives is reported. This is taken as the contribution of the responsible party for the internalisation of the costs caused by him.