COMPETITIVE AND SUSTAINABLE GROWTH

(GROWTH)

PROGRAMME

Deliverable 8 Annex 5 The Pilot Accounts for Ireland - Appendix Report

Version 1.2

June 2003

Authors:

Margaret O’Mahony (TCD),  Heike Link, Louise Stewart (DIW) and Peter Bickel (IER)

Contract: 1999-AM.11157 Project Coordinator: ITS, University of Leeds Funded by the European Commission 5th Framework – Transport RTD

UNITE Partner Organisations ITS/UNIVLEEDS (UK), DIW (De), NEI (Nl), CES/KUL (Be), TIS.PT (Pt), IWW/UNIKARL (De), VTI (Se), IER/USTUTT (De), CERAS/ENPC (Fr), HERRY (Au), EIET/ULPGC (Es), ISIS (It), STRATEC (Be), SYSTEMA (Gr), JP-TRANSPLAN (Fi), VATT (Fi), ECOPLAN (Ch), INFRAS (Ch), EKONO (Fi), EKI (Se)

UNITE

1999-AM.11157

UNIfication of accounts and marginal costs for Transport Efficiency

Deliverable 5:	  The Pilot Accounts of Ireland - Appendix Report 5

This document should be referenced as:

Margaret O’Mahony (Trinity College Dublin), Heike Link, Louise Stewart (DIW) and Peter Bickel (IER).  The Pilot Accounts of Ireland - Appendix Report (UNIfication of accounts and marginal costs for Transport Efficiency) Deliverable 5. Funded by 5th Framework RTD Programme. ITS, University of Leeds, Leeds, July 2001.

June 2003

Draft Version 1.2

Authors:	as above, with contributions from other UNITE partners

PROJECT  INFORMATION

Contract no: 1999-AM.11157:

UNIfication of accounts and marginal costs for Transport Efficiency

Website:	www.its.leeds.ac.uk/unite

Commissioned by: European Commission – DG TREN; Fifth Framework Programme

Lead Partner:   Institute for Transport Studies, University of Leeds (UK)

Partners:  ITS/UNIVLEEDS (UK), DIW (De), NEI (Nl), CES/KUL (Be), TIS.PT (Pt), IWW/UNIKARL (De), VTI (Se), IER/USTUTT (De), CERAS/ENPC (Fr), HERRY (Au), EIET/ULPGC (Es), ISIS (It), STRATEC (Be), SYSTEMA (Gr), JP-TRANSPLAN (Fi) VATT (Fi), ECOPLAN (Ch), INFRAS (Ch), EKONO (Fi), EKI (Se)

DOCUMENT  CONTROL  INFORMATION

Status: 	Accepted

Distribution:	Partners and EC

Availability:	Public (once status is accepted)

Filename:	k/j/5th/…Irish Annex 5

Quality assurance:

Coordinator’s review:

Signed:          			   			Date:

Table of Contents	Page:

1.	Introduction										11

1.1	Study context and objectives								11

1.2	The accounts approach of UNITE							11

1.2.1	Aims of the Pilot Accounts							11

1.2.2	Core, supplementary and excluded data in the Pilot Accounts		12

1.2.3	The six UNITE Pilot Account cost categories				12

1.2.4	The transport modes of the Pilot Accounts					14

1.3	Results presentation and guidelines for interpretation				16

1.4	The structure of this report								16

2.	Input Data										17

2.1	General input data									17

2.1.1	Basic economic and structural data						17

2.1.2	Basic transport data								18

2.2	Infrastructure costs									22

2.2.1	Road										22

2.2.2	Rail										23

2.2.3	Urban public transport							23

2.2.4	Aviation									23

2.2.5	Inland waterway shipping							23

2.2.6	Maritime shipping								23

2.2.7	Summary of input data							23

2.3	Supplier operating costs								24

2.3.1	Rail										24

2.3.2	Urban public transport							24

2.3.3	Maritime shipping								25

2.4	Congestion costs									25

2.4.1	Values of time									25

2.4.2	Input data by mode								26

2.5	Accident costs										27

2.5.1	Road										28

2.5.2	Rail										30

2.5.3	Urban public transport							31

2.5.4	Aviation									32

2.6	Environmental costs									33

2.6.1	Air pollution									33

2.6.2	Global warming								37

2.6.3	Noise										37

2.7	Taxes, charges and subsidies								37

3.	Methodological issues								38

3.1	Infrastructure costs									38

3.1.1	Road										39

3.1.2	Rail										40

3.1.3	Aviation									40

3.1.4	Maritime shipping								40

3.2	Supplier operating costs								40

3.2.1	Rail										40

3.2.2	Urban public transport							41

3.2.3	Maritime shipping								41

3.3	Congestion costs									41

3.3.1	Road										41

3.3.2	Rail										43

3.3.3	Urban public transport							43

3.4	Accident costs										43

3.5	Environmental costs									45

3.5.1	Air pollution									45

3.5.2	Noise										45

3.6	Taxes, charges and subsidies								46

3.6.1	Road										46

3.6.2	Rail										47

4.	The results in detail									48

4.1	Infrastructure costs									48

4.1.1	Overview									48

4.1.2	Road										48

4.1.3	Rail										49

4.1.4	Aviation									49

4.2	Supplier operating costs								50

4.2.1	Rail										50

4.2.2	Urban public transport							51

4.2.3	Regaional public transport (Bus)						52

4.2.4	Aviation									52

4.2.5	Maritime shipping								52

4.3	Congestion costs									53

4.3.1	Roads										53

4.3.1.1	National road network						53

4.3.1.2	Dublin									53

4.3.2	Rail										54

4.4	Accident costs										55

4.5	Environmental costs									58

4.5.1	Air pollution									58

4.5.2	Global warming								61

4.5.3	Noise										61

4.6	Taxes, charges and subsidies								61

4.6.1	Road										61

4.6.2	Rail										62

4.6.3	Urban public transport							63

4.6.4	Aviation									63

4.6.5	Inland waterway shipping							63

5.	Summary tables									64

5.1	Road											64

5.2	Rail											65

5.3	Aviation										70

References											72

Abbreviations

AR	Aer Rianta (Irish Airport Authority)

BAC	Bus Atha Cliath (Dublin Bus)

BE 	Bus Eireann

CIE	Coras Iompair Eireann

GDP	Gross domestic product

HGV	Heavy goods vehicles

IR	Iarnrod Eireann (Irish Rail)

LGV	Light goods vehicles

MTOW	Maximum take-off weight

PPP	Purchasing power parity

PT	Public transport

UPT	Urban public transport

VAT	Value added tax

VOSL	Value of statistical life

VOT	Value of time

WTP	Willingness-to-pay

List of tables

1   	The modes, network differentiation, transport means and user breakdown  			15

in the Irish Pilot Accounts

1	Basic economic and structural data for Ireland (Central Statistics Office - CSO)		18

3	Basic transport related indicators for Ireland 1998 per mode			            19

4	Transport volumes of road transport in Ireland, in mill. Vkm 				20

(National Roads Authority – NRA)

5	Transport volumes of rail transport in Ireland, in mill. train-km (Irish Rail – IR)		20

6	Transport volumes of urban public transport in Ireland, in mill. Vkm			21

(Dublin Bus – Bus Atha Cliath – BAC)

7  	Transport volumes of aviation in Ireland, in 1 000 aircraft movements (Aer Rianta)	21

8	Transport volumes of maritime shipping in Ireland, in 1 000 vessel				21

movements (Central Statistics Office)

9  	Input data for the computation of infrastructure costs by transport mode			24

10  	Input data for the computation of supplier operating costs by transport mode		25

11	VOT-Values for Ireland - PPP-adjusted, (Nellthorp et al.,2001), in € per hour, 		26

1998 prices

12	VOT-Values estimated for Ireland DKM (1994)						26

13  	Input data, uncertainties and level of disaggregation for the 				27

computation of congestion costs for road transport

14  	Cost blocks of the total traffic accident costs						27

15	Number of killed and injured victims of road accidents in Ireland, 1998			29

16	Input data for the valuation of the road accidents						30

17	Number of killed and injured victims of rail accidents, 1955 – 1998 (IRMS 1999)		31

18  	Rail accident data 1993 –1997 (IRMS 1999)						31

19	Equivalent fatalities per annum for each incident type (IRMS, 1999)			31

20  	Number of killed and injured victims of aviation accidents in 1996 (IRMS, 2001)		32

21  	Number of killed and injured victims of aviation accidents in 1998 (IRMS, 1999)		32

22	Emission of air pollutants of road transport, in t/a, 1996 (EPA)				34

23   	Emission of air pollutants of road transport, in t/a, 1998 (EPA)				35

24  	Particulate matter emissions from road transport, in t/a, 1996, 1998 (EPA)			36

25	Rail emissions estimates in Ireland in t/a, 1996 (Oscar Faber, 1999)			36

26 	Emissions from aviation in Ireland, in t/a, 1996 (Oscar Faber, 1999)			36

27  	No. of persons in millions exposed to each noise level (Tinch, 1997)			37

28	Input data and data quality for the calculation of taxes, charges and subsidies		38

29	Overview of the procedure for calculating infrastructure costs per transport mode		39

30  	Travel time estimates for a selection of towns on the Irish road network (NRA, 1998)	42

31	Structure of vehicle registration taxes in Ireland						46

32	Annual vehicle taxes in Ireland (DOE, 2001)						47

33  	Fuel taxes in Ireland 									47

34	Road infrastructure costs in million €, in 1998 prices					48

35	Road infrastructure costs per vehicle type in million €, in 1998 prices			48

36	Infrastructure costs rail, in million €, 1998 prices (IR yearly financial accounts)		49

37 	Infrastructure costs for rail – per train-km, in €, 1998 prices 				49

38  	Asset value and net book accounts of Irish railways (IR annual accounts)			49

39	Infrastructure costs for air 1998 million € (IR yearly accounts)				50

40	Infrastructure costs for air 1996 million €, 1998 prices (IR yearly accounts)		50

41	Infrastructure costs for Dublin port for 1996 and 1998 in million €, 1998 prices 		50

42	Supplier Operating Costs of Rail 1998, 1996 (1997) and 2005, in million €, 1998 prices	51

43	Average Supplier Operating Costs of Rail 1998, 1996 and 2005, in €/train-km, 		51

1998 prices

44	Supplier Operating Costs of Public Transport – Bus Atha Cliath (BAC) 			51

1996, 1998 and 2005, in million €, 1998 prices

45  	Average Supplier Operating Costs of Public Transport- Bus Atha Cliath (BAC) 		52

1996, 1998 and 2005, in €/vkm, 1998 prices

46   	Supplier Operating Costs of Public Transport – Bus Eireann (BE) 				52

1996, 1998 and 2005, in million €, 1998 prices

47  	Average Supplier Operating Costs of Aer Rianta 						52

(covering Dublin, Cork and Shannon airports) 1996 and 1998, in Million €, 1998 prices

48 	Operating costs for Dublin port for 1996 and 1998 in million €, 1998 prices 		53

49  	Road Congestion Costs 1998 in million €, 1998 prices 					53

50 	Performance levels – rail punctuality (Irish Rail, 2001)					55

51  	Social costs of transport accidents, in million €, 1998 (Bacon, 1999) 			51

52	Transport system external costs of transport accidents 1998, in million €			56

53  	Social accident costs 1998 – detailed results per mode, in million €				56

54  	Social accident costs of road transport 1998 – detailed results per road type, in million €	57

55	Average social costs per unit of performance in 1998, in million €, 1998 prices		58

56	Cost of air pollution from roads sector for Ireland for 1996 in million €, in 1998 prices	58

57	Cost of air pollution from roads sector for Ireland for 1998 in million €			59

58	Total and average air pollution costs per km from road sector 				59

59	Cost of emissions from rail (diesel traction) for 1996 in million € in 1998 prices		60

60	Cost of emissions from rail (electric traction) for 1996 in million € in 1998 prices		60

61  	Cost of air emissions from aviation sector for 1996 in million €, in 1998 prices 		60

62	Noise costs of road and rail for 1996 in million € for road and rail, in 1998 prices.		61

63	Different sources of the revenues from road transport 1998, in million €, 1998 prices.	61

64	Fuel price structure in Ireland								62

65	Supplier revenues and subsidies of mainline rail transport 					62

1998, 1996 and 2005, in million €, 1998 prices (IR Annual Accounts)

66	Supplier revenues and subsidies of urban rail transport in Dublin				62

1998, 1996 and 2005, in million €, 1998 prices (IR Annual Accounts)

67  	Supplier revenues and subsidies of interregional bus transport in Ireland 			63

1998, 1996 and 2005, in million €, 1998 prices (BE Annual Accounts)

68	Supplier revenues and subsidies of urban bus transport in Dublin 				63

1998, 1996 and 2005, in million €, 1998 prices (BAC Annual Accounts)

69	Aviation revenues for the Irish national airports, in million €, 1998 prices.			63

(Aer Rianta annual accounts)

70	Irish road account for 1996, 1998 and 2005 						64
- € million at 1998 prices

71	Average variable costs of road transport per vehicle km: Germany				65
- €/km at 1998 prices

72	Total costs of road transport: Germany							66
- € million at 1998 prices

73	Irish rail account for 1996, 1998 and 2005 						67
– € million at 1998 prices –

74	Average variable costs of rail transport per vehicle km: Irish Rail				68
€/train km at 1998 prices

75	Total costs of rail transport: Irish Rail (IR)						69
- € million at 1998 prices –

76	Irish air transport account for 1996, 1998 and 2005 					70
- € million at 1998 prices –

77	Average variable costs of Aviation per vehicle km or movement: 				71
Ireland – €/km at 1998 prices –

# Introduction

## Study Context and Objectives

This Appendix Report contains the full version of the pilot accounts developed within the UNITE project for Ireland. It is a background report for the results presented in the core body of deliverable 5 (Summary Report of D5 containing the Tranche B accounts) and gives more detailed descriptions on the methodology used and the input data and their reliability and quality. However, the general and detailed discussion of the accounts approach has been presented in Link et al. 2000 and will only be summarised in this document. The Appendix Report discusses methodologies only in so far as they are necessary background information for understanding the results. In addition to the core accounts for 1998 the Appendix Report also presents results for 1996 and a forecast for 2005, the two other years covered by UNITE.

In order to put this report into the context of the UNITE project, a summary of the aims and research areas of UNITE is provided here.

The UNITE project aims to provide accurate information about the costs, benefits and revenues of all transport modes including the underlying economic, financial, environmental and social factors. To achieve this goal, three main areas of research are carried out, known as “transport accounts”, “marginal costs” and “integration of approaches”.

This Appendix Report belongs to the research area “transport accounts”. For a better understanding of the results presented here it has to be borne in mind that the UNITE project distinguishes between ideal accounts on the one hand and pilot accounts on the other hand. The ideal accounts reflect the perfect situation with the utmost disaggregation, showing factors such as the time and location and duration of individual trips, all the relevant economic data as well as the individuals response to possible policy or infrastructure changes. The pilot accounts are the actual, feasible accounts given the available data for the 18 countries that UNITE covers. They can be used to assess the costs and revenues of transport per transport mode. The costs are reported and documented at the current level of transport demand for the reference years 1996, 1998 and for the forecast year 2005. Reported transport costs are allocated to user groups, where possible without arbitrary allocation methods.

## The Accounts Approach of UNITE

### Aims of the Pilot Accounts

The pilot accounts attempt to show the general relationship between costs of transport and the revenues from transport pricing and charging in the country studied. The aims and role of the pilot accounts are discussed in detail in “The Accounts Approach” Link (2000). It should be stressed that the accounts are aimed at providing the methodological and the empirical basis for in-depth policy analysis (monitoring control) rather than serving as a guide for immediate policy actions such as setting higher / lower prices and charges or shutting-down transport services/ links in order to achieve cost coverage. The pilot accounts are defined as stated in the box below.

The pilot accounts compare social costs and / charges on a national level in order to monitor the development of costs, the financial taxes balance and the structure and level of prices. Accounts can therefore be seen as monitoring and strategic instruments at the same time. They have to consider the country-specific situation and the institutional frameworks.

The pilot accounts show the level of costs and charges as they were in 1998 (and 1996 respectively) and provide a workable methodological framework to enable regular updating of transport accounts. Furthermore, an extrapolation for 2005 is given. The choices of additional accounting years (1996 and 2005) were motivated to show a comparison between years and to give a good indication of trends in transport for the near future. Furthermore, the inclusion of 1996 enables identification of any major statistical abnormalities that may occur only in one year, for example very high infrastructure cost due to tunnelling operations or higher than average accident costs because of major accidents occurring in 1998.

### Core, Supplementary and Excluded Data in the Pilot Accounts

The pilot accounts have been divided into two classes: “core data” and “supplementary data”. Core data is the data necessary to do a full basic review of the country accounts. Supplementary data falls into three categories.

- Data that provides additional information to the core accounts is described as supplementary data.

- For several cost categories,  there is no standard methodology for the valuation of effects.

- Some costs which can be estimated and valuated are borne by the transport users themselves (for example delay costs). These costs and the methods used to valuate them present useful further information to the reader, but cannot be considered to be part of the overall costs of transport as defined by UNITE.

### The six UNITE Pilot Account Cost Categories

Data for the pilot accounts have been collected within six cost and revenue categories that are described in “The Accounts Approach” (Link et al., 2000) and are summarised in the following:

Infrastructure Costs

For the pilot accounts, data for the assessment of infrastructure costs are structured to show the capital costs of transport infrastructure (including new investments and the replacement of assets) and the running costs of transport infrastructure (maintenance, operation and administration) for all modes of transport studied. As far as possible with current methodological knowledge, infrastructure costs are allocated to user groups and types of transport. Where it is possible to quantify the share of joint costs they are sorted out and are not allocated.

Supplier Operating Costs

All monetary costs incurred by transport operators for the provision of transport services are documented in the category supplier operating costs. Ideally, the data is structured to show what costs are incurred for vehicles, for personnel and for administration. However, this depends on data availability and will differ from country to country. Since collecting these data for all modes is extremely time consuming, the UNITE project focuses on estimating supplier operating costs only for those modes where significant state intervention and subsidisation is present. The main emphasis in this category is thus on public transport (excluding rail) and on rail transport. Whether other modes also have to be covered depends on the degree of state intervention in the respective countries. The corresponding revenues from the users of transport are included when supplier operating costs are estimated. The difference between such costs and revenues is the net public sector contribution (economic subsidy).

Transport Congestion Costs

In the European Commission’s White Paper “Fair payment for infrastructure use” (1998), costs caused by transport delays, accidents and environmental effects of transport are estimated to be the three major causes of external transport costs. In the category transport 'congestion costs', the costs of delay and delay-caused additional operating costs are estimates. This estimation is intended to provide supplementary data for the accounts and is carried out for all transport modes, provided data are available. This data is classified as supplementary data because the bulk of these costs are borne by transport users as a whole (transport system internal costs).

Accident Costs

The loss of lives and the reduction of health and prosperity through transport accidents are of major concern to all countries and to the European Commission. In this section of the accounts, the health related accident costs are calculated by assessing the loss of production, the risk value and the medical and non-medical rehabilitation of accident victims. Where the available data allows, the damage to property and the administrative costs of accidents are considered, too. The external part of accident costs (defined in this report as accident costs imposed by transport users on the rest of society) is included in the core section of the accounts. Total accident costs however, include a substantial proportion of costs imposed by one user on others and are therefore treated as supplementary costs (transport system internal costs).

Environmental Costs

A wide range of transport related environmental impacts and effects is considered in the environmental costs section of the accounts. Included in this cost category are: air pollution, global warming, noise, changes to nature and landscape and nuclear risks. The valuation of these environmental effects is carried out for all transport modes, provided adequate data is available.

It is the aim of UNITE to calculate these costs according to a harmonised methodology. Against this background, it was decided to carry out all calculations for air pollution and noise with the EcoSense model running at the Institute of Energy Economics and the Rational Use of Energy of the University of Stuttgart. The model was developed within the series of ExternE Projects on "External Costs of Energy" funded by the European Commission. A description is given in the annex of this report.

Taxes, Charges and Subsidies

Transport taxes and charges are exceptionally heterogeneous throughout Europe. In this section, the level of charging and taxation for the transport sector is documented for each mode of transport. Wherever possible, the revenues from taxes and charges are shown for fixed taxes and charges and variable ones. This information plays an important part in the ongoing discussions about the level of taxation between transport modes and countries. The comparison between taxes levied and the costs of infrastructure provision and use accrued per mode is central to this debate and holds a high level of political significance. Environmental taxes that apply to transportation are separately considered in this section. Taxes such as VAT that do not differ from the standard rate of indirect taxes are excluded from this study.

A further part in this area is reporting on subsidies. The need to maintain free and undistorted competition is recognised as being one of the basic principles upon which the EU is built. State aid or subsidies are considered to distort free competition and eventually cause inefficiency. Subsidies to the transport sector provided by the Member States are not exempted from the general provisions on state aid set out in the Amsterdam Treaty. There are, however, special provisions set out in the treaty in order to promote a Common Transport Policy for the transport sectors of the Member States (Treaty establishing the European Community : Articles 70 – 80). The subsidies of the transport sector are considered in this section but it should be noted that a full report on subsidies would require extremely time-consuming analyses of public budget expenditures at all administrative levels. Furthermore, the subsidies reported in the pilot accounts refer mainly to direct subsidies (e. g. monetary payments from the state to economic subjects). Indirect subsidies (e. g. tax reductions and tax exemptions that cause lower revenues of state budgets) are not quantified.

### The Transport Modes of the Pilot Accounts

The modes covered in UNITE are road, rail, other urban public transport (e.g. tram trolley bus), inland waterway shipping, maritime shipping and aviation. The level of disaggregation into types of networks and nodes, means of transport and user groups depends on data availability and relevance per country. Table 1 summarises this disaggregation for the Irish Pilot Account.

In the case of Ireland, inland waterways are of very minor relevance.  Urban public transport consists of buses mainly (in some cases included in the road account) and suburban rail in Dublin.  Trolley buses or trams would not have been part of the system in 1996 or 1998 although two trams routes will be in operation by 2005.  In the other cost areas, the Irish accounts include the modes road, rail,  aviation and maritime shipping.

Table 1

The modes, network differentiation, transport means and user breakdown

in the Irish Pilot Accounts

| Transport modes | Network differentiation | Means and user breakdown |
| --- | --- | --- |
| Road | Motorways National Primary Roads National Secondary Roads Regional Local | Motorcycles Passenger cars Buses Light Goods vehicles Heavy Goods Vehicles Others |
| Rail National rail - (Iarnrod Eireann - Irish Rail) | – – | Passenger transport Freight transport |
| Urban public transport* | – | Urban diesel buses Tramways (not currently in operation but will be by 2005 - included for 2005) |
| Aviation | National Airports (Dublin, Cork, Shannon) | – |
| Inland waterway shipping | - | – |
| Maritime shipping | Ports (Dublin, Cork) |  |

* The infrastructure costs of local/urban buses are included in the road infrastructure costs. Against this background, the figures estimated for this mode are not in all cases comparable. Attention should be paid to the footnotes specifying which categories of urban public transport have been taken into account.

## Results presentation and guidelines for interpretation

The aim of the data collection and estimation of cost and revenues in each category was a level of disaggregation that shows the pertinent costs and charges of the relevant transport mode. From the available, but very heterogeneous input data and results, a structure for reporting transport accounts has been developed. All results are documented separately for each cost category and are summarised in modal accounts covering all cost and revenue categories.

Additionally, a set of data needed as basic data for all cost categories was collected to ensure that commonly used data are consistent between the cost categories. Minor discrepancies in the basic data used between cost categories are due to the fact that the level of disaggregation in the input data required for each cost category differed.

The cost categories and taxes, charges and subsidies present a comprehensive estimation of transport costs and revenues. They are however, not a total estimation of transport costs. Each cost category could include data in further areas but limits are consistent with the resources of the project.  For example, the estimation of environmental costs does not include the environmental costs incurred during the manufacturing of vehicles, even though these costs could be estimated. These costs would be included in an ideal account, but lie outside the scope of the pilot accounts. Further transport cost categories such as vibration as attributing to environmental costs are not evaluated because no acceptable valuation method has been developed.

It should be noted that due to the separation into core and supplementary data with different levels of uncertainty and with different types (costs borne by transport users themselves versus external costs) a simple summing up of the different cost and revenue categories to totals and the calculation of a cost recovery rate is not sensible.

## The Structure of this Report

This annex report contains four major parts:

- In Chapter 2, the input data used to calculate the figures given in the UNITE Pilot Accounts are presented. The calculations require the collection of a large amount of data and information. Chapter 2 does not aim to reproduce all of them but concentrates on the most important input data.

- Chapter 3 deals with methodological issues.  In some cases, it was not possible to apply the methodology developed in Link et al (2000). Chapter 3 concentrates on these deviations from the general accounts approach of UNITE.   Furthermore, the procedure to derive the results for the years 1996 and 2005 is described.

- The fourth chapter of the report deals with the results of the calculations and their interpretation in detail. The descriptions in these chapters are organised along the categories stated in section 1.2.3 above.

- In the last chapter, the detailed results of chapter 4 are finally brought together per mode. This final presentation of the results is the same for the different UNITE countries.

# Input Data

- This chapter is divided into seven sections. The first section 2.1 presents the basic economic and transport data used for the calculations of several types of costs. The subsequent six sections discuss the most important specific input data on which the assessment of the cost and revenue categories of UNITE are based. These categories are:

- Infrastructure costs (section 2.2)

- Supplier operating costs (2.3)

- Congestion costs (2.4)

- Accident costs (2.5)

- Environmental costs (2.6)

- Taxes, charges and subsidies (2.7)

- Each of the six sections starts with a very short description of the main data needs of each individual cost and revenue category.  The methodology applied for the calculations of transport costs and revenues is given in Link et al. (2000). The presentation of the input data is restricted to the most important values and it is important to note that it mdoes not aim to reproduce all the data that has been collected for the calculations.

- The data availability, the level of disaggregation and the quality of the input data differs considerably between and within the different cost categories. These differences undoubtedly influence the level of uncertainty of the results of our calculations. In order to make them transparent for the reader, the following sections also contains a judgement of the data quality, where it perhaps is considered useful for the reader.

## General Input Data

### Basic Economic and Structural Data

- In some cases, basic economic data is needed as inputs for the calculation of transport costs. Nelllthorp et al. (2001) suggests that values (e.g. value of time, value of statistical life) grow with real incomes.  Accordingly, the cost estimates for the year 2005 need information about the growth of the Irish GDP.   Table 2 contains the most important basic economic data used in our cost estimations.  It is given for the base year of UNITE (1998) and for 1996.

Table 2

Basic economic and structural data for Ireland

(Central Statistics Office - CSO)

|  | unit | 1996 | 1998 |
| --- | --- | --- | --- |
| Land area | sqkm | 70 273 | 70 273 |
| Population | 1 000 | 3 626 | 3 680 |
| Population density | inhabitants/sqkm | 51.6 | 52.4 |
| Population employed | 1 000 | 1 643 | 1 825 |
| Employment Rate | % | 45.3 | 49.5 |
| GDP1) | € billion | 58 | 77 |
| GDP per capita | € million | 0.015 | 0.02 |
| GDP growth rate (change to previous year) | % (in prices of 1995) | 7.7 | 8.6 |
| Consumer price index | 1996 = 100 | 100 | 103 |
| 1) At market prices. Sources: Irish Central Statistics Office 1999, 2001. |  |  |  |

### Basic Transport Data

- Most of the transport data used for the calculation of the different costs and revenues is summarised in the following sections 2.2 to 2.7.    Here, the focus is on the basic information about transport volumes in the three UNITE years, i.e. the base year 1998 and the backcast and forecast years 1996 and 2005 respectively.

- Table 3 shows a summary of base data for the different modes.

Table 3
Basic transport related indicators for Ireland 1998 per mode

| Indicator | Unit | Road | Rail | Public transport4) | Aviation | Inland waterway navigation 6) | Maritime shipping | Total10) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Transport performance |  |  |  |  |  |  |  |  |
| Passengers carried | Mill. | 2) | 32 29a) | • | 14.8b) | • | • |  |
| Vehicle-km | Mill. Veh.km | 38150 | 15.5 | • | 0.59) | • | • |  |
| Goods transported | Mill. t |  | 2.92)c) | • | 0.2b) | • | 1904) |  |
| Tonne-km | mill. tkm | 5.9 | 0.52)c) | • | 3) | • | 3) |  |
| Network length | 1000 km | 97d) | 2.8c) | • | • | • | • |  |
| Employees | 1000 |  | 4.9e) | • | 2.25)b) | • | • |  |
| Gross investments2) | € mill. | 498 1)c) | 2311)c) | • | 95.7b) | • | • |  |
| Gross capital stock3) | € mill. | 3 052 | 15 7 ) e) | • | 975) | • | 148) f) |  |
| Accidents |  |  |  |  |  |  |  |  |
| Number of injuries(footnote removed) | Casualties | 7 831 | 3 | • | 10 | • | • | 7 844 |
| Number of fatalities | Casualties | 408 | 0 | • | 2 | • | • | 410 |
| Environment |  |  |  |  |  |  |  |  |
| Direct transport emissions |  |  |  |  |  |  |  |  |
| CO2 | Mill. t | 8.3 | 0.1 | • | 1.9 | • | • | 10.3 |
| PM | 1 000 t | 3.6 | 0.01 | • |  | • | • |  |
| NOx | 1 000 t | 52.8 | 1.2 | • | 5.2 | • | • | 59.2 |
| SO2 | 1 000 t | 7.9 | 0.02 | • | 0.06 | • | • | 8.0 |
| NMVOC | 1 000 t | 65.7 | 0.01 | • | 3.0 | • | • | 68.7 |
| 1) 1997 values 2) not available, 3) 1999 values, 4) metro, tram and trolley buses (no such services existed in Ireland in 1998), 5) airports only, 6) Inland waterways in Ireland are for recreational purposes only – data excluded here, 7) railway lines and works, 8) Dublin port only 9) aircraft movements, 10) In most cases, data are lacking or units are different, making it difficult to arrive at totals. Where it is possible to calculate totals, they are presented. Source: a) (CSO, 2001), b) (Aer Rianta, 1998), c) (CSO, 1999), d) (NRA, 2001), e) (Iarnrod Eireann, 1998), f) (Dublin Port, 2001) |  |  |  |  |  |  |  |  |

Table 4

Transport volumes of road transport in Ireland, in mill. Vkm

(National Roads Authority – NRA)

| Mode | Road Infrastructure | 1996 | 1998 | 2005 |
| --- | --- | --- | --- | --- |
| Motorcycles | Total | 227 | 450 | 554 |
|  | National Primary Rural | 51 | 100 | 123 |
|  | Other Rural | 101 | 200 | 246 |
|  | Urban | 76 | 150 | 184 |
| Passenger Cars | Total | 23799 | 27174 | 33420 |
|  | National Primary Rural | 5312 | 6065 | 7459 |
|  | Other Rural | 14315 | 16346 | 20103 |
|  | Urban | 4171 | 4763 | 5858 |
| Buses and coaches | Total | 305 | 454 | 559 |
|  | National Primary Rural | 56 | 83 | 102 |
|  | Other Rural | 194 | 289 | 356 |
|  | Urban | 56 | 83 | 102 |
| Heavy Goods Vehicles | Total | 5639 | 10071 | 12386 |
|  | National Primary Rural | 1329 | 2373 | 2919 |
|  | Other Rural | 3429 | 6123 | 7531 |
|  | Urban | 882 | 1575 | 1937 |

- The transport volumes on roads for 1996 were obtained from the NRA but similar data for 1998 was not available.    Estimates for 1998 were obtained using an energy balance done using the Copert model for 1998 by Feeney (2001).  Similar breakdowns per road type were used in 1998 as in 1996 and a 3% increase per year was used to estimate the 2005 levels of traffic volumes on the road network.  In table 4, transport volume data (million vehicle kilometres) is presented for Ireland for roads for the years 1996, 1998  and 2005.  The car is the predominant mode of transport making up to 71% of all million v.kms travelled in 1998, light and heavy goods vehicles make up 26%, buses about 1% and the remainder motorcycles.

Table 5  shows similar data for rail transport but the units in this case are in million train kilometres.  The estimates for 2005 were done using trend extrapolation based on previous years.  A 16% increase in passenger transport from 1998 to 2005 and a reduction in freight of 23% between 1998 and 2005 are used for the predictions.

Table 5

Transport volumes of rail transport in Ireland, in mill. train-km (Irish Rail – IR)

| Irish Rail | 1996 | 1998 | 2005 |
| --- | --- | --- | --- |
| Passenger transport | 11.05 | 11.29 | 13.12 |
| Freight transport | 4.34 | 4.18 | 3.21 |

- As mentioned in section 1.2, the UNITE mode category "Urban public transport" covers modes which are normally contained in other mode categories: diesel buses are part of road transport and urban rail services are included in the mode rail transport.   In the case of bus transport, only Dublin Bus operating areas are included in table 6.  Dublin Bus (Bus Atha Cliath – BAC) is the company providing the service.

Table 6

Transport volumes of urban public transport in Ireland, in mill. Vkm

(Dublin Bus – Bus Atha Cliath – BAC)

| Mode | 1996 | 1998 | 2005 |
| --- | --- | --- | --- |
| Total | 52.1 | 52.9 | 57.2 |
| Urban/Local Bus | 52.1 | 52.9 | 55.6 |
| Tram | - | - | 1.56 |

- In table 7, traffic volumes for aviation are presented for Ireland.  The table shows the numbers of aircraft movements of commercial and charter traffic for the three national airports Dublin, Cork and Shannon.   A modest increase of 3% per year was used for air traffic in 2005.

Table 7

Transport volumes of aviation in Ireland, in 1 000 aircraft movements (Aer Rianta)

| Total air transport | 1996 | 1998 | 2005 |
| --- | --- | --- | --- |
| Commercial traffic | 228 | 246 | 302 |
| Scheduled and charter traffic | 137 | 162 | 200 |
| Non-scheduled | 25 | 23 | 28 |
| Training | 38 | 33 | 40 |
| Others | 28 | 28 | 35 |
| National airports |  |  |  |
| Dublin | 140 | 162 | 199 |
| Cork | 41 | 38 | 46 |
| Shannon | 47 | 46 | 56 |

- In Table 8, transport volumes for maritime shipping from Irish ports are shown in terms of the number of vessel arrivals at all 22 Irish ports.  Also included are the gross tonnage arriving at each port.  An increase of 3% per year has been applied to estimate the 2005 levels.

Table 8

Transport volumes of maritime shipping in Ireland, in 1 000 vessel movements

(Central Statistics Office)

| 1996 | 1996 | 1998 | 1998 | 2005 | 2005 |
| --- | --- | --- | --- | --- | --- |
| Number | Gross Tonnage (‘000) | Number | Gross Tonnage (‘000) | Number | Gross Tonnage (‘000) |
| 14424 | 45637 | 16669 | 176228 | 20501 | 216738 |

## Infrastructure Costs

### Road

There are several types of road in Ireland, classified as urban and rural.  Within each category there are motorway, divided and undivided.  The main arteries are the national roads, then there are regional followed by county roads, which form the lowest class in the rural road category.   In the urban context, the main roads are called arterials, the next are collectors followed by the local roads which would as in the case of county roads be undivided roads.  The main rural roads are the responsibility of the National Roads Authority and the remainder fall under the responsibility of the local authorities.  Available data enables some consideration of the national roads to be considered for UNITE but data on the others is not centralised and is not as robust.

The cost allocation method is based on five types of vehicles:

- cars

- light goods vehicles

- buses

- heavy goods vehicles

- goods vehicles with trailers

The capital charges are estimated using the perpetual inventory method based on a published series of capital spending on road construction on the Irish road network from 1967/68 to 1998.  This was first done in a report entitled Reviewing methodologies for the determination of road track costs by NERA for input to a project for the EU using data up to 1994, in which DIW was a partner.  Spending was converted to 1998 prices using a road construction price series published by the Department of the Environment.  An additional allowance of 5 % was added to interest charges to allow for the value of the land on which the roads are built.  Expenditure for earlier years has been extrapolated backwards to give an estimated series for 45 years.   A real discount rate of 5%, recommended for use in highway investment appraisals using cost-benefit analysis in Ireland by the National Road Authority was used to calculate annual capital charges.   Capital charges are split into those for new construction and enhancement and those for capital maintenance (surface reconstruction).

Current expenditure on the Irish network consists of:

- surface dressing

- cleaning and drainage

- road sign maintenance

- road markings

- winter maintenance

For the UNITE project, the expenditure data etc for the years between 1994 and 1998 were obtained from the National Roads Authority and the PIM model was run again with the additional years to 1998.

### Rail

Coras Iompair Eireann (CIE) and Iarnrod Eireann (Irish Rail - IR) publish the figures for the Irish rail account every year.  Some data are also included in the CSO Statistical Abstracts (1999 and 2000).  These figures show costs and revenues for the railway company for different cost and revenue categories and provide a basis for the cost requirements for UNITE for 1996 and 1998.

### Urban Public Transport

The infrastructure costs for buses are covered in the Irish road account mentioned above, although only at national level.    Trams will be introduced on two routes in Dublin in 2003/2004.

### Aviation

The accounts for air infrastructure in the Irish pilot account cover the three national airports (Dublin, Cork and Shannon). The basic information stems from the annual financial reports of Aer Rianta. For the pilot accounts consideration of all the regional airports and private airfields was not undertaken, since they are of minor relevance for civil aviation. The airports of Dublin, Cork and Shannon catered for 98% of passengers in 1996 and 97.7% in 1998 (Central Statistics Office, Statistics Bulletin, 2001).

### Inland Waterway Shipping

Inland waterways in Ireland are only used for leisure activities and therefore are not of interest in the pilot accounts of UNITE.

### Maritime Shipping

Estimates of infrastructure costs for Dublin port were obtained from the port authority.  The numbers provided were quite aggregate and little information is readily available for the other ports.

### Summary of Input Data

The following table gives an overview of the characteristics of the input data used per transport mode.

Table 9

Input data for the computation of infrastructure costs by transport mode

| Infrastructure | Input data | Level of disaggregation | Quality of data, level of uncertainty |
| --- | --- | --- | --- |
| Road | Financial data (costs, revenues) from the National Roads Authority. | Some disaggregation available | Satisfactory |
| Rail | Financial data (costs, revenues, subsidies). Data sources are the annual reports of CIE and IR. Asset and depreciation accounts of IR. Financial data from IR and the Central Statistics Office Statistical Abstracts 1999 and 2000. | Within the cost accounts the disaggregation was available as follows: Personnel, materials and services, depreciation and railway line renewals, exceptional operating costs and interests. | Satisfactory |
| Aviation | Financial data (costs, revenues, subsidies). Data sources are the annual reports of all the three national airports. | Some disaggregation | Satisfactory |
| Maritime Shipping | Financial data (costs, revenues, subsidies) from the two main ports - Dublin and Cork. Data sources are the annual reports of all the Dublin and Cork ports. | Aggregate values | Satisfactory |

## Supplier Operating Costs

### Rail

For the estimation of rail supplier operating costs the same input data as for infrastructure was used.  The basis for all computations are the Irish rail accounts (provided every year by the IR), which contain data on costs and revenues for all railways in Ireland in aggregated cost/revenue categories (personnel costs, material costs and running costs).

### Urban Public Transport

Financial data of urban and regional public transport official statistics are divided into two main categories:

- UPT (Urban Public Transport): Category focuses on Dublin Bus (urban rail is dealt with under the rail category)

- PT (Regional Public Transport): Category concentrates on Bus Eireann – the regional bus operator.  This operator also provides urban services for cities other than Dublin.

The main data source for supplier operating cost data in public transport consists of the official statistics of the Irish Central Statistical Office and the financial reports of the companies making up Coras Iompair Eireann (CIE).

The table below shows the relevant input data categories, their level of disaggregation and uncertainty:

Table 10

Input data for the computation of supplier operating costs by transport mode

|  | Input data | level of disaggregation | Quality of data, level of uncertainty |
| --- | --- | --- | --- |
| UPT | a) Transport data for urban buses: Passenger, Passenger-km, Vehicle-km. | Modes: Bus | Satisfactory |
|  | b) Financial Data (costs, revenues): | Modes: Bus | Satisfactory |
| PT | a) Transport data: Passenger, Passenger-km, Vehicle-km. | Modes: PT bus | Satisfactory |
|  | b) Financial Data (costs, revenues, subsidies): | Within the cost accounts only a rough disaggregation was available | Satisfactory |

### Maritime Shipping

Operating costs at an aggregate level.were provided by Dublin port

## Congestion Costs

### Values of Time

The values of time per passenger-hour are taken from the UNITE valuation paper (Nellthorp et al., 2001), purchasing power parity (PPP)-adjusted and converted into factor costs (commuting and leisure values only). According to the valuation paper (Nellthorp et al., (2001) it was assumed that Values of Time grow over time in line with real incomes (elasticity of 1.0 to the country's real GDP per capita). The respective values (GDP per capita) are taken from table 2.

Table 11

VOT-Values for Ireland - PPP-adjusted, (Nellthorp et al. ,2001), in € per hour,

1998 prices

|  |  | 1996 |  |  | 1998 |  |  | 2005 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Business | Commuting | Leisure | Business | Commuting | Leisure | Business | Commuting | Leisure |
| Car | 31.99 | 9.14 | 6.09 | 32.98 | 9.42 | 6.28 | 35.03 | 10.01 | 6.67 |
| Inter-urban rail | 31.99 | 9.75 | 7.16 | 32.98 | 10.05 | 7.38 | 35.03 | 10.67 | 7.84 |
| Coach | 31.99 | 9.14 | 6.09 | 32.98 | 9.42 | 6.28 | 35.03 | 10.01 | 6.67 |
| Air | 43.42 | 15.23 | 15.23 | 44.76 | 15.71 | 15.71 | 47.53 | 16.68 | 16.68 |
| Urban bus/tramway | 31.99 | 9.14 | 4.87 | 32.98 | 9.42 | 5.03 | 35.03 | 10.01 | 5.34 |

For information purposes, Irish estimates of values of time were estimated by DKM in 1994 to be 11.68 €/hr for working time by and 4.7 €/hr for non-working time and are presented in table 12.

The values of time used in the Dublin Transportation Initiative in 1995 were as follows:

Table 12

VOT-Values estimated for Ireland DKM (1994)

|  | Peak (€/hr) | Off-peak (€/hr) |
| --- | --- | --- |
| Car transport | 3.9 | 4.7 |
| Public transport | 1.5 | 4.7 |
| Slow modes | 3.1 | 9.4 |
| Freight | 14 | 14 |

- The UNITE VOT is considerably higher than the DKM estimates for business/working time somewhat higher but to a lesser degree for leisure time.

### Input Data

The following table shows the different data sets used for road transport to estimate congestion costs.   Unfortunately, insufficient data was available to conduct congestion cost estimation for rail, air or shipping.  In the case of rail, information was provided on aggregate punctuality of the train services.  IR's punctuality target is to have 90% of train journeys  on time.  While this information is of interest, it is not sufficient to make useful estimates of rail congestion costs.

Table 13

Input data, uncertainties and level of disaggregation for the computation of congestion costs for road transport

|  | Input data | Level of disaggregation | Quality of data, level of uncertainty |
| --- | --- | --- | --- |
| Road | a) NRA Roads Needs Study 1998 | All vehicles treated the same | Information on a small selection of towns i.e. estimates of travel times on average days and on extremely busy days |
|  | b) Model calculations for Dublin using SATURN network model and TRENEN functions | Modes: Car | Traffic model – calculation for Dublin – estimates of congestion in one peak hour and one off-peak hour |
|  | c) Fuel prices 1996, 1998: Average prices of the respective year provided by the Automobile Association | Gasoline, Diesel | Average values |

## Accident Costs

- The methodology and steps to calculate the costs of transport accidents is described in Link et al. (2000) and especially in Doll et al. (2000). In these documents, comprehensive lists of data requirements are stated. For the assessment of total accident costs, input data concerning several cost blocks has to be collected. Table 14 below summarises the main cost blocks that add up to the total costs of traffic accidents.

Table 14

Cost blocks of the total traffic accident costs

| - Cost blocks | - Valuation issue |
| --- | --- |
| - Material damages | - Valuation of damages to vehicles, parts of the infrastructure etc. involved in traffic accidents |
| - Administrative costs | - Costs associated with the treatment of accidents by the police, the legal system, the social security and insurance companies |
| - Medical costs | - Costs associated with the treatment of injuries caused by traffic accidents |
| - Production losses | - Current and future lost output of victims of traffic accidents, replacement costs of the firms |
| - Risk costs | - Valuation of fatalities and injuries caused by accidents |

- In the following section "Input data", a summary of the most important values and their sources that were used for the calculations of the different cost blocks for the UNITE transport modes are presented.

### Road

- The number of road accidents is published annually by the National Roads Authority (NRA) in their yearly Road Accident Facts reports at a level of detail which shows the numbers killed, numbers injured, age group, gender, road user type and area type (location of accident – urban/rural).

The issue of lack of reporting of accidents is an important one.  There does not appear to be agreement between the levels of accidents reported by the NRA and insurance industry data (Bacon, 1999).  The official data includes reporting by the police force (Gardai) and hospital admissions while industry data is based on the needs of individual insurance companies to meet claims.  The industry data is claims based, not accident based, so that it is impossible to infer the total number of accidents, and hence those in addition to those notified to the Gardai.  A research study looking at hospital admissions which is currently underway is likely to generate possibilities in providing estimates on accidents but this work is not yet complete.

- In table 15, the official figures for fatalities and injuries are shown. The rows show the vehicle categories of the victims of road accidents, the columns show the involvement of the vehicle categories.  Public service vehicles (PSV) refers to vehicles in the public service including public transport, ambulances etc.  GV refers to goods vehicles (including both light goods vehicles LGV and heavy goods vehicles HGV).

- A study conducted for the National Safety Council (Bacon, 1999) conducted an examination of road accidents in Ireland using existing available data in Ireland and translation of UK costs to Irish conditions.  Another study conducted for the Department of Public Enterprise on the environmental impact of Irish transport growth an of related sustainable policies and measures (Oscar Faber et al, 1999) makes estimates of accidents costs for some modes.  These two studies are used as the basis of estimation of accident costs.

- Table 16 gives a summary of the data available and its quality for estimation of road accident costs.

Table 15

Number of killed and injured victims of road accidents in Ireland, 1998

| Involved | ALL |  | REPORTED ACCIDENTS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Element |  | Single | Two Party Accidents |  |  |  |  |  |
|  |  | Accidents | Pedal Cycle | Motor Cycle | Car | PSV | GV | Other |
| All |  |  |  |  |  |  |  |  |
| Accidents | 31843 | 2567 |  |  |  |  |  |  |
| Fatalities | 408 | 200 | 21 | 37 | 253 | 6 | 21 | 6 |
| Injuries | 7831 | 2367 | 566 | 1085 | 8210 | 138 | 798 | 143 |
| Pedal Cycle |  |  |  |  |  |  |  |  |
| Accidents |  | 13 | 0 | 14 | 432 | 14 | 78 | 20 |
| Fatalities |  | 0 | 0 | 0 | 13 | 1 | 7 | 0 |
| Injuries |  | 13 | 0 | 19 | 437 | 13 | 73 | 20 |
| Motor Cycle |  |  |  |  |  |  |  |  |
| Accidents |  | 95 | 14 | 6 | 717 | 4 | 94 | 26 |
| Fatalities |  | 2 | 0 | 0 | 5 | 0 | 6 | 3 |
| Injuries |  | 93 | 19 | 9 | 842 | 4 | 95 | 25 |
| Car |  |  |  |  |  |  |  |  |
| Accidents |  | 1047 | 432 | 717 | 2264 | 88 | 783 | 192 |
| Fatalities |  | 54 | 13 | 5 | 69 | 3 | 51 | 12 |
| Injuries |  | 993 | 437 | 842 | 4511 | 164 | 1325 | 319 |
| Public Service Vehicles |  |  |  |  |  |  |  |  |
| Accidents |  | 91 | 14 | 4 | 88 | 0 | 23 | 4 |
| Fatalities |  | 16 | 1 | 0 | 3 | 0 | 5 | 0 |
| Injuries |  | 75 | 13 | 4 | 164 | 0 | 57 | 13 |
| Goods Vehicles |  |  |  |  |  |  |  |  |
| Accidents |  | 48 | 78 | 94 | 783 | 23 | 95 | 32 |
| Fatalities |  | 1 | 7 | 6 | 51 | 5 | 4 | 2 |
| Injuries |  | 47 | 73 | 95 | 1325 | 57 | 135 | 43 |
| Other |  |  |  |  |  |  |  |  |
| Accidents |  | 31 | 20 | 26 | 192 | 4 | 32 | 7 |
| Fatalities |  | 2 | 0 | 3 | 12 | 0 | 2 | 0 |
| Injuries |  | 29 | 20 | 25 | 319 | 13 | 43 | 7 |

Table 16

Input data for the valuation of the road accidents

| - Cost category | - Data sources, main input data | - Level of disaggregation | - Quality of data, level of uncertainty |
| --- | --- | --- | --- |
| - Material damages | - Insurance industry data | - Aggregate estimates - not much detail | - Sufficient disaggregation not available |
| - Administrative costs | - Total payment for motor insurance to the Department of Enterprise, Trade and Employment. | - Little disaggregation between cost category | - Sufficient disaggregation not available |
| - Medical costs | - Data not available - transfer from UK costs to Irish conditions used. | - Average values. | - Estimate |
| - Production losses | - Irish data from the CSO and the NRA on age profile and GNP per capita . | - Average values | - Estimate |
| - Risk value (Value of Statistical Life VOSL) | - Source: Valuation conventions of UNITE (Nellthorp, 2001) - VOSL: 1.5 mill. € - Adjustments: - - Income, adjustment factor: 1.0877 - - Factor costs, adjustment factor: 0.0 - VOSL after adjustment: 1.63€ - Value for injuries in % of VOSL: - - Severe permanent: 32% - - Severe temporary: 4% - - Light: 1% | - - - | - An Irish study contains an estimate of 0.857 mill. € / fatality (Bacon, 1999) |

### Rail

- The number of fatalities and injuries caused by rail accidents are available from Irish Rail (IR). The statistics contain detailed information about the different type of accidents (e.g. accidents during shunting, involved categories of persons (employees, passengers and third parties), accidents on level crossings). However, the distinction between freight and passengers trains is not made.   In contrast to road transport, the problem of underreporting is of minor relevance in the case of rail transport. The data available for rail accident costs is not as well developed as in the case of roads.  The accidents involving multiple fatalities since 1955 are presented in Table 17 and Table 18 shows the accident data for the period 1993 - 1997.  IRMS (1999) in a study of rail safety for the Department of Public Enterprise estimated the equivalent fatalities per annum for each incident type and these values are shown in Table 19.

Table 17

Number of killed and injured victims of rail accidents, 1955 – 1998 (IRMS 1999)

| Date | Location | Circumstances | Fatalities |
| --- | --- | --- | --- |
| 21 12 55 | Cahir | Train crashed through buffer | 2 staff (driver & fireman) |
| 21 10 74 | Gormanston | Rear-end collision | 2 passengers |
| 31 12 75 | Gorey | Derailment on bridge | 1 staff and 2 passengers |
| 01 08 80 | Buttevant | Train derailment at speed | 2 staff and 16 passengers |
| 21 08 83 | Cherryville | Rear-end collision | 7 passengers |

Table 18

Rail accident data 1993 –1997(IRMS 1999)

| Train Accidents |  |  | Other Accidents |  |  |
| --- | --- | --- | --- | --- | --- |
| Incident Type | Total no. of | Total no. of |  | Total no. of |  |
|  | Incidents | incidents |  | Incidents |  |
|  |  | resulting in |  |  |  |
|  |  | injury |  |  |  |
|  |  |  |  | Staff | Passengers/ |
|  |  |  |  |  | Public |
| Derailments | 460 | 3 | Struck by | 329 | 227 |
| Collision (all types) | 90 | 4 | Other | 207 | 137 |
| Running into obstructions | 102 | 5 | Injured while handling | 509 | 8 |
| Train fires | 20 | 0 | Slips, falls | 409 | 594 |
| Breakloose | 40 | 0 | Trapped | 60 | 76 |
| Signal passed at danger (SPAD) | 90 | 0 | Exposure | 103 | 68 |
|  |  |  | Miscellaneous | 209 | 215 |

Table 19

Equivalent fatalities per annum for each incident type (IRMS, 1999)

|  |  |  | ALL ACCIDENTS |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Equivalent Fatalities |  |  |  | Equivalent Fatalities |  |  |
|  | Passengers | Public | Staff |  | Passengers | Public | Staff |
| Incident type |  |  |  | Incident type |  |  |  |
| Derailments | 0.02 | 0.02 | 0.02 | Struck by | 0.62 | 0.23 | 1.09 |
| Collision | 0.005 | 0 | 0.001 | Other | 0.39 | 0.63 | 0.6 |
| Running into obstructions | 0.08 | 0.24 | 0.2 | Injured while handling | 0.008 | 0.0002 | 0.95 |
| Train fires | 0 | 0 | 0 | Slips, falls | 2.37 | 0.09 | 0.9 |
| Break loose | 0 | 0 | 0 | Trapped | 0.14 | 0.009 | 0.21 |
| SPADs | 0 | 0 | 0 | Exposure | 0.07 | 0.002 | 0.18 |
|  |  |  |  | Miscellaneous | 0.48 | 0.03 | 0.48 |

### Urban Public Transport

- Available accident data for urban public transport was limited.

### Aviation

- The number of people killed and injured in aviation accidents in 1996 are presented in Table 20 followed by a similar table for 1998.

Table 20

Number of killed and injured victims of aviation accidents in 1996 (IRMS, 2001)

- * MTOW  =  Maximum take-off weight

Table 21

Number of killed and injured victims of aviation accidents in 1998 (IRMS, 1999)

- * Air Accident Investigation Unit, Department of Public Enterprise

- The data available is insufficient to enable an estimate of air accident costs.

## Environmental costs

- The environmental cost category contains different types of costs, namely

- Air pollution

- Global warming

- Noise

Estimates could not be made for nature, landscape and further environmental effect or for nuclear risk for Ireland.

- The discussion of the input data below is divided according to these different cost categories.

### Air Pollution

- For quantifying the costs due to airborne pollutants the Impact Pathway Approach, the methodology developed in the ExternE project series has been applied. It comprises the steps

- emission estimation,

- dispersion and chemical conversion modelling,

- calculation of physical impacts, and

- monetary valuation of these impacts.

- For the calculation of the costs of direct emissions from vehicle operation, emission inventories in spatial disaggregation are needed, i.e. a geo-coded data set for the different air pollutants. For each emission inventory, Europe-wide impacts are calculated and subtracted from impacts resulting from a reference inventory without these emissions. This procedure using a reference inventory is required, because of air chemistry processes where “background” emissions play an important role.

- Besides emission data, the distribution of the population over space is the second central input for the calculations of the most important costs of air pollution, i.e. the health costs.  This type of exposure data is not available for Ireland.

- In the case of Ireland, estimates of emissions are estimated by the Environmental Protection Agency using an energy balance model for fuel used in all road vehicles.

Table 22 shows the emissions estimate for road transport using the energy balance done by the Environmental Protection Agency for 1996 and it is followed by a similar set of estimates for 1998.

Table 22

Emission of air pollutants of road transport, in t/a, 1996 (EPA)

| Sector | PM | SO2 | NOx | NMVOC | CH4 | CO | CO2 | N2O | NH3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Passenger Cars |  | 3391 | 34989 | 58306 | 1783 | 218946 | 4396103 | 425 | 577 |
| Gasoline |  | 2127 | 31682 | 57429 | 1742 | 215165 | 3394761 | 364 | 571 |
| Diesel | 1165 | 1264 | 3224 | 841 | 39 | 3599 | 994132 | 61 | 6 |
| LPG |  | 0 | 83 | 36 | 2 | 182 | 7210 | 0 | 0 |
| Light Duty Vehicles |  | 1570 | 4802 | 2388 | 55 | 11702 | 1266526 | 73 | 5 |
| Gasoline <3.5t |  | 38 | 392 | 945 | 28 | 6162 | 61367 | 1 | 0 |
| Diesel <3.5t | 1383 | 1532 | 4410 | 1443 | 27 | 5540 | 1205159 | 72 | 5 |
| Heavy Duty Vehicles |  | 791 | 5263 | 1345 | 36 | 2803 | 624415 | 38 | 3 |
| Gasoline >3.5t |  | 2 | 47 | 29 | 1 | 370 | 3466 | 0 | 0 |
| Diesel | 464 | 789 | 5216 | 1316 | 35 | 2433 | 620949 | 38 | 3 |
| Buses |  | 417 | 4784 | 511 | 44 | 1384 | 328096 | 11 | 1 |
| Urban Busses | 125 | 223 | 2776 | 258 | 25 | 915 | 175121 | 5 | 0 |
| Coaches | 100 | 194 | 2008 | 253 | 19 | 469 | 152975 | 6 | 1 |
| Mopeds & Motorcycles |  | 26 | 72 | 1791 | 82 | 11937 | 41702 | 1 | 1 |
| Mopeds |  | 5 | 3 | 910 | 10 | 1467 | 7786 | 0 | 0 |
| Motorcycles |  | 21 | 69 | 881 | 72 | 10470 | 33916 | 1 | 1 |
| Grand Totals: | 3239 | 6195 | 49910 | 64341 | 2000 | 246772 | 6656842 | 548 | 587 |

Table 23

Emission of air pollutants of road transport, in t/a, 1998 (EPA)

| Sector | PM | SO2 | NOx | NMVOC | CH4 | CO | CO2 | N2O | NH3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Passenger Cars |  | 3470 | 33458 | 58130 | 1961 | 215582 | 4767413 | 658 | 1034 |
| Gasoline |  | 2525 | 30929 | 57532 | 1928 | 212835 | 4018551 | 609 | 1029 |
| Diesel | 730 | 945 | 2446 | 562 | 31 | 2565 | 741652 | 49 | 5 |
| LPG |  | 0 | 83 | 36 | 2 | 182 | 7210 | 0 | 0 |
| Light Duty Vehicles |  | 2657 | 6849 | 3637 | 85 | 17377 | 2127714 | 121 | 8 |
| Gasoline <3.5t |  | 55 | 555 | 1327 | 40 | 8736 | 87001 | 2 | 1 |
| Diesel <3.5t | 1987 | 2602 | 6294 | 2310 | 45 | 8641 | 2040713 | 119 | 7 |
| Heavy Duty Vehicles |  | 1176 | 7440 | 1797 | 51 | 3549 | 923743 | 54 | 6 |
| Gasoline >3.5t |  | 3 | 58 | 35 | 1 | 457 | 4290 | 0 | 0 |
| Diesel | 642 | 1173 | 7382 | 1762 | 50 | 3092 | 919453 | 54 | 6 |
| Buses |  | 535 | 5010 | 544 | 56 | 1350 | 419569 | 14 | 2 |
| Urban Busses | 127 | 273 | 2899 | 273 | 31 | 846 | 214419 | 6 | 1 |
| Coaches | 112 | 262 | 2111 | 271 | 25 | 504 | 205150 | 8 | 1 |
| Mopeds & Motorcycles |  | 28 | 72 | 1609 | 81 | 11123 | 44457 | 1 | 1 |
| Mopeds |  | 5 | 3 | 915 | 10 | 1476 | 7832 | 0 | 0 |
| Motorcycles |  | 23 | 69 | 694 | 71 | 9647 | 36625 | 1 | 1 |
| Grand Totals: | 3598 | 7866 | 52829 | 65717 | 2234 | 248981 | 8282896 | 848 | 1051 |

- The particulate matter estimates for 1996 and 1998 from road transport are shown as follows:

Table 24

Particulate matter emissions from road transport, in t/a, 1996, 1998 (EPA)

|  | 1996 | 1998 | 1996 | 1998 | 1996 | 1998 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Hot | Hot | Cold Start | Cold Start | Total | Total |
| Passenger Cars | 863 | 543 | 302 | 187 | 1165 | 730 |
| Diesel <2.0 l | 766 | 480 | 269 | 165 | 1035 | 645 |
| Diesel >2.0 l | 97 | 63 | 34 | 22 | 130 | 85 |
| Light Duty Vehicles | 1102 | 1569 | 281 | 418 | 1383 | 1987 |
| Diesel <3.5 t | 1102 | 1569 | 281 | 418 | 1383 | 1987 |
| Heavy Duty Vehicles | 690 | 881 |  |  | 690 | 881 |
| Diesel 3.5 - 7.5 t | 110 | 107 |  |  | 110 | 107 |
| Diesel 7.5 - 18 t | 354 | 535 |  |  | 354 | 535 |
| Diesel 16 - 32 t | 0 | 0 |  |  | 0 | 0 |
| Diesel > 32 t | 0 | 0 |  |  | 0 | 0 |
| Urban Buses | 125 | 127 |  |  | 125 | 127 |
| Coaches | 100 | 112 |  |  | 100 | 112 |
| Total | 2655 | 2993 | 583 | 605 | 3238 | 3598 |

- For other modes, less detailed information is available.  Some estimates for rail and air transport are presented in the study of the environmental impact of Irish transport growth and of related sustainable policies and measures commissioned by the Department of Public Enterprise.  Estimates for rail are presented in table 25.

Table 25

Rail emissions estimates in Ireland in t/a, 1996 (Oscar Faber, 1999)

|  | Diesel Trains | Electric Trains | Total |
| --- | --- | --- | --- |
| CO2 | 95236 | 12868 | 108104 |
| NMVOC | 141 | - | 141 |
| NOX | 1202 | 39.4 | 1241.4 |
| SO2 | 85 | 66.8 | 151.8 |
| PM | 139 | 4.3 | 143.3 |

- Estimates for emissions from the aviation sector are presented in table 26.

Table 26

Emissions from aviation in Ireland, in t/a, 1996 (Oscar Faber, 1999)

|  | Air Traffic Emissions |
| --- | --- |
| CO2 | 1942026 |
| NMVOC | 2994 |
| NOX | 5183 |
| SO2 | 616 |

### Global Warming

- The input data for the calculation of the costs of CO2 have been calculated from fuel consumption data and from information on energy consumption and the electricity production mix (rail). In the case of aviation, CO2 emissions estimates were made based on fuel consumption.

### Noise

- Insufficient data is available to quantify the proportion of the population that is exposed to different levels of noise accurately.  Tinch (1997) estimated levels for a range of EU countries.  In the case of Ireland the following table gives an indication of the proportion of the population exposed to different noise levels.

Table 27

No. of persons in millions exposed to each noise level dB(A) (Tinch, 1997)

|  | 50-55 | 55-60 | 60-65 | 65-70 | 70-75 | 75+ |
| --- | --- | --- | --- | --- | --- | --- |
| Road | 0.8 | 0.59 | 0.38 | 0.22 | 0.07 | 0.02 |
| Rail | 0.15 | 0.11 | 0.07 | 0.03 | 0.01 | 0.01 |

- Data was not available to estimate exposure of population to air noise.

## Taxes, Charges and Subsidies

Taxes, charges and subsidies are calculated and estimated within the respective infrastructure or supplier operating costs section of each mode. The following table presents the main data sources which are basically the same as data sources for infrastructure and supplier operating costs.

Table 28

Input data and data quality for the calculation of taxes, charges and subsidies

|  | Input data | level of disaggregation | Quality of data, level of uncertainty |
| --- | --- | --- | --- |
| Road | Infrastructure cost accounts (revenue part), based on national road accounts | Info on vehicle registration tax, annual tax, fuel tax and company car tax. | Satisfactory |
| Rail | Rail company accounts | Revenues Subsidies | Satisfactory |
| Urban and regional PT | National accounts | Revenues Subsidies | Satisfactory |
| Aviation | Business accounts of airports | Type of revenue | Satisfactory – aggregate values |
| Ports | Business accounts (ports) | Type of revenue | Satisfactory – aggregate values |

# Methodological Issues

- The methodology on how to proceed within the different cost fields to assess the transport costs is described in detail in Link et al. (2000) and very briefly summarised in section 1.2 of this Appendix Report. Against this background, we concentrate on the following issues:

- deviations from the general methodology developed in Link et al. (2000) and/or specific methodological issues that should be discussed in more detail within UNITE

## Infrastructure Costs

For roads, we carried out the methodological working steps for the calculation of infrastructure costs which correspond to Link et al. (2000) (see figure 3-1 below).

The applied methodology differs for all modes from Link et al. (2000) in the following points :

- Aggregation level: Depreciation and interests for a cost category are shown overall, not further differentiated.

- Cost structure: No differentiation between fixed and variable costs because we had not enough information existed for a reliable and accurate split.

- Data availability: For airports, data is presented at an aggregate level.

Table 29

Overview of the procedure for calculating infrastructure costs per transport mode

### Road

Base year data was obtained from the National Roads Authority as well as data for 1996.  The starting point of the forecast for 2005 is from the results for 1998.  The results of the PIM model were extrapolated from the 1996 and 1998 data for the year 2005 using standard trend extrapolation.

### Rail

Data sources are the profit and loss accounts divided into the categories of infrastructure, transportation of passengers and transportation of freight as well as the asset account.   Infrastructure costs are reported as well as the asset value and net book amounts.

The accounts for 1996 (1997) were available and trend extrapolation using data from 1996 – 2000 was used to estimate 2005 levels.

### Aviation

The base information was taken from the business accounts (annual reports) from the three national airports Dublin, Cork and Shannon.  It was not possible to separate the information from the annual reports into flight related infrastructure and non-flight related infrastructure (on the cost side it is totally impossible, on the revenue side some parts would be applicable). Therefore the whole profit and loss account is used for this pilot account.

For 1996 the methodology used was the same and trend extrapolation is used to make estimates for 2005.

### Maritime Shipping

Costs were provided on request from Dublin port for 1996, 1998 and 2000.  The infrastructure costs provided include infrastructure and some machinery in the form of Ro-Ro ramps and cranes.

## Supplier Operating Costs

### Rail

Irish Rail provides annual data on costs and revenues of all Irish railways in which aggregated running costs (among other categories such as depreciation, interest, personnel costs, etc.) are accounted.

The following list indicates the available cost categories but there is no disaggregation within the accounts between passenger and freight.  Allocations of cost were made to passenger and freight using the ratio of train km made by each.

- Material, goods, services (includes consumables, fuel costs, maintenance, cleaning and servicing)

- Personnel

- Other running costs (e.g. tenure and rental, fees, compensation for expenses or employees, insurances and compensation)

- Depreciation

- Interest

- VAT

Revenues (as an aggregate) and subsidies are also displayed.

The IR annual report for 1997 is readily available and this was used instead of the 1996 accounts.  No projections were supplied by IR and therefore trend extrapolation is used for 2005.

### (Urban) Public Transport

The Dublin Bus financial accounts are used to estimate the supplier operating costs for urban public transport.

Supplier operating costs of urban and regional public transport services are taken directly from the transport company financial accounts for the base year.

The following list shows the allocation categories of the Irish pilot accounts:

- Material, goods, services

- Personnel

- Other running costs

- Depreciation

- Interest

A similar set of data was available for 1997 and this was used instead of 1996.

Future supplier operating costs are estimated by means of trend-extrapolation of the relevant transport figures (i.e. mileage of rolling stock, number of transported passengers)

### Maritime Shipping

Operating costs were supplied by Dublin Port at quite an aggregate level.

## Congestion Costs

### Road

The congestion costs on the national road network are examined below and are followed by estimates for road congestion costs for the largest urban area in Ireland - Dublin.

#### National Road Network

The NRA includes some travel time data for a selection of towns in the National Roads Needs Study conducted in 1998.  The table below gives estimates for an average day and for an extra busy day.  Kildare and Portlaoise are large towns which have or will be by-passed in the near future.  Similarly other towns of their nature on the national primary network are to be by-passed in the future.  The delay estimates for those two towns, when looking at the table below, are higher than the delay experienced in the other smaller towns when comparing an extra busy day to an average day.   Anecdotal evidence would suggest however, that longer delays are experienced at very busy times in some of the towns listed.

Table 30

Travel time estimates for a selection of towns on the Irish road network (NRA, 1998)

| TOWN | AVERAGE DAY | EXTRA BUSY DAY | DIFF (MINS) |
| --- | --- | --- | --- |
| KILDARE | 4.72 | 7.47 | 2.75 |
| MONASTEREVAN | 2.18 | 2.22 | 0.04 |
| PORTLAOISE | 7.54 | 10.36 | 2.82 |
| MOUNTRATH | 2.48 | 2.59 | 0.11 |
| BORRIS | 1.32 | 1.32 | 0 |
| ROSCREA | 3.92 | 3.9 | 0 |
| MONEYGALL | 1.29 | 1.32 | 0.03 |
| TOOMEVARA | 1.3 | 1.31 | 0.01 |
| PATRICKSWELL | 2.74 | 2.85 | 0.11 |
| NEWCASTLE | 3.32 | 3.45 | 0.13 |
| CASTLEISLAND | 2.54 | 2.57 | 0.03 |
| ADARE | 3.14 | 3.32 | 0.18 |
| ABBEYFEALE | 4.03 | 4.11 | 0.08 |
| NENAGH | 4.53 | 5.51 | 0.98 |

In order to estimate congestion costs on the entire network, a series of simplifying assumptions were required.  For its travel time estimates, the NRA uses a simulation model which treats a year on the basis of 250 working days.    The model has five traffic conditions – traffic is at average levels (median), days on which traffic is moderately light, moderately busy days, extremely busy days and extremely light days.  The 250 days are divided as follows by the NRA:

Median:  80 days

Moderately light:  80 days

Moderately busy:  80 days

Extremely busy:   5 days

Extremely light:   5 days

One of the difficulties when concentrating only on 250 working days is that weekend travel is excluded.  Significant delays can be experienced on Sunday evenings particularly on the main routes to Dublin.  The other point is that there can be considerable fluctuations within any one day and this may generate sizeable errors in any estimate of congestion costs based on this breakdown of traffic.

However, in the absence of detailed delay studies, the data available was used with a series of assumptions to estimate congestion costs on the road network.  Where possible sensitivity analysis is conducted.

The first assumption made was the level of delay on any one of the particular types of day.  It was assumed that for extremely busy days the average delay in the bigger towns (Type 1) was 2.78 mins taking an average of the two larger towns in the above table and in the case of the others (Type 2) 0.14 minutes.  It was assumed on moderately light days and on extremely light days that there was no delay.  On moderately busy days it was assumed that the delay was 50% that experienced on extremely busy days and in the case of the median days, it was assumed there would be some delay on those days also, of the order of 25% or the extremely busy days.

Another assumption regarding the average trip length to be used in the calculations was required.  The calculation is done for an average trip length of 70 km assuming that a vehicle goes through 1 larger town and 6 smaller towns.  Sensitivity to trip length was testsed using an average trip length of 40 km in another calculation was done assuming that a vehicle passed through 1 large town and 2 small towns.

The values of time used in the calculation are presented earlier in chapter 2.  A report on forecasts of vehicle numbers and traffic volumes for Ireland by DKM (1998) gives an estimate of the proportion of time that vehicles spend on the network for business purposes, commuting and leisure.  For the purposes of this calculation it was assumed that 0.2 of traffic on the network is business related, 0.6 is commuting and 0.2 is leisure traffic.  The corresponding VOTs for these purposes in Section 2 of this report are used in the calculation.

To estimate the additional fuel costs due to congestion the same rate used in the German accounts is used here i.e. that a vehicle consumes 0.05 litres/km more in congested conditions.

It should be noted that the delay costs estimated here are rough and significant errors may be associated with them.

#### Dublin

The Dublin Transportation Office estimated congestion costs in Dublin to be 635 million Euro for 1996.  This was done using the SATURN network model of the city comparing the loss of time spent in traffic queues measured in passenger car unit hours /hour compared with a traffic free scenario.  A VOT of 4.53 Euro was used in the analysis.

The delay - volume relationships used in the TRENEN model have also been calculated for 1996 and 1998 for UNITE.  The idea behind the TRENEN model which uses a macro approach is to estimate a relationship between the time taken to travel one km and the level of travel on the network in veh.kms is to have one relationship to represent delay in a city.  This was used to estimate the difference in travel time compared with no traffic on the network and using the VOTs for Ireland presented earlier, estimates of congestion costs can be made.

### Rail

Methodology for congestion costs rail passenger

Irish Rail's target on punctuality is that 90% of trains should run on time.  Irish Rail provided the aggregate performance levels of their routes for the first six months of 2001.  These results are presented but are not sufficient to enable an estimate of congestion costs associated with rail transport.

### (Urban) public transport

For Ireland no study on congestion or delay costs in public transport exists so far.   The public transport companies did not respond to requests for data on public transport delay.  It is assumed therefore that they do not collect it.

## Accident Costs

Beside the back- and forecast methodology, two issues concerning the calculation of the accident costs deserve some further methodological discussion in this section:

- the consequences of the victim perspective of UNITE with regard to cost allocation

- the distinction between system-internal and -external costs

- In section 2.5.1, it is mentioned that UNITE follows, in the case of accident costs, the "victims perspective" (monitoring perspective). This perspective results in allocation problems for road transport:

- Another important methodological aspect is the distinction between internal and external costs with regard to the risk value, i.e. the willingness-to-pay (WTP) for avoiding death casualties or injuries caused by traffic accidents. Looking at the order of magnitude of this "cost block", this distinction crucially influences the total level of the external accident costs. In the theoretical part of UNITE it is mentioned that this distinction depends on the risk awareness of the individuals participating in transport. In our view, it is rather the perspective that is decisive for where to draw the line:

- Perspective of the transport system: Under this perspective the risk value (WTP-value) is an internal cost block because it refers to actors of the transport system. Only the grief and suffer of the relatives would be transport system external. However, in Doll et al. (2000) it is made clear that due to data and information problems it is not possible within UNITE to separate this part of the WTP-value from the part referring to the victim of the traffic accident itself. We do not recommend use of the notion of  "the risk value is internalised" because we understand internalisation as cost allocation according to the "polluter-pays-principle" which isn't the case here. The external cost part is limited to those transfer payments from the social security to persons involved in accidents that are not covered by payments of the auto liability insurances (i.e. transport system internal insurances) to the social security. With regard to the objective of the accounts, namely to prepare information about total costs and revenues of the transport system, this perspective - or rather the very similar perspective per transport mode road, rail etc. - is the relevant one.

- With the methodological discussion above we would like to emphasise that the choice of the victims perspective in UNITE has advantages with regard to data collection (the information about the causer of accidents is in many countries not immediately available) but it has considerable disadvantages when it comes to cost allocation and the distinction between internal and external cost parts that is very relevant for pricing. The latter, however, is not the main goal of the figures collected in transport accounts.

- For the derivation of the accident costs of the years 1996 and 2005 the procedure described below is chosen.

The total number of reported accidents is available for the year 1998 for road transport.  In the case of rail transport, data over several years is used to estimate an equivalent number of accidents per year.  In the case of road, data is available for the distribution of the accidents to the different vehicle categories and according to the severity of the accidents.  For aviation, accident data is also available.

Forecast methodology: For the assessment of the accident costs for the year 2005, forecasts concerning the changes in the accident rates are needed. The calculations of the 2005 accident  costs are done using a 20% increase in costs from 1998.  Although this rate is high, it is considered justified as accident costs are likely to increase significantly in coming years.

No adjustment was made for rail because the yearly accident rates are derived from data over several years.  No adjustment was made for aviation because the accident rates in 1996 and 1998 are quite similar

Available forecasts about the development of the GDP, salary level etc. were used to adjust the cost figures in the case of road transport.

## Environmental Costs

### Air Pollution

The calculations carried out with the ExternE model followed the method as described in Link et al. (2000).

- In the case of road transport, the total emissions of most air pollutants increased in Ireland matching a growth in traffic.  Unlike other countries though e.g. Switzerland the presence of 'cleaner cars' does not appear to have initiated a decrease in emissions from the road sector.  Ireland experienced increased economic activity during this period and it is thought that this has had an effect on the higher levels of demand for road transport which may mask any improvements as a result of clean cars.  Emissions estimates were provided by the Environmental Protection Agency from energy balances for 1996 and 1998.   Modest increases are used for 2005.

- The emissions from the road sector in Ireland were presented in Section 2.1 of this report.

- In the case of rail transport, a report commissioned for the Department of Public Enterprise (Oscar Faber, 1999) gave some estimates of emissions for 1996 but few further estimates or measurements have been made.  The number of train kms in 1996 was 15.39 million compared with 15.47 million in 1998.  As the increase is only marginal, the same emissions estimates are used in 1998 as in 1996.

In the case of air transport, similar estimates are made in the same report as in the case of rail i.e. for 1996.  An estimate for 1998 is made based on the increased level of air transport experienced between 1996 and 1998.

### Noise

Noise estimates could not be found for any of the modes.  Tinch (1997) estimated the exposure of the Irish population to various noise levels.   These estimates were already presented in Section 2.    Given the lack of data on this topic and the difficulty in predicting changes given the non-linearity of the relationship between traffic growth and noise levels, it is proposed to use the set of Tinch estimates for 1996 and 1998.

## Taxes, Charges and Subsidies

Ireland has a wide range of taxes across all transport modes.  Each mode is dealt with below in more detail.

### Road

Transport taxes in the road sector comprise the following:

- Vehicle registration tax

- Annual vehicle tax

- Excise tax on fuels

- Company car tax

The structure of vehicle registration taxes (on purchase of a vehicle) in 1998 was as follows:

Table 31

Structure of vehicle registration taxes in Ireland

| Engine size (cc) | Rate (% of car value) |
| --- | --- |
| <2500 | 22.5 |
| >2500 | 28 |

A new three-band system was introduced in 1999 as follows:

0 – 1400 cc:    	22.5%

1401 – 2000 cc:  	25%

2001 + cc:   		30%

Rates of 13.3% are applied to commercial cars and 50.8 € to other commercial vehicles.

Annual vehicle taxes are based on engine power in the case of cars and weight in the case of goods vehicles.  The current Irish rates are presented as follows:

Table 32

Annual vehicle taxes in Ireland (DOE, 2001)

| Vehicle / engine capacity (cc) | Rate (€) | Vehicle | Rate (€) |
| --- | --- | --- | --- |
| Cars |  | Goods Vehicles |  |
| <1001 | 129.51 | < 3000 kg | 216 |
| 1001 – 1300 | 193 – 231 | 3001 – 8000 kg | 273 – 829 |
| 1301 – 1600 | 249 – 333 | 8001 – 11000 kg | 1025 – 1416 |
| 1601 – 1900 | 353 – 436 | 11001 – 14000 | 1611 – 2002 |
| 1901 – 2200 | 458 – 615 | 14001 – 20000 | 2003 + 196 € / 1000kg or part thereof over 14000 kg |
| 2201 – 2500 | 643 – 698 | >20001 | 3357 |
| 2501 – 3000 | 818 – 943 | Electrical (<1500 kg) | 68.56 |
| >3001 | 1143 | Large Public Service Vehicles (seating capacity) | 99 – 262 |
| Electric cars | 124 | Taxis | 62.2 |
|  |  | School bus | 62.2 |
|  |  | Motorcycles (cc) | 31.74 – 57.13 |
| Goods vehicles (< 8 tonnes) | 190.5 – 907.9 | Electrical | 27.93 |
| Goods vehicles (>20 tonnes) | 2984 |  |  |

The current taxes on fuel are as follows:

Table 33

Fuel taxes in Ireland

| Fuel type | Tax rate (€/kl) |
| --- | --- |
| Leaded petrol | 459 |
| Unleaded petrol | 374 |
| Diesel | 325 |
| LPG | 52 – 105 / tonne |
| Kerosene | 325 |

The use of company cars is treated as a benefit  (benefit-in-kind – bik).  Generally the rate applied is 30% of the original market value.  If the employee pays for all private motoring fuel the rate is reduced to 25.5% and where the employee pays for either the insurance or for all servicing and repairs the rate is reduced to 27% (if they pay both, the amount by which the bik is reduced is 2 * 3 = 6%).

There is no bik on a car if it is a car pool vehicle.  This is where the car can be used by more than one employee and is not used by one employee to the exclusion of others, private use is incidental to business and the car is not kept in the vicinity of the employees' homes.

### Rail, Urban Public Transport and Regional Public Transport

The levels of state grant to public transport companies in Ireland are included in the annual company accounts.  This includes a subvention and any capital grants awarded.

# The Results in Detail

## Infrastructure Costs

### Overview

The methodology for infrastructure costs according to the relevant deliverable sought a differentiation between infrastructure and the operating level for all modes. This was possible in the case of road and rail but not in the case of air. Therefore, the aggregated figures for airports are shown in the infrastructure chapter.  Additionally, for all modes a split into variable and fixed costs was requested. In the following accounts only total costs are shown. Basic information for all modes was not detailed enough for the requested split.

### Road

The infrastructure data for roads is presented in the following table.

Table 34

Road infrastructure costs in million €, in 1998 prices

|  | 1996 | 1998 | 2005 |
| --- | --- | --- | --- |
| Gross capital value 1) | 2634 | 3052 | 3470 |
| Net capital value 1) | 2342 | 2359 | 2375 |
| Depreciation | 61 | 61 | 61 |
| Interests | 70 | 71 | 71 |
| Capital costs2) | 131 | 131 | 132 |
| Land value in mill. € | 1051 | 1067 | 1084 |
| 1) Including land value |  |  |  |
| 2) Including land costs, calculated as average over the financial year |  |  |  |

2005 values were estimated using extrapolations from 1996 and 1998 where there is an overall increase of about 14% between 1998 and 2005.  The cost data are further broken down per vehicle type in the following table.

Table 35

Road infrastructure costs per vehicle type in million €, in 1998 prices

|  | 1996 | 1998 | 2005 |
| --- | --- | --- | --- |
| Motorcycles | 1.99 | 3.11 | 3.12 |
| Cars | 208.00 | 187.30 | 188.02 |
| Buses/Coaches | 2.67 | 3.13 | 3.14 |
| Heavy Goods Vehicles | 49.29 | 69.42 | 69.68 |

### Rail

Infrastructure costs for rail have been divided into passenger transport related and freight transport related costs. Irish Rail only provides overall infrastructure figures. The separation has been done by using the breakdown between passenger kms and tonne kms.  The ratio between passenger kms and freight tonne kms is 2.65 / 1 (IR, 2001)

Table 36

Infrastructure costs rail, in million €, 1998 prices (IR yearly financial accounts)

|  | 1996(1997) |  | 1998 |  | 2005 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Pass | Freight | Pass | Freight | Pass | Freight |
| Operating (signalling) and other expenses | 9 | 4 | 9 | 4 | 11 | 4 |
| Depreciation | 3 | 1 | 3 | 1 | 4 | 1 |
| Interest | 4 | 2 | 4 | 1 | 5 | 2 |
| Totals | 23 |  | 22 |  | 27 |  |

A small part of the above costs are allocated to urban rail infrastructure in Dublin.  In 1997 and 1998, the infrastructure costs for urban rail (Dublin Area Rapid Transit) were 11% and 15% of the totals above.

Table 37

Infrastructure costs for rail – per train-km, in €, 1998 prices

| 1996(1997) |  | 1998 |  | 2005 |  |
| --- | --- | --- | --- | --- | --- |
| Passenger | Freight | Passenger | Freight | Passenger | Freight |
| 1.50 | 1.44 | 1.42 | 1.45 | 1.63 | 2.06 |

The cost per train km is presented above.  Rates for passenger and freight are similar in 1996 and 1998.  It is difficult to forecast to 2005 but the method used for extrapolation is the use of linear trends both in terms of infrastructure costs and predicted train-km.  The latter was supplied by Irish Rail.

Asset value and net book amounts are presented here in million € for 1998

Table 38

Asset value and net book accounts of Irish railways (IR annual accounts)

|  | 1998 |
| --- | --- |
| Asset value of railway lines and works | 14.49 |
| Net book value of railway lines and works | 14.86 |

### Aviation

Aer Rianta is the airport authority which manages the three national airports – Dublin, Cork and Shannon.  There is insufficient disaggregation in the accounts to provide an adequate breakdown between infrastructure and operating costs.  Some reference to operating costs is mentioned and they are included in the Supplier Operating Accounts of this report.  At the end of 1998,  the tangible fixed assets, depreciation and net book value were as follows in million €.  Only terminal complexes, lands and airfields and plant and equipment are taken into account for the pilot accounts.

Table 39

Infrastructure costs for air 1998 million € (IR yearly accounts)

|  | Terminal Complexes | Lands and airfields | Plant & Equipment | Hotels | Other property | Total |
| --- | --- | --- | --- | --- | --- | --- |
| Cost | 148 | 118 | 59 | 54 | 84 | 464 |
| Depreciation | 27 | 18 | 31 | - | 17 | 93 |
| Net book value | 122 | 100 | 29 | 54 | 66 | 371 |

Similar data for 1996 are presented as follows in 1998 prices.

Table 40

Infrastructure costs for air 1996 million €, 1998 prices (IR yearly accounts)

|  | Terminal Complexes | Lands and airfields | Plant & Equipment | Hotels | Other property | Total |
| --- | --- | --- | --- | --- | --- | --- |
| Cost | 71.80 | 91.86 | 42.46 | 48.65 | 65.55 | 320.32 |
| Depreciation | 19.03 | 13.13 | 24.27 | 0.21 | 12.87 | 69.51 |
| Net book value | 47.33 | 76.34 | 17.27 | 45.01 | 49.04 | 234.99 |

### Maritime Shipping

The infrastructure costs of maritime shipping are given below.  They include infrastructure and some machinery in the form of RoRo ramps and cranes.

Table 41

Infrastructure costs for Dublin port for 1996 and 1998 in million €, 1998 prices

|  | 1996 | 1998 | 2000 |
| --- | --- | --- | --- |
| Capital expenditure | 15.24 | 14.16 | 11.12 |

## Supplier Operating Costs

### Rail

The tables below present supplier operating costs in rail transport in Ireland in 1996 (1997) and 1998 and a forecast for 2005 (total and average costs).

Table 42

Supplier Operating Costs of Rail 1998, 1996 (1997) and 2005, in million €, 1998 prices

|  |  | 1996(1997) |  |  | 1998 |  |  | 2005 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cost Category | Pass | Freight | Total | Pass | Freight | Total | Pass | Freight | Total |
| Materials | 53 | 20 | 74 | 50 | 19 | 69 | 55 | 21 | 75 |
| Personnel | 92 | 35 | 126 | 97 | 36 | 133 | 106 | 40 | 146 |
| Other running costs | 10 | 4 | 14 | 11 | 4 | 15 | 12 | 4 | 16 |
| Depreciation | 15 | 6 | 20 | 15 | 6 | 21 | 16 | 6 | 23 |
| Interest | 12 | 4 | 16 | 11 | 4 | 15 | 12 | 4 | 16 |
| Non-deductible VAT | 0 | 0 | 0 | 2 | 1 | 3 | 3 | 1 | 4 |
| Totals |  |  | 250 |  |  | 255 |  |  | 280 |

Table 43

Average Supplier Operating Costs of Rail 1998, 1996 and 2005, in €/train-km,

1998 prices

| 1996(1997) |  | 1998 |  | 2005 |  |
| --- | --- | --- | --- | --- | --- |
| Passenger | Freight | Passenger | Freight | Passenger | Freight |
| 16.44 | 15.80 | 16.39 | 16.70 | 15.51 | 23.92 |

Supplier operating costs for rail transport are similar in 1996 and 1998.  It is difficult to predict for 2005 but an increase of 10% from 1998 to 2005 was applied in any case.

### (Urban) Public Transport

Table 44 presents supplier operating costs for Dublin Bus public transport services in 1996, 1998 and a forecast for 2005.

Table 44

Supplier Operating Costs of Public Transport – Bus Atha Cliath (BAC) 1996, 1998 and 2005, in million €, 1998 prices

| Cost Category | 1996(1997) | 1998 | 2005 |
| --- | --- | --- | --- |
| Materials | 26 | 26 | 29 |
| Personnel | 85 | 85 | 94 |
| Other running costs | 14 | 14 | 15 |
| Depreciation | 14 | 13 | 15 |
| Interest | 0 | 0 | 0 |
| Non-deductible VAT | 1 | 0 | 0 |

Extrapolations for 2005 are done using a 10% increase between 1998 and 2005.  The average rate per vkm is presented in table 45.

Table 45

Average Supplier Operating Costs of Public Transport- Bus Atha Cliath (BAC) 1996, 1998 and 2005, in €/vkm, 1998 prices

|  | 1996(1997) | 1998 | 2005 |
| --- | --- | --- | --- |
| Cost per veh.km | 2.70 | 2.64 | 2.64 |

Supplier operating costs per vkm fall slightly between 1997 and 1998.  The extrapolation for 2005 is done using a 10% increase between 1998 and 2005.

### Regional Public Transport (Bus)

Table 46

Supplier Operating Costs of Public Transport – Bus Eireann (BE) 1996, 1998 and 2005, in million €, 1998 prices

| Cost Category | 1996(1997) | 1998 | 2005 |
| --- | --- | --- | --- |
| Materials | 31 | 29 | 41 |
| Personnel | 61 | 63 | 78 |
| School bus contractors | 26 | 26 | 35 |
| Other running costs | 10 | 9 | 14 |
| Depreciation | 10 | 9 | 9 |
| Non-deductible VAT | 0 | 2 | 0 |

The supplier operating costs for Bus Eireann (the national regional bus company) are presented above.  Extrapolations for 2005 were done using an increase between 1998 and 2005 of 10%.

### Aviation

It is difficult to differentiate between air related activity and retail activity in the accounts of Aer Rianta.  The operating costs for 1996 and 1998 are presented in 1998 prices in million € as follows:

Table 47

Average Supplier Operating Costs of Aer Rianta (covering Dublin, Cork and Shannon airports) 1996 and 1998, in Million €, 1998 prices

|  | 1996 | 1998 |
| --- | --- | --- |
| Cost of goods for resale | 105 | 108 |
| Payroll and related costs | 84 | 88 |
| Materials and services | 56 | 63 |
| Depreciation | 14 | 17 |

### Maritime Shipping

The supplier operating costs for Dublin Port are as follows:

Table 48

Operating costs for Dublin port for 1996 and 1998 in million €, 1998 prices

|  | 1996 | 1998 | 2000 |
| --- | --- | --- | --- |
| Operating Costs | 27.74 | 22.78 | 22.21 |

## Congestion Costs

### Roads

#### National road network

The following tables show the results for congestion costs in road, rail and air transport in Ireland for 1998.

Table 49

Road Congestion Costs 1998 in million €, 1998 prices

| Average trip length assumed (km) | Total Veh.kms /year (mill) | Delay (mill hrs) | Cost of delay (mill €) | Cost (€/1000km) | Fuel costs in congested conditions (€) | Fuel costs /1000km (€/1000km) | Avge user costs |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | €/1000km |
| 70 | 38150.00 | 8.55 | 115.45 | 3.03 | 10.42 | 0.27 | 3.30 |
| 40 | 38150.00 | 27.24 | 367.84 | 9.64 | 33.20 | 0.87 | 10.51 |

As can be seen from the table the calculation is sensitive to the average trip length used in the calculation.  No estimates are made for 1996 and 2005 given the many assumptions that had to be made for 1998 and the difficulties in making such calculations without more data.

#### Dublin

The relationships obtained from the SATURN network model for 1996 and 1998 for Dublin are as follows:

t=1.52+0.001e4.8x

where t = time taken to travel 1 km and x is the total distance travelled on the network in million pcu.km.

In terms of deriving the relationship above, the network matrix was factored before conducting a modelled run.  For example, a factor of 1 was used for current conditions, a factor of 0.5 was used to estimate delay if distance travelled on the network was reduced by half etc.

For 1996 the distance travelled on the network in an hour was 0.74 million passenger car unit kms and in 1998 this had risen to 0.78 million pcu.kms.  From the relationships above it can be found that at those respective levels of demand in 1996 and 1998 the time taken to travel 1 km is 1.55 and 1.61 minutes.

The relationship predicted for 2005 in Gibbons (1998) is:

2005:  t=1.418+0.075e1.183x

Delay is predicted by the Dublin Transportation Office to decrease by 2005 given the major improvements planned for the Dublin region.

If a comparison is made between the travel time with no traffic on the network and in conditions as they were in 1996, then the difference is 0.03 minutes to travel one km.  If one estimates the total travel time on the network due to the current condition demand level, assuming 15 hour days and 340 days in a year and if a similar breakdown of travel per travel purpose is made as above i.e. 0.2 for business, 0.6 for commuting and 0.2 for leisure travel then the total delay costs are 25.48 million euros and the rate per veh. km is 13.5 Euro/veh.km.  A similar estimate for 1998 results in a total delay time million pcu hours, the total cost of which is 80.58 million Euro.  This converts to a rate per veh.km of 13.5 Euro/veh.km.   It is not possible to calculate the additional fuel consumed due to congestion although estimates of the total fuel consumed are possible.

### Rail

The following data was supplied by Irish Rail on performance for the first six months of 2001.  Although the data is not sufficient to calculate congestion costs, they do supply some indication of the level of performance of the rail services in Ireland.

Table 50

Performance levels – rail punctuality (Irish Rail, 2001)

|  | % of trains which ran on time | Target (%) |
| --- | --- | --- |
| Interurban Routes |  |  |
| Westport | 84.7 | 90 |
| Galway | 96.8 | 90 |
| Limerick | 94.4 | 90 |
| Cork | 90.5 | 90 |
| Tralee | 91.3 | 90 |
| Waterford | 97.3 | 90 |
| Average | 91.8 |  |
| Urban |  |  |
| Dublin (DART) | 93 | 95 |
| Outer suburban | 90 | 90 |

There is a higher target for the Dublin Area Rapid Transit (DART) (a heavy urban rail corridor on east side of the city) of 05%.  The outer suburban services mentioned are other urban rail services offered in the Dublin region.

## Accident Costs

In the following tables the results of the detailed calculations of the social and the external accident costs are summarised.

Table 51 shows the total social accident costs for 1998, i.e. the sum of transport system internal and external accident costs for transport modes (road and rail).   In the case of roads, estimates of all costs are given in Bacon (1999).  In the case of rail, a value for preventing a fatality of 2.66 million euro was calculated by IRMS.  This was used with the equivalent fatality rates to estimate the accident cost for rail and is included in the WTP column below in the absence of detailed knowledge of the breakdown in the costs.

Table 51

Social costs of transport accidents, in million €, 1998 (Bacon, 1999)

|  | Output Loss | Human (WTP) | Medical | Damage to | Insurance | Professional | Police | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Costs | Expenses | property | Admin | Services | Costs |  |
| Fatality | 0 | 747 | 0 | 1 | 7 | 0 | 0 | 755 |
| Serious Injury | 0 | 406 | 0 | 6 | 43 | 0 | 0 | 455 |
| Slight Injury | 0 | 171 | 0 | 33 | 10 | 0 | 0 | 214 |
| Total | 0 | 1323 | 0 | 41 | 60 | 0 | 0 | 1425 |

The social accident costs amount to almost € 1.4 billion  per year.  Similar estimates for 1996 and 2005 can be made with total social accident costs of € 1.3 billion and € 1.7 billion respectively.

By far the largest part of the social accident costs "remain" within the transport system. Table 52 shows the external part of the social accident costs. As discussed in section 3.4 on methodological issues the difference between internal and external is large because the cost block "risk costs" is considered as transport system internal with the exception of uncovered transfer payments of the social security.   External accident costs in 1998 amounted to € 240 million and for 1996 and 2005 totals of € 226 and € 289 were found.

Table 52

Transport system external costs of transport accidents 1998, in million €

|  | Output Loss | Human (WTP) | Medical | Damage to | Insurance | Professional | Police | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Costs | Expenses | property | Admin | Services | Costs |  |
| Fatality | 44 | 0 | 3 | 0 | 0 | 6 | 1 | 53 |
| Serious Injury | 34 | 0 | 22 | 0 | 0 | 51 | 0 | 106 |
| Slight Injury | 21 | 0 | 10 | 0 | 0 | 50 | 0 | 81 |
| Total | 99 | 0 | 34 | 0 | 0 | 107 | 1 | 240 |

In section 3.4 it is explained why vehicle specific statements cannot be made for the external costs under the victims perspective chosen in UNITE because of the problem of arbitrary cost allocation. Therefore, the vehicle specific figures given in table 53 refer to the total social accident costs.

Table 53

Social accident costs 1998 – detailed results per mode, in million €

|  | Output | Human (WTP) | Medical | Damage to | Insurance | Professional | Police | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Loss | Costs | Expenses | property | Admin | Services | Costs |  |
| Road |  |  |  |  |  |  |  |  |
| Pedestrians | 18 | 265 | 5 | 5 | 9 | 15 | 0 | 317 |
| Cycle | 4 | 53 | 1 | 2 | 2 | 4 | 0 | 66 |
| Motorcycle | 9 | 117 | 3 | 4 | 6 | 10 | 0 | 149 |
| Car | 59 | 781 | 22 | 27 | 38 | 68 | 1 | 996 |
| PSV | 1 | 14 | 0 | 0 | 1 | 1 | 0 | 17 |
| GV | 6 | 73 | 2 | 3 | 4 | 7 | 0 | 94 |
| Other | 1 | 17 | 0 | 0 | 1 | 1 | 0 | 22 |
| Rail |  | 3 |  |  |  |  |  | 3 |
| Total | 99 | 1320 | 34 | 41 | 60 | 107 | 1 | 1662 |

Table 53 shows that relatively high social costs are associated with victims in the category of unprotected road users (i.e. pedestrians and cyclists) and in the "dangerous" category -  motorcycle. In absolute terms, the social costs in the passengers car category dominate.

For road transport it is possible to present the results for the different types of road infrastructure, using some general assumptions in Table 54.

Table 54

Social accident costs of road transport 1998 – detailed results per road type,

in million €

|  | Output | Human (WTP) | Medical | Damage to | Insurance | Professional | Police | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Loss | Costs | Expenses | property | Admin | Services | Costs |  |
| Pedestrian |  |  |  |  |  |  |  |  |
| Inter-urban | 5.29 | 84.47 | 0.78 | 0.70 | 1.64 | 2.15 | 0.08 | 95.12 |
| Urban | 13.05 | 181.46 | 4.26 | 4.34 | 7.74 | 12.53 | 0.19 | 223.58 |
| Pedal cyclists |  |  |  |  |  |  |  |  |
| Inter-urban | 0.83 | 12.32 | 0.18 | 0.26 | 0.31 | 0.58 | 0.01 | 14.50 |
| Urban | 3.13 | 41.28 | 1.02 | 1.63 | 1.68 | 3.53 | 0.05 | 52.30 |
| Motor cyclists |  |  |  |  |  |  |  |  |
| Inter-urban | 3.63 | 53.97 | 0.87 | 0.87 | 1.64 | 2.52 | 0.05 | 63.55 |
| Urban | 5.34 | 64.67 | 2.54 | 2.75 | 4.42 | 7.67 | 0.08 | 87.46 |
| Car users |  |  |  |  |  |  |  |  |
| Inter-urban | 41.62 | 568.90 | 13.50 | 16.51 | 23.74 | 42.18 | 0.62 | 707.07 |
| Urban | 19.03 | 225.37 | 8.95 | 11.37 | 15.07 | 28.55 | 0.29 | 308.63 |
| PSV users |  |  |  |  |  |  |  |  |
| Inter-urban | 0.91 | 13.08 | 0.22 | 0.32 | 0.38 | 0.72 | 0.01 | 15.64 |
| Urban | 0.24 | 2.36 | 0.13 | 0.22 | 0.20 | 0.46 | 0.00 | 3.61 |
| GV |  |  |  |  |  |  |  |  |
| Inter-urban | 4.15 | 54.52 | 1.57 | 1.81 | 2.75 | 4.82 | 0.06 | 69.68 |
| Urban | 1.62 | 19.66 | 0.74 | 0.87 | 1.28 | 2.31 | 0.02 | 26.50 |
| Other |  |  |  |  |  |  |  |  |
| Inter-urban | 1.03 | 14.14 | 0.36 | 0.36 | 0.65 | 1.04 | 0.01 | 17.59 |
| Urban | 0.33 | 4.15 | 0.14 | 0.15 | 0.25 | 0.43 | 0.00 | 5.45 |

There is only a small proportion of Irish roads in the motorway category so the accident costs are presented in the table above in terms of inter-urban/rural roads and urban roads based on the accident statistics presented in NRA RAF (1998).   There may be some small discrepancies between the total generated and those above due to rounding errors and also the fact that when the NRA presents inter-urban and urban statistics, accident cases where the speed limit is not known are omitted. The costs of accidents in urban areas is 42% of total costs.

From the figures presented above cost rates per unit of performance can be derived. With the victims perspective of UNITE ("monitoring") only the rate for the social accident costs can be calculated unless arbitrary cost allocation is accepted (see the discussion in section 3.4). The social accident cost rates using the UNITE perspective are given in table 55.

Table 55

Average social costs per unit of performance in 1998, in million €, 1998 prices

| Road |  |  |  | Total Costs |  | Cost/veh.km |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | (million Euro) |  |  |
| Motorcycles | Inter-urban |  | 64 |  | 0.21 |  |
|  | Urban |  | 87 |  | 0.58 |  |
| Cars | Inter-urban |  | 707 |  | 0.03 |  |
|  | Urban |  | 309 |  | 0.06 |  |
| Buses & Coaches | Inter-urban |  | 16 |  | 0.04 |  |
|  | Urban |  | 4 |  | 0.04 |  |
| Goods vehicles | Inter-urban |  | 70 |  | 0.01 |  |
|  | Urban |  | 26 |  | 0.02 |  |

## Environmental costs

### Air Pollution

The costs of air pollution for 1996 for road transport are presented in the next table.  The costs were computed by Bickel (2001).  The total costs in 1996 amount to € 446 million, see table 56.  A similar set of estimates is presented for 1998.   The total costs in 1998 were found to be € 477 million, see table 57.  This is 85% of the total costs associated with road, rail and air emissions.

Table 56

Cost of air pollution from roads sector for Ireland for 1996 in million €, in 1998 prices

|  | PM 2.5 | SO2 | NOx | NMVOC | CH4 | CO | CO2 | N2O | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Passenger Cars | 36.9 | 6.9 | 99.9 | 38.5 | 0.8 | 0.1 | 93.3 | 2.8 | 279.2 |
| Gasoline | 0.0 | 4.3 | 90.5 | 37.9 | 0.8 | 0.1 | 72.0 | 2.4 | 208.0 |
| Diesel | 36.9 | 2.6 | 9.2 | 0.6 | 0.0 | 0.0 | 21.1 | 0.4 | 70.8 |
| LPG | 0.0 | 0.0 | 0.2 | 0.0 | 0.0 | 0.0 | 0.2 | 0.0 | 0.4 |
| Light Duty Vehicles | 43.8 | 3.2 | 13.7 | 1.6 | 0.0 | 0.0 | 26.9 | 0.5 | 89.7 |
| Gasoline <3.5t | 0.0 | 0.1 | 1.1 | 0.6 | 0.0 | 0.0 | 1.3 | 0.0 | 3.1 |
| Diesel <3.5t | 43.8 | 3.1 | 12.6 | 1.0 | 0.0 | 0.0 | 25.6 | 0.5 | 86.6 |
| Heavy Duty Vehicles | 14.7 | 1.6 | 15.0 | 0.9 | 0.0 | 0.0 | 13.2 | 0.2 | 45.8 |
| Gasoline >3.5t | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 | 0.2 |
| Diesel | 14.7 | 1.6 | 14.9 | 0.9 | 0.0 | 0.0 | 13.2 | 0.2 | 45.5 |
| Buses | 7.1 | 0.8 | 13.7 | 0.3 | 0.0 | 0.0 | 7.0 | 0.1 | 29.0 |
| Urban Busses | 4.0 | 0.5 | 7.9 | 0.2 | 0.0 | 0.0 | 3.7 | 0.0 | 16.3 |
| Coaches | 3.2 | 0.4 | 5.7 | 0.2 | 0.0 | 0.0 | 3.2 | 0.0 | 12.8 |
| Mopeds & Motorcycles | 0.0 | 0.1 | 0.2 | 1.2 | 0.0 | 0.0 | 0.9 | 0.0 | 2.4 |
| Mopeds | 0.0 | 0.0 | 0.0 | 0.6 | 0.0 | 0.0 | 0.2 | 0.0 | 0.8 |
| Motorcycles | 0.0 | 0.0 | 0.2 | 0.6 | 0.0 | 0.0 | 0.7 | 0.0 | 1.6 |
| Grand Totals: | 102.6 | 12.6 | 142.5 | 42.5 | 0.9 | 0.1 | 141.2 | 3.6 | 446.0 |

Table 57

Cost of air pollution from roads sector for Ireland for 1998 in million €

|  | PM 2.5 | SO2 | NOx | NMVOC | CH4 | CO | CO2 | N2O | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Passenger Cars | 21.8 | 6.7 | 90.1 | 36.2 | 0.8 | 0.0 | 95.3 | 4.1 | 255.0 |
| Gasoline | 0.0 | 4.8 | 83.3 | 35.8 | 0.8 | 0.0 | 80.4 | 3.8 | 208.9 |
| Diesel | 21.8 | 1.8 | 6.6 | 0.3 | 0.0 | 0.0 | 14.8 | 0.3 | 45.7 |
| LPG | 0.0 | 0.0 | 0.2 | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 | 0.4 |
| Light Duty Vehicles | 59.4 | 5.1 | 18.4 | 2.3 | 0.0 | 0.0 | 42.6 | 0.8 | 128.5 |
| Gasoline <3.5t | 0.0 | 0.1 | 1.5 | 0.8 | 0.0 | 0.0 | 1.7 | 0.0 | 4.2 |
| Diesel <3.5t | 59.4 | 5.0 | 16.9 | 1.4 | 0.0 | 0.0 | 40.8 | 0.7 | 124.3 |
| Heavy Duty Vehicles | 19.2 | 2.3 | 20.0 | 1.1 | 0.0 | 0.0 | 18.5 | 0.3 | 61.4 |
| Gasoline >3.5t | 0.0 | 0.0 | 0.2 | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 | 0.3 |
| Diesel | 19.2 | 2.2 | 19.9 | 1.1 | 0.0 | 0.0 | 18.4 | 0.3 | 61.1 |
| Buses | 7.1 | 1.0 | 13.5 | 0.3 | 0.0 | 0.0 | 8.4 | 0.1 | 30.5 |
| Urban Busses | 3.8 | 0.5 | 7.8 | 0.2 | 0.0 | 0.0 | 4.3 | 0.0 | 16.6 |
| Coaches | 3.3 | 0.5 | 5.7 | 0.2 | 0.0 | 0.0 | 4.1 | 0.0 | 13.9 |
| Mopeds & Motorcycles | 0.0 | 0.1 | 0.2 | 1.0 | 0.0 | 0.0 | 0.9 | 0.0 | 2.2 |
| Mopeds | 0.0 | 0.0 | 0.0 | 0.6 | 0.0 | 0.0 | 0.2 | 0.0 | 0.7 |
| Motorcycles | 0.0 | 0.0 | 0.2 | 0.4 | 0.0 | 0.0 | 0.7 | 0.0 | 1.4 |
| Grand Totals: | 107.5 | 15.1 | 142.2 | 40.9 | 0.9 | 0.1 | 165.7 | 5.3 | 477.6 |

The total and average costs for emissions from road transport are given in table 58.  Average costs for most vehicle types are similar although there is a noticeable decrease for the cost per veh.km for buses.

Table 58

Total and average air pollution costs per km from road sector

|  | 1996 |  | 1998 |  |
| --- | --- | --- | --- | --- |
|  | Total | Average Cost | Total | Average Cost |
|  | Million € | €/km | Million € | €/km |
| Passenger cars | 279 | 0.012 | 255 | 0.009 |
| Light and Heavy Duty Vehicles | 135 | 0.024 | 190 | 0.019 |
| Buses | 29 | 0.095 | 30 | 0.067 |
| Mopeds and Motorcycles | 2 | 0.01 | 2 | 0.005 |
| Total | 446 | 0.015 | 477 | 0.012 |

The air pollution costs for emissions associated with rail transport are presented below for 1996.  Given that there was only a marginal increase in the number of train kms between 1996 and 1998 and that estimates rather than measurements are being used in the calculations, the costs associated with rail transport in 1996 are assumed to be similar for 1998.  In table 59 the costs associated with diesel trains are presented but this is likely to be an underestimate because the costs associated with fuel production are not included.  Table 60 presents the costs associated with electric traction.  Again the pollution costs associated with production of the electricity used are not included and therefore the estimates are likely to be lower than actual.

Table 59

Cost of emissions from rail (diesel traction) for 1996 in million € in 1998 prices

Table 60

Cost of emissions from rail (electric traction) for 1996 in million € in 1998 prices

|  | Air pollution | (excl. GHG) | GHG | (direct) | Air pollution + CO2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1996 | Total | Average | Total | Average | Total | Average |
|  | [million €] | [€ / Train-km] | [million €] | [€ / Train-km] | [million €] | [€ / Train-km] |
| Passenger Trains | 0.24 | 0.12 | 0.29 | 0.15 | 0.53 | 0.27 |
| Freight Trains | - | - | - | - | - | - |
| Total |  |  |  |  |  |  |

The cost of air emissions from the aviation sector is presented in table 61.  No estimates could be obtained for PM10 and therefore the estimates presented in the table are an underestimation of possible costs.  .

Table 61

Cost of air emissions from aviation sector for 1996 in million €, in 1998 prices

| 1996 | Total (excl. | fuel prod. | PM10 | NOx | SO2 | NMVOC | CO2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | direct CO2) |  | tonnes | tonnes | tonnes | tonnes | tonnes |
| Airports (LTO-cycle) | 9.6 | 2.7 | no data | 4.7 | 0.4 | 1.8 | 12.5 |
| Flights |  | 8.9 |  |  |  |  | 38.8 |
| Total | 18.5 | 11.6 | 0.0 | 4.7 | 0.4 | 1.8 | 53.7 |

### Global Warming

Global warming due to road transport was € 165 million compared with € 2 million for rail.

### Noise

The results for costs of noise exposure from using the ExternE method using the exposure rates suggested by Tinch (1997) are shown in the following table.  No estimates were available for air exposure and so the table concentrates on road and rail only.

Table 62

Noise costs of road and rail for 1996 in million € for road and rail, in 1998 prices.

|  | 1996 |  | 1998 |  | 2005 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Road | Rail | Road | Rail | Road | Rail |
| Myocardial infarction | 9 | 1 | 9 | 1 | 10 | 1 |
| Angina pectoris | 0 | 0 | 0 | 0 | 0 | 0 |
| Hypertension | 0 | 0 | 0 | 0 | 0 | 0 |
| Sleep disturbance | 18 | 2 | 19 | 2 | 20 | 2 |
| Amenity losses | 305 | 24 | 324 | 26 | 343 | 27 |
| Total | 332 | 27 | 352 | 29 | 373 | 31 |

## Taxes, Charges and Subsidies

### Road

Sources of revenue from road transport are presented in table 63.

Table 63

Different sources of the revenues from road transport 1998, in million €, 1998 prices.

| Different sources of revenues | 1996 (1997) | 1998 (1999) | 2005 |
| --- | --- | --- | --- |
| Fuel tax (leaded petrol) | 235 | - | - |
| Fuel tax (unleaded petrol) | 592 | 643 | 707 |
| Fuel tax (diesel)* | 456 | 580 | 638 |
| Fuel tax (LPG) | - | 0 | 0 |
| Annual vehicle tax | 371 | 373 | 410 |
| Vehicle registration tax | 816 | 770 | 847 |
| Total | 2470 | 2366 | 2603 |

In terms of the UNITE accounting methods these revenues are considered as other revenues i.e. they go to the general budget and are not retained within the transport sector.

Fuel price structures in € per 1000 litres in 1998 (European Environment Agency) were as follows:

Table 64

Fuel price structure in Ireland

|  | Leaded petrol |  |  | Unleaded petrol |  |  | Diesel |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Price excl taxes | Tax | Sales Price | Price excl taxes | Tax | Sales Price | Price excl taxes | Tax | Sales Price |
| 264.2 | 617.8 | 882 | 243.6 | 510.6 | 754.2 | 212.1 | 505.4 | 717.5 |

VAT on fuels is charged at the standard 21% rate except for kerosene which has a VAT rate of 12.5%. (European Commission Eco-tax database).

Two bridges in the Dublin area are currently tolled.  The concession is held by National Toll Roads under a Public Private Partnership.  The revenues from the tolling operations in 1996 were 20.99 million € in 1997 and 26.69 million € in 1998.  Some of this revenue is used in operating the toll plazas but more is likely not to be allocated to transport activities as the company has activities in other areas.

### Rail

The revenues and state grants for mainline rail (interregional services) are presented in table 65.  The growth assumption used for 2005 was a 10% increase from 1998 to 2005.

Table 65

Supplier revenues and subsidies of mainline rail transport 1998, 1996 and 2005, in million €, 1998 prices (IR Annual Accounts)

|  |  | 1997 |  |  | 1998 |  |  | 2005 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cost category | pass | freight | total | pass | freight | total | pass | freight | total |
| Revenues | 80 | 23 | 103 | 85 | 21 | 107 | 94 | 23 | 117 |
| Subsidies | - | - | 43 | - | - | 38 | - | - | 42 |

The revenues for the urban rail service in Dublin are presented in table 66.  A 3% growth rate per year was used to make predictions for 2005.

Table 66

Supplier revenues and subsidies of urban rail transport in Dublin 1998, 1996 and 2005, in million €, 1998 prices (IR Annual Accounts)

|  | 1996(1997) | 1998 | 2005 |
| --- | --- | --- | --- |
| Revenues | 19 | 20 | 25 |
| Subsidies | 8 | 4 | 5 |

The revenues and subsidies for interregional bus transport in Ireland are presented below (BE annual accounts).

Table 67

Supplier revenues and subsidies of interregional bus transport in Ireland 1998, 1996 and 2005, in million €, 1998 prices (BE Annual Accounts)

|  | 1996(1997) | 1998 | 2005 |
| --- | --- | --- | --- |
| Revenues | 134 | 134 | 165 |
| Subsidies | 7 | 7 | 9 |

A 3% growth rate per year was applied for the years between 1998 and 2005.

### (Urban) Bus Transport

Table 68

Supplier revenues and subsidies of urban bus transport in Dublin 1998, 1996 and 2005, in million €, 1998 prices (BAC Annual Accounts)

|  | 1996(1997) | 1998 | 2005 |
| --- | --- | --- | --- |
| Revenues | 134 | 134 | 165 |
| Subsidies | 9 | 11 | 14 |

Table  68 above gives revenues and subsidies for bus transport in Dublin.  A rate of 3% growth per year was used for the years between 1998 and 2005.   It is not clear what conditions will be imposed on this sector regarding competition by 2005 but competition is likely.

### Aviation

The revenue from aeronautical activities and commercial activities are presented in Table 69.

Table 69

Aviation revenues for the Irish national airports, in million €, 1998 prices. (Aer Rianta annual accounts)

|  | 1996 (1997) | 1998 | 2005 |
| --- | --- | --- | --- |
| Aeronautical revenue | 134 | 134 | 165 |
| Commercial activities | 9 | 11 | - |

### Shipping

Accounts were found only for one large Irish shipping operator – Irish Ferries.  Their turnover for their Irish operations was 75 million €, in 1999 was 85.9 million € and in 2000 was 98.7 million €.  Stenaline does not publish its accounts at the moment.

5.	Summary Tables

5.1	Road Transport

Table 70

Irish road account for 1996, 1998 and 2005 
- € million at 1998 prices -

| Costs |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Core information | 1996 |  | 1998 |  | 2005 |  |
| Infrastructure Costs | 262 |  | 263 |  | 264 |  |
| Fixed |  |  |  |  |  |  |
| Variable |  |  |  |  |  |  |
| Accident costs (user external)1) | 226 |  | 240 |  | 289 |  |
| Environmental costs | 778 |  | 829 |  | 879 |  |
| Air pollution | 305 |  | 312 |  | 331 |  |
| Global warming | 141 |  | 165 |  | 175 |  |
| Noise | 3323) |  | 3523 |  | 373 |  |
| Total | 1266 |  | 1332 |  | 1432 |  |
| Additional information |  |  |  |  |  |  |
| Congestion costs2) | 401 |  | 401 |  | 463 |  |
| Time costs | 367 |  | 367 |  | 425 |  |
| Fuel costs | 344) |  | 344) |  | 38 |  |
| Accident costs (user internal)5) | 1342 |  | 1425 |  | 1714 |  |
| From this: risk value | 12479) |  | 1323 |  | 1592 |  |
| Environmental costs |  |  |  |  |  |  |
| Nature and landscape, soil and water pollution |  |  |  |  |  |  |
| Nuclear risk | 0 |  | 0 |  | 0 |  |
| Revenues |  |  |  |  |  |  |
| Directly related to a specific cost category |  |  |  |  |  |  |
| Charges for infrastructure usage |  |  |  |  |  |  |
| Fixed | 216) |  | 27 |  | 407) |  |
| Variable | 0 |  | 0 |  | 0 |  |
| Total | 21 |  | 27 |  | 408) |  |
| Other transport specific revenues |  |  |  |  |  |  |
| Annual circulation tax | 371 |  | 373 |  | 410 |  |
| Fuel tax | 1 283 |  | 1 223 |  | 1346 |  |
| Eco tax1) | 0 |  | 0 |  |  |  |
| VAT2) | 816 |  | 770 |  | 847 |  |
| Total | 2 470 |  | 2 366 |  | 2603 |  |
| Subsidies | 0 |  | 0 |  | 0 |  |
| 1) Refers to those parts of road accident costs which are not borne by road users and insurance companies but by the public sector and third parties. 2) Expressed as delay costs 3) Limited data available for noise 4) Estimates of delay were difficult due to lack of data, same value used for 1996 and 1998. 3% / year increase used for 2005. 5) Refers to those parts of accident costs which are caused by and borne by road users and insurance companies 6) Refers to revenue generated by 2 toll bridges in Dublin, the value inserted for 1996 is actually 1997 revenue. 7) 8 stretches of main road will be tolled in the future but estimates are not available for revenue for 2005. The charges will go towards recouping investment by private investors under public private partnership arrangements 8) Rough estimate 9) Bacon (1999) Sources: NRA, CSO. |  |  |  |  |  |  |

Table 71

Average variable costs of road transport per vehicle km: Ireland
- €/km at 1998 prices –

| All Roads |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1998 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Motor-cycles |  |  | Passenger cars |  |  | Buses |  |  |  | LGV & HGV |  |
| Core information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Infrastructure costs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fixed | : |  |  |  | : |  |  |  |  | : | : |  |  |  |
| Variable | : |  |  |  | : |  |  |  |  | : | : |  |  |  |
| External accident costs1) |  | 0.04 |  |  |  |  |  |  |  |  |  |  |  |  |
| Environmental costs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Air pollution | 0.0029 |  |  | 0.0059 |  |  | 0.0486 |  |  |  | 0.0128 |  |  |  |
| Global warming | 0.0019 |  |  | 0.0035 |  |  | 0.0185 |  |  |  | 0.0061 |  |  |  |
| Noise |  |  |  | 0.009 |  |  |  |  |  |  |  |  |  |  |
| Total I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Additional information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Delay costs |  | 0.01 |  |  |  |  |  |  |  |  |  |  |  |  |
| Internal accident costs2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Material damages |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Risk value |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Environmental costs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Nature, landscape, soil and water pollution |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total II |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Revenues |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fixed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Vignette |  | : |  |  |  | : |  | : |  |  |  | : |  | : |
| Annual circulation tax |  | : |  |  |  | : |  | : |  |  |  | : |  | : |
| Variable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fuel tax |  | : |  |  |  | : |  | : |  |  |  | : |  | : |
| Eco tax |  | • |  |  |  | • |  | • |  |  |  | • |  | • |
| Distance related infrastructure charges |  | : |  |  |  | : |  | : |  |  |  | : |  | : |
| VAT |  | : |  |  |  | : |  | : |  |  |  | : |  | : |
| Basic data |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Million vehicle km |  | 450 |  |  |  | 27174 |  | 454 |  |  |  |  |  | 10071 |
| Million passenger km |  |  |  |  |  |  |  |  |  |  |  | • |  |  |
| Million tonne km |  | • |  |  |  | • |  | • |  |  |  |  |  |  |
| 1) Both external and internal accident costs. – 2) Figures are included in item "External accident costs" of the core information section. – Source: O'Mahony et al, 2001. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 72

Total costs of road transport: Ireland
- € million at 1998 prices -

| All Roads |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1998 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Motor-cycles |  |  | Passenger cars |  |  | Buses |  |  | LGV2) |  |  |  | HGV |  | Total |  |  |
| Core information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Infrastructure costs |  |  | 3 |  |  | 187 |  |  | 3 |  |  | 69 |  |  |  |  |  | 263 |
| Fixed |  |  | : |  |  | : |  |  | : |  |  | : |  |  |  |  |  |  |
| Variable |  |  | : |  |  | : |  |  | : |  |  | : |  |  |  |  |  |  |
| External accident costs |  |  | 3 |  |  | 171 |  |  | 3 |  |  | 63 |  |  |  |  |  | 240 |
| Administrative |  |  | 1 |  |  | 77 |  |  | 1 |  |  | 28 |  |  |  |  |  | 107 |
| Health costs |  |  | 0.5 |  |  | 24 |  |  | 0.5 |  |  | 9 |  |  |  |  |  | 34 |
| Production loss |  |  | 1 |  |  | 70 |  |  | 1 |  |  | 26 |  |  |  |  |  | 98 |
| Environmental costs |  |  | 90 |  |  | 343 |  |  | 118 |  |  | 278 |  |  |  |  |  | 829 |
| Air pollution |  |  | 1 |  |  | 160 |  |  | 22 |  |  | 129 |  |  |  |  |  | 312 |
| Global warming |  |  | 1 |  |  | 95 |  |  | 8 |  |  | 61 |  |  |  |  |  | 165 |
| Noise |  |  | 3521) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total I |  |  | 701 |  |  |  |  |  | 124 |  |  | 410 |  |  |  |  |  | 1332 |
| Additional information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Delay costs3) |  |  | 0 |  |  | 289 |  |  | 5 |  |  | 107 |  |  |  |  |  | 401 |
| Internal accident costs |  |  | 17 |  |  | 1014 |  |  | 17 |  |  | 376 |  |  |  |  |  | 1424 |
| Material damages |  |  | 1 |  |  | 72 |  |  | 1 |  |  | 27 |  |  |  |  |  | 101 |
| Risk value |  |  | 16 |  |  | 942 |  |  | 16 |  |  | 349 |  |  |  |  |  | 1323 |
| Environmental |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Nature, landscape, soil and water pollution |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total II |  |  | 13204) |  |  |  |  |  | 22 |  |  | 483 |  |  |  |  |  | 1825 |
| Revenues |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fixed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Vignette |  |  | • |  |  | • |  |  | • |  |  | • |  |  |  |  |  |  |
| Annual circulation tax |  |  | : |  |  | : |  |  | : |  |  | : | : |  |  |  |  | 373 |
| Vehicle registration tax |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 770 |
| Variable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fuel tax |  |  | : |  |  | : |  |  | : |  |  | : | : |  |  |  |  | 1223 |
| Eco tax |  |  | • |  |  | • |  |  | • |  |  | • | • |  |  |  |  | • |
| Distance related infrastructure charges |  |  | • |  |  | • |  |  | • |  |  | • | • |  |  |  |  | • |
| VAT |  |  | : |  |  | : |  |  | : |  |  | : | : |  |  |  |  |  |
| Total |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2366 |
| Basic data |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Number of vehicles (thousand) |  |  | 24 |  |  | 1197 |  |  | 18 |  |  | 2715) |  |  |  |  |  | 1510 |
| Million vehicle km |  |  | 451 |  |  | 27174 |  |  | 454 |  |  | 10071 |  |  |  |  |  | 38150 |
| Million passenger km |  |  |  |  |  |  |  |  |  |  |  | • | • |  |  |  |  |  |
| Million tonne km |  |  | • |  |  | • |  |  | • |  |  |  |  |  |  |  |  |  |
| 1) noise is divided equally between cars, motorcycles, LGVs+HGVs and buses for compiling totals 2) LGV and HGV where more disaggregated data not available 3) in the absence of more detailed aggregation, total costs are proportioned according to mileage per vehicle type but in the case of delay, motorcycles are assumed not to be delayed 4) Motor cycle and passenger cars. 5) includes other vehs in addition to LGV and HGV such as dumpers, mobile machines, forklifts etc Source: O'Mahony et al., 2001 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

5.2	Rail Transport

Table 73

Irish rail account for 1996, 1998 and 2005 
– € million at 1998 prices –

| Costs |  |  |  |
| --- | --- | --- | --- |
| Core information | 1996 | 1998 | 2005 |
| Infrastructure Costs3) | 23 | 22 | 27 |
| Fixed | : | : | : |
| Variable | : | : | : |
| Supplier operating costs | 250 | 255 | 280 |
| Accident costs (external)1) |  |  |  |
| Environmental costs | 37 | 39 | 44 |
| Air pollution | 8 | 8 | 9 |
| Global warming | 2 | 2 | 3 |
| Noise | 27 | 29 | 32 |
| Total core social costs | 310 | 316 | 351 |
| Additional information |  |  |  |
| Congestion costs1) |  |  |  |
| Accident costs (internal) | : | : | : |
| From this: risk value2) | 3 | 3 | 4 |
| Environmental costs |  |  |  |
| Nature and landscape, soil and water pollution |  |  |  |
| Nuclear risk |  |  |  |
| Revenues | 121 | 127 | 142 |
| Directly related to Supplier Operating Costs |  |  |  |
| Subsidies | 51 | 42 | : |
| User Tariffs |  |  | : |
| Additional Information |  |  |  |
| Revenues directly related to infrastructure costs |  |  |  |
| Track charges |  |  |  |
| Fixed |  |  |  |
| Variable |  |  |  |
| Station charges |  |  |  |
| Other transport specific revenues |  |  |  |
| Fuel tax |  |  |  |
| Eco tax9) |  |  |  |
| VAT10) |  |  |  |
| Non-transport related revenues of rail companies | : | : | : |
| 1) Insufficient data, 2) IRMS (1998), 3) Operating, signalling and depreciation, Sources: Iarnrod Eireann. |  |  |  |

Table 74

Average variable costs of rail transport per vehicle km: Irish Rail
€/train km at 1998 prices

| Irish Rail (IR) |  |  |
| --- | --- | --- |
|  | 1998 |  |
|  | Passenger | Freight |
| Core information |  |  |
| Infrastructure costs | 1.4 | 1.4 |
| Fixed | : | : |
| Variable | : | : |
| External accident costs |  |  |
| Administrative |  |  |
| Health costs |  |  |
| Production loss |  |  |
| Environmental costs |  |  |
| Air pollution | 0.6 | 0.6 |
| Global warming | 0.2 | 0.2 |
| Noise | 1 | 1 |
| Total I | 1.8 | 1.8 |
| Additional Information |  |  |
| Delay costs |  |  |
| Internal accident costs |  |  |
| Material damages |  |  |
| Risk value |  |  |
| Environmental costs |  |  |
| Nature, landscape, soil and water pollution |  |  |
| Nuclear risk |  |  |
| Total II |  |  |
| Revenues |  |  |
| User tariffs | : | : |
| Track charges | : | : |
| Station charges | : | : |
| Fuel tax | : | : |
| Eco tax | • | • |
| VAT | : | : |
| Subsidies | : | : |
| Basic data |  |  |
| Veh. km (mill) | 11.3 | 4.2 |
| Source: O'Mahony et al., 2001. |  |  |

Table 75

Total costs of rail transport: Irish Rail (IR)
- € million at 1998 prices –

| Irish Rail (IR) |  |  |  |
| --- | --- | --- | --- |
|  | 1998 |  |  |
|  | Passenger | Freight | Total |
| Core information |  |  |  |
| Infrastructure costs | 16 | 6 | 22 |
| Tracks |  |  |  |
| Fixed | : | : | : |
| Variable | : | : | : |
| Stations |  |  |  |
| Fixed | : | : | : |
| Variable | : | : | : |
| Supplier operating costs | : | : | 255 |
| Out of these: track + station charges | : | : |  |
| External accident costs1) |  |  |  |
| Administrative |  |  |  |
| Health costs |  |  |  |
| Production loss |  |  |  |
| Environmental costs1) | 27 | 12 | 39 |
| Air pollution | 5 | 3 | 8 |
| Global warming | 2 | 2 | 2 |
| Noise | 20 | 9 | 29 |
| Total I |  |  | 316 |
| Additional information |  |  |  |
| Delay costs |  |  |  |
| Internal accident costs |  |  |  |
| Material damages | : | : | : |
| Risk value | : | : | 2.66 |
| Environmental costs |  |  |  |
| Nature, landscape, soil and water pollution | : | : |  |
| Nuclear risk |  |  |  |
| Total II |  |  | 2.66 |
| Revenues |  |  |  |
| User tariffs |  |  |  |
| Track charges |  |  |  |
| Station charges |  |  |  |
| Fuel tax |  |  |  |
| Eco tax |  |  |  |
| VAT |  |  |  |
| Total |  |  | 127 |
| Subsidies |  |  | 42 |
| Basic data |  |  |  |
| Passenger km (mill) | 1 |  |  |
| Tonne km (bill) |  | 0.5 |  |
| Source: O'Mahony et al., 2001. |  |  |  |

5.3	Aviation

Table 76
Irish air transport account for 1996, 1998 and 2005 
- € million at 1998 prices –

| Costs |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Core information | 1996 |  | 1998 |  | 2005 |  |
| Infrastructure Costs1) | 262 |  | 401 |  | 493 |  |
| Fixed | : |  | : |  | : |  |
| Variable | : |  | : |  | : |  |
| Accident costs (external) |  |  |  |  |  |  |
| Environmental costs | 73 |  | 77 |  | 95 |  |
| Air pollution | 19 |  | 20 |  | 25 |  |
| Global warming | 54 |  | 57 |  | 70 |  |
| Noise2) |  |  |  |  |  |  |
| Total | 335 |  | 478 |  | 588 |  |
| Additional information |  |  |  |  |  |  |
| Congestion costs2) |  |  |  |  |  |  |
| Accident costs (internal) | : |  | : |  | : |  |
| From this: risk value |  |  |  |  |  |  |
| Environmental costs |  |  |  |  |  |  |
| Nature and landscape, soil and water pollution |  |  |  |  |  |  |
| Nuclear risk | : |  | : |  | : |  |
| Revenues |  |  |  |  |  |  |
| Directly related to a specific cost category |  |  |  |  |  |  |
| Charges for infrastructure usage |  |  |  |  |  |  |
| Aeronautical revenue | 134 |  | 134 |  | 165 |  |
| Commercial activities | 9 |  | 11 |  |  |  |
| Total | 143 |  | 145 |  | 165 |  |
| Other transport specific revenues |  |  |  |  |  |  |
| Fuel tax |  |  |  |  | : |  |
| Eco tax3) |  |  |  |  | : |  |
| VAT4) |  |  |  |  | : |  |
| Subsidies6) | : |  | : |  | : |  |
| Non-transport related revenues of airports | : |  | : |  | : |  |
| 1) Includes terminal complexes, lands and airfields and plant and equipment 2) Insufficient data Source: Aer Rianta |  |  |  |  |  |  |

Table 77

Average variable costs of Aviation per vehicle km or movement: 
Ireland – €/km at 1998 prices –

|  | 1998 |  |
| --- | --- | --- |
|  | Passenger | Cargo |
| Core information |  |  |
| Infrastructure costs | : | : |
| Fixed | : | : |
| Variable | : | : |
| External accident costs2) |  |  |
| Administrative |  |  |
| Health costs |  |  |
| Production loss |  |  |
| Environmental costs1) |  |  |
| Air pollution | 18.4 |  |
| Global warming | 54.6 |  |
| Noise |  |  |
| Total I |  |  |
| Additional information |  |  |
| Delay costs: per arriving flight |  |  |
| Internal accident costs |  |  |
| Material damages |  |  |
| Risk value |  |  |
| Environmental costs |  |  |
| Nature, landscape, soil and water pollution |  |  |
| Nuclear risk | • | • |
| Total II |  |  |
| Revenues |  |  |
| Charges for infrastructure usage |  |  |
| Airport revenues | : | : |
| ATM charges | : | : |
| Meteorological services | : | : |
| Fuel tax |  |  |
| Eco tax4) |  |  |
| VAT |  |  |
| Subsidies |  |  |
| Exemption for kerosene tax | : | : |
| Exemption of VAT on ticket price | : | : |
| Basic data |  |  |
| Passenger km (bill)1) | 3.9 | • |
| Tonne km (bill) | • |  |
| 1) per movement, (all movements) Source: O'Mahony et al., 2001. |  |  |

References

Aer Rianta (1996)  Annual Accounts for 1996, Aer Rianta, Dublin Airport, Dublin, Ireland

Aer Rianta (1998)  Annual Accounts for 1998, Aer Rianta, Dublin Airport, Dublin, Ireland

Aer Rianta (2000)  Annual Accounts for 2000, Aer Rianta, Dublin Airport, Dublin, Ireland

Bacon, P. (1999).  An economic assessment and preliminary cost benefit analysis of the Government Strategy for Road Safety 1998-2002, National Safety Council, Dublin, Ireland.

Bickel P, Schmid S, Friedrich R, Maibach M, Gehrig S, Doll C, Tervonen J, Enei R (2000): Accounts Approach for Environmental Costs, UNITE (UNIfication of accounts and marginal costs for Transport Efficiency) Working Funded by 5th Framework RTD Programme, Interim Report 9.2. Version 0.6, ITS, University of Leeds, Leeds.

Bus Atha Cliath (1998)  Annual Accounts for 1998, Bus Atha Cliath, Dublin, Ireland.

Bus Atha Cliath (2000)  Annual Accounts for 2000, Bus Atha Cliath, Dublin, Ireland.

Bus Eireann (1998)  Annual accounts for 1998, Bus Eireann, Dublin, Ireland

Bus Eireann (2000)  Annual accounts for 2000, Bus Eireann, Dublin, Ireland

Central Statistics Office (2001) Economic Series, Government Publications Sales Office, Dublin, Ireland.

Central Statistics Office (1996), Pilot Environmental Accounts for Ireland Government Publications Sales Office, Dublin, Ireland.

Central Statistics Office (1999), Statistical Abstract, 1998 – 1999, Government Publications Sales Office, Dublin, Ireland.

Central Statistics Office (2001), Statistical Bulletin, Government Publications Sales Office, Dublin, Ireland.

Commission of the European Communities (1998), Fair payment for infrastructure use. White Paper, Brussels.

DKM (1994)  Parameter values and application rules for transport infrastructure projects, DKM, Dublin, Ireland.

DKM (1998)  Update of forecasts of vehicle numbers and traffic volumes, DKM, Dublin, Ireland.

DOE (1996) Department of the Environment Irish Bulletin of Vehicle and Driver Statistics for 1996, Department of the Environment and Local Government, Ireland.

DOE (1998)  Department of the Environment Irish Bulletin of Vehicle and Driver Statistics for 1998, Department of the Environment and Local Government, Ireland.

DOE (2001).  Construction industry review 2000 – Outlook 2000 – 2003,  Department of Environment and Local Government, Dublin, Ireland.

Doll C, Lindberg G and Niskanen E (2000), Accounts Approach for Accidents. UNITE (UNIfication of accounts and marginal costs for Transport Efficiency) Working Funded by 5th Framework RTD Programme, Interim Report 8.2. ITS, University of Leeds, Leeds.

Dublin Port (2001) Data provision on costs and demand, Dublin Port, Ireland.

Dublin Transportation Initiative (DTI) (1995) Final Report, Steer Davies Gleave, Department of the Environment, Dublin, Ireland.

Dublin Transportation Office (1999)  Estimates of congestion costs for Dublin, DTO, Dublin, Ireland.

ECMT - European Conference of Ministers of Transport (1998), Efficient Transport for Europe. Policies for Internalisation of External Costs, Paris.

Environmental Protection Agency (2001).  Emissions data spreadsheets for 1996 – 1999.  EPA, Dublin, Ireland.

European Commission (1999), ExternE Externalities of Energy. Vol 10 – National Implementation. A report produced for the EC – DG XII, Luxembourg, Office of Publications for the European Communities, Luxembourg.

Feeney (2001) Estimates of veh.km per veh.type for 1998, Goodbody Economic Consultants, Dublin, Ireland.

Gibbons, E.  (1998).  Application of an economics model to transport externalities in Dublin, MSc Thesis, Department of Civil, Structural & Environmental Engineering, Trinity College Dublin, Ireland.

Irish Ferries (2001) website http://www.irishferries.ie/

Irish Rail (Iarnrod Eireann) (1998)  Annual accounts for 1998, Irish Rail, Dublin, Ireland

Irish Rail (Iarnrod Eireann) ( 2000)  Annual accounts for 2000, Irish Rail, Dublin, Ireland.

IRMS (1998)  A review of railway safety in Ireland, Department of Public Enterprise, Dublin, Ireland.

Link H, Stewart L, Maibach M, Sansom T and Nellthorp J (2000), The Accounts Approach. UNITE (UNIfication of accounts and marginal costs for Transport Efficiency) Working Funded by 5th Framework RTD Programme, Deliverable D2. ITS, University of Leeds, Leeds.

National Roads Authority (1997)  Road Accident Facts for 1996,  National Roads Authority, Dublin, Ireland.

National Roads Authority (1998)  Road Accident Facts for 1998, National Roads Authority, Dublin, Ireland.

National Road Needs Study (1998).  MCOSullivan, National Roads Authority

Nellthorp J, Sansom T, Bickel P, Doll C and Lindberg G (2001), Valuation Conventions for UNITE. UNITE (UNIfication of accounts and marginal costs for Transport Efficiency) Working Funded by 5th Framework RTD Programme. ITS, University of Leeds, Leeds.

Oscar Faber, Ecotec, Goodbody Economic Consultants, ESRI (1999).  Study of the environmental impact of Irish transport growth and of related sustainable policies and measures.  Study commissioned by the Department of Public Enterprise, Summary and Technical Annexes, Department of Public Enterprise, Dublin, Ireland.

Tinch R. (1997) ‘The Valuation of Environmental Externalities’, Conference Paper, Determining the Monetary Values of Environmental  Impacts, University of Westminster, October 1997

- European Commission (1999) ExternE Externalities of Energy.

There are a number of other payments made to companies including fuel rebate (which is also available to private transport companies) and reimbursements, effectively, for free travel (old age pensioners are entitled to free travel on public transport in Ireland).  Contracts for school transport are in effect commercial arrangements but transfers are made for  them from the Department of Education.