- COMPETITIVE AND SUSTAINABLE GROWTH

(GROWTH)

PROGRAMME

Deliverable 8 Annex 6 The Pilot Accounts for the Netherlands

- Version 2.2

- June 2003

Authors: Corina Certan, Marten van den Bossche, Eline Devillers, Hellen Boersma (NEI),

with contributions from DIW, IER and other partners

Contract: 1999-AM.11157 Project Co-ordinator: ITS, University of Leeds Funded by the European Commission 5th Framework – Transport RTD

UNITE Partner Organisations ITS/UNIVLEEDS (UK), DIW (De), NEI (Nl), CES/KUL (Be), TIS.PT (Pt), IWW/UNIKARL (De), VTI (Se), IER/USTUTT (De), CERAS/ENPC (Fr), HERRY (Au), EIET/ULPGC (Es), ISIS (It), STRATEC (Be), SYSTEMA (Gr), JP-TRANSPLAN (Fi), VATT (Fi), ECOPLAN (Ch), INFRAS (Ch), EKI (Se)

- UNITE

1999-AM.11157

- UNIfication of accounts and marginal costs for Transport Efficiency

- Pilot Accounts – Results for the Netherlands

- This document should be referenced as:

Corina Certan, Marten van den Bossche, Eline Devillers, Hellen Boersma (NEI), UNITE (UNIfication of accounts and marginal costs for Transport Efficiency) Working Funded by 5th Framework RTD Programme. ITS, University of Leeds, Leeds, July 2002.

June 2003

Version No: 2.2

Authors: as above

- PROJECT INFORMATION

- Contract no: 1999-AM.11157:

- UNIfication of accounts and marginal costs for Transport Efficiency

- Website: www.its.leeds.ac.uk/unite

- Commissioned by: European Commission – DG TREN; Fifth Framework Programme

- Lead Partner:	Institute for Transport Studies, University of Leeds (UK)

Partners: 	ITS/UNIVLEEDS (UK), DIW (De), NEI (Nl), CES/KUL (Be), TIS.PT (Pt), IWW/UNIKARL (De), VTI (Se), IER/USTUTT (De), CERAS/ENPC (Fr), HERRY (Au), EIET/ULPGC (Es), ISIS (It), STRATEC (Be), SYSTEMA (Gr), JP-TRANSPLAN (Fi), VATT (Fi), ECOPLAN (Ch), INFRAS (Ch), EKONO (Fi), EKI (Se)

- DOCUMENT CONTROL INFORMATION

- Status: 	Accepted

Distribution:	DIW

Availability:	Public (once status is Accepted)

Filename:	D8NL Annex 6

Quality assurance:

Co-ordinator’s review:

Signed: 	Date:

- Table of contents

1	Introduction	7

1.1	Study context and objectives of this annex report	7

1.2	The accounts approach of UNITE	7

1.2.1	Aims of the pilot accounts	7

1.2.2	Core, supplementary and excluded data in the pilot accounts	8

1.2.3	The six UNITE pilot account cost categories	8

1.2.4	The transport modes covered in the pilot accounts	10

1.3	Background information for the transport accounts in the Netherlands	11

1.4	Results presentation and guidelines for interpretation	13

1.5	The structure of this report	13

2	Description of input data	14

2.1	Background for the Dutch transport sector and basic input data	14

2.1.1	Road transport	15

2.1.2	Rail transport	16

2.1.3	Public transport – tram, metro, bus	17

2.1.4	Aviation	18

2.2.5	Waterborne transport: inland waterway navigation and

maritime shipping	18

2.2	Input data per cost/revenue category	18

2.2.1	Infrastructure costs	18

2.2.2	Supplier operating costs	20

2.2.3	Delay costs due to congestion	21

2.2.3.1	Values of time	21

2.2.3.2	Road transport	22

2.2.3.3	Rail transport	24

2.2.3.4	Aviation transport	25

2.2.3.5	Waterborne transport	26

2.2.4	Accident costs	26

2.2.5	Environmental costs	32

2.2.5.1.	Global warming	34

2.2.5.2.	Noise	35

2.2.5.3.	Nature and landscape	35

2.2.5.4.	Nuclear risk	36

2.2.6	Taxes, charges, subsidies	37

3	Methodological issues	38

3.1	Methodology for estimating infrastructure costs	38

3.2	Methodology for estimating supplier operating costs	40

3.3	Methodology for estimating delay costs due to congestion	40

3.4	Methodology for estimating accident costs	41

3.5	Methodology for estimating environmental costs	45

3.5.1	Air pollution	45

3.5.2	Global warming	46

3.5.3	Noise	47

3.5.4	Nature and landscape	48

3.6	Methodology for estimating taxes, charges and subsidies	51

3.7	Methodology for the forecasts to 2005	53

3.7.1	General background	53

3.7.2	Basic input data	54

3.7.2.6	Road transport	54

3.7.2.7	Public transport and rail transport	54

3.7.2.8	Aviation	55

3.7.2.9	Waterborne transport	56

3.7.3	Cost specific input data	56

3.7.3.10	Infrastructure costs	56

3.7.3.11	Supplier operating costs	57

3.7.3.12	Delay costs due to congestion	57

3.7.3.13	Accident costs	57

3.7.3.14	Environmental costs	57

3.7.3.15	Taxes, charges and subsidies	59

4	Results	60

4.1	Infrastructure costs	60

4.1.1	Road	60

4.1.2	Rail	61

4.1.3	Public Transport	61

4.1.4	Waterborne transport - inland waterways	62

4.1.5	Sea Harbours	63

4.1.6	Aviation	64

4.2	Supplier operating costs	64

4.2.1	National rail carrier NS Reizigers	64

4.2.2	Public transport	65

4.3	Delay costs due to congestion	66

4.4	Accident costs	70

4.5	Environmental Costs	73

4.5.1	Overview of the total environmental costs	73

4.5.2	Air pollution and global warming	76

4.5.3	Noise costs	78

4.5.4	Nature and landscape	79

4.6	Taxes, charges, subsidies	80

4.6.1	Road transport	80

4.6.2	Rail transport	81

4.6.3	Public transport excluding rail	82

4.6.4	Aviation	82

4.6.5	Waterborne transport	83

5	Summary of results for the Netherlands	84

5.1	Road transport	84

5.2	Rail transport – National rail carrier Nederlandse Spoorwegen NS	91

5.3	Public transport	95

5.4	Aviation	98

5.5	Inland waterway transport	102

5.6	Maritime shipping	104

6	Conclusions	108

6.1	The relevance of the pilot accounts for transport policy	109

6.2	Open questions and future improvements	110

References	112

Glossary	117

Abbreviations	120

Abbreviations used in data tables	121

List of Tables

Table 1: 	Disaggregation used in the Dutch pilot accounts	11

Table 2: 	Basic indicators for the Netherlands	14

Table 3: 	Basic transport related indicators for the Netherlands per mode, 1998	15

Table 4: 	Road mileage driven in the Netherlands in 1998, in million vehicle-km	16

Table 5: 	Basic indicators of the Dutch Railways (NS)	17

Table 6: 	Core data public transport in the Netherlands1)	17

Table 7: 	Basic indicators for aviation	18

Table 8: 	Sources and quality of input data for estimating infrastructure costs	19

Table 9: 	Sources and quality of input data for estimating supplier operating costs	20

Table 10:    VOT values in 1998 prices (UNITE conventions and PPP-adjusted for the Netherlands)	21

Table 11:	Sources and quality of the input data for road	22

Table 12:	Input data for estimation of delays costs on road, 1998	23

Table 13:	Sources and quality of the input data for rail	24

Table 14:	Data input for estimation of delay costs for rail, 1998	24

Table 15:	Sources and quality of the input data for aviation	25

Table 16: 	Data input for estimation of delays costs in aviation, 1998	26

Table 17:	Input data for valuation of accident costs	27

Table 18:	Number of registered accidents with casualties and material damage by road network type, 1998.	28

Table 19:	Number of accident victims by mode, 1998	29

Table 20:	Number of accidents with material damage by mode, 1998	30

Table 21:	Environmental data in the EcoSense database	32

Table 22:	Input data for air pollution	33

Table 23: 	Direct transport emissions in the Netherlands in million kg, 1998	34

Table 24: 	Persons exposed to different transport noise levels, thousands 1998	35

Table 25:	Input data for nature and landscape environmental costs	35

Table 26: 	Network data for Dutch transport infrastructure	36

Table 27:	Infrastructure characteristics, 1998	36

Table 28: 	Input data and data quality for the calculation of taxes, charges and subsidies	37

Table 29: 	Average costs for medical treatment (€ per casualty), 1998	42

Table 30:	Composition of the lost working time per degree of injury, 1998	43

Table 31:	Average unit costs per material damage of road accidents, € 1998	44

Table 32:	Monetary values (factor costs) for health impacts due to airborne pollutants (€1998)	46

Table 33: 	Monetary values (factor costs) for impacts due to noise, Euro 1998	48

Table 34:	Valuation of effects on nature and landscape	50

Table 35: 	Forecasts for the basic input data (EC scenario)	54

Table 36: 	Yearly growth forecasted for fuel use and mileage for toad transport	54

Table 37: 	Traffic output forecasts for the public transport (EC scenario)	55

Table 38: 	Traffic forecasts for public transport	55

Table 39: 	Traffic forecast for Amsterdam Schiphol airport	55

Table 40: 	Forecast for Amsterdam Schiphol airport	56

Table 41: 	Volume forecasts for the Amsterdam Schiphol airport	56

Table 42: 	Forecasts for energy use	58

Table 43: 	Forecast for emissions	58

Table 44:	Input data for road emissions for 2005	58

Table 45:	Forecasts for number of persons exposed to noise for 2005, million	59

Table 46: 	Capital value and costs, million Euro 1998	60

Table 47: 	Infrastructure costs for road (million Euro prices 1998)	60

Table 48:	Infrastructure costs for rail (million Euro prices 1998)	61

Table 49:	Infrastructure costs for public transport (million Euro prices 1998)	62

Table 50: 	Infrastructure costs for IWW (million Euro, 1998 prices)	63

Table 51:	Infrastructure costs for sea harbours (million Euro prices 1998)	63

Table 52:	Infrastructure costs for aviation (million Euro prices 1998)	64

Table 53: 	Supplier operating costs for rail transport (million Euro)	65

Table 54:	Supplier Operating Costs public transport (million Euro, prices 1998)	65

Table 55:	Total delay costs for the Netherlands, € million 1998	66

Table 56: 	Total and average delay costs, € million 1998	67

Table 57: 	Total and average delay costs in 1996, € million prices of 1998	68

Table 58: 	Relative changes of delay costs in 1996 to 1998 in prices of 1998, %	69

Table 59: 	Total and average delay costs for 2005, € million prices of 1998	70

Table 60:	Social costs of transport accidents, in € million, 1998 prices	70

Table 61: 	Total social accident costs by cost bearer, in € million, 1998	71

Table 62:	Total costs of road accidents per type of road, € million, 1998	71

Table 63: 	Average costs of road accidents per type of road, €/1000vkm, 1998	72

Table 64: 	Social costs of transport accidents in 1996 and 2005, in € million	72

Table 65: 	Total social accident costs by cost bearer in 1996 and 2005, in € million	73

Table 66:	Average costs of road accidents per type of road in 1996 and 2005, €/1000vkm	73

Table 67:	Environmental costs per mode in € million, 1998	74

Table 68:	Environmental costs for roads transport in € million, 1998	75

Table 69:	Environmental costs per mode in 1996, € million prices 1998	76

Table 70:	Total and average costs for air pollution and global warming, 1998	77

Table 71: 	Air pollution and global warming costs for 2005, in million €	78

Table 72: 	Air pollution and global warming costs in 2005 for road transport	78

Table 73:	Total noise costs by mode, € million	78

Table 74:	Noise costs for road and rail transport by vehicle category.	79

Table 75:	Total and average costs of nature and landscape, 1998	80

Table 76:	Road transport revenues, € million 1998	81

Table 77: 	Revenues from taxes, charges and tariffs in rail transport, € million	81

Table 78: 	Revenues of the public transport (excl. rail), € million	82

Table 79:	Revenues of aviation infrastructure, € million	82

Table 80:	Revenues of maritime shipping, € million	83

Table 81: 	Dutch Road Account for 1996, 1998 and 2005 (€ million at 1998 prices)	84

Table 82: 	Fully allocated costs of road transport per vehicle km in Netherlands,  €/km at 1998 prices	86

Table 83: 	Total costs of road transport in Netherlands, € million at 1998 prices	87

Table 84: 	Total costs of road transport in Netherlands, € million at 1998 prices	88

Table 85:	Total costs of road transport in Netherlands, € million at 1998 prices	89

Table 86: 	Total costs of road transport in Netherlands, € million at 1998 prices	90

Table 87:	Dutch Rail Account for 1996, 1998 and 2005 (€ million at 1998 prices)	91

Table 88: 	Fully allocated costs of rail transport per vehicle km in Netherlands, €/train km at 1998 prices	93

Table 89: 	Total costs of rail transport: Dutch Railways (NS), € million at 1998 prices	94

Table 90: 	Dutch public transport account (bus) for 1996, 1998 and 2005 (€ million at 1998 prices)	95

Table 91: 	Fully allocated costs of bus per vehicle km in Netherlands, €/km at 1998 prices	97

Table 92: 	Dutch air transport account for 1996, 1998 and 2005 (€ million at 1998 prices)	98

Table 93:	Fully allocated costs of aviation per vehicle km for Netherlands (€/km at 1998 prices)	100

Table 94: 	Total costs of aviation in Netherlands (€ million at 1998 prices)	101

Table 95:	Dutch inland waterway account for 1996, 1998 and 2005 (€ million at 1998 prices)	102

Table 96: 	Dutch inland waterway account for 1996, 1998 and 2005 (€ million at 1998 prices)	104

Table 97: 	Fully allocated costs of IWW and maritime per vehicle km in Netherlands (€/km at 1998 prices)	106

# Introduction

## Study context and objectives of this annex report

This annex report contains the full version of the Dutch pilot account developed within the UNITE project. It serves as background report for the results presented in the core body of “Pilot Accounts – Results for tranche B countries” and gives more detailed descriptions on the methodology used and the input data and their reliability and quality. However, the general and detailed discussion of the accounts approach was presented in Link et al. (2000 b) and will be summarised only in this document. This annex report discusses methodologies only in so far as they are necessary background information for understanding the results and describes rather the application of methodology to the Dutch case. Furthermore, in addition to the core accounts for 1998 this annex report also presents the results for 1996 and to a certain degree for 2005. Due to the lack of reliable and detailed (for the purpose of this pilot account) information forecasts for 2005 were selectively estimated. This annex report was produced by NEI (overall responsibility), with contribution with DIW and IER.

In order to put this annex report into the context of the UNITE project a summary of the aims and research areas of UNITE is given here. The UNITE project endeavours to provide accurate information about the costs, benefits and revenues of all transport modes including the underlying economic, financial, environmental and social factors. To achieve this goal, three main areas of research are carried out, known as “transport accounts”, “marginal costs” and “integration of approaches”. This report belongs to the research area “transport accounts”.

For a better understanding of the results presented here it has to be borne in mind that the UNITE project distinguishes between ideal accounts on the one hand and the pilot accounts on the other hand. The ideal accounts reflect the perfect situation with the utmost disaggregation, showing factors such as the time and location and duration of individual trips, all the relevant economic data as well as the individuals response to possible policy or infrastructure changes. The pilot accounts are the actual, feasible accounts given the available data for the 18 countries that UNITE covers. They can be used to assess the costs and revenues of transport per transport mode. The costs are reported and documented at the current level of transport demand for the reference years 1996, 1998 and 2005. Reported transport costs are allocated to user groups, where possible without arbitrary allocation methods.

## The accounts approach of UNITE

### Aims of the pilot accounts

The pilot accounts attempt to show the general relationship between costs of transport and the revenues from transport pricing and charging in the country studied. The aims and role of the pilot accounts are discussed in detail in “The Accounts Approach” Link et al. (2000). It should be stressed that the accounts are aimed at providing the methodological and the empirical basis for in-depth policy analysis and monitoring rather than serving as a guide for immediate policy actions such as setting higher/lower prices and charges or opening up/shutting-down transport services/links in order to achieve cost coverage. The pilot accounts are defined as follows:

The pilot accounts compare social costs and / charges on a national level in order to monitor the development of costs, the financial taxes balance and the structure and level of prices. Accounts can therefore be seen as monitoring and strategic instruments at the same time. They have to consider the country-specific situation and the institutional frameworks.

The pilot accounts show the level of costs and charges as they were in 1998 (and 1996 respectively) and provide a workable methodological framework to enable regular updating of transport accounts. Furthermore, an extrapolation for 2005 is presented. The choices of additional accounting years (1996 and 2005) were motivated by the need to show a comparison between years and to give a good indication of trends in transport for the near future. Also, the inclusion of 1996 provides a double check on any major statistical abnormalities that may occur in one year, for example very high infrastructure cost due to tunnelling operations or higher than average accident costs because of major accidents occurring in 1998. Note, however, that the core year of the pilot accounts is 1998. The results for 1996 and 2005 are derived from this core year.

Due to their weakness, as a result of the limited time and the lack of reliable and detailed forecasts for the input data as required in the pilot accounts, additional results for the year 2005 are not fully covered in the Dutch pilot account. This remains a challenge for further development of the pilot accounts for the Netherlands.

### Core, supplementary and excluded data in the pilot accounts

The pilot accounts have been divided into the classes “core data” and “supplementary data”. Core data is the data necessary to do a full basic review of the country accounts. Core data is data within the following categories; infrastructure costs; the external costs of transport accidents; the environmental categories air pollution, noise and global warming and supplier operating costs. Transport revenues and taxes are also documented here. Supplementary data falls into two categories.

Firstly, for several cost categories being evaluated there is no standard methodology for the valuation of effects. An example of this is the valuation of loss of biodiversity due to transport infrastructure. Even though a valuation method has been developed for the UNITE Pilot Accounts, we feel that the level of uncertainty (due to lack of comparative studies) is high enough to warrant the information to be classified outside of the core data where efficient and well tried valuation methods have been utilised. Secondly, some costs which can be estimated and valuated are borne by the transport users themselves (for example delay costs). These costs and the methods used to valuate them present valuable further information to the reader, but can not be considered to be part of the overall costs of transport as defined by UNITE. Supplementary data is data within the following categories, congestion costs; the internal part of accident costs including the risk value; and, the environmental costs risk due to the provision of nuclear power and the costs associated with nature and landscape, soil and water pollution. Subsidies also fall within the category supplementary data.

### The six UNITE pilot account cost categories

Data for the pilot accounts are collected within six cost and revenue categories that are described in Link et al. (2000 b) and are summarised in the following section.

Infrastructure costs. For the pilot accounts, data for the assessment of infrastructure costs are structured to show the capital costs of transport infrastructure (including new investments and the replacement of assets) and the running costs of transport infrastructure (maintenance, operation and administration) for all modes of transport studied. As far as possible with current methodological knowledge, infrastructure costs are allocated to user groups and types of transport. Where it is possible to quantify the share of joint costs they are separated out and are not allocated.

Supplier operating costs. All monetary costs incurred by transport operators for the provision of transport services are documented in the category supplier operating costs. Ideally, the data is structured to show what costs are incurred for vehicles, for personnel and for administration. However, this depends on data availability and will differ from country to country. Since collecting and supplementing this data for all modes is extremely time consuming the UNITE project focuses on estimating supplier operating costs only for those modes where significant state intervention and subsidisation is present. The main emphasis in this category is thus on rail transport and other public transport (tram, metro, bus). Whether other modes also have to be covered depends on the degree of state intervention in the respective countries. The corresponding revenues from the users of transport are included when supplier operating costs are estimated. The difference between such costs and revenues is the net public sector contribution (economic subsidy).

Delay costs due to congestion. In the European Commission’s White Paper “Fair payment for infrastructure use” (1998), costs caused by transport delays, accidents and environmental effects of transport are estimated to be the three major causes of external transport costs. In the category congestion costs, the costs of delay and delay-caused additional operating costs are estimated. Note, within the pilot accounts the term congestion costs is used even though delay costs only were calculated. The name of the cost category “user costs” (Link et al. 2000 b, Doll et al. 2000) signifies that we are aware that this category does not cover all aspects of costs related to congestion. The estimation of delay costs as defined here is carried out for all transport modes, provided data is available. This data is classified as supplementary data because the bulk of these costs are borne by transport users as a whole.

Accident costs. The loss of lives and the reduction of health and prosperity through transport accidents are of major concern to all countries and to the European Commission. In this section of the accounts, the health related accident costs are calculated by assessing the loss of production, the risk value and the medical and non-medical rehabilitation of accident victims. Where the available data basis allows, the damage to property and the administrative costs of accidents are also considered. The external part of accident costs (defined in this report as accident costs imposed by transport users on the whole society) is included in the core section of the accounts. The internal part of accident costs however, costs imposed by one user on other users and are therefore treated as supplementary costs.

Environmental costs. A wide range of transport related environmental impacts and effects, presently being hotly debated in all countries, is considered in this section of the accounts. Included in this cost category are: air pollution, global warming, noise, changes to nature and landscape, soil and water pollution and nuclear risks. The valuation of these environmental effects is carried out for all transport modes, provided adequate data is available.

Taxes, charges and subsidies. In this section, the level of charging and taxation for the transport sector is documented for each mode of transport. Wherever possible, the revenues from taxes and charges are shown for fixed taxes and charges and variable ones. This information plays an important part in the ongoing discussions about the level of taxation between transport modes and countries. The comparison between taxes levied and the costs of infrastructure provision and use accrued per mode is central to this debate and holds a high level of political significance. Environmental taxes that apply to transportation are separately considered in this section. Taxes such as VAT that do not differ from the standard rate of indirect taxes are excluded from this study.

A further part in this area is reporting on subsidies. The need to maintain free and undistorted competition is recognised as being one of the basic principles upon which the EU is built. State aid or subsidies are considered to distort free competition and eventually cause inefficiency. Subsidies to the transport sector provided by the member states are not exempted from the general provisions on state aid set out in the Amsterdam Treaty. There are, however, special provisions set out in the treaty in order to promote a common transport policy for the transport sectors of the member states (Treaty establishing the European Community: Articles 70 – 80). The subsidies of the transport sector are considered in this section. It should be noted that a complete reporting on subsidies would require an extremely time-consuming analyses of public budget expenditures at all administrative levels. Furthermore, the subsidies reported in the pilot accounts refer mainly to direct subsidies (e. g. monetary payments from the state to economic subjects) at the federal state level but generally not at the municipal level. Indirect subsidies (e. g. tax reductions and tax exemptions that cause lower revenues of state budgets) are quantified where possible.

### The transport modes covered in the pilot accounts

The modes covered in UNITE are road, rail, other public transport (tram, metro, trolley bus), aviation, inland waterway navigation and maritime shipping. The level of disaggregation into types of networks and nodes, means of transport and user groups depends on data availability and relevance per country. Table 1 summarises this disaggregation for the Dutch pilot account. Section 2.1 provides in addition some indicators per mode in order to show the importance and relevance of each mode in the Dutch transport system.

Table 1: 	Disaggregation used in the Dutch pilot accounts

| Transport modes | Network and institutional differentiation | Means and user breakdown |
| --- | --- | --- |
| Road | Motorways Trunk roads Urban roads | Motorcycles Passenger cars Buses Light goods vehicles Heavy goods vehicles Special and agricultural vehicles |
| Rail | National Rail (NS) | Passenger transport Freight transport |
| Other public transport | – | Busses |
| Aviation | Airports Air transport | Passenger transport Freight transport |
| Inland waterway shipping | Inland waterways Inland waterways harbours | – |
| Maritime shipping | Seaports | – |
| Source: NEI. |  |  |

## Background information for the transport accounts in the Netherlands

Currently in the Netherlands there are no accounts presented in the way and based on the principles as proposed in UNITE. However, within national transport companies and within transport departments of local, regional and national bodies quite detailed cost accounts exists although not harmonised and not covering all aspects relevant to UNITE.

As the practice shows, in the Netherlands cost and expenditure information is used within several modes (rail, public transport) to compare costs and revenues, and to show cost coverage ratios. This serves as a basis for pricing and taxation policies. For other modes (road, air, inland waterways, short sea shipping) pricing and taxation policies are available, but not necessarily based on cost recovery ratios.

Airports and ports are mostly owned by state-owned/municipal bodies. However, in practice they can operate to a certain degree autonomously, based on business-oriented practices.

Within the Netherlands there is a great deal of information available within private firms and governmental bodies. However, this information is generally not structured and presented at the national level, but rather on the level of individual transport firms accounts. The category of firms is as follows:

- Inland waterway companies;

- Road transport firms, passenger and road;

- Airline companies;

- Airports

- Sea transport companies,

- Sea ports;

- Dutch railways.

Besides quantitative data on traffic and on supply (length of networks etc.) by mode, the aggregated business accounts provide information about total expenses and revenues of each transport organisation - final consumption, investments, imports and exports, and public revenues and expenses. However, it was observed that all business accounts present only total results of activities. Very rarely the results are presented for individual services, or specific categories apart. In addition, all available accounts differ across modes and across transport enterprises within the same mode, which complicates the harmonisation of the input data.

Also it has to be born in mind that transport operators in the Netherlands officially have very little obligations towards the level of detail of the business units accounts (BUA) they have formally to submit at their local branch of the Chamber of Commerce, who gathers all BUAs, and can provide copies of them upon request.

The pilot account work for the Netherlands as presented in this report has an innovative character for EU as a whole and for the Netherlands in particular. Alike accounts could be found in the Netherlands in the seventies (Nationale verkeers- en vervoerrekening, 1977), however, this is not the practices anymore. However, under the request of the Ministry of Transport and Waterways a number of studies were undertaken in the last years which implicitly include a number of UNITE aspects. There is no single study which covers all modes and all costs categories as wide as in UNITE. Most of the research and results are available for road sector (i.e. accident and congestion). Some of these studies and research have a periodical character, other are one-time studies.

An exemplification to that are:

- a periodical update on the costs of traffic jams in the Netherlands on the main road

network (AVV, 1997);

- a periodical update of the costs of road accidents in the Netherlands (AVV, 1999);

- a study of the social costs of the transport sector (CE, 1999);

- periodical studies on the environmental costs.

On the behalf of the Dutch Ministry of Transport the Centre for Energy Savings and Clean Technology has done an overview of the international studies related to “Social Costs of the Transport Sector” (1994) with a special accent on external costs – congestion, environment, safety. One of the conclusions/recommendation of their research to the Dutch authorities was the need for annual overviews of the costs and revenues in the transport sector. According to the study, they should be done systematically and should distinguish between fixed and variable costs, as well as between different types of vehicles. Moreover, it should include information about the relation between marginal costs and average costs.

Following these recommendations, a number of studies related to estimation of the social costs of transport use have been undertaken in the past years at the request of the Dutch Ministry of Transport. In particular they aimed at analysing and estimating the social costs associated with various modes of transport operating in the Netherlands. One of the most exhaustive recent studies – “Efficient Prices for Transport” - has been carried out by the Centre for Energy Conservation and Environmental Technology in 1999. It investigated the marginal social costs and their initial effect on prices, in both quantitative and qualitative terms.

The results of the already implemented studies (including UNITE)y proves the possibility of gathering and structuring the existent statistical information at different levels to a general/national level; and the possibility of estimating the total social costs and revenues of all modes. The most problematic aspect is the allocation of the costs to different users. This is mainly due to the lack of disaggregated input information.

Summarising, the state-of-the-art of the transport accounts in Netherlands can be characterised as “emerging - awareness” towards the concept as such. Even more, certain topics/aspects of the accounts do attract a great deal of attention in the transport policy debates. The trade-off between building versus pricing, the concept “the user pays”, variabilisation of transport costs, inclusion of external costs in pricing – all those are elements that are currently debated to a great extent. However, the present report is the first attempt to present a national transport account in the Netherlands.

## Results presentation and guidelines for interpretation

The goal of the data collection and estimation of cost and revenues in each category was a level of disaggregation that shows the pertinent costs and charges of the relevant transport mode. From the available, but very heterogeneous input data and results, a structure for reporting transport accounts has been developed. All results are documented separately for each cost category and are summarised in modal accounts covering all cost and revenue categories. Additionally, a set of data needed as basic data for all cost categories was collected to ensure that commonly used data have consistency between the cost categories. Minor discrepancies in the basic data used between cost categories are due to the fact that the level of disaggregation in the input data required for each cost category differed. However every effort was used to consolidate the basic data to ensure consistent results for all cost categories.

The categories studied present a comprehensive estimation of transport costs and revenues. They are however, not a total estimation of transport costs. Each cost category could include data in further areas and a definite border had to be drawn around the data to be collected for this project. For example, the estimation of environmental costs does not include the environmental costs incurred during the manufacturing of vehicles, even though these costs could be estimated. These costs would be included in an ideal account, but lie outside the scope of the pilot accounts. Further transport costs categories such as vibration as attributing to environmental costs are not evaluated because no acceptable valuation method has been developed.

It should be noted that due to the separation into core and supplementary data with different levels of uncertainty and with different types (costs borne by transport users themselves versus external costs) care is needed when comparing costs and revenues.

## The structure of this report

This report contains four major parts. Chapter 2 briefly explains firstly the organisation of the Dutch transport sector and the importance of each mode in order to provide some background information for the interpretation of the pilot accounts. Secondly, the input data that was used in the accounts is described here. The main methodological issues which have appeared during the elaboration of the accounts for the Netherlands are discussed in chapter 3. The results are presented and discussed in chapter 4. The descriptions in these chapters are organised along the categories infrastructure costs, supplier operating costs, congestion costs, accident costs, environmental costs and taxes, charges and subsidies. Chapter 5 presents the summary tables on the Dutch pilot accounts and chapter 6 draws conclusions.

# Description of input data

## Background for the Dutch transport sector and basic input data

This section aims at providing some basic information on the features of the Dutch transport sector, the organisational structure and the importance of transport modes as far as necessary for understanding and interpreting the pilot accounts. Table 2 therefore presents some main social and economic indicators.

Table 2: 	Basic indicators for the Netherlands

|  | unit | 1996 | 1998 |
| --- | --- | --- | --- |
| Land area | sqkm | 34 | 34 |
| Population | 1 000 | 15494 | 15654 |
| Population density | inhabitants/sqkm | 457 | 462 |
| Population employed | 1 000 | 6681 | 6957 |
| Employment Rate | % | 43.12 | 44.44 |
| GDP1) | € million | 315059 | 352207 |
| GDP per capita | € | 17333 | 19370 |
| GDP growth rate (change to previous year) | % | 3.0 | 3.7 |
| GDP2) | € million | 281885 | 314236 |
| DGP deflator | 1995 = 100 | 101.2 | 105.1 |
| Consumer prices (inflation) | % | 2.0 | 2.0 |
| 1) At market prices. – 2) At factor costs. Sources: Statistics Netherlands, CBS (1999, 2000). |  |  |  |

Table 3 gives an overview on transport related indicators per mode which will be summarised in the subsequent text. Additionally, in order to present results based on the same basic data among partners, a set of basic reference data for the Netherlands was developed. This data was commonly used for the calculations in the specific cost categories and is also presented in sections 2.1.1-2.1.5.

Table 3: 	Basic transport related indicators for the Netherlands per mode, 1998

| Indicator | Unit | Road | Rail | Public transport | Aviation | Inland waterway navigation | Maritime shipping |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Transport performance |  |  |  |  |  |  |  |
| Passengers carried | mill. | • | 321 | 6.112) | 35.7 | : | 1.8 |
| Passenger-km | bill. pkm | 191.5 | 14.94) | 6.3 | 58* | : | : |
| Goods transported | mill. t | 5461)3) | 24.73) | • | 1 | 237.31)3) | 404.82) |
|  | % |  |  |  |  |  |  |
| Tonne-km | bill. tkm | 46.5 | 3.84) | • | 6* | 59.13) | 96.9 |
| Network length | 1000 km | 124.55) | 2.808 | • | 1510) | 5.046 | 1611) |
| Employees | 100 | : | : | 26713) | 398 | 135 | 106 |
| Gross fixed capital formation7) | € mill. | 10496) | : | 419 | 3348) | 224 | 377 |
| Production7) | € mill. | 8.011 | : | 1.003 | 6.297 | 1.123 | 2.947 |
| Consumption7) | € mill. | 3.602 | : | 0.712 | 3.924 | 0.526 | 2.193 |
| Value added9) | € mill. | 4.409 | : | 0.290 | 2.373 | 0.597 | 0.755 |
| 1) Including commercial and private transport. – 2) Loaded and unloaded. – 3) National and international. – 4) On national track. – 5) as for 1996. – 6) Freight transport. – 7) Data for 1997. – 8) Companies with more than 20 employees. – 9) Gross market prices. – 10) Number of airports. – 11) Number of ports. – 12) By Dutch companies. - 13) Only tram/metro and bus companies. Source: Official statistics |  |  |  |  |  |  |  |

### Road transport

As in most countries, road transport is the main mode in the Netherlands for passenger and freight transport. In 1998, the modal split shares of road transport in Netherlands were 88 % (passengers carried) and 45 % (goods transported). The German road network in 1998 had a length of about 124 500 km representing. The gross fixed capital formation for the road sector amounted to € 761 million in 1998. This value is about 43% of the gross fixed capital formation in the whole transport sector (excl. rail). More than one half of all transport investments were spent for roads. The Dutch road network is exclusively in state ownership.

In 1998, the core year of the pilot accounts, about 117 billion vehicle-km were driven in Netherlands with almost 40% on motorways (see table 4).

Table 4: 	Road mileage driven in the Netherlands in 1998, in million vehicle-km

|  | All Roads | Motorways | Regional Roads | Urban Roads |
| --- | --- | --- | --- | --- |
| Total | 117798 | 43688 | 41374 | 32736 |
| Passenger cars | 93830 | 36552 | 34743 | 22535 |
| Motorcycles | 1432 | 316 | 441 | 675 |
| Mopeds | 1110 | 0 | 111 | 999 |
| Buses | 604 | 223 | 151 | 230 |
| Light goods vehicles | 14163 | 2833 | 4249 | 7081 |
| Heavy goods vehicles | 3356 | 1909 | 1030 | 416 |
| Special vehicles | 361 | 54 | 90 | 217 |
| Agricultural vehicles | 2942 | 1801 | 559 | 583 |
| Source: CBS statline |  |  |  |  |

### Rail transport

The Dutch rail market is characterised by one dominating company, the national rail company Nederlandse Spoorwegen (NS). Up to the eighties the Dutch railways were a state owned company, owning and operating the rails infrastructure as well as the stations. A few years ago the company has been divided in various business units. Among them are NS passengers, NS Cargo, NS Stations, NS Real Estate, NS Security and NS Rolling Stock, NS Infrastructure Management, NS Rail Construction. The only shareholder of NS is the Dutch Government.

Rail infrastructure is divided in a public and a market part. The latter consists of stations, NS telecommunication infrastructure and other real estate. As concerns stations, the law secured that competitors have access to the services. The infrastructure part is under the responsibility of Railned, which is responsible for financing, maintenance and capacity management. Although fully independent in its decisions regarding capacity allocation, Railned is a unit of Dutch railways.

The government finances the NS from the government budget. Most of the operational units are independent of governmental subsidies. An exception is NS Passenger which is subsidised, i.e. NS can receive subsidy for operating non-profitable lines.

Although NS has by far the largest market share, recently another operator (Lovers Rail) has started providing services along with other NS operators. Operators are free to ask for capacity and to start providing a new service. Still there are political discussions aside the regulation and consequences.

NS Cargo became an independent business unit in 1995 following a reorganisation process. As a result of the reorganisation, the total number of employees was reduced. Because of profitability requirements, NS is concentrating on international transport. The reduction of national services has resulted in the closer of some small terminals. NS Cargo is not subsidised. The liberalisation has opened opportunities for new companies to provide services. Their number may increase in future. In the east and north of the Netherlands there are some private companies providing rail passenger services (Synthus, NoordNed).. The NS owns a share of both companies. There is more competition in goods, although the former NS Cargo remains the largest provider. Despite of the opening up of the Dutch network and the beginning of on-track competition, the role of the non-NS rail companies is still minor.

Table 5 shows the main indicators of the NS.

Table 5: 	Basic indicators of the Dutch Railways (NS)

|  |  | 1998 | 1996 |
| --- | --- | --- | --- |
| Train-km |  |  |  |
| Passenger transport | mill. | 110 | 115 |
| Freight transport | mill. | 9* | 9* |
| Passenger-km | billion | 14.879 | 14.131 |
| Number of journeys | mill. | 321 | 306 |
| Productivity |  |  |  |
| Passengers per train | units | 128 | 127 |
| Seats per train | units | 371 | 362 |
| Freight | bill. ton-km | 3.778 | 3.123 |
| National | bill. ton-km | 0.819 | 0.763 |
| International | bill. ton-km | 2.959 | 2.360 |
| Freight | mill. tonnes | 24.7 | 20.8 |
| National | mill. tonnes | 4.4 | 4.7 |
| International | mill. tonnes | 20.3 | 17.9 |
| *) NEI estimation (train-km for passenger transport estimated based on passenger-km). Source: NS, NEI. |  |  |  |

### Public transport – tram, metro, bus

The public transport in the Netherlands is divided into two groups; municipal companies (such as RET, HTM) and private companies (such as Connexxion). The private companies are mostly state-owned although operating as a private company. The non state-owned companies which perform on-demand public transport are partly subsidised by regional governments. Due to the lack of statistical information available for individual modes (i.e. tram, metro), the pilot account will consider only transport by bus.

Table 6: 	Core data public transport in the Netherlands1)

|  | Unit | 1996 | 1998 |
| --- | --- | --- | --- |
| Number of passengers | million | 710 | 725 |
| Municipal companies | million | 52 | 53 |
| Private companies | million | 658 | 672 |
| Passenger-kilometres | million | 6.210 | 6.348 |
| Municipal companies | million | 1.626 | 16.663 |
| Private companies | million | 4.584 | 4.685 |
| Vehicle-kilometres | million | 423 | 421 |
| Municipal companies | million | 312 | 310 |
| Private companies | million | 11 | 110 |
| 1) Excluding international passenger transport by Dutch bus companies (1998: 6 million passengers, 4 billion passenger-kilometres and 100 million vehicle-kilometres) Source: CBS |  |  |  |

### Aviation

There are five major airports in the Netherlands with the following percentage of flight movements, Schiphol (80%), Rotterdam (8%), Maastricht (4%), Eindhoven (4%) and Groningen (4%). Table 7 shows basic data for aviation.

Table 7: 	Basic indicators for aviation

|  | Unit | 1996 | 1998 |
| --- | --- | --- | --- |
| Takeoffs and landings1) | 1000 | 438 | 485 |
| Passengers embarking/disembarking2) | mill. | 28.349 | 35.119 |
| Cargo loading/unloading | mill. t | 1.127 | 1.202 |
| Amsterdam Schiphol |  |  |  |
| Passengers | mill. | 27.262 | 33.952 |
| scheduled flights | mill. | 23.789 | 29.956 |
| non-scheduled flights |  | 3.473 | 3.996 |
| Freight | mill. tonnes | 1.083 | 1.171 |
| Aircraft movements (commercial flights) | 1000 | 321.779 | 321.779 |
| Scheduled flights | 1000 | 293.778 | 348.037 |
| Passenger | 1000 | 285.801 | 339.195 |
| Freight | 1000 | 7.977 | 8.842 |
| Non-scheduled flights | 1000 | 28.001 | 28.773 |
| Passenger | 1000 | 23.048 | 24.897 |
| Freight | 1000 | 4.953 | 3.876 |
| 1) Flight movements on 5 major airports in the Netherlands (Schiphol, Rotterdam, Maastricht, Eindhoven, Groningen) – 2) Passengers counted at each boarding/de-boarding airport. Source: Statistics Netherlands, Schiphol |  |  |  |

### Waterborne transport: inland waterway navigation and maritime shipping

The centre of goods transport is the port of Rotterdam, which possesses excellent natural inland waterway connections in particular to German hinterlands over the Rhine and other rivers. Inland waterways transport is an important mode for freight transport. More than half of the total freight transport is done by waterborne means of transport. When looking at the tonne-km indices this figure overruns 80% of total for all modes

## Input data per cost/revenue category

### Infrastructure costs

The main required input data is a long and disaggregated investment time series per mode. This is needed for the perpetual inventory model which is used to calculate the value of the capital stock and the capital costs. Furthermore, data for running costs is either collected from official statistics or estimated based on surveys or (in some modes) on available business reports. Further required input data are the parameters used in the perpetual inventory model, mainly used to estimate the life expectancies of assets. The input data and an evaluation of the quality are summarised in the table presented below.

Table 8: 	Sources and quality of input data for estimating infrastructure costs

| Input data | Figures estimated | Quality of data, level of uncertainty |
| --- | --- | --- |
| Road |  |  |
| Network length and land area form statistical yearbooks. Investments estimated based upon figures from the Statistics Netherlands (CBS). Price indexes also from CBS. Landprice from Ministry of Transport (1999)1) and corrected with indexes from CBS. | Investments and running costs are derived from total expenditures. | The infrastructure indicators are of high quality. Investments data and running costs must be estimated for this time period. Price indexes are of high quality, while land prices are calculated. |
| Rail |  |  |
| Infrastructure indicators received from CBS. Investments estimated based upon figures from the Statistics Netherlands. Running costs from year report Railway Infrastructure Administration (RIB). | Price index is not specified and thus the index of road construction is being used. Investments are estimated based upon total investments. | The quality of the infrastructure indicators is good. Investment costs are estimated and specific price indexes for rail construction were not available. Running costs available from year report. |
| Public Transport |  |  |
| Infrastructure indicators estimated based upon statistics from CBS and standard values of Ministry of Transport. | Tram, metro and sneltram are included. Investments in PT estimated based upon total investments. Price index is not specified for PT; index for road construction is used. | Good infrastructure data. Investment costs are estimated and specific price indexes for rail construction were not available. Running costs available from annual reports and CBS. |
| Air |  |  |
| Land area based upon figures from CBS. Running cost information based upon year reports from largest Dutch airports | Investments estimated based upon total investments. Price index for road construction used. | Good infrastructure data. Investment costs are estimated and specific price indexes for airport construction were not available. Running costs specified for largest airports. |
| Inland waterway |  |  |
| Land area based upon figures from CBS. Running cost information based upon year reports from the two largest harbours | Investments estimated based upon total investments. Price index for road construction used. | Good infrastructure data. Note: the length of the infrastructure did not change significantly over the years, but width. Although the length of the channels does not change significantly, the investments are mainly directed to their improvement and deepness. Investment costs are estimated and specific price indexes for airport construction were not available. Poor information on running costs. |

| Input data | Figures estimated | Quality of data, level of uncertainty |
| --- | --- | --- |
| Maritime |  |  |
| Land area seaports2) received from National Harbour Council (year reports). Land area inland waterway harbours3) estimated with data from Ministry of Transport1). | Investments estimated based upon total investments. Running costs from largest harbours extrapolated for country. | Good quality of data on land area. Investment costs are estimated and specific price indexes for harbour construction were not available. Information on running costs was not available. |
| 1) ‘Raming maatschappelijke kosten van ruimtegebruik door het verkeer; Efficiënte prijzen voor verkeer’, juli 2000, Vrij Universiteit Amsterdam. 2) Seaports in the Netherlands are: Eemshaven, Harlingen, Den Helder, Amsterdam, Velsen, Zaanstad, Beverwijk, Rotterdam, Maassluis, Vlaardingen, Schiedam, Dordrecht, Moerdijk, Vlissingen, Terneuzen and Scheveningen. 3) Inland waterway ports include: Bergen, Bergen op Zoom, Cuijk, Den Bosch, Geerttruidenberg, Maasbracht, Oosterhout, Roermond, Stein, Utrecht and West Maas en Waal. |  |  |

### Supplier operating costs

As already stated, supplier-operating costs are calculated only for public transport and rail services. The main data sources are annual reports from both the national railway company (NS) and several providers of public transport (Rotterdam (RET), Amsterdam (GVB), Den Haag (HTM) and east-Netherlands (Hermes)). However, not all public transport companies in the Netherlands are taken into account separately. It is however known from the Statistics Netherlands (CBS) what the total revenues of public transport are in the Netherlands. The following table summarises the input data used.

Table 9: 	Sources and quality of input data for estimating supplier operating costs

| Input data | Level of disaggregation | Quality of data, level of uncertainty |
| --- | --- | --- |
| Rail |  |  |
| The NS Groep publishes a year report. The infrastructure costs are the responsibility of the Railway Infrastructure Administration (RIB) and are thus not included in the numbers mentioned in the year report. | For the NS the following disaggregation could be made: revenues (separated to different activities); nett turnover; personnel costs; depreciation; material consumption; energy use; maintenance; and interest. There is no separate information regarding insurance and housing. | Good although rental payments (insurance, housing, advertising) could not be specified separately. |
| Public transport |  |  |
| Year reports from four major providers of public transport (representing 43% of total turnover) | Tram/metro and busses cannot be separated since the same providers operate them. The year reports do not supply sufficient data to split all costs. | Good, although not separately available for tram/metro. |

### Delay costs due to congestion

#### Values of time

The values of time (VOT) per vehicle kilometre were obtained on the basis of the UNITE valuation conventions (Nellthrop et al., 2001). According to these conventions the UNITE values were adjusted for the Netherlands using PPP and converted into factor costs (commuting and leisure values).

Table 10 shows the VOT values which have been used in the estimation of the delay costs due to congestion in a normal situation. For congested situations values were increased with 50%.

Table 10: VOT values in 1998 prices (UNITE conventions and PPP-adjusted for the Netherlands)

|  |  |  | UNITE values 1998 |  | Netherlands 1996 | Netherlands 1998 |
| --- | --- | --- | --- | --- | --- | --- |
| PASSENGER (Euro/h) |  |  |  |  |  |  |
| Road |  | Car |  |  |  |  |
|  |  | Business |  | 21.00 | 22.00 | 22.89 |
|  |  | Commuting/private |  | 6.00 | 5.18 | 5.39 |
|  |  | Leisure |  | 4.00 | 3.45 | 3.59 |
|  |  | Coach |  |  |  |  |
|  |  | Business |  | 21.00 | 22.00 | 22.89 |
|  |  | Commuting/private |  | 6.00 | 5.18 | 5.39 |
|  |  | Leisure |  | 4.00 | 3.45 | 3.59 |
|  |  | Urban bus/tramway |  |  |  |  |
|  |  | Business |  | 21.00 | 22.00 | 22.89 |
|  |  | Commuting/private |  | 6.00 | 5.18 | 5.39 |
|  |  | Leisure |  | 3.20 | 2.76 | 2.88 |
| Interurban-rail |  | Business |  | 21.00 | 22.00 | 22.89 |
|  |  | Commuting/private |  | 6.40 | 5.53 | 5.75 |
|  |  | Leisure |  | 4.70 | 4.06 | 4.22 |
| Air traffic |  | Business |  | 28.50 | 29.85 | 31.07 |
|  |  | Commuting/private |  | 10.00 | 8.64 | 8.99 |
|  |  | Leisure |  | 10.00 | 8.64 | 8.99 |
| FREIGHT (Euro/v-hour) |  |  |  |  |  |  |
| Road |  | LGV |  | 40.00 | 41.90 | 43.60 |
|  |  | HGV |  | 43.00 | 45.04 | 46.87 |
| Rail |  | Full trainload |  | 725.00 | 759.43 | 790.25 |
|  |  | Wagon load |  | 30.00 | 31.42 | 32.70 |
|  |  | Average per tonne |  | 0.76 | 0.80 | 0.83 |
| IWW |  | Full ship load |  | 200.00 | 209.50 | 218.00 |
|  |  | Average per tonne |  | 0.18 | 0.19 | 0.20 |
| Maritime |  | Full ship load |  | 200.00 | 209.50 | 218.00 |
|  |  | Average per tonne |  | 0.18 | 0.19 | 0.20 |
| Air |  | Average per tonne |  | 4.00 | 4.19 | 4.36 |
| Source: Nellthorp et al., 2001 and NEI |  |  |  |  |  |  |

#### Road transport

Table 11 offers a brief description of the input data for the estimation of the delays in the road transport while table 12 presents the input data used.

Table 11:	Sources and quality of the input data for road

| Input data | Disaggregation | Sources, quality of data, level of uncertainty |
| --- | --- | --- |
| Vehicle occupancy rate per purpose of trip | Car, motorcycle, scooter Bus, tram/metro Business, private/commuting, leisure | For the car data from the results of the study-survey on mobility patterns and AVV Filekosten 1997); for motorcycle CE (Efficiente prijzen voor het verkeer, 1999) For bus and tram data from Vervoer gegevens van het GVB-net in "Amsterdam in cijfers", Het Amsterdamse Bureau voor Onderzoek en statistiek, 1999 |
| Travel purpose split | Car, motorcycle, scooter Bus, tram/metro Business, private/commuting, leisure Motorway, trunk road, urban road, other roads | Good quality data from CBS, Mobiliteit van Nederlandse bevolking. |
| Average speed | Car, motorcycle, bus, tramway/metro, LGV, HGV Motorways, trunk roads, urban roads Congested, normal | Average speed for passenger cars and HGV in normal conditions on motorways and trunk roads calculated from the monthly registered speeds provided by AVV Average speed in congested situation taken from DIW based on Tremod traffic model. |
| Fuel price | Car, motorcycle, LDV, HDV Gasoline, diesel, LPG | Good quality of average values from CBS |
| Fuel consumption | Car, motorcycle, LDV, HDV Motorways, trunk roads, urban roads Normal, congested | Good quality data from CBS statline |
| Traffic volume (total mileage in vkm) | Car, motorcycle, bus/metro/tram, LDV, HDV Motorways, trunk roads, urban roads, all roads Total, congested | On total traffic volume - good quality of statistical data from CBS statline. Share of congested traffic - rough estimation of 1.5% (CE, 1999) |

Table 12:	Input data for estimation of delays costs on road, 1998

|  | Fuel consumption l/km |  | Traffic volume million vkm |  | Average speed kph |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | normal | congested | normal | congested | normal | congested |
| Car |  |  |  |  |  |  |
| -motorways | 0.08 | 0.15 | 36 552 | 548.3 | 113.54 | 15.00 |
| -trunk roads | 0.07 | 0.15 | 34743 | 521.1 | 97.05 | 15.00 |
| -urban roads | 0.11 | 0.26 | 22535 | 338.0 | 27.13 | 10.00 |
| Motorcycle |  |  |  |  |  |  |
| -motorways | 0.06 | 0.10 | 316 | 4.7 | 115.00 | 19.00 |
| -trunk roads | 0.06 | 0.10 | 441 | 6.6 | 61.00 | 19.00 |
| -urban roads | 0.06 | 0.06 | 675 | 10.1 | 26.50 | 20.00 |
| Scooter |  |  |  |  |  |  |
| -motorways | 0.00 | 0.00 | 0.00 | 0.0 | 0.00 | 0.00 |
| -trunk roads | 0.03 | 0.05 | 111 | 1.7 | 61.00 | 19.00 |
| -urban roads | 0.03 | 0.03 | 999 | 15.0 | 26.50 | 20.00 |
| Bus/coach |  |  |  |  |  |  |
| -motorways | . | . | 2344* | 35.2* | 84.60 | 5.80 |
| -trunk roads | . | . | 1587* | 23.8* | 56.80 | 5.80 |
| -urban roads | . | . | 2417* | 36.3* | 27.80 | 5.80 |
| LGV |  |  |  |  |  |  |
| -motorways | 0.10 | 0.14 | 2833 | 42.5 | 112.00 | 9.25 |
| -trunk roads | 0.08 | 0.14 | 4249 | 63.7 | 75.30 | 9.30 |
| -urban roads | 0.12 | 0.23 | 7081 | 106.2 | 38.50 | 5.15 |
| HGV |  |  |  |  |  |  |
| -motorways | 0.32 | 1.01 | 1909 | 28.6 | 88.93 | 5.80 |
| -trunk roads | 0.31 | 1.01 | 1030 | 15.5 | 83.96 | 5.80 |
| -urban roads | 0.45 | 0.90 | 416 | 6.2 | 33.80 | 5.40 |
| Source: CBS, AVV, NEI estimation * - in pkm |  |  |  |  |  |  |

Regarding public road passenger transport, it includes all collective passenger transport services carried out on the road network. In contrast to individual road transport by car, where in addition to time costs, fuel costs (and possibly other variable operating costs) influence the decisions of the passenger, this is not the case for public transport. Since only time costs and fares are costs perceived by the passengers, the fuel cost component is omitted here. Passenger services utilising cars or station wagons (taxis etc.) fall under the category motorised individual passenger transport and are covered within road transport.

Ideally, data on the share of vehicle kilometres by public transport carried out on networks separated from the common road network is required. As such information is not available, the following simplifying assumptions were made:

- Buses, tramways and trolley buses are operated on the common road network and can be    fully considered within the available data for road congestion.

- Underground, urban light rail and rapid mass transport services are normally operated on      their own networks, and are not affected by road congestion.

- As no delay data is collected in Netherlands for public transport but buses no estimation of delays was possible. The estimation of delays for buses is a rough estimation since no reliable data on mileage only for buses exists. Official statistics for this figure include buses, metro and trams.

#### Rail transport

Table 13 gives a brief overview of the sources and quality of the input data used for the estimation of the delays in rail transport while table 14 presents the input data used.

Table 13:	Sources and quality of the input data for rail

| Input data | Disaggregation | Sources, quality of data, level of uncertainty |
| --- | --- | --- |
| Travel purpose split | Passenger total | Good quality data from NS; the same share was used for local, regional and intercity services. |
| Average delay | Average delay | Rough assumption of 15 minutes for both passenger and freight. |
| Punctuality index (percentage of trains arriving less than 3 minutes late) | Passenger: local traffic, regional traffic, inter-city services | Yearly figures from NS. For the freight services the weighted average of the punctuality indexes for passenger services was used. |
| Total demand | Passenger: local, regional and intercity services Freight: national, international | Official data from NS annual reports and official statistics |

Table 14:	Data input for estimation of delay costs for rail, 1998

|  | Delay probability % | Total demand Million trips/tons |
| --- | --- | --- |
| Passenger transport |  |  |
| Local traffic | 6.7 | 131.61 |
| Regional traffic | 6.7 | 57.78 |
| Inter-city services | 10 | 73.83 |
| Freight transport |  |  |
| National | 17 | 4.4 |
| International | 17 | 20.3 |
| Source: NS, NEI estimation |  |  |

The delays in rail passenger transport were valued on a trip basis. Out of the number of trips per year made by rail passenger services and the delay probability by type of service, the annual number of delay hours was determined. Rail operating costs were not considered, as they are not directly borne by the users.

The shares of travel purposes in rail transport in 1998 were distributed as follows: 5% business, 51% private/commuting, 44% leisure. With the respective values of normal and delayed travel time average values of travel time were computed to be € 8.90 per passenger hour for all train services.

Unfortunately only rough estimates could be used for the estimation of delays for freight rail mainly based on the information available for the passenger rail. The assumptions used in the calculation of freight delays for average delay is 15 minutes. Traffic volumes for freight were available from NS for national and international traffic.

#### Aviation transport

Table 15 briefly describes the input data for estimation of delays in aviation transport while table 16 present the respective data. Unfortunately no information on delays at other airports than Amsterdam Schiphol could be obtained. Consequently, the delays costs were calculated only for Schiphol airport. Since the Schiphol airport covers about 65% of total flights the estimates could be considered representative.

Table 15:	Sources and quality of the input data for aviation

| Input data | Disaggregation | Sources, quality of data, level of uncertainty |
| --- | --- | --- |
| Travel purpose split | Passenger: domestic scheduled, domestic charter Business, private/commuting, leisure | NEI estimation based on good quality data provided by AVV which are the results of passenger surveys at Schiphol airport. These figures include charter, and scheduled transfer and non-transfer. Weighted average was applied to estimate the travel split for scheduled flights. The categories from the surveys were combined to reflect the UNITE categorisation. For international scheduled and international charter the same travel purpose split as for the respective domestic flights was used. |
| Average delay | Passenger: arrival scheduled and non-scheduled, departure scheduled and non-scheduled Freight: scheduled, non-scheduled Mail: scheduled and mail non-scheduled | Official figures on average delay on arrival and departure from Eurocontrol/CODA For all freight and mail categories the average passenger figures was applied. |
| Punctuality index | Passenger: arrival scheduled and non-scheduled, departure scheduled and non-scheduled Freight: scheduled, non-scheduled Mail: scheduled and mail non-scheduled | Statistical annual reviews of Schiphol airport. For freight weighted averages of arrivals and departures as for passengers were used (for mail scheduled and non-scheduled freight figures were applied). |
| Total demand | Passenger: arrival scheduled and non-scheduled, departure scheduled and non-scheduled Freight: scheduled, non-scheduled Mail: scheduled and mail non-scheduled | Official statistics and statistical annual review of Schiphol airport |

In air passenger transport, punctuality is the percentage of flights departing/arriving within the time period of 15 minutes after scheduled time on/off blocks, regarding scheduled flights only. Amsterdam airport Schiphol provides monthly figures on punctuality of passenger services for scheduled and non-scheduled arrivals and departures. Figures for average delays at arrivals and departures were taken from Eurocontrol/ECAC (2000).

The available statistics does not distinguish between delays in passenger and freight aviation transport. The punctuality index for passenger transport has been applied for freight since a considerable amount of freight is loaded on passenger aircrafts. It has been calculated using the weighted average of punctuality of arrivals and departures.

Table 16: Data input for estimation of delays costs in aviation, 1998

|  | Average delay minutes | Delay probability % | Total demand million trips/tonnes |
| --- | --- | --- | --- |
| Passenger transport |  |  |  |
| Arrivals scheduled | 26.50 | 28.8 | 14.94 |
| Arrivals non-scheduled | 26.50 | 53.5 | 1.96 |
| Departures scheduled | 21.40 | 41.6 | 15.02 |
| Departures non-scheduled | 21.40 | 55.9 | 2.04 |
| Freight transport |  |  |  |
| Scheduled | 24.0 | 35 | 0.37 |
| Non-scheduled | 24.0 | 55 | 0.20 |
| Mail |  |  |  |
| Scheduled | 24.0 | 35 | 0.05 |
| Non-scheduled | 24.0 | 55 | 0.0 |
| Source: Statistical Annual Review 2000, NEI estimation |  |  |  |

#### Waterborne transport

No information on delays in inland waterborne transport could be found. Therefore, respective costs for these delays were not quantified.

### Accident costs

The methodological approach and the corresponding data requirements for the calculation of the accidents costs is elaborated in Link et al. (2000) and in Doll et al. (2000). This approach includes the calculation of five cost components: material damage costs; administrative costs; medical costs; costs due to production losses; and, the costs of suffering and grief (risk value).

The input data per transport mode required for the calculation of accident costs includes on one hand physical information about the number of accidents, number and severity of injuries, fatalities and material damages, and on the other hand, accident unit costs, and information about the payment of insurance companies and transfers of the social security. The sources and remarks on the data quality are given in table 17; the input data is shown in tables 18-20. A more detailed description of the input data per mode will follow below.

Table 17:	Input data for valuation of accident costs

| Input data by cost category | Disaggegation | Sources, quality of data, level of uncertainty |
| --- | --- | --- |
| Physical units |  |  |
| Number of accidents | Mode Reported, underreported (road) Type of accident (material damage, casualties) Degree of severity (slight, severe, death) Vehicle type (incl. passenger) and/or property type | Good input data on the number of road accidents (however, not dissagregated by network type). Source: AVV. No official information about railway accidents. Good input/database on IWW and maritime accidents, however the quality of filled in information sometimes is poor. Source: Good official statistics for aviation accidents (source: ) |
| Number of casualties | Mode Reported, underreported Degree of severity (slight, severe, death) Victim type | Good input data on accident casualties for all modes. Disaggregation by victim type not possible for IWW and maritime, only totals available. Classification of casualties for all modes: deaths and injuries (official distinction between slight and severe not available); for road: slight, sever and deaths. |
| Total payments of the social security and liability insurance | Average figures by type of accident Cases reported and cases not-reported to liability insurance Degree of severity Per vehicle category | No data available on the cases reported to the liability insurance and the total payment. |
| Material damage |  |  |
| Average damage costs of cases reported to liability insurance and average costs per non-reported case | Road Type of vehicle | Average costs of accidents are not available. In the case of average damage cost for road data for Germany have been used with appropriate adjustments. Unit cost per case not reported to insurance was estimated based on SWOV reports. |
| Average liability insurance payments | Only for road for material damage | Data on average payment for material damage in a road accident made available from the Union of insurance Companies. |
| Administrative costs |  |  |
| Cost rate Euro/hour | Police | Statistic and other information from insurance companies and police department on administrative costs related to accidents not available. The average hourly salary has been used for the hourly police cost rate for all modes. No information available for the legal system, liability and health insurance companies. |
| Time required by police, legal system, liability and health insurance companies | Rode, average accident | For time required by police to administer road accidents German information has been used. No information available for other modes. Data for road has been transferred to other roads, therefore being an estimation. |
| Medical costs |  |  |
| Cost values of the health sector and shares of cases with steady reduction of working power | Average costs for injuries by severity class and reduction of working power | Indirect information from hospitals not available. DIW estimates adjusted for Netherlands |

| Input data by cost category | Disaggregation | Sources, quality of data, level of uncertainty |
| --- | --- | --- |
| Production losses |  |  |
| Production related figures | GNP per employed Share of production factor “work” Production potential/GNP Total private consumption | Production related figures from national statistics are of good quality. Number of victims in employable age group: NEI estimation. |
| Replacement cost at the victim’s former place |  | Estimate based on DIW data |
| Risk value |  |  |
| VOSL | Value for injuries in % of VOSL: Severe– 15 % Slight – 1 % | Unite standard value (Nellthorp et al. 2000) |

Table 18:	Number of registered accidents with casualties and material damage by road network type, 1998.

|  | Passenger, car, bus, motorcycle | LGV | HGV | Other | Total |
| --- | --- | --- | --- | --- | --- |
| Highways | 42 502 | 5619 | 5727 | 13907 | 67755 |
| deaths | 161 | 27 | 44 | 95 | 327 |
| injuries | 5 719 | 663 | 525 | 1705 | 8612 |
| material damage | 36 622 | 4929 | 5158 | 12107 | 58816 |
| Regional roads | 31 216 | 3988 | 2590 | 106613 | 48407 |
| deaths | 254 | 27 | 34 | 161 | 476 |
| injuries | 5 105 | 657 | 321 | 2787 | 8870 |
| material damage | 25 857 | 3304 | 2235 | 7665 | 39061 |
| Other roads | 341 222 | 42267 | 19664 | 139208 | 542361 |
| deaths | 616 | 70 | 67 | 689 | 1442 |
| injuries | 32 908 | 3470 | 1092 | 31757 | 69227 |
| material damage | 307 698 | 38727 | 18505 | 106762 | 471692 |
| All roads | 414 940 | 51874 | 27981 | 163728 | 658523 |
| deaths | 1 031 | 124 | 145 | 945 | 2245 |
| injuries | 43 732 | 4790 | 1938 | 36249 | 86709 |
| material damage | 370 177 | 46960 | 25898 | 126534 | 569569 |
| Source: AVV |  |  |  |  |  |

Table 19:	Number of accident victims by mode, 1998

|  | Reported to police |  |  | Total number of casualties |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Slight | Severe | Deaths | Slight | Severe | Deaths |
| ROAD | 16989 | 11733 | 1066 | 105000 | 18620 | 1149 |
| Car passenger | 8124 | 5390 | 582 | 22700 | 6120 | 627 |
| Motorcycle drivers | 692 | 740 | 76 | 4900 | 1110 | 82 |
| PT passengers | 3588 | 2320 | 89 | 17900 | 3110 | 96 |
| PT personnel | 3448 | 2329 | 194 | 53400 | 6760 | 209 |
| Truck drivers | 158 | 96 | 9 | 1200 | 90 | 10 |
| Pedestrians/cyclists | 927 | 827 | 110 | 3700 | 1340 | 119 |
| Others | 52 | 31 | 6 | 1200 | 90 | 6 |
| RAIL | : | 211 | 223 | : | 211 | 223 |
| Passengers | : | 117 | 3 | : | 117 | 3 |
| Level crossing | : | 25 | 32 | : | 25 | 32 |
| Personnel | : | 36 | 3 | : | 36 | 3 |
| Suicide | : | 29 | 181 | : | 29 | 181 |
| Third party | : | 4 | 4 | : | 4 | 4 |
| AVIATION | 6 | 4 | 0 | 6 | 4 | 0 |
| Passengers | 6 | 3 | 0 | 6 | 3 | 0 |
| Crew | 0 | 1 | 0 | 0 | 1 | 0 |
| Third party | 0 | 0 | 0 | 0 | 0 | 0 |
| IWW | : | 26 | 6 | : | 26 | 6 |
| MARITIME | : | 7 | 2 | : | 7 | 2 |
| Sources: Road: Verkeersongevallen 1998, CBS/AVV; AVV website Rail: Tweede kamer der Staten-Genraal, 26 699, nr.2, Railveiligheid. Aviation: NLR IWW and Maritime: NEI estimation based on accident database from AVV |  |  |  |  |  |  |

Table 20: Number of accidents with material damage by mode, 1998

| Mode | Accidents reported to police |  |
| --- | --- | --- |
|  | Slight | Severe |
| ROAD - total | 564870 | 88954 |
| Damage to vehicles |  |  |
| passenger car | 362831 | 41863 |
| motorcycle | 3441 | 2386 |
| bus/coach | 3905 | 514 |
| tramway | 698 | 184 |
| goods vehicles | 68159 | 6997 |
| others | 32231 | 28211 |
| Public property | 32614 | 6260 |
| Other private property | 60991 | 2539 |
| AVIATION – total | 2 | 6 |
| Aircrafts | 1 | 6 |
| Other company assets | 0 | 0 |
| Third party | 1 | 0 |
| IWW |  |  |
| Damage to vessels | 502 | 45 |
| Damage to transported goods | 3 | 6 |
| Damage to waterways | 109 | 267 |
| Environmental damage | 740 | 4 |
| MARITIME |  |  |
| Damage to vessels | 90 | 9 |
| Damage to transported goods | 2 | 1 |
| Damage to waterways | 83 | 76 |
| Environmental damage | 209 | 9 |
| Source: AVV, NEI estimation No information available for rail accidents with material damage. Estimation of accidents in aviation, IWW and maritime have been based on the accident databases from AVV. In the case of IWW and maritime the value is sub estimated because of a number of accidents which do not register the existence or non-existence of material damage. |  |  |

It has to be noted that for the calculation of medical costs and material damage in road transport, the specific problem of underreporting is of special importance. The problem of underreporting occurs only for road transport. Statistics reported for rail, aviation and inland waterway transport can be considered to be correct.

Road. A transport accident on road in the national statistics is defined as an accident on the road related to transport activities, and where at least one moving vehicle is involved and as a consequence one or more road users die and/or are injured. The accidents are registered by the police, which supplies this information to the Netherlands Transport Research Centre, Department of Statistics and Data Management (AVV/BG) who administers the Traffic Accidents Registration System (VOR). The accidents not reported to police are, therefore, not included in this system. In reality, the number of registered accidents depends on the degree of accident and the victim.

Starting with 1997 within the Integration Framework for Road Traffic Casualties (IVO), a collaboration between AVV/BG and Statistics Netherlands (CBS), the actual number of accident casualties is estimated with a methodology elaborated by Institute for Road Safety Research (SWOV). This methodology is based on comparison of information on accident victims as registered in the AVV’s transport accident register and other registration files (e.g. LMR database of hospital registrations of Prismat, LIS of Consumer and Safety, OVO of CBS). The data is published annually in a common publication of the above-mentioned organisation and replaces the earlier CBS publication “Traffic accidents on public roads”.

As concerns the accidents with material damages, there is no official estimate of unreported accidents. Currently, SWOV is developing a methodology for estimation of those. For the purpose of the current analysis the registration degree for slight and severe injuries have been used for the estimation of unreported accidents respectively with slight and severe material damage.

The casualties as well as the accidents in the statistics are classified in deaths, hospital admissions which in this report are considered as severe injuries, injured persons treated in the hospital’s Emergency Care Department – considered here as slight injuries, and accidents with only material damage. At the same time, accidents and casualties are subdivided according to their basic characteristics. Casualties are classified according to how they were involved in the accident, their ages and genders. Accidents on their turn are categorised based on time of the accident (month, day of week, period during the day) and the place of the accident (per province).

Statistical information on road accidents is provided by AVV and CBS in a very detailed way and covers total number of accidents by different vehicles involved, severity of accidents, different regions, by month and day, victims by age, sex. On a more detailed level, however, it was not possible to gather this detailed information per severity degree of the accident per type of the vehicle involved in the accident and on different type of network.

Rail. Statistical information on accidents is available from Railned. The number of fatalities and injuries caused by rail accidents is provided for different types of victims: staff, passengers, people on level crossings, suicides and third parties. A distinction is made between deaths and injuries. No diversification between slight and severe injuries available.

It was not possible to get any information on rail accidents with material damages. This information is not included in the Railned’ database and is hold by individual carriers.

As opposed to the road sector, the problem of underreporting for railways is of minor relevance at least within the scope of accident cost estimation in this report. Therefore, the number of registered casualties is assumed to show the total.

Aviation. For the estimation of the number of victims and material damage involved in aviation accidents, the summary provided by NLR of the accidents which occurred in the respective years has been used. This information covers only accidents where injuries have taken place. Therefore, the information about material damages is not comprehensive because it does not cover accidents only with material damage leading to a possible underestimation of material damage. In the information provided by NLR balloons are not included.

IWW and Maritime. The waterborne accident database has been provided by AVV. It include very detailed information on accidents with different types of material damage in particular damage to vessels, damage to transported goods, damage to waterways and environmental damage.

The quality of the database depends on the quality of the records provided by the police. Although the database is very detailed, the recorded information is sometimes of poor quality, fact that led to an underestimate of the number with accidents with material damage. The reason for that is a number of accident records which did not contain any information about the presence of a particular damage.

Information about the deaths and injuries in waterborne accidents was not available for different types of parties involved. The total figure for injuries and deaths in IWW and maritime accidents have been used.

### Environmental costs

The input data for the estimation of the environmental costs was collected for specific type of environmental costs, namely:

- air pollution,

- global warming,

- noise,

- nature and landscape,

- nuclear risks.

One of the main input data sources for estimating costs of air pollution was the EcoSense database which was used within the impact pathway method. The basic data used within EcoSense is summarised as follows.

Table 21:	Environmental data in the EcoSense database

|  | Resolution | Source |
| --- | --- | --- |
| Receptor distribution |  |  |
| Population | administrative units, EMEP 50 grid | EUROSTAT REGIO Database, The Global Demography Project |
| Production of wheat, barley, sugar beat, potato, oats, rye, rice, tobacco, sunflower | administrative units, EMEP 50 grid | EUROSTAT REGIO Database, FAO Statistical Database |
| Inventory of natural stone, zinc, galvanized steel, mortar, rendering, paint | administrative units, EMEP 50 grid | Extrapolation based on inventories of some European cities |
| Critical Loads/Levels for nitrogen-deposition for various ecosystems | EMEP 150 grid | UN-ECE |
| Meteorological data |  |  |
| Wind speed | EMEP 50 grid | European Monitoring and Evaluation Programme (EMEP) |
| Wind direction | EMEP 50 grid | European Monitoring and Evaluation Programme (EMEP) |
| Precipitation | EMEP 50 grid | European Monitoring and Evaluation Programme (EMEP) |
| Emissions |  |  |
| SO2, NOx, NH3, NMVOC, particles | administrative units, EMEP 50 grid | CORINAIR 1994/1990, EMEP 1998 TNO particulate matter inventory (Berdowski et al., 1997) |
| Source: IER. |  |  |

Table 22:	Input data for air pollution

|  | Dissaggregation | Source and quality of the input data |
| --- | --- | --- |
| Emissions data | CO, NMVOC, NOx, PM2.5, SO2, CO2, N2O, CH2 | Good data on emissions available in the official statistics; published periodically. |
| Road | Infrastructure network: motorways, regional roads, urban roads Passenger transport: passenger car, motorcycles (petrol), scooters (petrol), busses Freight transport: LGV, HGV, tractors, special vehicles Type of fuel: petrol, LPG, diesel | Data on road emissions available in a very detailed form in the official statistics (CBS-statline). For the public transport only emissions for busses was available. No information available for trams and metros. |
| Rail | Passenger, freight | Total figures for rail emissions available in official statistics. No information available on emission per type of traction (diesel, electric). For passenger trains and freight trains input data from Statistics Netherlands |
| Aviation | Total | Only totals on emissions available in the official statistics; published periodically. |
| IWW | Passenger, freight, recreation, fishing | Good data on emissions available in the official statistics; published periodically. |
| Maritime | Total | Only totals on emissions available in the official statistics; published periodically. |
| Electricity production mix | Rail transport | No direct data on electricity production mix was available for railways. Therefore, the Dutch average production mix for transport sector is assumed (99% oil, 1% electricity) |
| Fuel use | Road: types of vehicles as above | Official statistics |

Table 23: Direct transport emissions in the Netherlands in million kg, 1998

|  | CO | CO2 | NOx | N2O | SO2 | NMVOC | CH2O | PM2.5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Road, total | 397.6 | 28338 | 180.4 | 5.25 | 4.8 | 107.08 | 4.44 | 9 |
| Passenger | 361.6 | 18591 | 96.1 | 3.62 | 1.9 | 94.78 | 3.93 | 4.1 |
| Freight | 34.8 | 9461 | 81.2 | 1.57 | 2.8 | 11.78 | 0.49 | 4.7 |
| Passenger cars, total | 319.7 | 17785 | 87.9 | 3.5 | 1.7 | 75.84 | 3.21 | 3.4 |
| -petrol | 300.4 | 12478 | 68.2 | 2.67 | 0.6 | 70.29 | 3.01 | 0.7 |
| -diesel | 9.1 | 3776 | 12.3 | 0.25 | 1.1 | 2.41 | 0.1 | 2.6 |
| -LPG | 10.3 | 1531 | 7.5 | 0.58 | 0 | 3.13 | 0.1 | 0.1 |
| Motorcycles, petrol | 29.3 | 213 | 0.4 | 0 | 0 | 8.69 | 0.35 | 0.2 |
| Bromcycles, petrol | 11.1 | 66 | 0.1 | 0 | 0 | 9.23 | 0.33 | 0 |
| Busses, total | 1.4 | 527 | 7.7 | 0.12 | 0.2 | 1.03 | 0.04 | 0.5 |
| -petrol | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| -diesel | 1.4 | 526 | 7.7 | 0.12 | 0.2 | 1.02 | 0.04 | 0.5 |
| -LPG | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| LGV, total | 23.9 | 3656 | 14.8 | 0.31 | 1 | 5.96 | 0.25 | 2.3 |
| -petrol | 13.1 | 282 | 2.6 | 0.04 | 0 | 2.71 | 0.12 | 0 |
| -diesel | 9.5 | 3279 | 11.3 | 0.23 | 1 | 2.77 | 0.12 | 2.2 |
| -LPG | 1.3 | 95 | 0.8 | 0.04 | 0 | 0.47 | 0.01 | 0 |
| HGV, total | 5.3 | 2940 | 31 | 0.67 | 0.9 | 2.79 | 0.12 | 1.4 |
| -petrol | 0.1 | 2 | 0 | 0 | 0 | 0.02 | 0 | 0 |
| -diesel | 5.2 | 2938 | 31 | 0.67 | 0.9 | 2.77 | 0.12 | 1.4 |
| -LPG | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| Agricultural vehicles, total | 5.6 | 2865 | 35.4 | 0.59 | 0.9 | 3.03 | 0.13 | 1.1 |
| -petrol | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| -diesel | 5.6 | 2865 | 35.4 | 0.59 | 0.9 | 3.03 | 0.13 | 1.1 |
| -LPG | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Special vehicles, total | 1.2 | 286 | 3.1 | 0.06 | 0.1 | 0.52 | 0.02 | 0.2 |
| -petrol | 0.5 | 7 | 0.1 | 0 | 0 | 0.1 | 0 | 0 |
| -diesel | 0.7 | 278 | 3.1 | 0.06 | 0.1 | 0.42 | 0.02 | 0.2 |
| -LPG | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Rail | 0.35 | 102 | 1.69 | 0.023 | 0.111 | 0.106 | 0.004 | 0.066 |
| Passenger trains | 0.25 | 52 | 0.59 | 0.012 | 0.057 | 0.084 | 0.003 | 0.05 |
| Freight trains | 0.1 | 50 | 1.1 | 0.011 | 0.054 | 0.022 | 0.001 | 0.016 |
| Air transport | 6 | 773 | 3 | 0.051 | 0.25 | 1.4 | 0.11 | 0.17 |
| IWW | 9.299 | 2131 | 39.88 | 0.476 | 2.234 | 3.659 | 0.161 | 2.802 |
| Passenger vessels | 0.36 | 113 | 1.8 | 0.025 | 0.12 | 0.22 | 0.009 | 0.22 |
| Freight vessels | 1.8 | 1869 | 36 | 0.41 | 2 | 1.8 | 0.072 | 2.4 |
| Recreation vessels | 7.1 | 108 | 1.3 | 0.032 | 0.07 | 1.6 | 0.078 | 0.13 |
| Fishing vessels | 0.039 | 41 | 0.78 | 0.009 | 0.044 | 0.039 | 0.002 | 0.052 |
| Maritime | 2.8 | 1119 | 22 | 0.25 | 13 | 0.95 | 0.038 | 1.7 |
| Source: CBS statline and Statistics Netherlands |  |  |  |  |  |  |  |  |

#### Global warming

The input data for the calculation of the costs of CO2 are based directly on the level of CO2 emission given in the previous section for all modes of transport. The monetary values used for cost calculation are described in chapter 3.

#### Noise

The quality of data offered by official statistics on noise exposure is rather low. The available information regards the percentage of Dutch population exposed to noise from road, rail and air transport. This information is presented in total rather than for different noise levels.

Information including a differentiation of the noise exposure per levels in 1999 was available in CE (1999). The same shares have been applied for the total figures available for 1996 and 1998. Table 24 shows the estimated number of people exposed to different levels of noise.

Table 24: Persons exposed to different transport noise levels, thousands 1998

|  | Road | Rail | Air |
| --- | --- | --- | --- |
| 55-60 dB | 2759.8 | 578.5 | 1,735.0 |
| 60-65 dB | 1298.7 | 119.1 | 991.4 |
| 65-70 dB | 216.5 | 42.5 | 165.2 |
| 70-75 dB | 81.2 | 25.5 | 55.1 |
| >75 dB | 27.1 | 17.0 | 27.5 |
| Total | 4383.2 | 782.7 | 2974.3 |
| Source: NEI based on CE (1999) and CBS |  |  |  |

A further disaggregation of the exposure by night or day, passenger or goods transport, vehicles type, or network type was not possible.

#### Nature and landscape

The basic input data required for the calculation of the nature and landscape effects such as soil and water pollution relates to the type and length characteristics of the infrastructure, of which sealed and impaired areas derive. Table 25 summarises the input data and its sources.

Table 25:	Input data for nature and landscape environmental costs

|  | Level of disaggregation | Quality of data, level of uncertainty |
| --- | --- | --- |
| Road |  |  |
| Network data (length) | Motorways, trunk roads | Data on roads length taken from CBS statline |
| Traffic performance (vkm) | Motorways, trunk roads Passenger cars, motorcycles, busses, LGV, HGV, special vehicles, agricultural vehicles | Good quality data available from official statistics (CBS-statline) |
| Rail |  |  |
| Network data (length) | Double track, single track | Official statistics for totals and double (multiple) track; length of the single track estimated |
| Traffic performance (train-km) | Passenger, freight | NEI estimation on passenger train-km based on passenger-km, freight train-km NEI estimation |
| Waterborne |  |  |
| Network data (size, length) | Channels and rivers | CBS statline |
| Traffic performance | Total vkm IWW freight | Official statistics |
| General cost values | Habitat loss/barrier effect, unsealing, soil decontamination, ground water |  |

Due to the lack of reliable network data nature and landscape effects for aviation and ports were not calculated

The following years have been taken as the reference year for the comparison of the network data: motorways – 1950 and trunk roads - 1983; rail – 1980, IWW - 1971. The input for network data is presented in tables 26.

Table 26: 	Network data for Dutch transport infrastructure

|  | Reference year | 1996 | 1998 |
| --- | --- | --- | --- |
| Length by road class and width, km |  |  |  |
| Motorways (>20m) | 121 | 2207 | 2250 |
| Motorways (<20m) | 551 | 936 | 985 |
| Trunk roads (>20m) | 7130 | 6360 | 6314 |
| Trunk roads (<20m) | 42460 | 48700 | 49300 |
| Length of rail network, km |  |  |  |
| Two or more tracks | 1595 | 1853 | 1877 |
| Single track | 1146 | 960 | 931 |
| Length of waterways, km |  |  |  |
| Rivers | 4028 | 3745 | 3745 |
| Channels | 1092 | 841 | 841 |
| Source: NEI based on CBS statline |  |  |  |

Table 27:	Infrastructure characteristics, 1998

|  | Original width m | Compens. factor % | Sealing factor % | Additional impaired width m | Lost habitats ha/km | Sealed area ha/km | Contamin. area ha/km |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Road |  |  |  |  |  |  |  |
| Motorway, >20 m | 29 | 100 | 100 | 10 | 4.9 | 3.9 | 1.0 |
| Motorway, <20 m | 9 | 100 | 100 | 10 | 2.9 | 1.9 | 1.0 |
| Trunk road >9 m | 15 | 100 | 100 | 8 | 3.3 | 2.5 | 0.8 |
| Trunk road <9 m | 7 | 100 | 100 | 8 | 2.5 | 1.7 | 0.8 |
| Rail |  |  |  |  |  |  |  |
| Double track | 15 | 100 | 50 | 6 | 3.1 | 1.8 | 0.6 |
| Single track | 6 | 100 | 50 | 6 | 2.2 | 1.3 | 0.6 |
| IWW |  |  |  |  |  |  |  |
| Channels | 50 | 70 | 0 | 0 | 4.5 | 1.0 | 0.0 |
| Rivers | 200 | 0 | 0 | 0 | 1.0 | 1.0 | 0.0 |
| Source: NEI |  |  |  |  |  |  |  |

#### Nuclear risk

No mode specific information was available about the production of electricity in nuclear power stations. For the general transport sector the supply of power from nuclear energy is nihil. Therefore, the nuclear risk due to the production of electricity in nuclear power stations is assumed to be negligible and is not considered in the pilot account.

### Taxes, charges, subsidies

Table 28 gives a descriptive overview of the input data used for the calculation of the taxes, charges and subsidies. The data was collected from Statistics Netherlands, business account of transport companies, and the Dutch Ministry of Finance.

Table 28: Input data and data quality for the calculation of taxes, charges and subsidies

| Input data | Level of disaggregation | Quality of data |
| --- | --- | --- |
| Road | Type of vehicle: motorcycles, passenger cars, buses, coaches, lorries, trailers, special vehicles By type of charge/tax | National accounts |
| Rail | Charges: passenger, freight Subsidies | Business accounts of NS |
| Other public transport | Revenues Subsidies | Business accounts, generally of poor information; no disaggregation per type of activity or revenue. |
| Aviation | Airport tax, tax on noise | Business accounts of airports (Amsterdam Schiphol and Maastricht-Aachen), official statistics on noise tax at airports |
| Inland waterway shipping | Subsidies | An indication of total subsidies only available. For the other revenues, poor quality and lack of information |
| Maritime shipping | Charges for infrastructure use: seaports, inland ports | Business account of Rotterdam and Amsterdam, which are representative for the Netherlands. |

For road, rail and air transport the data quality is considered to be good although it is rather poor for the proposed disaggregation in UNITE. In addition, due to the lack of relevant information and in order to avoid arbitrary allocation, the figures for the all modes but road are presented as total. For the remaining modes various estimations had to be made leading to rough results. Therefore, one should be careful when interpreting the presented results.

# Methodological issues

The methodology used in developing the UNITE pilot accounts has been documented in the publication “D2 - The Accounts Approach” by Link et al. (2000 b). In this annex report on the German pilot accounts we will only summarise the methodology as far as it is necessary to understand and interpret the accounting results. We will focus on new methodology or deviations from the general methodology developed in Link et al. (2000 b) and on the methods used to compile the results for 1996 and 2005.

## Methodology for estimating infrastructure costs

The methodology used in developing the UNITE pilot accounts has been documented in the publication “D2 - The Accounts Approach” by Link, Stewart, Maibach et al. 2000. In this annex report on the Dutch pilot accounts we will only summarise the methodology as far as it is necessary to understand and interpret the accounting results. We will focus on new methodology or deviations from the general methodology developed in Link, Stewart, Maibach et al. 2000.

Infrastructure costs contain capital costs (depreciation and interests) for new investments and for replacement of assets on the one hand and running costs for maintenance, operation and administration/ overheads on the other hand. The basis for estimating capital costs is the value of the capital stock.

For the Dutch pilot accounts the perpetual inventory method (see box 1 for a summary description) was applied for all modes, with underlying long time series for the different modes. For the life expectancies standard values are used. It was not possible to collect disaggregated data for the entire time series. It was also not possible to achieve a complete separation between transport related parts of airports and harbours and non-transport related parts.

For roads and inland waterways, statistical data from the CBS could be used to estimate the running costs. For rail the annual reports of the Railway Infrastructure Administration (RIB) were used to estimate the running costs and for public transport statistical data from the CBS was combined with some specific annual reports of public transport companies. For airports the annual reports of the largest airports were used and the figures were extrapolated to a national level. The method for estimating missing values is described in the next chapter.

Box 1.	The perpetual inventory model used for the Dutch pilot accounts

The main idea of the perpetual inventory concept, a concept which is used by most OECD-countries for estimating the capital stock of industrial branches, is to capitalises time series of annual investment expenditures by cumulating the annual investments and by subtracting the value of those assets which exceeded their life-expectancy (written down assets) as expressed in the equations below: VG t+1 = VG t + It,t+1 - At,t+1 (1) VN t+1 = VN t + It,t+1 - Dt,t+1 (2) with: VG t : Gross value of assets at time t VN t : Net value of assets at time t It,t+1 : Investments during t, t+1 At,t+1 : Written down assets during t, t+1 (assets which exceeded life-expectancy) Dt,t+1 : Depreciation during t, t+1 As shown in these formulas the perpetual inventory method can be applied for estimating the gross value (gross concept) and the net value (net concept) of infrastructure assets. The gross value contains the value of all assets which still exist physically in the considered year, e.g. which have not yet exceeded their life expectancy. Thus, At,t+1 denotes those assets which could not be used any longer or which were shut down. It is assumed that the assets are properly maintained and can be used until they exceed their defined life expectancy. Within the net-concept the annual depreciation Dt,t+1 are considered. The net value of assets describes the time-value of all assets which have not yet exceeded life-expectancy. According to the international conventions of the SNA, most countries use a linear depreciation method. The general principle as described above can be refined by more sophisticated approaches which use probability functions for the written down assets. This type of perpetual inventory model was used for the German pilot accounts. In contrast to simple perpetual inventory models, the refined models assume that the life expectancies of assets within an investment vintage are dispersed over the mean value. A probability function, the so-called survival function, is estimated, which describes the share of assets which are still in use. The inverse function which describes the written down assets At,t+1 was estimated as a polynomial of the third degree in Germany meaning that the probability function of the written-down assets has a right-skewed shape. This approach considers the fact that the investment spent for an asset group consists of parts with different life expectancies which are dispersed within an interval around the mean. Although also in the German method for all elements of the investment I1 - In a linear depreciation is applied, the overall asset group shows in fact a regressive depreciation due to the underlying type of probability function for the written-down assets. The perpetual inventory model requires in general long time series on annual investment expenditures, information on life expectancies of assets, and initial values of the capital stock (except the investment time series is as long as the life expectancy). Due to the fact that the use of probability functions in the refined concept implies that not single assets but technically homogeneous groups of assets (earthworks, bridges/tunnels, terminal buildings, pavement and equipment) are considered, investment time series for asset groups (for example pavement, tunnels/bridges, equipment) have to be available.

All results are values without VAT which was eliminated both from the depreciations and from the running costs.

As far as the UNITE accounting years 1996 and 2005 are concerned the general approach was to carry out separate model runs with the perpetual inventory model to calculate capital stocks and derive capital costs. For 2005 investment paths were not extrapolated. In future they could be extrapolated by using existing planning or plausible past trends.

## Methodology for estimating supplier operating costs

For the UNITE pilot accounts it was decided to calculate supplier operating costs only for transport modes where the revenues from the transport users do not cover the costs of the supplier. This is mainly true for public transport and rail transport and is considered to be core data for these transport modes.

The ideal methodology proposed for supplier operating costs within the Pilot Accounts entries is the disaggregated cost model, as defined by Allport 1981. The chief advantages of this method are its comprehensiveness and ability to be clearly understood in relation to the inclusion of all relevant cost categories that make up the total costs of public transport services (excluding external costs like the Mohring effect, environmental and accident costs). Since no universal procedure can be established for the different operational cost components, consequently a step-by-step procedure has to be established for each component of which the generalised steps are:

- Disaggregation of costs into fixed and variables

- Confirmation of temporal escapability for each cost component

- Data collection;

- Assessment of estimation needs and respective computation for lacking data

- Cost calculation

The evaluation of supplier operating costs using the Allport method has proven itself to be valuable for determining the costs of single companies. The use of the method to evaluate a transport mode in total is, however, unrealistic and was not attempted. As far as possible the categories suggested in the Allport method (materials, goods and services, personnel, depreciation, other running costs and interest) were used.

## Methodology for estimating delay costs due to congestion

The estimation of the delay costs followed the methodology outlined in Link et al. (2000). The UNITE methodology defines congestion costs as the sum of those time and operating costs perceived by transport users which exceed average time and operating costs. Users are defined as the users of traffic infrastructure in individual and private commercial motorised road traffic (including passengers and drivers of cars and motorcycles and road haulers) and of passengers and shippers (represented by units of cargo) in public passenger and freight transport. Congested traffic conditions or late arrivals are defined per mode, taking into consideration characteristic fluctuations in travel time and the system-specific consequences of delays. In general the UNITE approach values late arrivals rather than late departures or longer in-vehicle travel times in public transport. Between all road-related transport modes, which include individual motorised traffic, bus, coach and tramway services and road haulage, interdependencies are to be considered and thus a common road model is applied to quantify delays.

Time and operating costs spent under delayed or congested conditions are estimated by using normal or acceptable travel times and operating costs in order to obtain a value of extra time and other resources lost by the users. For all road modes, acceptable traffic conditions are defined by off-peak travel speeds and the related operating costs, while for rail and air traffic scheduled travel times are used. The valuation of delays or extra travel time costs is restricted to serious delays. Small delays or simply disturbed traffic are considered to be normal attributes of traffic systems.

Delay cost information does not form part of the core data in the UNITE core section of the accounts. It relates to costs that are internal to transport users as a group, and is therefore classed as supplementary data only.

The estimation of delay costs for 1996 followed the same principles as for 1998. VOT values were adjusted following the common rules laid down in Nellthrop et al. (2001). Most of the information which was available for 1998 was available for 1996, too. A number of assumptions were made. It is assumed that the increase in traffic demand and the extension of the roads capacity equal out, thus the share of congested traffic in total road traffic for 1996 equals that in 1998. Average delay in air transport for 1998 was not available; figures for 1997 were applied.

## Methodology for estimating accident costs

The methodology applied for the calculation of accident costs followed the recommendations of Interim Report 8.2 “Accounts Approach for Accidents” of the UNITE project (Doll et al. 2000). The definition of cost categories is broadly in line with the SWOV approach for the estimation of costs as result of road passenger accidents (SWOV, 1993,1995). While the latter includes more medical and administrative cost elements, the former was extended to other vehicles and other modes. Because of the lack of available and reliable information, the presentation of a matrix of cost responsibility and cost bearers as proposed in Doll et al. (2000) respectively could not be presented.

Evaluation of accident costs is based on the calculation of the following cost categories: materials damage, administration costs, medical costs, production losses and the valuation of the risk associated with using transportation. Each of these subcategories is valued through the use of the number of incidents and the costs arising from the incident.

In the estimation of accident costs the distinction between internal and external costs is of great importance. This distinction has a great influence on the total level of the external accident costs. Within UNITE pilot accounts internal accident costs are those internal to the transport system, and external costs are those which transport accidents impose on the rest of the society. In the pilot account internal costs embrace all costs borne by the individual transport users that is damages to property not covered by insurance companies and the risk associated with using transport, costs borne by the community of transport users (including all costs covered by traffic insurance companies). Explicitly external costs are administrative costs for police or the legal system, the costs of medical treatment not covered by traffic insurance companies and production losses.

Due to the lack of information it was not possible to divide medical costs into internal and external costs, and thus in a simplified approach this cost component was considered to be totally external. The remaining internal costs therefore comprise only of the costs of material damages and the risk value. Risk value is considered to be internal for the purpose of UNITE. This means that we implicitly assume that accident risks are fully anticipated by individuals when they decide to take part in transport. External accident costs are considered to be core data while internal accident costs, because the costs are borne by the transport users and not society as a whole, are considered to be additional information only.

The numbers and costs from materials damage, administration and medical subcategories were respectively obtained from accident statistics and partly from insurance companies. Production losses represent an estimation of the losses to the national economy due to replacement costs, lost output of employed persons and lost non-market production (e.g. domestic work) resulting from accidents. The emphasis within this cost category was placed on medical costs and the cost arising from transport related fatalities. All valuations are documented in the publication “Valuation Conventions for UNITE” Nellthorp et al. (2001).

- a)	The costs of medical treatment

The costs of medical treatment of traffic casualties can be broken down into a number of different activities as shown in table 29. Injuries are divided into two categories, with a steady reduction of working power (SRWP) and without SRWP (see Doll et al. 2000). The share of injuries entailing a steady reduction of working power (SRWP-cases) were based on the results of a survey among the accident victims (SCV, 1997/1998) with 3% for slight injuries (corresponding to injuries without medical assistance) and 10% for severe injuries (corresponding to injuries with medical assistance) in road transport. These figures were used for injuries in all modes.

Reliable information on the coverage of costs for the medical system by transport users’ insurance companies could not be retrieved. Therefore, all costs related to the medical treatment of accident casualties were considered as external to the transport sector.

Table 29: 	Average costs for medical treatment (€ per casualty), 1998

| Type of action | Slight injuries |  | Severe injuries |  | Deaths |
| --- | --- | --- | --- | --- | --- |
|  | with SRWP | No SRWP | with SRWP | No SRWP |  |
| Stationary treatment | 0 | 0 | 16 857.09 | 3 574.91 | 687.48 |
| Ambulant treatment | 797.48 | 192.50 | 962.48 | 329.99 | 55.00 |
| Transport | 55.00 | 55.00 | 879.98 | 247.49 | 412.49 |
| Follow-up treatment | 110.00 | 27.50 | 467.49 | 55.00 | 0 |
| Aids | 0 | 0 | 1 292.47 | 110.00 | 0 |
| Supporting measures | 2 639.94 | 0 | 2 639.94 | 0 | 0 |
| Rehabilitation | 0 | 0 | 687.48 | 27.50 | 0 |
| Nursing | 0 | 0 | 522.49 | 55.00 | 0 |
| TOTAL per casualty | 3 602.41 | 274.99 | 24 309.40 | 4 399.89 | 1 154.97 |
| Source: NEI using values from DIW with appropriate benefit transfers SRWP – Steady reduction of working power |  |  |  |  |  |

- b)	Production losses

According to the methodology described in Doll et al. (2000) the cost category “Production Loss” comprises two elements:

- The loss of the production power of steadily disabled or traffic fatalities.

- The temporary costs for the victim’s employer.

The lost production time per victim takes into consideration the duration of various medical actions and the duration of partial disability preventing the victim taking part in the production process. The effective loss of productive time further considers the degree of disability to work (25% for SRWP-cases and all severe injuries) and the share of victims of employable age (BMVBW/DIW 2000). The respective input data is given in table 30.

Table 30:	Composition of the lost working time per degree of injury, 1998

| Category of treatment | Slight injuries |  | Severe injuries |  | Deaths |
| --- | --- | --- | --- | --- | --- |
|  | with SRWP | No SRWP | with SRWP | No SRWP |  |
| Stationary treatment (days) | . | . | 17 | 65 | . |
| Rehabilitation time (days) | . | . | . | 6 | . |
| Nursing (days) | . | . | 2 | 6 | . |
| Disability to work (days) | 17 | 79 | 64 | 224 | . |
| Duration of temporal reduction of working power (days) | . | 294 | 294 | 7 448 | 7 448 |
| Degree of reduction of working power (%) | . | 25% | 25% | 25% | 100% |
| Share of victims in employable age | 70.9% | 70.9% | 70.7% | 70.7% | 71.3% |
| Employment rate | 96.0% | 96.0% | 96.0% | 96.0% | 96.0% |
| Net value factor | 0 | 0.00067 | 0.00067 | 0.00067 | 0.00270 |
| Lost working time (years) | 0.032 | 0.284 | 0.291 | 4.022 | 13.962 |
| Source: NEI using values from DIW (2001) and CBS |  |  |  |  |  |

The gross production loss per lost year of working time refers to the production potential of the national economy rather than to the actual GNP. Thus the gross production loss is composed of the GNP per capita in employable age (€ 16 951) and the relation between GNP and the production potential (1.04 according to the Cochrane-Orcutt production function). In order to avoid double counting with the Risk Value the future consumption (€ 13 828 per capita and year) was subtracted from the gross production potential. The resulting net production potential then was discounted to 1998 using a social interest rate of 3%.

For direct replacement costs a value of € 3 155 per fatality or severe injury has been used based on DIW (2001) applying benefit transfer techniques.

c)	Valuation of administrative costs

Administrative costs are composed of the costs for police, justice and for the insurance sector. No reliable information could be retrieved for the time required by police, legal system and insurance for dealing with a traffic accident. Estimated administrative costs include only costs of police. In this case the data for time required by police in Germany (DIW, 2001) have been applied for the Netherlands with an average hourly wage rate for police officers of € 18.25, which in fact represents the average wage in the public administration and insurance sector as reported by CBS.

Further, it is assumed that the time required by police to deal with casualties in other modes is similar to the road case that is 1.97, 12.79 and12.79 hours to administer respectively slight injuries, severe injuries and fatalities. This assumption cannot be made in the case of accidents with material damages due to their different essence and extent. Consequently, the administrative costs of accidents with material damages have been calculated only for road accidents.

The administrative costs for traffic police, as considered here, are totally external to the transport sector as they are covered by the general budget and are the only administrative costs evaluated.

- d)	Valuation of material damages

No direct information on the average costs of accidents for any mode could be found. Average damage costs of road vehicles were estimated based on DIW (2001) with appropriate techniques for value transfer. Average material damages for other modes or other types of public or private property were not available, therefore the respective costs could not be estimated.

In addition, no reliable information could be found on the average value of deductibles borne by the accident parties. For accidents of passenger cars and motorcycles internal estimates of € 198 per accident were applied, while for other vehicles no deductibles were assumed. No estimates of average unit costs was attempted for accidents not reported to insurance companies.

Table 31:	Average unit costs per material damage of road accidents, € 1998

| Mode of transport & damage category | Unit costs per case reported to liability insurance |  | Insurance coverage per case reported |  |
| --- | --- | --- | --- | --- |
|  | Slight | Severe | Slight | Severe |
| Damage to vehicles |  |  |  |  |
| Passenger car | 1 397 | 12 759 | 1 198 | 12 561 |
| Motorcycle | 898 | 8 204 | 700 | 8 006 |
| Bus / Coach | 781 | 7 132 | 781 | 7 132 |
| Tramway | 768 | 7 016 | 768 | 7 016 |
| Goods vehicles | 781 | 7 132 | 781 | 7 132 |
| Others | 768 | 7 016 | 768 | 7 016 |
| Public property | : | : | : | : |
| Other private property | : | : | : | : |
| Source: NEI For passenger cars calculation are based on the average amount per claim for material damages of passenger cars € 1198 as reported by Centre for Insurance Statistics (CVS) and the assumption of own coverage of € 198 (SWOV, 1993). Average costs for other vehicles were estimated based on DIW information. |  |  |  |  |

Due to the non-availability of data estimating the costs of damage to public or non-transport-related private property, all costs of material damages are considered to be totally internal to the transport sector.

- e)	The risk value

The Risk Value was set according to the recommendations of the UNITE valuation conventions:

- € 1401484 million for fatalities,

- 15% of € 1.4 million = € 210223 for severe injuries.

- 1% of the value of statistical life = € 14015 for slight injuries.

Risk values for relatives and friends were not considered. For the UNITE accounts, risk value is considered to be fully internal.

Backcast methodology. The accident cost accounts 1996 follow the same methodology as for 1998. The number of physical units is available from the same source. Cost units have been used as for the core year 1998, only the benefit transfer rules given in Nellthorp et al. 2001 had to be applied.

## Methodology for estimating environmental costs

This chapter text provides a brief overview of the methodology used for the calculation of environmental costs. For a detailed description of the approach for the estimation of costs due to air pollution, global warming and noise the reader is referred to Bickel et al. (2001).

### Air pollution

For quantifying the costs due to airborne pollutants the Impact Pathway Approach, the methodology developed in the ExternE project series was applied. A detailed description of the approach can be found in European Commission (1999a). The impact pathway approach utilises the following steps: emission estimation, dispersion and chemical conversion modelling, calculation of physical impacts and monetary valuation of these impacts.

The ideal approach, which was applied in Tranche A countries (Germany and Switzerland), is to use emission inventories in spatial disaggregation (i.e. a geo-coded data set for the different air pollutants) for the calculation of the costs of direct emissions from vehicle operation. As such detailed data was not available for Tranche B (Netherlands being one of the countries), a simplified approach was used. Country-specific damage costs per tonne of pollutant emitted were calculated based on the emission inventory included in EcoSense, which contains information on the spatial distribution of emissions. For this, emissions were modified compared to the reference inventory and Europe-wide impacts were calculated and subtracted from impacts resulting from the reference inventory without unchanged emissions. This procedure using a reference inventory was required, because of air chemistry processes where “background” emissions play an important role. Then the resulting costs were divided by the difference of emissions to obtain the costs per tonne of pollutant. A description of the computer model EcoSense, which was used for the calculations, including exposure-response functions and monetary values is given below.

In addition to these regional scale calculations, damages on the local scale – up to about 20 km to each side of a line emission source (e.g. road) – were quantified. In lack of detailed geo-coded emission data, specific local-scale costs for the categories “urban roads”, “extra-urban roads” and “motorways” were transferred from detailed calculations for Germany in Tranche A.

Then the emissions provided by the official emissions statistics for the Netherlands were multiplied with the respective damage factor to obtain the costs caused by the different modes and vehicle categories.

Note: primary particle emissions from internal combustion engines were treated as PM2.5, primary particles from fossil power plants were treated as PM10 (higher deposition rate and lower impact compared to PM2.5).

Monetary values. According to Nellthorp et al. (2001) average European values should be used for transboundary air pollution costs, except for the source country, where country specific values were used. These were calculated according to the benefit transfer rules given in Nellthorp et al. (2001). The values used for the Netherlands are given in the table 32.

Table 32:	Monetary values (factor costs) for health impacts due to airborne pollutants (€1998)

| Impact |  | Avg. Europe | Netherlands |
| --- | --- | --- | --- |
| Year of life lost (chronic effects) | per YOLL | 74 700 | 84 600 |
| Year of life lost (acute effects) | per YOLL | 128 500 | 145 500 |
| Chronic bronchitis | per new case | 137 600 | 155 800 |
| Cerebrovascular hospital admission | per case | 13 900 | 15 740 |
| Respiratory hospital admission | per case | 3 610 | 4 090 |
| Congestive heart failure | per case | 2 730 | 3 090 |
| Chronic cough in children | per episode | 200 | 220 |
| Restricted activity day | per day | 100 | 110 |
| Asthma attack | per day | 69 | 78 |
| Cough | per day | 34 | 38 |
| Minor restricted activity day | per day | 34 | 38 |
| Symptom day | per day | 34 | 38 |
| Bronchodilator usage | per day | 32 | 36 |
| Lower respiratory symptom | per day | 7 | 8 |
| Source: Bickel et al. (2001) |  |  |  |

### Global warming

The method of calculating costs of CO2 emissions basically consists of multiplying the amount of CO2 emitted by a cost factor. Due to the global scale of the damage caused, there is no difference how and where the emissions take place.

A shadow value of € 20 per tonne of CO2 emitted was used for valuing CO2 emissions, which reflects the costs of meeting the Kyoto targets in Germany (Fahl et. al. 1999) and Belgium (Duerinck 2000). This value lies within a range of values of € 5 to € 38 per tonne of CO2 avoided presented by Capros and Mantzos (2000). These authors calculated shadow prices for the EU to meet the Kyoto targets with and without emission trading.

Looking further into the future, more stringent reductions than the Kyoto aims are assumed to be necessary to reach sustainability. Based on a reduction target of 50% in 2030 compared to 1990, INFRAS/IWW (2000) use avoidance costs of € 135 per t of CO2; however one could argue that this reduction target has not yet been accepted.

A valuation based on the damage cost approach, as e.g. presented by ExternE (Friedrich and Bickel 2001), would result in substantially lower costs. Due to the enormous uncertainties involved in the estimation process, such values have to be used very cautiously.

For those country, where emissions of methane (CH4) and nitrous oxide (N2O) were available, the shadow value for CO2 was multiplied by the global warming potential of 21 and 310 respectively, leading to values of 420 €/t CH4 and 6 200 €/t N2O.

### Noise

Noise costs were quantified for a number of health impacts calculated with new exposure-response functions, plus amenity losses estimated by hedonic pricing.

The methodology for quantifying noise costs was extended to the calculation of physical impacts. Costs for the following endpoints were quantified:

- Myocardial infarction (fatal, non-fatal)

- Angina pectoris

- Hypertension

- Subjective sleep quality

In addition, the willingness-to-pay for avoiding amenity losses were quantified based on hedonic pricing studies. A large number of such studies has been conducted, giving NSDI values (Noise Sensitivity Depreciation Index – the value of the percentage change in the logarithm of house price arising from a unit increase in noise) ranging from 0.08% to 2.22% for road traffic noise. Soguel (1994) conducted a hedonic pricing study in the town of Neuchatel in Switzerland. Rather than using housing prices, the dependent variable was monthly rent, net of charges. The coefficient on the noise variable in this study suggested a NSDI of 0.9. This value is similar to the average derived from European studies and was taken for our calculations.

As railway noise is perceived as less annoying than road noise, a bonus of 5 dB(A) was applied. This is in line with noise regulations in a number of European countries (e.g. Switzerland, France, Denmark, Germany; see INFRAS/IWW 2000).

For the quantification of the WTP for avoiding amenity losses a threshold value of 55 dB(A) was applied. It is assumed that noise levels equal to and over this value cause disamenity. The average rent which was the basis for the calculations is given in the table below.

The following table presents the monetary values used for valuing the health effects.

Table 33: 	Monetary values (factor costs) for impacts due to noise, Euro 1998

| Impact | Avg. Europe | Netherlands |
| --- | --- | --- |
| Myocardial infarction (fatal, 7 YOLL) |  |  |
| Total per case | 522 900 | 592 000 |
| Myocardial infarction (non-fatal, 8 days in hospital, 24 days at home) |  |  |
| Medical costs | 4 720 | 5 680 |
| Absentee costs | 2 820 | 3 140 |
| WTP | 15 070 | 17 630 |
| Total per case | 22 600 | 26 450 |
| Angina pectoris (severe, non-fatal, 5 days in hospital, 15 days at home) |  |  |
| Medical costs | 2 960 | 3 560 |
| Absentee costs | 1 760 | 1 960 |
| WTP | 9 440 | 11 040 |
| Total per case | 14 160 | 16 560 |
| Hypertension (hospital treatment, 6 days in hospital, 12 days at home) |  |  |
| Medical costs | 1 830 | 2 210 |
| Absentee costs | 1 580 | 1 760 |
| WTP | 550 | 620 |
| Total per case | 3 960 | 4 590 |
| Medical costs due to sleep disturbance (per year) | 197 | 223 |
| Average (net) rent per person per year (basis of calculation of WTP for avoiding amenity losses) |  | 1 285 |
| Source: Bickel et al. (2001) |  |  |

### Nature and landscape

The methodology applied to determine the annual costs of the year of investigation (here 1998) follows the approach taken by INFRAS/IWW (2000). According to this methodological approach, the costs of nature and landscape are defined as the share of the accounting period at the total loss of ecological resources caused by the construction of transport infrastructure from a defined base year until the year of accounting.

The damages to nature and landscape are monetarised by estimating the costs of compensating nature for the land taken by new infrastructure. This includes the “installation” of new biotopes were natural areas are destroyed, the remediation of soil and groundwater and the alternative unsealing of sealed ground. The result of this approach was total compensation or repair costs discounted to the year of accounting.

In contrast to the cost categories accidents and environmental health, in the cost category nature and landscape we do not explicitly consider the infrastructure built in the accounting year. In order to express the long-term impacts of consuming natural resources we consider the development of ecology from a state were nature is considered to be more or less intact.

The costs for the reporting period are determined by subdividing the total costs after the reference year by the respective number of years. We do not apply a discount rate on past costs caused to changes of nature and landscape, damages in the future are valued as high as damages caused today.

We assume further that the average damage to natural resources caused by the installation of infrastructure projects did not differ in general and that accordingly the average costs per additional square metre of transport assets constructed is equal over time. Transport infrastructure was considered separately for each mode.

The estimation of costs associated with repair and compensation measures is a very complex task, because the growth of alternative biotopes e.g. takes a long time. In this field no time series of respective cost estimates, which would allow to determine the development of the scarcity of nature are available. Further, the minor importance of the costs associated with nature and landscape effects in comparison to other cost blocks does not justify evaluate the total project installation reports of the infrastructure constructed in the period between the reference year and accounting year. Accordingly, it is assumed that the negative scarcity effect and the positive influence of improved construction practices are balancing out and that the costs of the accounting period is determined as the total costs divided by the number of years since the reference year.

The cost values used are characterised by the cost category, the type of area affected and the type of infrastructure. These elements are briefly discussed below.

Due to the similarity of the costs of nature and landscape and the costs of soil and water pollution they were commonly treated and estimated in the UNITE accounts (see Bickel et al. 2000). The cost categories considered under the joint cost item “Nature, landscape, soil and groundwater effects” are:

- Habitat losses and deterioration of biodiversity. Included are the loss of natural habitats and barrier effects caused by the existence of transport infrastructure. For the monetarisation a compensation costs approach is used, which estimates the costs for establishing new natural areas of the same type destroyed somewhere else.

- Sealing effects: Starting from the idea that every newly sealed area has to be unsealed at another location, sealing effects are valued by a compensation cost approach, estimating the de-sealing costs per square metre of directly covered ground.

- Soil and groundwater contamination: The sources for soil and groundwater contamination are manifold (see Bickel et. al. 2000). Here, the single pollutants are considered jointly by applying a decontamination cost value per m3. Starting from a constant depth of pollution (20 cm), out of this a value per m2 of impaired land can be computed.

- Groundwater pollution and winter maintenance: These two effects are summarised together as (1) they are both hardly quantifiable and as (2) the use of de-icing salts for winter maintenance directly impacts the quality of groundwater.

Table 34 summarises the cost values applied by cost type and type of biotopes. In case of sealed and impaired soil, the costs for the reporting year are determined by subdividing the total costs since the reference year by the respective number of years. We do not apply a discount rate on past costs caused to nature and landscape because no average life span of repaired soil and therefore no amortisation costs can be assumed. Further reasons against discounting rates are given in the German case study. Yet a PPP-Adjustment is necessary because the original repair cost figures for unsealing and soil decontamination are based on original German-specific figures.

Table 34:	Valuation of effects on nature and landscape

| Nomenclature according to the CORINE land use data set | Habitat loss | Ground sealing | Soil & water | Other effects |
| --- | --- | --- | --- | --- |
| Affected area: Description | Sealed + Impaired | Sealed | Impaired | Sealed |
|  | Euro/m2 | Euro/m2 | Euro/m2 | Euro / m2 |
| Artificial surfaces |  | 26.9 | 7.6 | 10 |
| Agricultural areas | 7.7 | 26.9 | 7.6 | 10 |
| Forests and semi-natural areas | 42.0 | 26.9 | 7.6 | 10 |
| Wetlands | 49.2 | 26.9 | 7.6 | 10 |
| Water bodies | 54.2 | 26.9 | 7.6 | 10 |
| Average | 10.7 |  |  |  |
| Source: NEI based on DIW. |  |  |  |  |

- Habitat loss and biodiversity. For the estimation of the economic costs due to the loss or deterioration of natural habitats a compensation cost approach according to IWW et. al (1998) was applied. The costs associated are:

- Annual costs for building up an alternative habitat (€ 300-400), discounted over the time the biotope requires to grow (up to 50 years).

- Opportunity costs of the land used for the compensation habitat (up to € 700 /a) discounted over the period of lost use (150 years).

- Initial costs for establishing the compensation habitat (up to € 90 000).

- Costs of purchasing the area of the alternative habitat (€ 60 000).

For the Dutch pilot account the following average values for habitat loss per m2 were used: agricultural areas € 7.7, forests and semi-natural areas € 42, wetlands € 49.2, and water bodies € 54.2.

- Unsealing costs. The costs of unsealing ground covered by solid transport infrastructure contain the costs for transport and deposit of materials sealing ground elsewhere. According to INFRAS/IWW (2000) cost estimates range between € 13 and 42 per m2. Here, a mean value of € 26.9 per square metre was applied.

Unsealing costs are applied to the sealed area associated to transport infrastructure whereby sealing factors were considered.

- Soil and groundwater pollution. Ground material contaminated by transport activities along infrastructure assets has to be carried off and deposited. Therefore, the costs applied per m2 need to take into consideration the treatment costs per m3 of ground and the depth of contamination. In both cases we followed INFRAS/IWW (2000) and assumed costs of € 35 / m3 and a contamination depth of 20 cm. The resulting cost value of € 7.6 / m2 was applied to the contaminated area (impaired) area alongside or around the infrastructure facilities for all types of biotopes.

- Further Effects. For estimating the costs of further effects on nature and landscape such as barrier effects and visual intrusion a unit value of € 10 per m² from INFRAS/IWW (2000) which was based on expert estimates was used.

Backcast methodology. Methodological approach used for the estimation of environmental costs for 1996 does not differ from the approach for the base year. Input data on network characteristics and traffic volumes were available for 1996. According to Nellthorp et al. (2001) values change proportionally to real incomes. Hence, all monetary values for all cost categories and modes were adjusted according to changes in real GDP per capita.

Concerning environmental costs, the quantifiable differences between the account years 1996 and 1998 are quite small. Firstly, the activities (vehicle mileage, number of starts and landings of aircraft) and emission factors do not change considerably within two years. Secondly, the actual changes are difficult to detect, as much of the required data is not available in sufficient detail. It has to be born in mind that the estimated changes from 1996 to 1998 are comparably rough and thus have to be interpreted with caution.

## Methodology for estimating taxes, charges and subsidies

The calculation of taxes, charges and subsidies is a great detail connected with the calculation of infrastructure and supplier operating costs. Since the availability of the respective input data are linked, the overall description of the data, steps used in the calculations and general assumptions are shown in the respective chapters.

The estimation of subsidies are based on the available figures mainly from the business accounts of transport companies and official statistics. State subsidies have different nature, that is direct of indirect. Direct subsidies include aid for operating and exploitation of the public transportation systems, while indirect subsidies could take the form of a tax deduction for the cost of commuting, reduction of or even exemption from VAT. The estimation of the latter is more complicated since it is not explicitly shown in company or national accounts. Indirect subsidies as reduction of VAT rates are applied to public transport and aviation (zero VAT).

Road. For the road transport the following charges and taxes were estimated: fuel excise duty, vehicle circulation tax (MRB), passenger car and motorcycle purchase tax (BPM) and Eurovignet. The MRB, BPM and Eurovignet data was collected from the official statistics.

Fuel excise duty was collected from CBS. The figures are estimated by multiplying the vehicle-km per vehicle type with the fuel excise duty.

BPM is a tax applied for the registration (purchase) of a passenger car or a motor bicycle. The tax is payable only once and it is levied on the list price exclusive of VAT and BPM. For the passenger cars the BPM rate is 45.2% of the list price exclusive of VAT and BPM. The result is to be reduced by NLG 3394 or, for passenger cars with diesel engines, NLG 2116. For the motor bicycles the rate is 10.2% for up to a list price of NLG 4700 exclusive of VAT and BPM; for higher prices the rate is 20.7% with a further reduction of NLG 494. The state is the beneficiary of the income from this tax.

| Vehicle | Characteristic | BPM |
| --- | --- | --- |
| Passenger car | Benzene | 0.452 x price – NLG 3394 |
| Passenger car | Diesel | 0.452 x price – NLG 2116 |
| Passenger car | LPG | 0.452 x price – NLG 3394 |
| Motorcycle | price<4700 | 0.102 x price |
| Motorcycle | price>4700 | 0.207 x price – NLG 494 |

MRB is a tax charged for the possession of passenger cars, LGVs and motorcycles (for the use of road for HGV – vignette). The tax is collected annually or quarterly, on the basis of the taxpayer’s returns, and a State car registration and its value depends on the type of the vehicle, its weight and the type of fuel used. The beneficiary of the income from MRB tax is partly the State and partly the provinces. Although the tax differs between provinces, the difference is not significant.

Eurovignet is the tax on heavy vehicles (of a gross maximum weight of 12000 kg or more) applied for the use of a motorway. The rate varies according to the number of axles of the vehicle and the period for which the tax is paid, which is optional to the keeper of the vehicle (daily, weekly, monthly or annually).

Parking tax is applied to the person parking a vehicle or requesting a license for parking a vehicle. The rates are different across municipalities; they are based on the duration and moment of parking, the area used for parking and the position of the plots or road sections. The revenues from the parking tax are collected by the municipalities. In the time framework of the proposed work it was impossible to estimate these revenues.

Rail. Data availability for rail transport is linked to the infrastructure costs and to the supplier operating costs. This data is collected and estimated by using the annual reports of the national railway company (NS). The methodological assumptions are shown in the accompanying chapters.

Public transport excluding rail. Data availability for public transport and rail is linked to the infrastructure costs and to the supplier operating costs. This data is collected and estimated by using the annual reports of several providers of public transport. The methodological assumptions are shown in the accompanying chapters.

Aviation. Charges for the use of aviation infrastructure include airport landing fees, aircraft parking fees and security charges. These revenues are collected by the airports. Therefore, information could be partly found in airports reports. However, not always the revenues are explicitly presented. In the pilot account the revenues were obtained from the annual reports from the airports Amsterdam Schiphol and Maastricht Aachen. It was not possible to get the airport taxes for Schiphol for 1998 (1997 was available), so we extrapolated this figure. An indirect subsidy to the air sector is the exemption of aviation from fuel tax. This could not be estimated due to the lack of available information.

Tax on noise pollution in aviation is charged to the user of the aircraft for landing on an airport in the Netherlands serving for civilian landing traffic. The tax is collected by means of a surcharge of the landing fee due by the user of the aircraft, by the Ministry of Transport and Waterways through the intermediary of the operator of the airport. The rate of the tax is determined by the Aviation Law and it is based on the proportion of the total noise nuisance within the noise area caused by the aircraft. This ratio depends on type of the aircraft and the level of the noise produced.

Inland waterway transport. No data on revenues of inland waterway harbours were available. Inland waterborne transport is exempted from paying fuel tax which can be considered as an indirect subsidy. However, due to the great number of IWW operators and the lack of required information, it was not possible to provide any estimates on taxes and charges for inland waterways.

Maritime shipping. Revenues in maritime shipping which have a direct relation to infrastructure costs are charges, fees and other payments at seaports and pilotage charges, as far as they do not belong to such coastal areas which are still defined as inland waterways. Information results from the annual reports from the two ports in Rotterdam and Amsterdam.

## Methodology for the forecasts to 2005

### General background

Elaboration of the pilot account for 2005 was done partially depending on the availability of specific forecasts for the respective year. In the cases when no forecasts or targets were available no costs for 2005 were calculated. The forecasts were carried out for the year 2005 using 2010 or 2020 as reference years and interpolating for 2005 by assuming a constant yearly growth rate between the base year and the reference year. One has to note, that the estimates for 2005 are only informative.

The central planning Bureau (CPB) elaborates long term forecast for they year 2020 for the Dutch economy on the basis of three scenarios: Divided Europe (DE), European Co-ordination (EC), and Global Co-ordination (GC). The EC-scenario models the situation under the assumption of the European Co-ordination. These three scenarios serve a starting point for the estimation of different effect for the NVVP (National Plan for Transport and Traffic) 2001-2020 with the base year 1995.

In the elaboration of scenario’s the CPB makes use of modelling of freight and passenger mobility and its consequences for accessibility undertaken by AVV with the help of LMS model (National Model Transport and Traffic System) and TEM-II (Transport Economic Model).

In the CPB modelling, the effects of different policies were estimated in different scenarios and for different time periods (2010 and 2020). Examples of the policy packages scenarios are: efficiency related measure to improve the use within modes; modal shift aimed at improving efficiency between modes; pricing package including the pricing for the use of infrastructure; as well as combination of the previously-named policies. Taking into account that 2005 relates to a rather shorter period of time within which limited policy decision could be taken, the forecasts for the pilot account are built on the results of the EC-scenario with no policy effects.

The forecasts for 2005 were developed autonomously of the policy framework, namely without taking into account specific policy modifications. The forecasts will be modified if such policies as variabilisation and internalisation of costs will be considered.

For the elaboration of pilot account for 2005 for road transport forecast figures from the EC-scenario elaborated by CPB (Central Planbureau) were used.

For rail, aviation and waterborne transport specific existing targets and forecasts indicators were used as well as interpolating approach when appropriate.

### Basic input data

The table below show the forecasts at the macro level for the Netherlands for the year 2005 according to the EC scenario. The yearly growth for the forecast in the pilot account is considered equal between 1998 and 2005 and correspond with the one given below.

Table 35: Forecasts for the basic input data (EC scenario)

|  | 1996-2020 |
| --- | --- |
| GDP, yearly growth | 2.75% |
| Population, yearly growth | 0.55% |
| Real price inflation, yearly | 0.5% |
| Source: Personen- en goederenmobiliteit in 2010 en 2020, Prognoses in het kader van de CPB-LT scenario’s en de Nationale Milieuverkenning, oktober 1997. |  |

#### Road transport

Time series for the last 15 years were available for the mileage driven and the fuel used by road transport on different types of roads and for different types of vehicles. These time series were used to interpolate forecast for 2005 since the existing forecast for mileage in the road transport according to NVVP does not cover all vehicles as categorised in Unite. Moreover, it is presented as total rather than by type of road. For the purpose of pilot account for 2005, mileage data for 2005 for different vehicles and on different roads were interpolated from 1985.

Table 36: Yearly growth forecasted for fuel use and mileage for toad transport

|  | Fuel use | vkm |
| --- | --- | --- |
| Total on road network |  |  |
| Passenger cars | 1.19% | 1.81% |
| Motorcycles | 3.97% | 3.96% |
| Bromcycles | -2.99% | -5.20% |
| LGV | 4.67% | 5.14% |
| HGV | -0.25% | -0.24% |
| Agricultural vehicles | 2.93% | 3.10% |
| Buses | -0.12% | -0.09% |
| Special vehicles | -2.24% | -0.84% |
| Source: NEI’s calculation based on mileage data from staline (CBS) |  |  |

#### Public transport and rail transport

Based on the forecast of the EC scenario the following data for vehicles mileage, passenger traffic and output result for the year 2005.

Table 37: Traffic output forecasts for the public transport (EC scenario)

|  | 1995 | 2010 | Yearly growth | 1998 million | 2005 million |
| --- | --- | --- | --- | --- | --- |
| Passenger-kilometres |  |  |  |  |  |
| Train | 100% | 120% | 1.22% | 14000 | 15243 |
| BTM | 100% | 104% | 0.26% | 6000 | 6111 |
| BTM – bus/tram/metro. Source: Personen- en goederenmobiliteit in 2010 en 2020, Prognoses in het kader van de CPB-LT scenario’s en de Nationale Milieuverkenning, oktober 1997. |  |  |  |  |  |

Table 38: Traffic forecasts for public transport

|  | 1996 | 1998 | 2005 |
| --- | --- | --- | --- |
| Passengers, million |  |  |  |
| -train, million | 306 | 321 | 366 |
| Vehicle-kilometres |  |  |  |
| -train-km, million | 119 | 124 | 124 |
| -BTM, vkm million | 646 | 630 | 694 |
| Passenger kilometres |  |  |  |
| -BTM, billion | 6 | 6 | 6.1111) |
| -train, billion | 14 | 14 | 15.2431) |
| 1) –Based on yearly growth in EC scenario between 1995 and 2010. BTM – bus/tram/metro. Source: NEI’s calculations. |  |  |  |

#### Aviation

For the calculation of transport costs for air transport forecasts from the CPB for the development at the Amsterdam Schiphol airport were used. They are presented below.

Table 39: Traffic forecast for Amsterdam Schiphol airport

|  | 1998 | 2010-L1) | 2010-H1) | 2010-M2) | 2005-M2) |
| --- | --- | --- | --- | --- | --- |
| Passengers, million | 34 | 43 | 63 | 52 | 44 |
| Movements, thousand | 377 | 460 | 590 | 472 | 430 |
| Tonnes, thousand | 576.207 | : | : | : | 877.7793) |
| 1) - Range of forecasts developed by CPB in collaboration with aviation sector covering only Schiphol. Forecasts for low and high scenarios available. 2) - NEI estimate, average yearly growth calculated as average from the low and high scenario. 3) – NEI estimate extrapolated from the development in the last ten years. Source: Ministerie van Verkeer en Waterstaat, Toekomst van de nationale luchthaven, 17 December 1999. |  |  |  |  |  |

The tables below present the figures for the forecasted traffic at Schiphol airport used in the pilot account for 2005.

Table 40: Forecast for Amsterdam Schiphol airport

|  | Movements |  | Tonnes |  |
| --- | --- | --- | --- | --- |
|  | 1998 | 2005, thousand | 1998 | 2005, thousand |
| Total | 376810 | 430 | 1171256 | 877.779 |
| scheduled flight | 348037 | 397 | 959950 | 719.419 |
| -passenger | 339195 | 387 | 588098 | 440.741 |
| -freight | 8842 | 10 | 371852 | 278.679 |
| non-scheduled | 28773 | 33 | 211306 | 158.360 |
| -passenger | 24897 | 28 | 6951 | 5.209 |
| -freight | 3876 | 4 | 204355 | 153.151 |
| Source: NEI’s calculations |  |  |  |  |

Table 41: Volume forecasts for the Amsterdam Schiphol airport

|  | 1998 | 2005, million |
| --- | --- | --- |
| Passengers | 33952148 | 43.625 |
| scheduled flight | 29956021 | 38.491 |
| -arrival | 14940737 | 19.197 |
| -departure | 15015284 | 19.293 |
| non-scheduled | 3996127 | 5.135 |
| -arrival | 1958402 | 2.516 |
| -departure | 2037725 | 2.618 |
| Source: NEI’s calculations |  |  |

#### Waterborne transport

The available in the official prognosis information on changes in the waterborne transport (relevant to the UNITE pilot account) is limited and is not satisfactory for the pilot account for 2005.

### Cost specific input data

In the following section cost categories specific input data used for the pilot account for 2005 will be presented.

#### Infrastructure costs

No capital costs were calculated. Running costs for road transport were extrapolated from 1998 by assuming an average growth of 1% p.a. The necessary input data, namely vehicle mileage per vehicle category, was forecasted by extrapolation of vehicle-kilometres since 1985.

To determine the running costs for 2005 for rail transport, core data for rail traffic were used. Both vehicles and passenger kilometres are expected to grow for the modes train and BTM (bus/tram/metro). The latter however will increase more compared to 1998. The growth in vehicle kilometres was used to estimate the running costs.

The running costs in 2005 for IWW transport are estimated based on the growth in total amount of vehicle kilometres.

#### Supplier operating costs

The supplier operating costs for rail and public transport are estimated based on an extrapolation of passenger kilometres and an analysis of developments in train materials. The output in terms of passenger kilometres will increase with 4.3 percent in 2005 compared to 1998, while the amount of vehicle kilometres will increase with 1.9 percent. This is consistent with the trend that the amount of passengers in a train will increase. This is caused by a higher occupancy rate and the fact that the operator uses longer trains. The same approach is used to estimate the supplier operating costs for the public transport.

#### Delay costs due to congestion

Values of travel time and fuel costs were transferred from 1998 to 2005 using the common rules laid down in Nellthorp et al. (2001).

In terms of delays probabilities the following assumptions were made:

- No changes in the congestion probability in comparison with 1998.

- For air traffic the same delay probability as for 1998 were used.

- For rail transport a forecast (target) presented in the yearly report of NS was applied.

#### Accident costs

Unit costs per injury, fatality and material damage for 2005 were derived from the 1998 values by considering the estimated growth in real prices.

The number of physical units for the road accidents for 2005 were determined on the basis of targets from NVVP (National Transport and Traffic Plan) for 2010. The respective yearly growth/decline was applied for 2005. The number of casualties involving hospitalisation and deaths will respectively decrease with 2.75% and 2.39%. The yearly decrease of the number of accidents with material damages was assumed to be 2.5%.

For rail transport accident rates are assumed to remain constant. The largest share of rail casualties belongs to suicides. This number oscillates around 200 in the last years. We assume that the number of total casualties will be 400 with equal share of deaths and injuries.

The number of accidents and casualties for 2005 was considered as the average for the last 5 years. The target in NVVP is to avoid any accidents, however taking into account the specific character of IWW accidents, the average over the last 5 years seems to be appropriate for the estimation of total accident costs.

#### Environmental costs

Prognoses for the emissions and fuel efficiency for the future are available form the NVVP. The next tables show the expected yearly growth (EC scenario) for particular emissions and energy use.

Table 42: Forecasts for energy use

|  | 1995 | 2010 | Yearly growth |
| --- | --- | --- | --- |
| Energy use1) (PJ) |  |  |  |
| -Passenger | 239 | 247 | 0.22% |
| -Freight | 135 | 183 | 2.05% |
| 1) – EC scenario. Source: Personen- en goederenmobiliteit in 2010 en 2020, Prognoses in het kader van de CPB-LT scenario’s en de Nationale Milieuverkenning, oktober 1997. |  |  |  |

Table 43: Forecast for emissions

|  | Air pollution, yearly growth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | CO2 | NOx | CO | VOS | PM10 |
| Total road transport | 0.18% | -9.62% | -8.83% | -7.51% | -8.36% |
| -passenger car | 2.48% | 1.22% | 1.28% | -1.23% | 1.76% |
| -LGV | 2.33% | -0.98% | 0.39% | -0.85% | -2.35% |
| -HGV (incl. agr. vehicles) | -11.26% | 0.00% | -6.76% | -5.62% | -2.73% |
| -other1) | 0.87% | -3.81% | 7.28% | 6.13% | 7.43% |
| 1) - includes motorcycles, buses, special vehicles and others. Source: NEI’s calculations based on EC scenario forecasts. |  |  |  |  |  |

Based on the above-presented assumption, the following input data was used for the calculation of the air pollution costs and global warming.

Table 44:	Input data for road emissions for 2005

|  | Fuel use, million volume-units | Air pollution, mill. kg |  |  |  |  | vkm, mill. |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | CO2 | NOx | CO | VOS | PM10 |  |
| Total road transport | 12445 | 29651 | 133 | 231 | 68 | 7 | 135700 |
| -passenger car | 7763 | 18016 | 43 | 167 | 44 | 2 | 106414 |
| -LGV | 1642 | 4340 | 16 | 26 | 5 | 3 | 17992 |
| -HGV (incl. agr. vehicles) | 2546 | 6821 | 62 | 11 | 5 | 2 | 7709 |
| -other1) | 493 | 473 | 11 | 26 | 13 | 1 | 3584 |
| 1) - includes motorcycles, buses, special vehicles and others. Source: NEI’s calculations based on EC scenario forecasts. |  |  |  |  |  |  |  |

For the calculation of the noise costs, forecasts for the number of people exposed to noise (as percentage of the total population older than 18 years) from RIVM (Milieucompendum 1999, Het milieu in cijfers, CBS, RIVM) were used. These are 9, 15 and 28 percent respectively for air transport, rail transport and road transport. The same shares for different noise levels were applied as for the year 1998. The table below shows the input data used.

Table 45: Forecasts for number of persons exposed to noise for 2005, million

|  | Road | Rail | Air |
| --- | --- | --- | --- |
| Total | 4.5751 | 0.8170 | 3.1045 |
| 55-60 dB | 2.8806 | 0.6039 | 1.8110 |
| 60-65 dB | 1.3556 | 0.1243 | 1.0348 |
| 65-70 dB | 0.2259 | 0.0444 | 0.1725 |
| 70-75 dB | 0.0847 | 0.0266 | 0.0575 |
| >75 dB | 0.0282 | 0.0178 | 0.0287 |
| Source: NEI’s calculation based on RIVM prognoses. |  |  |  |

#### Taxes, charges and subsidies

No estimation made for the taxes, charges and subsidies.

# Results

## Infrastructure costs

The table below presents the results obtained with the PIM for all modes.

Table 46: Capital value and costs, million Euro 1998

|  | ROAD | RAIL | PUBLIC | IWW |  | SEA HARBOURS | AVIATION |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | waterways | harbours |  |  |
| Life expectancies | 45 years | 30 years | 30 years | 35 years | 30 years | 30 years | 30 years |
| Gross capital value1) | 59122 | 10323 | 1797 | 111146 | 1548 | 4620 | 1834 |
| Net capital value1) | 39186 | 6140 | 998 | 109654 | 1107 | 4179 | 1271 |
| Depreciation | 1506 | 490 | 96 | 141 | 52 | 52 | 60 |
| Interests | 1176 | 184 | 30 | 3290 | 33 | 125 | 38 |
| Capital costs2) | 2682 | 674 | 126 | 3431 | 85 | 177 | 98 |
| 1) Including land value. 2) Including land costs. Calculated as average over the financial year. Source: NEI, DIW |  |  |  |  |  |  |  |

### Road

The next table shows the infrastructure costs for road. Capital stock and capital costs were obtained from the perpetual inventory model. Running costs were based upon data from the Statistics Netherlands (CBS). The same methodology as for 1998 was applied for accounting year 1996. Running costs for 2005 were estimated.

Table 47: Infrastructure costs for road (million Euro prices 1998)

|  | 1996 | 1998 | 2005 |
| --- | --- | --- | --- |
| Life expectancies | 45 years | 45 years | : |
| Gross capital value1) | 56189 | 59122 | : |
| Net capital value1) | 36090 | 39186 | : |
| Depreciation | 1507 | 1506 | : |
| Interests | 1083 | 1176 | ; |
| Capital costs2) | 2590 | 2682 | : |
| Running costs | 1347 | 1729 | 1915 |
| Total infrastructure costs | 3937 | 4411 | : |
| 1) Including land value. - 2) Including land costs. Calculated as average over the financial year. Source: NEI, DIW |  |  |  |

### Rail

The following table summarises the total infrastructure costs for rail. Capital stock and capital costs were calculated by using the perpetual inventory model. The investment time series for the perpetual inventory model were partly received from the national Statistics Netherlands. Missing data was extrapolated by using network length and land area. The investments only include infrastructure (no stations). The running costs were estimated from the annual reports of Railinfrabeheer, the Dutch Railway Infrastructure Administration.

Table 48:	Infrastructure costs for rail (million Euro prices 1998)

|  | 1996 | 1998 | 2005 |
| --- | --- | --- | --- |
| Life expectancies | 30 years | 30 years |  |
| Gross capital value1) | 10261 | 10323 | : |
| Net capital value1) | 5920 | 6140 | : |
| Depreciation | 496 | 490 | : |
| Interests | 178 | 184 | : |
| Capital costs2) | 674 | 674 | : |
| Running costs | 337.8 | 421.1 | 445 |
| Overhead wire and superstructure | 86.2 | 98.9 | : |
| Large scale maintenance | 59.9 | 68.5 | : |
| Small-scale maintenance | 174.5 | 231.9 | : |
| System development | 9.5 | 13.2 | : |
| Allocation to reserve millennium | 7.7 | 8.6 | : |
| 1) Including land value. - 2) Including land costs. Calculated as average over the financial year. Source: NEI, DIW |  |  |  |

### Public Transport

Since infrastructure costs relevant for busses were included in the road account, the estimation of infrastructure costs for public transport cover only the tram and metro infrastructure. The capital stock and capital costs for tram and metro infrastructure were – as for the other modes – calculated with the perpetual inventory model.

In the Netherlands only a small part of the subway is underground; the only underground lines are situated in Amsterdam and Rotterdam (which represent 50% of total tram/metro network). The percentage of underground lines in these cities is roughly 30%, leading to a total percentage of underground lines in the Netherlands of 6% of the metro lines (excluding tramlines).

The official transport statistics do not provide any information on expenditures or costs of public transport. Furthermore, the companies which provide public transport services, do not have separate bookkeeping for infrastructure. In additions, there is no separate bookkeeping for different type of transport merely because the infrastructure is not (completely) their responsibility. The only way to create data was to use the factor ‘investments versus running costs’ from the railways and use this factor on investments for PT infrastructure.

The same methodology as for 1998 was applied. The same data problems did allow only a very rough estimation of running costs.

Table 49:	Infrastructure costs for public transport (million Euro prices 1998)

|  | 1996 | 1998 | 2005 |
| --- | --- | --- | --- |
| Life expectancies | 30 years | 30 years |  |
| Gross capital value1) | 1777 | 1797 | : |
| Net capital value1) | 982 | 998 | : |
| Depreciation | 95 | 96 | : |
| Interests | 29 | 30 | : |
| Capital costs2) | 124 | 126 | : |
| Running costs | 67.9 | 83.5 | 95 |
| Total infrastructure costs | 191.9 | 209.5 | : |
| 1) Including land value. - 2) Including land costs. Calculated as average over the financial year. Source: NEI, DIW |  |  |  |

### Waterborne transport - inland waterways

The perpetual inventory model was used for calculating capital stock and capital costs, separately for inland waterways, inland waterway harbours and seaports. Information on running costs of inland waterways were obtained from both the Statistics Netherlands (CBS) and CE (1999). For the running costs the same methodology as for the investments costs was used. Based upon total expenditures and a rate for investments and running costs we were able to create rather accurate data.

The table below shows the results for capital and running costs.

Table 50: 	Infrastructure costs for IWW (million Euro, 1998 prices)

|  |  | 1996 |  |  | 1998 |  | 2005 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | waterways | harbours | total | waterways | harbours | total | Total |
| Life expectancies | 35 years | 30 years |  | 35 years | 30 years |  |  |
| Gross capital value1) | 97027 | 1457 | 98484 | 111146 | 1548 | 112694 | : |
| Net capital value1) | 95452 | 1005 | 96457 | 109654 | 1107 | 110761 | : |
| Depreciation | 144 | 52 | 196 | 141 | 52 | 193 | : |
| Interests | 2864 | 30 | 2894 | 3290 | 33 | 3323 | : |
| Capital costs2) | 3008 | 82 | 3090 | 3431 | 85 | 3516 | : |
| Running costs | : | : | 192.4 | : | : | 178.3 | 235 |
| Total infrastructure costs | : | : | 3282.4 | : | : | 3694.3 | : |
| 1) Including land value. - 2) Including land costs. Calculated as average over the financial year. Source: NEI, DIW |  |  |  |  |  |  |  |

### Sea Harbours

The perpetual inventory model was used for calculating capital costs for sea harbours. It was very difficult to obtain information regarding running costs. Information was obtained for the two largest harbours, namely Rotterdam and Amsterdam. These harbours represent about 75% of the total added value of the harbours in the Netherlands.

A source of information for the running costs of infrastructure could be the National Board of Harbours (NHR). Within the time framework of this report no specific information could be obtained. Therefore, running costs could not be calculated precisely. The infrastructure costs for sea harbours are presented in the next table.

Table 51:	Infrastructure costs for sea harbours (million Euro prices 1998)

|  | 1996 | 1998 |
| --- | --- | --- |
| Life expectancies | 30 years | 30 years |
| Gross capital value1) | 4037 | 4620 |
| Net capital value1) | 3584 | 4179 |
| Depreciation | 52 | 52 |
| Interests | 108 | 125 |
| Capital costs2) | 160 | 177 |
| Running costs | : | : |
| Total infrastructure costs | 160 | 177 |
| 1) Including land value. - 2) Including land costs. Calculated as average over the financial year. Source: NEI, DIW |  |  |

### Aviation

The next table shows the result of the PIM for the capital costs. Due to the lack of input data the running costs could not be estimated. The annual reports of the airports do not cover explicitly the running costs of the infrastructure.

Table 52:	Infrastructure costs for aviation (million Euro prices 1998)

|  | 1996 | 1998 |
| --- | --- | --- |
| Life expectancies | 30 years | 30 years |
| Gross capital value1) | 1765 | 1834 |
| Net capital value1) | 1146 | 1271 |
| Depreciation | 63 | 60 |
| Interests | 34 | 38 |
| Capital costs2) | 97 | 98 |
| Running costs | : | : |
| Total infrastructure costs | 97 | 98 |
| 1) Including land value. - 2) Including land costs. Calculated as average over the financial year. Source: NEI, DIW |  |  |

## Supplier operating costs

### National rail carrier NS Reizigers

The previously state owned rail carrier NS is split up. The infrastructure is still owned by the government and administrated by Railinfrabeheer. All freight and passenger transport is however privatised. The newly named NS Reizigers provides public transport by rail in the Netherlands, although a few lines in the north and east are operated by other companies (Syntus and NordNed) in both of which NS Reizigers participate.

In the annual report a balance sheets and profit/ loss statements is presented for the total NS Group. Most of the entries are specified for passenger transport, such as turnovers, depreciation, personnel and operating results. These figures were used for the estimations of supplier operating costs. The same methodology was applied as for the core year 1998.

Table 53: Supplier operating costs for rail transport (million Euro)

|  | 1996 | 1998 | 2005 |
| --- | --- | --- | --- |
| Vehicle related costs | 1626 | 1583.7 | 1671.4 |
| Service related costs | 53.7 | 51.7 | 55.9 |
| Administrative and commercial costs (incl. insurance) | 590.7 | 699.7 | 755.6 |
| Financial costs | 18.9 | 4.1 | 4.2 |
| Infrastructure use costs and maintenance costs | : | : | : |
| Total supplier operating costs | 2289.3 | 2339.2 | 2487.2 |
| Source: NEI |  |  |  |

### Public transport

Ideally, supplier operating costs would have to be estimated separately for companies with tram and metro operations (or their respective business units) and for companies operating bus services (or their respective business units). Furthermore, an analysis of supplier operating costs would require a separate treatment of municipal companies with (at least partly) public ownership on the one hand and private companies on the other hand.

However data problems complicated a disaggregated estimation of supplier costs for public transport. In three large cities (Amsterdam, Rotterdam and Den Haag) one operator (partly owned by municipality) provides both bus and tram/metro transport. These three companies are responsible for 38.7% of all operating incomes in public transport. The yearly reports do not present individual figures for these three kinds of modes. Therefore, the estimation of the operating costs of the public sector is treated as whole rather than individually for bus, tram and metro.

The overall turnover in the Netherlands presented in annual statistics (CBS) was split up. The summation of bus/tram providers (HTM, RET and GVB) was used and the remaining amount according to the aggregation of a bus operator (which all other public transport providers are) split up.

Furthermore, it is rather difficult to determine if and how much the three tram/bus operators invest in infrastructure maintenance.

The following table summarises the total estimated supplier operating costs for public transport.

Table 54:	Supplier Operating Costs public transport (million Euro, prices 1998)

|  | 1996 | 1998 | 2005 |
| --- | --- | --- | --- |
| Vehicle related costs | 1042.9 | 946.6 | 1080.3 |
| Service related costs | 133 | 131.1 | 149.9 |
| Administrative and commercial costs | 264 | 261.4 | 297.0 |
| Insurance and financial costs | 150 | 105.7 | 119.8 |
| Infrastructure use costs and maintenance costs | : | : | : |
| Total supplier operating costs | 1589.9 | 1448 | 1647.0 |
| Source: NEI |  |  |  |

## Delay costs due to congestion

The following table shows the results for total delay costs for the Netherlands in 1998. The traffic delay costs consisted about 1% of GDP in 1998. The EU Green Book on Fair and Efficient Pricing reports an average of 2% of GDP for road congestion costs which means that the obtained results are much lower.

Table 55:	Total delay costs for the Netherlands, € million 1998

|  | Additional costs due to disturbed traffic or delays |  |  | Total |
| --- | --- | --- | --- | --- |
|  | due to road congestion | additional fuel costs | due to late arrivals |  |
| Road | 2989.38 | 114.06 | 0.00 | 3103.44 |
| Rail | 0.00 | 0.00 | 45.22 | 45.22 |
| Aviation | 0.00 | 0.00 | 89.34 | 89.34 |
| TOTAL | 2989.38 | 114.06 | 134.56 | 3238.00 |
| Road includes passenger vehicles (cars, motorcycles, scooters, buses) and freight vehicles (LGV, HGV). |  |  |  |  |

The delays costs for public transport, rail and aviation have been poorly investigated in the Netherlands so far. On behalf of AVV, NEA undertakes periodical update of the estimation of the costs of traffic jams on the main roads network. Congestion on the other types of infrastructure is not considered. The congestion costs in this study covers mainly the costs due to additional fuel consumption and the costs of the time lost by drivers and the passengers in the traffic jams. Unlike the methodology used in the pilot account, NEA approach is based on the number and length of the registered traffic jams with a correction for underreporting. The estimation of delay costs is done with several scenarios for VOT values. The congestion costs on the main roads network according to the latter study amounts to about € 769 million in 1997 (AVV, 1997). Although a direct comparison with the results of the pilot account is not possible due to the different categorisation of network and vehicles used, one can note that the respective results of the both studies do not different essentially. The results show that almost 96% of the total transport delays costs were borne by road traffic users. The remaining were borne by rail and air traffic users.

Delay costs for rail transport are calculated for the first time in the UNITE pilot account. The results are rough estimates and based on a number of assumptions, therefore, they should be carefully interpreted. They can be improved when more detailed data on delays for different types of services and the distribution of delays during the day is made available. NS holds detailed information. However, for different reasons it is not disclosed. Delays costs in rail transport were estimated to € 45 million. This result is highly sensitive in regard to the used benchmark of the average delay time.

Delays costs estimated for Amsterdam Schiphol airport amounted to € 89 million in 1998. The delay costs for freight transport are much lower in comparison with the passenger transport.

The table below shows detailed results of the estimated total and average delay costs for different types of vehicles for 1998.

Table 56: Total and average delay costs, € million 1998

|  | Additional costs due to disturbed traffic or delays |  | Total | Mileage | Average costs |
| --- | --- | --- | --- | --- | --- |
|  | time | fuel |  | million vkm | per 1000 vkm |
| Road traffic | 2989.38 | 114.06 | 3103.44 | 114494 |  |
| Private vehicles | 1271.02 | 86.78 | 1357.80 | 96372 | 29.32 |
| Motorways | 468.54 | 25.65 | 494.19 | 36868 | 35.24 |
| Trunk roads | 439.45 | 27.93 | 467.38 | 35295 | 19.51 |
| Urban roads | 363.02 | 33.20 | 396.22 | 24209 | 34.61 |
| Bus | 115.73 | . | 115.73 | 604 | 93.20 |
| Motorways | 43.32 | . | 43.32 | 223 | 105.73 |
| Trunk roads | 28.64 | . | 28.64 | 151 | 58.53 |
| Urban roads | 43.77 | . | 43.77 | 230 | 103.82 |
| HGV | 378.04 | 17.70 | 395.74 | 3355 | 91.00 |
| Motorways | 216.31 | 10.47 | 226.78 | 1909 | 105.73 |
| Trunk roads | 116.23 | 5.73 | 121.96 | 1030 | 58.53 |
| Urban roads | 45.51 | 1.49 | 47.00 | 416 | 103.82 |
| LGV | 1224.59 | 9.58 | 1234.17 | 14163 | 45.31 |
| Motorways | 183.76 | 0.95 | 184.70 | 2833 | 52.86 |
| Trunk roads | 261.90 | 2.13 | 264.03 | 4249 | 29.26 |
| Urban roads | 778.93 | 6.51 | 785.44 | 7081 | 51.91 |
|  |  |  |  | mill. train-km | per 1000 train-km |
| Rail transport | 45.22 | . | 45.22 | 125 | 361.75 |
| Passenger | 44.35 | . | 44.35 | 116 | 382.30 |
| Freight | 0.87 | . | 0.87 | 9 | 96.81 |
|  |  |  |  | movements | per movement |
| Aviation | 89.34 | . | 89.34 | 376810 | 237.10 |
| Passenger | 88.89 | . | 88.89 | 364092 | 244.14 |
| Cargo | 0.45 | . | 0.45 | 12718 | 35.63 |
| In the case of road, additional time costs are dues to road congestion, while for the other modes time costs are due to late arrivals. |  |  |  |  |  |

Average costs per vehicle kilometre were calculated using PCU-kilometres for each network aggregate. In rail and air transport total costs were divided by respectively train kilometres and aircraft movements.

The following table gives an overview of the development of the total and average delay costs in 1996. The total delay costs in 1996 amounted to more than € 3100 million which is about 4% less than the respective estimate for 1998. The delays costs for individual vehicle categories and modes developed in a different way.

The delay costs exhibit an increase in 1998 for all considered vehicles with exception of private vehicles. This can be mainly explained by the increased traffic volumes. The most severe increase in the delay costs between 1996 and 1998 is observed in the aviation traffic. The delays costs in two years doubled. Delay costs of LGVs and rail transport increased with respectively 20% and 15%.

Table 57: Total and average delay costs in 1996, € million prices of 1998

|  | Additional costs due to disturbed traffic or delays |  | Total | Mileage | Average costs |
| --- | --- | --- | --- | --- | --- |
|  | time | fuel |  | million vkm | per 1000 vkm |
| Road traffic | 2917.47 | 100.33 | 3017.80 | 108380 |  |
| Private vehicles | 1458.77 | 75.67 | 1534.44 | 92517 | 30.42 |
| Motorways | 537.85 | 22.37 | 560.22 | 35314 | 37.41 |
| Trunk roads | 505.55 | 24.36 | 529.91 | 33835 | 20.74 |
| Urban roads | 415.37 | 28.93 | 444.30 | 23368 | 33.86 |
| Bus | 108.11 | . | 108.11 | 653 | 95.68 |
| Motorways | 40.45 | . | 40.45 | 223 | 112.22 |
| Trunk roads | 26.71 | . | 26.71 | 151 | 62.23 |
| Urban roads | 40.95 | . | 40.95 | 230 | 101.57 |
| HGV | 372.80 | 17.19 | 390.00 | 3443 | 95.55 |
| Motorways | 213.26 | 10.18 | 223.43 | 1959 | 112.22 |
| Trunk roads | 114.66 | 5.57 | 120.23 | 1057 | 62.23 |
| Urban roads | 44.89 | 1.45 | 46.33 | 427 | 101.57 |
| LGV | 877.79 | 7.47 | 985.26 | 11767 | 45.95 |
| Motorways | 146.67 | 0.74 | 147.41 | 2353 | 56.11 |
| Trunk roads | 209.10 | 1.66 | 210.76 | 3530 | 31.12 |
| Urban roads | 622.02 | 5.07 | 627.09 | 5884 | 50.79 |
|  |  |  |  | mill. train-km | per 1000 train-km |
| Rail transport | 38.32 | . | 38.32 | 120 | 319.34 |
| Passenger | 37.61 | . | 37.61 | 111 | 338.86 |
| Freight | 0.71 | . | 0.71 | 9 | 78.58 |
|  |  |  |  | movements | per movement |
| Aviation | 44.55 | . | 44.55 | 321779 | 138.46 |
| Passenger | 44.29 | . | 44.29 | 308849 | 143.40 |
| Cargo | 0.27 | . | 0.27 | 12930 | 20.58 |
| In the case of road, additional time costs are dues to road congestion, while for the other modes time costs are due to late arrivals. |  |  |  |  |  |

The average delays figures show the same increases in unit costs within respectively road, rail and aviation market segments. This is due to the relatively rough input data used.

Table 58: Relative changes of delay costs in 1996 to 1998 in prices of 1998, %

|  | Additional costs due to disturbed traffic or delays |  | Total | Mileage | Average costs |
| --- | --- | --- | --- | --- | --- |
|  | time | fuel |  | million vkm | per 1000 vkm |
| Road traffic | -2.41 | -12.04 | -2.76 | -5.34 |  |
| Private vehicles | 14.77 | -12.81 | 13.01 | -4.00 | 3.74 |
| Motorways | 14.79 | -12.79 | 13.36 | -4.22 | 6.14 |
| Trunk roads | 15.04 | -12.77 | 13.38 | -4.14 | 6.33 |
| Urban roads | 14.42 | -12.85 | 12.13 | -3.47 | -2.16 |
| Bus | -6.59 | . | -6.59 |  | 2.66 |
| Motorways | -6.63 | . | -6.63 | 8.07 | 6.14 |
| Trunk roads | -6.74 | . | -6.74 | 7.95 | 6.33 |
| Urban roads | -6.45 | . | -6.45 | 8.26 | -2.16 |
| HGV | -1.39 | -2.83 | -1.45 | 2.62 | 5.00 |
| Motorways | -1.41 | -2.83 | -1.48 | 2.62 | 6.14 |
| Trunk roads | -1.35 | -2.83 | -1.42 | 2.62 | 6.33 |
| Urban roads | -1.36 | -2.81 | -1.41 | 2.64 | -2.16 |
| LGV | -20.15 | -22.07 | -20.17 | -16.92 | 1.42 |
| Motorways | -20.18 | -22.10 | -20.19 | -16.94 | 6.14 |
| Trunk roads | -20.16 | -22.08 | -20.18 | -16.92 | 6.33 |
| Urban roads | -20.14 | -22.06 | -20.16 | -16.90 | -2.16 |
|  |  |  |  | mill. train-km | per 1000 train-km |
| Rail transport | -15.26 | . | -15.26 | -4.00 | -11.72 |
| Passenger | -15.18 | . | -15.18 | -4.31 | -11.36 |
| Freight | -18.83 | . | -18.83 | 0.00 | -18.83 |
|  |  |  |  | movements | per movement |
| Aviation | -50.13 | . | -50.13 | -14.60 | -41.60 |
| Passenger | -50.17 | . | -50.17 | -15.17 | -41.26 |
| Cargo | -41.28 | . | -41.28 | 1.67 | -42.25 |
| In the case of road, additional time costs are dues to road congestion, while for the other modes time costs are due to late arrivals. |  |  |  |  |  |

The following table presents the results obtained for the delay costs for the forecast year 2005.

Table 59: Total and average delay costs for 2005, € million prices of 1998

|  | Additional costs due to disturbed traffic or delays |  | Total | Mileage | Average costs |
| --- | --- | --- | --- | --- | --- |
|  | time | fuel |  | million vkm | per 1000 vkm |
| Road traffic | 3586.74 | 133.75 | 3720.49 |  |  |
| Private vehicles | 1500.31 | 102.11 | 1602.42 | 110486 |  |
| Motorways | 613.97 | 34.18 | 648.15 | 46707 | 32.61 |
| Trunk roads | 512.59 | 33.13 | 545.72 | 39811 | 20.88 |
| Urban roads | 373.75 | 34.80 | 408.55 | 23968 | 37.24 |
| Bus | 122.04 |  | 122.04 |  |  |
| Motorways | 45.68 | . | 45.68 | 314 | 97.83 |
| Trunk roads | 30.20 | . | 30.20 | 136 | 62.65 |
| Urban roads | 46.15 | . | 46.15 | 188 | 111.71 |
| HGV | 406.07 | 19.59 | 425.67 | 3468 |  |
| Motorways | 283.60 | 13.73 | 297.33 | 2417 | 97.83 |
| Trunk roads | 111.71 | 5.51 | 117.22 | 956 | 62.65 |
| Urban roads | 10.76 | 0.35 | 11.11 | 95 | 111.71 |
| LGV | 1558.32 | 12.05 | 1570.37 | 18107 |  |
| Motorways | 274.31 | 1.41 | 275.73 | 4084 | 48.91 |
| Trunk roads | 399.37 | 3.25 | 402.61 | 6257 | 31.32 |
| Urban roads | 884.64 | 7.39 | 892.03 | 7766 | 55.85 |
|  |  |  |  | mill. train-km | per 1000 train-km |
| Rail transport | 18.15 | . | 18.15 |  | 145.18 |
| Passenger | 16.87 | . | 16.87 | 116 | 145.41 |
| Freight | 1.28 | . | 1.28 | 9 | 142.30 |
|  |  |  |  | movements | per movement |
| Aviation | 118.60 | . | 118.60 | 429000 | 276.45 |
| Passenger | 118.27 | . | 118.27 | 415000 | 284.98 |
| Cargo | 0.33 | . | 0.33 | 14000 | 23.50 |
| In the case of road, additional time costs are dues to road congestion, while for the other modes time costs are due to late arrivals. |  |  |  |  |  |

## Accident costs

The following tables present the results of the calculations of internal and external accident costs. Table 60 shows the total social accident costs by mode.

Table 60:	Social costs of transport accidents, in € million, 1998 prices

|  | Material damage | Risk Value | Administrative costs | Medical treatment | Production losses | Total internal costs | Total external costs | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Road | 1 923.6 | 6 997.0 | 35.8 | 159.7 | 1 225.5 | 8 920.6 | 1 421.0 | 10 341.6 |
| Rail | 0 | 97.2 | 0.1 | 1.6 | 56.6 | 97.2 | 58.3 | 155.5 |
| Aviation | 0 | 0.9 | 0 | 0 | 0.3 | 0.9 | 0.4 | 1.3 |
| IWW | 0 | 13.9 | 0 | 0.2 | 3.4 | 13.9 | 3.6 | 17.5 |
| Maritime | 0 | 4.3 | 0 | 0 | 1.0 | 4.3 | 1.1 | 5.3 |
| Total | 1 923.6 | 7 113.3 | 35.9 | 161.5 | 1 286.8 | 9 036.9 | 1 484.3 | 10 521.2 |

The social accident costs, including both internal and external components, amounted in 1998 to more than € 10.5 billion which correspond to about 3% of Netherlands GDP. Of total costs 86% were internal. Because internal costs are carried by the transport sector, the interpretation of the results should be based on external costs that are carried by society as a whole. The most important cost driver is the risk value which account for 67% of the total social costs. Material damage and production losses accounted respectively for 19% and 12% of total while medical and administrative costs are of minor importance. In particular it is due to the underestimation of these costs due to the lack of reliable data.

Of the total social accident costs 98% represent road accidents. However, one should take into account that the social accidents for the other modes is underestimated because it does not cover the cost of material damages.

Table 61 shows in more detail the total social costs by bearer of the costs. It can be seen that the largest part, more than 85%, is borne by transport system. The high share is explained by the fact that the risk value is assumed to be an internal accident cost. The largest part of external to the transport system accident costs is borne by third parties through uncovered payments of the insurance.

Table 61: Total social accident costs by cost bearer, in € million, 1998

|  | TOTAL | Private user | Transport sector | Public sector | Third parties |
| --- | --- | --- | --- | --- | --- |
| Road | 10 341.6 | 8 313.5 | 607.1 | 195.5 | 1 225.5 |
| Rail | 155.5 | 97.2 | 0.0 | 1.7 | 56.6 |
| Aviation | 1.3 | 0.9 | 0.0 | 0.0 | 0.3 |
| IWW | 17.5 | 13.9 | 0.0 | 0.2 | 3.4 |
| Maritime | 5.3 | 4.3 | 0.0 | 0.0 | 1.0 |
| Total | 1 0521.2 | 8 429.8 | 607.1 | 197.5 | 1 286.8 |

The allocation of total social accident costs is rather complicated since the principle of UNITE is to avoid arbitrary allocation. In the allocation with the “monitoring (victim) perspective” it is actually impossible to assess vehicle category specific internal and external costs unless arbitrary cost allocation is accepted. A way to avoid this problem is to apply the “causer perspective”. However, suitable data on the distribution of accident responsibilities are not available. Nevertheless, an attempt has been made to meet the minimum disaggregation possible of road accidents under a number of assumptions.

In each mode the costs which are directly borne by the users of a specific mode are respectively seen as cause by mode. All other costs are distributed to the vehicle types by the share of vehicle specific costs. For the allocation of road accident costs to different types of roads, records of accidents, injuries and fatalities on different roads is used.

Table 62:	Total costs of road accidents per type of road, € million, 1998

|  | Motorways | Regional roads | Other roads | All roads |
| --- | --- | --- | --- | --- |
| Total, of which: | 1064 | 760 | 8517 | 10342 |
| Private vehicles | 667 | 490 | 5 359 | 6 516 |
| HGV | 90 | 41 | 309 | 440 |
| LGV | 88 | 62 | 664 | 814 |
| other | 219 | 167 | 2185 | 2571 |

Table 62 shows the results for different types of road infrastructure. It can be seen that the accident costs on other roads are higher than on motorways and regional roads, fact which proves that they are safer.

From the total social accident costs per mode and type of vehicle cost rates per unit of performance can be derived. Average costs were calculated based on vehicle-km (road), train-km, aircraft-km and vessel-km. Table 63 presents the results of the average cost estimates for road.

Table 63: 	Average costs of road accidents per type of road, €/1000vkm, 1998

|  | Motorways | Regional roads | Other roads | All roads |
| --- | --- | --- | --- | --- |
| Private vehicles | 17.97 | 13.22 | 174.37 | 55.65 |
| HGV | 53.90 | 39.65 | 523.12 | 107.70 |
| LGV | 26.95 | 19.82 | 261.56 | 142.11 |
| Private vehicles include passenger cars, motorcycles, mopeds, and buses. HGV – rigid and articulated goods vehicles with a gross weight > 3.5t. LGV – goods vehicles with a gross weight < 3.5t. |  |  |  |  |

For other transport modes data on vessel-km, aircraft-km was unreliable therefore no average estimates are presented.

For the estimation of accident costs in 1996 and 2005 the respective data on physical units were used. Unit costs values were transferred in line with the UNITE valuation conventions. Tables 64-66 show the results for total accident costs, i.e. internal and external costs and average costs for road transport.

Table 64: 	Social costs of transport accidents in 1996 and 2005, in € million

|  | Material damage | Risk Value | Administrative costs | Medical treatment | Production losses | Total internal costs | Total external costs | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1996 |  |  |  |  |  |  |  |  |
| Road | 1 830 | 6 845.7 | 30.2 | 153.7 | 1 190.9 | 8 675.7 | 1 374.8 | 10 050.6 |
| Rail | 0 | 28.2 | 0.1 | 1.0 | 48.7 | 28.2 | 49.8 | 78.0 |
| Aviation | 0 | 47.7 | 0 | 0.1 | 7.6 | 47.7 | 7.7 | 55.3 |
| IWW | 0 | 20.2 | 0 | 0.3 | 4.8 | 20.2 | 5.1 | 25.3 |
| Maritime | 0 | 8.0 | 0 | 0 | 1.5 | 8.0 | 1.5 | 9.5 |
| Total | 1 830 | 6 949.8 | 30.4 | 155.1 | 1 253.5 | 8 779.7 | 1 439.0 | 10 218.7 |
| 2005 |  |  |  |  |  |  |  |  |
| Road | 1668.5 | 6070.7 | 31.1 | 139.8 | 1205.2 | 7739.2 | 1376.1 | 9115.2 |
| Rail | 0 | 334.0 | 0.1 | 1.6 | 59.7 | 334.0 | 61.4 | 395.3 |
| Aviation | : | : | : | : | : | : | : | : |
| IWW | 0 | 15.8 | 0 | 0.2 | 4.2 | 15.8 | 4.4 | 20.2 |
| Maritime | 0 | 9.6 | 0 | 0 | 2.0 | 9.6 | 2.0 | 11.6 |
| Total | 1668.5 | 6430.1 | 31.2 | 141.5 | 1271.1 | 8098.5 | 1443.8 | 9542.3 |

Table 65: Total social accident costs by cost bearer in 1996 and 2005, in € million

|  | TOTAL | Private user | Transport sector | Public sector | Third parties |
| --- | --- | --- | --- | --- | --- |
| 1996 |  |  |  |  |  |
| Road | 10 050.6 | 8 137.6 | 538.1 | 184.0 | 1190.9 |
| Rail | 78.0 | 28.2 | 0 | 1.1 | 48.7 |
| Aviation | 55.3 | 47.7 | 0 | 0.1 | 7.6 |
| IWW | 25.3 | 20.2 | 0 | 0.3 | 4.8 |
| Maritime | 9.5 | 8.0 | 0 | 0 | 1.5 |
| Total | 10 218.7 | 8 241.6 | 538.1 | 185.5 | 1 253.5 |
| 2005 |  |  |  |  |  |
| Road | 9115.2 | 7212.6 | 526.6 | 170.9 | 1205.2 |
| Rail | 395.3 | 334.0 | 0 | 1.7 | 59.7 |
| Aviation | : | : | : | : | : |
| IWW | 20.2 | 15.8 | 0 | 0.2 | 4.2 |
| Maritime | 11.6 | 9.6 | 0 | 0 | 2.0 |
| Total | 9542.3 | 7571.9 | 526.6 | 172.7 | 1271.1 |

Table 66:	Average costs of road accidents per type of road in 1996 and 2005, €/1000vkm

|  | Motorways | Regional roads | Other roads | All roads |
| --- | --- | --- | --- | --- |
| 1996 |  |  |  |  |
| Private vehicles | 17.88 | 14.67 | 180.82 | 58.01 |
| HGV | 53.63 | 44.01 | 542.46 | 111.30 |
| LGV | 26.81 | 22.01 | 271.23 | 147.59 |
| 2005 |  |  |  |  |
| Private vehicles | 12.34 | 10.02 | 154.62 | 42.43 |
| HGV | 37.01 | 30.07 | 463.87 | 46.79 |
| LGV | 18.50 | 15.03 | 231.94 | 108.84 |
| Private vehicles include passenger cars, motorcycles, mopeds, and buses. HGV – rigid and articulated goods vehicles with a gross weight > 3.5t. LGV – goods vehicles with a gross weight < 3.5t. |  |  |  |  |

The total accident costs in 1996 were about 3% lower than in 1998. Accident costs have developed different for different modes and different vehicle classes. The total cost of road accidents in 1996 was about two times less than in 1998. This can be explained mainly by the increase in the number of accident as result of the traffic growth in 1998. In respect to the other modes, the total accident costs figures for 1996 are substantially higher. This is not strange since accidents in these modes have different character than in the road sector.

## Environmental Costs

### Overview of the total environmental costs

Table 67 and table 69 present the results of environmental costs in the Netherlands for 1998 and 1996. One has to note, that the presented costs are only the costs which are currently quantifiable. For some modes or cost categories no appropriate data was available (e.g. nuclear risk). In addition, there are effects, for which currently no consistent monetary values exist (e.g. costs of ecosystem impairment due to nitrogen deposition).

The highest share of costs are due to air pollution and nature and landscape effects. The sector causing the highest costs is road transport, reflecting its dominating role in transport performance. Air pollution is one of the most important costs category, for both passenger and freight transport. Costs are dominated by impacts due to primary and secondary particles, above all loss of life expectancy and increased morbidity rates.

Noise, the least important cost category according to the results, is dominated by amenity losses. Further cost components here are health impacts due to ischaemic heart disease and hypertension and the subjective impairment of sleeping quality. Noise exposure estimates were only available for the whole road transport sector. The resulting costs were not broken down to passenger and freight transport to avoid arbitrary cost allocation.

Table 67:	Environmental costs per mode in € million, 1998

|  | Air Pollution excl. CO2 | Global Warming | Noise | Nature, Landscape, Soil and Water pollution | Total |
| --- | --- | --- | --- | --- | --- |
| Road | 1481.5 | 686.3 | 311.03 | 1896.84 | 4375.67 |
| Passenger transport* | 814.7 | 455.8 | : | 1617.17 | 2887.67 |
| Freight transport | 644.1 | 223.8 | : | 279.67 | 1147.57 |
| Rail | 9.6 | 2.2 | 22.42 | 73.65 | 107.87 |
| Passenger transport | 4.6 | 1.1 | : | 68.31 | 74.01 |
| Freight transport | 5.1 | 1.1 | : | 5.35 | 11.55 |
| Aviation | 24.9 | 15.46 | 186.08 | : | 226.44 |
| Inland Waterways | 279.8 | : | : | 100.31 | 380.11 |
| Total | 1795.8 | 703.96 | 519.53 | 2070.8 | 5090.09 |
| Costs due to short sea shipping are not estimated individually for the Netherlands, but rather for Europe as a whole. Results will be presented in a separate report. Nuclear risks not estimated. No split available for noise according to UNITE principle of non-arbitrary allocation. *) Passenger transport includes busses Source: NEI |  |  |  |  |  |

Table 68 shows the environmental costs of road transport for different vehicle types. Noise costs are not included, because a break down would contradict the UNITE principle of non-arbitrary cost allocation. Passenger cars cause the highest total costs, followed by light and heavy goods vehicles.

Table 68:	Environmental costs for roads transport in € million, 1998

|  | Air Pollution | Global Warming | Noise | Nature, Landscape, Soil and Water pollution | Total |
| --- | --- | --- | --- | --- | --- |
| Motorcycles | 26.3 | 4.4 | : | 16.88 | 47.58 |
| Scooters | 12.9 | 1.5 | : | 2.48 | 16.88 |
| Passenger Cars | 709.1 | 378.7 | : | 1589.84 | 2677.64 |
| Buses | 66.4 | 11.3 | : | 8.34 | 86.04 |
| Light Goods Vehicles | 238.6 | 75.3 | : | 157.93 | 471.83 |
| Heavy Goods Vehicles | 199.2 | 63.0 | : | 65.54 | 327.74 |
| Agricultural vehicles | 206.3 | 61.0 | : | 52.63 | 319.93 |
| Special vehicles | 22.7 | 6.0 | : | 3.21 | 31.91 |
| Total | 1481.5 | 601.2 | : | 1896.85 | 3979.55 |
| Source: NEI, IER. Noise costs not broken down according to UNITE principle of non-arbitrary cost allocation. |  |  |  |  |  |

Table 69 shows the costs for the 1996 account. Changes compared to the account year 1998 are not essential but they exhibit a different pattern for different modes and cost categories.

For road transport, costs due to air pollution declined in 1998, reflecting progress in reducing emissions in 1998. In the case of road transport, the higher share of vehicles with cleaner technologies results in lower emissions and costs of air pollution. Traffic volume on road significantly increased over two years. The overcompensation of the volume effect by the technology effect can only be observed for road transport.

For air and waterborne transport, overall environmental costs were lower in 1996, reflecting lower traffic in 1996. For the rail transport, air pollution costs increased during the two years as a result of the increased emissions. Noise costs have increased for all modes in 1998 in comparison with 1996 reflecting the increased traffic volume for all modes.

Table 69:	Environmental costs per mode in 1996, € million prices 1998

|  | Air Pollution excl. CO2 | Global Warming | Noise | Nature, Landscape, Soil and Water pollution | Total |
| --- | --- | --- | --- | --- | --- |
| Road* | 1738.7 | 686.9 | 292.744 | 2056.80 | 4775.1 |
| Passenger Transport | 962.4 | 463.5 | : | 1770.15 | 3196.1 |
| Freight Transport | 752.7 | 216.8 | : | 286.65 | 1256.2 |
| Rail | 7.6 | 1.97 | 21.490 | 80.34 | 111.4 |
| Passenger Transport | 3.1 | 1.03 | : | 74.27 | 78.4 |
| Freight Transport | 4.5 | 0.94 | : | 6.07 | 11.5 |
| Aviation | 21.5 | 14.2 | 162.757 | : | 198.5 |
| Inland Waterways | 236.2 | : | : | 104.27 | 340.5 |
| Total | 2004.0 | 703.1 | 476.99 | 2241.41 | 5425.5 |
| Costs due to short sea shipping are not estimated individually for the Netherlands, but rather for Europe as a whole. Results will be presented in a separate report. Nuclear risks not estimated. No split available for noise according to UNITE principle of non-arbitrary allocation. *) Passenger transport includes busses; freight transport includes LGV and HGV. The total figure includes agricultural and special vehicles, too. Source: NEI, IER |  |  |  |  |  |

### Air pollution and global warming

As was mentioned above, the total costs due to air pollution are almost exclusively caused by road transport. The second important mode for air pollution is waterborne transport. Rail remains an unimportant with regard to air pollution.

The total and average costs of air pollution for road are summarised in table 58 for different types of vehicles. The average costs per vehicle kilometre are estimated based on the shares of the vehicle categories on the total emissions and the relative harmfulness of the different air pollutants.

The figures show the high costs per vehicle kilometres for public buses many of them travelling in urban areas. The high cost rate is caused by high emission factors (in g/vkm) that result first of all from the frequent stop and go of public buses.

Table 70:	Total and average costs for air pollution and global warming, 1998

|  | Air pollution (excl. GHG) |  | GHG (direct) |  | GHG direct + indirect |  | Air pollution + GHGs |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | total | average | total | average | total | average | total | average |
|  | mill. € | €/100vkm | mill. € | €/100vkm | mill. € | €/100vkm | mill. € | €/100vkm |
| Road, total | 1481.5 | 1.26 | 601.2 | 0.51 | 686.3 | 0.58 | 2167.8 | 1.84 |
| -Passenger, total | 814.7 | 0.84 | 395.8 | 0.41 | 455.8 | 0.47 | 1270.5 | 1.31 |
| -Freight, total | 644.1 | 3.15 | 199.3 | 0.97 | 223.8 | 1.09 | 867.9 | 4.24 |
| Passenger cars | 709.1 | 0.76 | 378.7 | 0.40 | 436.3 | 0.46 | 1145.3 | 1.22 |
| -petrol | 459.3 | 0.72 | 267.4 | 0.42 | 311.4 | 0.49 | 770.7 | 1.21 |
| -diesel | 211.8 | 1.00 | 77.1 | 0.36 | 86.7 | 0.41 | 298.5 | 1.41 |
| -LPG | 37.9 | 0.41 | 34.2 | 0.37 | 38.2 | 0.41 | 76.1 | 0.82 |
| Motorcycles, petrol | 26.3 | 1.84 | 4.4 | 0.31 | 5.2 | 0.36 | 31.5 | 2.20 |
| Bromcycles, petrol | 12.9 | 1.16 | 1.5 | 0.13 | 1.7 | 0.15 | 14.6 | 1.31 |
| Busses, diesel | 66.4 | 11.07 | 11.3 | 1.88 | 12.6 | 2.11 | 79.0 | 13.17 |
| LGV | 238.6 | 1.68 | 75.3 | 0.53 | 84.9 | 0.60 | 323.5 | 2.28 |
| -petrol | 14.9 | 1.23 | 6.0 | 0.50 | 7.0 | 0.58 | 21.9 | 1.81 |
| -diesel | 219.3 | 1.76 | 67.1 | 0.54 | 75.5 | 0.61 | 294.8 | 2.36 |
| -LPG | 4.3 | 0.90 | 2.2 | 0.45 | 2.4 | 0.51 | 6.8 | 1.41 |
| HGV | 199.2 | 5.94 | 63.0 | 1.88 | 70.5 | 2.10 | 269.7 | 8.04 |
| -petrol | 0.0 | 0.66 | 0.0 | 2.00 | 0.0 | 2.00 | 0.1 | 2.66 |
| -diesel | 199.2 | 5.94 | 63.0 | 1.88 | 70.5 | 2.10 | 269.6 | 8.04 |
| -LPG | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Agric. vehicles, diesel | 206.3 | 7.01 | 61.0 | 2.07 | 68.3 | 2.32 | 274.7 | 9.34 |
| Special vehicles | 22.7 | 6.29 | 6.0 | 1.67 | 6.8 | 1.87 | 29.5 | 8.16 |
| -petrol | 0.2 | 1.44 | 0.1 | 1.17 | 0.2 | 1.38 | 0.3 | 2.82 |
| -diesel | 22.5 | 6.47 | 5.9 | 1.69 | 6.6 | 1.89 | 29.1 | 8.37 |
| -LPG | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Source: NEI, IER |  |  |  |  |  |  |  |  |

The general picture of the results for global warming is very similar to the one of the costs of air pollution. By far the largest share of the costs is contributed by the road transport sector, too. As in the case of air pollution and because of the same reasons, the buses cause after agricultural vehicles the highest costs per vkm. However, the difference between buses and cars, for example, is much less marked.

Looking at total costs of only € 9.6 million and the annual performance of about 115 million train-kilometre, the average costs of air pollution exclusive global warming and including global warming for rail transport amount to respectively about € 7.78 and 9.54 / train-km. The average costs of rail transport per train-kilometre are negligibly small.

In the case of air transport a total estimate of € 24.9 million results in about 13.53 per movement.

The following tables show the results for air pollution and global warming for 2005 for the road transport.

Table 71: 	Air pollution and global warming costs for 2005, in million €

|  | CO2 direct | CO2 indirect1) | Air poll.1) | NOx | CO | VOS | PM2.5 | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Passenger car | 360 | 58 | 96 | 162 | 0.08 | 58 | 112 | 846 |
| LGV | 87 | 11 | 18 | 60 | 0.01 | 7 | 158 | 341 |
| HGV (incl. agr. vehicles) | 136 | 17 | 28 | 232 | 0.01 | 7 | 129 | 549 |
| Other | 9 | 4 | 6 | 42 | 0.01 | 17 | 45 | 124 |
| Total | 593 | 90 | 148 | 496 | 0.11 | 89 | 444 | 1861 |
| 1) - Fuel production. Assumptions: fuel split as for 1998; for “other” 50% petrol, 50% diesel; 50% of PM-emissions urban, 50% extra-urban; 50% of CO-emissions urban, 50% extra-urban. Source: IER, NEI |  |  |  |  |  |  |  |  |

Table 72: 	Air pollution and global warming costs in 2005 for road transport

|  | Air pollution (excl. CO2) |  | CO2 (direct) |  | CO2 direct+ indirect |  | Air pollution + CO2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Total | Average | Total | Average | Total | Average | Total | Average |
|  | mill. € | €/100 vkm | mill. € | €/100 vkm | mill. € | €/100 vkm | mill. € | €/100 vkm |
| Passenger car | 428 | 0.40 | 360 | 0.34 | 419 | 0.39 | 846 | 0.80 |
| LGV | 244 | 1.35 | 87 | 0.48 | 98 | 0.54 | 341 | 1.90 |
| HGV* | 396 | 5.13 | 136 | 1.77 | 154 | 1.99 | 549 | 7.12 |
| Other | 111 | 3.08 | 9 | 0.26 | 13 | 0.37 | 124 | 3.45 |
| Total | 1177 | 0.87 | 593 | 0.44 | 683 | 0.50 | 1861 | 1.37 |
| *) - HGV include agricultural vehicles. Source: IER, NEI |  |  |  |  |  |  |  |  |

### Noise costs

In the next table the noise costs as estimated with the ExternE methodology are summarised.

Table 73:	Total noise costs by mode, € million

|  | 1998 |  |  | 1996 |  |  | 2005 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Road | Rail | Aviation | Road | Rail | Aviation | Road | Rail | Aviation |
| Myocardial infarction | 11.179 | 1.591 | 7.636 | 11.496 | 1.636 | 7.439 | 11.268 | 1.603 | 7.697 |
| Angina pectoris | 0.115 | 0.016 | 0.079 | 0.119 | 0.017 | 0.077 | 0.116 | 0.017 | 0.085 |
| Hypertension | 0.011 | 0.001 | 0.005 | 0.011 | 0.001 | 0.005 | 0.011 | 0.001 | 0.006 |
| Sleep disturbance | 52.525 | 6.544 | * | 54.014 | 6.729 | * | 52.943 | 6.596 | * |
| Amenity losses | 247.199 | 14.267 | 178.359 | 227.103 | 13.107 | 155.235 | 319.464 | 18.438 | 230.500 |
| Total | 311.029 | 22.419 | 186.079 | 292.744 | 21.490 | 162.757 | 383.803 | 26.654 | 238.288 |
| *) no data for night time noise available; assumption: no flights during the night, therefore no sleep disturbance. Source: NEI, IER |  |  |  |  |  |  |  |  |  |

Noise costs are dominated by amenity losses with the second most important effect being sleep disturbance. Further cost components here are health impacts due to ischaemic heart disease and hypertension.

An attempt was made to allocate the total costs for road and rail transport to different types of vehicles. The resulting costs are presented in the table below.

Table 74:	Noise costs for road and rail transport by vehicle category.

| Vehicle category | Weight * | 1996 |  | 1998 |  | 2005 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Total | Average | Total | Average | Total | Average |
|  |  | mill. € | €/100 vkm | mill. € | €/100 vkm | mill. € | €/100 vkm |
| Total road |  | 292.7 |  | 311.0 |  | 383.8 |  |
| Motor Cycles | 5 | 29.4 | 10.791 | 26.3 | 10.358 | 29.9 | 10.719 |
| Passenger Cars | 1 | 70.4 | 0.784 | 70.9 | 0.755 | 87.5 | 0.813 |
| Buses | 6 | 4.9 | 7.467 | 4.3 | 7.187 | 4.4 | 6.618 |
| LGV | 7 | 134.4 | 11.423 | 155.9 | 11.009 | 212.1 | 11.713 |
| HGV | 14 | 53.6 | 8.184 | 53.6 | 8.042 | 49.9 | 6.771 |
| Train category |  | 21.5 |  | 22.4 |  | 26.7 |  |
| Passenger train | 1 | 18.5 | 167.893 | 19.4 | 167.305 | 23.1 | 198.914 |
| Goods train | 2 | 3.0 | 335.785 | 3.0 | 334.610 | 3.6 | 397.828 |
| Noise emission relative to a passenger car derived from differences in noise emissions from Bundesumweltministerium (Hrsg.): Dokumentation Motorrad und Umwelt. Bericht der Projektgruppe "Motorrad und Umwelt", Berlin 1999 Source: IER, NEI |  |  |  |  |  |  |  |

This split has to be interpreted with caution, because it represents only a very rough approximation. Noise exposure estimates and vehicle mileage stem from different sources and the procedure of splitting the total costs does NOT necessarily represent the vehicle categories’ true share in causing noise exposure. Noise exposure estimates were available only for the whole road and rail transport sectors. The resulting costs are broken down by splitting the total based on weighted vehicle kilometres. As the lion’s share of noise exposure occurs in urban areas, the vehicle kilometres driven on urban roads are used for determining the vehicle categories’ shares in the total noise costs on all roads.

### Nature and landscape

The table below shows the detailed results for the total and average costs of the environmental costs due to nature, landscape soil and water pollution.

Table 75:	Total and average costs of nature and landscape, 1998

| Mode of Transport | Total Costs in € million |  |  |  | Average costs |
| --- | --- | --- | --- | --- | --- |
|  | Habitat loss and bio- diversity | Unsealing of sealed ground | Soil decontamina-tion | TOTAL | € per 1000 vkm |
| Road |  |  |  |  |  |
| Motorways | 135.61 | 51.07 | 4.03 | 190.71 | 4.37 |
| Trunk roads | 1470.36 | 208.21 | 27.56 | 1706.13 | 41.24 |
| TOTAL | 1605.97 | 259.28 | 31.59 | 1896.84 | 22.30 |
| Rail |  |  |  |  |  |
| Single | 48.85 | 6.74 | 0.65 | 56.24 |  |
| Double | 17.42 | 0.00 | 0.00 | 17.42 |  |
| TOTAL | 66.27 | 6.74 | 0.65 | 73.65 | 593.98 |
| Waterborne transport |  |  |  |  |  |
| Channels | 95.54 | 0.00 | 0.00 | 95.54 |  |
| Rivers | 4.77 | 0.00 | 0.00 | 4.77 |  |
| TOTAL | 100.31 | 0.00 | 0.00 | 100.31 | 1238.39 |
| ALL MODES | 1773 | 266 | 32 | 2071 |  |

The nature and landscape estimates should be treated with precaution. The relatively high estimates are to be investigated in more details in the further research. The overestimation of the costs for road and waterways can be explained by the method which was applied, that is the assumption that the infrastructure increases over time. In the case of roads, it can be noted that the length of trunk roads (>9m) decreased in comparison with the basic year. The reason for that is the change in the registration system of the roads at the beginning of eighties. For the moment being it was not possible to adjust the input data for this change.

In the case of waterways the length of the rivers and canals decreased over time, fact due to the fact that efforts are done towards improving the quality of the waterways rather than prolong them. The nature and landscape effects in this case relate to increase in the width and the deepness rather than length. The model has, therefore, to be adjusted to these effects. Consequently, the results for IWW and road are rough estimates and have to be improved in the future.

## Taxes, charges, subsidies

This section reports on the transport related taxes and charges which can be compared with the related costs. Furthermore, as far as the available data did allow to do so, subsidies were quantified.

### Road transport

The following table shows the revenues related to road infrastructure costs for 1996 and 1998 in 1998 prices. Revenues from fuel excise duty, circulation and purchase tax, and Eurovignet amounted to more than € 9 billion in 1998 and represents an increase with more than 5 % in comparison with 1996. When including the VAT on fuel tax, the total revenues in 1998 amounted to about € 10.2 billion. The fuel tax is the main contributor in the overall revenues, followed by circulation and purchase tax respectively.

Table 76:	Road transport revenues, € million 1998

|  | All roads |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | 1996 | 1996 in 1998 prices | 1998 |
| Fuel excise duty |  | 4370 | 4540.4 | 5040 |
| Motorcycles2) |  | 65 | 67.5 | 67 |
| Passenger Cars |  | 3137 | 3259.3 | 3619 |
| Buses |  | 68 | 70.7 | 68 |
| Vans |  | 386 | 401.1 | 498 |
| Lorries |  | 359 | 373.0 | 380 |
| Trailers |  | 323 | 335.6 | 370 |
| Special Vehicles |  | 32 | 33.2 | 38 |
| Vehicle Circulation Tax (MRB) |  | 2260 | 2348.1 | 1873 |
| Purchase tax |  | 1892 | 1965.8 | 2425 |
| Eurovignet |  | 69 | 71.7 | 91 |
| TOTAL |  | 8591 | 8926.0 | 9429 |
| Additional information: VAT* |  | 743 | 772.0 | 857 |
| *) VAT on exise duty Source: NEI |  |  |  |  |

### Rail transport

The next table shows the revenues from track access charges, from station charges and from tariffs in passenger and freight transport. The highest share in the revenues is due to the revenues from passenger transport. These revenues are accountable for more than 90% of the total revenues. The increase in 1998 as compared to 1996 is due to the increase in the number of rail passenger as well as the tariffs.

Table 77: 	Revenues from taxes, charges and tariffs in rail transport, € million

|  | 1996 | 1996 in prices 1998 | 1998 |
| --- | --- | --- | --- |
| Charges |  |  |  |
| Revenues passenger transport | 976 | 1014.1 | 1210 |
| Revenues freight transport | 143 | 148.6 | 155 |
| Subsidies |  |  |  |
| For unrewardable lines | 30 | 31.2 | 81 |
| Total revenues | 1149 | 1193.9 | 1446 |
| Source: NEI based on NS year reports |  |  |  |

The revenues from fuel tax paid for Diesel consumption could not be estimated due to the lack of information on energy consumption.

Subsidies to the trail sector have different purposes. In the accounting years subsides were given by the government for provision of services on non-profitable lines. No information could be retrieved for other subsidies.

### Public transport excluding rail

Table below summarises the revenues including subsidies for public transport excluding rail.

Table 78: Revenues of the public transport (excl. rail), € million

|  | 1996 | 1996 in 1998 prices | 1998 |
| --- | --- | --- | --- |
| Revenues from passenger transport | 1393 | 1447.3 | 1396 |
| Subsidies | 406 | 421.8 | 508 |
| Total revenues | 1799 | 1869.1 | 1904 |
| Source: NEI |  |  |  |

Due to lack of data on energy consumption as a separate figure for public transport excluding rail it was not possible to estimate revenues of the state from fuel taxation. However, it should be borne in mind that the road account contains the fuel tax paid by buses, most of this item referring to public transport.

Subsidies reported for the public transport are compensation payments for reduced tariffs. Public transport receives as well subsidies for running costs. From the business account of the transport companies the different types of subsidies could not be specified. Therefore, only a total is presented.

### Aviation

Table below shows the revenues of the airports Amsterdam Schiphol and Maastricht Aachen.

Table 79:	Revenues of aviation infrastructure, € million

|  | 1996 | 1996 in prices 1998 | 1998 |
| --- | --- | --- | --- |
| Airport taxes | 203 | 210.9 | 224 |
| Tax on Noise | 1.7 | 1.8 | 1.3 |
| Total revenues | 204.7 | 212.7 | 225.3 |
| Source: NEI |  |  |  |

Note: According to CBS the subsidies for airports (with 20 employee’s or more) amounted to less than € 1 million for the years 1996 and 1997.

### Waterborne transport

Due to lack of information it was not possible to obtain or estimate the charges of inland waterways. According to CBS statistics, for the years 1996 and 1997 the subsidies amounted to € 25 million.

Revenues in maritime shipping that relate to the use of infrastructure are charges, fees and other payments at seaports, and pilotage charges. No information on subsidies could be retrieved. The table below shows the estimated results for total revenues of the maritime shipping.

Table 80:	Revenues of maritime shipping, € million

|  | 1996 | 1996 in prices 1998 | 1998 |
| --- | --- | --- | --- |
| Charges for infrastructure use |  |  |  |
| Sea ports | 198 | 205.7 | 209 |
| Inland ports | 12 | 12.5 | 12 |
| Subsidies | : | : | : |
| Total revenues | 210 | 218.2 | 221 |
| Source: NEI |  |  |  |

# Summary of results for the Netherlands

## Road transport

The total cost of road transport from the Dutch pilot account are presented in the next table.

Table 81: Dutch Road Account for 1996, 1998 and 2005 (€ million at 1998 prices)

| Costs |  |  |  |
| --- | --- | --- | --- |
| Core information | 1996 | 1998 | 2005 |
| Infrastructure Costs | 3937 | 4411 | : 10) |
| Fixed | : | : | : |
| Variable | : | : | : |
| Accident costs (external)3)8) | 1374.8 | 1421.0 | 1376.1 |
| Environmental costs9) | 2718.3 | 2478.8 | 2243.8 |
| Air pollution9) | 1738.7 | 1481.5 | 1177 |
| Global warming9) | 686.9 | 686.3 | 683 |
| Noise9) | 292.7 | 311.03 | 383.80 |
| Total | 8030.1 | 8310.8 | : |
| Additional information |  |  |  |
| Congestion costs4) 9) | 3017.80 | 3103.44 | 3720.49 |
| Time costs | 2917.47 | 2989.38 | 3586.74 |
| Fuel costs | 100.33 | 114.06 | 133.75 |
| Accident costs (user internal)6)8) | 8675.7 | 8920.6 | 7739.2 |
| From this: risk value | 6845.7 | 6970.0 | 6070.7 |
| Environmental costs |  |  |  |
| Nature and landscape, soil and water pollution7)9) | 2056.8 | 1896.84 |  |
| Nuclear risk7) |  |  |  |
| Revenues |  |  |  |
| Directly related to a specific cost category |  |  |  |
| Charges for infrastructure usage1) | 71.7 | 91 | : |
| Fixed | : | : | : |
| Variable | : | : | : |
| Total | 71.7 | 91 | : |
| Other transport specific revenues |  |  |  |
| Annual circulation tax | 2348.1 | 1873 | : |
| Fuel tax9) | 4540.4 | 5040 | : |
| Purchase tax | 1965.8 | 2425 | : |
| VAT2) | 772.0 | 857 | : |
| Total | 9626.3 | 10195 | : |
| Subsidies5) | 0 | 0 | : |
| 1) Covers only Eurovignet. – 2) VAT levied on fuel tax. – 3) Refers to those parts of accident costs which are not borne by road users and insurance companies but by the public sector and third parties. – 4) Expressed as delay costs. – 5) Subsidies included here refer subsidies given for debt relief, for the provision of services etc. These subsidies can clearly not be allocated to either the cost or to the revenue side of this table. Subsidies are in cash flow terms and are not on the same basis as the economic costs. – 6) Refers to those parts of accident costs which are borne by road users and insurance companies and includes risk value. – 7) Because there is no standardised methodology for the calculation of these costs, the figures given here are to be regarded only as approximate indications that may change greatly over time with the development of a standard methodology. – 8) Passenger cars, motorcycles and goods vehicles. – 9) Buses are included. Figures only for buses are presented in the public transport account. – 10 Running costs are estimated at 1915 million Euro. Source: Certan et al., 2001 |  |  |  |

The total cost for road as a sum of different road cost categories amounts to € 22 billion which correspond to around 7% of the Dutch GDP at factor costs. In 1998, the core year of the pilot accounts, the largest cost block was the total social accident costs. Total accident costs amounted to € 10 billion, out of these about 14% (€ 1.4 billion) were external accident costs, i.e. those parts of accident costs which are not borne by road users themselves or by transport insurance companies. Infrastructure costs and environmental costs were the second largest cost blocks (each with around € 4.4 billion). Congestion costs, which in the UNITE accounts refer to costs of delay (e.g. time and fuel costs) and not to the dead-weight welfare loss of congestion, were at € 3.1 billion.

On the revenue side we have estimated road transport related revenues of € 10.3 billion in 1998. The share of such charges which relate directly to infrastructure usage was  € 91 million which is less than 1% of all road transport related taxes and charges. This fact shows a low share of revenues directly raised for covering infrastructure costs. This revenue is due to the HGV on the use of the motorways. It is expected that the total road transport related revenues will increase because of the existing plans of introducing kilometre charges for the use of infrastructure.

Infrastructure costs. Capital stock and capital costs were calculated using the perpetual inventory model. Running costs were calculated based upon data from the Statistics Netherlands (CBS). In order to avoid the arbitrary allocation the total costs were not allocated to different users.

Congestion costs. The road congestion costs were estimated by using a modelling approach. The estimated costs cover fuel and time costs of road users due to delays. Values of time used are the UNITE standard values adjusted for the Netherlands.

Accident costs. The input data for the calculation of the road accident costs comes form the official statistics regarding the number and severity of the accidents and is of a good quality. The calculation took into account the underreporting of accidents by using the available official estimates for registration degree for different types of casualties. The obtained estimate of the costs is highly dependant on the valuation of the risk value which was standardised within UNITE. The total accident costs for road do not comprise costs of the legal system and insurance, thus leading to an underestimate of the administration costs.

Environmental costs. The sector causing the highest environmental costs is road transport, reflecting its dominating role in transport performance. Air pollution is one of the most important costs category, for both passenger and freight transport. For the estimation of the environmental costs for road a very detailed input data on emissions was used. Costs are dominated by impacts due to primary and secondary particles, above all loss of life expectancy and increased morbidity rates. The high cost rate per vehicle kilometres for public buses - many of them travelling in urban areas - is caused by high emission factors (in g/vkm) that results first of all from the frequent stop and go of public buses. The general picture of the results for global warming is very similar to the one of the costs of air pollution. By far the largest share of the costs is contributed by the road transport sector, too. Noise is the least noteworthy cost category according to the results. Noise exposure estimates were only available for the whole road transport sector. The resulting costs were not broken down to passenger and freight transport to avoid arbitrary cost allocation.

Taxes, charges and subsidies. The input data was obtained from available official statistics. Based on the available information the respective revenues for road sector were estimated. The costs were allocated to different users only when it was possible without arbitrary allocation.

In table 82 the fully allocated costs of road transport for all roads and vehicle types are presented. In tables 83 – 86 the total costs of road transport are shown per road type (all roads, motorways, trunk roads and urban roads) and disaggregated by vehicle type (motorcycles, passenger car, buses, light goods vehicles weighting 3.5 tonnes or less and heavy goods vehicles weighting over 3.5 tonnes).

Table 82: 	Fully allocated costs of road transport per vehicle km in Netherlands,  €/km at 1998 prices

| All Roads |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1998 |  |  |  |  |  |  |
|  | Motor-cycles |  | Passenger cars |  | Buses | LGV | HGV |
| Core information |  |  |  |  |  |  |  |
| Infrastructure costs | : |  | : |  | : | : | : |
| Fixed | : |  | : |  | : | : | : |
| Variable | : |  | : |  | : | : | : |
| External accident costs1) | 0.05565 |  |  |  |  | 0.14211 | 0.10774) |
| Environmental costs |  |  |  |  |  |  |  |
| Air pollution | 0.01848) |  | 0.0076 |  | 0.1099 | 0.0168 | 0.05949) |
| Global warming | 0.00368) |  | 0.0046 |  | 0.0209 | 0.0060 | 0.02109) |
| Noise3) | 0.0104 |  | 0.0008 |  | 0.0072 | 0.0110 | 0.008 |
| Total I |  |  |  |  |  |  |  |
| Additional information |  |  |  |  |  |  |  |
| Delay costs | 0.02932 |  |  |  | 0.0932 | 0.04531 | 0.0914) |
| Internal accident costs2) |  |  |  |  |  |  |  |
| Material damages |  |  |  |  |  |  |  |
| Risk value |  |  |  |  |  |  |  |
| Environmental costs |  |  |  |  |  |  |  |
| Nature, landscape, soil and water pollution | : |  | : |  | : | : | : |
| Total II |  |  |  |  |  |  |  |
| Revenues |  |  |  |  |  |  |  |
| Fixed |  |  |  |  |  |  |  |
| Vignette | : |  | : |  | : | : | : |
| Annual circulation tax | : |  | : |  | : | : | : |
| Variable |  |  |  |  |  |  |  |
| Fuel tax | : |  | : |  | : | : | : |
| Eco tax5) | : |  | : |  | : | : | : |
| Distance related infrastructure charges | : |  | : |  | : | : | : |
| VAT 6) | : |  | : |  | : | : | : |
| Basic data |  |  |  |  |  |  |  |
| Million vehicle km | 2542 | 93830 |  | 604 |  | 14163 | 6659 |
| Million passenger km | : | 85500 |  | 63487) |  | . |  |
| Million tonne km | . | . |  | . |  | 161608 |  |
| 1) Both external and internal accident costs. – 2) Figures are included in item "External accident costs" of the core information section. – 3) Allocation to vehicle types not possible for different type of roads. – 4) Including special and agricultural vehicles. - 5) Ecological tax. – 6) VAT on fuel tax. – 7) Including by tram and metro. - 8) Only motorcycles, excluding scooters. - 9) Only HGV, excluding agricultural and special vehicles. Source: Certan et al, 2001. |  |  |  |  |  |  |  |

Table 83: 	Total costs of road transport in Netherlands, € million at 1998 prices

| All Roads |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1998 |  |  |  |  |  |  |  |
|  | Motor-cycles |  | Passenger cars |  | Buses | LGV | HGV2) | Total |
| Core information |  |  |  |  |  |  |  |  |
| Infrastructure costs | : |  | : |  | : | : | : | 4411.0 |
| Fixed | : |  | : |  | : | : | : | : |
| Variable | : |  | : |  | : | : | : | : |
| External accident costs | 90881)8) |  |  |  |  | 8151) | 4391) | 10342 |
| Administrative | : |  |  |  | : | : | : | 35.8 |
| Health costs | : |  |  |  | : | : | : | 159.7 |
| Production loss | : |  |  |  | : | : | : | 1225.5 |
| Environmental costs | 72.4 |  | 1216.3 |  | 83.3 | 479.4 | 627.4 | 2478.8 |
| Air pollution | 39.2 |  | 709.1 |  | 66.4 | 238.6 | 428.2 | 1481.5 |
| Global warming | 6.9 |  | 436.3 |  | 12.6 | 84.9 | 145.6 | 686.3 |
| Noise | 26.3 |  | 70.9 |  | 4.3 | 155.9 | 53.6 | 311.0 |
| Total I |  |  |  |  |  |  |  | 8310.8 |
| Additional information |  |  |  |  |  |  |  |  |
| Delay costs | 1357.8 |  |  |  | 115.73 | 1234.17 | 395.74 | 3103.44 |
| Internal accident costs | : |  |  |  |  | : | : | 8920.6 |
| Material damages | : |  |  |  | : | : | : | 1923.6 |
| Risk value | : |  |  |  | : | : | : | 6997 |
| Environmental |  |  |  |  |  |  |  |  |
| Nature, landscape, soil and water pollution | 19.36 |  | 1589.84 |  | 8.34 | 157.93 | 121.38 | 1896.85 |
| Total II |  |  |  |  |  |  |  | 13921 |
| Revenues |  |  |  |  |  |  |  |  |
| Fixed |  |  |  |  |  |  |  |  |
| Vignette | . |  | . |  | . | . | 91 | 91 |
| Purchase tax | : |  | : |  | : | : | : | 2425 |
| Annual circulation tax | : |  | : |  | : | : | : | 1873 |
| Variable |  |  |  |  |  |  |  |  |
| Fuel tax | 67 |  | 3619 |  | 68 | 498 | 788 | 5040 |
| Eco tax 4) | . |  | . |  | . | . | . | . |
| Distance related infrastructure charges | : |  | : |  | : | : | : | : |
| VAT3) | : |  | : |  | : | : | : | 857 |
| Total |  |  |  |  |  |  |  | 10286 |
| Basic data |  |  |  |  |  |  |  |  |
| Number of vehicles, million | 373 | 5931 |  | 11 |  | 649 | 792) | 7043 |
| Million vehicle km | 2542 | 93830 |  | 604 |  | 14163 | 6659 | 117798 |
| Million passenger km | : | 85500 |  | 63487) |  | . |  |  |
| Million tonne km | . | . |  | . |  | 161608 |  |  |
| 1) Internal and external accident cost. – 2) Including special and agricultural vehicles. – 3) VAT on fuel tax. - 4) Ecological tax. – 5) Motorcycle and passenger cars. – 6) Not including noise costs. – 7) Including by tam and metro. – 8) Of which € 6516 million are allocated to motorcycles, passenger cars and buses, and the rest to others. Source: Certan et al., 2001 |  |  |  |  |  |  |  |  |

Table 84: 	Total costs of road transport in Netherlands, € million at 1998 prices

| Motorways |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1998 |  |  |  |  |  |  |  |  |  |  |  |
|  | Motor-cycles | Passenger cars |  |  |  | Buses |  | LGV |  | HGV2) | Total |  |
| Core information |  |  |  |  |  |  |  |  |  |  |  |  |
| Infrastructure costs | : |  | : |  | : |  | : |  | : |  |  | : |
| Fixed | : |  | : |  | : |  | : |  | : |  |  | : |
| Variable | : |  | : |  | : |  | : |  | : |  |  | : |
| External accident costs (total incl. internal) | 885.95) |  |  |  |  |  | 88.2 |  | 89.9 |  |  | 1064 |
| Administrative |  |  |  |  |  |  |  |  |  |  |  |  |
| Health costs |  |  |  |  |  |  |  |  |  |  |  |  |
| Production loss |  |  |  |  |  |  |  |  |  |  |  |  |
| Environmental costs3) | 3.8 |  | 454.5 |  | 17.6 |  | 54.0 |  | 275.9 |  |  | 805.8 |
| Air pollution |  |  |  |  |  |  |  |  |  |  |  |  |
| Global warming |  |  |  |  |  |  |  |  |  |  |  |  |
| Noise1) | : |  | : |  | : |  | : |  | : |  |  | : |
| Total I | : |  | : |  | : |  | : |  | : |  |  | : |
| Additional information |  |  |  |  |  |  |  |  |  |  |  |  |
| Delay costs | 494.19 |  |  |  |  | 43.32 |  | 184.70 |  | 226.78 | 948.99 |  |
| Internal accident costs |  |  |  |  |  |  |  |  |  |  |  |  |
| Material damages |  |  |  |  |  |  |  |  |  |  |  |  |
| Risk value |  |  |  |  |  |  |  |  |  |  |  |  |
| Environmental costs |  |  |  |  |  |  |  |  |  |  |  |  |
| Nature, landscape, soil and water pollution | : |  |  | : |  | : |  | : |  | : | 190.71 |  |
| Total II |  |  |  |  |  |  |  |  |  |  |  |  |
| Revenues |  |  |  |  |  |  |  |  |  |  |  |  |
| Fixed |  |  |  |  |  |  |  |  |  |  |  |  |
| Vignette | • | • |  |  |  | • |  | • |  | : | : |  |
| Annual circulation tax | : | : |  |  |  | : |  | : |  | : | : |  |
| Variable |  |  |  |  |  |  |  |  |  |  |  |  |
| Fuel tax | : | : |  |  |  | : |  | : |  | : | : |  |
| Eco tax | • | • |  |  |  | • |  | • |  | • | • |  |
| Distance related infrastructure charges | • | • |  |  |  | • |  | • |  | • | • |  |
| VAT4) | : | : |  |  |  | : |  | : |  | : | : |  |
| Total |  |  |  |  |  |  |  |  |  |  |  |  |
| Basic data |  |  |  |  |  |  |  |  |  |  |  |  |
| Million vehicle km | 316 |  | 36552 |  | 223 |  | 2833 |  | 3764 |  |  | 43688 |
| 1) Total road noise costs can not be allocated to road type. – 2) Including special and agricultural vehicle. – 3) Includes air pollution and global warming. – 4) VAT on fuel tax. – 5) Of which € 667 million are allocated to motorcycles, passenger cars and buses, and the rest to others. Source: Certan et al., 2001. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 85:	Total costs of road transport in Netherlands, € million at 1998 prices

| Trunk roads |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1998 |  |  |  |  |  |  |
|  | Motor-cycles | Passenger cars |  | Buses | LGV | HGV2) | Total |
| Core information |  |  |  |  |  |  |  |
| Infrastructure costs | : | : |  | : | : | : | : |
| Fixed | : | : |  | : | : | : | : |
| Variable | : | : |  | : | : | : | : |
| Ext. accident costs1) (total incl. internal) | 656.96) |  |  |  | 62.6 | 40.7 | 760.2 |
| Administrative | : |  |  | : | : | : | : |
| Health costs | : |  |  | : | : | : | : |
| Production loss | : |  |  | : | : | : | : |
| Environmental costs5) | 9.9 | 295.9 |  | 15.0 | 57.3 | 133.2 | 511.3 |
| Air pollution | : | : |  | : | : | : | : |
| Global warming | : | : |  | : | : | : | : |
| Noise3) | : | : |  | : | : | : | : |
| Total I |  |  |  |  |  |  |  |
| Additional information |  |  |  |  |  |  |  |
| Delay costs1) | 467.38 |  |  | 28.64 | 264.03 | 121.96 | 882.01 |
| Internal accident costs |  |  |  |  |  |  |  |
| Material damages |  |  |  |  |  |  |  |
| Risk value |  |  |  |  |  |  |  |
| Environmental costs |  |  |  |  |  |  |  |
| Nature, landscape, soil and water pollution | : |  | : | : | : | : | 1706.13 |
| Total II |  |  |  |  |  |  |  |
| Revenues |  |  |  |  |  |  |  |
| Fixed |  |  |  |  |  |  |  |
| Vignette | • | • |  | • | • | • | • |
| Annual circulation tax | : | : |  | : | : | : | : |
| Variable |  |  |  |  |  |  |  |
| Fuel tax | : | : |  | : | : | : | : |
| Eco tax | • | • |  | • | • | • | • |
| Distance related infrastructure charges | • | • |  | • | • | • | • |
| VAT4) | : | : |  | : | : | : | : |
| Total |  |  |  |  |  |  |  |
| Basic data |  |  |  |  |  |  |  |
| Million vehicle km | 552 |  | 34743 | 151 | 4249 | 1679 | 41374 |
| 1) Here trunk roads = other inter-urban roads – 2) Including special and agricultural vehicles – 3) Total road noise costs cannot be allocated to road type – 4) VAT on fuel tax. – 5) Includes air pollution and global warming. – 6) Of which € 490 million are allocated to motorcycles, passenger cars and buses, and the rest to others. Source: Certan et al., 2001. |  |  |  |  |  |  |  |

Table 86: Total costs of road transport in Netherlands, € million at 1998 prices

| Urban roads |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1998 |  |  |  |  |  |  |
|  | Motor-cycles | Passenger cars |  | Buses | LGV | HGV1) | Total |
| Core information |  |  |  |  |  |  |  |
| Infrastructure costs |  |  |  |  |  |  |  |
| Fixed | : | : |  | : | : | : | : |
| Variable | : | : |  | : | : | : | : |
| External accident costs (total incl. internal) | 7544.85) |  |  |  | 663.8 | 308.8 | 8517.4 |
| Administrative | : |  |  |  | : | : | : |
| Health costs | : |  |  |  | : | : | : |
| Production loss | : |  |  |  | : | : | : |
| Environmental costs4) | 32.3 | 395.0 |  | 46.4 | 212.2 | 164.7 | 850.6 |
| Air pollution | : | : |  | : | : | : | : |
| Global warming | : | : |  | : | : | : | : |
| Noise2) | : | : |  | : | : | : | : |
| Total I |  |  |  |  |  |  |  |
| Additional information |  |  |  |  |  |  |  |
| Delay costs | 396.22 |  |  | 43.77 | 785.44 | 47 | 1272.43 |
| Internal accident costs |  |  |  |  |  |  |  |
| Material damages |  |  |  |  |  |  |  |
| Risk value |  |  |  |  |  |  |  |
| Environmental costs |  |  |  |  |  |  |  |
| Nature, landscape, soil and water pollution | : |  | : | : | : | : | : |
| Total II |  |  |  |  |  |  |  |
| Revenues |  |  |  |  |  |  |  |
| Fixed |  |  |  |  |  |  |  |
| Vignette | • | • |  | • | • | • | • |
| Annual circulation tax | : | : |  | : | : | : | : |
| Variable |  |  |  |  |  |  |  |
| Fuel tax | : | : |  | : | : | : | : |
| Eco tax | • | • |  | • | • | • | • |
| Distance related infrastructure charges | • | • |  | • | • | • | • |
| VAT3) | : | : |  | : | : | : | : |
| Total |  |  |  |  |  |  |  |
| Basic data |  |  |  |  |  |  |  |
| Million vehicle km | 1674 |  | 22535 | 230 | 7081 | 1216 | 32736 |
| 1) Including special and agricultural vehicles. – 2) Total road noise costs of € 311million cannot be allocated to road. - 3) VAT on fuel tax. – 4) Includes air pollution and global warming. – 5) Of which € 4359 million are allocated to motorcycles, passenger cars and buses, and the rest to others. Source: Certan et al., 2001. |  |  |  |  |  |  |  |

## Rail transport – National rail carrier Nederlandse Spoorwegen NS

Table 87:	Dutch Rail Account for 1996, 1998 and 2005 (€ million at 1998 prices)

| Costs |  |  |  |
| --- | --- | --- | --- |
| Core information | 1996 | 1998 | 2005 |
| Infrastructure Costs | 1012 | 1095.1 | : 7) |
| Fixed | : | : | : |
| Variable | : | : | : |
| Supplier operating costs | 2289 | 2339.2 | 2487.2 |
| Out of these: |  |  |  |
| Track & station charges1) | : | : | : |
| Accident costs (external) | 50 | 58.3 | 61.4 |
| Environmental costs | 32 | 34.22 | : |
| Air pollution | 8 | 9.6 | : |
| Global warming | 2 | 2.2 | : |
| Noise | 22 | 22.42 | 26.65 |
| Total | 3389 | 3526.8 | : |
| Additional information |  |  |  |
| Congestion costs4) | 38.32 | 45.22 | 18.15 |
| Accident costs (internal) | 28.2 | 97.2 | 334.0 |
| From this: risk value | 28.2 | 97.2 | 334.0 |
| Environmental costs |  |  |  |
| Nature and landscape, soil and water pollution5) | 80.34 | 73.65 | : |
| Nuclear risk5) | : | : | : |
| Revenues |  |  |  |
| Directly related to a specific cost category |  |  |  |
| Track charges | 148.6 | 155 | : |
| Fixed | : | : | : |
| Variable | : | : | : |
| Station charges | : | : | : |
| Subsidies for concessionary fares | 31.2 | 81 | : |
| User Tariffs3) | 1014.1 | 1210 | : |
| Out of these: | : | : | : |
| Track & station charges1) | : | : | : |
| Total | 1194 | 1446 | : |
| Other transport specific revenues |  |  |  |
| Fuel tax | : | : | : |
| Eco tax | . | . | . |
| VAT2) | : | : | : |
| Total | : | : | : |
| Subsidies6) | : | : | : |
| Non-transport related revenues of rail companies | : | : | : |
| 1) Rail track charges appear on both the cost side as part of the supplier operating costs and on the revenue side as part of the tariff revenues and show a monetary transfer. - 2) VAT levied on fuel tax. – 3) Subsidies and VAT are excluded. - 4) Expressed as delay costs. – 5) Because there is no standardised methodology for the calculation of these costs, the figures given here are to be regarded only as approximate indications that may change greatly over time with the development of a standard methodology. – 6) Subsidies included here refer to subsidies given for debt relief, for the provision of rail services etc. These subsidies can clearly not be allocated to either the cost or to the revenue side of this table. Subsidies are in cash flow terms and are not on the same basis as the economic costs. – 7) Running costs are estimated at 445 million Euro. Source: Certan et al., 2001. |  |  |  |

The total costs estimated for rail transports is significantly lower than those estimated for road sector. Amounting to € 3.7 billion it is slightly lower than the total costs estimated for the Dutch inland waterways (€ 4 billion). The estimated total costs for rail transport represent less than 17% of the total costs for road. The largest cost block is the supplier operating costs with about one third the total costs for rail, followed by infrastructure costs.

The total amount of rail transport related revenues amounted in 1998 to € 1.4 billion, out of which more than 83% (€ 1.2 billion excluding subsidies for non profitable lines) were generated from user tariffs. Supplier operating costs for rail transport was € 2.3 billion. In comparison with the road sector, the rail sector boasts with a higher share of revenues directly related to infrastructure costs.

Infrastructure costs. Capital stock and capital costs were calculated by using the perpetual inventory model. The investment time series for the perpetual inventory model were partly received from the national Statistics Netherlands. Missing data was extrapolated by using network length and land area. The investments only include infrastructure (no stations). The running costs were estimated from the annual reports of Railinfrabeheer, the Dutch Railway Infrastructure Administration.

Supplier operating costs. The input data for the estimation of the supplier operating costs was taken from the balance sheets and profit/ loss statements of the NS Group and estimated by NEI.

Congestion costs. The estimation was made on the basis of the punctuality figures obtained form NS. This was the only available source of information which could be used in the accounts. Note however, that detailed delays statistics for rail is recorded but remains under the confidentiality of NS. The results for the delay costs for rail transport are rough estimates and based on a number of assumptions, therefore, they should be carefully interpreted.

Accident costs. As opposed to the road sector, the problem of underreporting for railways is assumed to be of minor relevance, therefore negligible, at least within the scope of accident cost estimation in this report. The total accident costs for railways do not cover the costs of material damage. Because of the lack of data material damages could not be estimated.

Environmental costs. The basic data for rail transport in the Netherlands is of good quality on the general level. On a disaggregated level there is no official data available for other modes than road. Rail remains an unimportant mode if compared with the other modes with regard to air pollution. The highest share of environmental costs for the rail transport belongs to nature and landscape followed by noise costs. The noise costs were estimated as total. Detailed information of the exposure to noise according to the level of noise and time of the day could not be obtained. The lack of relevant information did not allow the estimation of the costs of nuclear risks.

Taxes, charges and subsidies. The input data used for the estimation of the revenues were collected from the annual reports and balance sheets of NS Group. On the basis of the available information it is difficult to judge the completeness of the cost estimates in particular regarding subsidies. In addition, the available figures for subsidies did not contain information about the purpose of the subsidies.

The fully allocated costs of rail transport are shown in the next table.

Table 88: Fully allocated costs of rail transport per vehicle km in Netherlands, €/train km at 1998 prices

|  | 1998 |  |  |
| --- | --- | --- | --- |
|  | Passenger | Freight |  |
| Core information |  |  |  |
| Infrastructure costs |  |  |  |
| Fixed | : | : |  |
| Variable | : | : |  |
| External accident costs 1) | 1.34042 | : |  |
| Administrative | : | : |  |
| Health costs | : | : |  |
| Production loss | : | : |  |
| Environmental costs | 0.0131 | 0.0424 |  |
| Air pollution | 0.0084 | 0.0315 |  |
| Global warming | 0.0047 | 0.0109 |  |
| Noise | 0.1673 | 0.3346 |  |
| Total I |  |  |  |
| Additional Information |  |  |  |
| Delay costs | 0.3823 | 0.09681 |  |
| Internal accident costs |  |  |  |
| Material damages | : | : |  |
| Risk value | : | : |  |
| Environmental costs |  |  |  |
| Nature, landscape, soil and water pollution3) | 0.59398 |  |  |
| Nuclear risk |  |  |  |
| Total II |  |  |  |
| Revenues |  |  |  |
| User tariffs | : |  | : |
| Track charges | : |  | : |
| Station charges | : |  | : |
| Fuel tax | : |  | : |
| Eco tax | • |  | • |
| VAT2) | : |  | : |
| Subsidies | : |  | : |
| Basic data |  |  |  |
| Train Kilometre (million) | 116 |  | 9 |
| Passenger km (million) | 14879 |  | . |
| Tonne km (million) | . |  | 3778 |
| 1) Internal and external accident costs. - 2) VAT on fuel tax. - 3) No allocation to freight/passenger possible. Source: Certan et al., 2001. |  |  |  |

Table 89 shows the total costs of rail transport for passenger and freight transport.

Table 89: 	Total costs of rail transport: Dutch Railways (NS), € million at 1998 prices

|  | 1998 |  |  |
| --- | --- | --- | --- |
|  | Passenger | Freight | Total |
| Core information |  |  |  |
| Infrastructure costs | : | : | 1095.1 |
| Tracks |  |  |  |
| Fixed | : | : | : |
| Variable | : | : | : |
| Stations |  |  |  |
| Fixed | : | : | : |
| Variable | : | : | : |
| Supplier operating costs | : | : | 2339.2 |
| Out of these: track + station charges | : | : | : |
| External accident costs |  |  | 58.3 |
| Administrative | : | : | 0.1 |
| Health costs | : | : | 1.6 |
| Production loss | : | : | 56.6 |
| Environmental costs | 25.1 | 9.2 | 34.2 |
| Air pollution | 4.6 | 5.1 | 9.6 |
| Global warming | 1.1 | 1.1 | 2.2 |
| Noise | 19.4 | 3.0 | 22.4 |
| Total I |  |  | 3526.8 |
| Additional information |  |  |  |
| Delay costs | 44.35 | 0.87 | 45.22 |
| Internal accident costs | : | : |  |
| Material damages | : | : | : |
| Risk value | : | : | 97.2 |
| Environmental costs |  |  |  |
| Nature, landscape, soil and water pollution | 68.31 | 5.35 | 73.65 |
| Nuclear risk | : | : | : |
| Total II |  |  | 216.072) |
| Revenues | 1210 | 155 | 1365 |
| User tariffs | : | : | : |
| Track charges | : | : | : |
| Station charges | : | : | : |
| Fuel tax | : | : | : |
| Eco tax | • | • | • |
| VAT1) | : | : | : |
| Total | 1210 | 155 | 1365 |
| Subsidies | : | : | 81 |
| Basic data |  |  |  |
| Train kilometre (million) | 116 | 9 | 125 |
| Passenger km (million) | 14879 | . | 14879 |
| Tonne km (million) | . | 3778 | 3778 |
| 1) VAT on fuel tax. - 2) Excluding material damage. Source: Certan et al., 2001. |  |  |  |

## Public transport

Table 90: 	Dutch public transport account (bus) for 1996, 1998 and 2005 (€ million at 1998 prices)

| Costs |  |  |  |
| --- | --- | --- | --- |
| Core information | 1996 | 1998 | 2005 |
| Infrastructure Costs1) | 191.9 | 209.5 | : 7) |
| Fixed | : | : | : |
| Variable | : | : | : |
| Services |  |  |  |
| Supplier operating costs | 1589.9 | 1448 | 1647 |
| Accident costs (external) 2) | : | : | : |
| Environmental costs | : | : | : |
| Air pollution | 81.7 | 66.4 | : |
| Global warming | 14.0 | 12.6 | : |
| Noise2) | : | : | : |
| Additional information |  |  |  |
| Congestion costs | 108 | 116 | : |
| Accident costs (internal)2) | : | : | : |
| From this: risk value | : | : | : |
| Environmental costs |  |  |  |
| Nature and landscape, soil and water pollution4) | 10.28 | 8.34 | : |
| Nuclear risk | : | : | : |
| Revenues |  |  |  |
| Directly related to a specific cost category | : | : | : |
| Charges for infrastructure usage | : | : | : |
| Fixed | : | : | : |
| Variable | : | : | : |
| Subsidies for concessionary fares | : | : | : |
| User Tariffs3) | 1447 | 1396 | : |
| Other transport specific revenues |  |  |  |
| Fuel tax6) | 70.7 | 68 | : |
| Eco tax | : | : | : |
| VAT | : | : | : |
| Subsidies5) | 421.8 | 508 | : |
| 1) Tram and metro infrastructure. – 2) Buses are included in the road account, tram/metro not included. – 3) Subsidies and VAT are excluded. – 4) Because there is no standardised methodology for the calculation of these costs, the figures given here are to be regarded only as approximate indications that may change greatly over time with the development of a standard methodology. – 5) Subsidies included here include subsidies given for the provision of infrastructure, debt relief, provision of rail services etc. These subsidies can clearly not be allocated to either the cost or to the revenue side of this table. Subsidies are in cash flow terms and are not on the same basis as the economic costs. – 6) Buses only, they are included in the road account, too. – 7) Running costs are estimated at 95 million Euro. Source: Certan et al. (2001). |  |  |  |

Due to the organisation of the public transport and its specific characteristics it was not possible to draw a complete picture. The reasons behind that are mainly due to the fact that public transport is a representative part of the road sector and sometimes the respective shares of only public transport cannot be retrieved from the overall activity in the road sector. The table presents the obtained results for the public sector. One has to note that buses are included in the road account, too. Due to methodological and input data constraints the infrastructure costs, accident costs (busses included in the road account) and noise costs could not be quantified.

The overall costs of public transport amount to € 1.6 billion. The largest share of these costs is due to supplier operating costs, followed by congestion costs and environmental costs. The high share of congestion costs is not surprising taking into account that this is a mode operating in the urban areas.

The most considerable component of the revenues is the user charges which accounted for almost 85 % of the total estimated revenues.

Infrastructure costs. Since infrastructure costs relevant for busses were included in the road account, the estimation of infrastructure costs for public transport cover only the tram and metro infrastructure. The capital stock and capital costs for tram and metro infrastructure were calculated with the perpetual inventory model. The available data allowed only a very rough estimation of running costs.

Supplier operating costs. Data problems complicated a disaggregated estimation of supplier costs for public transport. Therefore, the estimation of the operating costs of the public sector is treated as whole rather than individually for bus, tram and metro.

Congestion costs. The estimation of congestion costs for public transport expressed as delay costs was possible only for busses and was based on modelling approach.

Accident costs. The transport system internal and external accident costs of urban public transport (that is buses) are included in the figures for road transport and presented separately in the public transport account. No information could be acquired for metros and trams.

Environmental costs. In the estimation of the environmental costs buses were assigned to the road account. However, when possible the results are presented in the public transport account separately. The costs of nuclear risk were not estimated due to the lack of information. It is believed, thought, that these costs play a minor role.

Taxes, charges and subsidies. Taxes and charges directly allocable to the use of infrastructure for public transport do not exist. Fuel taxation is included in the road account and presented in the public transport account separately. The figures for subsidies are total to the public transport (excluding rail); no information on subsides per individual mode was available.

In table 91 we have attempted to show the fully allocated costs of metro, tram and trolley bus services. As can be seen from the table more research is needed in this area. In order to obtain better figures for this transport mode some form of central data collection must be developed.

Table 91: Fully allocated costs of bus per vehicle km in Netherlands, €/km at 1998 prices

|  | 1998 |  |
| --- | --- | --- |
|  | Metro and tram | Bus |
| Core information |  |  |
| Infrastructure costs |  |  |
| Fixed | : | : |
| Variable | : | : |
| Supplier operating costs | : | : |
| External accident costs1) |  |  |
| Administrative | : | : |
| Health costs | : | : |
| Production loss | : | : |
| Environmental costs |  |  |
| Air pollution | : | 0.1099 |
| Global warming | : | 0.0209 |
| Noise | : | 0.0072 |
| Total I |  |  |
| Additional information |  |  |
| Delay costs | : | 0.0932 |
| Internal accident costs 1) | : | : |
| Material damages | : | : |
| Risk value | : | : |
| Environmental costs |  |  |
| Nature, landscape, soil and water pollution | : | : |
| Nuclear risk | : | : |
| Total II |  |  |
| Revenues |  |  |
| User tariffs | : | : |
| Subsidies | : | : |
| Basic data |  |  |
| Vehicle km (million) | : | 604 |
| Passenger km (million) | 6348 |  |
| 1) Bus included in road account. Source: Certan et al., 2001. |  |  |

## Aviation

Table 92: Dutch air transport account for 1996, 1998 and 2005 (€ million at 1998 prices)

| Costs |  |  |  |
| --- | --- | --- | --- |
| Core information | 1996 | 1998 | 2005 |
| Infrastructure Costs1) | 97 | 98 | : |
| Fixed | : | : | : |
| Variable | : | : | : |
| Accident costs (external) | 7.7 | 0.4 | : |
| Environmental costs | 198.46 | 226.44 | : |
| Air pollution | 21.5 | 24.9 | : |
| Global warming | 14.2 | 15.46 | : |
| Noise | 162.757 | 186.08 | 238.29 |
| Total | 303.16 | 324.84 | : |
| Additional information |  |  |  |
| Congestion costs | 44.55 | 89.34 | 118.6 |
| Accident costs (internal) | 47.7 | 0.9 | : |
| From this: risk value | 47.7 | 0.9 | : |
| Environmental costs | : | : | : |
| Nature and landscape, soil and water pollution2) | : | : | : |
| Nuclear risk2) | : | : | : |
| Revenues3) |  |  |  |
| Directly related to a specific cost category | : | : | : |
| Charges for infrastructure usage | : | : | : |
| Airport revenues | 210.9 | 224 | : |
| ATM charges | : | : | : |
| Meteorological services | : | : | : |
| Total |  |  |  |
| Loss of revenues due to tax exemptions |  |  |  |
| Kerosene tax | : | : | : |
| VAT on ticket price | : | : | : |
| Other transport specific revenues |  |  |  |
| Fuel tax | : | : | : |
| Eco tax | 1.8 | 1.3 | : |
| VAT | : | : | : |
| Subsidies | : | : | : |
| Non-transport related revenues of airports | : | : | : |
| 1) Only capital costs as part of total infrastructure costs. No running cost estimates available. - 2) Because there is no standardised methodology for the calculation of these costs, the figures given here are to be regarded only as approximate indications that may change greatly over time with the development of a standard methodology. - 3) Revenues for Amsterdam Schiphol airport and Maastricht Aachen airport. Source: Certan et al. (2001). |  |  |  |

Before presenting the results, one should note that due to the poor quality and incompleteness of the input data for air sector as a whole, the result have to be interpreted carefully on the basis of the assumptions, which were made. On the revenue side, only Amsterdam-Schiphol and Maastricht-Aachen were considered. The delay costs were quantified only for Schiphol airport. On the other hand, infrastructure costs, accident costs and environmental costs were estimated for the aviation as a whole.

The total costs of the air sector as quantified in the current aviation account amount to about € 0.4 billion, which is significantly lower than the costs of the other modes (except maritime). The largest share of costs is due to the environmental costs with slightly less than half of total costs. About 82% of the environmental costs represent the costs of noise.

Infrastructure related revenues amounted to € 224 million in 1998 which is more than the quantified infrastructure costs.

Infrastructure costs. The capital costs were calculated with the PIM. Due to the lack of input data the running costs could not be estimated.

Congestion costs. Delay costs were estimated for Amsterdam Schiphol airport using punctuality information. The results were not extrapolated for the rest of the airports. A benchmark of 15 minutes was applied according to the international practice in air transport.

Accident costs. The accident costs were calculated based on the information on the aviation accidents in the accounting years including injuries and material damage. Based on the available information the accidents were classified in classes of accidents per degree of severity. The major accident cost for aviation is the risk value.

Environmental costs. For the estimation of environmental costs input data only at the total level could be obtained. The major role of the environmental cost can be attributed to the noise costs. Costs due to nature, landscape, soil and water pollution were not calculated as a result of the insufficient information necessary to perform the calculations.

Taxes, charges and subsidies. On the revenue side estimation cover only airport taxes and taxes on noise for Amsterdam-Schiphol and Maastricht-Aachen airports. No specific information on subsides could be obtained. Only rough figures could be obtained; they were not included in the accounts.

In table 93 the fully allocated costs of passenger and cargo air transport per movement are shown.

Table 93:	Fully allocated costs of aviation per movement for Netherlands (€/km at 1998 prices)

|  | 1998 |  |
| --- | --- | --- |
|  | Passenger | Cargo |
| Core information |  |  |
| Infrastructure costs | : | : |
| Fixed | : | : |
| Variable | : | : |
| External accident costs |  |  |
| Administrative |  |  |
| Health costs |  |  |
| Production loss |  |  |
| Environmental costs1) | : | : |
| Air pollution |  |  |
| Global warming2) | 13.53 |  |
| Noise |  |  |
| Total I |  |  |
| Additional information |  |  |
| Delay costs | 244.14 | 35.63 |
| Internal accident costs | : | : |
| Material damages |  |  |
| Risk value |  |  |
| Environmental costs | : | : |
| Nature, landscape, soil and water pollution |  |  |
| Nuclear risk | • | • |
| Total II |  |  |
| Revenues |  |  |
| Charges for infrastructure usage |  |  |
| Airport revenues | : | : |
| ATM charges | : | : |
| Meteorological services | : | : |
| Fuel tax | : | : |
| Eco tax | : | : |
| VAT | : | : |
| Subsidies |  |  |
| Exemption for kerosene tax | : | : |
| Exemption of VAT on ticket price | : | : |
| Basic data |  |  |
| Passenger (movements) 3) | 364092 | • |
| Cargo (movements) 3) | • | 12718 |
| 1) No allocation to passenger/cargo possible. – 2) CO2 direct, per movement. - 3) Amsterdam Schiphol airport only. Source: Certan et al., 2001. |  |  |

In table 94 the total costs are disaggregated between passenger and freight transport.

Table 94: 	Total costs of aviation in Netherlands (€ million at 1998 prices)

|  | 1998 |  |  |
| --- | --- | --- | --- |
|  | Passenger | Cargo | Total |
| Core information |  |  |  |
| Infrastructure costs | : | : | 98 |
| Fixed | : | : | : |
| Variable | : | : | : |
| External accident costs | : | : | 0.4 |
| Administrative | : | : | 0 |
| Health costs | : | : | 0 |
| Production loss | : | : | 0.3 |
| Environmental costs1) | : | : | 226.44 |
| Air pollution | : | : | 24.9 |
| Global warming | : | : | 15.46 |
| Noise | : | : | 186.08 |
| Total I |  |  | 324.84 |
| Additional information |  |  |  |
| Delay costs | 88.89 | 0.45 | 89.34 |
| Internal accident costs | : | : | 0.9 |
| Material damages | : | : | 0 |
| Risk value | : | : | 0.9 |
| Environmental costs1) |  |  |  |
| Nature, landscape, soil and water pollution | : | : | : |
| Nuclear risk | : | : | : |
| Total II | 88.89 | 0.45 | 89.34 |
| Revenues1) 6) |  |  |  |
| Charges for infrastructure usage |  |  |  |
| Airport revenues | : | : | 224 |
| ATM charges | : | : | : |
| Meteorological services | : | : | : |
| Fuel tax | 0 | 0 | 0 |
| Eco tax2) | : | : | 1.3 |
| VAT3) | 0 | 0 | 0 |
| Total |  |  | 225.3 |
| Subsidies | : | : | : |
| Total |  |  |  |
| Basic data |  |  |  |
| Passenger km (bill) | 584) | . | . |
| Tonne km (mill) | . | 65) | . |
| 1) No allocation to passenger/cargo possible. - 2) Tax on noise. – 3) VAT on fuel tax. – 4) Estimation for KLM only. – 5) Estimation for KLM and Martinair. – 6) For Amsterdam Schiphol airport and Maastricht Aachen airport. Source: Certan et al., 2001. |  |  |  |

## Inland waterway transport

Table 95:	Dutch inland waterway account for 1996, 1998 and 2005 (€ million at 1998 prices)

| Costs |  |  |  |
| --- | --- | --- | --- |
| Core information | 1996 | 1998 | 2005 |
| Infrastructure costs – inland waterways | 3282.42) | 3694.33) | :7) |
| Fixed | : | : | : |
| Variable | : | : | : |
| Infrastructure costs – inland waterway harbours1) | 82 | 85 | : |
| Fixed | : | : | : |
| Variable | : | : | : |
| Accident costs (external) | 5.1 | 3.6 | 4.4 |
| Environmental costs | 236 | 280 | : |
| Air pollution | 236 | 280 | : |
| Global warming | : | : | : |
| Noise | : | : | : |
| Total | 3523.5 | 3977.9 | : |
| Additional information |  |  |  |
| Congestion costs4) | : | : | : |
| Accident costs (internal) | 20.2 | 13.9 | 15.8 |
| From this: risk value | 20.2 | 13.9 | 15.8 |
| Environmental costs |  |  |  |
| Nature and landscape, soil and water pollution6) | 104 | 100 | : |
| Nuclear risk6) | . | . | . |
| Revenues |  |  |  |
| Directly allocable | : | : | : |
| Charges for infrastructure usage | : | : | : |
| Fixed |  |  |  |
| Variable |  |  |  |
| Total |  |  |  |
| Other transport specific revenues | : | : | : |
| Fuel tax |  |  |  |
| Eco tax |  |  |  |
| VAT |  |  |  |
| Subsidies 5) | : | : | : |
| Non-transport related revenues of ports | : | : | : |
| 1) Only capital costs. – 2) From this € 192.4 million are running costs. Waterways capital costs amount to € 3008 million. – 3) From this € 178.3 million are running costs. Waterways capital costs amount to € 3431 million.. – 4) Expressed as delay costs. – 5) Subsidies included here include subsidies given for the provision of infrastructure, for debt relief, for the provision of services etc. These subsidies can clearly not be allocated to either the cost or to the revenue side of this table. Subsidies are in cash flow terms and are not on the same basis as the economic costs. – 6) Because there is no standardised methodology for the calculation of these costs, the figures given here are to be regarded only as approximate indications that may change greatly over time with the development of a standard methodology. – 7) Running costs were estimated at 235 million Euro. Source: Certan et al., 2001 |  |  |  |

The results for the inland waterways show its relative significance in comparison with other modes. The infrastructure costs play the major role in IWW transport. In 1998 infrastructure costs consisted more than 90% of the total quantified costs. The comparative low figures for environmental and accident costs show that IWW is an environmental friendly mode with low accident costs.

Within the UNITE project it was not possible to quantify the revenues from the inland navigation.

Infrastructure costs. The perpetual inventory model was used for calculating capital stock and capital costs, separately for inland waterways, inland waterway harbours and seaports. Information on running costs of inland waterways were obtained from both the Statistics Netherlands (CBS) and CE (1999).

Congestion costs. No information on delays in the inland waterway transport could be found. Based on the information supplied by the operators, delays are negligible, therefore, the delay costs were not estimated.

Accident costs. Accident costs for inland waterway transport were calculated on the basis of a detailed database on all accidents. Based on the records regarding different types of damage and injuries which occurred, accidents were classified in classes by severity of the accident and damage involved.

Environmental costs. Environmental costs for inland waterway transport were estimated for air pollution and nature and landscape cost categories. Input data on emissions was of good quality and covered different types of vessels. Waterborne transport is the second important mode for air pollution after road sector. The costs of nature and landscape are rough estimates due to the specific features of the waterways infrastructure which are not taken into account in the model which was used.

Taxes, charges and subsidies. Due to lack of information it was not possible to estimate the revenues of inland waterways.

## Maritime shipping

Table 96: 	Dutch inland waterway account for 1996, 1998 and 2005 (€ million at 1998 prices)

| Costs |  |  |  |
| --- | --- | --- | --- |
| Core information | 1996 | 1998 | 2005 |
| Infrastructure costs – maritime | : | : | : |
| Fixed |  |  |  |
| Variable |  |  |  |
| Infrastructure costs – sea harbours1) | 160 | 177 | : |
| Fixed | : | : | : |
| Variable | : | : | : |
| Accident costs (external) | 1.5 | 1.1 | 2.0 |
| Environmental costs2) |  |  |  |
| Air pollution | : | : | : |
| Global warming | : | : | : |
| Noise | : | : | : |
| Total | 161.5 | 178.1 | : |
| Additional information |  |  |  |
| Congestion costs4) | : | : | : |
| Accident costs (internal) | 8 | 4.3 | 9.6 |
| From this: risk value | 8 | 4.3 | 9.6 |
| Environmental costs2) |  |  |  |
| Nature and landscape, soil and water pollution6) | : | : | : |
| Nuclear risk6) | . | . | . |
| Revenues |  |  |  |
| Directly allocable | : | : | : |
| Charges for infrastructure usage | 218.2 | 221 | : |
| Fixed | : | : | : |
| Variable | : | : | : |
| Total | 218.2 | 221 | : |
| Other transport specific revenues | : | : | : |
| Fuel tax |  |  |  |
| Eco tax |  |  |  |
| VAT3) |  |  |  |
| Subsidies 5) | : | : | : |
| Non-transport related revenues of ports | : | : | : |
| 1) Only capital costs. – 2) Environmental costs presented for Europe in a separate report. – 3) VAT levied on fuel tax. – 4) Expressed as delay costs. – 5) Subsidies included here include subsidies given for the provision of infrastructure, for debt relief, for the provision of services etc. These subsidies can clearly not be allocated to either the cost or to the revenue side of this table. Subsidies are in cash flow terms and are not on the same basis as the economic costs. – 6) Because there is no standardised methodology for the calculation of these costs, the figures given here are to be regarded only as approximate indications that may change greatly over time with the development of a standard methodology. Source: Certan et al., 2001 |  |  |  |

For maritime sector results only for infrastructure costs and accident costs on the costs side, and rough estimates on revenues can be presented. Environmental costs were estimated for Europe as a whole with emissions data from all relevant countries, including the Netherlands.

Based on the obtained result, infrastructure costs are lower than revenues directly related to the use of infrastructure. The total accidents of maritime amounted to € 5 million and were the lowers as comparative to the other sectors. Infrastructure costs together with accident costs amounted to around € 182 million in 1998.

Infrastructure costs. The perpetual inventory model was used for calculating capital costs for sea harbours. It was very difficult to obtain information regarding running costs. Therefore, running costs could not be calculated precisely.

Accident costs. Accident costs for maritime were calculated on the basis of a detailed database on all accidents. Based on the records regarding different types of damage and injuries which occurred, accidents were classified in classes by severity of the accident and damage involved.

Environmental costs. The environmental costs for maritime shipping – covering air pollution costs - are calculated within UNITE for Europe as a whole and do not cover countries individually. The results will be available at a latter stage of UNITE. The other cost categories were not estimated.

Taxes, charges and subsidies. Revenues in maritime shipping that relate to the use of infrastructure are charges, fees and other payments at seaports, and pilotage charges. No detailed information could be gathered, therefore only rough estimation of total is provided. No quantitative information on subsidies was available.

Table 97 and 98 show the fully allocated and total costs of water transport disaggregated by shipping mode. It is obvious, that more research is needed in these areas.

Table 97: 	Fully allocated costs of IWW and maritime per vehicle km in Netherlands (€/km at 1998 prices)

|  | 1998 |  |
| --- | --- | --- |
|  | IWW | Maritime |
| Core information |  |  |
| Infrastructure costs | : | : |
| Inland waterway harbours | : | : |
| Fixed | : | : |
| Variable | : | : |
| Inland waterways | : | : |
| Fixed | : | : |
| Variable | : | : |
| Sea harbours | : | : |
| Fixed | : | : |
| Variable | : | : |
| External accident costs1) | 0.215 | : |
| Administrative | : | : |
| Health costs | : | : |
| Production loss | : | : |
| Environmental costs | : | : |
| Air pollution | 2.884) | : |
| Global warming | : | : |
| Noise | : | : |
| Total I |  | : |
| Additional information |  |  |
| Delay costs | 0 | 0 |
| Internal accident costs3) | : | : |
| Material damages | : | : |
| Risk value | : | : |
| Environmental costs | : | : |
| Nature, landscape, soil and water pollution | : | : |
| Nuclear risk | : | : |
| Total II |  |  |
| Revenues |  |  |
| Charges for infrastructure usage | : | : |
| fixed | : | : |
| variable | : | : |
| Fuel tax | : | : |
| Eco tax | : | : |
| VAT | : | : |
| Subsidies | : | : |
| Basic data |  |  |
| Vessel km | 81 | : |
| 1) Both external and internal accident costs. - 2) Ecological tax. - 3) Within core account. – 4) Exclusive direct CO2 for freight vessels. Source: Certan et al., 2001. |  |  |

Table 98:	Total costs of IWW and maritime in Netherlands (€ million at 1998 prices)

|  | 1998 |  |
| --- | --- | --- |
|  | Inland waterways | Maritime shipping |
| Core information |  |  |
| Infrastructure costs | 3694.3 | 177 |
| Inland waterways | : | • |
| Fixed | : | • |
| Variable | : | • |
| Inland waterway harbours | : | • |
| Fixed | : | • |
| Variable | : | • |
| Sea harbours | • | 177 |
| Fixed | • | : |
| Variable | • | : |
| External accident costs |  |  |
| Administrative | 0 | 0 |
| Health costs | 0.2 | 0 |
| Production loss | 3.4 | 1.0 |
| Environmental costs | 380.11 | : |
| Air pollution | 279.8 | : |
| Global warming | : | : |
| Noise | : | : |
| Total I |  |  |
| Additional information |  |  |
| Delay costs | 0 | 0 |
| Internal accident costs |  |  |
| Material damages | 0 | 0 |
| Risk value | 13.9 | 4.3 |
| Environmental costs |  |  |
| Nature, landscape, soil and water pollution | 100.31 | : |
| Nuclear risk | : | : |
| Total II |  |  |
| Revenues |  |  |
| Charges for infrastructure usage | : | 221 |
| fixed | : | : |
| variable | : | : |
| Fuel tax | : | : |
| Eco tax | : | : |
| VAT | : | : |
| Subsidies | : | : |
| Basic data |  |  |
| Tonne km (million) | 59.1 | 96.9 |
| Source: Certan et al., 2001. |  |  |

# Conclusions

This reports present the results of the first attempt to develop a pilot transport account for the Netherlands including total social costs of the transport sector per mode and per specific category. The methodological approach for the presented account was elaborated within UNITE project as described in Link et al. (2000) based on valuation conventions as presented in Nellthrop et al. (2001). The results show that most of the proposed objectives were achieved. However, there is a lot of space for future improvements, development and deepness of the results. The presented in this paper transport account can be further completed with results from relevant studies available in the Netherlands (this was not within the purpose of this report). In general lines, all cost categories described in Link et al. (2000) could be qualified covering all transport modes.

Infrastructure costs could be estimated for road, national rail, airports and waterborne system. Good results were obtained for capital costs. Due to the characteristics of the road infrastructure, infrastructure costs could not be estimated for the public transports as a separate mode. The allocation of costs was not done to avoid the arbitrary allocation to transport users.

Supplier operating costs were calculated for the public transport, that is national railways and buses, trams and metro. It was not possible to estimates operating costs for individual transport means, one operator can provide services for several modes. Consequently, it is not possible to derive from the business accounts the respective shares of costs.

Congestion costs (calculated as delay costs) could be calculated for all modes of transport studied but waterborne transport. These delays are assumed to be negligible and therefore not attempt for their estimation was done.

Accident costs were estimated for all transport modes. The major parts of accident costs, namely the risk value, the costs due to production losses and the health costs were calculated for all transport modes. The further parts of accident costs, e. g. administrative costs of accidents and costs of material damages to vehicles were estimated for some of the transport modes depending on the data situation. The results have to be interpreted with care since a number of assumptions were made and some cost categories (e.g. legal costs) were not estimated. Accident costs for public transport could not be estimated. Only estimation of accidents related to bus accidents was possible. No information was available of the accidents of trams and metros.

Within the environmental cost category air pollution costs and the costs of global warming were estimated for all transport modes except marine shipping. These costs are estimated for the EU as a whole based on data from individual countries and will be presented in a separate report within UNITE project. Noise costs were calculated for road, rail and air transport. The cost associated with nuclear risk arising form electricity production were not estimated. Furthermore, it was also possible to compile figures for the costs associated with nature, landscape, soil and water pollution road, rail and air transport. The interpretation of these results is sensible, however. One should note that in the case of waterways and trunk roads (>9m) the network length has decreased between the base year and accounting year. In the road sector this might be due to the change in the methodological registration of the length or any other reasons which could not be clarified within the framework of this reporting period. In the waterborne transport, the decreasing length of the waterways is due to and compensated by the increasing attention and investments to widening and deepening of the waterways. The elaborated method does not take into account these aspects. Therefore, these issues remain an open question for future research.

The taxes and charges for road, rail, public transport and air transport could be calculated partially. Subsidies for rail, public transport were documented but might be incomplete, therefore considered rough estimates. Partial revenues for maritime shipping were estimated, but no actual data can be presented for inland waterways.

Compared to existing transport accounts type of information the pilot accounts presented in this report have achieved considerable progress in terms of methodologies used, consistency of both methodologies and data across modes of transport and types of costs and quality of data and empirical estimates. In the following we can draw conclusions with respect to two questions:

- How can the results be interpreted and used for transport policy?

- What are the future challenges to improve the pilot accounts?

## The relevance of the pilot accounts for transport policy

Sansom et al. (2000) raises the question of how the estimation of total and average costs and revenues contribute to the priority areas of transport policy identified to be relevant for the UNITE project. Indeed, this question is important since first best pricing rules refer to marginal cost, not average cost. Sansom et al. (2000) identifies than three main areas to which the UNITE accounts contribute: (1) equity, (2) efficiency, (3) financial viability. In the light of the results within UNITE so far and also considering the remaining gaps in the Netherlands pilot accounts it is now possible to clarify more precisely how the accounting results can be used in these areas.

Equity: As stated in Sansom et al. (2000) there is no unique definition of equity, but equity quite obviously refers to the relation between the costs imposed by an economic subject and the charges paid. This relationship can have different dimensions: income classes or even individual transport users, vehicle classes (for example HGV versus passenger cars), regional differences or country differences (for example port charging, non-discriminatory road user charging in cross-country transport, international rail track access charging). The pilot accounts presented in this report give indications on equity between modes (intermodal comparisons), between types of transport (passenger versus freight transport) and between vehicle classes.

For the Dutch transport policy this is an important issue in the ongoing policy debate on the kilometre charge. Here it can contribute to the question whether the general charge level and the charge levels between vehicle classes are properly defined. However, the pilot accounts cannot be used to assess equity between individual users or user groups defined by income classes.

Efficiency: If cost recovery is a binding constraint, second best pricing principles are relevant. This, however, requires information on the costs to be covered in order to guarantee that the mark-ups on marginal costs are sufficient to meet the cost-recovery goal. On the other hand, this information is essential in order to monitor that there is no overcharging. The issue of avoiding overcharging is also dealt with in the directive on rail infrastructure charging which states that mark-ups over marginal costs must not exceed total costs. The UNITE pilot accounts provides this total cost information. Furthermore, although not estimated in the current pilot account, when the shares of fixed and variable costs can be estimated, the results give an indication to what extent it would be worthwhile to subsidise parts of the fixed costs from tax revenues. This refers to the information which the pilot accounts provide both at the cost side and the revenue side.

Financial viability: Again, if cost recovery is a binding constraint, either since private operators have to recover their costs or due to political/budget reasons, it is necessary to have knowledge on the level of total costs as presented here in the pilot accounts. It is extremely important for an appropriate monitoring by governments and regulators. One example for this is the rail sector: if marginal cost pricing is introduced and the revenues from track access charges are not sufficient to recover total cost, the state has to subsidies the deficit. In this case where rail companies negotiate with the government on subsidies it is essential for the government and/or the regulator to know the total costs to be covered and the extent of subsidies really necessary for covering the deficit.

For all potential uses of the pilot accounts it should, however, be noted that they reflect the actual, rather than the ideal accounts and cannot be considered to supply the absolute total of all transport related costs and revenues. Therefore a simple adding up and comparison of the costs and revenues within the modal accounts described in this report supplies the reader only with the specific costs and revenues found using the methodology described in Link (2000b). Although the accounts are comprehensive they can not be considered to be all inclusive. This leads to the conclusions for future work.

## Open questions and future improvements

During the elaboration of the pilot account a number of difficulties in its elaboration have were met. The main difficulty occurred in the collection of input data. Taking into account the high ambitions of the proposed pilot account, some of the information could not be obtained because it is not currently collected in statistics (neither at national level not at business unit level), other information could not be obtained in the proposed time-period, and finally, there is very detailed information which is available but could not be obtained due to its confidentiality (e.g. delays in rail sector). Regarding the data problems for public transport with tram, metro and trolley buses they refer mainly to infrastructure and supplier operating costs, e. g. those costs which are monetary costs (in contrast to environmental, accident and congestion costs which have to be monetarised). These companies do not have a separate bookkeeping for infrastructure and operation and they usually do not provide separate figures for buses, trams, metros.

Having compiled the current pilot account, a number of weaknesses could be underlined which serve as basis for future improvements and further research. The main gaps in the Dutch pilot account refer to the modes with a relatively big number of transport operators (tram, metro, bus and waterways) or modes with relatively big node infrastructure (waterways, maritime, aviation). The pilot accounts for these modes and specific cost categories do not fully cover the specific mode or total costs.

An exemplification to that is the maritime account which contains very limited in information. Delays in the aviation sector cover only the Amsterdam-Schiphol airport. Supplier operating costs for public transport cover only the largest companies. In general, it was not possible to consider bus transport in a systematic way for all cost categories. This results in a split between the road account (for example for infrastructure costs) and the public transport account (for example for supplier operating costs). This can cause confusion when researching and interpreting the accounts and must be documented clearly for transparency in the results.

The above being mentioned, there are lot of opportunities for future improvements and developments. Some of them are summarised in the following text.

- Improve the quality of the existing input data (accidents: unit costs, time requirements

for the accidents, insurance coverage; supplier operating costs; taxes, charges and

subsidies etc).

- It should be mentioned that the estimation of subsidies was not based on a systematic

definition and analysis which would have been too time consuming. So far, figures refer

to parts of subsidies only. Here clearly a potential for future improvement is given

although it might be time-consuming.

- Develop the input data by its further disaggregation (by different vehicle type, by type

of infrastructure, by motives, by time periods etc).

- Develop the methodology (e.g. nature and landscape in general and in particular for

waterways).

- Develop methodologies for the allocation of costs and diversification of fixed costs

from variable costs. This will enlarge the use of the current pilot account in particular

for equity issues.

- Develop further the analytical and empirical analysis of the results. This will allow the

understanding of the facts behind figures and values.

Compare the obtained results (as well as methodologies and input data used) with the

relevant results from other studies and research as well as looking for opportunities to

incorporate already existing results in the current pilot account.

- Periodical update of the Dutch pilot account. In addition, future efforts could be

attributed to provide pilot account for future years. This might serve as an important

building block for future policies. It has to be noted that due to the lack of reliable input

data and lack of forecasts for the input values required for the estimation of individual

cost categories no forecasts were estimated in this report.

# References

AEA (2001) Association of European Airlines: AEA Punctuality Data. Circulate of the AEA Secretariat. Brussels, February.

Amsterdam Airport Schiphol (1998), Jaarverslag Schiphol 1998, Schiphol.

Amsterdam Airport Schiphol (2000), Statistical Annual Review 2000, Schiphol.

Beleidsplan verkeersveiligheid goederen vervoer over de weg 2001-2006, versie 11 juli 2001, Ministerie van Verkeer en Waterstaat, Directie Transportveiligheid.

Berdowski, J., Mulder, W., Veldt, C., Visschedijk, A., Zandveld, P. (1997) Particulate matter (PM10 - PM2.5 - PM0.1) emissions in Europe. TNO-report TNO-MEP-R 96/472, TNO, Appeldoorn, The Netherlands.

Bickel P, Schmid S, Friedrich R Maibach M, Doll C, Tervonen J, Enei R (2000): Accounts Approach for Environmental Costs, Version 0.6, Stuttgart, UNITE (UNIfication of accounts and marginal costs for Transport Efficiency). Contained in Link et al. (2000), Accounts Approach. Funded by 5th Framework RTD Programme. ITS, University of Leeds, Leeds October.

Brink van den R.M.M. (2000), Verkeer en Vervoer in de Milieubalans 2000, RIVM, December.

Builtjes, P. (1992) The LOTOS-Long Term Ozone Simulation project, Summary report, TNO-report, IMW-R 92/240, 1992.

Capros, P. and Mantzos, L. (2000) Kyoto and technology at the European Union: costs of emission reduction under flexibility mechanisms and technology progress, Int. J. Global Energy Issues, 14, pp. 169-183.

CBS, Statistiek van het goederenvervoer 1996-1999, Voorburg/Heerlen.

CBS, Statistiek van het personenvervoer 1996-1999, Voorburg/Heerlen.

CBS, Statistiek van de wegen 1996, Voorburg/Heerlen.

CBS, Statistiek van de motorvoertuigen 1996, Voorburg/Heerlen.

CBS, Statistiek van de luchtvaart 1998, Voorburg/Heerlen.

CBS, De mobiliteit van de Nederlandse bevolking 1999, Voorburg/Heerlen.

CBS, Zakboek verkeer en vervoer 1997, Voorburg/Heerlen.

CBS, Tijdreeksen verkeer en vervoer 1997, Voorburg/Heerlen.

CBS, Statistisch Jaarboek 1996-1999, Voorburg/Heerlen.

CBS (1998), Verkeersongevallen 1998, Voorburg/Heerlen.

CBS (1999), Milieucompendium 1999, Het milieu in cijfers, CBS, RIVM.

CBS (2000), Statistical Yearbook 2000 of the Netherlands, Voorburg/Heerlen.

CE (1999), Efficiënte prijzen voor het verkeer, Delft, October 1999.

CEC (1998) White paper, Fair Payment for Infrastructure Use: A phased approach to a common transport infrastructure charging framework in the EU. COM(98) 466 final. European Commission, Brussels.

Derwent, RG, Dollard, GJ, Metcalfe, SE (1988) On the nitrogen budget for the United Kingdom and north-west Europe. Q.J.R. Meteorol. Soc. (1988), 114, pp.1127-1152.

Dijkstra W.J., Dings J.M.W. (1997), Specific energy consumption and emissions of freight transport, A comparison between road, water, rail and air, CE, December.

Doll C, Lindberg G, Niskanen, E. (2000) Accounts Approach for Accidents. UNITE (UNIfication of accounts and marginal costs for Transport Efficiency). Contained in Link et al. (2000), Accounts Approach. Funded by 5th Framework RTD Programme. ITS, University of Leeds, Leeds October.

Doll C. (2001) Pilot accounts nature and landscape Netherlands 1998 (and 1996), Draft. UNITE (UNIfication of accounts and marginal costs for transport efficiency). Funded by 5th Framework RTD Programme.

Duerinck, J. (2000) Personal communication based on: Duerinck, J. et al. (1999) Prospective study of emissions in Belgium until 2008/2012 of the greenhouse gases included in the Kyoto Protocol. Costs and potential measures and policy instruments to reduce GHG emissions, Vito (Mol) & KU-Leuven.

ECMT (1999) Round Table 110: Road Traffic Congestion.

Eurocontrol/CODA (2000): Annual report on AFTM-delays 1998.

European Commission (1999) ExternE Externalities of Energy. Vol 10 – National Implementation. A Report produced for the EC – DG XII, Luxembourg, Office of Publications for the European Communities, Luxembourg.

European Commission (1999a) ExternE Externalities of Energy. Vol 7 – Methodology 1998 update. A Report produced for the EC – DG XII, Luxembourg, Office of Publications for the European Communities, Luxembourg.

European Commission (1999b) ExternE Externalities of Energy. Vol 10 – National Implementation. A Report produced for the EC – DG XII, Luxembourg, Office of Publications for the European Communities, Luxembourg.

European Environment Agency (1998): Spatial and Ecological Assessment of the TEN; Demonstration of Indicators and GIS Methods. Progress Report of the DG VII-DG-XI-Eurostat-EEA Working Group of the SEA of the TEN, Copenhagen.

Friedrich, R. and Bickel, P. (eds.) (2001) Environmental External Costs of Transport. Springer Verlag, Heidelberg.

INFRAS (1999) Handbook of emission factors (Handbuch Emissionsfaktoren des Straßenverkehrs, HBEFA). Umweltbundesamt (UBA), Berlin, and Bundesamt für Umwelt, Wald und Landschaft (BUWAL), Bern.

INFRAS/IWW (2000) External Costs of Transport, Study for the International Union of Railways, Paris.

Link H, Dodgson J, Maibach M, Herry M (1999) The Costs of Road Infrastructure and Congestion in Europe, Heidelberg, New York.

Link, H. (2000 a) Accounts Approach for Infrastructure Costs. UNITE (UNIfication of accounts and marginal costs for Transport Efficiency). Contained in Link et al. (2000), Accounts Approach. Funded by 5th Framework RTD Programme. ITS, University of Leeds, Leeds October.

Link H, Stewart L, Maibach M et al. (2000 b) Accounts Approach. UNITE (UNIfication of accounts and marginal costs for Transport Efficiency) Deliverable 2 Funded by 5th Framework RTD Programme. ITS, University of Leeds, Leeds October.

Link H, Stewart L et al. (2001a) The Pilot Account for Germany. UNITE (UNIfication of accounts and marginal costs for Transport Efficiency) Deliverable 5 Funded by 5th Framework RTD Programme. ITS, University of Leeds, Leeds September.

Link H, Stewart L et al. (2001b) Pilot Accounts – Results for Germany and Switzerland. Draft, UNITE (UNIfication of accounts and marginal costs for Transport Efficiency) Deliverable 5 Funded by 5th Framework RTD Programme. ITS, University of Leeds, Leeds September.

Maastricht Aachen Airport (1998), Jaarverslag 1998, Maastricht.

Maibach M, Rothengatter W, Banfi S, Doll C, Sieber N, Zuber J (2000): External Costs of Transport. Study for the International Railway Union (UIC). Zurich, Karlsruhe, Paris, April 2000.

Macario et al. (2000) Accounts approach for Taxes, Charges and Subsidies. UNITE (UNIfication of accounts and marginal costs for Transport Efficiency). Contained in Link et al. (2000), Accounts Approach. Funded by 5th Framework RTD Programme. ITS, University of Leeds, Leeds, October.

McInnes, G. (1996) Atmospheric Emission Inventory Guidebook, A joint EMEP/CORINAIR Production, First Edition. EMEP Task Force on Emission Inventories, European Environment Agency, Copenhagen.

Ministerie van Verkeer en Waterstaat (1997), Filekosten op het Nederlandse hoofdwegennet in 1997.

Ministerie van Verkeer en Waterstaat (1998a), Jaarbericht Vervoerend Nederland 1998.

Ministerie van Verkeer en Waterstaat (1998b), Vervoer-Economische Verkenningen 1998-2003,Sdu Uitgevers, Den Haag, 1998.

Ministerie van Verkeer en Waterstaat (1999a), Meerjarenprogramma Infrastructuur en Transport 1999-2003.

Ministerie van Verkeer en Waterstaat (1999b), Kosten ten gevolge van verkeersongevallen in Nederland, 1997, Heerlen, April 1999.

Ministerie van Verkeer en Waterstaat (2000), Verkeersgegevens Jaarrapport 1999.

Nellthorp, J., Sansom, T., Bickel, P., Doll, C. and Lindberg, G. (2001) Valuation Conventions for UNITE, UNITE (UNIfication of accounts and marginal costs for Transport Efficiency). Contained in Link et al. (2001), Pilot Accounts – Results for Netherlands and Switzerland. Funded by 5th Framework RTD Programme. ITS, University of Leeds, Leeds, April.

Nederlandse Spoorwegen, Jaarverslagen 1996-1999, Utrecht.

NVVP Beleidsopties verkend, Ministerie van Verkeer en Waterstaat, Directoraat-Generaal Rijkswaterstaat, Advies Verkeer en Vervoer, Rotterdam, januari 2001.

Personen- en goederenmobilitiet in 2010 en 2020, Prognoses in het kader van de CPB-LT scenario’s en de Nationale Milieuverkenning 4, Ministerie van Verkeer en Waterstaat, Directoraat-Generaal Rijkswaterstaat, Advies Verkeer en Vervoer, october 1997.

Posch M, Hettelingh JP, de Smet P, Downing RJ (1997) Calculation and Mapping of Critical Thresholds in Europe: Status Report 1997. RIVM Report No. 259101007. Coordination Center for Effects, National Institute of Public Health and the Environment (RIVM), Bilthoven, Netherlands.

Rabl, A, Spadaro, J (1999) Environmental Damages and Costs: an analysis of Uncertainties. Paper accepted for publication in Environment International.

RailNed (1998), Jaarverslag 1998, Utrecht.

Richardson, S. (2000) Atmospheric Emission Inventory Guidebook, A joint EMEP/CORINAIR Production, Second Edition. European Environment Agency, Technical Report, No. 30, Copenhagen.

Rotterdam Airport (1998), Jaarverslag 1998, Rotterdam.

Sansom, T., Nellthorp, J., Proost, S., Mayers., et al. (2000) The Overall UNITE Methodology UNITE (UNIfication of accounts and marginal costs for Transport Efficiency). Funded by 5th Framework RTD Programme. ITS, University of Leeds, Leeds, May.

Simpson, D, Olendrzynski, K, Semb, A, Storen, E, Unger, S (1997) Photochemical oxidant modelling in Europe: multi-annual modelling and source-receptor relationships. EMEP/MSC-W Report 3/97, Oslo.

Simpson, D. and Eliassen, A. (1997): Control strategies for ozone and acid deposition - an iterative approach, Norwegian Meteorological Institute, EMEP MSC-W Note 5/97.

Stichting Consument en Veilgheid (1997), Seminar Kosten van letsels, Preceedings van het seminar over kosten van letsels, gehouden op 19 november 1996 in Breukelen, Februari.

Stichting Consument en Veilgheid (2000), Hertog, P.C. et al., Ongevallen in Nederland 1997/1998, een enqueteponderzoek onder slachtoffers van ongevallen. April.

Suter, S., Sommer, H., Marti, M. et al. (2001) The Pilot Accounts for Switzerland Contained in Link et al. (2001), Pilot Accounts – Results for Netherlands and Switzerland. Funded by 5th Framework RTD Programme. ITS, University of Leeds, Leeds, April.

SWOV (1995), Kosten van verkeersonveilligheid in Nederland 1993, Muizelaar, J. e.a., Leidschendam.

Transport en Logistiek Nederland (2000), Transport in cijfers, TLN 2000.

Tweede Kamer der Staten-Generaal, Railveiligheid, 26 699, vergaderjaar 1998-1999, Sdu Uitgevers, ‘s-Gravenhage 1999.

UN (1993) United Nations, Systems Approach to National Accounts Compilation, A technical Report.

Van A naar Beter, Mobiliteit verkend, feiten achter het NVVP, Ministerie van Verkeer en Waterstaat, Directoraat-Generaal Rijkswaterstaat, Advies Verkeer en Vervoer, Rotterdam, november 2000.

Van A naar Beter, Nationaal verkeers- en vervoersplan 2001-2020, Kabinetsstandpunt (deel 3), Ministerie van Verkeer en Waterstaat, Den Haag, mei 2001.

Vestreng, V. (2000) Tables of 1998 sector emissions in the ECE region. Pollutants: sulphur, nitrogen oxides, ammonia, NMVOCs, carbon monoxide, methane, carbon dioxide. Co-operative programme for monitoring and evaluation of the long range transmission of air pollutants in Europe (EMEP). http://projects.dnmi.no/~emep/emis_tables/ stab1.html, Last Update: 9/2000, Status from: 3/2001

Vestreng, V. and Støren, E. (2000) Analysis of UNECE/EMEP Emission Data, MSC-W Status Report 2000. Co-operative programme for monitoring and evaluation of long range transmission of air pollutants in Europe (EMEP), EMEP MSC-W Note, 1/00, Blindern, Norway.

Wesemann, P., Bos, J.M.J. (1998), Recente kerncijfers verkeersonveiligheid, SWOV, Leidschendam.

# Glossary

| Accident Costs | Costs caused by transport accidents. These costs are directly related to material damage costs and medical costs, the administrative costs of police and insurance companies, the costs associated with production loss through accident related illness and fatalities and the costs of „suffering„ associated with accidents (risk value). |
| --- | --- |
| Capital costs | The capital costs comprise the consumption of fixed capital and interest. Capital costs represent a high share of total infrastructure costs and are different to the annual capital expenditures. |
| Capital value | The capital value is the value of fixed capital measured either as a gross or a net value. The gross value represents the capital value of all assets still physically existing in the capital stock. It can thus be considered as an equivalent of production capacity. The net value represents the value of assets minus the meanwhile consumed fixed capital. The difference to the gross value is thus the loss of value due to foreseen obsolence and the normal amount of accidental damage which is not made good by normal repair, as well as normal wear and tear. Methods for estimating capital values are the direct method (synthetic method) and the indirect method (perpetual inventory concept). |
| Congestion | Congestion arises when traffic exceeds road capacity so that the travelling speed of vehicles is slowed down. It can be defined as a situation where traffic is slower than it would be if traffic flows were at low levels. The definition of these „low levels„ (reference level) is complicated and varies from country to country (e.g. six service levels in the American HCM). |
| CORINAIR | Programme to establish an inventory of emissions of air pollutants in Europe. It was initiated by the European Environment Agency Task Force and was part of CORINE (COoRdination d’Information Environmentale) work programme set up by the European Council of Ministers in 1985. End of 1994 the EEA’s European Topic Centre on Air Emissions (ETC/AEM) took over the CORINAIR programme. |
| Earmarking | Direct interlinkages between the financial source and the financial purpose, in order to secure financial resources. In practice, specific funds are used therefore (e.g. earmarking road pricing revenues and financing of road infrastructure or environmental measures). |
| GDP | (= Gross Domestic Product). The GDP is the sum of all goods and services produced within a country and a year. GDP per capita can be regarded as the relative economic power of a country per inhabitant. |
| GVW | GVW is the gross vehicle weight and contains the weight of the vehicle itself and the weight of the payload. |
| HGV | HGV means heavy goods vehicles. Within this study they are defined as all goods vehicles with a maximum GVW equal or more than 3,5 tons. |
| Impact Pathway Approach (IPA) | Methodology for externality quantification developed in the ExternE project series. It follows the chain of causal relationships from pollutant emission via dispersion (including chemical transformation processes), leading to changes in ambient air concentrations from which impacts can be quantified using exposure-response functions. Damages are then calculated using monetary values based on the WTP approach. |
| Individual transport | Transport performed on the own account of users with their own vehicle for private reasons. |
| Infrastructure Cost | Cost category which comprises capital costs (depreciation and interests) and running costs for maintenance and repair, operation and administration, overheads and traffic police. |
| Infrastructure suppliers | are defined as the totality of public and private enterprises which are financing the provision and maintenance of the transport infrastructure for all modes (road, rail and water) within the urban area analysed. |
| NUTS | Nomenclature of territorial units for statistics; level 0 = countries, level III = départements, Kreise, etc. (depending on country considered). |
| Opportunity costs | The expressions "opportunity costs" and "shadow prices" are used synonymously within the Real Cost Scheme. They determine the value added for an individual in the case a good would not have been bought or built or in case negative effects of transport would not be present. Opportunity values are used for the evaluation of investments (capital costs), lost lives (statistical value of human life) or for the assessment of noise nuisance. |
| Passenger car unit | (= PCU) PCU is used in order to standardise vehicles in relation to a passenger car. Speed and lengths differentials are most common. |
| Perpetual-inventory method | Perpetual inventory model: This is a method to estimate the asset value from a time series of annual investment expenditures. Annual new investments are cumulated and - according to their remaining life time - a depreciation will be calculated. The sum of these annual remaining asset values is equal to the total amount of the asset value. |
| PPP | PPP means purchasing power parity. PPPs are the rates of currency conversions which equalise the purchasing power of different countries. This means that a given sum of money, when converted into different currencies at the PPP rates, will buy the same basket of goods and services in all countries. In particular, PPPs are applied if figures for specific products or branches shall be expressed in foreign currency (for example in ECU or in US $) because in these cases the use of official exchange rates is not appropriate. |
| Primary particles | Particles, that are directly emitted. |
| Public Transport | PT subsumes all services that are supplied according to a pre-defined timetable in passenger and freight transport. The final user here pays an average fare. Typical PT is rail, bus, air and ferry services. The transport of an additional person or unit of goods does not cause in the short run additional vehicle kilometres, as scheduled vehicles are used, which are running anyway. In the long run, due to increased capacity use, additional or larger vehicles have to be scheduled. In the former case the marginal costs are zero, in the latter case the marginal costs are the costs per vehicle kilometre divided by the capacity use. |
| Replacement value/cost | The cost of replacing a particular asset of a particular quality with an asset of equivalent quality. Replacement cost may exceed the original purchase cost because of changes in the prices of the assets. |
| Risk value | The risk value represents the society’s willingness to pay for avoiding death casualties or injuries in transport. It reflects the decrease in social welfare due to the suffering and grief of the victims and their relatives and friends. The relevant cost elements are: Own risk value and suffering and grief of relatives and friends |
| Secondary particles | Particles, such as nitrates and sulphates, that are formed in the atmosphere through atmospheric chemical reactions. |
| Supplier Operating Cost | Costs mainly related to costs incurred by supplier in its operations. |
| Survival function | Survival functions are used in rather refined perpetual inventory models. The survival function g (i) is based on the assumption that the service lives of assets within an investment vintage are dispersed around the mean. g (i) explains then which share of investments within an investment-vintage still exists in the capital stock after i years. The survival function is characterised by a downwards slope of shares between 100 % (in the first year of investment) and 0 % (after exceeding the maximal lifetime of all assets in the investment vintage). |
| Synthetic method | One of the two main methods to value the existing road network (see also: perpetual inventory method). The synthetic method values the road network by estimating what it would cost to replace the road network with assets of equivalent quality. The method therefore involves measuring the existing physical assets, in terms of road length of particular types, bridges, etc, and then multiplying these measures of physical assets by unit replacement costs, such as the cost of constructing a motorway with the same physical characteristics as the existing one. |
| Vehicle category | Road: passenger car, motorcycle, bus, goods transport vehicles. Public transport: bus, tram, trolley bus, metro. Rail: electric passenger train, diesel passenger train, electric goods train, diesel goods train. Inland Waterways / Marine: Goods transport. Air: passenger, goods transport |
| VOSL | Value of statistical life: An unit often used to express individuals willingness-to-pay (WTP) for safety. The individual state (or reveal) a WTP for a small reduction in risk (dz) for a fatal accident; he is never asked the question about the value of life per se. If this risk change is summed over (n) individuals so that statistical the risk reduction will save one life we can also sum their WTP; this sum of the WTP then becomes the Value of statistical life (VOSL). VOSL = WTP*n = WTP/dz if n*dz = 1 |
| VOT | Value of time. The value of time is standardised within the UNITE accounts. |
| WTP | Willingness to pay: The direct or indirect response to questionnaire about individuals willingness-to-pay for a good. For example the WTP for higher safety. |
| YOLL | Year of life lost |

# Abbreviations

| AVV/BG | Netherlands Transport Research Centre, Department of Statistics and Data Management |
| --- | --- |
| bill. | billion |
| CBS | Statistics Netherlands |
| CE | Centre for Energy Consumption and Environmental Technology |
| CO2 | Carbon dioxide |
| COI | Cost of illness |
| dB(A) | Decibel, weighted with the A-filter. Logarithmic unit of sound pressure level. |
| EMEP | European Monitoring and Evaluation Programme |
| GDP | Gross Domestic Product |
| GIS | Geographical Information System |
| GVW | Gross vehicle weight (weight of the vehicle itself and the weight of the payload) |
| HGV | Heavy goods vehicles (goods vehicles with a maximum GVW equal or more than 3,5 tonnes) |
| kph | Kilometres per hour |
| kWh | Kilowatt hour |
| LAeq | Energy equivalent noise level |
| LGV | Light goods vehicles (goods vehicles with a maximum GVW less than 3,5 tonnes) |
| LTO | Landing and take-off cycle |
| mill. | Million |
| MWh | Megawatt hour |
| n.a. | No data available |
| NMHC | Hydrocarbon |
| NMVOC | Non-methane volatile organic compounds |
| NOx | Nitrogen oxides (mix of NO and NO2) |
| NS | Dutch railways |
| NUTS | Nomenclature of territorial units for statistics; level 0 = countries, level III = départements, Kreise, etc. (depending on country considered) |
| PCU | Passenger car unit |
| PIM | Perpetual Inventory Model |
| PM10 | Fine particles with a diameter of 10 µm and less |
| PM2.5 | Fine particles with a diameter of 2.5 µm and less |
| PPP | Purchasing power parity |
| PT | Public transport |
| RIVM | Research for Man and Environment |
| RLD | Aviation Department |
| SOC | Supplier operating costs |
| SO2 | Sulphur dioxide |
| SRWP | Steady reduction of working power |
| SWOV | Institute for Road safety Research |
| UPT | Urban public transport |
| v-hours | Vehicle hours |
| v-km | Vehicle kilometres |
| VOC | Volatile organic compounds |
| VOT | Value of time |
| WTP | Willingness to pay |
| YOLL | Years of life lost |

# Abbreviations used in data tables

| – | No existing data category (for example sea ports in Switzerland) |
| --- | --- |
| 0 | Zero or approximately zero when compared to other data entries |
| . | Not applicable (for example the length of a sea harbour) |
| : | No data available |

A differentiation of the share of travel purposes by train class was not available. Thus, a constant distribution for the whole passenger transport market was used.

Due to the non-availability of monetary estimates of groundwater pollution this item is - as stated in IR 9.2 - not considered explicitly.