COMPETITIVE AND SUSTAINABLE GROWTH

(GROWTH)

PROGRAMME

Deliverable 9: Accident Cost Case Studies (Case Study 8c) The marginal cost of road/rail level crossing accidents on Swedish railways

Version 1.7

10 February 2002

Authors: 
Gunnar Lindberg (VTI)

Contract: 1999-AM.11157 Project Coordinator: ITS, University of Leeds Funded by the European Commission 5th Framework – Transport RTD

UNITE Partner Organisations ITS/UNIVLEEDS (UK), DIW (De), NEI (Nl), CES/KUL (Be), TIS.PT (Pt), IWW/UNIKARL (De), VTI (Se), IER/USTUTT (De), CERAS/ENPC (Fr), HERRY (Au), EIET/ULPGC (Es), ISIS (It), STRATEC (Be), SYSTEMA (Gr), JP-TRANSPLAN (Fi), VATT (Fi), ECOPLAN (Ch), INFRAS (Ch), EKONO (Fi), EKI (Se)

UNITE

1999-AM.11157

UNIfication of accounts and marginal costs for Transport Efficiency

Deliverable 9:	Accident Cost Case Studies: Case Study 8c/1 - The marginal cost of road/rail level crossing accidents on Swedish railways

This document should be referenced as:

Lindberg, G, Deliverable 9: Accident Cost Case Studies, Case Study 8c/1: The marginal cost of road/rail level crossing accidents on Swedish railways. (UNIfication of accounts and marginal costs for Transport Efficiency) Deliverable 9. Funded by 5th Framework RTD Programme. ITS, University of Leeds, Leeds..

10 February 2002

Version 1.7

Authors:	as above, with contributions from other UNITE partners

PROJECT  INFORMATION

Contract no: 1999-AM.11157:

UNIfication of accounts and marginal costs for Transport Efficiency

Website:	www.its.leeds.ac.uk/unite

Commissioned by: European Commission – DG TREN; Fifth Framework Programme

Lead Partner:   Institute for Transport Studies, University of Leeds (UK)

Partners:  ITS/UNIVLEEDS (UK), DIW (De), NEI (Nl), CES/KUL (Be), TIS.PT (Pt), IWW/UNIKARL (De), VTI (Se), IER/USTUTT (De), CERAS/ENPC (Fr), HERRY (Au), EIET/ULPGC (Es), ISIS (It), STRATEC (Be), SYSTEMA (Gr), JP-TRANSPLAN (Fi) VATT (Fi), ECOPLAN (Ch), INFRAS (Ch), EKONO (Fi), EKI (Se)

DOCUMENT  CONTROL  INFORMATION

Status: 	Accepted

Distribution:	UNITE partners

Availability:	Public once accepted

Filename:	D9 8b

Quality assurance

Coordinator’s review:	C A Nash

Signed:          		Date:

Table of Contents	Page:

1	Introduction	3

1.1	Swedish accident charges	3

1.2	What should be included in a charge?	4

1.3	Swedish railway accidents	4

2	The marginal cost theory	5

3	Database	6

4	Marginal accident cost at level crossings	11

4.1	Accident model	12

4.2	Accident function	14

4.3	External marginal accident cost	17

5	Conclusion	19

References	30

Appendix	32

# Introduction

Since the pioneering separation of the Swedish railway monopoly in 1988, into a Track Authority (Banverket) and a Railway company (SJ), the operator(s) have been charged for the use of the infrastructure. The charge shall in principle be based on the marginal costs of infrastructure damage, including damage on aerial lines, marginal accident and environmental costs. The accident charge is today 0.90 SEK/trainkm, which is 1/3 of the charges related to train kilometre.

While proposals on new charges over the years adhere to the principle of marginal costs, no proper theory has been developed and consequently, no proper marginal cost analysis have been carried out. The present paper will discuss a theory of external accident cost and analyse the external marginal cost at rail/road level crossing.

After section 1.1 below, which describes the Swedish charges, follows a short discussion on what to include in an accident charge and a presentation of Swedish Railway accident statistics. Section 2 presents the marginal cost theory and section 3 the used database. This section contains a rather extensive presentation of the rail/road level crossing accidents during the years 1995 to 1999. In section 4 we discuss the model structure (4.1) and estimate a road/rail level crossing accident model (4.2) from which we develop the external marginal accident cost (4.3). Section 5 offers some conclusions.

## Swedish accident charges

The original accident charge, introduced 1988, was based on a cost allocation principle. The total external accident cost in railway accidents where allocated to the total number of train kilometre. External cost was defined as all accident costs excluding costs for rail passengers and staff. In the original proposal, the charge was differentiated between track categories, based on the standard of the track. However, the final charge was a uniform charge of 1.60 SEK/trainkm. Subsequently updating of the accident charge has increased the charge but not accepted new valuation of accidents.

In a proposal 1997, the Swedish National Rail Administration (Banverket) took the position that the costs for road/rail accidents were irrelevant for the charge (Banverket 1997). In addition, the cost of other accidents where in the proposal reduced (to ¼) based on the argument that these accidents occurred when people illegally walked on the track. The rest of the principles from 1988 were retained. Based on this proposal a new charge was introduced in 1998, which did not include road/rail accidents. However, the government did not accept the argument that the charge for other accidents should be reduced due to illegal behaviour. The charge was differentiated around an average of 0.90 SEK/trainkm with 1.10 SEK/trainkm for passenger train and 0.55 SEK/trainkm for freight train (SFS 1998:1827). Since 1998 the charge is, also in the legal text, named ‘accident charge’.

Table 1-1: Swedish rail accident charges (total cost, traffic volume and charge per kilometre)

| Included costs | Charge 1988 (SFS 1988:1378) | Banverket 1997 proposal | Accident charge 1998 (SFS 1998:1827) |
| --- | --- | --- | --- |
| Passengers/Staff (MSEK) | 17 | 5.6 | 5.6 |
| Rail-Road crossings (MSEK) | 89 | 0 | 0 |
| Other accidents (MSEK) | 59 | 27 | 85.9 |
| Total cost (MSEK) | 165 | 32.6 | 91.5 |
| Train km (million) | 103.7 | 100 | 100 |
| Charge (SEK/km) | 1.60 A) | 0.33 | 0.91 |

A) SFS 1988:1378, 8§ Driftsavgift 1.60 SEK/trainkm for diesel train. For electrical train the ‘drifstavgift’ is 1.75 SEK/trainkm, which includes a charge for maintainance of the aerial line.

## What should be included in a charge?

The discussion about what should be included in the charge has been rather woolly. It has been a ‘feeling’ that accidents due to illegal behaviour should not be included. It has also been a discussion on how to allocate the cost between rail and road users. The solution, as often with cost allocation models, has been an arbitrary allocation of the cost.

In an optimal safety policy both the protection device, the legal system, enforcement and the traffic volumes will be optimal. The optimal protection device will probably not ensure zero accidents independently of the human behaviour. With the legal system and enforcement, the society tries to ensure optimal safety behaviour. However, this optimal behaviour will probably not be a ‘zero-fault’ behaviour. Consequently, in this optimal system, safety issues will affect the optimal railway traffic. In addition, in the decision on the optimal railway traffic, illegal behaviour among other users has to be expected.

On the other hand, the system may not be in an optimal stage. We will then have a discussion on the optimal traffic volume based on a second best approach. While the outcome of this second best is not clear, it is clear that it exists principles that will guide us through such a discussion. We do not need to ‘feel’ that it should be in a certain way.

With the marginal cost theory and empirical estimation lined out in the rest of this paper, the allocation problem is resolved with a simple marginal cost theory.

## Swedish railway accidents

Banverket is since 1997 responsible for the collection of Swedish railway accident statistics on state owned railways. All the main tracks are owned by the state (75% of the length). Of the private railways are 59% tracks at industrial sites and the rest museum tracks and similar (Inlandsbanan).

A railway accident is defined as an accident involving train in motion and where persons have been killed or severe injured, within or outside the train, or the cost is above 10.000 ECU. Fatality is defined in the same way as in road accident statistics, i.e. fatality within 30 days of the accidents. Severe injury is defined as persons that have a certificate of illness for more than 14 days.

Table 1-2: Railway accident statistics, 1995 - 1999

| Accidents |  |  |  |  |  | Personal injuries | Fatality |  |  |  |  | Severe Injury |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1995 | 1996 | 1997 | 1998 | 1999 |  | 1995 | 1996 | 1997 | 1998 | 1999 | 1995 | 1996 | 1997 | 1998 | 1999 |
| Train in motion |  |  |  |  |  | Passenger | .. | 0 | 2 | 1 | 0 |  | 2 | 3 | 1 |  |
| collision | .. | 2 | 2 | 4 | 16 | Railway employees | .. | 0 | 11 | 1 | 2 |  | 6 | 10 | 7 | 2 |
| derailment | .. | 12 | 17 | 10 | 15 | Other at level crossings | .. | 14 | 8 | 7 | 12 |  | 3 | 5 | 2 | 17 |
| other | .. | 8 | 6 | 9 | 3 | Other incl suicideA) | .. | (2) | 72 | 60 | 58 |  | (1) | 8 | 2 | 3 |
| Shunting |  |  |  |  |  | of which suicide | .. | .. | 66 | 46 | 52 |  | n.a. | n.a. | n.a. | n.a. |
| derailment | .. | 12 | 12 | 13 | 17 | Other electricity acc. | .. | - | 3 | 2 | 1 |  | - | 1 | 1 | 2 |
| collision | .. | 6 | 3 | 7 | 6 | TOTALA) | (27) | (16) | 96 | 71 | 73 | (13) | 12 | 27 | 13 | 26 |
| other | .. | 3 | 5 | 1 | 6 |  |  |  |  |  |  |  |  |  |  |  |
| Other accidents |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| fire | .. | 7 | 18 | 8 | 24 |  |  |  |  |  |  |  |  |  |  |  |
| level crossings | .. | 34 | 35 | 25 | 42 |  |  |  |  |  |  |  |  |  |  |  |
| TOTAL | .. | 84 | 98 | 77 | 129 |  |  |  |  |  |  |  |  |  |  |  |

Source: Banverket Statistik over olyckor på statens spåranläggningar 1998 and dito 1990; for 1995, SIKA, Transport och Kommunikation, Årsbok 1998

A) Excluding suicide 1995 and 1996; Not available (..)

If we ignore the frightening amount of suicides, road/rail level crossing accidents was the most frequent cause fatalities in the railway system 1999 (57%). The second largest categpry were other accidents with non-rail staff or passengers (33% in 1999). Railway is a safe system for the train passengers (0 fatalities 1999) while railway employees have a higher probability to be killed or injured (10% of the fatalities 1999). Turning to the accidents the same pattern occur; around 1/3 of all accidents were road/rail level crossing accidents in 1999.

# The marginal cost theory

The number of accidents where trains are involved (A), is a function of the traffic volume of trains (QT) and other explanatory variables, including the traffic volume of cars at level crossings (equation (1) below). Naturally, A should be seen as a vector with rows representing different degree of severity, which also is the case for the cost components; willingness-to-pay of the involved user (a), ditto of relatives and friends (b) and finally system external cost (c), i.e. mainly medical costs paid by the social security system. The marginal cost (MC) with respect to the train traffic volume (QT) follows naturally from the total cost (TC) (equation 2 and 3). We derive the external marginal cost as (4) where PMC is the private marginal cost already internalised by the train operator.

A =f(QT,......) 	(1)

TC =Aa+b+c	(2)

MC = ∂A∂QT (a+b+c)	(3)

MCe = MC−PMC	(4)

The legal system allocates the full responsibility to the road user in road/rail level crossing accidents (XX). Consequently, the train operator does not bear any of the road users accident cost. Ignoring the train operators own accident cost, the external marginal cost at road/rail level crossings is the same as the marginal cost. However, the model can easily be generalised to comprise a situation where the cost burden is split between the parts in the collision. Based on Lindberg (2001), we introduce r as the accident risk for trains (5) and θ as the share of total accident costs that falls on the train operator. The comprehensive expression on the external marginal cost can be written as (7) where we have introduced a risk elasticity E.

r = AQ 	(5)

E = ∂r∂Q Qr	(6)

MCe =r(1−θ+E)a+b+rc(1+E)	(7)

As we have assumed θ to be nil for train operators in road/rail level crossings, the external marginal cost (7) is equal to expression 4 above. The key function to understand is the relationship between accidents and train traffic volume. However, the costing of an accident is complicated and uncertain, so we will here use existing official values.

# Database

Data on road/rail level crossing accidents during five years, 1995-1999, have been assembled from Banverket. The data includes information on location of the crossing, total number of accidents (ACCID), number of fatalities (FATAL), severe (SEVERE) and light injuries (LIGHT) and if the involved train was a passenger train or not. The total cost (TC) of each accident has been calculated using official Swedish accident valuations (which excludes the b-value). This total cost excludes the train operator’s own damage cost.

Table 3-3: Fatalities, personal injuries, accidents and accident cost per year 1995 to 1999

|  | 95-99 | 95 | 96 | 97 | 98 | 99 |
| --- | --- | --- | --- | --- | --- | --- |
| ACCID | 794 | 165 | 188 | 139 | 135 | 167 |
| FATAL | 48 | 6 | 13 | 8 | 6 | 15 |
| SEVERE | 16 | 3 | 2 | 4 | 2 | 5 |
| LIGHT | 63 | 8 | 11 | 12 | 24 | 8 |
| PACCID | 81 | 15 | 19 | 14 | 14 | 19 |
| TC (kSEK) | 732650 | 94200 | 191450 | 125800 | 94000 | 227200 |
| Prop double track | 0.081 | 0.081 | 0.115 | 0.080 | 0.060 | 0.061 |
| Prop pass.train | 0.514 | 0.235 | 0.444 | 0.577 | 0.556 | 0.525 |

PACCID = Number of personal injury accidents

CAS = Number of casualties (=FATAL+SEVERE+LIGHT)

The dataset consist of 794 accidents, of which 81 included personal injuries. A total of 127 persons were killed or injured in road/rail level crossing accidents over the five-year period. The total cost of level crossing accidents was 146 MSEK annually. This is 65% above the cost used in the original estimates of the charge in 1988. Around 8% of the accidents occurred at a double track and in 51% of the cases a passenger train was involved.

The definition of accidents (ACCID) is not the same in Banverket’s database as in their accident report presented in Table 1-2 above. The published reports have a restriction on the minimum cost of the accident to be included while the database includes all accidents. We expect ACCID to include a number of minor accidents.

Data has also been collected from Banverket covering all existing rail/road level crossings in Sweden 1997 with information on protection device (P), type and name of the passing road (R), if the track is a single or double track (UNE =1 for double) and information of the location of the crossing. We have excluded pedestrian crossings. For each segment, the annual number of passenger (QP) and freight trains (QF) for year 1996 and 1998 have been collected from the Swedish Railways (SJ). The dataset consist of 8600 road/rail level crossings on tracks, which were used in 1996 or(and?) 1998.

Table 3-4: Road type and Protection device

| Code in database (dummy=1) |  | Name | Code at Banverket |
| --- | --- | --- | --- |
| Road Type |  |  |  |
| R1 | Main road (Riksväg) |  | GR,R |
| R2 | County road (Länsväg) |  | GL,L |
| R3 | Street (Gågata, övrig gata, gata) |  | GG,ÖG,G |
| R4 | Other road (Övrig väg) |  | Ö |
| R5 | Minor private road (Ägoväg) |  | ÄG,Ä |
| Protection device |  |  |  |
| P1 | Full barriers |  | A,A+,Ab,AF,AFH,AH |
| P2 | Half barriers |  | B,BF,BH,BOM |
| P3 | Open crossing w. flash light |  | C,C1,C3,CD,CD3,CDGF,D,DGF |
| P4 | Open crossing w. St Andrew cross |  | K,K+,K1,K2,K3,K4,K5 |
| P4 | Minor private road light (Ägovägslampa) |  | E,E+ |
| P0 | No protection device |  | -+,- |

Road traffic volume has been included manually for main roads and county roads based on information from Swedish National Road Administration (Vägverket). In a large part of Banverkets database road number was missing. This has been manually corrected. In addition, the road-number of the nearest road/road crossing has manually been included to be able to identify the unique segment of the road that crosses the railway. The road traffic flow (QR) on these segments has been included in the database. However, it was only possible to add road traffic flow for 981 of the 8600 crossings.

The accident database is linked to the database of crossings 1997. For 20 crossings with accidents, we could not find the relevant crossing in the database of crossings.

Table 3-5: Descriptive statistics, all crossings and crossings where road traffic was observed 1995-1999

| 5 years accidents | All road/rail level crossings |  |  |  |  | Road/rail level Crossings where road traffic was observed |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mean | Std.dev | Min | Max | NumCases | Mean | Std.dev | Min | Max | NumCases |
| UNE | 0.026 | 0.159 | 0 | 1 | 8538 | 0.044 | 0.205 | 0 | 1 | 975 |
| QR | 1040.760 | 1519.400 | 0 | 10700 | 982 | 1041.820 | 1519.810 | 12 | 10700 | 981 |
| QP96 | 3380.510 | 4599.890 | 0 | 66800.9 | 8600 | 4947.790 | 6191.230 | 0 | 62640.9 | 981 |
| QF96 | 1973.390 | 2881.490 | 0 | 18232.3 | 8600 | 2662.000 | 3301.090 | 0 | 18232.3 | 981 |
| QTOT96 | 5353.900 | 6333.700 | 0 | 71160.7 | 8600 | 7609.790 | 8043.350 | 0 | 66638.2 | 981 |
| QP98 | 3426.860 | 4726.890 | 0 | 66800.9 | 8600 | 5122.330 | 6356.080 | 0 | 62640.9 | 981 |
| QF98 | 1988.610 | 2958.240 | 0 | 18232.3 | 8600 | 2666.400 | 3464.540 | 0 | 18232.3 | 981 |
| QTOT98 | 5415.470 | 6439.760 | 0 | 71160.7 | 8600 | 7788.730 | 8148.640 | 71 | 66638.2 | 981 |
| FATAL | 0.004 | 0.088 | 0 | 5 | 8600 | 0.006 | 0.078 | 0 | 1 | 981 |
| SEVERE | 0.001 | 0.039 | 0 | 2 | 8600 | 0.000 | 0.000 | 0 | 0 | 981 |
| LIGHT | 0.005 | 0.107 | 0 | 7 | 8600 | 0.015 | 0.131 | 0 | 2 | 981 |
| CAS | 0.010 | 0.148 | 0 | 7 | 8600 | 0.021 | 0.158 | 0 | 2 | 981 |
| PACCID | 0.007 | 0.085 | 0 | 1 | 8600 | 0.019 | 0.138 | 0 | 1 | 981 |
| ACCID | 0.077 | 0.413 | 0 | 12 | 8600 | 0.298 | 0.774 | 0 | 9 | 981 |
| TC | 63.483 | 1257.350 | 0 | 71000 | 8600 | 89.144 | 1109.650 | 0 | 14350 | 981 |
| P1 | 0.139 | 0.346 | 0 | 1 | 8520 | 0.417 | 0.493 | 0 | 1 | 981 |
| P2 | 0.118 | 0.322 | 0 | 1 | 8520 | 0.401 | 0.490 | 0 | 1 | 981 |
| P3 | 0.081 | 0.273 | 0 | 1 | 8520 | 0.154 | 0.361 | 0 | 1 | 981 |
| P4 | 0.132 | 0.338 | 0 | 1 | 8520 | 0.027 | 0.161 | 0 | 1 | 981 |
| P0 | 0.530 | 0.499 | 0 | 1 | 8520 | 0.002 | 0.045 | 0 | 1 | 981 |
| R1 | 0.007 | 0.083 | 0 | 1 | 7282 | 0.036 | 0.186 | 0 | 1 | 977 |
| R2 | 0.136 | 0.343 | 0 | 1 | 7282 | 0.822 | 0.383 | 0 | 1 | 977 |
| R3 | 0.112 | 0.316 | 0 | 1 | 7282 | 0.016 | 0.127 | 0 | 1 | 977 |
| R4 | 0.352 | 0.478 | 0 | 1 | 7282 | 0.125 | 0.331 | 0 | 1 | 977 |
| R5 | 0.392 | 0.488 | 0 | 1 | 7282 | 0.001 | 0.032 | 0 | 1 | 977 |

Half of the observed crossings (53%) had no protection device (P0), 14% had full barriers (P1), 13% was open crossings with S:t Andrews cross or similar (P4), 12% had half barriers (P2) and 8% had flashing lights or similar (P3).

The annual number of passenger trains was 3427 (Qp98) and 1989 freight trains (Qf98) in 1998. The total number of trains was 5415 (Qtot98). For the subset of the data where we have identified the road traffic flow the total number of trains where 44% higher, 7789 (Qtot98). The highest train traffic flow was identified at crossings with full and half barriers (P1 and P2),(see figure below).

Figure 1 Train passages per year by protection device

Around 0.7% of the crossing roads where trunk roads (R1), 14% county roads (R2), 11% other street (R3) and as much as 35% other road (R4) and 39% minor private roads (R5). For the subset of the data where we have found reliable estimates on road traffic flow as much as 82% where county roads. Level crossings which where passed by trunk or country roads had usually full (P1), half barriers (P2) or flash lights (P3). The unprotected crossings (P0) was most often used for minor private roads and the open crossings with S:t Andrew cross were used for other roads (R4).

Figure 2: Type of passing road by crossing type

The average traffic of crossing road vehicles (QR) was 1041 per day (AADT). No information exists on the traffic volume on small private roads used mainly by farmers and the forest industry.

Figure 3 Passing cars per day by protection device (977 observations)

The risk per crossing was 77 accidents (ACCID) during a five-year period per 1000 crossings or 7 personal injury accidents (PACCID) per 1000 crossings during the same period. The relationship between all accidents (ACCID) and only personal injury accidents (PACCID) by protection device suggests that the reporting principles differ between crossing types. For the level crossings with full barriers or half barriers (P1, P2), the personal injury accidents is below 10% of all reported accidents while it for unprotected (P0) and unknown crossings are around 50% (Table 3-6). In Figure 4 below we have summarised the risk based on PACCID, ACCID and similar information from the manuals of the railway cost-benefit analysis at Banverket.

Figure 4: Annual accidents per crossing, ACCID and PACCID

The accidents cost per crossing and year is 12.7 kSEK. The lowest cost is for full barriers (P1) and the highest cost, 39.7 kSEK, is for the open crossings with S:t Andrew cross or similar (P4).

Table 3-6: Annual accidents by severity and total accident cost per crossing and cost per train passages 1995 - 1999

| Annual | unknown | 0 | 1 | 2 | 3 | 4 | All |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Accidents and costs per crossing |  |  |  |  |  |  |  |
| FATAL | 0.000000 | 0.00013 | 0.00051 | 0.00199 | 0.00144 | 0.00268 | 0.00084 |
| SEVERE | 0.000000 | 0.00009 | 0.00051 | 0.00020 | 0.00058 | 0.000535 | 0.00026 |
| LIGHT | 0.000000 | 0.00018 | 0.00068 | 0.00279 | 0.00260 | 0.001784 | 0.00095 |
| CAS | 0.000000 | 0.00040 | 0.00169 | 0.00498 | 0.00462 | 0.004996 | 0.00205 |
| PACCID | 0.000000 | 0.00031 | 0.00135 | 0.00299 | 0.00375 | 0.003568 | 0.00146 |
| ACCID | 0 | 0.00062 | 0.05616 | 0.04726 | 0.01241 | 0.0067797 | 0.01546 |
| PACCID/ACCID |  | 50% | 2% | 6% | 30% | 53% | 9% |
| TC/crossing(kSEK) | 0 | 2.143 | 8.600 | 29.252 | 22.381 | 39.661 | 12.696 |
| Accidents per million passing train |  |  |  |  |  |  |  |
| PACCID/Q | 0 | 0.085 | 0.137 | 0.341 | 0.842 | 0.665 | 0.271 |
| ACCID/Q | 0 | 0.170 | 5.710 | 5.392 | 2.785 | 1.263 | 2.856 |
| Accident cost per passing train (SEK/train) |  |  |  |  |  |  |  |
| AC |  | 0.586 | 0.875 | 3.337 | 5.023 | 7.389 | 2.344 |

Per passing train, the cost varies from 0.59 SEK/train on the unprotected crossings (P0) to 7.4 SEK/train on the open crossings with S:t Andrew cross (P4). The average cost is 2.3 SEK/train.

Figure 5: Average cost per train passage by crossing type 1995 – 1999 (SEK/train)

# Marginal accident cost at level crossings

We only consider road/rail level crossing accidents and costs related to personal injuries. This means that all costs fall on the road user and consequently, we do not need to split the cost between different user groups. The key function to understand becomes the relationship between train traffic volume and accident cost. Will the risk increase, be constant or decline with increased train traffic? Our data presented in the figure below suggest that the risk will decline with increased train traffic. The marginal cost can be expected to fall below the average cost.

Figure 6: Accident risk, all accidents (divided with 10) and personal injury accidents only 1995-1999

## Accident model

An influential Swedish study, TFD (1981), studied 489 road/rail crossing accidents in Sweden during the period 1973 to 1977. Crossings with different types of protection devices showed great variation in accident risks. Full and half barriers have the lowest risk. Open crossings with flash lights have about ten times as high risk and open crossings with S:t Andrew cross had four times higher than the flash light crossings.

The result showed that the sight distance appeared to be an important factor of risk at open crossings with flash light and crossings with S:t Andrew cross. At a number of open crossings w. flashlight the road and railway run in parallel, and close to each other, before the crossing. When the car and train runs in the same direction the risk was seven times higher in these crossings. Driver age affect the number of accidents such that cars driven by the youngest group are more frequent than expected. Lindberg (1994) add distracting elements such as buildings and T-road crossings as another important factor.

In a following report, the problem with open crossings with flash lights were analysed with the focus on road users behaviour (TFD (1983)). The results suggested that in crossings with restricted vision, fewer drivers moved their head and very few look behind them when the road and railway run in parallel. The number of drivers showing movement of their head increased with the number of tracks as well as with the number of trains per day. A higher road vehicle speed reduced the number of drivers that moved their head.

A number of quantitative models, but with a more aggregated approach, can be found in the literature. Coleman and Stewart (1977) predict the number of accidents per crossing as a function of only the number of passing trains and passing cars for different protection devices in urban and non-urban areas. Validation between observed and predicted values suggested good fit for S:t Andrew cross and flashlight but less accurate for automatic barriers and stop sign.

Coulombre et al. (1982) use an index for exposure based on the train/road traffic flow product (QRQT). In addition, the model includes numbers of tracks, passing trains during daytime, train speed, passing cars, road type and number of road lanes. Three different models were estimated for different protection devices, flashlights, barriers and other. The predicted result was in a second stage combined with historical data to predict the number of accidents. Hauer and Persaud (1987) also advocates a dual approach.

Banverket (BVH106 sid 4-61) employs an accident function, inspired by Coulombre et al, for each of the protection devices presented in Table 3-4. The accident risk is based on the average risk at crossings with the same protection device, adjusted for the deviation of the traffic volume product of the actual crossing (QT*QR) from the average (TFP).

A separate group of models contains exposure models. These models explain the probability that a car and a train are on the track at the crossing during the same seconds (e.g. HRB (1968)).

Based on the survey of the literature and discussions with Rail safety staff a basic model structure has been developed. We assume the number of accidents (A) to be a function of exposure (E) and user misbehaviour (B). As the exposure increases, the number of accidents will increase and as the probability of misbehaviour increases, the number of accident will increase.

The exposure can be seen as the probability that a car and a train will meet exactly on the track. This probability depends on the number of passing trains and cars as well as the speed when passing the crossing. It is also possibly that the number of tracks, measuring the width of the crossing, will increase the number of accidents.

We assume that the train has the right to pass and are not expected to take any action. Misbehaviour of the train operator is then ruled out. For road users misbehaviour, we divide the protection devices into barriers (full and half P1,P2) and non-barriers and non-protected crossings (P3,P4,P0).

For the last group we expect sight length, and if the road and railway run in parallel, to influence the number of accidents. A higher car speed increases the risk for misbehaviour and we assume that increased train speed increases this risk. If the road user get used to pass the crossing the risk for misbehaviour increase, but this type of mistakes are reduced with increased number of trains and increased number of tracks. In addition, the type of protection device affects the risk for misbehaviour, open crossings with flash light (P3) or S:t Andrew cross (P4) are safer than unprotected crossing (P0) and flash lights (P3) are safer then S:t Andrew cross (P4). Additionally, it seems that young drivers have a higher proportion of misbehaviour.

For crossings with barriers (P1, P2), we expect car speed to increase the risk and assume train speed can affect the risk in connection with attempts to pass barriers. The protection device affects the risk for misbehaviour such that full barriers have the lowest risk. The road surface and time of day may also affect the accident risk.

We will estimate two main groups of accident models. In Group A we use the crossings where we have observed road traffic volume and in Group B we use all crossings. As a proxy for road traffic volume, we use road type.

Table 4-7: Model structure

|  |  | Correlation with A | Model Group A | Model Group B |
| --- | --- | --- | --- | --- |
| Exposure (E) |  |  |  |  |
| Passing trains | Trains per day | + | QTOT98 | = |
| Passing cars | Cars per day | + | QR | R1R2;R3;R4R5 |
| Train speed | Km/h | - | Speed = QP98/QF98 | = |
| Car speed | Km/h | - | R1R2;R3;R4R5 | = |
| Crossing width | Number of tracks | + | UNE | = |
| Train operator Misbehaviour (Btrain) |  |  |  |  |
| Not assumed |  |  |  |  |
| Road User Misbehaviour (Bcar) |  |  |  |  |
| Non-barriers (P0, P3, P4) |  |  |  |  |
| Road sight | Sight length | - | R1R2;R3;R4R5 | = |
| Running parallel | Crossing angel (0-90˚) | - | n.a. | = |
| Car speed | Km/h | + | R1R2;R3;R4R5 | = |
| Train speed | Km/h | + | S = QP98/QTOT98 | = |
| Habit | Trains per day | - | QT98 | = |
| Habit | Number of tracks | - | UNE | = |
| Protection device | P3, P4 | - s.t. P3<P4 | P3;P4 | = |
| Road surface | Surface standard |  | n.a. | = |
| Darkness | Time of day |  | n.a. | = |
| Driver age | Young drivers | + | n.a. | = |
| Barriers (P1, P2) |  |  |  | = |
| Car speed | Km/h | + | R1R2;R3;R4R5 | = |
| Train speed | Km/h | + | S = QP98/QTOT98 | = |
| Protection device | P1, P2 | - s.t. P1<P2 | P1;P2 | = |
| Road surface | Surface standard |  | n.a. | = |
| Darkness | Time of day |  | n.a. | = |

= same as Model Group A

## Accident function

Accidents are unlucky events that depends on a number of known and unknown reasons. For one individual crossing the annual risk that an accident will happen is a certain probability. The probability (p) that an accident will happen can be written as p=A/Y where A is the number of (personal injury) accidents and Y is the number of crossings. With a logit model this probability can be expressed as (8), were x, the independent variable, β the parameters that will be estimated, and Λ(β’x) indicates the logistic cumulative distribution function.

P(A=1)=eβ'x1+eβ'x=Λ(β'x)				(8)

The estimation is done with the LIMDEP 7.0 software. Table 4-9 presents the result for the Group A models, which is based on the subset of the data where we have information on road traffic. Group B in Table 4-9 includes all observations.

Table 4-8 Estimated accident models, Group A.

|  | MODEL A0 |  | MODEL A1 |  | MODEL A2 |  | MODEL A3 |  | MODEL A4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Variable | Parameter | t-ratio | Parameter | t-ratio | Parameter | t-ratio | Parameter | t-ratio | Parameter | t-ratio |
| Constant | -7.097 | -4.420** | -6.026 | -4.809** | -5.844 | -4.692** | -3.396 | -8.740** | -3.096 | -8.555** |
| LNQR | 0.536 | 2.712** | 0.471 | 2.4764** | 0.447 | 2.360** |  |  |  |  |
| QTOT98 | 0.543 10-4 | 1.680* | 0.490 10-4 | 2.316** | 0.460 10-4 | 2.231** |  |  |  |  |
| QR |  |  |  |  |  |  | 0.245 10-3 | 2.350** |  |  |
| QTDAY |  |  |  |  |  |  | 0.017 | 2.277** |  |  |
| QTQRDAY |  |  |  |  |  |  |  |  | 7.56E-06 | 2.964** |
| S | -0.299 | -0.424 |  |  |  |  |  |  |  |  |
| UNE | 0.209 | 0.165 |  |  |  |  |  |  |  |  |
| R1R2 | 1.089 | 1.304 |  |  |  |  |  |  |  |  |
| R3 | 3.165 | 2.234** | 1.859 | 1.590 |  |  |  |  |  |  |
| P1 | -2.901 | -3.676** | -2.571 | -3.544** |  |  |  |  |  |  |
| P2 | -1.783 | -2.758** | -1.473 | -2.580** |  |  |  |  |  |  |
| P1P2 |  |  |  |  | -1.885 | -3.482** | -1.751 | -3.268** | -1.491 | -2.985** |
| Wald Statistics | ΒR1R2-βR3=0 ΒP1-βP2=0 | 4.456o | βP1-βP2=0 | 2.596o |  |  | βQR-βQTDAY=0 | 5.055** |  |  |
| Log-L | -82.55 |  | -84.31 |  | -86.33 |  | -87.17 |  | -87.78 |  |
| Log-L(0) | -93.57 |  | -93.7 |  | -93.8 |  | -93.8 |  | -93.8 |  |
| LR | 22.05 |  | 18.72 |  | 14.84 |  | 13.15 |  | 11.95 |  |
| LRI | 0.118 |  | 0.100 |  | 0.079 |  | 0.070 |  | 0.064 |  |
| dP/dQ | 6.065 10-7 | 1.641o | 5.98 10-7 | 2.2120** | 6.19 10-7 | 2.22** | 0.239 10-3 | 2.304** | 1.115 10-3 | 2.765** |
| 1/5 dP/dQ | 1.21 10-7 |  | 1.20 10-7 |  | 1.24 10-7 |  | 1.31 10-7 1) |  |  |  |

**) =Significant at 5%, *) Significant at 10%,

1) Per annual train traffic volume

Both the volume of trains (QTOT98) and the logarithm of road vehicles (LNQR) explains significantly the number of personal injury accidents (PACCID) with expected sign; more trains or cars will increase the number of expected accidents. However, in the introducing model A0, neither the proxy for train speed (s), car speed (R1R2,R3) nor the crossing width (UNE) are significant. The existence of urban road increases the risk, while barriers (P1) and half barriers (P2) reduces the risk as expected.

After reducing insignificant variables in a number of steps we conclude on a second model (A1), where we get significant effects for urban road (R3), full (P1) and half barriers (P2). A Wald test suggests that we cannot distinguish between the effects of half and full barriers. The chosen model in Group A (A2) only includes train and road traffic and the existence of barriers (full or half, P1P2).

Model A3 includes separately the daily flow of train traffic (QTDAY) and the flow of road traffic (QR). Both significantly affect the accident risk, and the parameter significantly differs between the modes. This suggests that the commonly used ‘traffic-product measure’ (Qtrain*Qroad), which treat the marginal influence of the different modes as equal, is not the most suitable measure of the traffic in road/rail level crossings. However, a model including this measure (QTQRDAY) suggests that the measure significantly explains the risk (Model A4).

The observed and estimated probability by protection device, train traffic and road traffic class is presented in Annex Table 5-14. The model A0 fits the observed probability by protection device very well. Model A2 overestimates the probability for full barriers and S:t Andrew cross while it underestimates the probability for half barriers. All models predict the probability by train traffic volume well expect at the lowest train traffic volume.

We only have observations on road traffic volume for a subset of the Swedish rail/road level crossings. In Group B we estimate an accident model excluding this information, using road type as a proxy. Train traffic volume (QTOT98) affects the accident probability, and it appears that road type works well as a proxy for road traffic volume. The existence of barriers (P1P2) reduces the risk. A Wald test suggests that we cannot distinguish between the detailed road categories in model B0. An aggregate of main and county roads (R1R2) and streets and other roads (R3R4) suggests that the former roads  have a higher risk than the latter smaller roads (model B1). The inclusion of streets (R3) does not improve the model very much; model B2 excludes R3. The group B models predict the observed probability less accurate than Group A (see Table 5-15).

Table 4-9: Estimated models, Group B

|  | MODELB0 |  | MODELB1 |  | MODELB2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Variable | Parameter | t-ratio | Parameter | t-ratio | Parameter | t-ratio |
| Constant | -6.517 | -14.396** | -6.500 | -14.369** | -6.049 | -18.539** |
| QTOT98 | 3.97 10-5 | 3.248** | 3.65 10-5 | 3.008** | 3.93 10-5 | 3.208** |
| R1 | 3.945 | 4.329** |  |  |  |  |
| R2 | 2.772 | 4.814** |  |  |  |  |
| R3 | 1.411 | 2.193** |  |  |  |  |
| R4 | 2.101 | 4.323** |  |  | 1.609 | 4.389** |
| P1P2 | -0.863 | -2.442** | -0.864 | -2.514** | -0.731 | -2.050** |
| R1R2 |  |  | 2.871 | 5.062** | 2.279 | 4.877** |
| R3R4 |  |  | 1.983 | 4.112** |  |  |
| Wald statistics | βRx-βRy=0 | Not signf. | βR1R2-βR3R4=0 | 6.706** | βR1R2-βR4=0 | 3.509* |
| Log-L | -327.821 |  | -329.869 |  | -330.938 |  |
| Log-L(0) | -351.914 |  | -351.914 |  | -351.914 |  |
| LR | 48.19 |  | 44.09 |  | 41.95 |  |
| LRI | 0.068 |  | 0.063 |  | 0.060 |  |
| dP/dQ | 2.12 10-7 | 2.947** | 2.00 10-7 | 2.762** | 2.29 10-7 | 3.018** |
| dP/dQ 1/5 | 0.423 10-7 |  | 0.400 10-7 |  | 0.459 10-7 |  |

**) =Significant at 5%, *) Significant at 10%, o) Significant at 15%

The annual marginal effect of train volume on the probability (1/5 dP/dQ) is 1.24 10-7 for model A2, which include road traffic volume. For B1, with a proxy for road traffic volume, the marginal effect is 0.40 10-7, i.e. only ⅓ of the effect in the Group A model.

Given the same speed, we do not expect to find different risk for passenger and freight trains. However, we cannot observe the speed but expect freight trains to have a lower speed. Our proxy for speed (the proportion passenger train on the track) not significant (model A0). A model with only passenger train (Qp) and passenger train accidents (PACCID*ELEMENT) gives similar result as B1 above, but a much lower marginal effect Table 4-10. To use only passenger train accidents does not result in a workable model for Group A. Freight train (Qf) does not significantly explain the ‘all’ accident risk (both involving passenger and freight train accidents) or the accidents where only freight trains where involved.

Table 4-10: Estimated models,

|  | MODEL B1 (PACCID_P) |  |
| --- | --- | --- |
| Variable | Parameter | t-ratio |
| Constant | -7.010 | -12.630** |
| QP98 | 5.108 10-5 | 2.662** |
| LNQR |  |  |
| P1P2 | -0.927 | -1.808* |
| R1R2 | 2.071 | 2.499** |
| R3R4 | 1.891 | 3.012** |
| Wald statistics |  |  |
| Log-L | -174.29 |  |
| Log-L(0) | -183.40 |  |
| LR | 18.22 |  |
| LRI |  |  |
| dP/dQ | 1.140 10-7 | 2.385** |
| dP/dQ 1/5 | 0.228 10-7 |  |

**) =Significant at 5%, *) Significant at 10%, o) Significant at 15%

## External marginal accident cost

In this section, we examine the marginal cost as a product of the average cost per accident (C) and the marginal effect on the accident function.

We write the cost per accidents as C (C=TC/PACCID). The average accident cost is 8 666 kSEK per personal injury accidents. Coleman and Stewart (1977) predict the consequence of a rail accident as a function of the speed for cars and trains. We have not been able to explain the accident cost as a function of the available variables.

The total expected annual accident cost per crossing (TC) can be written as in equation (9), where we have corrected for the number of years in the observation period. The average cost (AC) per passing train (10) and the marginal cost (=marginal external cost) (11) follows from this.

TC=P5C						(9)

AC=TCQT						(10)

MCe=MC=dPdQT15C					(11)

The marginal effect on the probability can be written as (12) (See Greene (1990)) and was presented in the Tables 4-2 - 4-4 above.

dPdx=Λ(β'x)(1−Λ(β'x))β				(12)

The marginal effects can also be expressed in terms of risk elasticity E as follows from (13) and (14) below.

r = AQ =PQY	(13)

E = ∂r∂Q Qr=dPdQQP−1	(14)

We evaluate the marginal effect for all crossings (Pall) and for different protection devices. The presented elasticities is based on the predicted annual probability (P/5 model). The model A2 is based on a subset of the data where crossings with full (P1) and half barriers (P2) dominate. The marginal effect is only significant for these two protection types; the elasticity EA is –0.65 for P1, -0.71 for P2 and –0.68 for an aggregate of barriers (P1P2).

For the B model (B1) the marginal effect is of the same magnitude; the elasticity EB is -0.72 for the aggregate of barriers (P1P2). For other barriers the elasticity is lower; for the aggregate of open crossings with light or S:t Andrew cross EB = -0.85 and for unprotected crossings EB = -0.92. The overall elasticity is –0.75 for model A and –0.87 for model B.

Table 4-11: Accident probability and marginal effect

| Model A2 | Pall | P1 | P2 | P1P2 | P3 | P4 | P3P4 | P0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1/5 dP/dQ (103) | 0.00012** | 0.00011** | 0.00009** | 0.00010** | 0.00033* | 0.00039* | 0.00034* | 0.00020o |
| P/5 model (103) | 3.88 | 3.10 | 2.36 | 2.74 | 8.64 | 10.90 | 8.98 | 5.38 |
| P/5 observed (103) | 3.88 | 1.96 | 3.56 | 2.74 | 9.28 | 7.70 | 9.04 | 0.00 |
| QTOT98 | 7789 | 9717 | 8088 | 8919 | 2428 | 4380 | 2715 | 3579 |
| EA | -0.75 | -0.65 | -0.71 | -0.68 | (-0.91) | (-0.84) | (-0.90) | (-0.87) |
| Model B1 | Pall | P1 | P2 | P1P2 | P3 | P4 | P3P4 | P0 |
| 1/5 dP/dQ (103) | 0.00004** | 0.00007** | 0.00007** | 0.00007** | 0.00011** | 0.00007** | 0.00008** | 0.00002** |
| P/5 model (103) | 1.69 | 2.28 | 2.13 | 2.21 | 2.04 | 2.28 | 2.66 | 0.93 |
| P/5 observed (103) | 1.46 | 1.34 | 3.00 | 2.10 | 3.76 | 3.56 | 3.64 | 0.30 |
| QTOT98 | 5415 | 9834 | 8765 | 9344 | 4455 | 5367 | 5019 | 3654 |
| EB | -0.87 | -0.71 | -0.72 | -0.72 | -0.76 | -0.84 | -0.85 | -0.92 |

**) =Significant at 5%, *) Significant at 10%, o) Significant at 15%

The sub sample used in Group A has higher average probability than the whole sample, as was seen in Table 3-5. To estimate a marginal cost, applicable on the whole rail network, based on the model A probabilities would be artificial.

Instead, we use the model B to estimate the marginal cost. First, the estimated marginal effect from model B is used and the MC is estimated based on equation (11) (MC model B in Table 4-12). Secondly, we use the observed probability, with the estimated elasticity EB, and estimate the MC based on equation (7) (MC r_obs EB). The model B overestimates the average probability for Pall, and for P0 (and P1), while it is underestimated for P3P4 (and P2, P3, P4). These differences are mirrored in the marginal costs. Thirdly, the estimated elasticity from model A (EA) is used with the observed probabilities for the whole database (MC r_obs EA).

Table 4-12 Marginal cost by passing train (SEK/passage)

|  | All | P1 | P2 | P1P2 | P3 | P4 | P3P4 | P0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MC model B | 0.35 | 0.58 | 0.58 | 0.58 | 0.94 | 0.58 | 0.70 | 0.18 |
| MC r_obs EB | 0.30 | 0.34 | 0.82 | 0.55 | 1.73 | 0.91 | 0.96 | 0.06 |
| MC r_obs EA | (0.58) | 0.42 | 0.87 | 0.62 | (0.69) | (0.89) | (0.65) | (0.10) |

The marginal costs based on observed probabilities and model B elasticities are 0.30 SEK/passages for all crossings, 0.55 SEK/passages for barriers (P1P2), 0.96 SEK/passages for open crossings with light or S:t Andrew cross (P3P4) and 0.06 SEK/passages for unprotected crossings. With model A elasticity the marginal cost at barriers are 13% higher.

Table 4-13: Marginal cost by passing train (SEK/passage)

# Conclusion

The first Swedish railway accident charge was based on the average accident cost per train kilometre. For road/rail level crossing accidents, the implicit charge was around 0.86 SEK/trainkm. The level crossing part of the accident charge has subsequently been abolished.

We have concluded that it does not exist any welfare economic reasons to abolish the level crossing accidents in a calculation of the relevant accident charge. However, we have also concluded that the average cost is not the right approximation for the marginal cost and that an appropriate welfare economic based theory exists.

We have estimated an accident function based on discrete choice model. We have observation on all (8600) Swedish road/rail level crossings where the tracks have been used in 1998. However, we have only information for road traffic on a limited number of crossings (977). The effect of both road and rail traffic are significant and the protection devices have the expected sign. The accident probability will decline if full or half barriers are installed. The proxy used for train speed or car speed was not successful. The model that includes the full dataset employs road type as a proxy for road traffic volume. The same conclusion can be drawn vis-à-vis barriers and, as expected, the probability is higher for crossings with main and county roads than for streets and other roads. The lowest probability is found at minor private roads. This reflects the expected road traffic volume.

The marginal cost is estimated based on the probabilities in the full dataset and is on average around 0.30 SEK/passages. For barriers the marginal cost is 0.55 SEK/passages and for open crossings with light or S:t Andrew cross 0.96 SEK/passages. For unprotected crossings, the marginal cost is 0.06 SEK/passages.

We cannot distinguish the marginal cost for freight and passenger train separately although we expect that slower freight trains have a lower marginal cost.

It is a strong case to re-introduce the road/rail level crossing charge. With our analysis, this can be done individually for each tracks segment. If the charge is differentiated according to protection device new incentives will be created in the railway safety system. If the number of dangerous crossings can be reduced, the charge will go down. This will give incentives to train operators to co-finance road/rail level crossing protection devices.

The study is based on a large dataset. However, the quality of the result is restrained by the limited information on road traffic flow, car and train speed. This should be improved in the future.

References

Abrahamsson, A., K. Ohlsson and K. Sjölinder (1991): ”Litteratursökning gällande olycksprediktionsmodeller och riskindex för plankorsningar mellan väg och järnväg” Statens Väg- och Trafikinstitut, VTI notat T 105, Linköping

Banverket (1990): ”Ökad observans av plankorsningar: Slutrapport: Plankorsningsdelegationen. Rapport 5”, Borlänge

Banverket (2000): ”Beräkningshandledning, BVH106”, Borlänge

Cairney, PT (ed) (1991): “Improving safety at railway level crossings”. Conference 26-27 September 1991 in Adelaide. Proceedings Australian Road Research Board, ARRB Conference proceedings, Vermont South

Coleman, J. and G. Stewart (1977): “Investigation of Accident Data for Railroad-Highway Grade Crossings”, Federal Highway Administration, U.S. department of transportation, TRR 611.

Coulombre, R., J. Poage and HF. Edwin (1982): “Summary of the DOT Rail-Highway Crossing Prediction Formulas and Resource Allocation Model”, U.S. Department of Transportation Federal Research and Special Programs Administration, Cambridge, September1982

Dahlman, J. (1999): ”Väntetider vid plankorsningar - upplevda och faktiska: en litteraturstudie Statens väg- och transportforskningsinstitut”. VTI notat 41-1999, Linköping

Hauer, E. and BN.Persaud (1987): “How to estimate the safety of rail-highway grade crossings and the safety effects of warning devices”, Transportation Research Record, 1987

Highway Research Board. (1968): “Factors Inflencing Safety at Highway-Rail Grade Crossings”, National Cooperative Highway Research Program Report No. 50

Lindberg, E. and K.Ohlsson (1991): ”Forskning om säkerhet i plankorsningar mellan väg och järnväg”: Statens Väg- och Trafikinstitut. VTI notat J 05: Linköping

Lindberg, E. (1994), ”Riskfyllda vägtrafikmiljöer i plankorsningar mellan väg och järnväg: En förstudie”: KFB & VTI forskning/research 13: Linköping

Ogden, KW., TA. Patton, N. Clark (1973): “A review of railway crossings in relation to road safety”, Australian Department of Transport. Report 10 Canberra.

Shahriari, M. (1991): “Risk analysis at railway/highway level crossings: A fault tree analysis”, Chalmers Tekniska Högskola. Transportteknik. Arbetsrappor

Shahriari, M. (1993): “Safety at rail/road level crossings”, Chalmers Tekniska Högskola. Transportteknik. Rapport 19 Göteborg

TFD (1981): ”Olyckor i plankorsningar mellan väg och järnväg”, TFD S 1981:4, Stockholm

TFD (1983): ”Olyckor i plankorsningar mellan väg och järnväg. Förarbeteende i korsningar med ljus- och ljudsignaler”, TFD S-publikation 1983:2, Stockholm

TRB (1976), “Railroad-highway grade crossings”, Transportation Research Board. Bibliography nr 57, Washington DC

Öhgren, P. (1998): ”Regeringsuppdrag: Utredning: Orsaker till döds- och personskadeolyckor vid plankorsningar mellan väg och järnväg”, Vägverket, Publikation 1998:36: Borlänge

Table 5-14; 5 years accident (PACCID) probability, observed and for models Group A

|  | P all | P1 | P2 | P3 | P4 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PACCID | 0.0194 | 0.0098 | 0.0178 | 0.0464 | 0.0385 |  |  |  |  |  |
| PO1 | 0.0194 | 0.0099 | 0.0179 | 0.0455 | 0.0434 |  |  |  |  |  |
| PA1 | 0.0194 | 0.0098 | 0.0179 | 0.0433 | 0.0543 |  |  |  |  |  |
| PA2 | 0.0194 | 0.0155 | 0.0118 | 0.0432 | 0.0545 |  |  |  |  |  |
| PA3 | 0.0194 | 0.0153 | 0.0121 | 0.0434 | 0.0525 |  |  |  |  |  |
| PA4 | 0.0194 | 0.0151 | 0.0123 | 0.0444 | 0.0463 |  |  |  |  |  |
| QTOT98 | 0-1000 | 1001-2000 | 2001-3000 | 3001-4000 | 4001-5000 | 5001-6000 | 6001-7000 | 7001-8000 | 8001-9000 | 9001 - |
| PACCID | 0.037 | 0.018 | 0.000 | 0.020 | 0.020 | 0.000 | 0.000 | 0.010 | 0.000 | 0.021 |
| PO1 | 0.032 | 0.019 | 0.014 | 0.017 | 0.015 | 0.028 | 0.013 | 0.009 | 0.013 | 0.021 |
| PA1 | 0.025 | 0.019 | 0.019 | 0.020 | 0.018 | 0.029 | 0.013 | 0.011 | 0.014 | 0.021 |
| PA2 | 0.025 | 0.020 | 0.020 | 0.020 | 0.019 | 0.016 | 0.013 | 0.012 | 0.015 | 0.020 |
| PA3 | 0.024 | 0.024 | 0.018 | 0.022 | 0.019 | 0.016 | 0.013 | 0.012 | 0.013 | 0.020 |
| PA4 | 0.025 | 0.023 | 0.021 | 0.021 | 0.020 | 0.016 | 0.014 | 0.013 | 0.014 | 0.019 |
| QR | 0-1000 | 1001-2000 | 2001-3000 | 3001-4000 | 4001-5000 | 5001-6000 | 6001-7000 | 7001- |  |  |
| PACCID | 0.016 | 0.018 | 0.000 | 0.111 | 0.042 | 0.000 | 0.000 | 0.059 |  |  |
| PO1 | 0.015 | 0.027 | 0.029 | 0.029 | 0.038 | 0.025 | 0.066 | 0.030 |  |  |
| PA1 | 0.015 | 0.026 | 0.027 | 0.031 | 0.044 | 0.022 | 0.048 | 0.034 |  |  |
| PA2 | 0.015 | 0.025 | 0.028 | 0.029 | 0.043 | 0.025 | 0.039 | 0.039 |  |  |
| PA3 | 0.017 | 0.018 | 0.020 | 0.023 | 0.039 | 0.027 | 0.046 | 0.071 |  |  |
| PA4 | 0.018 | 0.017 | 0.019 | 0.023 | 0.038 | 0.018 | 0.019 | 0.058 |  |  |

Table 5-15; 5 years accident (PACCID) probability, observed and for models Group B

|  | P0 | P1 | P2 | P3 | P4 | P all |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PACCID | 0.0015 | 0.0067 | 0.0150 | 0.0188 | 0.0178 | 0.0073 |  |  |  |  |
| B0 | 0.0044 | 0.0109 | 0.0112 | 0.0157 | 0.0124 | 0.0085 |  |  |  |  |
| B1 | 0.0046 | 0.0114 | 0.0107 | 0.0162 | 0.0114 | 0.0085 |  |  |  |  |
| B3 | 0.0048 | 0.0107 | 0.0115 | 0.0142 | 0.0122 | 0.0085 |  |  |  |  |
| QTOT98 | 0-1000 | 1001-2000 | 2001-3000 | 3001-4000 | 4001-5000 | 5001-6000 | 6001-7000 | 7001-8000 | 8001-9000 | 9001 - |
| PACCID | 0.0041 | 0.0060 | 0.0076 | 0.0056 | 0.0098 | 0.0035 | 0.0113 | 0.0107 | 0.0060 | 0.0110 |
| B0 | 0.0065 | 0.0068 | 0.0070 | 0.0070 | 0.0078 | 0.0083 | 0.0075 | 0.0084 | 0.0088 | 0.0135 |
| B1 | 0.0064 | 0.0069 | 0.0072 | 0.0069 | 0.0077 | 0.0078 | 0.0079 | 0.0086 | 0.0090 | 0.0136 |
| B3 | 0.0065 | 0.0067 | 0.0069 | 0.0070 | 0.0079 | 0.0079 | 0.0077 | 0.0086 | 0.0085 | 0.0136 |
| QR | 0-1000 | 1001-2000 | 2001-3000 | 3001-4000 | 4001-5000 | 5001-6000 | 6001-7000 | 7001- |  |  |
| PACCID | 0.0160 | 0.0183 | 0 | 0.111 | 0.0417 | 0 | 0 | 0.0566 |  |  |
| B0 | 0.0163 | 0.0159 | 0.0151 | 0.0200 | 0.0206 | 0.0235 | 0.0233 | 0.0212 |  |  |
| B1 | 0.0173 | 0.0160 | 0.0140 | 0.0155 | 0.0151 | 0.0131 | 0.0142 | 0.0133 |  |  |
| B3 | 0.0172 | 0.0163 | 0.0142 | 0.0160 | 0.0157 | 0.0132 | 0.0137 | 0.0135 |  |  |

SFS 1988:1378

The charge for infrastructure damage is approximately 3.50 SEK/trainkm for an average train of 495 tonne/train.

Source: XXXXX Category I had a charge on 1.16 SEK/trainkm and category II/III had a charge on 3.36 SEK/trainkm

Increased to 1.83 SEK/trainkm in 1991 (SFS 1991:2038, 8§)

The ¼ is related to the number of children and mentally ill persons in accidents (BVP 1997/4 p.24)

Source: Banverket 1999. Statistik över olyckor på statens spåranläggningar 1999 (p.4 ff).

We can view the general equation (7) as a product of the average cost per accident (a, b, c) times the derivation of the total number of accidents (A) w.r.t. the traffic flow (Q) of the examined mode [dA/dQ=r(1+E)] or it can be viewed as the product of the average cost per exposure r(a+b+c), for example per kilometre driven, times the elasticity (E).

Accident information has also been collected from Vägverket. While this dataset is a double check on the number of accidents, it also includes traffic volume by road, road surface condition, time of day, classification of the involved vehicle and driver.

By track-segment, distance in kilometre and metre from the beginning of the segment.

Cost per fatality 14.200.00 SEK, severe injury 2.600.000 SEK and light injury 150.000 SEK.

BVH 106, 2-71; Average risk (Omf) full barriers (P1) 0.0033; automatical full barriers (‘P1’) 0.0055; Half barrier (P2) 0.0076, Open crossings w light. (P3) 0.0156; Open crossings w. S:t A. (P4) 0.0080; no prtection (P0) 0.0008. Personal injury accidents per accident (1-0.37).

Crossings are grouped by number of passing trains. The first eleven groups have an interval of 2500 passing train. The number of observations in each group are 3382; 2229; 941; 652; 419; 459; 264; 77; 28; 36; 23. The twelfth and last group contains crossings with 30000-75000 passing trains (108 obs).

Log10A = αo + α1log10QR+α2log10QT+α3(log10QT)2

And a subdivision of the full barriers into automatic full barriers and other.

A = r*QT = QT2 QR /TFP f(sth) Omf . Where r is the predicted accident risk, Omf the observed average risk and TFP the average traffic volume product for that group of crossings, f(sth) is a correction factor for speed which increases for increased speed.