COMPETITIVE AND SUSTAINABLE GROWTH

(GROWTH)

PROGRAMME

Deliverable 9: Accident Cost Case Studies Case Study 8e Marginal External Accident Costs in Maritime transport on the Baltic sea

Version 1.0

May 2002

Author: 
Gunnar Lindberg (VTI)

Contract: 1999-AM.11157 Project Coordinator: ITS, University of Leeds Funded by the European Commission 5th Framework – Transport RTD

UNITE Partner Organisations ITS/UNIVLEEDS (UK), DIW (De), NEI (Nl), CES/KUL (Be), TIS.PT (Pt), IWW/UNIKARL (De), VTI (Se), IER/USTUTT (De), CERAS/ENPC (Fr), HERRY (Au), EIET/ULPGC (Es), ISIS (It), STRATEC (Be), SYSTEMA (Gr), JP-TRANSPLAN (Fi), VATT (Fi), ECOPLAN (Ch), INFRAS (Ch), EKONO (Fi), EKI (Se)

UNITE

1999-AM.11157

UNIfication of accounts and marginal costs for Transport Efficiency

Deliverable 9:Marginal external accident costs in Maritime transport on the Baltic sea

This document should be referenced as:

Lindberg, G (VTI), Deliverable 9: Accident Cost Case Studies, Case Study 8e: Marginal external accident costs in Maritime transport on the Baltic sea (UNIfication of accounts and marginal costs for Transport Efficiency) Deliverable 9. Funded by 5th Framework RTD Programme. ITS, University of Leeds, Leeds..

May 2002

Version 1.0

Authors:	as above, with contributions from other UNITE partners

PROJECT  INFORMATION

Contract no: 1999-AM.11157:

UNIfication of accounts and marginal costs for Transport Efficiency

Website:	www.its.leeds.ac.uk/unite

Commissioned by: European Commission – DG TREN; Fifth Framework Programme

Lead Partner:   Institute for Transport Studies, University of Leeds (UK)

Partners:  ITS/UNIVLEEDS (UK), DIW (De), NEI (Nl), CES/KUL (Be), TIS.PT (Pt), IWW/UNIKARL (De), VTI (Se), IER/USTUTT (De), CERAS/ENPC (Fr), HERRY (Au), EIET/ULPGC (Es), ISIS (It), STRATEC (Be), SYSTEMA (Gr), JP-TRANSPLAN (Fi) VATT (Fi), ECOPLAN (Ch), INFRAS (Ch), EKONO (Fi), EKI (Se)

DOCUMENT  CONTROL  INFORMATION

Status: 	Accepted

Distribution:	  UNITE partners

Availability:	  Public once accepted

Filename:	D9 8e

Quality assurance:

Coordinator’s review:	C A Nash

Signed:

Date:

Table of Contents	Page:

1	Introduction	3

2	Fairway charges	4

3	Theory of external marginal cost	5

3.1	Basic approach	5

3.2	Liability	6

4	The Swedish fleet and its accidents on Swedish water	7

4.1	Qualitative survey of accidents	8

4.1.1	Single accidents	11

4.1.2	Collision between ships	11

5	Limits of compensation	13

5.1	Limits of compensation in Swedish Maritime law	13

5.2	Limits of compensation compared to value of statistical life	14

6	Consequence for the fairway charges	15

6.1	Single accidents/collision with other objects	15

6.2	Collision with other ships	15

6.3	Indication for the fairway charge	16

7	Conclusion	16

References	18

# Introduction

Maritime transport is considered a safe mode of transport. Nevertheless, a number of horrifying accidents have occurred the last decade (e.g. Estonia), which makes the accident risk real. In Sweden, a charging scheme exists in the maritime sector. The charge is based on the infrastructure cost and includes environmental differentiations. However, accident cost is not included. The question is, does a marginal external accident cost exist in the maritime sector and if so, is it relevant to include it in a distance based charging scheme?

We define five categories of ships; fishing vessel which needs licenses (above 5 metre), passenger ships including ‘road-ferries’, tanker for oil, gas and chemicals, ship for transport of dry cargo (bulk) including ro-ro, car- and container ship and other ships including barges, working platforms and ice breakers. Nine types of accidents are defined; fire, the ship running aground, collision with bridges pier etc, collision between ships, leakage, engine failure and personal injury, including illness which in some tables are presented separately, and oil spill discharge.

A qualitative study is carried out on a detailed database of all accidents with Swedish ships on Swedish water during 1998. The statistics includes around 140 such reported accidents. The limitation to Swedish ships is because we need to know the number of ships, to be able to estimate a risk per ship. In addition, accidents occur for foreign ships on Swedish water (59 reported accidents in 1998) and of Swedish ships on other water (36 accidents). The table below summarise the Maritime accidents with commercial ships on Swedish water 1998.

Table 1: Accidents on Swedish water involving Swedish and foreign ships 1998

| Swedish Ships | Fire | Aground | Collision w. other | Collision w. ship | Leakage | Engine failure | Personal injury | Dis- charge | Other | TOTAL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fishing-vessel | 3 | 9 | 1 | 1 | 6 | 5 | 1 | 0 |  | 26 |
| Passenger ship | 10 | 24 | 10 | 6 | 3 | 7 | 9 | 1 | 1 | 71 |
| Tanker | 2 | 3 | 0 | 4 | 0 | 0 | 1 | 1 |  | 11 |
| Bulk | 1 | 4 | 4 | 2 | 0 | 2 | 1 | 1 | 1 | 16 |
| Other |  | 4 | 0 | 2 | 4 | 2 | 4 |  | 2 | 18 |
| TOTAL | 16 | 44 | 15 | 15 | 13 | 16 | 16 | 3 | 4 | 142 |
| Foreign Ships |  |  |  |  |  |  |  |  |  |  |
| Fishing vessel |  | 0 | 1 | 1 |  |  | 1 |  |  | 3 |
| Passenger ship | 2 | 0 | 3 |  |  |  | 3 | 1 |  | 9 |
| Tanker |  | 1 | 2 | 2 |  | 1 |  | 2 |  | 8 |
| Bulk |  | 13 | 4 | 5 |  | 4 | 4 | 1 | 2 | 33 |
| Other |  | 1 | 1 | 3 |  |  | 1 |  |  | 6 |
| TOTAL | 2 | 15 | 11 | 11 | 0 | 5 | 9 | 4 | 2 | 59 |

Source: Maritime Administration database SOS.

The paper is organised as follows. Section 2 below presents shortly the current fairway charges in Sweden. Section 3 presents the theory of external marginal accident cost and section 4 discusses the accident risk. In section 5 the limits of compensation compared to the accident cost is examined and section 6 discuss the consequence for the fairway charges. Finally, section 7 offers some conclusions.

# Fairway charges

Swedish charges for the provision of maritime infrastructure and services consist of two parts; (i) a charge based on gross tonnage (GT) introduced in January 1998, environmentally differentiated, and (ii) a part based on the amount of loaded/unloaded goods. The environmentally differentiation aims at establishing an incentive for ship owners to reduce emissions of sulphur and nitrogen oxides. The Swedish Maritime organisation is financed by the charges. The revenue from the fairway charge was 100.4 million € in 1998.

The first charge (based on GT) is differentiated according to the certified emission levels of NOx per kWh for the ships’ machinery and the (two) levels of sulphur in the bunker. For ferries and other ships (not tankers) the charge is 3.40 SEK/GT (0.38 €/GT) if emissions of NOx are 2g/kWh or less. The charge increases linearly up to the level of 5 SEK/GT (0.56 €/GT) if emissions are 12 g/kWh or more. In addition, if a ship certifies that it is only using low bunker fuel, a reduction of 0.9 SEK/GT (0.10 €/GT) will be given. A maximum limit is set for other ships in such a way that the charge will not increase beyond 24 000 GT. Charges are not levied for ships of less than 400 GT. The charge is only levied at a maximum 18 times a year for passenger ferries and a maximum of 12 times a year for each individual cargo ship.

Before the introduction of the environmental differentiation, the charge was 3.60 SEK/GT (0.40 €/GT) for all ships. The new policy was introduced with a restriction to leave the total amount of charges unchanged. Restitution is given up to 40% of the cost of introducing the NOx reducing technology.

In addition, some harbours have introduced a differentiation of the charges according to the sulphur content of the fuel and NOx emissions (Sjöbris et.al. 1999 p.4). The revenues from harbour fees was approximately 389 million € in 1998, i.e. almost four times higher than the fairway charges.

The fairway charging system is only aimed at financing the infrastructure cost with the additional aim to give incentives to improve the environmental performance. This means that the level of charges is below the marginal cost but the structure may be appropriate for environmental reasons. Accident cost has not been included in the charge.

# Theory of external marginal cost

In this section we first presents the basic approach of external marginal cost (3.1), which then is extended to a discussion on liability (3.2).

## Basic approach

The number of accidents where ships are involved (A), is a function of the traffic volume (Q) and other explanatory variables, including the traffic volume of other ships (1). Naturally, A should be seen as a vector representing different degree of severity, which also is the case for the cost components; willingness-to-pay of the involved user (a), ditto of relatives and friends (b) and system external cost (c), mainly medical costs paid by the social security system. The marginal cost (MC) with respect to the traffic volume of ships (Q) follows naturally from the total cost (TC) (2 and 3). Finally, we derive the external marginal cost as (4) where PMC is the private marginal cost already internalised by the captain/shipowner.

A =f(Q,......) 	(1)

MC = ∂A∂Q (a+b+c)	(2)

TC =Aa+b+c	(3)

MCe = MC−PMC	(4)

We introduce r as the accident risk (5) and assume it may be affected by an increase in the volume of traffic of ships. This effect is conveniently written as a risk-elasticity (E) (6).

r = AQ 	(5)

E = ∂r∂Q Qr	(6)

The marginal cost then takes the following simple form (7).

MC =r(1+E)a+b+c	(7)

However, a part of the cost is internalised by the captain/shipowner. We introduce θ as the share of total accident costs that is internalised by the captain/shipowner. The external marginal accident cost can be expressed as (8) .

MCe =r(1+E)a+b+c−rθ(a+b) =r(1−θ+E)a+b+rc(1+E)	(8)

Assume first a single accident, where only one ship is involved. The compensation shipowner has to pay his customer is often limited. In such cases, he does not pay the whole accident cost (a+b) and θ has to be adjusted according to his actual cost. This means that it exists an externality in relation to his own customers.

## Liability

The other source of externality is in collisions between two or more ships. We divide the users into two groups, the ship of the first group (A) is damaged and the second group (B) is the other part in the accident. Without any liability or limits of compensation, the owner of the damaged ship (A) will bear all costs (θ=1) and the other part (B) will not bear any cost (θ=0). The external marginal cost for each group is then:

MCAe =rA(a+b+c)1+EA−rA(a+b)=rA(a+b+c)EA+rAc		(9)

MCBe =rB(a+b+c)1+EB				(10)

Under a negligence rule, the prevailing rule in Maritime traffic, a user of group B will not bear any cost as long as he is not responsible for the accident. He nevertheless will generate an externality equal to equation (10) above. If he is responsible, he will have to pay compensation in relation to his responsibility compared to shipowner A. We assume he will only compensate the other shipowner, and not the rest-of-society, with a share (α) of ship A’s accident cost. The expected compensation (α(a+b)) will be included in his private marginal cost and the external cost will decrease (11). At the same time, the compensation (α(a+b)) will reduce the expected cost of shipowner A; consequently user A’s external marginal cost will increase (12).

Responsible user B =>  MCBe =rB(a+b+c)1+EB−rB(α(a+b))	(11)

Compensated user A =>  MCAe =rA(a+b+c)EA+rAc+rA(α(a+b))	(12)

Assume for simplicity that we find the risk of collision accidents to be constant, i.e. the risk elasticity for each individual ship is –0.5 (see Lindberg 2001). If shipowner B pays half the accident cost (α=0.5) as a compensation to shipowner, A the only remaining externality is the cost for the rest of society (c).

# The Swedish fleet and its accidents on Swedish water

The Swedish commercial maritime fleet consisted in 1998 of 5 371 registered ships. The largest group, 39%, was fishing vessels and especially small fishing vessels below 19 gross tonnage (GT) (33%). The second largest group is ‘other’ (31%). The third group is passenger ships with ¼ of the fleet. Most passenger ships are small ships and so-called ‘road-ferries’. The transporters of cargo, tankers and bulk ship, consist together only of 6% of the fleet.

Table 2: Registered Swedish ships by type and gross tonnage 1998

| Type/ GT | -19 | 20-99 | 100-499 | 500-999 | 1000-4999 | 5000-9999 | 10000 | TOTAL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fishing vessel | 1777 | 221 | 107 | 8 |  |  |  | 2113 |
| Passenger ship | 642 | 396 | 225 | 10 | 4 | 6 | 17 | 1300 |
| Tanker | 1 | 23 | 15 | 5 | 39 | 23 | 7 | 113 |
| Bulk | 10 | 41 | 37 | 3 | 39 | 11 | 49 | 190 |
| Other | 942 | 413 | 236 | 33 | 25 | 5 | 1 | 1655 |
| TOTAL | 3372 | 1094 | 620 | 59 | 107 | 45 | 74 | 5371 |

Source: Sjöfartsmeddelanden Nr 2 1999, p9

In 1998, 143 accidents involving Swedish ships on Swedish water were reported into the SOS database. Most accidents were reported for fishing vessels (26) and passenger ships (72). Only 17 accidents were reported for bulk ships and 11 for tankers.

Table 3: Reported Accidents by ship type and gross tonnage 1998

| Type / GT | -19 | 20-99 | 100-499 | 500-999 | 1000-4999 | 5000-9999 | 10000 | na | TOTAL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fishing vessel | 2 | 11 | 5 | 0 | 0 | 0 | 0 | 8 | 26 |
| Passenger ship | 2 | 15 | 29 | 3 | 1 | 5 | 17 | 0 | 72 |
| Tanker | 0 | 1 | 0 | 1 | 5 | 3 | 1 | 0 | 11 |
| Bulk | 0 | 0 | 0 | 0 | 13 | 1 | 3 | 0 | 17 |
| Other | 1 | 6 | 2 | 1 | 3 | 0 | 0 | 4 | 17 |
| TOTAL | 5 | 33 | 36 | 5 | 22 | 9 | 21 | 12 | 143 |

Source: Maritime Administrations database

The risk per registered ship is highest for the tankers and bulk ships. If we examine the risk by weight, an increasing risk with increasing weight can be observed. However, it should be noted that we examine registered accidents, i.e. the probability to report an accident may increase with ship size.

Table 4: Accident risk by ship type and gross tonnage 1998 (Acc/1000ships)

| Type / GT | -19 | 20-99 | 100-499 | 500-999 | 1000-4999 | 5000-9999 | 10000 | TOTAL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fishing vessel | 1.1 | 49.8 | 46.7 | 0.0 | 0.0 | 0.0 | 0.0 | 12.3 |
| Passenger ship | 3.1 | 37.9 | 128.9 | 300.0 | 250.0 | 833.3 | 1000.0 | 55.4 |
| Tanker | 0.0 | 43.5 | 0.0 | 200.0 | 128.2 | 130.4 | 142.9 | 97.3 |
| Bulk | 0.0 | 0.0 | 0.0 | 0.0 | 333.3 | 90.9 | 61.2 | 89.5 |
| Other | 1.1 | 14.5 | 8.5 | 30.3 | 120.0 | 0.0 | 0.0 | 10.3 |
| TOTAL | 1.5 | 30.2 | 58.1 | 84.7 | 205.6 | 200.0 | 283.8 | 26.6 |

The fatality risk per registered ship is 1.9 killed person per 1 000 registered Swedish ships. Compared to road traffic the fatality risk per vehicle/ship is ten times higher in Maritime transport; in road transport 0.012 fatalities occurred per registered vehicle in 1998. However, the number of passenger per road vehicle is of course not comparable to the number of passengers in a ship. The COMMUTE project suggested the fatality risk per billion passenger kilometre for ferries in European water to be 3.3, which is half of the risk in road transport.

The fatality risk is nevertheless surprisingly high in maritime transport. In addition to the annual ten fatalities in commercial traffic, 33 fatalities occurred in pleasure boat traffic. It is an open question what this mean for the risk, as we have no information on the number of pleasure boats, which includes a substantial number of minor boats.

Table 5: Accident risk in Maritime and road transport

|  | Fatality | Vehicles | Fatalities per 1000 vehicles | Million passenger km (Mpkm) | Fatalities per 1000 Mpkm |
| --- | --- | --- | --- | --- | --- |
| Maritime | 10 | 5 371 | 1.9 | na | 3.3A |
| Road | 531 | 4 246 000 | 0.1 | 80 842 | 6.6 |

Note A. Information from the COMMUTE project

In section 4.1 below all accidents with Swedish ships on Swedish water during 1998 is examined.

## Qualitative survey of accidents

The theory of marginal cost was discussed in section 3 above. It was clear that to estimate a distance related marginal cost we need to find the relationship between the accident risk and the distance travelled. The data, summarised above, do not allow us to estimate such functions.

Instead, we employ a qualitative method and survey all the reported accidents with Swedish ships on Swedish waters in 1998. For each accident, a report is written into the SOS-system (SjöOlycksSystemet). The report contains detailed information about the ships involved, type of accident, time and place, weather conditions and accident consequences.

The most common accident type is ship that runs aground (31%), followed by collision accidents with other things than ships (14%), fire (12%), engine failure (10%) and collision with other ships (10%). Around 11% were not maritime accidents but accidents where people got ill or were injured of other reasons. Around 2% were environmental accidents (discharge). In summary, 90% of the accidents where ‘single’ accidents.

The classified cause of the accidents where in 42% of the cases unspecified human failure, and 6% navigation errors and in two cases (1%) was the skipper influenced by alcohol. The second most common reason was engine failure (24%). Bad whether (strong wind or bad sight) was the cause behind 6% of the accidents.

The gross tonnage of the average ship involved in a reported accident was 4 727 tonne and the ship was 27 years old. In 4% of the cases had the ship a pilot onboard.

The type of water where the accident occurred was evenly distributed between open sea (19%), open coast (10%), inner coast (19%), narrow coast (17%), inland waterway including lakes (16%) and harbours (19%). However, 45% of the accidents occurred in some type of coastal area. Around half of the accidents occurred in the Baltic Sea.

Most of the accidents occurred in daytime with a relatively good sight distance (13 km) and low wind (6.6 m/s) and not rough sea (0.61 metre). However, collisions with other ships occurred more often when the sight distance was shorter (9.3 km). Accidents caused by the weather occurred in strong wind (20 m/s) and high seaway (5.2 m).

The most frequent consequence of the accident was hull damage (42%). In 7% was the consequence a leakage, but most of these accidents occurred as a primary leakage

Table 6: Descriptive statistics of accidents with Swedish ships on Swedish water 1998.

| Accient type | Fire | Aground | Coll.other | Coll.ship | Leackage | Enginef. | Personinj. | Illness | Discharge | Weather | Other | TOTAL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Accidents | 17 | 45 | 20 | 14 | 6 | 15 | 10 | 7 | 3 | 3 | 4 | 144 |
| Type and number of ship |  |  |  |  |  |  |  |  |  |  |  |  |
| Fishing vessel | 3 | 9 | 3 | 1 | 4 | 5 | 1 | 0 | 0 | 0 | 0 | 26 |
| Pass. ship | 11 | 24 | 11 | 5 | 2 | 6 | 5 | 5 | 1 | 1 | 1 | 72 |
| Tanker | 2 | 3 | 0 | 3 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 11 |
| Bulk | 1 | 5 | 4 | 3 | 0 | 2 | 0 | 1 | 1 | 0 | 0 | 17 |
| Other | 0 | 4 | 2 | 2 | 0 | 2 | 3 | 1 | 0 | 1 | 3 | 18 |
| Cause of accident |  |  |  |  |  |  |  |  |  |  |  |  |
| Engine failure | 8 | 2 | 5 | 1 | 3 | 15 |  |  |  |  |  | 34 |
| Construction |  |  |  |  | 1 |  |  |  |  |  |  | 1 |
| Human | 4 | 31 | 10 | 6 |  |  | 6 |  | 3 | 1 |  | 61 |
| Alcohol |  | 2 |  |  |  |  |  |  |  |  |  | 2 |
| Navigation |  | 3 |  | 5 |  |  |  |  |  |  |  | 8 |
| Weather |  | 2 | 1 | 2 | 1 |  | 1 |  |  | 2 |  | 9 |
| Other |  | 1 | 2 |  |  |  | 2 | 5 |  |  | 2 | 12 |
| Unknown | 5 | 4 | 2 |  | 1 |  | 1 | 2 |  |  | 2 | 17 |
| Ship characteristics, average |  |  |  |  |  |  |  |  |  |  |  |  |
| GT | 13 418 | 584 | 2 793 | 3 302 | 49 | 3 806 | 5 840 | 19 621 | 11 057 | 3 727 | 7 735 | 4 727 |
| Prod Year | 1 970 | 1 966 | 1 977 | 1 979 | 1 954 | 1 959 | 1 978 | 1 986 | 1 982 | 1 975 | 1 973 | 1 971 |
| Pilot | 0.00 | 0.04 | 0.00 | 0.21 | 0.00 | 0.00 | 0.00 | 0.14 | 0.00 | 0.00 | 0.00 | 0.04 |
| Seamen on board | 23.4 | 5.4 | 10.4 | 7.9 | 2.3 | 7.6 | 3.0 | 113.0 | 46.0 | 0.0 |  | 11.76 |
| TotOnboard | 169.0 | 28.7 | 47.5 | 18.6 | 9.0 | 46.2 | 56.0 | 923.0 |  | 0.0 |  | 78.41 |
| Type of Water |  |  |  |  |  |  |  |  |  |  |  |  |
| Open Sea | 5 | 1 | 3 | 3 | 5 | 2 | 1 | 3 | 1 | 2 | 1 | 27 |
| Open Coast |  | 8 |  | 1 |  | 1 | 1 | 1 |  | 1 | 1 | 14 |
| Inner Coast | 5 | 8 | 2 | 1 |  | 7 | 2 | 2 |  |  |  | 27 |
| Narrow Coast |  | 17 | 4 | 1 |  | 2 |  |  |  |  |  | 24 |
| Inland waterw. | 3 | 7 | 6 | 3 | 1 | 3 |  |  |  |  |  | 23 |
| Harbour | 3 | 4 | 5 | 5 |  |  | 6 | 1 | 2 |  | 1 | 27 |
| Other | 1 |  |  |  |  |  |  |  |  |  | 1 | 2 |
| Baltic 1/0 | 0.65 | 0.58 | 0.35 | 0.43 | 0.33 | 0.33 | 0.10 | 0.29 | 0.67 | 1.00 | 1.00 | 0.48 |
| Weather conditions, average |  |  |  |  |  |  |  |  |  |  |  |  |
| Light (1-0.5-0) | 0.74 | 0.68 | 0.80 | 0.75 | 0.75 | 0.87 | 0.80 | 0.50 | 1.00 | 0.50 | 1.00 | 0.74 |
| Sight (km) | 15.00 | 12.28 | 12.82 | 9.33 | 13.67 | 12.44 | 15.00 | 15.00 | 15.00 | 11.00 | 15.00 | 12.58 |
| Wind (m/s) | 2.73 | 5.95 | 7.14 | 5.66 | 8.58 | 8.03 | 7.93 | 4.52 | 0.90 | 20.98 | 9.35 | 6.60 |
| Seaway (m) | 0.22 | 0.41 | 0.64 | 0.16 | 1.10 | 0.89 | 0.79 | 0.19 | 0.00 | 5.25 | 2.50 | 0.61 |
| Accident consequences, average |  |  |  |  |  |  |  |  |  |  |  |  |
| Tot Fat/Inj/Mis | 0.06 | 0.00 | 0.25 | 0.00 | 0.00 | 0.00 | 1.10 | 1.00 | 0.00 | 0.67 | 0.00 | 0.18 |
| Hull damage | 0.00 | 0.69 | 0.65 | 0.64 | 0.50 | 0.13 | 0.00 | 0.00 | 0.00 | 0.67 | 0.00 | 0.42 |
| Leakage | 0.00 | 0.09 | 0.05 | 0.00 | 0.67 | 0.00 | 0.00 | 0.00 | 0.00 | 0.33 | 0.00 | 0.07 |
| Othe damage | 0.00 | 0.00 | 0.20 | 0.07 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.03 |

Light: daytime=1, dark=0, dawn/twilight=0.5

In the table below, the casualties in commercial shipping 1998 are presented. Of the 11 fatalities that occurred, seven was due to illness (64%) and the rest in collisions accidents with other objects than ships.

Table 7: Casualties in Swedish Maritime accidents 1998

| Accident type | Fire | Aground | Coll.other | Coll.ship | Leackage | Engine. | Personinj. | Illness | Discharge | Weather | Other | TOTAL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fatality – seamen | 0 | 0 | 4 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 5 |
| Fatality – passenger | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 5 | 0 | 0 | 0 | 5 |
| Fatality – other | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 |
| Injury – seamen | 0 | 0 | 0 | 0 | 0 | 0 | 8 | 0 | 0 | 0 | 0 | 8 |
| Injury – passenger | 1 | 0 | 1 | 0 | 0 | 0 | 2 | 0 | 0 | 2 | 0 | 6 |
| Injury – other | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 |
| TOTAL | 1 | 0 | 5 | 0 | 0 | 0 | 11 | 7 | 0 | 2 | 0 | 26 |

### Single accidents

Around 90% of the accidents can be classified as single accidents. As long as the cost of the accident is born by the shipowner, no obvious externality occurs.

However, we know that not all cost of personal injury accidents are paid by the shipowner. A part of the accident cost is born by the society. Consequently, for personal injury accidents an externality occur, even if the accident is a single accident. Personal injuries took place for fire accidents, collision with other objects, personal injuries, and illness and weather accidents. Personal injury accidents and illness is the dominant type (70% of the personal injuries) but has nothing to do with transportation. Example of these accidents includes accidents with staff in the kitchen. Of the remaining eight personal injuries, two were passengers that got hurt in bad weather and one was a passenger hurt in a fire. It is difficult to see that this type of accidents should have a clear relationship with the transportation distance and relevant for a distance related charge.

The remaining accident with personal injuries is the most serious. Collisions with other objects killed four seamen and injured one passenger during 1998.

### Collision between ships

From the SOS database, collision matrixes can be constructed. The most common collision during 1998 was collisions between passenger ships and pleasure boats. All three of the pleasure boats were sailing boats.

Table 8 Collision matrix involving at least one Swedish shipc1998

|  | Fishing vessel | Passenger | Tanker | Bulk | Other | Pleasure | Total |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Fishing vessel | 1 |  |  |  |  |  | 1 |
| Passenger | - | - |  |  |  |  | 0 |
| Tanker | - | 1 | 1 |  |  |  | 2 |
| Bulk | - | - | 2 | 1 |  |  | 3 |
| Other | - | 2 | - | 1 | - |  | 3 |
| Pleasure | - | 3 | - | - | 1 | n.a. | 4 |
| Unknown | - | - | - | 1 | - | n.a. | 1 |
| Total | 1 | 6 | 3 | 3 | 1 | - | 14 |

From the theoretical discussion above we know that an externality occur for both ship in a collision accident but the magnitude of the externality depends on the elasticity (E) and the proportion of cost that is born by each ship (θ). In the table below detailed information is given for each of the collision accidents during 1998.

The quality of the information is not very good. We have only information on damages on both ships for five of the fourteen accidents. If we assume that pleasure boats are damaged in collision accidents with commercial ships we can add four accidents with information.

Table 9: Detailed information on Collision accidents

| Nr: | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ship I type | Fish. | Pass. | Pass. | Pass. | Pass. | Pass. | Tank | Tank | Tank | Bulk | Bulk | Bulk | Other (Pilot) | Other |
| GT | 30 | 122 | 294 | 505 | 424 | 6 643 | 2 406 | 4 230 | 10 908 | 2 787 | 3 800 | 7 486 | - | - |
| Prod year | 1956 | 1975 | 1979 | 1993 | 1991 | 1979 | 1980 | 1973 | 1993 | 1973 | 1983 | 1976 | 1979 | - |
| Cause | navig. | human | human | sight | navig. | human | weath. | navig. | navig. | engine | navig. | human | human | human |
| Pilot | no | no | no | no | no | no | yes | yo | yes | yes | no | no | no | no |
| Seamen | - | 4 | 4 | 3 | - | 15 | 8 | 15 | 10 | - | 11 | 11 | 2 | 4 |
| TotOnboard | - | 86 | 36 | 3 | - | 15 | 9 | 15 | 12 | - | 11 | 11 | 3 | 4 |
| TypeWater | Open sea | Narrow coast | Har-bour | Inland wat | Har-bour | Har-bour | Inland wat | Har-bour | Lake | Inner coast | Open sea | Open sea | Open coast | Har-bour |
| Baltic 1/0 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 0 |
| Activity | Sailing | Sailing | Sailing | Sailing | Park | Sailing | Sailing | Load | Sailing | Sailing | Icebre. | Sailing | Other | Sailing |
| Cargo | fish | pass. | pass | pass | empty | empty | oil | oil | ballast | bulk | ballast | ballast | empty | empty |
| Light | 1 | 1 | 1 | 1 | 0 | 1 | 0.5 | 1 | 0 | 1 | 1 | 1 | 0 | 1 |
| Sight km | 0.25 | 15 | 15 | 0.25 | - | 15 | 15 | 15 | 15 | 15 | 0.25 | 0.25 | 0.25 | 15 |
| Wind m/s | - | 12.3 | 4.4 | 0.1 | - | 4.4 | 15.5 | 6.7 | 2.45 | 0.9 | 6.7 | 4.4 | 4.4 | - |
| Seaway (m) | - | 0.5 | 0 | 0 | - | 0 | 0.25 | 0.25 | 0 | 0 | 0 | 0.25 | 0.5 |  |
| Fat/Inj/Mis | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Damage to hull | - | - | - | 1 | 1 | - | 1 | 1 | 1 | 1 | 1 | 1 | 1 | - |
| Leakage | - | - | - | 0 | 0 | - | 0 | 0 | 0 | 0 | - | 0 | 0 | - |
| Fault | No | - | - | - | - | - | - | - | Pilots | - | - | - | - | - |
| Ship II - type | Fish. | Pleas. | Pleas. | Other. | Other. | Pleas | Pass. | Tank | Bulk. | Bulk. | Tank. | ? | Bulk | Pleas |
| Nationality | LIT | - | - | GB | DK | - |  | IT | RYSS | DK |  | - | D | - |
| GT | - | - | - | 113 | - | - | 29 691 | 2 349 | 3 086 | 1 275 | 6 060 | - | 4 169 | - |
| Prod year | 1987 | - | - | 1994 | - | - | 1996 | 1993 | 1988 | 1977 | 1981 | - | 1990 | - |
| Pilot | no | - | - | no | no | - | no | no | yes | yes | no | - | no | - |
| Seamen | - | - | - |  | - | - | 88 | - | 12 |  | 15 | - | - | - |
| TotOnboard | - | - | - |  | - | - | 1141 | - | 14 |  | 15 | - | - | - |
| Activity | Sailing | - | - |  | Sailing | - | Sailing | Sailing | Sailing | Sailing | Icebre. | - | Sailing | - |
| Cargo | fish | - | - |  | pass | - | pass | - | bulk | bulk | chem. | - | - | - |
| Fat/Inj/Miss | 0 | 0 | 0 | 0 | 0 | 0 | 0 | - | 0 | 0 | 0 | - | - | - |
| Damage to hull | - | (1) | (1) | 1 | - | (1) | 1 | - | 1 | 0 | 1 | - | - | (1) |
| Leakage | - | - | - | 0 | - | - | 0 | - | 1 | 0 | 0 | - | - | - |

# Limits of compensation

Sea transport is an international business and international treaties at sea have regulated safety for a long time. The most important treaty concerning safety of commercial ships are the SOLAS convention. The first version was adopted in 1914 as a response to the Titanic disaster. The current version was adopted in 1974 but has subsequently been amended (IMO(2002)). In addition, convention exists which regulates the limits of compensation for fatalities and personal injures and oil damages. In section 5.1 below we summarise the limits and the limits are compared to the value of statistical life in section 5.2. A discussion on the consequences for the external cost is made in section 5.3.

## Limits of compensation in Swedish Maritime law

The limits for personal injuries on own passengers are written in the Athens Convention relating to Carriage of Passengers and their Luggage by Sea (PAL) 1974 and on other passenger in the Convention on Limitation of Liability for Maritime Claims 1976. Limits to compensation for oil spills are regulated in the International Convention on Civil Liability for Oil Pollution Damage, 1969 (IMO(2002)). These limits are codified in the Swedish Maritime law (Sjölag (1994:1009)) in section 3.

The law is based on negligence, which means for collision accidents, the responsible parts have to pay the cost of damage. If both are responsible, the cost is split according the proportion of responsibility (8 ch:1§). The shipowner has to pay the cost of an accident caused by an employee but the cost can be reclaimed from the responsible part (7 ch:1§).

The shipowner has the right to limit his liability according to chapter 9 in the law. He can limit the liability for personal injuries and damage to cargo but not for the cost of rescue. The limit of compensation follows from the table below. For oil damages the compensation has two sources, first direct compensation and, secondly, compensation from the international fund described in 1992 years protocol to 1971 international convention about an international fund. The fund covers damages above the shipowers limit

Table 10: Limits of compensation (€)

|  | Limit per certified passenger (€) | Total limit by GT (€) | Total limit € |
| --- | --- | --- | --- |
| Own passengers (§5.1) | 64 399 | - | 34 500 000 |
| Other passenger (GT<500) (§5.2) |  | - | 459 540 |
| per GT from 501-3 000 |  | 690 |  |
| per GT from 3 001-30 000 |  | 460 |  |
| per GT from 30 001-70 000 |  | 357 |  |
| per GT > 70 001 |  | 230 |  |
| Other damages (GT<500) (§5.3) |  | - | 230 460 |
| per GT from 501-30 000 |  | 230 |  |
| per GT from 30 001-70 000 |  | 173 |  |
| per GT > 70 001 |  | 115 |  |
| Damage from oil spill (GT=<5000) (Kap 10 §5) |  | - | 4 140 000 |
| per GT > 5 000 |  | 580 |  |
|  |  | - | 82 386 000 |
| Including compensation from fund |  | - | 186 300 000 |

SDR1=US$ 1.29 = € 1.38 = SEK 12.57 (Source: www.imf.org)

## Limits of compensation compared to value of statistical life

The total limit for compensation to own passengers is 34.5 million €. That means that ‘full’ compensation to a passenger (64 399 €) can be paid to 536 passengers. The ship with most passengers involved in an accident during 1998 had 1 554 passengers (fire). A large ferry, Silja Serenade, may carry 2 852 passengers. If an accident means injuries to passengers on other ships, the total limit is set according to gross tonnage. The largest passenger ship involved in an accident 1998 had a weight of 38 772 GT which means a total limit of 13 million € or 8 427 € per passenger. A large ferry may have a tonnage of 58 000 GT which means a total limit is 25 million € or 8 787 € per passenger.

The limits of compensation for personal injuries are compared to the risk value used in the UNITE accounts. It should be noted that the values used in the accounts are based on the willingness to pay for a reduction of the risk of fatality in a road accident. The willingness to pay for a reduction in the risk of fatality due to a maritime accident could be different. In Table 11 below, the risk value is presented. The cost of property damage are irrelevant for Maritime accidents.

The limits per personal injury of own passenger is 5% of the risk value for fatality, 30% for severe injuries and covers up to four times the cost of slight injuries.

Table 11 Risk value (€)

| Euro | Fatality | Severe Injured | Slight Injured |
| --- | --- | --- | --- |
| Willingness to pay | 1 231 884 | 160 145 | 12 319 |
| Net lost production | 108 210 | 28 516 | 1 561 |
| Hospital care | 3 948 | 21 387 | 1 201 |
| Administration | 6 416 | 1 307 | 601 |
| VOSL | 1 350 458 | 211 355 | 15 682 |
| Limitation compared to VOSL | 5% | 30% | 411% |

# Consequence for the fairway charges

In section 4 above, it was concluded that casualties occurred in two types of accidents that where relevant for a distance-based charge, collision with other objects and collision between ships. In the following, an indication is given what this means for the external marginal cost. We will assume a low cost, where only the system external cost should be included, and a high cost where the limits of compensation is employed.

## Single accidents/collision with other objects

If we assume the risk of single accidents to be constant (E=0) and that only the system external cost is relevant we find an external cost of 0.4 million € for the four fatalities and one injured person in collision accidents with other objects. Compared to the current fairway charges this is 0.4%. The annual cost per ship is 73 €, which is insignificant.

Assuming that the compensation is limited, following the limit per certified passenger above, the external cost increases to 5.4 million € which is around 5% of the current fairway charges. The cost per ship is 10 000 € annually.

## Collision with other ships

In collision between ships, no person was injured during 1998. This means that no externality related to personal injuries arises.

If we assume the individual elasticity to be –0.5, and that the damaged ship is compensated with half the accident cost from the other ship, no externality occur in collision accidents between ships. Although, this conclusion is based on rather uncertain assumption the magnitude of the possible external cost is limited.

## Indication for the fairway charge

Although the fatality risk in maritime transport was seen as surprisingly high, the resulting external cost related to personal injuries is low (0.4% of the fairway charge). Even if we employ a limit on the compensation, the external cost is relatively low (5% of the fairway charge). We have assumed that the fairway charge should internalise the externality due to maritime accidents but excluded personal injuries, illness, and fire accidents.

An externality may occur in accidents between ships. However, collision accidents between ships are only about 10% of the total number of accidents. No person was injured or killed in such accidents during 1998. An externality may occur but that depends on the actual compensation of the cost of hull damage of the involved ships.

In conclusion, for the year 1998 no significant externality occurred in Maritime transport which would suggest any dramatic shift in the fairway charges. We have not included the cost of oil spill in this analysis. Neither have we considered the risk for catastrophic accidents. Due to the limitation of compensation the degree of externality will increase with the size of the accident.

# Conclusion

Reliable estimates on the accident risk in maritime transport are rare. Two reasons can be found, first, maritime transport is a truly international mode and for almost every single trip a number of different territorial waters are passed which make the statistics difficult to collect. Secondly, the traditional measure of exposure, vehicle kilometre, is difficult to find at sea transport. In this paper, we have not been able to improve the estimates on maritime risk.

We have examined detailed accident reports from the Swedish Maritime administration for all accidents with Swedish ship on Swedish water during 1998. It is suggested that the fatality risk at sea are surprisingly high at a first glance. Per Swedish ship, the risk is 1.9 fatalities per ship compared to 0.1 per vehicle in road transport. The fatality risk per passenger kilometre for ferry traffic could nevertheless be around half of the risk in road transport.

A majority of the accidents can be classified as single accidents (90%). Many of the accidents in the statistics are not transport related. Around 70% of the fatalities and personal injuries occurred in ‘non-transport’ related accidents, such as illness or accidents in the kitchen. If these accidents are deducted from the risk estimate presented above the risk will fall in maritime sector.

In Sweden 14 collision accidents happened during 1998 which involved at least one Swedish commercial ship. No person was killed or injured in these accidents.

Very little in the material suggests that the accident risk should be dependent on the traffic volume or distance. A plausible hypothesis is that the ‘congestion’ type of externality, i.e. the fact that the cost changes with traffic volume, may be ignored in maritime accidents in Sweden. However, more collisions happen in narrow water, which suggests that ‘congestion’ could have an influence on the risk.

The second type of externality, the proportion internal and external cost depends on who is the victim, and if he is compensated or not by the injurer. A number of international conventions regulate the level of compensation. It is found that, for the most catastrophic accidents, the cost of accidents is much higher than the total limit. In addition, the limit per passenger is far below the accident cost if the willingness-to-pay based component is included. The limitation means that the compensation, and consequently the internalised cost, is around 5% for a fatal accident and 30% for a severe injury. The conclusion is that the shipowner does not internalise all accident cost in case of catastrophic accidents.

Although the fatality risk in maritime transport was seen as surprisingly high, the resulting external cost related to personal injuries is low and can be estimated to around 0.4% of the current fairway charge. Even if we employ a limit on the compensation, the external cost is relatively low and amounts to 5% of the current fairway charge. We have then assumed that the fairway charge should internalise the externality due to maritime accidents, but excluded personal injuries, illness, and fire accidents.

An externality may occur in accidents between ships. However, collision accidents between ships constitute only about 10% of the total number of accidents. No person was injured or killed in such accidents during 1998. An externality can take place, but that depends on the actual compensation of the cost of hull damage of the involved ships.

In summary, for the year 1998 no significant externality occurred in Maritime transport which would suggest any dramatic shift in the fairway charges. We have not included the cost of oil spill in this analysis. Neither have we considered the risk for catastrophic accidents. It can nevertheless be concluded that due to the limitation of compensation, the degree of externality will increase with the size of the accident.

References

IMO (2002), International Maritime Organisation, www.imo.org (2002-02-27)

Kågesson, P. (2000) Internalisering av sjöfartens externa kostnader PM 2000-11-27.

Lindberg, G. (2001) Traffic insurance and accident externality charges, Journal of Transport Economics and Policy, Vol. 35. Part 3. Sept 2002, pp 399 – 416.

Proposition 2001/02:139 Höjda ansvarsbelopp vid oljeskador till sjöss,¨Stockholm, 7 mars 2002.

Sjöbris, A (1999) Med vilka styrmedel ingår sjöfarten i den europeiska trafikpolitiken? Göteborg oktober 1999, MariTerm AB.

Sjöbris, A., E. Flodstöm, E-W. G. Behm, (1999) Utvärdering av miljödifferentierade avgifter för sjöfarten, Göteborg 13 december 1999, MariTerm AB.

Sjöfartsverket (1999) Sammanställning av rapporterade fartygsolyckor och tillbud samt personolyckor i svenska handels- och fiskefartyg år 1998, Sjöfartsverkets meddelande Nr 2 1999.

Sjöfartsverket (2000) Sjöolyckor i svenska farvatten år 2000, Sjöfartsinspektionen

Sjölagen 1994:1009.

Swahn, H. (2002) Environmentally differentiated fairway charges in practice – the Swedish experience, Swedish Maritime Administration

Includes also combined tanker/bulk ships.

An alternative presentation is to start with the average cost of the charged category (i) and the total cost of other categories (ii). The MC can be written as three components, which can be developed to the same expression as above.

ACj =ra+b+cθ    (i);   TCk =Aa+b+c1−θ   (ii);

MCje =Q∂ACjdQ+∂TCk∂Q+θrc  =θQ∂rdQ(a+b+c)+(1−θ)∂A∂Q(a+c)+θrc  =θrE(a+b+c)+(1−θ)r(1+E)(a+b+c)+θrc=r(a+b)1−θ+E+rc(1+E).

The reports contain one more accident than the summary table 1. This is, however, not important for our conclusions.

Communication with Mikko Malmivuo, VTT, Finland, 21 Mars 2002.

A new law (Prop 2001/02:139) is proposed which increases the limit to 4 510 000 SDR with an increase of 631 SDR/GT above 5 000 GT and an absolute maximum of 89 770 000 SDR. The total limit of compensation, including the fund, is proposed to be 203 000 000 SDR.

The system external cost si 90 679 € per fatality, 38 168 € for severe injury and 2 175 € for light in jury.